3GPP TSG-RAN WG5 Meeting #97	R5-226365r1

Toulouse, France, Nov 14th - 18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2606 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of Test freqs for R15 pending CADC configs from cl 4.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per the endorsed proposals 1-3 in R5-223337, R15 pending CADC configurations in cl 4.3.1 need to be removed |  |  |  |  |  |  |  |  |
| Summary of change: |  | R15 pending CADC configurations in cl 4.3.1 have been removed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The pending configurations may lead to misunderstanding that these configs are of interest to the industry or even ready for validation/certification which is not the fact. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.2.1, 4.3.1.2.3.1, 4.3.1.2.3.4, 4.3.1.2.3.5, 4.3.1.2.4.4, 4.3.1.4.1.2, 4.3.1.4.1.3, 4.3.1.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: DC_48A-48A_n260A config in Table 4.3.1.5.1.2-1 is reserved and kept in the table because this config is already assigned to Verizon as the Interested Operator and marked as “Ongoing” status in RAN5#97. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

4.3.1.1.2	NR inter-band CA configurations in FR1

4.3.1.1.2.1	NR inter-band CA configurations in FR1 (two bands)

Table 4.3.1.1.2.1-1: Inter-band NR CA configurations (FR1, two bands)

| NR CA configuration | Uplink NR CA configuration | NR CA downlink configuration band 1 (Note 3) | NR CA downlink configuration band 2 | NR CA uplink configuration band 1 | NR CA uplink configuration band 2 | Applicable for protocol testing (Note 2) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A | CA_n1A-n3A | n1A | n3A | n1A | n3A | Yes |
| CA_n1A-n8A | CA_n1A-n8A | n1A | n8A | n1A | n8A | Yes |
| CA_n1(2A)-n3A | - | CA_n1(2A) | n3A | n1A | n3A | No |
| CA_n1(2A)-n5A | - | CA_n1(2A) | n5A | - | - | No |
| CA_n1(2A)-n8A | - | CA_n1(2A) | n8A | - | - | No |
| CA_n1A-n77A | - | n1A | n77A | - | - | Yes |
| CA_n1A-n78A | CA_n1A-n78A | n1A | n78A | n1A | n78A | Yes |
| CA_n1A-n78(2A) | CA_n1A-n78A | n1A | CA_n78(2A) | n1A | n78A | No |
| CA_n1A-n78C | CA_n1A-n78A | n1A | CA_n78C | n1A | n78A | No |
| CA_n1(2A)-n78A | - | CA_n1(2A) | n78A | - | - | No |
| CA_n1A-n79A | CA_n1A-n79A | n1A | n79A | n1A | n79A | Yes |
| CA_n2A-n5A | CA_n2A-n5A | n2A | n5A | n2A | n5A | Yes |
| CA_n2A-n48A | CA_n2A-n48A | n2A | n48A | n2A | n48A | Yes |
| CA_n2A-n66A | CA_n2A-n66A | n2A | n66A | n2A | n66A | Yes |
| CA_n2A-n77A | CA_n2A-n77A | n2A | n77A | n2A | n77A | Yes |
| CA_n3A-n5A | CA_n3A-n5A | n3A | n5A | n3A | n5A | Yes |
| CA_n3(2A)-n5A | - | CA_n3(2A) | n5A | - | - | No |
| CA_n3(2A)-n8A | - | CA_n3(2A) | n8A | - | - | No |
| CA_n3A-n41A | CA_n3A-n41A | n3A | n41A | n3A | n41A | Yes |
| CA_n3A-n78A | CA_n3A-n78A | n3A | n78A | n3A | n78A | Yes |
| CA_n3A-n78(2A) | CA_n3A-n78A | n3A | CA_n78(2A) | n3A | n78A | No |
|  | CA_n78(2A) | n3A | CA_n78(2A) | - | CA_n78(2A) | No |
| CA_n3(2A)-n78A | - | CA_n3(2A) | n78A | - | - | No |
| CA_n3A-n79A | CA_n3A-n79A | n3A | n79A | n3A | n79A | Yes |
| CA_n5A-n7A | - | n5A | n7A | - | - | Yes |
| CA_n5A-n48A | CA_n5A-n48A | n5A | n48A | n5A | n48A | Yes |
| CA_n5A-n66A | CA_n5A-n66A | n5A | n66A | n5A | n66A | Yes |
| CA_n5A-n78A | - | n5A | n78A | - | - | Yes |
| CA_n5A-n78(2A) | CA_n5A-n78A | n5A | CA_n78(2A) | n5A | n78A | No |
| CA_n7A-n78A | - | n7A | n78A | - | - | Yes |
| CA_n8A-n78A | CA_n8A-n78A | n8A | n78A | n8A | n78A | Yes |
| CA_n8A-n78(2A) | CA_n8A-n78A | n8A | CA_n78(2A) | n8A | CA_n78(2A) | No |
| CA_n8A-n79A | CA_n8A-n79A | n8A | n79A | n8A | n79A | Yes |
| CA_n24A-n41A | CA_n24A-n41A | n24A | n41A | n24A | n41A | Yes |
| CA_n24A-n41(2A) | CA_n24A-n41A | n24A | CA_n41(2A) | n24A | n41A | No |
| CA_n24A-n48A | CA_n24A-n48A | n24A | n48A | n24A | n48A | Yes |
| CA_n24A-n48B | CA_n24A-n48A | n24A | CA_n48B | n24A | n48A | No |
| CA_n24A-n48(2A) | CA_n24A-n48A | n24A | CA_n48(2A) | n24A | n48A | No |
| CA_n24A-n77A | CA_n24A-n77A | n24A | n77A | n24A | n77A | Yes |
| CA_n24A-n77C | CA_n24A-n77A | n24A | CA_n77C | n24A | n77A | No |
| CA_n26A-n66A | CA_n26A-n66A | n26A | n66A | n26A | n66A | Yes |
| CA_n26A-n66(2A) | CA_n26A-n66A | n26A | CA_n66(2A) | n26A | n66A | No |
| CA_n26A-n70A | CA_n26A-n70A | n26A | n70A | n26A | n70A | Yes |
| CA_n28A-n41A | CA_n28A-n41A | n28A | n41A | n28A | n41A | Yes |
| CA_n28A-n78A | CA_n28A-n78A | n28A | n78A | n28A | n78A | Yes |
| CA_n28A-n79A | CA_n28A-n79A | n28A | n79A | n28A | n79A | Yes |
| CA_n29A-n66A | - | n29A | n66A (Note 1) | - | - | Yes |
| CA_n29B-n66B | - | n29A | CA_n66B (Note 1) | - | - | No |
| CA_n29A-n66(2A) | - | n29A | CA_n66(2A) (Note 1) | - | - | No |
| CA_n29A-n70A | - | n29A | n70A (Note 1) | - | - | Yes |
| CA_n29A-n71A | - | n29A | n71A (Note 1) | - | - | Yes |
| CA_n41A-n79A | CA_n41A-n79A | n41A | n79A | n41A | n79A | Yes |
| CA_n41C-n79A | CA_n41A-n79A | CA_n41C | n79A | n41A | n79A | No |
|  | CA_n41C | CA_n41C | n79A | CA_n41C | - | No |
| CA_n48A-n66A | CA_n48A-n66A | n48A | n66A | n48A | n66A | Yes |
| CA_n48A-n66(2A) | CA_n48A-n66A | n48A | CA_n66(2A) | n48A | n66A | No |
| CA_n48A-n70A | CA_n48A-n70A | n48A | n70A | n48A | n70A | Yes |
| CA_n48A-n71A | CA_n48A-n71A | n48A | n71A | n48A | n71A | Yes |
| CA_n48A-n71(2A) | CA_n48A-n71A | n48A | CA_n71(2A) | n48A | n71A | No |
| CA_n48B-n66A | CA_n48A-n66A | CA_n48B | n66A | n48A | n66A | No |
| CA_n48B-n70A | CA_n48A-n70A | CA_n48B | n70A | n48A | n70A | No |
| CA_n48B-n71A | CA_n48A-n71A | CA_n48B | n71A | n48A | n71A | No |
| CA_n48(2A)-n66A | CA_n48A-n66A | CA_n48(2A) | n66A | n48A | n66A | No |
| CA_n48(2A)-n66(2A) | CA_n48A-n66A | CA_n48(2A) | CA_n66(2A) | n48A | n66A | No |
| CA_n48(2A)-n70A | CA_n48A-n70A | CA_n48(2A) | n70A | n48A | n70A | No |
| CA_n48(2A)-n71A | CA_n48A-n71A | CA_n48(2A) | n71A | n48A | n71A | No |
| CA_n48(2A)-n71(2A) | CA_n48A-n71A | CA_n48(2A) | CA_n71(2A) | n48A | n71A | No |
| CA_n66A-n70A | - | n66A | n70A | - | - | Yes |
| CA_n66B-n70A | - | CA_n66B | n70A | - | - | No |
| CA_n66(2A)-n70A | - | CA_n66(2A) | n70A | - | - | No |
| CA_n66A-n71A | CA_n66A-n71A | n66A | n71A | n66A | n71A | Yes |
| CA_n66A-n71(2A) | CA_n66A-n71A | n66A | CA_n71(2A) | n66A | n71A | No |
| CA_n66B-n71A | CA_n66A-n71A | CA_n66B | n71A | n66A | n71A | No |
| CA_n66(2A)-n71A | CA_n66A-n71A | CA_n66(2A) | n71A | n66A | n71A | No |
| CA_n66(2A)-n71(2A) | CA_n66A-n71A | CA_n66(2A) | CA_n71(2A) | n66A | n71A | No |
| CA_n70A-n71A | CA_n70A-n71A | n70A | n71A | n70A | n71A | Yes |
| CA_n70A-n71(2A) | CA_n70A-n71A | n70A | CA_n71(2A) | n70A | n71A | No |
| CA_n77A-n79A | - | n77A | n79A | - | - | Yes |
| CA_n78A-n79A | - | n78A | n79A | - | - | Yes |
| Note 1: This band is used as PCell. Note 2: Protocol testing is limited to NR CA configurations with 2CC. Note 3: PCell is configured on this band unless otherwise stated. |  |  |  |  |  |  |

<<<UNCHANGED PARTS SKIPPED >>>

4.3.1.2.3.1	NR Intra-band contiguous CA configurations for CA_n257

4.3.1.2.3.1.1	CA_n257B

Editor’s note:	CA_n257B Test frequencies are FFS.

Table 4.3.1.2.3.1.1-1: NR Intra-Band contiguous CA configuration CA_n257B, SCS=120 kHz, ΔFRaster 120 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.3.1.2.3.1.2	CA_n257C

Editor’s note:	CBW=400 MHz for NR band n257 is only supported by for SCS 120kHz. Test frequencies for CA_n257C are currently limited to SCS 120kHz for all CCs. Test frequencies for mixed numerologies between CCs is FFS.

Table 4.3.1.2.3.1.2-1: NR Intra-Band contiguous CA configuration CA_n257C, SCS=120 kHz, ΔFRaster 120 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+400 | CC1 | 50 | 120 | 32 | Downlink | Low | 26525.04 | 2054583 | 26502 | 2054199 | 0 | 120 |  | 22388 | 2054683 |  | 2 |  | 6 | 1 (4) | 20 |
| +400 |  |  |  |  | & | Mid | 27600 | 2072499 | 27430.08 | 2069667 | 102 |  |  | 22450 | 2072539 |  | 8 |  | 3 | 1 (4) | 218 |
|  |  |  |  |  | Uplink | High | 28683 | 2090549 | 27934.2 | 2078069 | 504 |  |  | 22513 | 2090683 |  | 7 |  | 7 | 1 (4) | 1030 |
|  | Channel spacing CC1-CC2=216.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 400 | 120 | 264 | Downlink | Low | 26742 | 2058199 | 26551.92 | 2055031 | 0 | 120 |  | 22391 | 2055547 |  | 6 |  | 7 | 1 (4) | 22 |
|  |  |  |  |  | & | Mid | 27816.96 | 2076115 | 27480 | 2070499 | 102 |  |  | 22453 | 2073403 |  | 0 |  | 5 | 1 (4) | 222 |
|  |  |  |  |  | Uplink | High | 28899.96 | 2094165 | 27984.12 | 2078901 | 504 |  |  | 22515 | 2091259 |  | 11 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC2-CC3=399.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 400 | 120 | 264 | Downlink | Low | 27141.96 | 2064865 | 26951.88 | 2061697 | 0 | 120 |  | 22414 | 2062171 |  | 9 |  | 5 | 1 (4) | 18 |
|  |  |  |  |  | & | Mid | 28216.92 | 2082781 | 27879.96 | 2077165 | 102 |  |  | 22476 | 2080027 |  | 3 |  | 3 | 1 (4) | 218 |
|  |  |  |  |  | Uplink | High | 29299.92 | 2100831 | 28384.08 | 2085567 | 504 |  |  | 22539 | 2098171 |  | 2 |  | 7 | 1 (4) | 1030 |
| 100+400 | CC1 | 100 | 120 | 66 | Downlink | Low | 26550 | 2054999 | 26502.48 | 2054207 | 0 | 120 |  | 22388 | 2054683 |  | 10 |  | 5 | 1 (4) | 18 |
| +400 |  |  |  |  | & | Mid | 27600 | 2072499 | 27405.6 | 2069259 | 102 |  |  | 22448 | 2071963 |  | 8 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 28657.44 | 2090123 | 27884.16 | 2077235 | 504 |  |  | 22510 | 2089819 |  | 4 |  | 6 | 1 (4) | 1028 |
|  | Channel spacing CC1-CC2=242.52 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 400 | 120 | 264 | Downlink | Low | 26792.52 | 2059041 | 26602.44 | 2055873 | 0 | 120 |  | 22393 | 2056123 |  | 5 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 27842.52 | 2076541 | 27505.56 | 2070925 | 102 |  |  | 22454 | 2073691 |  | 3 |  | 3 | 0 (0) | 210 |
|  |  |  |  |  | Uplink | High | 28899.96 | 2094165 | 27984.12 | 2078901 | 504 |  |  | 22515 | 2091259 |  | 11 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC2-CC3=399.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 400 | 120 | 264 | Downlink | Low | 27192.48 | 2065707 | 27002.4 | 2062539 | 0 | 120 |  | 22417 | 2063035 |  | 8 |  | 6 | 1 (4) | 20 |
|  |  |  |  |  | & | Mid | 28242.48 | 2083207 | 27905.52 | 2077591 | 102 |  |  | 22477 | 2080315 |  | 6 |  | 1 | 0 (0) | 206 |
|  |  |  |  |  | Uplink | High | 29299.92 | 2100831 | 28384.08 | 2085567 | 504 |  |  | 22539 | 2098171 |  | 2 |  | 7 | 1 (4) | 1030 |
| 200+400 | CC1 | 200 | 120 | 132 | Downlink | Low | 26600.04 | 2055833 | 26505 | 2054249 | 0 | 120 |  | 22388 | 2054683 |  | 1 |  | 4 | 1 (4) | 16 |
| +400 |  |  |  |  | & | Mid | 27600 | 2072499 | 27358.08 | 2068467 | 102 |  |  | 22446 | 2071387 |  | 8 |  | 5 | 1 (4) | 222 |
|  |  |  |  |  | Uplink | High | 28605 | 2089249 | 27784.2 | 2075569 | 504 |  |  | 22504 | 2088091 |  | 9 |  | 3 | 1 (4) | 1022 |
|  | Channel spacing CC1-CC2=294.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 400 | 120 | 264 | Downlink | Low | 26895 | 2060749 | 26704.92 | 2057581 | 0 | 120 |  | 22399 | 2057851 |  | 3 |  | 1 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 27894.96 | 2077415 | 27558 | 2071799 | 102 |  |  | 22457 | 2074555 |  | 10 |  | 2 | 0 (0) | 208 |
|  |  |  |  |  | Uplink | High | 28899.96 | 2094165 | 27984.12 | 2078901 | 504 |  |  | 22515 | 2091259 |  | 11 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC2-CC3=399.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 400 | 120 | 264 | Downlink | Low | 27294.96 | 2067415 | 27104.88 | 2064247 | 0 | 120 |  | 22423 | 2064763 |  | 6 |  | 7 | 1 (4) | 22 |
|  |  |  |  |  | & | Mid | 28294.92 | 2084081 | 27957.96 | 2078465 | 102 |  |  | 22480 | 2081179 |  | 1 |  | 1 | 0 (0) | 206 |
|  |  |  |  |  | Uplink | High | 29299.92 | 2100831 | 28384.08 | 2085567 | 504 |  |  | 22539 | 2098171 |  | 2 |  | 7 | 1 (4) | 1030 |
| 400+400 | CC1 | 400 | 120 | 264 | Downlink | Low | 26700 | 2057499 | 26509.92 | 2054331 | 0 | 120 |  | 22388 | 2054683 |  | 8 |  | 0 | 1 (4) | 8 |
| +400 |  |  |  |  | & | Mid | 27600 | 2072499 | 27263.04 | 2066883 | 102 |  |  | 22440 | 2069659 |  | 8 |  | 3 | 0 (0) | 210 |
|  |  |  |  |  | Uplink | High | 28500 | 2087499 | 27584.16 | 2072235 | 504 |  |  | 22492 | 2084635 |  | 8 |  | 2 | 0 (0) | 1012 |
|  | Channel spacing CC1-CC2=399.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 400 | 120 | 264 | Downlink | Low | 27099.96 | 2064165 | 26909.88 | 2060997 | 0 | 120 |  | 22411 | 2061307 |  | 11 |  | 2 | 0 (0) | 4 |
|  |  |  |  |  | & | Mid | 27999.96 | 2079165 | 27663 | 2073549 | 102 |  |  | 22463 | 2076283 |  | 11 |  | 1 | 0 (0) | 206 |
|  |  |  |  |  | Uplink | High | 28899.96 | 2094165 | 27984.12 | 2078901 | 504 |  |  | 22515 | 2091259 |  | 11 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC2-CC3=399.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC3 | 400 | 120 | 264 | Downlink | Low | 27499.92 | 2070831 | 27309.84 | 2067663 | 0 | 120 |  | 22434 | 2067931 |  | 2 |  | 1 | 0 (0) | 2 |
|  |  |  |  |  | & | Mid | 28399.92 | 2085831 | 28062.96 | 2080215 | 102 |  |  | 22486 | 2082907 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 29299.92 | 2100831 | 28384.08 | 2085567 | 504 |  |  | 22539 | 2098171 |  | 2 |  | 7 | 1 (4) | 1030 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-2 [8], clause 5.4A.1 for the CC1-CC2 and CC2-CC3 channel bandwidth combinations. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.3.1.3	CA_n257D

Editor’s note:	CA_n257D Test frequencies are FFS.

Table 4.3.1.2.3.1.3-1: NR Intra-Band contiguous CA configuration CA_n257D, SCS=60 kHz, ΔFRaster 60 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 4.3.1.2.3.1.3-2: NR Intra-Band contiguous CA configuration CA_n257D, SCS=120 kHz, ΔFRaster 120 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] |  | point A [MHz] |  | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

<<<UNCHANGED PARTS SKIPPED >>>

4.3.1.2.3.1.9	CA_n257J

Editor’s note:	CA_n257J Test frequencies are FFS.

Table 4.3.1.2.3.1.9-1:Void

Table 4.3.1.2.3.1.9-2: NR Intra-Band contiguous CA configuration CA_n257J, SCS=60 kHz, ΔFRaster 60 kHz, nominal channel spacing

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 4.3.1.2.3.1.9-3: Void

Table 4.3.1.2.3.1.9-4: NR Intra-Band contiguous CA configuration CA_n257J, SCS=120 kHz, ΔFRaster 120 kHz, nominal channel spacing

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.3.1.2.3.1.10	CA_n257K

Editor’s note:	CA_n257K Test frequencies are FFS.

Table 4.3.1.2.3.1.10-1: Void

Table 4.3.1.2.3.1.10-2: NR Intra-Band contiguous CA configuration CA_n257K, SCS=60 kHz, ΔFRaster 60 kHz, nominal channel spacing

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 4.3.1.2.3.1.10-3: Void

Table 4.3.1.2.3.1.10-4: NR Intra-Band contiguous CA configuration CA_n257K, SCS=120 kHz, ΔFRaster 120 kHz, nominal channel spacing

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.3.1.2.3.1.11	CA_n257L

Editor’s note:	CA_n257L Test frequencies are FFS.

Table 4.3.1.2.3.1.11-1: Void

Table 4.3.1.2.3.1.11-2: NR Intra-Band contiguous CA configuration CA_n257L, SCS=60 kHz, ΔFRaster 60 kHz, nominal channel

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 4.3.1.2.3.1.11-3: Void

Table 4.3.1.2.3.1.11-4: NR Intra-Band contiguous CA configuration CA_n257L, SCS=120 kHz, ΔFRaster 120 kHz, nominal channel spacing

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.3.1.2.3.1.12	CA_n257M

Editor’s note:	CA_n257M Test frequencies are FFS.

Table 4.3.1.2.3.1.12-1: Void

Table 4.3.1.2.3.1.12-2: NR Intra-Band contiguous CA configuration CA_n257M, SCS=60 kHz, ΔFRaster 60 kHz, nominal channel

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 4.3.1.2.3.1.12-3: Void

Table 4.3.1.2.3.1.12-4: NR Intra-Band contiguous CA configuration CA_n257M, SCS=120 kHz, ΔFRaster 120 kHz, nominal channel spacing

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | CORESET#1 Offset from [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

<<<UNCHANGED PARTS SKIPPED >>>

4.3.1.2.3.4	NR Intra-band contiguous CA configurations for CA_n260

4.3.1.2.3.4.1	CA_n260B

Editor’s note:	CA_n260B Test frequencies are FFS.

Table 4.3.1.2.3.4.1-1: NR Intra-Band contiguous CA configuration CA_n260B (PCC=CC1 and SCC=CC2), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.3.1.2.3.4.2	CA_n260C

Editor’s note:	CA_n260C Test frequencies are FFS.

Table 4.3.1.2.3.4.2-1: NR Intra-Band contiguous CA configuration CA_n260C (PCC=CC1 and SCC=CC2, CC3), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.3.1.2.3.4.3	CA_n260D

Editor’s note:	CA_n260D Test frequencies are FFS.

Table 4.3.1.2.3.4.3-1: NR Intra-Band contiguous CA configuration CA_n260D (PCC=CC1 and SCC=CC2), SCS=60 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 4.3.1.2.3.4.3-2: NR Intra-Band contiguous CA configuration CA_n260D (PCC=CC1 and SCC=CC2), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

<<<UNCHANGED PARTS SKIPPED >>>

4.3.1.2.3.4.13	CA_n260O

Editor’s note:	CA_n260O Test frequencies are FFS.

Table 4.3.1.2.3.4.13-1: NR Intra-Band contiguous CA configuration CA_n260O (PCC=CC1 and SCC=CC2), SCS=60 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 4.3.1.2.3.4.13-2: NR Intra-Band contiguous CA configuration CA_n260O (PCC=CC1 and SCC=CC2), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.3.1.2.3.4.14	CA_n260P

Editor’s note:	CA_n260P Test frequencies are FFS.

Table 4.3.1.2.3.4.14-1: NR Intra-Band contiguous CA configuration CA_n260P (PCC=CC1 and SCC=CC2, CC3), SCS=60 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 4.3.1.2.3.4.14-2: NR Intra-Band contiguous CA configuration CA_n260P (PCC=CC1 and SCC=CC2, CC3), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.3.1.2.3.4.15	CA_n260Q

FFS

4.3.1.2.3.5	NR Intra-band contiguous CA configurations for CA_n261

4.3.1.2.3.5.1	CA_n261B

Editor’s note:	CA_n261B Test frequencies are FFS.

Table 4.3.1.2.3.5.1-1: NR Intra-Band contiguous CA configuration CA_n261B (PCC=CC1 and SCC=CC2), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.3.1.2.3.5.2	CA_n261C

Editor’s note:	CA_n261C Test frequencies are FFS.

Table 4.3.1.2.3.5.2-1: NR Intra-Band contiguous CA configuration CA_n261C (PCC=CC1 and SCC=CC2, CC3), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.3.1.2.3.5.3	CA_n261D

Editor’s note:	CA_n261D Test frequencies are FFS.

Table 4.3.1.2.3.5.3-1: NR Intra-Band contiguous CA configuration CA_n261D (PCC=CC1 and SCC=CC2), SCS=60 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 4.3.1.2.3.5.3-2: NR Intra-Band contiguous CA configuration CA_n261D (PCC=CC1 and SCC=CC2), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

<<<UNCHANGED PARTS SKIPPED >>>

4.3.1.2.3.5.13	CA_n261O

Editor’s note:	CA_n261O Test frequencies are FFS.

Table 4.3.1.2.3.5.13-1: NR Intra-Band contiguous CA configuration CA_n261O (PCC=CC1 and SCC=CC2), SCS=60 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 4.3.1.2.3.5.13-2: NR Intra-Band contiguous CA configuration CA_n261O (PCC=CC1 and SCC=CC2), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.3.1.2.3.5.14	CA_n261P

Editor’s note:	CA_n261P Test frequencies are FFS.

Table 4.3.1.2.3.5.14-1: NR Intra-Band contiguous CA configuration CA_n261P (PCC=CC1 and SCC=CC2, CC3), SCS=60 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 4.3.1.2.3.5.14-2: NR Intra-Band contiguous CA configuration CA_n261P (PCC=CC1 and SCC=CC2, CC3), SCS=120 kHz, nominal channel spacing

| CBW combination | CC | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 |  | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.3.1.2.3.5.15	CA_n261Q

FFS

4.3.1.2.4	NR intra-band non-contiguous CA configurations in FR2

4.3.1.2.4.1	NR Intra-band non-contiguous CA configurations for CA_n257

4.3.1.2.4.2	NR Intra-band non-contiguous CA configurations for CA_n258

4.3.1.2.4.3	FFS

4.3.1.2.4.4	NR Intra-band non-contiguous CA configurations for CA_n260

4.3.1.2.4.4.1	CA_n260(XA)

Editor's note:	This clause is reserved for test frequencies for CA_n260(XA) configurations where x is >= 2, e.g. CA_n260(2A), CA_n260(3A) and CA_n260(4A)

4.3.1.2.4.4.2	CA_n260(A-I)

Editor’s note:	CA_n260(A-I) Test frequencies are FFS.

Table 4.3.1.2.4.4.2-1: NR Intra-Band non-contiguous CA configuration CA_n260(A-I) without UL CA, SCS=120 kHz, Max Wgap

| CBW combination | Sub-block Note 2 | CC within Sub-Block Note 2 | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 4.3.1.2.4.4.2-2: NR Intra-Band non-contiguous CA configuration CA_n260(A-I) with UL CA, SCS=120 kHz, Max Wgap

| CBW combination | Sub-block Note 2 | CC within Sub-Block Note 2 | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

<<<UNCHANGED PARTS SKIPPED >>>

4.3.1.4	Test frequencies for EN-DC band combinations within FR1

4.3.1.4.1	Inter-band EN-DC configurations within FR1

4.3.1.4.1.1	General

For inter-band EN-DC configurations as listed in this clause, the following apply:

For the E-UTRA band and E-UTRA CA configurations, test frequencies as specified in TS 36.508 [2], clause 4.3.1 are used.

For the NR band and NR CA configurations, test frequencies as specified in clause 4.3.1.1 are used.

For the EN-DC inter-band configuration that includes an EN-DC contiguous configuration (e.g. DC_2A-(n)71AA) the EN-DC contiguous configuration is listed in the NR configuration column and the test frequencies as specified in clause 4.3.1.4.2 are used.

For the secondary NR band in inter-band signalling test cases, the band selected is based on the subset of NR bands supported within the EN-DC configurations specified in Table 4.3.1.4.1.2-1 for NR FR1.

4.3.1.4.1.2	Inter-band EN-DC configurations within FR1 (two bands)

Table 4.3.1.4.1.2-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC Configuration | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n3A | DC_1A_n3A | 1A | n3A | 1A | n3A | Yes |
| DC_1A_n5A | DC_1A_n5A | 1A | n5A | 1A | n5A | Yes |
| DC_1A_n7A | DC_1A_n7A | 1A | n7A | 1A | n7A | Yes |
| DC_1A_n28A | DC_1A_n28A | 1A | n28A | 1A | n28A | Yes |
| DC_1A_n77A | DC_1A_n77A | 1A | n77A | 1A | n77A | Yes |
| DC_1A_n78A | DC_1A_n78A | 1A | n78A | 1A | n78A | Yes |
| DC_1A_n78C | DC_1A_n78A | 1A | CA_n78C | 1A | n78A | Yes (NR 2CC) |
| DC_1A-1A_n78A | DC_1A_n78A | CA_1A-1A | n78A | 1A | n78A | No |
| DC_1A_n79A | DC_1A_n79A | 1A | n79A | 1A | n79A | Yes |
| DC_1A_n79C | DC_1A_n79A | 1A | CA_n79C | 1A | n79A | FFS (NR 2CC) |
| DC_2A_n5A | DC_2A_n5A | 2A | n5A | 2A | n5A | Yes |
| DC_2A_n41A | DC_2A_n41A | 2A | n41A | 2A | n41A | Yes |
| DC_2C_n41A | DC_2A_n41A | CA_2C | n41A | 2A | n41A | No |
|  | DC_2C_n41A | CA_2C | n41A | CA_2C | n41A | No |
| DC_2A_n66A | DC_2A_n66A | 2A | n66A | 2A | n66A | Yes |
| DC_2A_n71A | DC_2A_n71A | 2A | n71A | 2A | n71A | Yes |
| DC_2A_n77A | DC_2A_n77A | 2A | n77A | 2A | n77A | Yes |
| DC_2A_n78A | DC_2A_n78A | 2A | n78A | 2A | n78A | Yes |
| DC_3A_n1A | DC_3A_n1A | 3A | n1A | 3A | n1A | Yes |
| DC_3A_n5A | DC_3A_n5A | 3A | n5A | 3A | n5A | Yes |
| DC_3A_n7A | DC_3A_n7A | 3A | n7A | 3A | n7A | Yes |
| DC_3A_n28A | DC_3A_n28A | 3A | n28A | 3A | n28A | Yes |
| DC_3A_n41A | DC_3A_n41A | 3A | n41A | 3A | n41A | Yes |
| DC_3A_n77A | DC_3A_n77A | 3A | n77A | 3A | n77A | Yes |
| DC_3A_n78A | DC_3A_n78A | 3A | n78A | 3A | n78A | Yes |
| DC_3A_n78C | DC_3A_n78A | 3A | CA_n78C | 3A | n78A | Yes (NR 2CC) |
| DC_3C_n78A | DC_3A_n78A | CA_3C | n78A | 3A | n78A | No |
| DC_3A_n79A | DC_3A_n79A | 3A | n79A | 3A | n79A | Yes |
| DC_3A_n79C | DC_3A_n79A | 3A | CA_n79C | 3A | n79A | FFS (NR 2CC) |
| DC_5A_n2A | DC_5A_n2A | 5A | n2A | 5A | n2A | Yes |
| DC_5A_n66A | DC_5A_n66A | 5A | n66A | 5A | n66A | Yes |
| DC_5A_n77A | DC_5A_n77A | 5A | n77A | 5A | n77A | Yes |
| DC_5A_n78A | DC_5A_n78A | 5A | n78A | 5A | n78A | Yes |
| DC_5A_n78C | DC_5A_n78A | 5A | CA_n78C | 5A | n78A | Yes (NR 2CC) |
| DC_7A-7A_n78A | DC_7A_n78A | CA_7A-7A | n78A | 7A | n78A | No |
| DC_7A_n1A | DC_7A_n1A | 7A | n1A | 7A | n1A | Yes |
| DC_7A_n3A | DC_7A_n3A | 7A | n3A | 7A | n3A | Yes |
| DC_7A_n5A | DC_7A_n5A | 7A | n5A | 7A | n5A | Yes |
| DC_7A_n28A | DC_7A_n28A | 7A | n28A | 7A | n28A | Yes |
| DC_7A_n66A | DC_7A_n66A | 7A | n66A | 7A | n66A | Yes |
| DC_7A_n78A | DC_7A_n78A | 7A | n78A | 7A | n78A | Yes |
| DC_7C_n66A | DC_7A_n66A | CA_7C | n66A | 7A | n66A | No |
| DC_7C_n78A | DC_7A_n78A | CA_7C | n78A | 7A | n78A | No |
|  | DC_7C_n78A | CA_7C | n78A | 7C | n78A | No |
| DC_8A_n1A | DC_8A_n1A | 8A | n1A | 8A | n1A | Yes |
| DC_8A_n3A | DC_8A_n3A | 8A | n3A | 8A | n3A | Yes |
| DC_8A_n20A | DC_8A_n20A | 8A | n20A | 8A | n20A | Yes |
| DC_8A_n41A | DC_8A_n41A | 8A | n41A | 8A | n41A | Yes |
| DC_8A_n77A | DC_8A_n77A | 8A | n77A | 8A | n77A | Yes |
| DC_8A_n78A | DC_8A_n78A | 8A | n78A | 8A | n78A | Yes |
| DC_11A_n77A | DC_11A_n77A | 11A | n77A | 11A | n77A | Yes |
| DC_11A_n78A | DC_11A_n78A | 11A | n78A | 11A | n78A | Yes |
| DC_11A_n79A | DC_11A_n79A | 11A | n79A | 11A | n79A | Yes |
| DC_12A_n5A | DC_12A_n5A | 12A | n5A | 12A | n5A | Yes |
| DC_12A_n66A | DC_12A_n66A | 12A | n66A | 12A | n66A | Yes |
| DC_12A_n78A | DC_12A_n78A | 12A | n78A | 12A | n78A | Yes |
| DC_13A_n2A | DC_13A_n2A | 13A | n2A | 13A | n2A | Yes |
| DC_13A_n66A | DC_13A_n66A | 13A | n66A | 13A | n66A | Yes |
| DC_13A_n77A | DC_13A_n77A | 13A | n77A | 13A | n77A | Yes |
| DC_14A_n2A | DC_14A_n2A | 14A | n2A | 14A | n2A | Yes |
| DC_14A_n66A | DC_14A_n66A | 14A | n66A | 14A | n66A | Yes |
| DC_18A_n77A | DC_18A_n77A | 18A | n77A | 18A | n77A | Yes |
| DC_18A_n78A | DC_18A_n78A | 18A | n78A | 18A | n78A | Yes |
| DC_18A_n79A | DC_18A_n79A | 18A | n79A | 18A | n79A | Yes |
| DC_19A_n1A | DC_19A_n1A | 19A | n1A | 19A | n1A | Yes |
| DC_19A_n77A | DC_19A_n77A | 19A | n77A | 19A | n77A | Yes |
| DC_19A_n77(2A) | DC_19A_n77A | 19A | CA_n77(2A) | 19A | n77A | Yes (NR 2CC) |
| DC_19A_n78A | DC_19A_n78A | 19A | n78A | 19A | n78A | Yes |
| DC_19A_n78(2A) | DC_19A_n78A | 19A | CA_n78(2A) | 19A | n78A | Yes (NR 2CC) |
| DC_19A_n78C | DC_19A_n78A | 19A | CA_n78C | 19A | n78A | Yes (NR 2CC) |
| DC_19A_n79A | DC_19A_n79A | 19A | n79A | 19A | n79A | Yes |
| DC_19A_n79C | DC_19A_n79A | 19A | CA_n79C | 19A | n79A | FFS (NR 2CC) |
| DC_20A_n1A | DC_20A_n1A | 20A | n1A | 20A | n1A | Yes |
| DC_20A_n3A | DC_20A_n3A | 20A | n3A | 20A | n3A | Yes |
| DC_20A_n7A | DC_20A_n7A | 20A | n7A | 20A | n7A | Yes |
| DC_20A_n8A | DC_20A_n8A | 20A | n8A | 20A | n8A | Yes |
| DC_20A_n28A | DC_20A_n28A | 20A | n28A | 20A | n28A | Yes |
| DC_20A_n78A | DC_20A_n78A | 20A | n78A | 20A | n78A | Yes |
| DC_21A_n1A | DC_21A_n1A | 21A | n1A | 21A | n1A | Yes |
| DC_21A_n28A | DC_21A_n28A | 21A | n28A | 21A | n28A | Yes |
| DC_21A_n77A | DC_21A_n77A | 21A | n77A | 21A | n77A | Yes |
| DC_21A_n77(2A) | DC_21A_n77A | 21A | CA_n77(2A) | 21A | n77A | Yes (NR 2CC) |
| DC_21A_n78A | DC_21A_n78A | 21A | n78A | 21A | n78A | Yes |
| DC_21A_n78(2A) | DC_21A_n78A | 21A | CA_n78(2A) | 21A | n78A | Yes (NR 2CC) |
| DC_21A_n78C | DC_21A_n78A | 21A | CA_n78C | 21A | n78A | Yes (NR 2CC) |
| DC_21A_n79A | DC_21A_n79A | 21A | n79A | 21A | n79A | Yes |
| DC_21A_n79C | DC_21A_n79A | 21A | CA_n79C | 21A | n79A | FFS (NR 2CC) |
| DC_25A_n41A | DC_25A_n41A | 25A | n41A | 25A | n41A | Yes |
| DC_26A_n41A | DC_26A_n41A | 26A | n41A | 26A | n41A | Yes |
| DC_26A_n77A | DC_26A_n77A | 26A | n77A | 26A | n77A | Yes |
| DC_26A_n78A | DC_26A_n78A | 26A | n78A | 26A | n78A | Yes |
| DC_26A_n79A | DC_26A_n79A | 26A | n79A | 26A | n79A | Yes |
| DC_28A_n3A | DC_28A_n3A | 28A | n3A | 28A | n3A | Yes |
| DC_28A_n5A | DC_28A_n5A | 28A | n5A | 28A | n5A | Yes |
| DC_28A_n7A | DC_28A_n7A | 28A | n7A | 28A | n7A | Yes |
| DC_28A_n77A | DC_28A_n77A | 28A | n77A | 28A | n77A | Yes |
| DC_28A_n78A | DC_28A_n78A | 28A | n78A | 28A | n78A | Yes |
| DC_28A_n78C | DC_28A_n78A | 28A | CA_n78C | 28A | n78A | Yes (NR 2CC) |
| DC_28A_n79A | DC_28A_n79A | 28A | n79A | 28A | n79A | Yes |
| DC_30A_n5A | DC_30A_n5A | 30A | n5A | 30A | n5A | Yes |
| DC_30A_n66A | DC_30A_n66A | 30A | n66A | 30A | n66A | Yes |
| DC_38A_n78A | - | 38A | n78A | - | - | Yes |
| DC_39A_n41A | DC_39A_n41A | 39A | n41A | 39A | n41A | Yes |
| DC_39A_n79A | DC_39A_n79A | 39A | n79A | 39A | n79A | Yes |
| DC_40A_n1A | DC_40A_n1A | 40A | n1A | 40A | n1A | Yes |
| DC_40A_n41A | DC_40A_n41A | 40A | n41A | 40A | n41A | Yes |
| DC_40A_n78A | DC_40A_n78A | 40A | n78A | 40A | n78A | Yes |
| DC_40C_n78A | DC_40A_n78A | CA_40C | n78A | 40A | n78A | No |
|  | DC_40C_n78A | CA_40C | n78A | CA_40C | n78A | No |
| DC_40A_n79A | DC_40A_n79A | 40A | n79A | 40A | n79A | Yes |
| DC_40C_n79A | DC_40A_n79A | CA_40C | n79A | 40A | n79A | No |
| DC_41A_n77A | DC_41A_n77A | 41A | n77A | 41A | n77A | Yes |
| DC_41C_n77A | DC_41A_n77A | CA_41C | n77A | 41A | n77A | No |
| DC_41A_n78A | DC_41A_n78A | 41A | n78A | 41A | n78A | Yes |
| DC_41C_n78A | DC_41A_n78A | CA_41C | n78A | 41A | n78A | No |
| DC_41A_n79A | DC_41A_n79A | 41A | n79A | 41A | n79A | Yes |
| DC_42A_n1A | DC_42A_n1A | 42A | n1A | 42A | n1A | Yes |
| DC_42C_n1A | DC_42C_n1A | CA_42C | n1A | CA_42C | n1A | No |
| DC_42A_n77A | - | 42A | n77A | - | - | Yes |
| DC_42C_n77A | - | CA_42C | n77A | - | - | No |
| DC_42D_n77A | - | CA_42D | n77A | - | - | No |
| DC_42A_n78A | - | 42A | n78A | - | - | Yes |
| DC_42A_n78C | - | 42A | CA_n78C | - | - | Yes (NR 2CC) |
| DC_42C_n78A | - | CA_42C | n78A | - | - | No |
| DC_42C_n78C | - | CA_42C | CA_n78C | - | - | No |
| DC_42D_n78A | - | CA_42D | n78A | - | - | No |
| DC_42E_n78A | - | CA_42E | n78A | - | - | No |
| DC_42A_n79A | - | 42A | n79A | - | - | Yes |
| DC_42A_n79C | - | 42A | CA_n79C | - | - | FFS (NR 2CC) |
| DC_42C_n79A | - | CA_42C | n79A | - | - | No |
| DC_42C_n79C | - | CA_42C | CA_n79C | - | - | No |
| DC_42D_n79A | - | CA_42D | n79A | - | - | No |
| DC_42E_n79A | - | CA_42E | n79A | - | - | No |
| DC_48A_n5A | DC_48A_n5A | 48A | n5A | 48A | n5A | Yes |
| DC_48A_n66A | DC_48A_n66A | 48A | n66A | 48A | n66A | Yes |
| DC_66A_n2A | DC_66A_n2A | 66A | n2A | 66A | n2A | Yes |
| DC_66A_n5A | DC_66A_n5A | 66A | n5A | 66A | n5A | Yes |
| DC_66A_n41A | DC_66A_n41A | 66A | n41A | 66A | n41A | Yes |
| DC_66A_n71A | DC_66A_n71A | 66A | n71A | 66A | n71A | Yes |
| DC_66A_n77A | DC_66A_n77A | 66A | n77A | 66A | n77A | Yes |
| DC_66A_n78A | DC_66A_n78A | 66A | n78A | 66A | n78A | Yes |
| Note 1: Protocol testing is limited to EN-DC configurations with 1 CC E-UTRA and 1CC or 2CC NR configurations. |  |  |  |  |  |  |

4.3.1.4.1.3	Inter-band EN-DC configurations within FR1 (three bands)

Table 4.3.1.4.1.3-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC Configuration | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A_n28A | DC_1A_n28A | CA_1A-3A | n28A | 1A | n28A | No |
|  | DC_3A_n28A | CA_1A-3A | n28A | 3A | n28A | No |
| DC_1A-3A_n78A | DC_1A_n78A | CA_1A-3A | n78A | 1A | n78A | No |
|  | DC_3A_n78A | CA_1A-3A | n78A | 3A | n78A | No |
| DC_1A-3C_n78A | DC_1A_n78A | CA_1A-3C | n78A | 1A | n78A | No |
|  | DC_3A_n78A | CA_1A-3C | n78A | 3A | n78A | No |
| DC_1A-3C_n78(2A) | DC_1A_n78A | CA_1A-3C | CA_n78(2A) | 1A | n78A | No |
|  | DC_3A_n78A | CA_1A-3C | CA_n78(2A) | 3A | n78A | No |
|  | DC_3C_n78A | CA_1A-3C | CA_n78(2A) | CA_3C | n78A | No |
| DC_1A-1A-3A_n78A | DC_1A_n78A | CA_1A-1A-3A | n78A | 1A | n78A | No |
|  | DC_3A_n78A | CA_1A-1A-3A | n78A | 3A | n78A | No |
| DC_1A-1A-5A_n78A | DC_1A_n78A | CA_1A-1A-5A | n78A | 1A | n78A | No |
|  | DC_5A_n78A | CA_1A-1A-5A | n78A | 5A | n78A | No |
| DC_1A-1A-3C_n78A | DC_1A_n78A | CA_1A-1A-3C | n78A | 1A | n78A | No |
|  | DC_3A_n78A | CA_1A-1A-3C | n78A | 3A | n78A | No |
|  | DC_3C_n78A | CA_1A-1A-3C | n78A | CA_3C | n78A | No |
| DC_1A-3A_n79A | DC_1A_n79A | CA_1A-3A | n79A | 1A | n79A | No |
|  | DC_3A_n79A | CA_1A-3A | n79A | 3A | n79A | No |
| DC_1A-5A_n78C | DC_1A_n78A | CA_1A-5A | CA_n78C | 1A | n78A | No |
|  | DC_5A_n78A | CA_1A-5A | CA_n78C | 5A | n78A | No |
| DC_1A-7A_n3A | DC_1A_n3A | CA_1A-7A | n3A | 1A | n3A | No |
|  | DC_7A_n3A | CA_1A-7A | n3A | 7A | n3A | No |
| DC_1A-7A_n28A | DC_1A_n28A | CA_1A-7A | n28A | 1A | n28A | No |
|  | DC_7A_n28A | CA_1A-7A | n28A | 7A | n28A | No |
| DC_1A-7A_n78A | DC_1A_n78A | CA_1A-7A | n78A | 1A | n78A | No |
|  | DC_7A_n78A | CA_1A-7A | n78A | 7A | n78A | No |
| DC_1A-8A_n3A | DC_1A_n3A | CA_1A-8A | n3A | 1A | n3A | No |
|  | DC_8A_n3A | CA_1A-8A | n3A | 8A | n3A | No |
| DC_1A-8A_n78A | DC_1A_n78A | CA_1A-8A | n78A | 1A | n78A | No |
|  | DC_8A_n78A | CA_1A-8A | n78A | 8A | n78A | No |
| DC_1A-8A_n78(2A) | DC_1A_n78A | CA_1A-8A | CA_n78(2A) | 1A | n78A | No |
|  | DC_8A_n78A | CA_1A-8A | CA_n78(2A) | 8A | n78A | No |
| DC_1A-19A_n77(2A) | DC_1A_n77A | CA_1A-19A | CA_n77(2A) | 1A | n77A | No |
|  | DC_19A_n77A | CA_1A-19A | CA_n77(2A) | 19A | n77A | No |
| DC_1A-19A_n78A | DC_1A_n78A | CA_1A-19A | n78A | 1A | n78A | No |
|  | DC_19A_n78A | CA_1A-19A | n78A | 19A | n78A | No |
| DC_1A-19A_n78(2A) | DC_1A_n78A | CA_1A-19A | CA_n78(2A) | 1A | n78A | No |
|  | DC_19A_n78A | CA_1A-19A | CA_n78(2A) | 19A | n78A | No |
| DC_1A-19A_n79A | DC_1A_n79A | CA_1A-19A | n79A | 1A | n79A | No |
|  | DC_19A_n79A | CA_1A-19A | n79A | 19A | n79A | No |
| DC_1A-20A_n3A | DC_1A_n3A | CA_1A-20A | n3A | 1A | n3A | No |
|  | DC_20A_n3A | CA_1A-20A | n3A | 20A | n3A | No |
| DC_1A-20A_n8A | DC_1A_n8A | CA_1A-20A | n8A | 1A | n8A | No |
|  | DC_20A_n8A | CA_1A-20A | n8A | 20A | n8A | No |
| DC_1A-20A_n28A | DC_1A_n28A | CA_1A-20A | n28A | 1A | n28A | No |
|  | DC_20A_n28A | CA_1A-20A | n28A | 20A | n28A | No |
| DC_1A-20A_n78A | DC_1A_n78A | CA_1A-20A | n78A | 1A | n78A | No |
|  | DC_20A_n78A | CA_1A-20A | n78A | 20A | n78A | No |
| DC_1A-21A_n28A | DC_1A_n28A | CA_1A-21A | n28A | 1A | n28A | No |
|  | DC_21A_n28A | CA_1A-21A | n28A | 21A | n28A | No |
| DC_1A-21A_n77(2A) | DC_1A_n77A | CA_1A-21A | CA_n77(2A) | 1A | n77A | No |
|  | DC_21A_n77A | CA_1A-21A | CA_n77(2A) | 21A | n77A | No |
| DC_1A-21A_n78A | DC_1A_n78A | CA_1A-21A | n78A | 1A | n78A | No |
|  | DC_21A_n78A | CA_1A-21A | n78A | 21A | n78A | No |
| DC_1A-21A_n78(2A) | DC_1A_n78A | CA_1A-21A | CA_n78(2A) | 1A | n78A | No |
|  | DC_21A_n78A | CA_1A-21A | CA_n78(2A) | 21A | n78A | No |
| DC_1A-21A_n79A | DC_1A_n79A | CA_1A-21A | n79A | 1A | n79A | No |
|  | DC_21A_n79A | CA_1A-21A | n79A | 21A | n79A | No |
| DC_1A-28A_n3A | DC_1A_n3A | CA_1A-28A | n3A | 1A | n3A | No |
|  | DC_28A_n3A | CA_1A-28A | n3A | 28A | n3A | No |
| DC_1A-28A_n5A2 | DC_1A_n5A | CA_1A-28A | n5A | 1A | n5A | No |
|  | DC_28A_n5A | CA_1A-28A | n5A | 28A | n5A | No |
| <<<UNCHANGED PARTS SKIPPED >>> |  |  |  |  |  |  |

<<<UNCHANGED PARTS SKIPPED >>>

4.3.1.5	Test frequencies for EN-DC band combinations including FR2

4.3.1.5.1	Inter-band EN-DC configurations including FR2

4.3.1.5.1.1	General

For inter-band EN-DC configurations as listed in this clause, the following apply:

For the E-UTRA band and E-UTRA CA configurations, test frequencies as specified in TS 36.508 [2], clause 4.3.1 are used.

For the NR band and NR CA configurations, test frequencies as specified in clause 4.3.1.2 are used.

For the secondary NR band in inter-band signalling test cases, the band selected is based on the subset of NR bands supported within the EN-DC configurations specified in Table 4.3.1.4.1.2-1 for NR FR1 and 4.3.1.5.1.2-1 for NR FR2.

4.3.1.5.1.2	Inter-band EN-DC configurations including FR2 (two bands)

Table 4.3.1.5.1.2-1: Inter-band EN-DC configurations including FR2 (two bands)

| EN-DC configuration | Uplink EN-DC Configuration | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n257A | DC_1A_n257A | 1A | n257A | 1A | n257A | Yes |
| DC_1A_n257G | DC_1A_n257A | 1A | CA_n257G | 1A | n257A | Yes (NR 2CC) |
|  | DC_1A_n257G | 1A | CA_n257G | 1A | CA_n257G | No |
| DC_1A_n257H | DC_1A_n257A | 1A | CA_n257H | 1A | n257A | No |
|  | DC_1A_n257G | 1A | CA_n257H | 1A | CA_n257G | No |
|  | DC_1A_n257H | 1A | CA_n257H | 1A | CA_n257H | No |
| DC_1A_n257I | DC_1A_n257A | 1A | CA_n257I | 1A | n257A | No |
|  | DC_1A_n257G | 1A | CA_n257I | 1A | CA_n257G | No |
|  | DC_1A_n257H | 1A | CA_n257I | 1A | CA_n257H | No |
|  | DC_1A_n257I | 1A | CA_n257I | 1A | CA_n257I | No |
| DC_2A_n257A | DC_2A_n257A | 2A | n257A | 2A | n257A | Yes |
| DC_2A_n257(2A) | DC_2A_n257A | 2A | CA_n257(2A) | 2A | n257A | FFS (NR 2CC) |
| DC_2A_n260A | DC_2A_n260A | 2A | n260A | 2A | n260A | Yes |
| DC_2A_n260G | DC_2A_n260A | 2A | CA_n260G | 2A | n260A | Yes (NR 2CC) |
| DC_2A_n260H | DC_2A_n260A | 2A | CA_n260H | 2A | n260A | No |
| DC_2A_n260I | DC_2A_n260A | 2A | CA_n260I | 2A | n260A | No |
| DC_2A_n260J | DC_2A_n260A | 2A | CA_n260J | 2A | n260A | No |
| DC_2A_n260K | DC_2A_n260A | 2A | CA_n260K | 2A | n260A | No |
| DC_2A_n260L | DC_2A_n260A | 2A | CA_n260L | 2A | n260A | No |
| DC_2A_n260M | DC_2A_n260A | 2A | CA_n260M | 2A | n260A | No |
| DC_2C_n260A | DC_2A_n260A | CA_2C | n260A | 2A | n260A | No |
| DC_2A_n260(2A) | DC_2A_n260A | 2A | CA_n260(2A) | 2A | n260A | FFS (NR 2CC) |
| DC_2A-2A_n260A | DC_2A_n260A | CA_2A-2A | n260A | 2A | n260A | No |
| DC_2A-2A_n260G | DC_2A_n260A | CA_2A-2A | CA_n260G | 2A | n260A | No |
| DC_2A-2A_n260H | DC_2A_n260A | CA_2A-2A | CA_n260H | 2A | n260A | No |
| DC_2A-2A_n260I | DC_2A_n260A | CA_2A-2A | CA_n260I | 2A | n260A | No |
| DC_2A-2A_n260J | DC_2A_n260A | CA_2A-2A | CA_n260J | 2A | n260A | No |
| DC_2A-2A_n260K | DC_2A_n260A | CA_2A-2A | CA_n260K | 2A | n260A | No |
| DC_2A-2A_n260L | DC_2A_n260A | CA_2A-2A | CA_n260L | 2A | n260A | No |
| DC_2A-2A_n260M | DC_2A_n260A | CA_2A-2A | CA_n260M | 2A | n260A | No |
| DC_3A_n257A | DC_3A_n257A | 3A | n257A | 3A | n257A | Yes |
| DC_3A_n257D | DC_3A_n257A | 3A | CA_n257D | 3A | n257A | FFS (NR 2CC) |
| DC_3A_n257G | DC_3A_n257A | 3A | CA_n257G | 3A | n257A | Yes (NR 2CC) |  |
|  | DC_3A_n257B | 3A | CA_n257G | 3A | CA_n257B | No |  |
|  | DC_3A_n257D | 3A | CA_n257G | 3A | CA_n257D | No |  |
|  | DC_3A_n257G | 3A | CA_n257G | 3A | CA_n257G | No |  |
| DC_3A_n257H | DC_3A_n257A | 3A | CA_n257H | 3A | n257A | No |  |
|  | DC_3A_n257B | 3A | CA_n257H | 3A | CA_n257B | No |  |
|  | DC_3A_n257D | 3A | CA_n257H | 3A | CA_n257D | No |  |
|  | DC_3A_n257G | 3A | CA_n257H | 3A | CA_n257G | No |  |
|  | DC_3A_n257H | 3A | CA_n257H | 3A | CA_n257H | No |  |
| DC_3A_n257I | DC_3A_n257A | 3A | CA_n257I | 3A | n257A | No |  |
|  | DC_3A_n257B | 3A | CA_n257I | 3A | CA_n257B | No |  |
|  | DC_3A_n257D | 3A | CA_n257I | 3A | CA_n257D | No |  |
|  | DC_3A_n257G | 3A | CA_n257I | 3A | CA_n257G | No |  |
|  | DC_3A_n257H | 3A | CA_n257I | 3A | CA_n257H | No |  |
|  | DC_3A_n257I | 3A | CA_n257I | 3A | CA_n257I | No |  |
| DC_5A_n257A | DC_5A_n257A | 5A | n257A | 5A | n257A | Yes |
| DC_5A_n260A | DC_5A_n260A | 5A | n260A | 5A | n260A | Yes |
| DC_5A_n260B | DC_5A_n260A | 5A | CA_n260B | 5A | n260A | FFS (NR 2CC) |
| DC_5A_n260C | DC_5A_n260A | 5A | CA_n260C | 5A | n260A | No |
| DC_5A_n260D | DC_5A_n260A | 5A | CA_n260D | 5A | n260A | FFS (NR 2CC) |
| DC_5A_n260E | DC_5A_n260A | 5A | CA_n260E | 5A | n260A | No |
| DC_5A_n260F | DC_5A_n260A | 5A | CA_n260F | 5A | n260A | No |
| DC_5A_n260G | DC_5A_n260A | 5A | CA_n260G | 5A | n260A | Yes (NR 2CC) |
| DC_5A_n260H | DC_5A_n260A | 5A | CA_n260H | 5A | n260A | No |
| DC_5A_n260I | DC_5A_n260A | 5A | CA_n260I | 5A | n260A | No |
| DC_5A_n260J | DC_5A_n260A | 5A | CA_n260J | 5A | n260A | No |
| DC_5A_n260K | DC_5A_n260A | 5A | CA_n260K | 5A | n260A | No |
| DC_5A_n260L | DC_5A_n260A | 5A | CA_n260L | 5A | n260A | No |
| DC_5A_n260M | DC_5A_n260A | 5A | CA_n260M | 5A | n260A | No |
| DC_5A_n260O | DC_5A_n260A | 5A | CA_n260O | 5A | n260A | FFS (NR 2CC) |
| DC_5B_n260A | DC_5A_n260A | CA_5B | n260A | 5A | n260A | No |
|  | DC_5B_n260A | CA_5B | n260A | CA_5B | n260A | No |
| DC_5A_n260(2A) | DC_5A_n260A | 5A | CA_n260(2A) | 5A | n260A | FFS (NR 2CC) |
| DC_5A_n260(3A) | DC_5A_n260A | 5A | CA_n260(3A) | 5A | n260A | No |
| DC_5A_n260(4A) | DC_5A_n260A | 5A | n260(4A) | 5A | n260A | No |
| DC_5A_n260(A-I) | DC_5A_n260A | 5A | CA_n260(A-I) | 5A | n260A | No |
| DC_5A_n260(G-I) | DC_5A_n260A | 5A | CA_n260(G-I) | 5A | n260A | No |
| DC_5A-5A_n260A | DC_5A_n260A | CA_5A-5A | n260A | 5A | n260A | No |
| DC_5A_n261A | DC_5A_n261A | 5A | n261A | 5A | n261A | Yes |
| DC_5A_n261B | DC_5A_n261A | 5A | CA_n261B | 5A | n261A | FFS (NR 2CC) |
| DC_5A_n261C | DC_5A_n261A | 5A | CA_n261C | 5A | n261A | No |
| DC_5A_n261D | DC_5A_n261A | 5A | CA_n261D | 5A | n261A | FFS (NR 2CC) |
| DC_5A_n261E | DC_5A_n261A | 5A | CA_n261E | 5A | n261A | No |
| DC_5A_n261F | DC_5A_n261A | 5A | CA_n261F | 5A | n261A | No |
| DC_5A_n261G | DC_5A_n261A | 5A | CA_n261G | 5A | n261A | Yes (NR 2CC) |
| DC_5A_n261H | DC_5A_n261A | 5A | CA_n261H | 5A | n261A | No |
| DC_5A_n261I | DC_5A_n261A | 5A | CA_n261I | 5A | n261A | No |
| DC_5A_n261J | DC_5A_n261A | 5A | CA_n261J | 5A | n261A | No |
| DC_5A_n261K | DC_5A_n261A | 5A | CA_n261K | 5A | n261A | No |
| DC_5A_n261L | DC_5A_n261A | 5A | CA_n261L | 5A | n261A | No |
| DC_5A_n261M | DC_5A_n261A | 5A | CA_n261M | 5A | n261A | No |
| DC_5A_n261O | DC_5A_n261A | 5A | CA_n261O | 5A | n261A | FFS (NR 2CC) |
| DC_5A_n261P | DC_5A_n261A | 5A | CA_n261P | 5A | n261A | No |
| DC_5A_n261Q | DC_5A_n261A | 5A | CA_n261Q | 5A | n261A | No |
| DC_5A_n261(2A) | DC_5A_n261A | 5A | CA_n261(2A) | 5A | n261A | Yes (NR 2CC) |
| DC_5A_n261(3A) | DC_5A_n261A | 5A | CA_n261(3A) | 5A | n261A | No |
| DC_5A_n261(4A) | DC_5A_n261A | 5A | CA_n261(4A) | 5A | n261A | No |
| DC_5A_n261(D-G) | DC_5A_n261A | 5A | CA_n261(D-G) | 5A | n261A | No |
| DC_5A_n261(D-H) | DC_5A_n261A | 5A | CA_n261(D-H) | 5A | n261A | No |
| DC_5A_n261(D-I) | DC_5A_n261A | 5A | CA_n261(D-I) | 5A | n261A | No |
| DC_5A_n261(D-O) | DC_5A_n261A | 5A | CA_n261(D-O) | 5A | n261A | No |
| DC_5A_n261(D-P) | DC_5A_n261A | 5A | CA_n261(D-P) | 5A | n261A | No |
| DC_5A_n261(D-Q) | DC_5A_n261A | 5A | CA_n261(D-Q) | 5A | n261A | No |
| DC_5A_n261(E-O) | DC_5A_n261A | 5A | CA_n261(E-O) | 5A | n261A | No |
| DC_5A_n261(E-P) | DC_5A_n261A | 5A | CA_n261(E-P) | 5A | n261A | No |
| DC_5A_n261(E-Q) | DC_5A_n261A | 5A | CA_n261(E-Q) | 5A | n261A | No |
| DC_7A_n257A | DC_7A_n257A | 7A | n257A | 7A | n257A | Yes |
| DC_7A-7A_n257A | DC_7A_n257A | CA_7A-7A | n257A | 7A | n257A | No |
| DC_8A_n257A | DC_8A_n257A | 8A | n257A | 8A | n257A | Yes |
| DC_8A_n257D | DC_8A_n257A | 8A | CA_n257D | 8A | n257A | FFS (NR 2CC) |
| DC_8A_n257E | DC_8A_n257A | 8A | CA_n257E | 8A | n257A | No |
| DC_8A_n257F | DC_8A_n257A | 8A | CA_n257F | 8A | n257A | No |
| DC_8A_n257G | DC_8A_n257A | 8A | CA_n257G | 8A | n257A | Yes (NR 2CC) |
| DC_8A_n257H | DC_8A_n257A | 8A | CA_n257H | 8A | n257A | No |
| DC_8A_n257I | DC_8A_n257A | 8A | CA_n257I | 8A | n257A | No |
| DC_8A_n257J | DC_8A_n257A | 8A | CA_n257J | 8A | n257A | No |
| DC_8A_n257K | DC_8A_n257A | 8A | CA_n257K | 8A | n257A | No |
| DC_8A_n257L | DC_8A_n257A | 8A | CA_n257L | 8A | n257A | No |
| DC_8A_n257M | DC_8A_n257A | 8A | CA_n257M | 8A | n257A | No |
| DC_8A_n258A | DC_8A_n258A | 8A | n258A | 8A | n258A | Yes |
| DC_11A_n257A | DC_11A_n257A | 11A | n257A | 11A | n257A | Yes |
| DC_12A_n260A | DC_12A_n260A | 12A | n260A | 12A | n260A | Yes |
| DC_14A_n260A | DC_14A_n260A | 14A | n260A | 14A | n260A | Yes |
| DC_14A_n260G | DC_14A_n260A | 14A | CA_n260G | 14A | n260A | Yes (NR 2CC) |
|  | DC_14A_n260G | 14A | CA_n260G | 14A | CA_n260G | No |
| DC_14A_n260H | DC_14A_n260A | 14A | CA_n260H | 14A | n260A | No |
|  | DC_14A_n260G | 14A | CA_n260H | 14A | CA_n260G | No |
|  | DC_14A_n260H | 14A | CA_n260H | 14A | CA_n260H | No |
| DC_14A_n260I | DC_14A_n260A | 14A | CA_n260I | 14A | n260A | No |
|  | DC_14A_n260G | 14A | CA_n260I | 14A | CA_n260G | No |
|  | DC_14A_n260H | 14A | CA_n260I | 14A | CA_n260H | No |
|  | DC_14A_n260I | 14A | CA_n260I | 14A | CA_n260I | No |
| DC_12A_n260G | DC_12A_n260A | 12A | CA_n260G | 12A | n260A | Yes (NR 2CC) |
| DC_12A_n260H | DC_12A_n260A | 12A | CA_n260H | 12A | n260A | No |
| DC_12A_n260I | DC_12A_n260A | 12A | CA_n260I | 12A | n260A | No |
| DC_12A_n260J | DC_12A_n260A | 12A | CA_n260J | 12A | n260A | No |
| DC_12A_n260K | DC_12A_n260A | 12A | CA_n260K | 12A | n260A | No |
| DC_12A_n260L | DC_12A_n260A | 12A | CA_n260L | 12A | n260A | No |
| DC_12A_n260M | DC_12A_n260A | 12A | CA_n260M | 12A | n260A | No |
| DC_13A_n257A | DC_13A_n257A | 13A | n257A | 13A | n257A | Yes |
| DC_13A_n260A | DC_13A_n260A | 13A | n260A | 13A | n260A | Yes |
| DC_18A_n257A | DC_18A_n257A | 18A | n257A | 18A | n257A | Yes |
| DC_19A_n257A | DC_19A_n257A | 19A | n257A | 19A | n257A | Yes |
| DC_19A_n257D | DC_19A_n257A | 19A | CA_n257D | 19A | n257A | FFS (NR 2CC) |
| DC_19A_n257G | DC_19A_n257A | 19A | CA_n257G | 19A | n257A | Yes (NR 2CC) |  |
|  | DC_19A_n257G | 19A | CA_n257G | 19A | CA_n257G | No |  |
| DC_19A_n257H | DC_19A_n257A | 19A | CA_n257H | 19A | n257A | No |  |
|  | DC_19A_n257G | 19A | CA_n257H | 19A | CA_n257G | No |  |
|  | DC_19A_n257H | 19A | CA_n257H | 19A | CA_n257H | No |  |
| DC_19A_n257I | DC_19A_n257A | 19A | CA_n257I | 19A | n257A | No |  |
|  | DC_19A_n257G | 19A | CA_n257I | 19A | CA_n257G | No |  |
|  | DC_19A_n257H | 19A | CA_n257I | 19A | CA_n257H | No |  |
|  | DC_19A_n257I | 19A | CA_n257I | 19A | CA_n257I | No |  |
| DC_20A_n257A | DC_20A_n257A | 20A | n257A | 20A | n257A | Yes |  |
| DC_21A_n257A | DC_21A_n257A | 21A | n257A | 21A | n257A | Yes |
| DC_21A_n257D | DC_21A_n257A | 21A | CA_n257D | 21A | n257A | FFS (NR 2CC) |
| DC_21A_n257G | DC_21A_n257A | 21A | CA_n257G | 21A | n257A | Yes (NR 2CC) |  |
|  | DC_21A_n257G | 21A | CA_n257G | 21A | CA_n257G | No |  |
| DC_21A_n257H | DC_21A_n257A | 21A | CA_n257H | 21A | n257A | No |  |
|  | DC_21A_n257G | 21A | CA_n257H | 21A | CA_n257G | No |  |
|  | DC_21A_n257H | 21A | CA_n257H | 21A | CA_n257H | No |  |
| DC_21A_n257I | DC_21A_n257A | 21A | CA_n257I | 21A | n257A | No |  |
|  | DC_21A_n257G | 21A | CA_n257I | 21A | CA_n257G | No |  |
|  | DC_21A_n257H | 21A | CA_n257I | 21A | CA_n257H | No |  |
|  | DC_21A_n257I | 21A | CA_n257I | 21A | CA_n257I | No |  |
| DC_28A_n257A | DC_28A_n257A | 28A | n257A | 28A | n257A | Yes |
| DC_30A_n260A | DC_30A_n260A | 30A | n260A | 30A | n260A | Yes |
| DC_30A_n260G | DC_30A_n260A | 30A | CA_n260G | 30A | n260A | Yes (NR 2CC) |
| DC_30A_n260H | DC_30A_n260A | 30A | CA_n260H | 30A | n260A | No |
| DC_30A_n260I | DC_30A_n260A | 30A | CA_n260I | 30A | n260A | No |
| DC_30A_n260J | DC_30A_n260A | 30A | CA_n260J | 30A | n260A | No |
| DC_30A_n260K | DC_30A_n260A | 30A | CA_n260K | 30A | n260A | No |
| DC_30A_n260L | DC_30A_n260A | 30A | CA_n260L | 30A | n260A | No |
| DC_30A_n260M | DC_30A_n260A | 30A | CA_n260M | 30A | n260A | No |
| DC_41A_n257A | DC_41A_n257A | 41A | n257A | 41A | n257A | Yes |
|  | DC_41C_n257A | CA_41C | n257A | CA_41C | n257A | No |
| DC_42A_n257A | DC_42A_n257A | 42A | n257A | 42A | n257A | Yes |
| DC_42C_n257A | DC_42A_n257A | CA_42C | n257A | 42A | n257A | No |
|  | DC_42C_n257A | CA_42C | n257A | CA_42C | n257A | No |
| DC_42A_n257D | DC_42A_n257A | 42A | CA_n257D | 42A | n257A | FFS (NR 2CC) |
| DC_42C_n257D | DC_42A_n257A | CA_42C | CA_n257D | 42A | n257A | No |
|  | DC_42C_n257A | CA_42C | CA_n257D | CA_42C | n257A | No |
| DC_42D_n257A | DC_42A_n257A | CA_42D | n257A | 42A | n257A | No |
|  | DC_42C_n257A | CA_42D | n257A | CA_42C | n257A | No |
| DC_42E_n257A | DC_42A_n257A | CA_42E | n257A | 42A | n257A | No |
|  | DC_42C_n257A | CA_42E | n257A | CA_42C | n257A | No |
| DC_48A_n260A | DC_48A_n260A | 48A | n260A | 48A | n260A | Yes |
| DC_48C_n260A | DC_48A_n260A | CA_48C | n260A | 48A | n260A | No |
|  | DC_48C_n260A | CA_48C | n260A | CA_48C | n260A | No |
| DC_48A-48A_n260A | DC_48A_n260A | CA_48A-48A | n260A | 48A | n260A | No |
| DC_66A_n257A | DC_66A_n257A | 66A | n257A | 66A | n257A | Yes |
| DC_66A_n257(2A) | DC_66A_n257A | 66A | CA_n257(2A) | 66A | n257A | FFS (NR 2CC) |
| DC_66A-66A_n257A | DC_66A_n257A | CA_66A-66A | n257A | 66A | n257A | No |
| DC_66A_n260A | DC_66A_n260A | 66A | n260A | 66A | n260A | Yes |
| DC_66A_n260D | DC_66A_n260A | 66A | CA_n260D | 66A | n260A | FFS (NR 2CC) |
| DC_66A_n260E | DC_66A_n260A | 66A | CA_n260E | 66A | n260A | No |
| DC_66A_n260F | DC_66A_n260A | 66A | CA_n260F | 66A | n260A | No |
| DC_66A_n260G | DC_66A_n260A | 66A | CA_n260G | 66A | n260A | Yes (NR 2CC) |
| DC_66A_n260H | DC_66A_n260A | 66A | CA_n260H | 66A | n260A | No |
| DC_66A_n260I | DC_66A_n260A | 66A | CA_n260I | 66A | n260A | No |
| DC_66A_n260J | DC_66A_n260A | 66A | CA_n260J | 66A | n260A | No |
| DC_66A_n260K | DC_66A_n260A | 66A | CA_n260K | 66A | n260A | No |
| DC_66A_n260L | DC_66A_n260A | 66A | CA_n260L | 66A | n260A | No |
| DC_66A_n260M | DC_66A_n260A | 66A | CA_n260M | 66A | n260A | No |
| DC_66A_n260O | DC_66A_n260A | 66A | CA_n260O | 66A | n260A | FFS (NR 2CC) |
| DC_66A_n260(2A) | DC_66A_n260A | 66A | CA_n260(2A) | 66A | n260A | FFS (NR 2CC) |
| DC_66A_n260(3A) | DC_66A_n260A | 66A | CA_n260(3A) | 66A | n260A | No |
| DC_66A_n260(4A) | DC_66A_n260A | 66A | CA_n260(4A) | 66A | n260A | No |
| DC_66A_n260(A-I) | DC_66A_n260A | 66A | CA_n260(A-I) | 66A | n260A | No |
| DC_66A_n260(G-I) | DC_66A_n260A | 66A | CA_n260(G-I) | 66A | n260A | No |
| DC_66A-66A_n260A | DC_66A_n260A | CA_66A-66A | n260A | 66A | n260A | No |
| DC_66A-66A_n260G | DC_66A_n260A | CA_66A-66A | CA_n260G | 66A | n260A | No |
| DC_66A-66A_n260H | DC_66A_n260A | CA_66A-66A | CA_n260H | 66A | n260A | No |
| DC_66A-66A_n260I | DC_66A_n260A | CA_66A-66A | CA_n260I | 66A | n260A | No |
| DC_66A-66A_n260J | DC_66A_n260A | CA_66A-66A | CA_n260J | 66A | n260A | No |
| DC_66A-66A_n260K | DC_66A_n260A | CA_66A-66A | CA_n260K | 66A | n260A | No |
| DC_66A-66A_n260L | DC_66A_n260A | CA_66A-66A | CA_n260L | 66A | n260A | No |
| DC_66A-66A_n260M | DC_66A_n260A | CA_66A-66A | CA_n260M | 66A | n260A | No |
| DC_66A_n261A | DC_66A_n261A | 66A | n261A | 66A | n261A | Yes |
| DC_66A_n261D | DC_66A_n261A | 66A | CA_n261D | 66A | n261A | FFS (NR 2CC) |
| DC_66A_n261E | DC_66A_n261A | 66A | CA_n261E | 66A | n261A | No |
| DC_66A_n261F | DC_66A_n261A | 66A | CA_n261F | 66A | n261A | No |
| DC_66A_n261G | DC_66A_n261A | 66A | CA_n261G | 66A | n261A | Yes (NR 2CC) |
| DC_66A_n261H | DC_66A_n261A | 66A | CA_n261H | 66A | n261A | No |
| DC_66A_n261I | DC_66A_n261A | 66A | CA_n261I | 66A | n261A | No |
| DC_66A_n261J | DC_66A_n261A | 66A | CA_n261J | 66A | n261A | No |
| DC_66A_n261K | DC_66A_n261A | 66A | CA_n261K | 66A | n261A | No |
| DC_66A_n261L | DC_66A_n261A | 66A | CA_n261L | 66A | n261A | No |
| DC_66A_n261M | DC_66A_n261A | 66A | CA_n261M | 66A | n261A | No |
| DC_66A_n261O | DC_66A_n261A | 66A | CA_n261O | 66A | n261A | FFS (NR 2CC) |
| DC_66A_n261P | DC_66A_n261A | 66A | CA_n261P | 66A | n261A | No |
| DC_66A_n261Q | DC_66A_n261A | 66A | CA_n261Q | 66A | n261A | No |
| DC_66A_n261(2A) | DC_66A_n261A | 66A | CA_n261(2A) | 66A | n261A | Yes (NR 2CC) |
| DC_66A_n261(3A) | DC_66A_n261A | 66A | CA_n261(3A) | 66A | n261A | No |
| DC_66A_n261(4A) | DC_66A_n261A | 66A | CA_n261(4A) | 66A | n261A | No |
| DC_66A_n261(D-G) | DC_66A_n261A | 66A | CA_n261(D-G) | 66A | n261A | No |
| DC_66A_n261(D-H) | DC_66A_n261A | 66A | CA_n261(D-H) | 66A | n261A | No |
| DC_66A_n261(D-I) | DC_66A_n261A | 66A | CA_n261(D-I) | 66A | n261A | No |
| DC_66A_n261(D-O) | DC_66A_n261A | 66A | CA_n261(D-O) | 66A | n261A | No |
| DC_66A_n261(D-P) | DC_66A_n261A | 66A | CA_n261(D-P) | 66A | n261A | No |
| DC_66A_n261(D-Q) | DC_66A_n261A | 66A | CA_n261(D-Q) | 66A | n261A | No |
| DC_66A_n261(E-O) | DC_66A_n261A | 66A | CA_n261(E-O) | 66A | n261A | No |
| DC_66A_n261(E-P) | DC_66A_n261A | 66A | CA_n261(E-P) | 66A | n261A | No |
| DC_66A_n261(E-Q) | DC_66A_n261A | 66A | CA_n261(E-Q) | 66A | n261A | No |
| Note 1: Protocol testing is limited to EN-DC configurations with 1 CC E-UTRA and 1CC or 2CC NR configurations. |  |  |  |  |  |  |

<< END OF CHANGES >>