3GPP TSG-RAN5 Meeting #97	R5-226564r1

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3273 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new Idle mode TC 6.1.2.15a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies, Verizon Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of new Idle mode TC 6.1.2.15a to cover requirements according to the discussion paper R5-226546. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New TC 6.1.2.15a Cell reselection in shared network environment / Cells broadcasting multiple PLMN IDs with unique TAC's, RAN areas, and cell identities is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some common deployment scenarios test coverage will be missing in the Idle mode WP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.15a(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-2 CR0279 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-226564r1: Missing Source to TSG added References to TC 6.1.2.15 is clarified in TP, conformance requirement and Pre-test conditions |  |  |  |  |  |  |  |  |

#### 6.1.2.15a	Cell reselection in shared network environment / Cells broadcasting multiple PLMN IDs with unique TAC's, RAN areas, and cell identities

6.1.2.15a.1	Test Purpose (TP)

Same as TC 6.1.2.15.1

6.1.2.15a.2	Conformance requirements

Same as TC 6.1.2.15.1

6.1.2.15a.3	Test description

6.1.2.15a.3.1	Pre-test conditions

Same as TC 6.1.2.15.1

6.1.2.15a.3.2	Test procedure sequence

Same as TC 6.1.2.15.1

6.1.2.15a.3.3	Specific message contents

Table 6.1.2.15a.3.3-1: SIB1 for NR Cell 1 (Preamble and all steps, Table 6.1.2.15a.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { |  |  |  |  |
| plmn-Identity[1] | Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.1.2.15a.3.3-2: SIB1 for NR Cell 11 (Preamble and all steps, Table 6.1.2.15a.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| PLMN-IdentityInfoList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo { | 2 entries |  |  |
| PLMN-IdentityInfo[1] SEQUENCE { | entry 1 |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { |  |  |  |
| plmn-Identity[1] | Set to the same Mobile Country Code stored in EFIMSI on the test USIM, MNC=02 | Same MCC like PLMN for NR Cell 1 but different MNC |  |
| } |  |  |  |
| trackingAreaCode | '00000000 00000000 00000010'B, |  |  |
| ranac | 2 |  |  |
| cellIdentity | '00000000 00000000 00000001 10000000 1011'B, |  |  |
| } |  |  |  |
| PLMN-IdentityInfo[2] SEQUENCE { | entry 2 |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { |  |  |  |
| plmn-Identity[1] | Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card | This is the same PLMN as NR Cell 1 |  |
| } |  |  |  |
| trackingAreaCode | ''00000000 00000000 00000011'B, |  |  |
| ranac | 1 |  |  |
| cellIdentity | ''00000000 00000000 00000001 01000000 1010'B, |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.15a.3.3-3: RRCSetupComplete (step 4, Table 6.1.2.15a.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupComplete::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |  |
| selectedPLMN-Identity |  | 2 | HPLMN |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |