3GPP TSG-RAN5 Meeting #97	R5-226661r1

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0395 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS for Redcap CSI test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Israel Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current version of the spec has ambiguity on the applicability of RedCap CSI test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified the Rx and Layer requirement for RedCap UE supporting FR1 and or FR2. Clarified HD-FDD PICS is only applicable for FR1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Applicability of CSI test cases for RedCap will remain unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: added the missing clause in coversheet Removed changes related to overlap contents in R5-226951 |  |  |  |  |  |  |  |  |

### A.4.3.12	RedCap Capabilities

According to TS 38.306 [17] clause 4.2.21:

-	PICS associated to the following features shall not be set to 'true':

-	CA: PICS defined in clause A.4.3.2A;

-	MR-DC: PICS defined in clause A.4.3.2B;

-	DAPS: PICS include pc_intraFreqDAPS_r16, interFreqDAPS_r16 and other DAPS related PICS;

-	CPAC: PICS include pc_condPSCellChange_r16 and other CPAC related PICS.

-	A RedCap UE shall not support features and corresponding capabilities related to more than 2 UE Rx branches or more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 1 UE Tx branches or more than 1 UL MIMO layers are not supported.

-	For FR1, 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported.

-	For FR2, either 1 or 2 DL MIMO layers can be supported, while 2 Rx branches are always supported.

Table A.4.3.12-1: RedCap Capabilities

| Item | UE Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support of 16 DRBs for RedCap UEs. | 38.306, 4.2.21.2 | Rel-17 | pc_16DRB_RedCap _r17 | No |  |  |
| 2 | Support of RedCap. | 38.306, 4.2.21.2 | Rel-17 | pc_RedCap_r17 | No |  |  |
| 3 | Support of 18 bit length of PDCP sequence for RedCap UEs. | 38.306, 4.2.21.3 | Rel-17 | pc_longSN_RedCap_r17 | No |  |  |
| 4 | Support of AM DRB with 18 bit length of RLC sequence number for RedCap UEs. | 38.306, 4.2.21.4 | Rel-17 | pc_ amWithLongSN_RedCap_r17 | No |  |  |
| 5 | Support of Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE. | 38.306 4.2.21.6.1 | Rel-17 | pc_halfDuplexFDD_TypeA_RedCap_r17 | No |  | FDD FR1 only |