3GPP TSG-RAN5 Meeting #97	R5-226812r1

# Toulouse, France, 14th - 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3307 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR V2X TC 12.2.1.5 Inter-carrier concurrent operation / Sidelink communication / RRC_CONNECTED / Transmission / Exceptional pool |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| nrNR |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | TDIA, CATT, Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of new NR V2X TC12.2.1.5 Inter-carrier concurrent operation / Sidelink communication / RRC_CONNECTED / Transmission / Exceptional pool. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New NR V2X TC added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete compared to work plan coverage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.1.5(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-2 CR 0280 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Fixed editorial issues, changed Test Mode (On) instead of Test Loop Function (On). |  |  |  |  |  |  |  |  |

<Start of modified section>

12.2.1.5	Inter-carrier concurrent operation / Sidelink communication / RRC_CONNECTED / Transmission / Exceptional pool

12.2.1.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and performing NR sidelink tranmission based on network scheduling with sl-ScheduledConfig, and sl-TxPoolExceptional is included in sl-ConfigDedicatedNR }

ensure that {

when { UE receives an RRCReconfiguration message including reconfigurationWithSync }

then { UE performs NR sidelink transmission using sl-TxPoolExceptional included in sl-ConfigDedicatedNR during handover procedure }

}

(2)

with { UE in RRC_CONNECTED state and performing NR sidelink tranmission based on network scheduling with sl-ScheduledConfig, and sl-TxPoolExceptional is included in sl-ConfigDedicatedNR }

ensure that {

when { UE detects physical layer problems by receiving N310 consecutive out-of-sync indications from lower layers }

then { UE performs NR sidelink transmission using sl-TxPoolExceptional included in sl-ConfigDedicatedNR until receiving RRCReestablishment message }

}

12.2.1.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 [22], subclause 5.3.5.14 and 5.8.8. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, subclause 5.3.5.14]

Upon initiating the procedure, the UE shall:

….

1>	if sl-FreqInfoToAddModList is included in sl-ConfigDedicatedNR within RRCReconfiguration:

2>	if configured to receive NR sidelink communication:

3>	use the resource pool(s) indicated by sl-RxPool for NR sidelink communication reception, as specified in 5.8.7;

2>	if configured to transmit NR sidelink communication:

3>	use the resource pool(s) indicated by sl-TxPoolSelectedNormal, sl-TxPoolScheduling or sl-TxPoolExceptional for NR sidelink communication transmission, as specified in 5.8.8;

[TS 38.331, subclause 5.8.8]

A UE capable of NR sidelink communication that is configured by upper layers to transmit NR sidelink communication and has related data to be transmitted shall:

1>	if the conditions for NR sidelink communication operation as defined in 5.8.2 are met:

2>	if the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12:

3>	if the UE is in RRC_CONNECTED and uses the frequency included in sl-ConfigDedicatedNR within RRCReconfiguration message:

4>	if the UE is configured with sl-ScheduledConfig:

5>	if T310 for MCG or T311 is running; and if sl-TxPoolExceptional is included in sl-FreqInfoList for the concerned frequency in SIB12 or included in sl-ConfigDedicatedNR in RRCReconfiguration; or

5>	if T301 is running and the cell on which the UE initiated RRC connection re-establishment provides SIB12 including sl-TxPoolExceptional for the concerned frequency; or

5>	if T304 for MCG is running and the UE is configured with sl-TxPoolExceptional included in sl-ConfigDedicatedNR for the concerned frequency in RRCReconfiguration:

6>	configure lower layers to perform the sidelink resource allocation mode 2 based on random selection using the pool of resources indicated by sl-TxPoolExceptional as defined in TS 38.321 [3];

5>	else:

6>	configure lower layers to perform the sidelink resource allocation mode 1 for NR sidelink communication;

5>	if T311 is running, configure the lower layers to release the resources indicated by rrc-ConfiguredSidelinkGrant (if any);

4>	if the UE is configured with sl-UE-SelectedConfig:

5>	if a result of sensing on the resources configured in sl-TxPoolSelectedNormal for the concerned frequency included in sl-ConfigDedicatedNR within RRCReconfiguration is not available in accordance with TS 38.214 [19];

6>	if sl-TxPoolExceptional for the concerned frequency is included in RRCReconfiguration; or

6>	if the PCell provides SIB12 including sl-TxPoolExceptional in sl-FreqInfoList for the concerned frequency:

7>	configure lower layers to perform the sidelink resource allocation mode 2 based on random selection using the pool of resources indicated by sl-TxPoolExceptional as defined in TS 38.321 [3];

5>	else, if the sl-TxPoolSelectedNormal for the concerned frequency is included in the sl-ConfigDedicatedNR within RRCReconfiguration:

6>	configure lower layers to perform the sidelink resource allocation mode 2 based on resource selection operation according to sl-AllowedResourceSelectionConfig (as defined in TS 38.321 [3] and TS 38.214 [19]) using the pools of resources indicated by sl-TxPoolSelectedNormal for the concerned frequency;

3>	else:

4>	if the cell chosen for NR sidelink communication transmission provides SIB12:

5>	if SIB12 includes sl-TxPoolSelectedNormal for the concerned frequency, and a result of sensing on the resources configured in the sl-TxPoolSelectedNormal is available in accordance with TS 38.214 [19]:

6>	configure lower layers to perform the sidelink resource allocation mode 2 based on resource selection operation according to sl-AllowedResourceSelectionConfig using the pools of resources indicated by sl-TxPoolSe	lectedNormal for the concerned frequency as defined in TS 38.321 [3];

5>	else if SIB12 includes sl-TxPoolExceptional for the concerned frequency:

6>	from the moment the UE initiates RRC connection establishment or RRC connection resume, until receiving an RRCReconfiguration including sl-ConfigDedicatedNR, or receiving an RRCRelease or an RRCReject; or

6>	if a result of sensing on the resources configured in sl-TxPoolSelectedNormal for the concerned frequency in SIB12 is not available in accordance with TS 38.214 [19]:

7>	configure lower layers to perform the sidelink resource allocation mode 2 based on random selection (as defined in TS 38.321 [3]) using one of the pools of resources indicated by sl-TxPoolExceptional for the concerned frequency;

2>	else:

3>	configure lower layers to perform the sidelink resource allocation mode 2 based on sensing (as defined in TS 38.321 [3] and TS 38.213 [13]) using the pools of resources indicated by sl-TxPoolSelectedNormal in SidelinkPreconfigNR for the concerned frequency.

12.2.1.5.3	Test description

12.2.1.5.3.1	Pre-test conditions

System Simulator:

-	SS-NW

-	NR Cell 1, NR Cell 2.

-	System information combination NR-14 as defined in TS 38.508-1 [4] clause 4.4.3.1 is used in NR Cells.

-	NR-SS-UE

-	NR-SS-UE1: operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3 except for those listed in Table 12.2.1.5.3.1-1.

- The UE uses GNSS as the synchronization reference source.

Table 12.2.1.5.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | service no. 119 V2X is available |  |
| EFVST |  | Service no.2 V2X policy configuration data over PC5 is supported, i.e. value is '01 02' HEX |  |
| EFV2XP_PC5 |  | SL-PreconfigurationNR field as defined in TS 38.508-1 [4] Table 4.10.1-1 |  |

Preamble:

-	The UE is in state 3N-B RRC_CONNECTED_with_SL and Test Mode (On) with UE test loop mode E as defined in TS 38.508-1 [4] subclause 4.4A on NR Cell 1 using generic procedure defined in TS 38.508-1[4] clause 4.5.4 with parameters Sidelink (On), Cast Type (Unicast), GNSS Sync (On) using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22.

12.2.1.5.3.2	Test procedure sequence

Table 12.2.1.5.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 12.2.1.5.3.2-2.

Table 12.2.1.5.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -91 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -79 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -85 | off |  |

Table 12.2.1.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS-NW sends an RRCReconfiguration message with sl-ConfigDedicatedNR including sl-ScheduledConfig and sl-TxPoolExceptional. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | UE sends an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS transmits a CLOSE UE TEST LOOP message | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 4 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 5 | Check: Does UE send SDAP SDUs on SL DRB#n in slots included in exceptional resource pool? | - | - | 1 | F |
| 6 | SS-NW adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 7 | The SS-NW transmits an RRCReconfiguration message including reconfigurationWithSync to order the UE to perform inter-frequency handover to NR Cell 2. | <-- | NR RRC: RRCReconfiguration | - | - |
| - | EXCEPTION: In parallel with step 8, parallel behaviour defined in table 12.2.1.5.3.2-3 is executed repeatedly. | - | - | - | - |
| 8 | Check: Does UE send SDAP SDUs on SL DRB#n in slots included in exceptional resource pool? | --> | - | 1 | P |
| 9 | The SS-NW transmits Random Access Response to respond to the latest preamble on NR Cell 2. | <-- | Random Access Response |  |  |
| 10 | The UE transmits RRCReconfigurationComplete message in NR Cell 2. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 11 | The SS transmits an OPEN UE TEST LOOP message | <-- | TC: OPEN UE TEST LOOP | - | - |
| 12 | The UE transmits an OPEN UE TEST LOOP COMPLETE message | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 13 | SS-NW sends an RRCReconfiguration message with sl-ConfigDedicatedNR including sl-ScheduledConfig and sl-TxPoolExceptional. | <-- | NR RRC: RRCReconfiguration | - | - |
| 14 | UE sends an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 15 | The SS transmits a CLOSE UE TEST LOOP message | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 16 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 17 | Check: Does UE send SDAP SDUs on SL DRB#n in slots included in exceptional resource pool? | - | - | 2 | F |
| 18 | SS-NW adjusts the cell-specific reference signal level according to row "T2". | - | - | - | - |
| - | EXCEPTION: In parallel with step 19, parallel behaviour defined in table 12.2.1.5.3.2-4 is executed. | - | - | - | - |
| 19 | Check: Does UE send SDAP SDUs on SL DRB#n in slots included in exceptional resource pool? | --> | - | 2 | P |
| 20 | The SS-NW transmits RRCReestablishment message. | <-- | NR RRC: RRCReestablishment | - | - |
| 21 | The UE transmits RRCReestablishmentComplete message. | --> | NR RRC: RRCReestablishmentComplete | - | - |
| 22 | The SS-NW transmits an RRCReconfiguration message to resume existing radio bearer. | <-- | NR RRC: RRCReconfiguration | - | - |
| 23 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 24 | The SS transmits an OPEN UE TEST LOOP message | <-- | TC: OPEN UE TEST LOOP | - | - |
| 25 | The UE transmits an OPEN UE TEST LOOP COMPLETE message | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |

Table 12.2.1.5.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits preamble to NR Cell 2. | --> | (PRACH Preamble) | - | - |

Table 12.2.1.5.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits preamble to NR Cell 1. | --> | (PRACH Preamble) | - | - |
| 2 | The SS-NW transmits Random Access Response to respond to the latest preamble on NR Cell 1. | <-- | Random Access Response | - | - |
| 3 | The UE sends RRCReestablishmentRequest message on NR Cell 1. | --> | NR RRC: RRCReestablishmentRequest | - | - |

12.2.1.5.3.3	Specific message contents

Table 12.2.1.5.3.3-1: SL-BWP-ConfigCommon in SIB12 for Cell 1 and Cell 2 (Preamble and all steps)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-BWP-ConfigCommon-r16 ::= SEQUENCE { |  |  |  |  |
| sl-BWP-PoolConfigCommon-r16 |  | SL-BWP-PoolConfigCommon with RXPOOL, SELECTED and EXCEPTIONAL |  |  |
| } |  |  |  |  |

Table 12.2.1.5.3.3-2: SIB1 for Cell 1 and Cell 2 (Preamble and all steps)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants SEQUENCE { |  |  |  |
| t301 | ms2000 |  |  |
| t310 | ms2000 |  |  |
| t311 | ms30000 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.2.1.5.3.3-3: RLF-TimersAndConstants (Preamble and all steps)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms2000 |  |  |
| t311 | ms30000 |  |  |
| } |  |  |  |

Table 12.2.1.5.3.3-4: RRCReconfiguration (step 1 and step 13, Table 12.2.1.5.3.2-2)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sl-ConfigDedicatedNR-r16 | SL-ConfigDedicatedNR with condition SCHEDULING |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.2.1.5.3.3-5: SL-BWP-Config (Table 12.2.1.5.3.3-4)

| Derivation path: TS 38.508-1 [4], Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SL-BWP-Config-r16 ::= SEQUENCE { |  |  |  |
| sl-BWP-PoolConfig-r16 | SL-BWP-PoolConfig with condition SCHEDULING and EXCEPTIONAL |  |  |
| } |  |  |  |

Table 12.2.1.5.3.3-6: RRCReconfiguration (step 7, Table 12.2.1.5.3.2-2)

| Derivation Path: TS 38.508-1 [4] Table 4.8.1-1A with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | OCTET STRING (CONTAINING CellGroupConfig) | Table 12.2.1.5.3.3-7 |  |
| masterKeyUpdate SEQUENCE { |  |  |  |
| keySetChangeIndicator | True |  |  |
| nextHopChainingCount | 0 |  |  |
| nas-Container | NASContainer | Intra N1 mode NAS transparent container, Table 12.2.1.5.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.2.1.5.3.3-7: CellGroupConfig (Table 12.2.1.5.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { | Same as default ServingCellConfigCommon |  |  |
| physCellId | Physical cell Id of NR Cell 2 |  |  |
| } |  |  |  |
| t304 | ms2000 |  |  |
| rach-ConfigDedicated CHOICE { |  |  |  |
| Uplink SEQUENCE { | Same as default RACH-ConfigDedicated |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric | Table 12.2.1.5.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| rlf-TimersAndConstants CHOICE { | RLF-TimersAndConstants | Table 12.2.1.5.3.3-3 |  |
| setup |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.2.1.5.3.3-8: RACH-ConfigGeneric (Table 12.2.1.5.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| ra-ResponseWindow | sl80 |  |  |
| } |  |  |  |

Table 12.2.1.5.3.3-9: NASContainer (Table 12.2.1.5.3.3-6)

| Derivation Path: TS 24.501, table 9.11.2.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Message authentication code | The calculated value of MAC-I for this message. | The value of MAC-I is calculated by SS using COUNT = 0xFFFFFFFF( as per TS 33.501[20], 6.9.2.3.3) |  |
| Type of ciphering algorithm | Set according to PIXIT px_NAS_5GC_CipheringAlgorithm for default ciphering algorithm |  |  |
| Type of integrity protection algorithm | Set according to PIXIT px_NAS_5GC_IntegrityAlgorithm for default integrity protection algorithm | This value should not be equal to the null integrity algorithm. |  |
| KACF | ‘1’B | a new KAMF has been calculated by the network |  |
| TSC | '0'B | native security context (for KSIAMF) |  |
| Key set identifier in 5G | KSIAMF that was created when the UE last registered to 5GCN |  |  |
| Sequence number | The internal counter of the SS | eight least significant bits of the downlink NAS COUNT |  |

Table 12.2.1.5.3.3-10: CLOSE UE TEST LOOP (step 3 and step 15, Table 12.2.1.5.3.2-2)

Derivation Path: TS 38.508-1 [4] Table 4.9.22.3-2

Table 12.2.1.5.3.3-11: RRCReestablishmentRequest (step 3, Table 12.2.1.5.3.2-4)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentRequest ::= SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of NR Cell 2 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS-NW |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |

<End of modified section>