3GPP TSG-RAN5 Meeting #97	R5-227299r1

# Toulouse, France,  14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2050 | rev | - | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of CA_n71A-n77A Spurious emissions requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DISH Network |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CA_n71A-n77A is ongoing in PRD21, but missing from this section of specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added CA_n71A-77A to relevant 6.5A.3 sections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA_n71A-n77A conformance requirement are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A.3.1, 6.5A.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP analysis provided in R5-227279r1 r1: Test point alignments |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.5A.3	Spurious emission for CA

#### 6.5A.3.1	General spurious emissions for CA

Editor’s Note:. The following aspects are either missing or not yet determined:

- Extending the coverage of the TCs with intra-band CA scenarios is FFS

##### 6.5A.3.1.0	Minimum conformance requirements

For inter-band carrier aggregation with uplink assigned to two NR bands, the spurious emission requirement Table 6.5.3.1.3-2 apply for the frequency ranges that are more than FOOB as defined in Table 6.5.3.1.3-1 away from edges of the assigned channel bandwidth on a component carrier. If for some frequency a spurious emission requirement of individual component carrier overlaps with the spectrum emission mask or channel bandwidth of another component carrier then it does not apply.

NOTE 1:	For inter-band carrier aggregation with uplink assigned to two NR bands the requirements in Table 6.5.3.1.3-2 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5.3.1.3-2 would be considered to be verified by the measurements verifying the one uplink inter-band CA spurious emission requirement.

For intra-band contiguous carrier aggregation the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5A.3.1.0-1 from the edge of the aggregated channel bandwidth. For frequencies ΔfOOB greater than FOOB as specified in Table 6.5A.3.1.0-1 the spurious emission requirements in Table 6.5.3.1.3-2 are applicable.

Table 6.5A.3.1.0-1: Boundary between out of band and spurious emission domain for intra-band contiguous carrier aggregation

| Aggregated Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| BWChannel_CA | BWChannel_CA + 5 |

For intra-band non-contiguous carrier aggregation transmission the spurious emission requirement is defined as a composite spurious emission requirement. Composite spurious emission requirement applies to frequency ranges that are more than FOOB away from the edges of each carrier in the gap and out of the gap. Composite spurious emission requirement is defined as follows

a)	Composite spurious emission requirement is a combination of individual sub-block spurious emission requirements

b)	In case the sub-block consist of one component carrier the sub-lock spurious emission requirement and FOOB are defined in subclause 6.5.3.1

c)	If for some frequency an individual sub-block spurious emission requirement overlaps with the general spectrum emission mask or the sub-block bandwidth of another sub-block then it does not apply

For the signalling is absent for dualPA-Architecture IE, if carrier leakage or I/Q image lands inside the gap spectrum between 2 UL CCs when UL CCs are synchronized with frequencies in the gap, exception to the general spurious requirement applies.

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5A.3.1

##### 6.5A.3.1.1	General spurious emissions for CA (2UL CA)

6.5A.3.1.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5A.3.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2UL CA.

6.5A.3.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.3.1.0.

6.5A.3.1.1.4	Test description

6.5A.3.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.3.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.1.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg for both PCC and SCC Highest NRB_agg for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  | Lowest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A | CP-OFDM QPSK | Outer_Full | Outer_Full |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A3-1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.3.1.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5A.3.1.1.4.3.

6.5A.3.1.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.1.1. Message contents are defined in clause 6.5A.3.1.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5A.3.1.1.4.1-1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5A.3.1.1.5-1. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5A.3.1.1.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots.

6.5A.3.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.5A.3.1.1.4.3-1: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 16 |  | Power class 3 and Inter-band CA |

6.5A.3.1.1.5	Test requirement

This clause specifies the requirements for the specified NR band for Transmitter Spurious emissions requirement with frequency range as indicated in Table 6.5A.3.1.1.5-1. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth. If for some frequency a spurious emission requirement of individual component carrier overlaps with the spectrum emission mask or channel bandwidth of another component carrier then it does not apply.

The measured average power of spurious emission, derived in step 6, shall not exceed the described value in Table 6.5A.3.1.1.5-1.

Table 6.5A.3.1.1.5-1: General spurious emissions test requirements

| Frequency Range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| Test requirements for CA_n3A-n78A Configuration |  |  |  |
| 270 MHz ≤ f ≤ 380 MHz | -36 dBm+TT | 100 kHz |  |
| 1515MHz ≤ f ≤ 2090 MHz 3270 MHz ≤ f ≤ 3830 MHz 4815 MHz ≤ f ≤ 5890 MHz 6720 MHz ≤ f ≤ 7370 MHz 8310 MHz ≤ f ≤ 9385 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n8A-n78A Configuration |  |  |  |
| 780 MHz ≤f ≤1000 MHz | -36 dBm+TT | 100 kHz |  |
| 1000MHz £ f £ 1015 MHz 1470 MHz £ f £ 2040 MHz 2385 MHz ≤ f ≤ 2920 MHz 3290 MHz ≤ f ≤ 3810 MHz 4180 MHz ≤ f ≤4715 MHz 5060 MHz ≤ f ≤5630 MHz 5685 MHz ≤ f ≤6720 MHz 7480 MHz ≤ f ≤8515 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n24A-n41A Configuration |  |  |  |
| 563 MHz ≤ f ≤ 825 MHz 835.5 MHz ≤ f ≤ 1000 MHz | -36 dBm+TT | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1063.5 MHz 3331.5 MHz ≤ f ≤ 3753.5 MHz 4122.5 MHz ≤ f ≤ 4350.5 MHz 5749 MHz ≤ f ≤ 6011 MHz 6618.5 MHz ≤ f ≤ 7040.5 MHz | -25 dBm+TT | 1 MHz |  |
| Test requirements for CA_n24A-n48A Configuration |  |  |  |
| 229 MHz ≤ f ≤ 447 MHz | -36 dBm+TT | 100 kHz |  |
| 1889.5 MHz ≤ f ≤ 2073.5 MHz 5176.5 MHz ≤ f ≤ 5360.5 MHz 5439.5 MHz ≤ f ≤ 5773.5 MHz 6803 MHz ≤ f ≤ 7021 MHz 8726.5 MHz ≤ f ≤ 9060.5 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n24A-n77A Configuration |  |  |  |
| 21 MHz ≤ f < 30 MHz | -36 dBm+TT | 10 kHz |  |
| 30 MHz ≤ f < 947 MHz | -36 dBm+TT | 100 kHz |  |
| 1639.5 MHz ≤ f ≤ 2573.5 MHz 4926.5 MHz ≤ f ≤ 7521 MHz 8226.5 MHz ≤ f ≤ 10060.5 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n26A-n66A Configuration |  |  |  |
| 12 MHz ≤ f < 30 MHz | -36 dBm+TT | 10 kHz |  |
| 30 MHz ≤ f ≤ 152 MHz 861 MHz ≤ f ≤ 966 MHz | -36 dBm+TT | 100 kHz |  |
| 2524 MHz ≤ f ≤ 2746 MHz 3338 MHz ≤ f ≤ 3478 MHz 4234 MHz ≤ f ≤ 4409 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n26A-n70A Configuration |  |  |  |
| 3 MHz ≤ f < 30 MHz | -36 dBm+TT | 10 kHz |  |
| 30 MHz ≤ f < 82 MHz | -36 dBm+TT | 100 kHz |  |
| 846 MHz ≤ f ≤ 896 MHz | -36 dBm+TT | 100 kHz |  |
| 2509 MHz ≤ f ≤ 2606 MHz 3323 MHz ≤ f ≤ 3408 MHz 4204 MHz ≤ f ≤ 4269 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n28A-n41A Configuration |  |  |  |
| 1000MHz ≤ f ≤ 1284 MHz 1748MHz ≤ f ≤ 1987 MHz 3199MHz ≤ f ≤ 3438 MHz 3902MHz ≤ f ≤ 4186 MHz 4244MHz ≤ f ≤ 4677 MHz 5695MHz ≤ f ≤ 6128MHz | -25 dBm+TT | 1MHz |  |
| Test requirements for CA_n41A-n79A Configuration |  |  |  |
| 8 MHz ≤ f ≤30 MHz | -36 dBm+TT | 10kHz |  |
| 30 MHz ≤ f ≤980 MHz | -36 dBm+TT | 100kHz |  |
| 1710MHz ≤ f ≤ 2504MHz | -30 dBm+TT | 1MHz |  |
| 6110MHz ≤ f ≤ 7690MHz |  |  |  |
| 9392MHz ≤ f ≤ 10380MHz |  |  |  |
| 11296MHz ≤ f ≤ 12690MHz |  |  |  |
| Test requirements for CA_n48A-n66A Configuration |  |  |  |
| 10 MHz ≤ f < 30 MHz | -36 dBm+TT | 10 kHz |  |
| 30 MHz ≤ f < 280 MHz | -36 dBm+TT | 100 kHz |  |
| 1770 MHz ≤ f ≤ 1990 MHz 5260 MHz ≤ f ≤ 5690 MHz 6970 MHz ≤ f ≤ 7260 MHz 8810 MHz ≤ f ≤ 9180 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n48A-n70A Configuration |  |  |  |
| 130 MHz ≤ f ≤ 310 MHz | -36 dBm+TT | 100 kHz |  |
| 1840 MHz ≤ f ≤ 2005 MHz 5245 MHz ≤ f ≤ 5705 MHz 6940 MHz ≤ f ≤ 7120 MHz 8795 MHz ≤ f ≤ 9110 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n48A-n71A Configuration |  |  |  |
| 2154 MHz ≤ f ≤ 2374 MHz 2852 MHz ≤ f ≤ 3037 MHz 4213 MHz ≤ f ≤ 4398 MHz 4876 MHz ≤ f ≤ 5096 MHz 6402 MHz ≤ f ≤ 6737 MHz 7763 MHz ≤ f ≤ 8098 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n66A-n71A Configuration |  |  |  |
| 314 MHz ≤ f < 454 MHz | -36 dBm+TT | 100 kHz |  |
| 1012 MHz ≤ f ≤ 1117 MHz 2373 MHz ≤ f ≤ 2478 MHz 2722 MHz ≤ f ≤ 2897 MHz 3036 MHz ≤ f ≤ 3176 MHz 4083 MHz ≤ f ≤ 4258 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n70A-n71A Configuration |  |  |  |
| 299 MHz ≤ f < 384 MHz 997 MHz ≤ f < 1000 MHz | -36 dBm+TT | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1047 MHz 2358 MHz ≤ f ≤ 2408 MHz 2692 MHz ≤ f ≤ 2757 MHz 3021 MHz ≤ f ≤ 3106 MHz 4053 MHz ≤ f ≤ 4118 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n71A-n77A Configuration |  |  |  |
| 1904 MHz ≤ f ≤ 3537 MHz 3963 MHz ≤ f ≤ 5596 MHz 5902 MHz ≤ f ≤ 9098 MHz | -30 dBm+TT | 1 MHz |  |

Table 6.5A.3.1.1.5-2: Test Tolerance for General spurious emissions

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0 | 0 |
| 40MHz < BW ≤ 100MHz | 0 | 0 |

#### 6.5A.3.2	Spurious emission for UE co-existence

Editor’s note: The following aspects are either missing or not yet determined:

- Extending the coverage of the TCs with intra-band CA scenarios is FFS.

##### 6.5A.3.2.0	Minimum conformance requirements

###### 6.5A.3.2.0.1	Spurious emissions for UE co-existence for intra-band contiguous CA

This clause specifies the requirements for the specified intra-band contiguous carrier aggregation configurations for coexistence with protected bands, the requirements in Table 6.5A.3.2.0.1-1 apply.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5A.3.2.0.1-1: Requirements for uplink intra-band contiguous carrier aggregation

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1, 2, 3 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1, 2, 3 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 1, 2 |
| CA_n41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2, 4 |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 6 |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 5, 6 |
| CA_n48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| CA_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| CA_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 5 |
| NOTE 1: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 2: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 3: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 4: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 6: This requirement applies when the NR carrier is confined within 2545 – 2575 MHz or 2595 – 2645 MHz and the channel bandwidth is 10 or 20 MHz |  |  |  |  |  |  |  |

###### 6.5A.3.2.0.2	Void

###### 6.5A.3.2.0.3	Spurious emissions for UE co-existence for Inter-band CA

For inter-band carrier aggregation with the uplink assigned to two NR bands, the requirements in Table 6.5A.3.2.0.3-1 and Table 6.5A.3.2.0.3-2 apply on each component carrier with both component carriers are active.

NOTE:	For inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in Table 6.5A.3.2.0.3-1 and Table 6.5A.3.2.0.3-2 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5A.3.2.0.3-1 and Table 6.5A.3.2.0.3-2 would be considered to be verified by the measurements verifying the one uplink inter-band CA UE to UE co-existence requirements.

Table 6.5A.3.2.0.3-1: Requirements for uplink inter-band carrier aggregation (two bands)

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n28 | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -30 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n40 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | NR band n77, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 4, 14 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 4, 7, 14 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 4, 7, 14 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n1-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n3-n28 | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31 38, 40, 41, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 32, 42, 43, 50, 51, 74, 75, 76 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -30 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 | 4 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n3-n40 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 43, 44. 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | UTRA Band 22, 42, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n78 | E-UTRA Band 1, 8, 11, 20, 21, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n40-n78 | UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n40-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 41, 42, 65, 74, NR band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n78-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. |  |  |  |  |  |  |  |

NOTE:	To simplify Table 6.5A.3.2.0.3-1, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

Table 6.5A.3.2.0.3-2: Requirements for uplink inter-band carrier aggregation (two bands) Rel-16

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n28 | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -30 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n40 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | NR band n77, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 4, 14 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 4, 7, 14 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 4, 7, 14 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n1-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n2A-n77A | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n3-n40 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 43, 44. 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | UTRA Band 22, 42, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n41 | E-UTRA Band 1, 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 34, 39, 44, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 42, NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n77 | E-UTRA Band 1, 2, 3, 4, 8, 11, 12, 13, 14, 17, 18, 19, 21, 25, 26, 28, 29, 30, 34, 40, 65, 66, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n39 | E-UTRA Band 1, 34, 40, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 41, 42 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n8-n40 | E-UTRA Bands 1, 5, 11, 18, 19, 20, 21, 26, 28, 31, 32, 33, 34, 38, 39, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Bands 3, 7, 22, 41, 42, 43, 52 NR Bands n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n78 | E-UTRA Band 1,8, 11, 20, 21, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n40 | E-UTRA Band 1, 3, 5, 7, 8, 18, 19, 20, 26, 27, 28, 31, 34, 38, 41, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21, 22, 32, 42, 43, 50, 51, 52, 65, 73, 74, 75, 76 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n41 | E-UTRA Band 2, 3, 5, 8, 25, 26, 27, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 42, 50, 51, 52, 65, 66, 73, 74 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -50 | 1 | 11 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n39-n40 | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 8 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n39-n41 | E-UTRA Band 1, 8, 26, 28, 34, 40, 42, 44, 45, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 4 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n39-n79 | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 4, 8 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n40-n41 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 27, 28, 34, 39, 42, 44, 45, 50, 51, 65, 73, 74, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n40-n78 | UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n40-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 41, 42, 65, 74, NR band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n41-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 42, 44, 45, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n48-n66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n66-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 43, 50, 51, 53, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 42, 48, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n66-n77 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n70-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 48, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 7, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). |  |  |  |  |  |  |  |

Table 6.5A.3.2.0.3-3: Requirements for uplink inter-band carrier aggregation (two bands) Rel-17

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n3-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n39 | E-UTRA Band 1, 34, 40, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 41, 42 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n8-n78 | E-UTRA Band 1,8, 11, 20, 21, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n24-n41 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n24-n48 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n24-n77 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n26-n66 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 43, 47, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 42, 48, 53 NR band 77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n26-n70 | E-UTRA Band 2, 5, 10, 12, 13, 14, 17, 24, 25, 29, 30, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n41 | E-UTRA Band 2, 3, 5, 8, 25, 26, 27, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 42, 50, 51, 52, 65, 66, 73, 74 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -50 | 1 | 11 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n39-n40 | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 8 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n39-n41 | E-UTRA Band 1, 8, 26, 28, 34, 40, 42, 44, 45, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 4 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n39-n79 | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 4, 8 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n41-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 42, 44, 45, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n48-n66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n48-n70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 | 16 |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2, 16 |
| CA_n48-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 50, 51, 53, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| CA_n66-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 43, 50, 51, 53, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 42, 48, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n70-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 48, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 7, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| CA_n71-n77 |  | E-UTRA Band 1, 3, 4, 5, 8, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 30, 39, 40, 44, 45, 50, 51, 53, 65, 66, 73, 74, 85, 103 |  | FDL_low |  | - |  | FDL_high | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  |  |  | 1915.7 | -41 |  | 0.3 |  | 3 |  |
|  |  | E-UTRA Band 2, 7, 25, 41, 34, 70 |  | FDL_low |  | - |  | FDL_high | -50 |  | 1 |  | 2 |  |
|  |  | E-UTRA Band 29 |  | FDL_low |  | - |  | FDL_high | -38 |  | 1 |  | 4 |  |
|  |  | E-UTRA Band 71 |  | FDL_low |  | - |  | FDL_high | -50 |  | 1 |  | 4 |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 16: The same configuration applies to corresponding NR-DC configuration in Table 4.3.1.1.7.1-1 in [5]. If UE supporting NR-DC configuration do not support the corresponding CA configuration, NR-DC configuration is used in this test. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5A.3.2.

##### 6.5A.3.2.1	Spurious emissions for UE co-existence for CA (2UL CA)

6.5A.3.2.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions for 2UL CA.

6.5A.3.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support 2UL CA.

6.5A.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.3.2.0.

6.5A.3.2.1.4	Test description

6.5A.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.3.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.2.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Normal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest SCS for default test points. CA configuration specific test points: Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  |  |  | CA Configuration / NRB_agg (Note 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  |  |  |  |  |  |  | UL Allocation (Note 2,3) |  |  |  |  |  |  |
|  |  |  |  | CA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCC NRB @SCS |  |  |  | SCC NRB@SCS |  |  | CC MOD |  |  |  | PCC & SCC RB allocation |  |  |  |  | CC MOD |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |  |
|  |  |  |  | PCC |  |  |  |  |  |  |  | SCC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCC | SCC |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Band |  |  |  | Range |  |  |  | Band |  |  |  | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_XA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | X |  |  |  | Low |  |  |  | Y |  |  |  | Low |  |  |  | Highest NRB@SCS |  |  | Highest NRB@SCS |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  | X |  |  |  | High |  |  |  | Y |  |  |  | High |  |  |  | Highest NRB@SCS |  |  | Highest NRB@SCS |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@RBmax |  |  |  | 1@RBmax |  |
| Test Settings for CA_n1A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 |  |  |  | High |  |  |  | n78 |  |  |  | High |  |  |  | 106@15kHz |  |  |  |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@105 |  |  | 1@272 |
| 2 | n1 |  |  |  | Low |  |  |  | n78 |  |  |  | 3455MHz |  |  |  | 106@15kHz |  |  |  |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 3 | n1 |  |  |  | Low |  |  |  | n78 |  |  |  | 3475MHz |  |  |  | 106@15kHz |  |  |  |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 4 | n1 |  |  |  | High |  |  |  | n78 |  |  |  | High |  |  |  | 106@15kHz |  |  |  |  |  | 106@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@105 |  |  | 106@0 |
| 5 | n1 |  |  |  | Low |  |  |  | n78 |  |  |  | High |  |  |  | 106@15kHz |  |  |  |  |  | 106@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  | 106@0 |
| Test Settings for CA_n2A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 |  |  |  | Low |  |  |  | n77 |  |  |  | Mid |  |  |  | 106@15kHz |  |  |  |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  | 1@272 |
| 2 | n2 |  |  |  | High |  |  |  | n77 |  |  |  | 3900MHz |  |  |  | 106@15kHz |  |  |  |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@99 |  |  | 1@200 |
| 3 | n2 |  |  |  | HIgh |  |  |  | n77 |  |  |  | 4000MHz |  |  |  | 106@15kHz |  |  |  |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@99 |  |  | 1@272 |
| Test Settings for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n3 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 160@15kHz |  |  | 270@15kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@160 |  |  | 1@270 |  |  |
| 2 |  |  | n3 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 78@30KHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@78 |  |  | 1@273 |  |  |
| 3 |  |  | n3 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 38@60KHz |  |  | 135@60KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@38 |  |  | 1@135 |  |  |
| Test Settings for CA_n5A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n5 |  |  |  | High |  |  |  | n77 |  |  |  | Mid |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  | 1@29 |  |  |
| 2 |  |  | n5 |  |  |  | Low |  |  |  | n77 |  |  |  | Mid |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@272 |  |  |
| 3 |  |  | n5 |  |  |  | Low |  |  |  | n77 |  |  |  | 4053.06MHz |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| 4 |  |  | n5 |  |  |  | Low |  |  |  | n77 |  |  |  | 3600MHz |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| 5 |  |  | n5 |  |  |  | Low |  |  |  | n77 |  |  |  | 3658.14MHz |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| 6 |  |  | n5 |  |  |  | Low |  |  |  | n77 |  |  |  | 3540.54MHz |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| 7 |  |  | n5 |  |  |  | High |  |  |  | n77 |  |  |  | High |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  | 1@272 |  |  |
| Test Settings for CA_n24A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n24 |  |  |  | High |  |  |  | n41 |  |  |  | Low |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@136 |  |  |
| 2 |  |  | n24 |  |  |  | Low |  |  |  | n41 |  |  |  | Low |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@52 |  |  | 1@108 |  |  |
| 3 |  |  | n24 |  |  |  | High |  |  |  | n41 |  |  |  | Low |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@136 |  |  |
| 4 |  |  | n24 |  |  |  | Low |  |  |  | n41 |  |  |  | Low |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@94 |  |  |
| 5 |  |  | n24 |  |  |  | High |  |  |  | n41 |  |  |  | High |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@192 |  |  |
| Test Settings for CA_n24A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n24 |  |  |  | High |  |  |  | n48 |  |  |  | Mid |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@136 |  |  |
| 2 |  |  | n24 |  |  |  | High |  |  |  | n48 |  |  |  | High |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| Test Settings for CA_n24A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n24 |  |  |  | Low |  |  |  | n77 |  |  |  | Mid |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@136 |  |  |
| 2 |  |  | n24 |  |  |  | Low |  |  |  | n77 |  |  |  | 3930 MHz |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@273 |  |  |
| 3 |  |  | n24 |  |  |  | Low |  |  |  | n77 |  |  |  | 3930 MHz |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@262 |  |  |
| 4 |  |  | n24 |  |  |  | Low |  |  |  | n77 |  |  |  | 3930 MHz |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@52 |  |  | 1@196 |  |  |
| Test Settings for CA_n26A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n26 |  |  |  | High |  |  |  | n66 |  |  |  | High |  |  |  | 106@15kHz |  |  | 216@15kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  | 1@0 |  |  |
| Test Settings for CA_n26A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n26 |  |  |  | Low |  |  |  | n70 |  |  |  | Low |  |  |  | 106@15kHz |  |  | 79@15kHz |  |  |  | QPSK/CP-OFDM QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| Test Settings for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n8 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 106@15kHz |  |  | 270@15kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| 2 |  |  | n8 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 106@15kHz |  |  | 270@15kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@106 |  |  | 1@270 |  |  |
| 3 |  |  | n8 |  |  |  | Low |  |  |  | n78 |  |  |  | Low |  |  |  | 51@30KHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@51 |  |  | 1@273 |  |  |
| 4 |  |  | n8 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 51@30KHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@51 |  |  | 1@273 |  |  |
| Test Settings for CA_n28A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n28 |  |  |  | High |  |  |  | n41 |  |  |  | High |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  | 1@272 |  |  |
| 2 |  |  | n28 |  |  |  | Low |  |  |  | n41 |  |  |  | Mid |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| 3 |  |  | n28 |  |  |  | Low |  |  |  | n41 |  |  |  | High |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@272 |  |  |
| 4 |  |  | n28 |  |  |  | Low |  |  |  | n41 |  |  |  | High |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| Test Settings for CA_n41A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | 4870MHz |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@0 |  |  |
| 2 |  |  | n41 |  |  |  | Mid |  |  |  | n79 |  |  |  | Low |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@136 |  |  | 1@136 |  |  |
| 3 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | Low |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@272 |  |  |
| 4 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | 4450MHz |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@136 |  |  |
| 5 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | 4500MHz |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@136 |  |  |
| 6 |  |  | n41 |  |  |  | Mid |  |  |  | n79 |  |  |  | Low |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@136 |  |  | 1@0 |  |  |
| 7 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | Mid |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@136 |  |  |
| Test Settings for CA_n48A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n48 |  |  |  | High |  |  |  | n66 |  |  |  | Low |  |  |  | 216@30kHz |  |  | 216@15kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@215 |  |  | 1@0 |  |  |
| Test Settings for CA_n48A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n48 |  |  |  | High |  |  |  | n70 |  |  |  | High |  |  |  | 216@15KHz |  |  | 79@15KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@215 |  |  | 1@78 |  |  |
| 2 |  |  | n48 |  |  |  | High |  |  |  | n70 |  |  |  | Low |  |  |  | 216@15KHz |  |  | 79@15KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@215 |  |  | 1@0 |  |  |
| Test Settings for CA_n48A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n48 |  |  |  | 3550 |  |  |  | n71 |  |  |  | 698 |  |  |  | 216@30kHz |  |  | 106@15kHz |  |  |  | QPSK/CP-OFDM QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@215 |  |  | 1@105 |  |  |
| 2 |  |  | n48 |  |  |  | High |  |  |  | n71 |  |  |  | Mid |  |  |  | 216@30kHz |  |  | 106@15kHz |  |  |  | QPSK/CP-OFDM QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@215 |  |  | 1@0 |  |  |
| Test Settings for CA_n66A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n66 |  |  |  | Low |  |  |  | n71 |  |  |  | Low |  |  |  | 216@15 KHz |  |  | 216@15 KHz |  |  |  | QPSK/CP-OFDM QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| Test Settings for CA_n66A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n66 |  |  |  | High |  |  |  | n77 |  |  |  | High |  |  |  | 216@15kHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@215 |  |  | 1@272 |  |  |
| 2 |  |  | n66 |  |  |  | Low |  |  |  | n77 |  |  |  | High |  |  |  | 216@15kHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@119 |  |  |
| 3 |  |  | n66 |  |  |  | Low |  |  |  | n77 |  |  |  | High |  |  |  | 216@15kHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@167 |  |  |
| 4 |  |  | n66 |  |  |  | Low |  |  |  | n77 |  |  |  | High |  |  |  | 216@15kHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@231 |  |  |
| 5 |  |  | n66 |  |  |  | High |  |  |  | n77 |  |  |  | Mid |  |  |  | 216@15kHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@215 |  |  | 1@100 |  |  |
| 6 |  |  | n66 |  |  |  | Low |  |  |  | n77 |  |  |  | 3892.11MHz |  |  |  | 216@15kHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| 7 |  |  | n66 |  |  |  | High |  |  |  | n77 |  |  |  | High |  |  |  | 216@15kHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@215 |  |  | 1@93 |  |  |
| 8 |  |  | n66 |  |  |  | Low |  |  |  | n77 |  |  |  | High |  |  |  | 216@15kHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@198 |  |  |
| 9 |  |  | n66 |  |  |  | Low |  |  |  | n77 |  |  |  | Mid |  |  |  | 216@15kHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@47 |  |  |
| 10 |  |  | n66 |  |  |  | Low |  |  |  | n77 |  |  |  | High |  |  |  | 216@15kHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@216 |  |  |
| Test Settings for CA_n70A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n70 |  |  |  | Low |  |  |  | n71 |  |  |  | Low |  |  |  | 79@15 KHz |  |  | 106@15 KHz |  |  |  | QPSK/CP-OFDM QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| Test Settings for CA_n71A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n71 |  |  |  | LOW |  |  |  | n77 |  |  |  | 3441,66 MHz (NOTE <X1>) |  |  |  | 79@15kHz |  |  | 52@15kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 51@0 |  |  |
| 2 |  |  | n71 |  |  |  | HIGH |  |  |  | n77 |  |  |  | LOW |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| 3 |  |  | n71 |  |  |  | HIGH |  |  |  | n77 |  |  |  | LOW |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@184 |  |  |
| 4 |  |  | n71 |  |  |  | HIGH |  |  |  | n77 |  |  |  | LOW |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  | 1@0 |  |  |
| 5 |  |  | n71 |  |  |  | LOW |  |  |  | n77 |  |  |  | MID |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  | 1@39 |  |  |
| 6 |  |  | n71 |  |  |  | LOW |  |  |  | n77 |  |  |  | 3951,51 MHz (NOTE <X6>) |  |  |  | 52@15kHz |  |  | 52@15kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| 7 |  |  | n71 |  |  |  | LOW |  |  |  | n77 |  |  |  | 3815,16 MHz (NOTE <X7>) |  |  |  | 79@15kHz |  |  | 52@15kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| NOTE <X1>: Test Point ID 1 for CA_n71A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3441.66MHz (NR ARFCN=629400). NOTE <X6>: Test Point ID 6 for CA_n71A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3951.51MHz (NR ARFCN=663400). NOTE <X7>: Test Point ID 7 for CA_n71A-n77A and Band n77 is defined as a carrier with CBW=10MHz and centre frequency at 3815.16MHz (NR ARFCN=654333). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] subclause4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.3.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5A.3.2.1.4.3.

6.5A.3.2.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, and Annex C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.5A.3.2.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5A.3.2.1.4.1-1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5.3.2.3-1. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5.3.2.3-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots.

6.5A.3.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.5A.3.2.1.4.3-1: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 16 |  | Power class 3 and Inter-band CA |

6.5A.3.2.1.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are the same as the minimum requirements. The measured average power of spurious emission, derived in step 6, shall not exceed the described value in Tables 6.5A.3.2.1.5-1, 6.5A.3.2.1.5-1a (Rel-16) and 6.5A.3.2.1.5-1 (Rel-17). The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.5A.3.2.1.5-1: Requirements for uplink inter-band carrier aggregation (two bands) Rel-15

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n28 | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR band n77, n78 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50+TT | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50+TT | 1 | 11, 12 |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50+TT | 1 | 11, 15 |
|  | Frequency range | 470 | - | 694 | -42+TT | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2+TT | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -30+TT | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50+TT | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2+TT | 6 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40+TT | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5+TT | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6+TT | 5 | 4, 6, 7 |
| CA_n1-n40 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR band n78 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Band 3, 34 | FDL_low | - | FDL_high | -50+TT | 1 | 4 |
|  | NR band n77, n79 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
|  | Frequency range | 1880 |  | 1895 | -40+TT | 1 | 4, 14 |
|  | Frequency range | 1895 |  | 1915 | -15.5+TT | 5 | 4, 7, 14 |
|  | Frequency range | 1915 |  | 1920 | +1.6+TT | 5 | 4, 7, 14 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| CA_n1-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40+TT | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5+TT | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6+TT | 5 | 4, 6, 7 |
| CA_n1-n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40+TT | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5+TT | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6+TT | 5 | 4, 6, 7 |
| CA_n3-n28 | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31 38, 40, 41, 72 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | E-UTRA Band 32, 42, 43, 50, 51, 74, 75, 76 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50+TT | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50+TT | 1 | 11, 12 |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50+TT | 1 | 11, 15 |
|  | Frequency range | 470 | - | 694 | -42+TT | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2+TT | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -30+TT | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50+TT | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2+TT | 6 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40+TT | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5+TT | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6+TT | 5 | 4, 6, 7 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50+TT | 1 | 4 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3, 11 |
| CA_n3-n40 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 43, 44. 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50+TT | 1 | 4 |
|  | UTRA Band 22, 42, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| CA_n3-n78 | E-UTRA Band 3, 34, 39 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| CA_n8-n78 | E-UTRA Band 8, 20, 28, 34, 39, 40 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | E-UTRA Band 3, 7,41 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| CA_n40-n78 | UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50+TT | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| CA_n40-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 41, 42, 65, 74, NR band n78 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| CA_n78-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50+TT | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41+TT | 0.3 | 3 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

Table 6.5A.3.2.1.5-1a: Requirements for uplink inter-band carrier aggregation (two bands) Rel-16

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band |  | Frequency range (Mhz) |  |  | Maximum Level (dBm) |  |  | MBW (MHz) |  | NOTE |
| CA_n1-n28 | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 |  | FDL_low | - | FDL_high | -50+TT |  |  | 1 |  |  |
|  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR band n77, n78 |  | FDL_low | - | FDL_high | -50+TT |  |  | 1 |  | 2 |
|  | E-UTRA Band 3, 34 |  | FDL_low | - | FDL_high | -50+TT |  |  | 1 |  | 4 |
|  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50+TT |  |  | 1 |  | 11, 12 |
|  | E-UTRA Band 1, 65 |  | FDL_low | - | FDL_high | -50+TT |  |  | 1 |  | 11, 15 |
|  | Frequency range |  | 470 | - | 694 | -42+TT |  |  | 8 |  | 4, 14 |
|  | Frequency range |  | 470 | - | 710 | -26.2+TT |  |  | 6 |  | 15 |
|  | Frequency range |  | 758 | - | 773 | -30+TT |  |  | 1 |  | 4 |
|  | Frequency range |  | 773 | - | 803 | -50+TT |  |  | 1 |  |  |
|  | Frequency range |  | 662 | - | 694 | -26.2+TT |  |  | 6 |  | 4 |
|  | Frequency range |  | 1880 | - | 1895 | -40+TT |  |  | 1 |  | 4, 6 |
|  | Frequency range |  | 1895 | - | 1915 | -15.5+TT |  |  | 5 |  | 4, 6, 7 |
|  | Frequency range |  | 1915 | - | 1920 | +1.6+TT |  |  | 5 |  | 4, 6, 7 |
| CA_n1-n40 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR band n78 |  | FDL_low | - | FDL_high | -50+TT |  |  | 1 |  |  |
|  | Band 3, 34 |  | FDL_low | - | FDL_high | -50+TT |  |  | 1 |  | 4 |
|  | NR band n77, n79 |  | FDL_low | - | FDL_high | -50+TT |  |  | 1 |  | 2 |
|  | Frequency range |  | 1880 |  | 1895 | -40+TT |  |  | 1 |  | 4, 14 |
|  | Frequency range |  | 1895 |  | 1915 | -15.5+TT |  |  | 5 |  | 4, 7, 14 |
|  | Frequency range |  | 1915 |  | 1920 | +1.6+TT |  |  | 5 |  | 4, 7, 14 |
|  | Frequency range |  | 1884.5 | - | 1915.7 | -41+TT |  |  | 0.3 |  | 3 |
| CA_n1-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 |  | FDL_low | - | FDL_high |  |  | -50+TT |  | 1 |  |
|  | Frequency range |  | 1880 | - | 1895 |  |  | -40+TT |  | 1 | 4, 6 |
|  | Frequency range |  | 1895 | - | 1915 |  |  | -15.5+TT |  | 5 | 4, 6, 7 |
|  | Frequency range |  | 1915 | - | 1920 |  |  | +1.6+TT |  | 5 | 4, 6, 7 |
| CA_n1-n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low |  | - | FDL_high |  | -50+TT |  | 1 |  |  |
|  | Frequency range | 1880 |  | - | 1895 |  | -40+TT |  | 1 |  | 4, 6 |
|  | Frequency range | 1895 |  | - | 1915 |  | -15.5+TT |  | 5 |  | 4, 6, 7 |
|  | Frequency range | 1915 |  | - | 1920 |  | +1.6+TT |  | 5 |  | 4, 6, 7 |
| CA_n2A-n77A | E-UTRA Band 4, 65, 66, 70 | FDL_low |  | - | FDL_high |  | -50 +TT |  | 1 |  |  |
|  | E-UTRA Band 2, 25 | FDL_low |  | - | FDL_high |  | -50 +TT |  | 1 |  | 2 |
| CA_n3-n40 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 43, 44. 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low |  | - | FDL_high |  | -50 |  | 1 |  |  |
|  | E-UTRA Band 3 | FDL_low |  | - | FDL_high |  | -50 |  | 1 |  | 4 |
|  | UTRA Band 22, 42, 52 NR Band n77, n78, n79 | FDL_low |  | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  | Frequency range | 1884.5 |  | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| CA_n3-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 |  | FDL_low | - | FDL_high | -50+TT |  |  | 1 |  |  |
|  | Frequency range |  | 1884.5 | - | 1915.7 | -41+TT |  |  | 0.3 |  | 3 |
| CA_n5-n77 | E-UTRA Band 1, 2, 3, 4, 8, 11, 12, 13, 14, 17, 18, 19, 21, 25, 26, 28, 29, 30, 34, 40, 65, 66, 70, 71, 74 |  | FDL_low | - | FDL_high | -50+TT |  |  | 1 |  |  |
|  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50+TT |  |  | 1 |  | 2 |
|  | Frequency range |  | 1884.5 | - | 1915.7 | -41+TT |  |  | 0.3 |  | 3 |
| CA_n8-n39 | E-UTRA Band 1, 34, 40, 50, 51, 74 |  | FDL_low | - | FDL_high | -50+TT |  |  | 1 |  |  |
|  | E-UTRA Band 22, 41, 42 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50+TT |  |  | 1 |  | 2 |
|  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50+TT |  |  | 1 |  | 4 |
| CA_n8-n40 | E-UTRA Bands 1, 5, 11, 18, 19, 20, 21, 26, 28, 31, 32, 33, 34, 38, 39, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  |  |
|  | E-UTRA Bands 3, 7, 22, 41, 42, 43, 52 NR Bands n77, n78, n79 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  | 2 |
|  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  | 4 |
|  | Frequency range |  | 1884.5 | - | 1915.7 | -41 |  |  | 0.3 |  | 3 |
| CA_n8-n78 | E-UTRA Band 1, 8, 11, 20, 21, 28, 34, 39, 40, 65, 74 |  | FDL_low | - | FDL_high | -50+TT |  |  | 1 |  |  |
|  | E-UTRA Band 3, 7,41 |  | FDL_low | - | FDL_high | -50+TT |  |  | 1 |  | 2 |
|  | Frequency range |  | 1884.5 | - | 1915.7 | -41+TT |  |  | 0.3 |  | 3 |
| CA_n28-n40 | E-UTRA Band 1, 3, 5, 7, 8, 18, 19, 20, 26, 27, 28, 31, 34, 38, 41, 72 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  |  |
|  | E-UTRA Band 11, 21, 22, 32, 42, 43, 50, 51, 52, 65, 73, 74, 75, 76 NR band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  | 2 |
|  | Frequency range |  | 1884.5 | - | 1915.7 | -41 |  |  | 0.3 |  | 3 |
| CA_n28-n41 | E-UTRA Band 2, 3, 5, 8, 25, 26, 27, 34 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  |  |
|  | E-UTRA Band 4, 42, 50, 51, 52, 65, 66, 73, 74 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  | 2 |
|  | E-UTRA Band 18, 19 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  | 11 |
|  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  | 11, 15 |
|  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  | 11, 12 |
|  | Frequency range |  | 470 | - | 694 | -42 |  |  | 8 |  | 4, 14 |
|  | Frequency range |  | 470 | - | 710 | -26.2 |  |  | 6 |  | 13 |
|  | Frequency range |  | 662 | - | 694 | -26.2 |  |  | 6 |  | 4 |
|  | Frequency range |  | 758 | - | 773 | -32 |  |  | 1 |  | 4 |
|  | Frequency range |  | 773 | - | 803 | -50 |  |  | 1 |  |  |
|  | Frequency range |  | 1884.5 | - | 1915.7 | -41 |  |  | 0.3 |  | 3, 11 |
| CA_n39-n40 | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  |  |
|  | NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  | 2 |
|  | Frequency range |  | 1805 |  | 1855 | -40 |  |  | 1 |  | 8 |
|  | Frequency range |  | 1855 |  | 1880 | -15.5 |  |  | 5 |  | 4, 7, 8 |
| CA_n39-n41 | E-UTRA Band 1, 8, 26, 28, 34, 40, 42, 44, 45, 50, 51, 74 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  |  |
|  | NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  | 2 |
|  | Frequency range |  | 1805 | - | 1855 | -40 |  |  | 1 |  | 4 |
|  | Frequency range |  | 1855 | - | 1880 | -15.5 |  |  | 5 |  | 4, 7, 8 |
| CA_n39-n79 | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  |  |
|  | NR Band n78 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  | 2 |
|  | Frequency range |  | 1805 | - | 1855 | -40 |  |  | 1 |  | 4, 8 |
|  | Frequency range |  | 1855 | - | 1880 | -15.5 |  |  | 5 |  | 4, 7, 8 |
| CA_n40-n41 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 27, 28, 34, 39, 42, 44, 45, 50, 51, 65, 73, 74, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  |  |
|  | NR Band n79 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  | 2 |
|  | Frequency range |  | 1884.5 | - | 1915.7 | -41 |  |  | 0.3 |  | 3 |
| CA_n40-n78 | UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  |  |
|  | NR Band n79 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  | 2 |
|  | Frequency range |  | 1884.5 | - | 1915.7 | -41 |  |  | 0.3 |  | 3 |
| CA_n40-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 41, 42, 65, 74, NR band n78 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  |  |
|  | Frequency range |  | 1884.5 | - | 1915.7 | -41 |  |  | 0.3 |  | 3 |
| CA_n41-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 42, 44, 45, 65 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  |  |
|  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 |  |  | 1 |  |  |
|  | Frequency range |  | 1884.5 | - | 1915.7 | -41 |  |  | 0.3 |  | 3 |
| CA_n48-n66 | E-UTRA Band 5, 25 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  |  |
| CA_n66-n71 | NR Band n77 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  | 2 |
| CA_n66-n77 | E-UTRA Band 2, 4, 12, 13, 14, 17, 29, 30, 65, 66, 70, 71, 103 |  | FDL_low | - | FDL_high | -50 +TT |  |  | 1 |  |  |
| CA_n70-n71 | E-UTRA Band 30 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  |  |
|  | NR Band n77 |  | FDL_low | - | FDL_high | -50 |  |  | 1 |  | 2 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart> 3. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5A.3.2.1.5-1b: Requirements for uplink inter-band carrier aggregation (two bands) Rel-17

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band |  | Frequency range (Mhz) |  |  |  |  | Maximum Level (dBm) |  |  | MBW (MHz) |  | NOTE |
| CA_n1-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 |  | FDL_low |  | - |  | FDL_high |  |  | -50+TT |  | 1 |  |
|  | Frequency range |  | 1880 |  | - |  | 1895 |  |  | -40+TT |  | 1 | 4, 6 |
|  | Frequency range |  | 1895 |  | - |  | 1915 |  |  | -15.5+TT |  | 5 | 4, 6, 7 |
|  | Frequency range |  | 1915 |  | - |  | 1920 |  |  | +1.6+TT |  | 5 | 4, 6, 7 |
| CA_n1-n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low |  | - |  | FDL_high |  |  | -50+TT |  | 1 |  |  |
|  | Frequency range | 1880 |  | - |  | 1895 |  |  | -40+TT |  | 1 |  | 4, 6 |
|  | Frequency range | 1895 |  | - |  | 1915 |  |  | -15.5+TT |  | 5 |  | 4, 6, 7 |
|  | Frequency range | 1915 |  | - |  | 1920 |  |  | +1.6+TT |  | 5 |  | 4, 6, 7 |
| CA_n3-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 | -41+TT |  |  | 0.3 |  | 3 |
| CA_n8-n39 | E-UTRA Band 1, 34, 40, 50, 51, 74 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
|  | E-UTRA Band 22, 41, 42 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 2 |
|  | E-UTRA Band 8 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 4 |
| CA_n8-n78 | E-UTRA Band 1, 8, 11, 20, 21, 28, 34, 39, 40, 65, 74 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
|  | E-UTRA Band 3, 7,41 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 2 |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 | -41+TT |  |  | 0.3 |  | 3 |
| CA_n24-n41 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 48, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
|  | NR Band n77 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 2 |
| CA_n24-n48 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
| CA_n24-n77 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
| CA_n26-n66 | E-UTRA Band 5, 26 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | E-UTRA Band 42 NR band 77 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 2 |
| CA_n26-n70 | E-UTRA Band 5 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 2 |
| CA_n28-n41 | E-UTRA Band 2, 3, 5, 8, 25, 26, 27, 34 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | E-UTRA Band 4, 42, 50, 51, 52, 65, 66, 73, 74 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 2 |
|  | E-UTRA Band 18, 19 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 11 |
|  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 11, 15 |
|  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 11, 12 |
|  | Frequency range |  | 470 |  | - |  | 694 | -42 |  |  | 8 |  | 4, 14 |
|  | Frequency range |  | 470 |  | - |  | 710 | -26.2 |  |  | 6 |  | 13 |
|  | Frequency range |  | 662 |  | - |  | 694 | -26.2 |  |  | 6 |  | 4 |
|  | Frequency range |  | 758 |  | - |  | 773 | -32 |  |  | 1 |  | 4 |
|  | Frequency range |  | 773 |  | - |  | 803 | -50 |  |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 | -41 |  |  | 0.3 |  | 3, 11 |
| CA_n39-n40 | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 2 |
|  | Frequency range |  | 1805 |  |  |  | 1855 | -40 |  |  | 1 |  | 8 |
|  | Frequency range |  | 1855 |  |  |  | 1880 | -15.5 |  |  | 5 |  | 4, 7, 8 |
| CA_n39-n41 | E-UTRA Band 1, 8, 26, 28, 34, 40, 42, 44, 45, 50, 51, 74 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 2 |
|  | Frequency range |  | 1805 |  | - |  | 1855 | -40 |  |  | 1 |  | 4 |
|  | Frequency range |  | 1855 |  | - |  | 1880 | -15.5 |  |  | 5 |  | 4, 7, 8 |
| CA_n39-n79 | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | NR Band n78 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 2 |
|  | Frequency range |  | 1805 |  | - |  | 1855 | -40 |  |  | 1 |  | 4, 8 |
|  | Frequency range |  | 1855 |  | - |  | 1880 | -15.5 |  |  | 5 |  | 4, 7, 8 |
| CA_n41-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 42, 44, 45, 65 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | E-UTRA Band 40 |  | FDL_low |  | - |  | FDL_high | -40 |  |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 | -41 |  |  | 0.3 |  | 3 |
| CA_n48-n66 | E-UTRA Band 2, 25 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
| CA_n48-n70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 16 |
|  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 2, 16 |
| CA_n48-n71 | E-UTRA Band 4, 30, 66 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
| CA_n66-n71 | NR Band n77 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 2 |
| CA_n70-n71 | E-UTRA Band 30 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | NR Band n77 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 2 |
| CA_n71-n77 | E-UTRA Band 1, 2, 4, 10, 25, 30, 39, 40, 53, 65, 66 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 | -41 |  |  | 0.3 |  | 3 |
|  | E-UTRA Band 7, 34, 41, 70 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 2 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart> 3. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 16: The same configuration applies to corresponding NR-DC configuration in Table 4.3.1.1.7.1-1 in [5]. If UE supporting NR-DC configuration do not support the corresponding CA configuration, NR-DC configuration is used in this test. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5A.3.2.1.5-2: Test Tolerance for uplink inter-band carrier aggregation (two bands)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0 | 0 |
| 40MHz < BW ≤ 100MHz | 0 | 0 |

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5.3.2.3-1 from the edge of the channel bandwidth. The spurious emission limits in Table 6.5.3.2.3-1 apply for all transmitter band configurations (NRB) and channel bandwidths for all CC combinations.

[END OF MODIFICATIONS]

Release 17

3GPP TS 38.521-1 V17.6.0 (2022-09)

Release 17

3GPP TS 38.521-1 V17.6.0 (2022-09)