3GPP TSG-RAN5 Meeting #97	R5-227313r1

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2135 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to CLI Measurement test case 5.6.4.1, 5.7.5.1, 7.6.4.1 and 7.7.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies Int |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CLI-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Test Tolerances for FR2 RLM test cases are available in 38.903 but missing on the current test case description. Test Tolerances are available for the following configurations: - Normal conditions- Test frequency f ≤ 40.8 GHz - UE PC3- Normal conditions |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated the Test Requirements tables according to the corresponding Test Tolerances. - Updated the Editor’s notes to reflect the Test Tolerance addition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case implementation will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.4.1, 5.7.5.1, 7.6.4.1, 7.7.5.1, Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS/TR 38.903 CR R5-227325,328 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: 38.903 Tdoc number referred on the cover page, editor’s note updated for all test cases, TT+MU allowance added to the test cases |  |  |  |  |  |  |  |  |

<< Start of Changes >>

### 5.6.4	CLI measurements

#### 5.6.4.0	Minimum conformance requirements

##### 5.6.4.0.1	Minimum conformance requirements for SRS-RSRP measurement period

The UE shall be capable of performing SRS-RSRP measurement based on the configured SRS resource, and the UE shall be capable of reporting SRS-RSRP measured over measurement period of TSRS_RSRP_measurement_period for FR1 and FR2.

Table 5.6.4.0.1-1 Measurement period TSRS_RSRP_measurement_period

| Configuration | TSRS_measurement_period (ms) |
| --- | --- |
| No DRX | Max(60, 3 X TSRS) |
| DRX cycle ≤ 320ms | Max(60, Ceil(1.5 X 3) X max(TSRS, TDRX)) |
| DRX cycle > 320ms | 3 X TDRX |
| Note: TSRS is SRS measurement periodicity configured SRS-PeriodicityAndOffset, and TDRX is the DRX cycle length. |  |

If the SRS resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TSRS_RSRP_measurement_period.

When configured by the network, the UE shall be able to perform SRS-RSRP measurements of configured srs-ResourceConfigCLI. The requirements apply when the subcarrier spacing for SRS-RSRP measurement resource configuration is the same as the subcarrier spacing of the active DL BWP of serving cell. The UE is not required to measure SRS using different SCS compared to the downlink active BWP SCS of the same carrier.

The requirements as provided:

-	SRS resources configured for SRS-RSRP measurements are measurable.

An SRS resource configured for SRS-RSRP shall be considered measurable when for each relevant SRS the following conditions are met:

-	SRS-RSRP related side conditions given in clauses 10.1.22.1 for FR1 and FR2 for a corresponding band,

-	SRS_RP and SRS Ês/Iot according to Annex B.2.7 for a corresponding band.

The UE shall send SRS-RSRP reports only for report configurations according to reportType which is cliPeriodical or cliEventTriggered when SRS-RSRP report is configured.

The UE shall report the SRS-RSRP value as a 7-bit value in the range [-140, -44] dBm with 1dB step size according to clause 10.1.22.1 for FR1 and FR2.

Reported SRS-RSRP measurements contained in periodically triggered measurement reports shall meet the requirements in clause 10.1.22.1.

Reported SRS-RSRP measurements contained in periodically triggered measurement reports shall meet the requirements in clauses 10.1.22.1.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.7.2.3.3.

Reported SRS-RSRP measurements contained in periodically triggered measurement reports shall meet the requirements in clause 10.1.22.1.

The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report on.

The normative reference for this requirement is TS 38.133 [6] clause 9.7.2.5 and 9.7.2.1, 9.7.2.2 and 9.7.2.3

#### 5.6.4.1	EN-DC FR2 SRS-RSRP measurement in non-DRX

Editor's Note: This test case is incomplete. Following aspects are either missing or TBD

- The test applicability is FFS

- The test procedure is incomplete

- The message content is FFS

- MU/TT analysis for UE PC3 and test frequency f ≤ 40.8 GHz is complete.

- MU/TT analysis for UE power class other than PC3 is incomplete.

- MU//TT analysis for test frequency f > 40.8 GHz is incomplete.

5.6.4.1.1	Test purpose

The purpose of this test case is to verify that the UE makes correct reporting of SRS-RSRP measurement in non-DRX within SRS-RSRP measurement requirements in TS 38.133 [6] clause 9.7.2.5

5.6.4.1.2	Test applicability

FFS

5.6.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 5.6.4.0.1

The normative reference for this requirement is TS 38.133 [6] clause 9.7.2.5 and A.5.6.4.1.

5.6.4.1.4	Test description

Two cells are deployed in the test, which are E-UTRAN PCell (Cell 1) and FR2 PSCell (Cell 2).

5.6.4.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 5.6.4.1.4.1-1.

Table 5.6.4.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 5.6.4.1 - 1 | NR 120 kHz SRS SCS, 100 MHz bandwidth, TDD duplex mode, LTE FDD |
| 5.6.4.1 - 2 | NR 120 kHz SRS SCS, 100 MHz bandwidth, TDD duplex mode, LTE TDD |
|  | Note: UE is only required to be tested in one of the supported test configurations |

Configure the test equipment and the DUT according to the parameters in Table 5.6.4.1.4.1-2.

Table 5.6.4.1.4.1-2: Initial conditions EN-DC FR2 SRS-RSRP measurement in non-DRX

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, E.1.1, E.1.3.1 and Table E.3-1 and TS 38.508-1 [14] clause 4.3.1 for E-UTRA and 7.2.3 for NR. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in clause C.2.1 |
| Connection Diagram | TE Part: A.3.3.1.1 DUT Part: A.3.4.1.1 | As specified in TS 38.508-1 [14] Annex A. |
| Exceptions to connection diagram | N/A |  |

1. The test parameters are given in Table 5.6.4.1.4.1-3 below.

2. Message contents are defined in clause 5.6.4.1.4.3.

3. Two cells are deployed in the test, which are E-UTRAN PCell (Cell 1) and FR2 PSCell (Cell 2). The test parameters for PSCell is given in Table A.5.6.4.1.4.1-3

.	The UE Rx beam peak direction has been obtained previously using one of the Rx Beam Peak Search procedures as described in Annex I.

Table 5.6.4.1.4.1-3: General test parameters for SRS-RSRP event triggered reporting for PSCell in FR2

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | E-UTRAN Cell 1 and NR Cell 2 |  |
| RF Channel Number |  | 1 | 1: Cell 1 2: Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| SRS configuration |  | 1 | SRSConf.1 | Table A.5.6.4.1.2-4 |
| CP length |  | 1 | Normal |  |
| i1-Threshold | dBm | 1 | -103 |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX | ms | 1 | DRX.11 |  |
| Time offset between DL from serving cell and SRS from test system | s | 1,2 | 10.67 |  |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 1 |  |

5.6.4.1.4.2	Test procedure

There are two cells are deployed in the test, which are E-UTRAN PCell (Cell 1) and FR2 PSCell (Cell 2)

In the measurement control information, a measurement object is configured for the frequency of the PSCell, and it is indicated to the UE that event-triggered reporting with Event I1 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively.

During the test, the test system transmits SRS resource for measurement in the DL slot according to the SRS configuration in Table 5.6.4.1.5-3 and the test parameters for the (virtual) neighbour cell UE in Table 5.6.4.1.5-23. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 2 data symbols before SRS to be transmitted.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 5.6.4.1.5-1 as appropriate.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1 with event I1 configured.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

Editor's note: the rest of the steps are FFS.

5.6.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

FFS

5.6.4.1.5	Test requirement

Table 5.7.1.1.5-1 defines the cell specific settings for all tests. Table 5.7.1.1.5-2 defines the OTA primary level settings including test tolerances for all tests.

Table 5.6.4.1.5-1: NR Cell specific test parameters for SRS-RSRP event triggered reporting for PSCell in FR2

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |
| OCNG Patterns |  | 1 | OP.1 |  |
| TRS configuration |  |  | TRS.2.1. TDD |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  |
| Propagation Condition |  | 1 | AWGN |  |

Table 5.6.4.1.5-2: NR OTA Cell specific test parameters for SRS-RSRP event triggered reporting for PSCell and Neighbour cell UE in FR2

| Parameter | Unit | Test configuration | Cell 2 |  | Neighbour cell UE |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1 | Setup 1 defined A.3.15.1 |  |  |  |
| Beam assumption Note 4 |  | 1 | Fine |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98-100.5 |  | -98-100.5 |  |
| Note 2 | dBm/SCS | 1 | -89 |  | -89 |  |
|  | dB | 1 | - | - | -infinity | 4 |
|  | dB | 1 | - | - | -infinity | 4 |
| SRS-RSRP Note 3 | dBm/SCS kHz | 1 | - | - | -infinity | -94 |
| Io | dBm/95.04 MHz | 1 | -70.01 | -68.82 | -70.01 | -68.82 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

Table 5.6.4.1.5-3: SRS configuration for measurement reporting

|  | Field | SRSConf.1 | Comments |
| --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 |  |
|  | srs-ResourceIdList | 0 |  |
|  | resourceType | Periodic |  |
|  | Usage | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 |  |
|  | nrofSRS-Ports | Port1 |  |
|  | transmissionComb | n2 |  |
|  | combOffset-n2 | 0 |  |
|  | cyclicShift-n2 | 0 |  |
|  | resourceMapping startPosition | 0 |  |
|  | resourceMapping nrofSymbols | n1 |  |
|  | resourceMapping repetitionFactor | n1 |  |
|  | freqDomainPosition | 0 |  |
|  | freqDomainShift | 0 |  |
|  | freqHopping c-SRS | 12 |  |
|  | freqHopping b-SRS | 0 |  |
|  | freqHopping b-hop | 0 |  |
|  | groupOrSequenceHopping | Neither |  |
|  | resourceType | Periodic |  |
|  | periodicityAndOffset | sl160, 25 |  |
|  | sequenceId | 0 | Any 10 bit number |

The UE shall send one Event I1 triggered measurement report, with a measurement reporting delay less than 60 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<<Some Unchanged Sections>>

### 5.7.5	SRS-RSRP

#### 5.7.5.0	Minimum conformance requirements

##### 5.7.5.0.1	Minimum conformance requirements for SRS-RSRP measurement accuracy

The SRS-RSRP measurement reported by the UE shall fulfil the accuracy requirements defined in Table 5.7.5.0.1-1 for FR1 and Table 5.7.7.0.1-2 for FR2, provided that the following conditions are met. The accuracy requirements in this clause are derived based on AWGN radio propagation conditions.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for SRS-RSRP measurements are fulfilled according to Annex B.2.z for a corresponding Band for each relevant SRS resource configured for measurement.

-	The time difference between UE’s DL reference timing in the serving cell and SRS arrival time is no larger than Terror_SRS_RSRP, where

-	Terror_SRS_RSRP = TC × NTA_offset + 4.67us for FR1

-	Terror_SRS_RSRP = TC × NTA_offset + 3.67us for FR2

-	NTA_offset is defined in Table 7.1.2-2

-	TC is 0.509ns

-	The number of SRS ports in the SRS resource configured for measurement is 1,

-	The number of symbols in the SRS resource configured for measurement is 1,

-	The number of repetitions in the SRS resource configured for measurement is 1,

-	Frequency hopping, sequence group hopping or sequence hopping is disabled in the SRS resource configured for measurement,

-	The bandwidth of the SRS resource is 48 PRBs.

-	One of the following conditions is met

-	There is no other SRS resource with the same root sequence and on the same symbol and with same comb as the relevant SRS resource.

-	If multiple SRS resources are on the same symbol and with same comb, the distance between cyclic shifts of any two resources is no less than 6 if transmissionComb = n4, and no less than 4 if transmissionComb = n2.

Table 5.7.5.0.1-1: SRS-RSRP absolute accuracy in FR1

| Accuracy |  |  |  |  |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  |  | Extreme condition |  |  | SRS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB |  |  |  |  |  | dB |  | dBm / SCSSRS |  |  | dBm/BW Channel | dBm/BW Channel |
| SCSSRS (kHz) |  |  | SCSSRS (kHz) |  |  |  |  |  |  |  |  |  |
| 15 | 30 | 60 | 15 | 30 | 60 |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |  |
| 3 | 3.5 | 5 | 7.5 | 8 | 9.5 | 1 | NR_TDD_FR1_A, | -120 | -117 | -114 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_C | -119 | -116 | -113 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_E | -118 | -115 | -112 | N/A | -70 |
| 6.5 | 7 | 8.5 | 9.5 | 10 | 11.5 | 1 | NR_TDD_FR1_A, NR_TDD_FR1_C, NR_TDD_FR1_D, NR_TDD_FR1_E | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.7.5.0.1-2: SRS-RSRP absolute accuracy in FR2

| Accuracy |  |  |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  | Extreme condition |  | SRS Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  |  |  | Minimum Io |  |  | Maximum Io |
| dB |  |  |  | dB | dBm / SCSSRS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
| SCSSRS (kHz) |  | SCSSRS (kHz) |  |  | SCSSRS = | SCSSRS = |  |  |
| 60 | 120 | 60 | 120 |  | 60kHz | 120kHz |  |  |
| 6 | 8.5 | 9 | 11.5 | ≥1 | Same value as SRS_RP in Table B.2.7-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 9 | 11.5 | 11 | 13.5 | ≥1 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |  |

5.7.5.0.1.1	SRS-RSRP report mapping

The reporting range of SRS-RSRP is defined from -140 dBm to -44 dBm with 1 dB resolution. The mapping of measured quantity is defined in Table 5.7.7.0.1.2-1. The range in the signalling may be larger than the guaranteed accuracy range.

Table 5.7.7.0.1.2-1: SRS-RSRP measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| SRS-RSRP_0 | SRS-RSRP<-140 | dBm |
| SRS-RSRP_1 | -140≤ SRS-RSRP<-139 | dBm |
| SRS-RSRP_2 | -139≤ SRS-RSRP<-138 | dBm |
| SRS-RSRP_3 | -138≤ SRS-RSRP<-137 | dBm |
| SRS-RSRP_4 | -137≤ SRS-RSRP<-136 | dBm |
| .. | .. | … |
| SRS-RSRP_95 | -46≤ SRS-RSRP<-45 | dBm |
| SRS-RSRP_96 | -45≤ SRS-RSRP<-44 | dBm |
| SRS-RSRP_97 | -44≤ SRS-RSRP | dBm |
| SRS-RSRP_98 | Infinity |  |
| Note: ‘Infinity’ means that UE cannot detect SRS due to too strong signal to measure. |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.22.1.

#### 5.7.5.1	EN-DC FR2 SRS-RSRP measurement accuracy

Editor's Note: This test case is incomplete. Following aspects are either missing or TBD:

- The test procedure is incomplete

- The message content is FFS

- MU/TT analysis for UE PC3 and test frequency f ≤ 40.8 GHz is complete.

- MU/TT analysis for UE power class other than PC3 is incomplete.

- MU//TT analysis for test frequency f > 40.8 GHz is incomplete.

5.7.5.1.1	Test purpose

The purpose of this test is to verify that the SRS-RSRP measurement accuracy is within the specified limits with SRS-RSRP measurement requirements in TS 38.133 [6] clause 10.1.22.1.1.

5.7.5.1.2	Test applicability

This test applies to all types of NR UE supporting E-UTRA and EN-DC from Release 16 onwards and CLI-SRS-RSRP

5.7.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 5.7.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.5.7.5.1.

5.7.5.1.4	Test description

5.7.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 5.7.5.1.4.1-1.

Table 5.7.5.1.4.1-1: EN-DC FR2 SRS-RSRP accuracy supported test configurations

| Config | Description |
| --- | --- |
| 5.7.5.1 - 1 | LTE FDD, NR 120 kHz SRS SCS, 100 MHz bandwidth, TDD duplex mode |
| 5.7.5.1 - 2 | LTE TDD, NR 120 kHz SRS SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 5.7.5.1.4.1-2.

Table 5.7.5.1.4.1-2: Initial conditions EN-DC FR2 SRS-RSRP measurement accuracy

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, E.1.1, E.1.3.1 and Table E.3-1 and TS 38.508-1 [14] clause 4.3.1 for E-UTRA and 7.2.3 for NR. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in clause C.2.1 |
| Connection Diagram | TE Part: A.3.3.1.1 DUT Part: A.3.4.1.1 | As specified in TS 38.508-1 [14] Annex A. |
| Exceptions to connection diagram | N/A |  |

1. Message contents are defined in clause 5.7.5.1.4.3.

2. In this set of test cases there are two cells in the test, E-UTRAN PCell (Cell 1), FR2 PSCell (Cell 2).

3.	The UE Rx beam peak direction has been obtained previously using one of the Rx Beam Peak Search procedures as described in Annex I.

5.7.5.1.4.2	Test procedure

There are two cells are deployed in the test, which are E-UTRAN PCell (Cell 1) and FR2 PSCell (Cell 2)

The test parameters and applicability for Cell 1 are defined in A.3.7.2 of 38.133. The test parameters for the Cell 2 are given in Table 5.7.5.1.5-1 and 5.7.5.1.5-2 below. The test parameter for the (virtual) neighbour cell UE transmitting SRS are given in Table 5.7.5.1.5-2.

Before the test UE is configured to perform SRS-RSRP measurement. During the test, the test system transmits SRS resources for measurement in the DL slots according to the SRS configuration in Table 5.7.5.1.5-3. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 2 data symbols before SRS to be transmitted.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 5.7.5.1.5-1 as appropriate.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

<rest of the steps are FFS>

5.7.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

FFS

5.7.5.1.5	Test requirement

Table 5.7.5.1.5-1 defines the cell specific settings for all tests. Table 5.7.5.1.5-2 defines the OTA primary level settings including test tolerances for all tests.

The SS-RSRP measurement accuracy shall fulfil the absolute accuracy requirements in table 5.7.5.1.5-3 and absolute accuracy requirements in table 5.7.5.1.5-4. The following requirements are to be verified:

During T1:

The UE is deemed to meet the requirement if the reported SRS-RSRP is in the range shown in table 5.7.5.1.5-4.

During T2:

The UE is deemed to meet the requirement if the reported SRS-RSRP is in the range shown in table 5.7.5.1.5-4.

Table 5.7.5.1.5-1: FR2 test parameters for SRS-RSRP accuracy

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 | freq1 |
| Duplex mode | 1~2 |  | TDD | TDD |
| TDD configuration | 1~2 |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1~2 |  | SR.3.1 TDD | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1~2 |  | CR.3.1 TDD | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1~2 |  | CCR.3.1 TDD | CCR.3.1 TDD |
| SSB configuration | 1~2 |  | SSB.3 FR2 | SSB.3 FR2 |
| OCNG Patterns | 1~2 |  | OP.1 | OP.1 |
| TRS configuration | 1~2 |  | TRS.2.1 TDD | TRS.2.1 TDD |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1~2 |  | SMTC.1 | SMTC.1 |
| Time offset between DL from serving cell and SRS from test system | 1~2 | s | 10.76 | 10.67 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Propagation condition | 1~2 |  | AWGN | AWGN |
| Antenna configuration | 1~2 |  | 1x2 | 1x2 |
| SRS configuration | 1~2 |  | SRSConf.1 | SRSConf.1 |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |

Table 5.7.5.1.5-2: SRS-RSRP accuracy OTA related test parameters for PSCell and Neighbour cell UE in FR2

| Parameter | Unit | T1 | T2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 defined A.3.15.1 | Setup 1 defined A.3.15.1 |
| Beam assumption Note 5 |  | Fine | Fine |
| Note1 | dBm/15kHzNote3 | -100 | N/A |
| Note1 | dBm/SCSNote3 | -91 | N/A |
|  | dB | 2 | N/A |
| Es | dBm/SCSNote3 |  | (Table B.2.7-2 Rx Beam Peak)+5 |
| SRS_RPNote2 | dBm/SCS | -89 | (Table B.2.7-2 Rx Beam Peak) +5 |
| BB Note4 | dB | >1 | 1 |
| IoNote2 | dBm/95.04 MHz Note3 | -57.89 | (Table B.2.7-2 Rx Beam Peak +50.79dB) |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SRS_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 2dB for UE multi-band relaxation factor ∑MBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |

Table 5.7.5.1.5-3: SRS configuration parameters for FR2 SRS-RSRP accuracy

|  | Field | SRSConf.1 |
| --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 |
|  | srs-ResourceIdList | 0 |
|  | resourceType | Periodic |
|  | Usage | Codebook |
| SRS-Resource | SRS-ResourceId | 0 |
|  | nrofSRS-Ports | Port1 |
|  | transmissionComb | n2 |
|  | combOffset-n2 | 0 |
|  | cyclicShift-n2 | 0 |
|  | resourceMapping startPosition | 0 |
|  | resourceMapping nrofSymbols | n1 |
|  | resourceMapping repetitionFactor | n1 |
|  | freqDomainPosition | 0 |
|  | freqDomainShift | 0 |
|  | freqHopping c-SRS | 12 |
|  | freqHopping b-SRS | 0 |
|  | freqHopping b-hop | 0 |
|  | groupOrSequenceHopping | Neither |
|  | resourceType | Periodic |
|  | periodicityAndOffset-p | sl160,25 |
|  | sequenceId | 0 |

Table 5.7.5.1.5-4: SRS-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
| SRS | SRS_RP -δ +Gmin ≤ Reported SRS-RSRP(dBm) ≤SRS_RP +δ +Gmax |
| Note 1: SRS_RP is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test Note 2: δ is the RSRP absolute accuracy requirement from Table 10.1.22.1.1-2, selected according to the Io used in the test Note 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

The SRS-RSRP measurement accuracy shall fulfil the absolute accuracy requirements in clauses 5.7.5.0.10.1 The following requirements are to be verified:

During T1:

The UE is deemed to meet the requirement if the reported SRS-RSRP is in the range shown in table 5.7.5.1.5-4.

During T2:

The UE is deemed to meet the requirement if the reported SRS-RSRP is in the range shown in table 5.7.5.1.5-4.

For the test to pass, the ratio of successful reported values for each requirement shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

<<Some Unchanged Sections>>

### 7.6.4	CLI measurements

#### 7.6.4.0	Minimum conformance requirements

##### 7.6.4.0.1	Minimum conformance requirements for SRS-RSRP measurement period

The UE shall be capable of performing SRS-RSRP measurement based on the configured SRS resource, and the UE shall be capable of reporting SRS-RSRP measured over measurement period of TSRS_RSRP_measurement_period for FR1 and FR2.

Table 7.6.4.0.1-1 Measurement period TSRS_RSRP_measurement_period

| Configuration | TSRS_measurement_period (ms) |
| --- | --- |
| No DRX | Max(60, 3 X TSRS) |
| DRX cycle ≤ 320ms | Max(60, Ceil(1.5 X 3) X max(TSRS, TDRX)) |
| DRX cycle > 320ms | 3 X TDRX |
| Note: TSRS is SRS measurement periodicity configured SRS-PeriodicityAndOffset, and TDRX is the DRX cycle length. |  |

If the SRS resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TSRS_RSRP_measurement_period.

When configured by the network, the UE shall be able to perform SRS-RSRP measurements of configured srs-ResourceConfigCLI. The requirements apply when the subcarrier spacing for SRS-RSRP measurement resource configuration is the same as the subcarrier spacing of the active DL BWP of serving cell. The UE is not required to measure SRS using different SCS compared to the downlink active BWP SCS of the same carrier.

The requirements as provided:

-	SRS resources configured for SRS-RSRP measurements are measurable.

An SRS resource configured for SRS-RSRP shall be considered measurable when for each relevant SRS the following conditions are met:

-	SRS-RSRP related side conditions given in clauses 10.1.22.1 for FR1 and FR2 for a corresponding band,

-	SRS_RP and SRS Ês/Iot according to Annex B.2.7 for a corresponding band.

The UE shall send SRS-RSRP reports only for report configurations according to reportType which is cliPeriodical or cliEventTriggered when SRS-RSRP report is configured.

The UE shall report the SRS-RSRP value as a 7-bit value in the range [-140, -44] dBm with 1dB step size according to clause 10.1.22.1 for FR1 and FR2.

Reported SRS-RSRP measurements contained in periodically triggered measurement reports shall meet the requirements in clause 10.1.22.1.

Reported SRS-RSRP measurements contained in periodically triggered measurement reports shall meet the requirements in clauses 10.1.22.1.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.7.2.3.3.

Reported SRS-RSRP measurements contained in periodically triggered measurement reports shall meet the requirements in clause 10.1.22.1.

The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report on.

The normative reference for this requirement is TS 38.133 [6] clause 9.7.2.5 and 9.7.2.1, 9.7.2.2 and 9.7.2.3

#### 7.6.4.1	NR SA FR2 SRS-RSRP measurement in non-DRX

Editor's Note:  This test case is incomplete. Following aspects are either missing or TBD

- The test applicability is FFS

- The test procedure is incomplete

- The message content is FFS

- MU/TT analysis for UE PC3 and test frequency f ≤ 40.8 GHz is complete.

- MU/TT analysis for UE power class other than PC3 is incomplete.

- MU//TT analysis for test frequency f > 40.8 GHz is incomplete.

7.6.4.1.1	Test purpose

The purpose of this test case is to verify that the UE makes correct reporting of SRS-RSRP measurement in non-DRX within SRS-RSRP measurement requirements in TS 38.133 [6] clause 9.7.2.5

7.6.4.1.2	Test applicability

FFS

7.6.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.4.0.1

The normative reference for this requirement is TS 38.133 [6] clause 9.7.2.5 and A.7.6.4.1.

7.6.4.1.4	Test description

One cell is deployed in the test, which is FR2 PCell (Cell 1).

7.6.4.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.4.1.4.1-1.

Table 7.6.4.1.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.4.1 - 1 | NR 120 kHz SRS SCS, 100 MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 7.6.4.1.4.1-2.

Table 7.6.4.1.4.1-2: Initial conditions SA FR2 SRS-RSRP measurement in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The test parameters are given in Table 7.6.4.1.4.1-3 below.

2. Message contents are defined in clause 7.6.4.1.4.3.

3. One cell is deployed in the test, which is FR2 PCell (Cell 1).. The test parameters for PSCell is given in Table A.7.6.4.1.4.1-3

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx Beam Peak Search procedures as described in Annex I.

Table 7.6.4.1.4.1-3: General test parameters for SRS-RSRP event triggered reporting for PCell in FR2

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 |  |
| RF Channel Number |  | 1 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| SRS configuration |  | 1 | SRSConf.1 | Table A.7.6.4.1.2-4 |
| CP length |  | 1 | Normal |  |
| i1-Threshold | dBm | 1 | -103 |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX | ms | 1 | OFF | Non-DRX |
| Time offset between DL from serving cell and SRS from test system | s | 1 | 10.67 |  |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 1 |  |

7.6.4.1.4.2	Test procedure

There is one cell is deployed in the test, which is FR2 PSCell (Cell 1)

In the measurement control information, a measurement object is configured for the frequency of the PSCell, and it is indicated to the UE that event-triggered reporting with Event I1 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively.

During the test, the test system transmits SRS resource for measurement in the DL slot according to the SRS configuration in Table 7.6.4.1.5-3 and the test parameters for the (virtual) neighbour cell UE in Table 7.6.4.1.5-23. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 2 data symbols before SRS to be transmitted.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.6.4.1.5-1 as appropriate.

3. 	The SS shall transmit an RRCReconfiguration message on Cell 1 with event I1 configured..

4. 	The UE shall transmit an RRCReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

<rest of the steps are FFS >

7.6.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

FFS

7.6.4.1.5	Test requirement

Table 7.6.4.1.5-1 defines the cell specific settings for all tests. Table 7.6.4.1.5-2 defines the OTA primary level settings including test tolerances for all tests.

Table 7.6.4.1.5-1: NR Cell specific test parameters for SA SRS-RSRP event triggered reporting for PCell in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |
| OCNG Patterns |  | 1 | OP.1 |  |
| TRS configuration |  |  | TRS.2.1. TDD |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  |
| Propagation Condition |  | 1 | AWGN |  |

Table 7.6.4.1.5-2: NR OTA Cell specific test parameters for SA SRS-RSRP event triggered reporting for PCell and neighbour cell UE in FR2

| Parameter | Unit | Test configuration | Cell 1 |  | Neighbour cell UE |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1 | Setup 1 defined in A.3.15.1 |  |  |  |
| Beam assumption Note 4 |  | 1 | Fine |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98-100.5 |  | -98-100.5 |  |
| Note 2 | dBm/SCS | 1 | -89 |  | -89 |  |
|  | dB | 1 | - | - | -infinity | 4 |
|  | dB | 1 | - | - | -infinity | 4 |
| SRS-RSRP Note 3 | dBm/SCS kHz | 1 | - | - | -infinity | -94 |
| Io | dBm/95.04 MHz | 1 | -70.01 | -68.82 | -70.01 | -68.82 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |

Table7.6.4.1.5-3: SRS configuration for measurement reporting

|  | Field | SRSConf.1 | Comments |
| --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 |  |
|  | srs-ResourceIdList | 0 |  |
|  | resourceType | Periodic |  |
|  | Usage | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 |  |
|  | nrofSRS-Ports | Port1 |  |
|  | transmissionComb | n2 |  |
|  | combOffset-n2 | 0 |  |
|  | cyclicShift-n2 | 0 |  |
|  | resourceMapping startPosition | 0 |  |
|  | resourceMapping nrofSymbols | n1 |  |
|  | resourceMapping repetitionFactor | n1 |  |
|  | freqDomainPosition | 0 |  |
|  | freqDomainShift | 0 |  |
|  | freqHopping c-SRS | 12 |  |
|  | freqHopping b-SRS | 0 |  |
|  | freqHopping b-hop | 0 |  |
|  | groupOrSequenceHopping | Neither |  |
|  | resourceType | Periodic |  |
|  | periodicityAndOffset | sl40, 25 |  |
|  | sequenceId | 0 | Any 10 bit number |

The UE shall send one Event I1 triggered measurement report, with a measurement reporting delay less than 60 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<<Some Unchanged Sections>>

### 7.7.5	SRS-RSRP

#### 7.7.5.0	Minimum conformance requirements

##### 7.7.5.0.1	Minimum conformance requirements for SRS-RSRP measurement accuracy

The SRS-RSRP measurement reported by the UE shall fulfil the accuracy requirements defined in Table 7.7.5.0.1-1 for FR1 and Table 5.7.7.0.1-2 for FR2, provided that the following conditions are met. The accuracy requirements in this clause are derived based on AWGN radio propagation conditions.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for SRS-RSRP measurements are fulfilled according to Annex B.2.z for a corresponding Band for each relevant SRS resource configured for measurement.

-	The time difference between UE’s DL reference timing in the serving cell and SRS arrival time is no larger than Terror_SRS_RSRP, where

-	Terror_SRS_RSRP = TC × NTA_offset + 4.67us for FR1

-	Terror_SRS_RSRP = TC × NTA_offset + 3.67us for FR2

-	NTA_offset is defined in Table 7.1.2-2

-	TC is 0.509ns

-	The number of SRS ports in the SRS resource configured for measurement is 1,

-	The number of symbols in the SRS resource configured for measurement is 1,

-	The number of repetitions in the SRS resource configured for measurement is 1,

-	Frequency hopping, sequence group hopping or sequence hopping is disabled in the SRS resource configured for measurement,

-	The bandwidth of the SRS resource is 48 PRBs.

-	One of the following conditions is met

-	There is no other SRS resource with the same root sequence and on the same symbol and with same comb as the relevant SRS resource.

-	If multiple SRS resources are on the same symbol and with same comb, the distance between cyclic shifts of any two resources is no less than 6 if transmissionComb = n4, and no less than 4 if transmissionComb = n2.

Table 7.7.5.0.1-1: SRS-RSRP absolute accuracy in FR1

| Accuracy |  |  |  |  |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  |  | Extreme condition |  |  | SRS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB |  |  |  |  |  | dB |  | dBm / SCSSRS |  |  | dBm/BW Channel | dBm/BW Channel |
| SCSSRS (kHz) |  |  | SCSSRS (kHz) |  |  |  |  |  |  |  |  |  |
| 15 | 30 | 60 | 15 | 30 | 60 |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |  |
| 3 | 3.5 | 5 | 7.5 | 8 | 9.5 | 1 | NR_TDD_FR1_A, | -120 | -117 | -114 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_C | -119 | -116 | -113 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_E | -118 | -115 | -112 | N/A | -70 |
| 6.5 | 7 | 8.5 | 9.5 | 10 | 11.5 | 1 | NR_TDD_FR1_A, NR_TDD_FR1_C, NR_TDD_FR1_D, NR_TDD_FR1_E | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.7.5.0.1-2: SRS-RSRP absolute accuracy in FR2

| Accuracy |  |  |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  | Extreme condition |  | SRS Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  |  |  | Minimum Io |  |  | Maximum Io |
| dB |  |  |  | dB | dBm / SCSSRS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
| SCSSRS (kHz) |  | SCSSRS (kHz) |  |  | SCSSRS = | SCSSRS = |  |  |
| 60 | 120 | 60 | 120 |  | 60kHz | 120kHz |  |  |
| 6 | 8.5 | 9 | 11.5 | ≥1 | Same value as SRS_RP in Table B.2.7-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 9 | 11.5 | 11 | 13.5 | ≥1 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |  |

7.7.5.0.1.1	SRS-RSRP report mapping

The reporting range of SRS-RSRP is defined from -140 dBm to -44 dBm with 1 dB resolution. The mapping of measured quantity is defined in Table 5.7.7.0.1.2-1. The range in the signalling may be larger than the guaranteed accuracy range.

Table 5.7.7.0.1.2-1: SRS-RSRP measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| SRS-RSRP_0 | SRS-RSRP<-140 | dBm |
| SRS-RSRP_1 | -140≤ SRS-RSRP<-139 | dBm |
| SRS-RSRP_2 | -139≤ SRS-RSRP<-138 | dBm |
| SRS-RSRP_3 | -138≤ SRS-RSRP<-137 | dBm |
| SRS-RSRP_4 | -137≤ SRS-RSRP<-136 | dBm |
| .. | .. | … |
| SRS-RSRP_95 | -46≤ SRS-RSRP<-45 | dBm |
| SRS-RSRP_96 | -45≤ SRS-RSRP<-44 | dBm |
| SRS-RSRP_97 | -44≤ SRS-RSRP | dBm |
| SRS-RSRP_98 | Infinity |  |
| Note: ‘Infinity’ means that UE cannot detect SRS due to too strong signal to measure. |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.22.1.

#### 7.7.5.1	NR SA FR2 SRS-RSRP measurement accuracy

Editor's Note: This test case is incomplete. Following aspects are either missing or TBD

- The test procedure is incomplete

- The message content is FFS

- MU/TT analysis for UE PC3 and test frequency f ≤ 40.8 GHz is complete.

- MU/TT analysis for UE power class other than PC3 is incomplete.

- MU//TT analysis for test frequency f > 40.8 GHz is incomplete.

7.7.5.1.1	Test purpose

The purpose of this test is to verify that the SRS-RSRP measurement accuracy is within the specified limits with SRS-RSRP measurement requirements in TS 38.133 [6] clause 10.1.22.1.1.

7.7.5.1.2	Test applicability

This test applies to all types of NR UE release 16 onwards and supporting CLI-SRS-RSRP.

7.7.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.7.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.7.5.1.

7.7.5.1.4	Test description

7.7.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.5.1.4.1-1.

Table 7.7.5.1.4.1-1: Applicable NR configurations for FR2 SRS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | 120 kHz SRS SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 7.7.5.1.4.1-2.

Table 7.7.5.1.4.1-2: Initial conditions SA FR2 SRS-RSRP measurement accuracy

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 7.7.5.1.4.3.

2. In this set of test cases there is one cell in the test, FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in Table 7.7.5.1.5-1 and Table 7.7.5.1.5-2 below. The test parameter for the (virtual) neighbour cell UE transmitting SRS are given in Table 7.7.5.1.5-2.

3.	The UE Rx beam peak direction has been obtained previously using one of the Rx Beam Peak Search procedures as described in Annex I.

7.7.5.1.4	Test description

7.7.5.1.4.1	Initial conditions

7.7.5.1.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.7.5.1.5-1 as appropriate.

3. 	The SS shall transmit an RRCReconfiguration message on Cell 1.

4. 	The UE shall transmit an RRCReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

<rest of the steps are FFS >

7.7.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

FFS

7.7.5.1.5	Test requirement

Table 7.7.5.1.5-1 defines the cell specific settings for all tests. Table 7.7.5.1.5-2 defines the OTA primary level settings including test tolerances for all tests.

Table 7.7.5.1.5-1: FR2 test parameters for SRS-RSRP accuracy

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 | freq1 |
| Duplex mode | 1 |  | TDD | TDD |
| TDD configuration | 1 |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR2 | SSB.3 FR2 |
| OCNG Patterns | 1 |  | OP.1 | OP.1 |
| TRS configuration | 1 |  | TRS.2.1 TDD | TRS.2.1 TDD |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.3 ULBWP.1.3 | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1 |  | SMTC.1 | SMTC.1 |
| Time offset between DL from serving cell and SRS from test system | 1 | s | 10.76 | 10.67 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Propagation condition | 1 |  | AWGN | AWGN |
| Antenna configuration | 1 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |

Table 7.7.5.1.5-2: SRS-RSRP accuracy OTA related test parameters for PCell and Neighbour cell UE in FR2

| Parameter | Unit | T1 | T2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 defined A.3.15.1 | Setup 1 defined A.3.15.1 |
| Beam assumption Note 5 |  | Fine | Fine |
| Note1 | dBm/15kHzNote3 | -100 | N/A |
| Note1 | dBm/SCSNote3 | -91 | N/A |
|  | dB | 2 | N/A |
| Es | dBm/SCSNote3 |  | (Table B.2.7-2 Rx Beam Peak) +5 |
| SRS_RPNote2 | dBm/SCS | -89 | (Table B.2.7-2 Rx Beam Peak) +5 |
| BB Note4 | dB | >1 | 1 |
| IoNote2 | dBm/95.04 MHz Note3 | -57.89 | (Table B.2.7-2 Rx Beam Peak +50.79dB) |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SRS_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 2dB for UE multi-band relaxation factor ∑MBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |

Table 7.7.5.1.5-3: SRS configuration parameters for FR2 SRS-RSRP accuracy

|  | Field | SRSConf.1 |
| --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 |
|  | srs-ResourceIdList | 0 |
|  | resourceType | Periodic |
|  | Usage | Codebook |
| SRS-Resource | SRS-ResourceId | 0 |
|  | nrofSRS-Ports | Port1 |
|  | transmissionComb | n2 |
|  | combOffset-n2 | 0 |
|  | cyclicShift-n2 | 0 |
|  | resourceMapping startPosition | 0 |
|  | resourceMapping nrofSymbols | n1 |
|  | resourceMapping repetitionFactor | n1 |
|  | freqDomainPosition | 0 |
|  | freqDomainShift | 0 |
|  | freqHopping c-SRS | 12 |
|  | freqHopping b-SRS | 0 |
|  | freqHopping b-hop | 0 |
|  | groupOrSequenceHopping | Neither |
|  | resourceType | Periodic |
|  | periodicityAndOffset-p | sl160,25 |
|  | sequenceId | 0 |

Table 7.7.5.1.5-4: SRS-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
| SRS | SRS_RP -δ +Gmin ≤ Reported SRS-RSRP(dBm) ≤SRS_RP +δ +Gmax |
| Note 1: SRS_RP is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test Note 2: δ is the RSRP absolute accuracy requirement from Table 10.1.22.1.1-2, selected according to the Io used in the test Note 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

The SRS-RSRP measurement accuracy shall fulfil the absolute accuracy requirements in clauses 7.7.5.0. The following requirements are to be verified:

During T1:

The UE is deemed to meet the requirement if the reported SRS-RSRP is in the range shown in table 7.7.5.1.5-4.

During T2:

The UE is deemed to meet the requirement if the reported SRS-RSRP is in the range shown in table 7.7.5.1.5-4.

For the test to pass, the ratio of successful reported values for each requirement shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

<< Some Unchanged Sections>>

Table F.1.1.2-5: Maximum test system uncertainty for RRM requirements for EN-DC FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 5.3.2.2.1 EN-DC FR2 contention based random access | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.3.2.2.2 EN-DC FR2 non-contention based random access | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB ±48Tc Uplink signal transmit timing relative to downlink ±40Tc Relative during UE timing adjustment | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.4.3.1 EN-DC FR2 UE timing advance adjustment accuracy | Noc ±5.65 dB, f<=40.8 GHz Ês1 / Noc ±0.3 dB ±40Tc timing advance adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.5.1.5 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Average Noc ±5.65 dB, f<=40.8 GHz Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Fading profile ±0.5 dB for 1Tx | Ês1 / Noc is the SNR of RLM-RS1 Ês2 / Noc is the SNR of RLM-RS2 |
| 5.5.1.6 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 5.5.1.5 | Same as 5.5.1.5 |
| 5.5.2.1 EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.5.2.2 EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Same as 5.5.2.1 | Same as 5.5.2.1 |
| 5.5.5.1 EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX | Noc averaged over BWConfig, ±5.65 dB, f<=40.8 GHz Noc averaged over measured PRBs, ±5.65 dB, f<=40.8 GHz Ês / Noc averaged over BWConfig: ±0.3 dB Ês / Noc averaged over measured PRBs: ±0.3 dB Fading profile ±0.7 dB, 2Tx | Noc averaged over BWConfig is the AWGN absolute power averaged over BWConfig Noc averaged over measured PRBs is the AWGN absolute power averaged over measured PRBs Ês / Noc averaged over BWConfig is the Ratio of cell signal / AWGN averaged over BWConfig Ês / Noc averaged over measured PRBs is the Ratio of cell signal / AWGN averaged over measured PRBs |
| 5.5.5.2 EN-DC FR2 SSB-based beam failure detection and link recovery in DRX | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.3 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB Fading profile ±0.7 dB for 2Tx | Note: Noc is the AWGN on cell 2 frequency Ês2 / Noc is the SNR for the CSI-RS |
| 5.5.5.4 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.3 | Same as 5.5.5.3 |
| 5.5.5.5 EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.6 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | Noc ±5.65 dB, f<=40.8 GHz Ês3 / Noc ±0.3 dB Fading profile ±0.7 dB for 2Tx | Note: Noc is the AWGN on cell 3 frequency Ês3 / Noc is the SNR for the CSI-RS |
| 5.5.5.7 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.6 | Same as 5.5.5.6 |
| 5.6.1.1 EN-DC FR2 event-triggered reporting without gap in non-DRX | Ês2 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês2 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz Ês3 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês3 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz | Ês2 averaged over BW is the absolute power of cell 2 signal averaged over BW Ês2 averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB Ês3 averaged over BW is the absolute power of cell 3 signal averaged over BW Ês3 averaged over measured PRB is the absolute power of cell 3 signal averaged over measured PRB |
| 5.6.1.2 EN-DC FR2 event-triggered reporting without gap in DRX | Noc ±5.65 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |
| 5.6.1.3 EN-DC FR2 event-triggered reporting without gap in non-DRX | Same as 5.6.1.1 | Same as 5.6.1.1 |
| 5.6.1.4 EN-DC FR2 event-triggered reporting with gap in DRX | Same as 5.6.1.2 | Same as 5.6.1.2 |
| 5.6.2.1 EN-DC FR2-FR2 event-triggered reporting in non-DRX | Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB Meas PRB Ês2 ±5.65 dB | Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.2 EN-DC FR2-FR2 event-triggered reporting in DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc ±0.3 dB Ês2 / Noc over meas PRBs ±0.3 dB | Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.3 EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.1 | Same as 5.6.2.1 |
| 5.6.2.4 EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.2 | Same as 5.6.2.2 |
| 5.6.2.5 EN-DC FR1-FR2 event-triggered reporting in non-DRX | Average Ês2 ±5.65, f <= 40.8 GHz | Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.6 EN-DC FR1-FR2 event-triggered reporting in DRX | Noc ±5.65 dB, f <= 40.8 GHz Ês2 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.6.2.7 EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 5.6.2.8 EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 5.6.3.1 EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.3.2 EN-DC FR2 SSB-based L1-RSRP measurement in DRX | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 5.6.3.3 EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-RSRP |
| 5.6.3.4 EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 5.6.3.3 | Same as 5.6.3.3 |
| 5.6.4.1 EN-DC FR2 SRS-RSRP measurement in non-DRX | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell2 signal/AWGN |
| 5.6.6.1 EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-SINR #RBJ to RBJ+275 |
| 5.6.6.2 EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-SINR#RBJ to RBJ+19 |
| 5.6.6.3 EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Same as 5.6.6.1 | Same as 5.6.6.1 |
| 5.7.1.1 EN-DC FR2 SS-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Ês2 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.1.2 EN-DC FR2-FR2 SS-RSRP measurement accuracy | Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over measPRBs ±5.65 dB, f <= 40.8 GHz Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over measPRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on the NR freq 1 Noc2 is the AWGN on the NR freq 2 Ês1 / Noc1 is the ratio of cell 2 signal / AWGN Ês2 / Noc2 is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.2.1 EN-DC FR2 SS-RSRQ measurement accuracy | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Ês2 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.2.2 EN-DC FR2-FR2 SS-RSRQ measurement accuracy | Same as 5.7.1.2 | Same as 5.7.1.2 |
| 5.7.3.1 EN-DC FR2 SS-SINR measurement accuracy | Same as 5.7.2.1 | Same as 5.7.2.1 |
| 5.7.3.2 EN-DC FR2-FR2 SS-SINR measurement accuracy | Same as 5.7.1.2 | Same as 5.7.1.2 |
| 5.7.4.1 EN-DC FR2 SSB based L1-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.4.2 EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for CSI-RS-RSRP Test 2 Ês1 is NR cell 2 signal Meas PRB is the measurement PRB for CSI-RS-RSRP |
| 5.7.5.1 EN-DC FR2 SRS-RSRP measurement accuracy | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell2 signal/AWGN |

Table F.1.1.2-6: Maximum test system uncertainty for RRM requirements for SA FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.3.1.4 NR SA FR1-FR2 synchronous DAPS handover | Noc, averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es2/Noc averaged over BWConfig ±0.3 dB Es2/Noc averaged over Measurement PRB ±0.3 dB | Noc averaged over BWConfig is the AWGN absolute power of RF channel 2 averaged over BWConfig Noc averaged over Measurement PRB is the AWGN absolute power of RF channel 2 averaged over Measurement PRB Es2/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |
| 7.3.1.5 NR SA FR1-FR2 asynchronous DAPS handover | Same as 7.3.1.4 | Same as 7.3.1.4 |
| 7.3.2.1.1 NR SA FR2 RRC re-establishment | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc ±0.3 dB Ês2 / Noc over meas PRBs ±0.3 dB | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.3.2.1.2 NR SA FR2 - FR2 RRC re-establishment | Average Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc1 ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Ês2 / Noc over meas PRBs ±0.3 dB | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.3.2.2.1 NR SA FR2 contention based random access | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 7.3.2.2.2 NR SA FR2 non-contention based random access | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 |
| 7.3.3.1 NR SA FR2 conditional handover | Noc averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es1/Noc averaged over BWConfig ±0.3 dB Es1/Noc averaged over Measurement PRB ±0.3 dB Es2/Noc ±0.3 dB Noc, averaged over BWConfig, ±5.65 dB Es2/Noc averaged over Measurement PRB ±0.3 dB | Noc averaged over BWConfig,is the AWGN absolute power of RF channel 1 averaged over BWConfig, Noc averaged over Measurement PRB,is the AWGN absolute power of RF channel 1 averaged over Measurement PRB, Es1/Noc averaged over BWConfig is Ratio of cell 1 signal / AWGN averaged over BWConfig Es1/Noc averaged over Measurement PRB is Ratio of cell 1 signal / AWGN averaged over Measurement PRB Es2/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |
| 7.4.1.1 SA FR2 UE transmit timing accuracy | Noc ±5.65 dB Ês1 / Noc ±0.3 dB ±48Tc Uplink signal transmit timing relative to downlink ±40Tc Relative during UE timing adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 7.4.3.1 NR SA FR2 UE timing advance adjustment accuracy | Noc ±5.65 dB Ês1 / Noc ±0.3 dB ±40Tc timing advance adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 7.5.5.1 NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 7.5.5.2 NR SA FR2 SSB-based beam failure detection and link recovery in DRX | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 7.5.5.3 NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Noc ±5.65 dB, f<=40.8 GHz Ês1 / Noc ±0.3 dB Fading profile ±0.7 dB for 2Tx | Note: Noc is the AWGN on cell 1 frequency Ês1 / Noc is the SNR for the CSI-RS |
| 7.5.5.4 NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX | Same as 7.5.5.3 | Same as 7.5.5.3 |
| 7.5.5.5 NR SA FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 7.5.5.6 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB Fading profile ±0.7 dB for 2Tx | Note: Noc is the AWGN on cell 2 frequency Ês2 / Noc is the SNR for the CSI-RS |
| 7.5.5.7 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Same as 7.5.5.6 | Same as 7.5.5.6 |
| 7.6.1.1 NR SA FR2 event-triggered reporting without gap in non-DRX | Ês1 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês1 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz Ês2 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês2 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz | Ês1 averaged over BW is the absolute power of cell 1 signal averaged over BW Ês1 averaged over measured PRB is the absolute power of cell 1 signal averaged over measured PRB Ês2 averaged over BW is the absolute power of cell 2 signal averaged over BW Ês2 averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB |
| 7.6.1.2 NR SA FR2 event-triggered reporting without gap in DRX | Noc averaged over BW, ±5.65 dB, f <= 40.8 GHz Noc averaged over measured PRB, ±5.65 dB, f <= 40.8 GHz Ês1 / Noc averaged over BW, ±0.3 dB, f <= 40.8 GHz Ês1 / Noc averaged over measured PRB, ±0.3 dB, f <= 40.8 GHz Ês2 / Noc averaged over BW, ±0.3 dB, f <= 40.8 GHz Ês2 / Noc averaged over measured PRB, ±0.3 dB, f <= 40.8 GHz | Noc averaged over BW is AWGN absolute power averaged over BW. Noc averaged over measured PRB is AWGN absolute power averaged over measured PRB. Ês1 / Noc averaged over BW is the ratio of cell 1 signal / AWGN averaged over BW Ês1 / Noc averaged over measured PRB is the ratio of cell 1 signal / AWGN averaged over measured PRB Ês2 / Noc averaged over BW is the ratio of cell 2 signal / AWGN averaged over BW. Ês2 / Noc averaged over measured PRB is the ratio of cell 2 signal / AWGN averaged over measured PRB. |
| 7.6.1.3 NR SA FR2 event-triggered reporting without gap in non-DRX | Same as 7.6.1.1 | Same as 7.6.1.1 |
| 7.6.1.4 NR SA FR2 event-triggered reporting with gap in DRX | Same as 7.6.1.2 | Same as 7.6.1.2 |
| 7.6.2.1 NR SA FR2-FR2 event-triggered reporting in non-DRX | Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB Meas PRB Ês2 ±5.65 dB | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.6.2.2 NR SA FR2-FR2 event-triggered reporting in DRX | Average Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc1 ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Ês2 / Noc2 over meas PRBs ±0.3 dB | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.6.2.3 NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 7.6.2.4 NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 7.6.2.2 | Same as 7.6.2.2 |
| 7.6.2.5 NR SA FR1-FR2 event-triggered reporting in non-DRX | Average Ês2 ±5.65 dB, f <= 40.8 GHz | Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.6.2.6 NR SA FR1-FR2 event-triggered reporting in DRX | Noc ±5.65 dB, f <= 40.8 GHz Ês2 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 7.6.2.7 NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 7.6.2.5 | Same as 7.6.2.5 |
| 7.6.2.8 NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 7.6.2.6 | Same as 7.6.2.6 |
| 7.6.3.1 NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.6.3.2 NR SA FR2 SSB-based L1-RSRP measurement in DRX | Same as 7.6.3.1 | Same as 7.6.3.1 |
| 7.6.3.3 NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-RSRP |
| 7.6.3.4 NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 7.6.3.3 | Same as 7.6.3.3 |
| 7.6.4.1 NR SA FR2 SRS-RSRP measurement in non-DRX | Same as 5.6.4.1 | Same as 5.6.4.1 |
| 7.6.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-SINR #RBJ to RBJ+275 |
| 7.6.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |
| 7.6.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Same as 7.6.6.1 | Same as 7.6.6.1 |
| 7.7.1.1 NR SA FR2 SS-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.7.1.2 NR SA FR2-FR2 SS-RSRP measurement accuracy | Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over measPRBs ±5.65 dB, f <= 40.8 GHz Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over measPRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on the NR freq 1 Noc2 is the AWGN on the NR freq 2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.7.2.1 NR SA FR2 SS-RSRQ measurement accuracy | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.7.2.2 NR SA FR2-FR2 SS-RSRQ measurement accuracy | Same as 7.7.1.2 | Same as 7.7.1.2 |
| 7.7.3.1 NR SA FR2 SS-SINR measurement accuracy | Same as 7.7.2.1 | Same as 7.7.2.1 |
| 7.7.3.3 NR SA FR2-FR2 SS-SINR measurement accuracy | Same as 7.7.1.2 | Same as 7.7.1.2 |
| 7.7.4.1 NR SA FR2 SSB based L1-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 1 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.7.4.2 NR SA FR2 CSI-RS based L1-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for CSI-RS-RSRP Test 2 Ês1 is NR cell 1 signal Meas PRB is the measurement PRB for CSI-RS-RSRP |
| 7.7.5.1 NR SA FR2 SRS-RSRP measurement accuracy | Same as 5.7.5.1 | Same as 5.7.5.1 |

<< Some Unchanged Sections>>

### F.1.3.2	Measurement of RRM requirements

Because the relationships between the test system uncertainties and the test tolerances are often complex, it is not always possible to give a simple derivation of the test requirement in this document. The analysis is recorded in 3GPP TR 38 903 [22].

The derivation of the test requirements for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.3.2-1.

The derivation of the test requirements for the NR/5GC FR1 test cases in chapter 6 is defined in Table F.1.3.2-2.

The derivation of the test requirements for the EN-DC FR2 test cases in chapter 5 is defined in Table F.1.3.2-3.

The derivation of the test requirements for the NR/5GC FR2 test cases in chapter 7 is defined in Table F.1.3.2-4.

<< Some Unchanged Sections>>

Table F.1.3.2-3: Derivation of test requirements for EN-DC FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 5.3.2.2.1 EN-DC FR2 contention based random access | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 5.3.2.2.2 EN-DC FR2 non-contention based random access | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB |
| 5.4.3.1 EN-DC FR2 UE timing advance adjustment accuracy | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE Timing Advance Adjustment Accuracy for 120kHz SCS = ±32 Tc | 0dB 0dB +/- 40 Tc | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE TAAA for 120kHz SCS = ±72 Tc |
| 5.5.1.5 EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -14 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -14 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB |
| 5.5.1.6 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -14 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -92.1dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -92.1dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -14 dB | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB During T4: -2.7 dB 0 dB 0 dB During T5: -2.7 dB 2.1 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -14 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -94.8 dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -94.8 dBm/15kHz Ês1 / Noc: 4.1 dB Ês2 / Noc: -14 dB |
| 5.5.5.1 EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX | During T1: SNR_SSB q0: 5 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: -3 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20.2 dB In signaling: rsrp-ThresholdSSB: -95 dBm/120kHz for Config 1,2 rsrp-ThresholdSSB: -92 dBm/240kHz for Config 3,4 | +8.70 dB 0 dB +8.70 dB 0 dB 0 dB -0.2 dB -14 dB -14 dB | During T1: SNR_SSB q0: 13.7 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: 5.7 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20 dB In signaling: rsrp-ThresholdSSB: -109 dBm/120kHz for Config 1,2 srp-ThresholdSSB: -106 dBm/240kHz for Config 3,4 |
| 5.5.5.2 EN-DC FR2 SSB-based beam failure detection and link recovery in DRX | Same as 5.5.5.1 | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.3 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdSSB: -95 dBm/SCS kHz | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdSSB: -109.0 dBm/SCS kHz |
| 5.5.5.4 EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.3 | Same as 5.5.5.3 | Same as 5.5.5.3 |
| 5.5.5.5 EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Same as 5.5.5.1 | Same as 5.5.5.1 | Same as 5.5.5.1 |
| 5.5.5.6 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês3 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês3 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdBFR: -95 dBm/SCS kHz | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês3 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês3 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdBFR: -109.0 dBm/SCS kHz |
| 5.5.5.7 EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Same as 5.5.5.6 | Same as 5.5.5.6 | Same as 5.5.5.6 |
| 5.6.1.1 EN-DC FR2 event-triggered reporting without gap in non-DRX | During T1: Ês2: -86dBm/120kHz Ês3: -infinity During T2: Ês2: -86dBm/120kHz Ês3: -86dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês2: -86dBm/120kHz Ês3: -infinity During T2: Ês2: -86dBm/120kHz Ês3: -86dBm/120kHz |
| 5.6.1.2 EN-DC FR2 event-triggered reporting without gap in DRX | During T1: Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +4.00dB Signalled A3-offset: -6.0 | During T1: -3.5dB 0dB 0dB During T2: -3.5dB 0dB 0dB Signalled A3-offset: -1.0 | During T1: Noc: -101.5dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Noc: -101.5dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +4.00dB Signalled A3-offset: -7.0 |
| 5.6.1.3 EN-DC FR2 event-triggered reporting with gap in non-DRX | Same as 5.6.1.1 | Same as 5.6.1.1 | Same as 5.6.1.1 |
| 5.6.1.4 EN-DC FR2 event-triggered reporting with gap in DRX | Same as 5.6.1.2 | Same as 5.6.1.2 | Same as 5.6.1.2 |
| 5.6.2.1 EN-DC FR2-FR2 event-triggered reporting in non-DRX | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -87 dBm/120kHz Ês2: -- infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz |
| 5.6.2.2 EN-DC FR2-FR2 event-triggered reporting in DRX | During T1: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +9.00dB Signalled A3-offset: -6.0 | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB Signalled A3-offset: -6.0 | During T1: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +9.00dB Signalled A3-offset: -12.0 |
| 5.6.2.3 EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.1 | Same as 5.6.2.1 | Same as 5.6.2.1 |
| 5.6.2.4 EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.2 | Same as 5.6.2.2 | Same as 5.6.2.2 |
| 5.6.2.5 EN-DC FR1-FR2 event-triggered reporting in non-DRX | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz | During T1: 0dB During T2: 0dB | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz |
| 5.6.2.6 EN-DC FR1-FR2 event-triggered reporting in DRX | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB |
| 5.6.2.7 EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.5 | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 5.6.2.8 EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.6 | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 5.6.3.1 EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB |
| 5.6.3.2 EN-DC FR2 SSB-based L1-RSRP measurement in DRX | Same as 5.6.3.1 | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 5.6.3.3 EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-RSRP values ± 29dB Reported Differential CSI-RSRP values ± 7dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: RSRP_40 to RSRP_99 CSI-RS#0: DIFFRSRP_1 to DIFFRSRP_7 |
| 5.6.3.4 EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 5.6.3.3 | Same as 5.6.3.3 | Same as 5.6.3.3 |
| 5.6.4.1 EN-DC FR2 SRS-RSRP measurement in non-DRX | During T1: Noc: -98 dBm/15kHz Io: -70.01 dBm/ 95.04 MHz During T2: Noc: -98 dBm/15kHz Io: -68.82 dBm/ 95.04 MHz | During T1: -2.5dB 0dB During T2: -2.5dB 0dB | During T1: Noc: -100.5 dBm/15kHz Io: -70.01 dBm/ 95.04 MHz During T2: Noc: -100.5 dBm/15kHz Io: -68.82 dBm/ 95.04 MHz |
| 5.6.6.1 EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-SINR values ± 5.5dB Reported Differential CSI-SINR values ± 4.5dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: SINR_51 to SINR_74 CSI-RS#0: DIFFSINR_4 to DIFFSINR_13 |
| 5.6.6.2 EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported SS-SINR values ± 4.0dB Reported Differential SS-SINR values ± 3.0dB | During T1: 0dB 1.5dB 0dB During T2: 0dB 1.5dB 0dB Via mapping | During T1: Noc: -105dBm/15kHz Ês0 / Noc: +1.50dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: +1.50dB Ês1 / Noc: +9.00dB SSB#1: SINR_54 to SINR_71 SSB#0: DIFFSINR_4 to DIFFSINR_10 |
| 5.6.6.3 EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-SINR values ± 4.5dB Reported Differential CSI-SINR values ± 3.5dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: SINR_53 to SINR_72 CSI-RS#0: DIFFSINR_5 to DIFFSINR_12 |
| 5.7.1.1 EN-DC FR2 SS-RSRP measurement accuracy | Test 1: Noc: -91.60dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês2: -110.0dBm/120kHz n257 Ês3: -110.0dBm/120kHz n260 Ês2: -107.4dBm/120kHz n260 Ês3: -107.4dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 7.70dB 7.70dB 7.70dB 7.70dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB Cell 2: RSRP_50 to RSRP_108 Cell 3: RSRP_46 to RSRP_103 Cell 3: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês2: -102.3dBm/120kHz n257 Ês3: -102.3dBm/120kHz n260 Ês2: -99.7dBm/120kHz n260 Ês3: -99.7dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |
| 5.7.1.2 EN-DC FR2-FR2 SS-RSRP measurement accuracy | Test 1 Config 1: Noc1: -90.60dBm/15kHz Noc2: -90.60dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 1: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 1 Config 2: Noc1: -93.70dBm/15kHz Noc2: -93.70dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 2: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1 Config 1: -5.70dB -5.70dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 1: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping Test 1 Config 2: -5.60dB -5.60dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 2: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping | Test 1 Config 1: Noc1: -96.30dBm/15kHz Noc2: -96.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 2 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 3: RSRP_52 to RSRP_109 Cell 3: n257: RSRP_x-15 to RSRP_x+25 n260: RSRP_x-15 to RSRP_x+27 Test 2 Config 1: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 2 n257: RSRP_33 to RSRP_101 n260: RSRP_34 to RSRP_104 Cell 3: n257: RSRP_32 to RSRP_87 n260: RSRP_34 to RSRP_90 Cell 3: n257: RSRP_x-29 to RSRP_x+11 n260: RSRP_x-29 to RSRP_x+13 Test 1 Config 2: Noc1: -99.30dBm/15kHz Noc2: -99.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 12 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 3: RSRP_52 to RSRP_109 Cell 3: n257: RSRP_x-15 to RSRP_x+25 n260: RSRP_x-15 to RSRP_x+27 Test 2 Config 2: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 2 n257: RSRP_36 to RSRP_104 n260: RSRP_37 to RSRP_107 Cell 3: n257: RSRP_35 to RSRP_90 n260: RSRP_37 to RSRP_93 Cell 3: n257: RSRP_x-29 to RSRP_x+11 n260: RSRP_x-29 to RSRP_x+13 |
| 5.7.2.1 EN-DC FR2 SS-RSRQ measurement accuracy | Test 1: Noc: -95.0dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Test 2: Noc: -95.0dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: -5.70dB 0dB 0dB Via Mapping Test 2: -1.70dB 0dB 0dB Via Mapping | Test 1: Noc: -100.70dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Cell 3: All bands: RSRQ_41 to RSRQ_73 Test 2: Noc: -96.70dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Cell 3: n257, n258, n261: RSRQ_35 to RSRQ_71 n260: RSRQ_34 to RSRQ_71 |
| 5.7.2.2 EN-DC FR2-FR2 SS-RSRQ measurement accuracy | Test 1: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Reported relative RSRQ values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Test 2: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported relative RSRQ values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: -1.90dB -1.90dB 0dB 0dB Via Mapping Via Mapping Test 2: -1.41dB -1.41dB 0dB 0dB Via Mapping Via Mapping | Test 1: Noc1: -95.93dBm/15kHz Noc2: -95.93dBm /15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Cell 2 and Cell 3: RSRQ_41 to RSRQ_73 Cell 3: RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -95.44dBm/15kHz Noc2: -95.44dBm /15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Cell 2 and Cell 3: RSRQ_37 to RSRQ_74 Cell 3: RSRQ_x-9 to RSRQ_x+9 |
| 5.7.3.1 EN-DC FR2 SS-SINR measurement accuracy | Test 1: Noc: -105.0dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +2.66dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc: -105.0dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB Via Mapping Test 2: 0dB 0.2dB 0.2dB Via Mapping | Test 1: Noc: -105.0dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +2.66dB Cell 3: n257, n258, n261: SINR_22 to RSRQ_58 n260: SINR_21 to RSRQ_58 Test 2: Noc: -105.0dBm/15kHz Ês2 / Noc: -2.8dB Ês3 / Noc: -2.8dB Cell 3: n257, n258, n261: SINR_18 to RSRQ_55 n260: SINR_18 to RSRQ_54 |
| 5.7.3.2 EN-DC FR2-FR2 SS-SINR measurement accuracy | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported absolute SINR values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB 0dB Via Mapping Via Mapping Test 2: -0.1dB -0.1dB 0dB 0dB Via Mapping Via Mapping Test 3: 0dB 0dB 0.9dB 0.9dB Via Mapping Via Mapping | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Cell 2 and Cell 3: n257, n258, n261: SINR_27 to SINR_62 n260: SINR_27 to SINR_61 Cell 3: SINR_x-8 to SINR_x+8 Test 2: Noc1: -105.1dBm/15kHz Noc2: -105.1dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Cell 2 and Cell 3: n257, n258, n261: SINR_48 to SINR_87 n260: SINR_48 to SINR_86 Cell 3: SINR_x-17 to SINR_x+17 Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -2.1dB Ês2 / Noc2: -2.1dB Cell 2 and Cell 3: n257, n258, n261: SINR_23 to SINR_60 n260: SINR_22 to SINR_59 Cell 3: SINR_x-9 to SINR_x+9 |
| 5.7.4.1 EN-DC FR2 SSB based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 SSB#0 Ês: -108.1dBm/120kHz n257 SSB#1 Ês: -105.5dBm/120kHz n260 SSB#0 Ês: -108.1dBm/120kHz n260 SSB#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -4.1dB 0dB 0.4dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -1.6dB SSB#0: RSRP_42 to RSRP_101 SSB#1: RSRP_31 to RSRP_89 SSB#1-SSB#0: -9 to -2 Test 2: n257 SSB#0 Ês: -102.4dBm/120kHz n257 SSB#1 Ês: -99.8dBm/120kHz n260 SSB#0 Ês: -102.4dBm/120kHz n260 SSB#1 Ês: -99.8dBm/120kHz SSB#0 and SSB#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 SSB#1-SSB#0: -4 to +4 |
| 5.7.4.2 EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 CSI-RS#0 Ês: -108.1dBm/120kHz n257 CSI-RS#1 Ês: -105.5dBm/120kHz n260 CSI-RS#0 Ês: -108.1dBm/120kHz n260 CSI-RS#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -4.1dB 0dB 0.2dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -1.8dB CSI-RS#0: RSRP_48 to RSRP_95 CSI-RS#1: RSRP_30 to RSRP_89 CSI-RS#1-CSI-RS#0: -9 to -2 Test 2: n257 CSI-RS#0 Ês: -102.4dBm/120kHz n257 CSI-RS#1 Ês: -99.8dBm/120kHz n260 CSI-RS#0 Ês: -102.4dBm/120kHz n260 CSI-RS#1 Ês: -99.8dBm/120kHz CSI-RS#0 and CSI-RS#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 CSI-RS#1-CSI-RS#0: -4 to +4 |
| 5.7.5.1 EN-DC FR2 SRS-RSRP measurement accuracy | During T1: Noc: -98 dBm/15kHz Io: -70.01 dBm/ 95.04 MHz During T2: Noc: -98 dBm/15kHz Io: -68.82 dBm/ 95.04 MHz SRS_RP n257,n258, n261 -110.1 n259 -106.5 n260 -107.5 n257,n258, n261 -99.2 n259 -93.7 n260 -94.9 | During T1: 0dB During T2: 5dB | During T1: Noc: -98 dBm/15kHz Io: -70.01 dBm/ 95.04 MHz During T2: Noc: -98 dBm/15kHz Io: -68.82 dBm/ 95.04 MHz SRS_RP n257,n258, n261 -105.1 n259 -101.5 n260 -102.5 n257,n258, n261 -94.2 n259 -88.7 n260 -89.9 |

Table F.1.3.2-4: Derivation of test requirements for NR SA FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 7.3.1.4 NR SA FR1-FR2 synchronous DAPS handover | During T1: Noc -104.7dBm/15kHz Es2/Noc -infinity During T2-T5: Noc -104.7 dBm/15kHz Es2/Noc 10 dB | During T1: 0 dB 0 dB During T2-T5: 0 dB 0 dB | During T1: Noc -104.7dBm/15kHz Es2/Noc -infinity During T2-T5: Noc -104.7 dBm/15kHz Es2/Noc 10 dB |
| 7.3.1.5 NR SA FR1-FR2 asynchronous DAPS handover | Same as 7.3.1.4 | Same as 7.3.1.4 | Same as 7.3.1.4 |
| 7.3.2.1.1 NR SA FR2 RRC re-establishment | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 4 dB Ês2 / Noc = 2 dB During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 4 dB Ês2 / Noc = 2 dB During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB |
| 7.3.2.1.2 NR SA FR2 - FR2 RRC re-establishment | During T1: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = 0 dB Ês2 / Noc 2= - Infinity During T2: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = - Infinity During T3: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = 0 dB | During T1: -0.2dB -0.2dB 0dB 0dB During T2: -0.2dB -0.2dB 0dB 0dB During T3: -0.2dB -0.2dB 0dB 0dB | During T1: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = 0 dB Ês2 / Noc 2= - Infinity During T2: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = - Infinity During T3: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = 0 dB |
| 7.3.2.2.1 NR SA FR2 contention based random access | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 7.3.2.2.2 NR SA FR2 non-contention based random access | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 |
| 7.3.3.1 NR SA FR2 conditional handover | During T1: Noc -104.7dBm/15kHz Es1/Noc 6 dB Es2/Noc -infinity During T2: Noc -104.7dBm/15kHz Es1/Noc 6 dB Es2/Noc 11dB In signaling: A3-offset = -1dB | During T1 -1.5dB 0dB 0dB During T2 -1.5dB 0dB 0dB In signaling: -1dB | During T1: Noc -106.2 dBm/15kHz Es1/Noc 6 dB Es2/Noc -infinity During T2: Noc -106.2 dBm/15kHz Es1/Noc 6 dB Es2/Noc 11dB In signaling: A3-offset = -2dB |
| 7.4.1.1 SA FR2 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB |
| 7.4.3.1 NR SA FR2 UE timing advance adjustment accuracy | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE Timing Advance Adjustment Accuracy for 120kHz SCS = ±32 Tc | 0dB 0dB +/- 40 Tc | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE TAAA for 120kHz SCS = ±72 Tc |
| 7.5.5.1 NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX | During T1: SNR_SSB q0: 5 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: -3 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20.2 dB In signaling: rsrp-ThresholdSSB: -95 dBm/120kHz for Config 1 rsrp-ThresholdSSB: -92 dBm/240kHz for Config 2 | +8.70 dB 0 dB +8.70 dB 0 dB 0 dB -0.2 dB -14 dB -14 dB | During T1: SNR_SSB q0: 13.7 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: 5.7 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20 dB In signaling: rsrp-ThresholdSSB: -109 dBm/120kHz for Config 1 srp-ThresholdSSB: -106 dBm/240kHz for Config 2 |
| 7.5.5.2 NR SA FR2 SSB-based beam failure detection and link recovery in DRX | Same as 7.5.5.1 | Same as 7.5.5.1 | Same as 7.5.5.1 |
| 7.5.5.3 NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês1 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês1 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdSSB: -95 dBm/SCS kHz | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês1 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês1 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdSSB: -109.0 dBm/SCS kHz |
| 7.5.5.4 NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX | Same as 7.5.5.3 | Same as 7.5.5.3 | Same as 7.5.5.3 |
| 7.5.5.5 NR SA FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Same as 7.5.5.1 | Same as 7.5.5.1 | Same as 7.5.5.1 |
| 7.5.5.6 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdBFR: -95 dBm/SCS kHz | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdBFR: -109.0 dBm/SCS kHz |
| 7.5.5.7 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Same as 7.5.5.6 | Same as 7.5.5.6 | Same as 7.5.5.6 |
| 7.6.1.1 NR SA FR2 event-triggered reporting without gap in non-DRX | During T1: Ês1: -86dBm/120kHz Ês2: -infinity During T2: Ês1: -86dBm/120kHz Ês2: -86dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -86dBm/120kHz Ês2: -infinity During T2: Ês1: -86dBm/120kHz Ês2: -86dBm/120kHz |
| 7.6.1.2 NR SA FR2 event-triggered reporting without gap in DRX | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Signalled A3-offset: -6.0 | During T1: -3.5dB 0dB 0dB During T2: -3.5dB 0dB 0dB Signalled A3-offset: -1.0 | During T1: Noc: -101.5dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -101.5dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Signalled A3-offset: -7.0 |
| 7.6.1.3 NR SA FR2 event-triggered reporting with gap in non-DRX | Same as 7.6.1.1 | Same as 7.6.1.1 | Same as 7.6.1.1 |
| 7.6.1.4 NR SA FR2 event-triggered reporting with gap in DRX | Same as 7.6.1.2 | Same as 7.6.1.2 | Same as 7.6.1.2 |
| 7.6.2.1 NR SA FR2-FR2 event-triggered reporting in non-DRX | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz |
| 7.6.2.3 NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 7.6.2.1 | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 7.6.2.5 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1) | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz | During T1: 0dB During T2: 0dB | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz |
| 7.6.2.6 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1) | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB |
| 7.6.2.7 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1) | Same as 5.6.2.5 | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 7.6.2.8 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1) | Same as 5.6.2.6 | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 7.6.3.1 NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | Same as 5.6.3.1 | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 7.6.3.2 NR SA FR2 SSB-based L1-RSRP measurement in DRX | Same as 5.6.3.1 | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 7.6.3.3 NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-RSRP values ± 29dB Reported Differential CSI-RSRP values ± 7dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: RSRP_40 to RSRP_99 CSI-RS#0: DIFFRSRP_1 to DIFFRSRP_7 |
| 7.6.3.4 NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 7.6.3.3 | Same as 7.6.3.3 | Same as 7.6.3.3 |
| 7.6.4.1 NR SA FR2 SRS-RSRP measurement in non-DRX | Same as 5.6.4.1 | Same as 5.6.4.1 | Same as 5.6.4.1 |
| 7.6.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Same as 5.6.6.1 | Same as 5.6.6.1 | Same as 5.6.6.1 |
| 7.6.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported SS-SINR values ± 4.5dB Reported Differential SS-SINR values ± 3.5dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB Via mapping | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB SSB#1: SINR_53 to SINR_72 SSB#0: DIFFSINR_5 to DIFFSINR_12 |
| 7.6.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-SINR values ± 4.0dB Reported Differential CSI-SINR values ± 3.0dB | 0dB 1.5dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: +1.50dB Ês1 / Noc: +9.00dB CSI-RS#1: SINR_54 to SINR_71 CSI-RS#0: DIFFSINR_4 to DIFFSINR_10 |
| 7.7.1.1 NR SA FR2 SS-RSRP measurement accuracy | Test 1: Noc: -91.60dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês1: -110.0dBm/120kHz n257 Ês2: -110.0dBm/120kHz n260 Ês1: -107.4dBm/120kHz n260 Ês2: -107.4dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 7.70dB 7.70dB 7.70dB 7.70dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB Cell 1: RSRP_50 to RSRP_108 Cell 2: RSRP_46 to RSRP_103 Cell 2: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês1: -102.3dBm/120kHz n257 Ês2: -102.3dBm/120kHz n260 Ês1: -99.7dBm/120kHz n260 Ês2: -99.7dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |
| 7.7.1.2 NR SA FR2-FR2 SS-RSRP measurement accuracy | Test 1 Config 1: Noc1: -90.60dBm/15kHz Noc2: -90.60dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 1: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 1 Config 2: Noc1: -93.70dBm/15kHz Noc2: -93.70dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 2: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1 Config 1: -5.70dB -5.70dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 1: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping Test 1 Config 2: -5.60dB -5.60dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 2: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping | Test 1 Config 1: Noc1: -96.30dBm/15kHz Noc2: -96.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 1 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 2: RSRP_52 to RSRP_109 Cell 2: n257: RSRP_x-15 to RSRP_x+25 n260: RSRP_x-15 to RSRP_x+27 Test 2 Config 1: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 1 n257: RSRP_33 to RSRP_101 n260: RSRP_34 to RSRP_104 Cell 2: n257: RSRP_32 to RSRP_87 n260: RSRP_34 to RSRP_90 Cell 2: n257: RSRP_x-29 to RSRP_x+11 n260: RSRP_x-29 to RSRP_x+13 Test 1 Config 2: Noc1: -99.30dBm/15kHz Noc2: -99.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 1 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 2: RSRP_52 to RSRP_109 Cell 2: n257: RSRP_x-15 to RSRP_x+25 n260: RSRP_x-15 to RSRP_x+27 Test 2 Config 2: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 1 n257: RSRP_36 to RSRP_104 n260: RSRP_37 to RSRP_107 Cell 2: n257: RSRP_35 to RSRP_90 n260: RSRP_37 to RSRP_93 Cell 2: n257: RSRP_x-29 to RSRP_x+11 n260: RSRP_x-29 to RSRP_x+13 |
| 7.7.2.1 NR SA FR2 SS-RSRQ measurement accuracy | Test 1: Noc: -95.0dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Test 2: Noc: -95.0dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: -5.70dB 0dB 0dB Via Mapping Test 2: -1.70dB 0dB 0dB Via Mapping | Test 1: Noc: -100.70dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Cell 2: All bands: RSRQ_41 to RSRQ_73 Test 2: Noc: -96.70dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Cell 2: n257, n258, n261: RSRQ_35 to RSRQ_71 n260: RSRQ_34 to RSRQ_71 |
| 7.7.2.2 NR SA FR2-FR2 SS-RSRQ measurement accuracy | Test 1: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Reported relative RSRQ values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Test 2: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported relative RSRQ values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: -1.90dB -1.90dB 0dB 0dB Via Mapping Via Mapping Test 2: -1.41dB -1.41dB 0dB 0dB Via Mapping Via Mapping | Test 1: Noc1: -95.93dBm/15kHz Noc2: -95.93dBm /15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Cell 1 and Cell 2: RSRQ_41 to RSRQ_73 Cell 2: RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -95.44dBm/15kHz Noc2: -95.44dBm /15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Cell 1 and Cell 2: RSRQ_37 to RSRQ_74 Cell 2: RSRQ_x-9 to RSRQ_x+9 |
| 7.7.3.1 NR SA FR2 SS-SINR measurement accuracy | Test 1: Noc: -105.0dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc: -105.0dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB Via Mapping Test 2: 0dB 0.2dB 0.2dB Via Mapping | Test 1: Noc: -105.0dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Cell 2: n257, n258, n261: SINR_22 to RSRQ_58 n260: SINR_21 to RSRQ_58 Test 2: Noc: -105.0dBm/15kHz Ês1 / Noc: -2.8dB Ês2 / Noc: -2.8dB Cell 2: n257, n258, n261: SINR_18 to RSRQ_55 n260: SINR_18 to RSRQ_54 |
| 7.7.3.2 NR SA FR2-FR2 SS-SINR measurement accuracy | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported absolute SINR values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB 0dB Via Mapping Via Mapping Test 2: -0.1dB -0.1dB 0dB 0dB Via Mapping Via Mapping Test 3: 0dB 0dB 0.9dB 0.9dB Via Mapping Via Mapping | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Cell 1 and Cell 2: n257, n258, n261: SINR_27 to SINR_62 n260: SINR_27 to SINR_61 Cell 2: SINR_x-8 to SINR_x+8 Test 2: Noc1: -105.1dBm/15kHz Noc2: -105.1dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Cell 1 and Cell 2: n257, n258, n261: SINR_48 to SINR_87 n260: SINR_48 to SINR_86 Cell 2: SINR_x-17 to SINR_x+17 Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -2.1dB Ês2 / Noc2: -2.1dB Cell 1 and Cell 2: n257, n258, n261: SINR_23 to SINR_60 n260: SINR_22 to SINR_59 Cell 2: SINR_x-9 to SINR_x+9 |
| 7.7.4.1 NR SA FR2 SSB based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 SSB#0 Ês: -108.1dBm/120kHz n257 SSB#1 Ês: -105.5dBm/120kHz n260 SSB#0 Ês: -108.1dBm/120kHz n260 SSB#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -4.1dB 0dB 0.4dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -1.6dB SSB#0: RSRP_42 to RSRP_101 SSB#1: RSRP_31 to RSRP_89 SSB#1-SSB#0: -9 to -2 Test 2: n257 SSB#0 Ês: -102.4dBm/120kHz n257 SSB#1 Ês: -99.8dBm/120kHz n260 SSB#0 Ês: -102.4dBm/120kHz n260 SSB#1 Ês: -99.8dBm/120kHz SSB#0 and SSB#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 SSB#1-SSB#0: -4 to +4 |
| 7.7.4.2 NR SA FR2 CSI-RS based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 CSI-RS#0 Ês: -108.1dBm/120kHz n257 CSI-RS#1 Ês: -105.5dBm/120kHz n260 CSI-RS#0 Ês: -108.1dBm/120kHz n260 CSI-RS#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -4.1dB 0dB 0.2dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -1.8dB CSI-RS#0: RSRP_45 to RSRP_104 CSI-RS#1: RSRP_30 to RSRP_89 CSI-RS#1-CSI-RS#0: -9 to -2 Test 2: n257 CSI-RS#0 Ês: -102.4dBm/120kHz n257 CSI-RS#1 Ês: -99.8dBm/120kHz n260 CSI-RS#0 Ês: -102.4dBm/120kHz n260 CSI-RS#1 Ês: -99.8dBm/120kHz CSI-RS#0 and CSI-RS#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 CSI-RS#1-CSI-RS#0: -4 to +4 |
| 7.7.5.1 NR SA FR2 SRS-RSRP measurement accuracy | Same as 5.7.5.1 | Same as 5.7.5.1 | Same as 5.7.5.1 |

<<End of changes>>