3GPP TSG-RAN5 Meeting #98	R5-230098

# Athens, Greece, 27th Feb 2023 – 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5430 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the table content of TC 8.7.5.1_H.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DEKRA Testing and Certification |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2023-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Incorrect content of Table 8.7.5.1_H.5.3-1, 8.7.5.1_H.5.4-1, 8.7.5.1_H.5.5-1 for TC 8.7.5.1_H.5 in 36.521-1 and aligned with RAN4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the content of Table 8.7.5.1_H.5.3-1, 8.7.5.1_H.5.4-1, 8.7.5.1_H.5.5-1. Aligned to RAN 4 updates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The content of Table 8.7.5.1_H.5.3-1, 8.7.5.1_H.5.4-1, 8.7.5.1_H.5.5-1 will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.5.1_H.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN 4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes >>

8.7.5.1_H.5	TDD FDD CA Sustained data rate performance for FDD PCell (6DL CA) for 256QAM in DL

8.7.5.1_H.5.1	Test purpose

Same test purpose as 8.7.1.1.

8.7.5.1_H.5.2	Test applicability

This test applies to Release 12 and forward of UE DL category 16 that support 256QAM in DL and E-UTRA FDD and TDD, 6DL TDD-FDD CA with FDD as PCell.

8.7.5.1_H.5.3	Minimum conformance requirements

For UE supporting 256QAM, the requirements for TDD FDD CA with FDD PCell are specified in Table 8.7.5.1_H.5.3-1 with the additional parameters specified in Table 8.7.5-1, and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category or UE DL category, and bandwidth combination with the maximum aggregated bandwidth as specified in Table 8.7.5.1_H.5.3-2. The TB success rate shall be sustained during at least 300 frames. For UE supporting 256QAM, the requirement in Table 8.7.5.1.5.3-1 is not applicable.

The applicability of the requirements are specified in Clause 8.1.2.3B. The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.7.5.1_H.5.3-1: Minimum requirement (TDD FDD CA 256QAM)

| Test number | Bandwidth (MHz) |  |  | Measurement channel |  | Reference value |
| --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC | FDD CC | TDD CC | TB success rate [%] |
| 27 | 20+5x20 | 20 | 5x20 | R.68 FDD | R.68 TDD | 85 |
| 28 | 2x20+4x20 | 2x20 | 4x20 | R.68 FDD | R.68 TDD | 85 |

Table 8.7.5.1_H.5.3-2: Test points for sustained data rate (FRC 256QAM)

| CA config | Maximum supported Bandwidth/ Bandwidth combination (MHz) |  |  | Cat. 11, 12 | DL Cat. 13 | DL Cat. 15 | DL Cat. 16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC | DL Cat. 11, 12 |  |  |  |  |  |
| CA with 6CCs | 20+5x20 | 20 | 5x20 | - | - | - | 27 |  |  |
|  | 2x20+4x20 | 2x20 | 4x20 | - | - | - | 28 |  |  |
| Note 1: If DL category is signalled by the UE under test, then select the test point according to UE DL Category. Otherwise, select the test point according to the UE category signalled. |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101[2] clause 8.7.5.1

8.7.5.1_H.5.4	Test description

Same test description as in clause 8.7.5.1.5.4 with the following exceptions:

-	Instead of Table 8.7.5.1.5.4.1-1 -> use Table 8.7.5.1_H.5.4-1.

-	Instead of Table 8.7.5.1.5.5-1 -> use Table 8.7.5.1_H.5.5-1.

-	Instead of Table 8.7.5.1.5.5-2 -> use Table 8.7.5.1_H.5.5-2.

Table 8.7.5.1_H.5.4-1: Further test parameters per test

| Test number |  | 27 | 28 |
| --- | --- | --- | --- |
| Bandwidth | FDD CC | 20 | 2x20 |
|  | TDD CC | 5x20 | 4x20 |
| DL Measurement channel | FDD CC | R.68 FDD | R.68 FDD |
|  | TDD CC | R.68 TDD | R.68 TDD |
| UL channel | FDD CC | R.1-4 FDD | R.1-4 FDD |
|  | TDD CC | NA | NA |
| TBsize per Codeword | FDD CC | 97896(Note 3) | 97896(Note 3) |
|  | TDD CC | 97896/0 (Note 4) | 97896/0 (Note 4) |
| Number of PDCP SDU per Codeword | FDD CC | 10 | 10 |
|  | TDD CC | 10/0 | 10/0 |
| PDCP SDU size [Octets] Calculation (Note 1) | FDD CC | FLOOR((TBsize - 352)/80) | FLOOR((TBsize - 352)/80) |
|  | TDD CC | FLOOR((TBsize - 352)/80) | FLOOR((TBsize - 352)/80) |
| PDCP SDU size [Octets] | FDD CC | 1219 | 1219 |
|  | TDD CC | 1219/0 | 1219/0 |
| Note 1: Transport block size under test according to applicable Fixed Reference Channel for sustained data-rate test in annex A.3.9. In case of varying TBS across SFs of the RMC, only the maximum TBS is used for PDCP SDU size calculation. In case of different resulting PDCP SDU sizes among component carriers, the smaller calculated PDCP SDU size is used across all the carriers. Note 2: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is: PDCP SDU size = (TBsize – N*PDCP header size - AMD PDU header size - MAC header size - Size of RLC STATUS PDU) / N, where PDCP header size is 16 bits for the RLC AM and 12-bit SN case; AMD PDU header size is CEIL[(16+(N-1)*12)/8] bytes which includes 16 bit standard AM header and (N-1) Length indicators; and MAC header size = R/R/E/LCID/F/L MAC subheader (24 bits for MAC SDU for RLC STATUS PDU with 15 bit LI) + R/R/E/LCID MAC subheader (8 bits for MAC SDU for RLC Data PDU) = 32 bits. The size of RLC STATUS PDU including one ACK_SN field and one NACK_SN field is 32 bits (if no STATUS PDU is sent or if the size of the STATUS PDU is less than 32 bits then padding will be used to fill the 32 bits). This gives: PDCP SDU size = 8*FLOOR((TBsize – N*16- 8*CEIL((16+(N-1)*12)/8) – 64)/(8*N)) bits. The calculation of PDCP SDU sizes does not consider timing advance MAC CE as timing advance is not transmitted by SS for RF test cases, and the header sizes are informative and may vary during the test. Note 3: 84760 bits for sub-frames 0,1,2,6,7 and 81176 bits for sub-frames 5. Note 4: 84760 bits for sub-frame 0, 4 and 81176 bits for sub-frame 5. |  |  |  |

8.7.5.1_H.5.5	Test Requirements

The requirements are specified in Table 8.7.5.1_H.5.5-1 depending on the UE category according to Table 8.7.5.1_H.5.5-2. The PDCP SDU success rate shall be sustained during at least 300 frames.

Table 8.7.5.1_H.5.5-1: Test requirements per test

| Test number | Bandwidth (MHz) |  |  | Measurement channel |  | Reference value |
| --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC | FDD CC | TDD CC | TB success rate [%] |
| 27 | 20+5x20 | 20 | 5x20 | R.68 FDD | R.68 TDD | 85 |
| 28 | 2x20+4x20 | 2x20 | 4x20 | R.68 FDD | R.68 TDD | 85 |

Table 8.7.5.1_H.5.5-2: Test applicability per UE category

| CA config | Maximum supported Bandwidth/ Bandwidth combination (MHz) |  |  | Cat. 11, 12 | DL Cat. 13 | DL Cat. 15 | DL Cat. 16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC | DL Cat. 11, 12 |  |  |  |  |  |
| CA with 6CCs | 20+5x20 | 20 | 5x20 | - | - | - | 27 |  |  |
|  | 2x20+4x20 | 2x20 | 4x20 | - | - | - | 28 |  |  |
| Note 1: Only one test point is tested. Select the largest aggregated CA bandwidth combination supported by the UE among the UE supported CA capabilities. Note 2: The test point is selected according to the UE DL Category. |  |  |  |  |  |  |  |  |  |

# << End of changes >>