3GPP TSG-RAN5 Meeting #98	R5-230158

# Athens, Greece, 27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0030 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 6.2 - File Distribution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The MO BYE in 36.579-1 Table 5.5.2.2.1-1 does not have a condition MO_CALL anymore - The MT BYE in 36.579-1 Table 5.5.2.2.2-1 does not have a condition MT_CALL anymore - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Condition MO_CALL removed from tables 6.2.9.3.3-11, 6.2.11.3.3-10 - Condition MT_CALL removed from tables 6.2.10.3.3-9, 6.2.12.3.3-8 - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.2	File Distribution

### 6.2.1	On-network / File Distribution (FD) / FD Using HTTP / One-to-one Standalone FD / Non-Mandatory Download / FILE DOWNLOAD REQUEST ACCEPTED / FILE DOWNLOAD COMPLETED / FILE DOWNLOAD REQUEST REJECTED / FILE DOWNLOAD DEFERRED / Client Originated (CO)

6.2.1.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { MCDATA User wants to send a file that is larger than <max-data-size-auto-recv-bytes> via a standalone one-to-one FD message with a non-mandatory download and with a disposition request of "FILE DOWNLOAD COMPLETED UPDATE", and, the UE (MCDATA Client) is unaware of the URL of the Media Storage Function }

then { UE (MCDATA Client) sends a SIP MESSAGE to find the URL of the Media Storage Function and responds to a SIP MESSAGE that contains the URL of the Media Storage Function with a SIP 200 (OK) message }

}

(2)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { MCDATA User wants to send a file that is larger than <max-data-size-auto-recv-bytes> via a standalone one-to-one FD message with a non-mandatory download and with a disposition request of "FILE DOWNLOAD COMPLETED UPDATE", and, the UE (MCDATA Client) is aware of the URL of the Media Storage Function }

then { UE (MCDATA Client) uploads the file to the Media Storage Function via an HTTP POST message and then sends the URL of the file location to the recipient via a SIP MESSAGE message }

}

(3)

with { MCDATA User having requesting the sending of a file that is larger than <max-data-size-auto-recv-bytes> via a standalone one-to-one FD message with a non-mandatory download and with a disposition request of "FILE DOWNLOAD COMPLETED UPDATE" and the UE (MCDATA Client) having sent the URL of the file location to the recipient }

ensure that {

when { UE (MCDATA Client) receives a FD notification via a SIP MESSAGE with disposition notification type of "FILE DOWNLOAD REQUEST REJECTED" }

then { UE (MCDATA Client) responds to the SIP MESSAGE with a SIP 200 (OK) message and delivers the notification that the remote Client has not accepted the download to the MCDATA User }

}

(4)

with { MCDATA User having requesting the sending of a file that is larger than <max-data-size-auto-recv-bytes> via a standalone one-to-one FD message with a non-mandatory download and with a disposition request of "FILE DOWNLOAD COMPLETED UPDATE" and the UE (MCDATA Client) having sent the URL of the file location to the recipient }

ensure that {

when { UE (MCDATA Client) receives a FD notification via a SIP MESSAGE with disposition notification type of "FILE DOWNLOAD DEFERRED", followed by a SIP MESSAGE with disposition notification type of "FILE DOWNLOAD REQUEST ACCEPTED", followed by SIP MESSAGE with disposition notification type of "FILE DOWNLOAD COMPLETED" }

then { UE (MCDATA Client) responds to each SIP MESSAGE with a SIP 200 (OK) message and delivers suitable notification on the respective remote Client action to the MCDATA User }

}

6.2.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 10.2.1.3.2, 10.2.2.1, 10.2.4.2.1, 12.2.1.2. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 10.2.1.3.2]

To discover the absolute URI of the media storage function, the MCData client shall generate a SIP MESSAGE request towards the participating MCData function, in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	shall follow the rules specified in subclause 6.4 for the handling of MIME bodies in a SIP message when processing the remaining steps in this subclause;

3)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with a <request-type> element containing the value "msf-disc-req";

4)	if the upload of a file is for a group standalone FD request, shall include in an application/vnd.3gpp.mcdata-info+xml MIME body, the <mcdata-calling-group-id> element set to the required MCData group identity; and

NOTE 1:	The absence of a group identity in the <mcdata-calling-group-id> element of the application/vnd.3gpp.mcdata-info+xml MIME body implies that the MCData client intends to upload a file for a one-to-one FD request. In this case, the participating MCData function identifies the MCData ID of the user from the binding between the public user identity and the MCData ID.

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

On receipt of a "SIP MESSAGE request for absolute URI discovery response", the MCData client:

1)	shall store the absolute URI found in the <mcdata-controller-psi> element;

2)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5]; and

3)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 10.2.2.1]

If the media storage client is not aware of the absolute URI of the media storage function, the media storage client shall request the MCData client to discover the absolute URI associated with the media storage function by following the procedures in subclause 10.2.1.3.

The media storage client shall send HTTP requests over a TLS connection as specified for the HTTP client in the UE in annex A of 3GPP TS 24.482 [24].

NOTE 1:	The HTTP client encodes the MCData ID in the bearer access token of the Authorization header field of an HTTP request as specified in 3GPP TS 24.482 [24].

NOTE 2:	The HTTP client always sends the HTTP requests to an HTTP proxy. Annex A of 3GPP TS 24.482 [24] indicates how the HTTP proxy forwards the HTTP request to the HTTP server.

To upload a file to media storage function, the media storage client:

1)	shall generate an HTTP POST request as specified in IETF RFC 7230 [22] and IETF RFC 7231 [23];

2)	shall set the Request-URI to the absolute URI identifying the resource on a media storage function;

3)	shall set the Host header field to a hostname identifying the media storage function;

4)	shall set the Content-Type header field to multipart/mixed and with a boundary delimiter parameter set to any chosen value;

5)	if the file upload is for one-to-one file distribution, shall insert an application/vnd.3gpp.mcdata-info+xml MIME body with:

a)	the <request-type> element set to a value of "one-to-one-fd"; and

b)	the <mcdata-calling-user-id> element set to the originating MCData ID;

6)	if the file upload is for group file distribution, shall insert an application/vnd.3gpp.mcdata-info+xml MIME body with:

a)	the <request-type> element set to a value of "group-fd";

b)	the <mcdata-request-uri> element set to the MCData group identity; and

c)	the <mcdata-calling-user-id> element set to the originating MCData ID;

7)	if end-to-end security is required for a one-to-one communication, the MCData client protects the binary data representing the file and prefixes the protected binary data with security parameters as described in 3GPP TS 33.180 [26];

8)	if

i)	end-to-end security is not required for a one-to-one communication, or

ii)	the file upload is for group file distribution;

shall include the binary data representing the file with Content-Type field set to application/octet-stream and Content-Length field set to the file size; and

9)	shall send the HTTP POST request towards the media storage function.

On receipt of a HTTP 201 Created containing a Location header field with a URL identifying the location of the resource where the file has been stored on the media storage function, then the media storage client shall store this information.

[TS 24.282, clause 10.2.4.2.1]

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	if a one-to-one standalone FD message is to be sent shall insert in the SIP MESSAGE request:

a)	an application/resource-lists+xml MIME body with the MCData ID of the target MCData user, according to rules and procedures of IETF RFC 4826 [9]; and

b)	an application/vnd.3gpp.mcdata-info+xml MIME body with a <request-type> element set to a value of "one-to-one-fd";

…

4)	shall generate a standalone FD message as specified in subclause 6.2.2.2; and

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 12.2.1.2]

Upon receipt of a:

"SIP MESSAGE request for SDS disposition notification for terminating MCData client"; or

"SIP MESSAGE request for FD disposition notification for terminating MCData client";

the MCData client:

1)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2)	shall deliver the notification to the user or application.

6.2.1.3	Test description

6.2.1.3.1	Pre-test conditions

System Simulator:

-	SS (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCData client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

-	Test File 1 for CO FD as specified in annex A.2.1 and test File 2 for CO FD as specified in aAnnex A.2.2 are available at the UE for upload.

Preamble:

-	In the <on-network> element of the MCData Service Configuration document the <max-data-size-auto-recv-bytes> element of the <tx-and-rx-control> element shall be set to 0 to indicate non-mandatory download independent from the file size.

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCData Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.2.1.3.2	Test procedure sequence

Table 6.2.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-12 | Void | - | - | - | - |
| 13 | Make the UE (MCData client) send test file 1 (TS 36.579-7 A.2.1) for CO one-to-one FD over HTTP for non-mandatory download and with disposition request "FILE DOWNLOAD COMPLETED UPDATE". (NOTE 1, NOTE 3) | - | - | - | - |
| 14 | Check: Does the UE (MCData client) correctly perform procedure 'discovery of the absolute URI of the media storage function (one-to-one communication)' as described in TS 36.579-1 [2] Table 5.3C.8.3-1 ? | - | - | 1 | P |
| 15 | Check: Does the UE (MCData client) correctly perform procedure 'FD file upload using HTTP' as described in TS 36.579-1 [2] Table 5.3C.10.3-1? | - | - | 2 | P |
| 15A | Check: Is the content of the uploaded file the same as specified in annex A.2.1? | - | - | 2 | P |
| 16-18 | Void | - | - | - | - |
| 19 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD REQUEST REJECTED" for the FD message sent at step 15? | - | - | 3 | P |
| 20 | Void | - | - | - | - |
| 21 | Check: Does the UE (MCData client) notify the user that the remote client has rejected the download? (NOTE 1) | - | - | 3 | P |
| 22 | Make the UE (MCData client) send test file 2 (TS 36.579-7 A.2.2) for CO one-to-one FD over HTTP for non-mandatory download and with disposition request "FILE DOWNLOAD COMPLETED UPDATE". (NOTE 1, NOTE 2, NOTE 3) |  |  |  |  |
| - | EXCEPTION: Step 23a1 describes behaviour that depends on UE implementation. | - | - | - | - |
| 23a1 | IF the client needs to discover again the absolute URI of the media storage function THEN the UE (MCData client) performs procedure 'Discovery of the absolute URI of the media storage function (one-to-one communication)' as described in TS 36.579-1 [2] Table 5.3C.8.3-1. | - | - | - | - |
| 23a2-23a4 | Void | - | - | - | - |
| 24 | Check: Does the UE (MCData client) correctly perform procedure 'FD file upload using HTTP' as described in TS 36.579-1 [2] Table 5.3C.10.3-1? | - | - | 2 | P |
| 24A | Check: Is the content of the uploaded file the same as specified in annex A.2.2? | - | - | 2 | P |
| 25-27 | Void | - | - | - | - |
| 28 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD DEFERRED" for the FD message sent at step 24? | - | - | 4 | P |
| 29 | Void | - | - | - | - |
| 30 | Check: Does the UE (MCData client) notify the user that the remote client has deferred the acceptance of the download? (NOTE 1) | - | - | 4 | P |
| 31 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD REQUEST ACCEPTED" for the FD message sent at step 24? | - | - | 4 | P |
| 32 | Void | - | - | - | - |
| 33 | Check: Does the UE (MCData client) notify the user that the remote client has accepted the download? (NOTE 1) | - | - | 4 | P |
| 34 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD COMPLETED" for the FD message sent at step 24? | - | - | 4 | P |
| 35 | Void | - | - | - | - |
| 36 | Check: Does the UE (MCData client) notify the user that the remote client has completed the download? (NOTE 1) | - | - | 4 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: To avoid unpredicted behaviour at the UE side due to the file upload in the previous steps, the UE shall attempt to upload a different file here. NOTE 3: Test file 1 and 2 for CO FD as specified in annex A.2.1 and A.2.2. |  |  |  |  |  |

6.2.1.3.3	Specific message contents

Table 6.2.1.3.3-1..6: Void

Table 6.2.1.3.3-7: HTTP POST from the UE (steps 15, 24, Table 6.2.1.3.2-1; 
step 2, TS 36.579-1 [2] Table 5.3C.10.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1, condition FD_HTTP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.1.3.3-8 |  |  |  |

Table 6.2.1.3.3-8: MCData-Info (Table 6.2.1.3.3-7)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "one-to-one-fd" |  |  |  |
| mcdata-calling-user-id | px_MCData_ID_User_A | NOTE: the element is not encrypted |  |  |

Table 6.2.1.3.3-9..10: Void

Table 6.2.1.3.3-11: HTTP 201 Created from the SS (step 15, Table 6.2.1.3.2-1; 
step 3, TS 36.579-1 [2] Table 5.3C.10.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.7-1, condition FD_HTTP

Table 6.2.1.3.3-11A: HTTP 201 Created from the SS (step 24, Table 6.2.1.3.2-1; 
step 3, TS 36.579-1 [2] Table 5.3C.10.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.7-1, condition FD_HTTP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Location |  |  |  |  |
| uri | tsc_MCData_MSF_URI & "/file-location-2" |  |  |  |

Table 6.2.1.3.3-12: SIP MESSAGE from the UE (steps 15, 24, Table 6.2.1.3.2-1; 
step 4, TS 36.579-1 [2] Table 5.3C.10.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.1.3.3-13 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD SIGNALLING PAYLOAD as described in Table 6.2.1.3.3-13A |  |  |  |

Table 6.2.1.3.3-13: MCData-Info (Table 6.2.1.3.3-12)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "one-to-one-fd" |  |  |  |

Table 6.2.1.3.3-13A: FD SIGNALLING PAYLOAD (Table 6.2.1.3.3-12)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.5-1, condition FD_HTTP

Table 6.2.1.3.3-14: SIP MESSAGE from the SS (step 19, Table 6.2.1.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.1.3.3-16 |  |  |  |

Table 6.2.1.3.3-15: Void

Table 6.2.1.3.3-16: FD NOTIFICATION (Table 6.2.1.3.3-14)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.8-1, condition FD_REJECTED

Table 6.2.1.3.3-17: SIP MESSAGE from the SS (step 28, Table 6.2.1.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.1.3.3-18 |  |  |  |

Table 6.2.1.3.3-18: FD NOTIFICATION (Table 6.2.1.3.3-17)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.8-1, condition FD_DEFERRED

Table 6.2.1.3.3-19: SIP MESSAGE from the SS (step 31, Table 6.2.1.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.1.3.3-20 |  |  |  |

Table 6.2.1.3.3-20: FD NOTIFICATION (Table 6.2.1.3.3-19)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.8-1, condition FD_ACCEPTED

Table 6.2.1.3.3-21: SIP MESSAGE from the SS (step 34, Table 6.2.1.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.1.3.3-22 |  |  |  |

Table 6.2.1.3.3-22: FD NOTIFICATION (Table 6.2.1.3.3-21)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.8-1, condition FD_COMPLETED

### 6.2.2	On-network / File Distribution (FD) / FD Using HTTP / One-to-one Standalone FD / Non-Mandatory Download / Before TDU2 Timers Expires / FILE DOWNLOAD REQUEST ACCEPTED / FILE DOWNLOAD COMPLETED / FILE DOWNLOAD REQUEST REJECTED / FILE DOWNLOAD DEFERRED / FD Client Terminated (CT)

6.2.2.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { UE (MCDATA Client) receives a SIP MESSAGE message for a standalone one-to-one FD message with a non-mandatory download and with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" }

then { UE (MCDATA Client) responds with a SIP 200 (OK) message and notifies the MCDATA User about the incoming FD request }

}

(2)

with { UE (MCDATA Client) having received a standalone one-to-one FD message with a non-mandatory download and with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" }

ensure that {

when { the MCDATA User requests to accept the FD request }

then { UE (MCDATA Client) generates an FD NOTIFICATION indicating acceptance of the FD request and attempts to download the file with an HTTP GET message }

}

(3)

with { MCDATA User having accepted an FD request with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" }

ensure that {

when { UE (MCDATA Client) has successfully downloaded the file }

then { UE (MCDATA Client) notifies the MCDATA User that the file has successfully downloaded and generates an FD NOTIFICATION indicating the successful download of the file }

}

(4)

with { UE (MCDATA Client) having received a standalone one-to-one FD message with a non-mandatory download and with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" }

ensure that {

when { the MCDATA User requests to reject the FD request }

then { UE (MCDATA Client) generates an FD NOTIFICATION indicating the rejection of the FD request }

}

(5)

with { UE (MCDATA Client) having received a standalone one-to-one FD message with a non-mandatory download and with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" }

ensure that {

when { the MCDATA User requests to defer the FD request before the expiration of the TDU2 (FD non-mandatory download timer) timer }

then { UE (MCDATA Client) generates an FD NOTIFICATION indicating the deferral of the FD request }

}

6.2.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 10.2.4.2.2, 10.2.1.2.3, 12.2.1.1, 10.2.3.1. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 10.2.4.2.2]

Upon receipt of a "SIP MESSAGE request for FD using HTTP for terminating MCData client", the MCData client:

1)	may reject the SIP MESSAGE request if there are not enough resources to handle the SIP MESSAGE request;

2)	if the SIP MESSAGE request is rejected in step 1), shall respond towards the participating MCData function with a SIP 480 (Temporarily unavailable) response and skip the rest of the steps of this subclause;

3)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5];

4)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5]; and

5)	shall handle the received message as specified in subclause 10.2.1.2.

[TS 24.282, clause 10.2.1.2.3]

The MCData client:

1)	if the FD SIGNALLING PAYLOAD message does not contain an Application ID IE:

a)	shall determine that the payload contained in the Payload IE in the FD SIGNALLING PAYLOAD message is for user consumption;

b)	shall notify the user about the incoming FD request; and

c)	if the FD SIGNALLING PAYLOAD message contains a Metadata IE, shall deliver the contents of the Metadata IE to the user;

2)	if the FD SIGNALLING PAYLOAD message contains an Application ID IE:

a)	shall determine that the payload contained in the Payload IE in the FD SIGNALLING PAYLOAD message is not for user consumption;

b)	if the Application ID value is unknown, shall discard the FD message and exit this subclause;

c)	if the Application ID value is known, shall notify the application of the incoming FD request; and

NOTE 1:	If FD request is addressed to a non-MCData application that is not running, the MCData client starts the local non-MCData application.

d)	if the FD SIGNALLING PAYLOAD message contains a Metadata IE, shall deliver the contents of the Metadata IE to the application;

3)	shall start a timer TDU2 (FD non-mandatory download timer) with the timer value as specified in subclause F.2.3;

4)	shall wait for the user or application to request to download the file indicated by file URL in the Payload data in the Payload IE in the FD SIGNALLING PAYLOAD message;

5)	if the user or application accepts or rejects or decides to defer the FD request, shall stop timer TDU2 (FD non-mandatory download timer);

6)	if the user deferred the FD request while the timer TDU2 (FD non-mandatory download timer) was running, shall generate an FD NOTIFICATION indicating deferral of the FD request as specified in subclause 12.2.1.1;

NOTE 2:	Once the timer TDU2 (FD non-mandatory download timer) has expired the FD request can only be accepted or rejected with an appropriate action by the MCData client.

NOTE 3:	Once the timer TDU2 (FD non-mandatory download timer) has expired, no action is taken by the MCData client if the FD request is deferred.

7)	if the user or application rejects the FD request, shall generate an FD NOTIFICATION indicating rejection of the FD request as specified in subclause 12.2.1.1 and shall exit this subclause; and

8)	if the user accepts the FD request:

a)	shall generate an FD NOTIFICATION indicating acceptance of the FD request as specified in subclause 12.2.1.1;

b)	if the FD SIGNALLING PAYLOAD message contains a new Conversation ID, shall instantiate a new conversation with the Message ID in the FD SIGNALLING PAYLOAD identifying the first message in the conversation thread;

c)	if the FD SIGNALLING PAYLOAD message contains an existing Conversation ID and:

i)	if the FD SIGNALLING PAYLOAD message does not contain an InReplyTo message ID, shall use the Message ID in the FD SIGNALLING PAYLOAD to identify a new message in the existing conversation thread; and

ii)	if the FD SIGNALLING PAYLOAD message contains an InReplyTo message ID, shall associate the message to an existing message in the conversation thread as identified by the InReplyTo message ID in the FD SIGNALLING PAYLOAD, and use the Message ID in the FD SIGNALLING PAYLOAD to identify the new message;

d)	may store the Conversation ID, Message ID, InReplyTo message ID and Date and time in local storage;

e)	shall attempt to download the file as identified by the file URL in the Payload IE in the FD SIGNALLING PAYLOAD message, as specified in subclause 10.2.3.1; and

f)	if the received FD SIGNALLING PAYLOAD message contains an FD disposition request type IE requesting a file download completed update, then after the file download has been successfully downloaded, shall generate an FD NOTIFICATION by following the procedures in subclause 12.2.1.1.

[TS 24.282, clause 12.2.1.1]

The MCData client shall follow the procedures in this subclause to:

-	indicate to an MCData client that an SDS message was delivered, read or delivered and read when the originating client requested a delivery, read or delivery and read report;

-	indicate to the participating MCData function serving the MCData user that an SDS message was undelivered. The participating MCData function can store the message for later re-delivery;

-	indicate to an MCData client that a request for FD was accepted, deferred or rejected; or

-	indicate to an MCData client that a file download has been completed;

Before sending a disposition notification the MCData client needs to determine:

-	the group identity related to an SDS or FD message request received as part of a group communication. The MCData client determines the group identity from the contents of the <mcdata-calling-group-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request; and

-	the MCData user targeted for the disposition notification. The MCData client determines the targeted MCData user from the contents of the <mcdata-calling-user-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request.

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	shall follow the rules specified in subclause 6.4 for the handling of MIME bodies in a SIP message when processing the remaining steps in this subclause;

3)	shall insert in the SIP MESSAGE request an application/resource-lists+xml MIME body containing the MCData ID of the targeted MCData user, according to rules and procedures of IETF RFC 5366 [18];

4)	void;

5)	if sending a disposition notification in response to an MCData group data request, shall include an <mcdata-calling-group-id> element set to the MCData group identity in the application/vnd.3gpp.mcdata-info+xml MIME body;

6)	if requiring to send an SDS notification, shall generate an SDS NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.1;

7)	if requiring to send an FD notification, shall generate an FD NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.2; and

8)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 10.2.3.1]

The media storage client on the MCData client shall send HTTP requests over a TLS connection as specified for the HTTP client in the UE, in annex A of 3GPP TS 24.482 [24].

NOTE 1:	The HTTP client encodes the MCData ID in the bearer access token of the Authorization header field of an HTTP request as specified in 3GPP TS 24.482 [24].

NOTE 2:	The HTTP client always sends the HTTP requests to an HTTP proxy. Annex A of 3GPP TS 24.482 [24] indicates how the HTTP proxy forwards the HTTP request to the HTTP server.

To download a file from the media storage function on the controlling MCData function, the media storage client on the MCData client:

1)	shall generate an HTTP GET request as specified in IETF RFC 7230 [22] and IETF RFC 7231 [23] with a Request-URI set to an absolute URI identifying the URL of the file being requested from the media storage function on the controlling MCData function; and

2)	shall send the HTTP GET request towards the media storage function on the controlling MCData function.

On receipt of a HTTP 200 OK response containing the requested file, the MCData client shall notify the user or application that the file has been successfully downloaded.

6.2.2.3	Test description

6.2.2.3.1	Pre-test conditions

System Simulator:

-	SS (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCData client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

-	Test files downloaded or received at previous test runs are deleted.

Preamble:

-	In the <on-network> element of the MCData Service Configuration document the <max-data-size-auto-recv-bytes> element of the <tx-and-rx-control> element shall be set to 0 to indicate non-mandatory download independent from the file size.

-	TDU2 (FD non-mandatory download timer) is set to the default value of 60 seconds.

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCData Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.2.2.3.2	Test procedure sequence

Table 6.2.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD message for one-to-one file distribution with disposition request "FILE DOWNLOAD COMPLETED UPDATE"? (NOTE 2) | - | - | 1 | P |
| 2 | Void | - | - | - | - |
| 3 | Check: Does the UE (MCData client) notify the user of the incoming FD request? (NOTE 1) | - | - | 1 | P |
| 4 | Make the UE (MCData client) accept the FD request and download the file before timer TDU2 expires. (NOTE 1) | - | - | - | - |
| 5 | Check: Does the UE (MCData client) correctly perform procedure 'FD file accept and download using HTTP' as described in TS 36.579-1 [2] Table 5.3C.11.3-1 to download test file 1? (NOTE 3) | - | - | 2,3 | P |
| 6-8 | Void | - | - | - | - |
| 9 | Check: Does the UE (MCData client) notify the user of the file download? (NOTE 1) | - | - | 3 | P |
| 9A | Check: Has the UE (MCData client) downloaded test file 1 (TS 36.579-7 A.3.1)? (NOTE 1) | - | - | 2 | P |
| 10-11 | Void | - | - | - | - |
| 12 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD message for one-to-one file distribution with disposition request "FILE DOWNLOAD COMPLETED UPDATE"? (NOTE 2) | - | - | 1 | P |
| 13 | Void | - | - | - | - |
| 14 | Check: Does the UE (MCData client) notify the user of the incoming FD request? (NOTE 1) | - | - | 1 | P |
| 15 | Make the UE (MCData client) reject the FD request before timer TDU2 expires. (NOTE 1) | - | - | - | - |
| 16 | Check: Does the UE (MCData client) correctly perform procedure 'CO SDS or FD message transfer using signalling plane' as described in TS 36.579-1 [2] Table 5.3C.1.3-1 to send an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD REQUEST REJECTED"? | - | - | 4 | P |
| 17 | Void | - | - | - | - |
| 18 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD message for one-to-one file distribution with disposition request "FILE DOWNLOAD COMPLETED UPDATE"? (NOTE 2) | - | - | 1 | P |
| 19 | Void | - | - | - | - |
| 20 | Check: Does the UE (MCData client) notify the user of the incoming FD request? (NOTE 1) | - | - | 1 | P |
| 21 | Make the UE (MCData client) defer the FD request before timer TDU2 expires. (NOTE 1) | - | - | - | - |
| 22 | Check: Does the UE (MCData client) correctly perform procedure 'CO SDS or FD message transfer using signalling plane' as described in TS 36.579-1 [2] Table 5.3C.1.3-1 to send an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD DEFERRED"? | - | - | 5 | P |
| 23 | Void | - | - | - | - |
| 24 | Make the UE (MCData client) accept the deferred FD request and download the file. (NOTE 1) | - | - | - | - |
| 25 | Check: Does the UE (MCData client) correctly perform procedure 'FD file accept and download using HTTP' as described in TS 36.579-1 [2] Table 5.3C.11.3-1 to download test file 2? (NOTE 3) | - | - | 2,3 | P |
| 26-28 | Void | - | - | - | - |
| 29 | Check: Does the UE (MCData client) notify the user of the file download? (NOTE 1) | - | - | 3 | P |
| 29A | Check: Has the UE (MCData client) downloaded test file 2 (TS 36.579-7 A.3.3)? (NOTE 1) | - | - | 2 | P |
| 30-31 | Void | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Timer TDU2 (FD non-mandatory download timer) starts on reception of the FD request via the SIP MESSAGE request. NOTE 3: Test file 1 and 2 for CT FD as specified in annex A.3.1 and A.3.2. |  |  |  |  |  |

6.2.2.3.3	Specific message contents

Table 6.2.2.3.3-1: SIP MESSAGE from the SS (steps 1, 12, 18, Table 6.2.2.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.2.3.3-2 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD SIGNALLING PAYLOAD as described in Table 6.2.2.3.3-2A |  |  |  |

Table 6.2.2.3.3-2: MCData-Info (Table 6.2.2.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3, condition MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "one-to-one-fd" |  |  |  |

Table 6.2.2.3.3-2A: FD SIGNALLING PAYLOAD (Table 6.2.2.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.6-1, condition FD_HTTP

Table 6.2.2.3.3-3: Void

Table 6.2.2.3.3-4: SIP MESSAGE from the UE (steps 5, 25, Table 6.2.2.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.11.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part | not present | MCData-Info |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.2.3.3-5 |  |  |  |

Table 6.2.2.3.3-5: FD NOTIFICATION (Table 6.2.2.3.3-4)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.7-1, condition FD_ACCEPTED

Table 6.2.2.3.3-6: HTTP GET from the UE (steps 5, 25, Table 6.2.2.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3C.11.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.2-1, condition FD_HTTP

Table 6.2.2.3.3-7: HTTP 200 (OK) from the SS (steps 5, 25, Table 6.2.2.3.2-1;
step 5, TS 36.579-1 [2] Table 5.3C.11.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.6-1, condition FD_HTTP

Table 6.2.2.3.3-8..9: Void

Table 6.2.2.3.3-10: SIP MESSAGE from the UE (steps 5, 25, Table 6.2.2.3.2-1;
step 6a1, TS 36.579-1 [2] Table 5.3C.11.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part | not present | MCData-Info |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.2.3.3-11 |  |  |  |

Table 6.2.2.3.3-11: FD NOTIFICATION (Table 6.2.2.3.3-10)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.7-1, condition FD_COMPLETED

Table 6.2.2.3.3-12: SIP MESSAGE from the UE (step 16, Table 6.2.2.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part | not present | MCData-Info |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.2.3.3-13 |  |  |  |

Table 6.2.2.3.3-13: FD NOTIFICATION (Table 6.2.2.3.3-12)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.7-1, condition FD_REJECTED

Table 6.2.2.3.3-14: SIP MESSAGE from the UE (step 22, Table 6.2.2.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part | not present | MCData-Info |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.2.3.3-15 |  |  |  |

Table 6.2.2.3.3-15: FD NOTIFICATION (Table 6.2.2.3.3-14)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.7-1, condition FD_DEFERRED

### 6.2.3	On-network / File Distribution (FD) / FD Using HTTP / Group Standalone FD / Non-Mandatory Download / FILE DOWNLOAD REQUEST ACCEPTED / FILE DOWNLOAD COMPLETED / FILE DOWNLOAD REQUEST REJECTED / Client Originated (CO)

6.2.3.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a Group Standalone FD message with a non-mandatory download and with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" and the UE (MCDATA Client) is unaware of the URL of the Media Storage Function }

then { UE (MCDATA Client) sends a SIP MESSAGE to find the URL of the Media Storage Function and responds to a SIP MESSAGE that contains the URL of the Media Storage Function with a SIP 200 (OK) message }

}

(2)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a Group Standalone FD message with a non-mandatory download and with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" and the UE (MCDATA Client) is aware of the URL of the Media Storage Function }

then { UE (MCDATA Client) uploads the file to the Media Storage Function via an HTTP POST message and then sends the URL of the file location to the recipient via a SIP MESSAGE message }

}

(3)

with { UE (MCDATA Client) having sent the URL of the file location to the recipient }

ensure that {

when { UE (MCDATA Client) receives a FD notification via a SIP MESSAGE message }

then { UE (MCDATA Client) responds to the SIP MESSAGE message with a SIP 200 (OK) message and delivers the notification to the MCDATA User }

}

6.2.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 10.2.1.3.2, 10.2.2.1, 10.2.4.2.1, 12.2.1.2. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 10.2.1.3.2]

To discover the absolute URI of the media storage function, the MCData client shall generate a SIP MESSAGE request towards the participating MCData function, in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	shall follow the rules specified in subclause 6.4 for the handling of MIME bodies in a SIP message when processing the remaining steps in this subclause;

3)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with a <request-type> element containing the value "msf-disc-req";

4)	if the upload of a file is for a group standalone FD request, shall include in an application/vnd.3gpp.mcdata-info+xml MIME body, the <mcdata-calling-group-id> element set to the required MCData group identity; and

NOTE 1:	The absence of a group identity in the <mcdata-calling-group-id> element of the application/vnd.3gpp.mcdata-info+xml MIME body implies that the MCData client intends to upload a file for a one-to-one FD request. In this case, the participating MCData function identifies the MCData ID of the user from the binding between the public user identity and the MCData ID.

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

On receipt of a "SIP MESSAGE request for absolute URI discovery response", the MCData client:

1)	shall store the absolute URI found in the <mcdata-controller-psi> element;

2)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5]; and

3)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 10.2.2.1]

If the media storage client is not aware of the absolute URI of the media storage function, the media storage client shall request the MCData client to discover the absolute URI associated with the media storage function by following the procedures in subclause 10.2.1.3.

The media storage client shall send HTTP requests over a TLS connection as specified for the HTTP client in the UE in annex A of 3GPP TS 24.482 [24].

NOTE 1:	The HTTP client encodes the MCData ID in the bearer access token of the Authorization header field of an HTTP request as specified in 3GPP TS 24.482 [24].

NOTE 2:	The HTTP client always sends the HTTP requests to an HTTP proxy. Annex A of 3GPP TS 24.482 [24] indicates how the HTTP proxy forwards the HTTP request to the HTTP server.

To upload a file to media storage function, the media storage client:

1)	shall generate an HTTP POST request as specified in IETF RFC 7230 [22] and IETF RFC 7231 [23];

2)	shall set the Request-URI to the absolute URI identifying the resource on a media storage function;

3)	shall set the Host header field to a hostname identifying the media storage function;

4)	shall set the Content-Type header field to multipart/mixed and with a boundary delimiter parameter set to any chosen value;

5)	if the file upload is for one-to-one file distribution, shall insert an application/vnd.3gpp.mcdata-info+xml MIME body with:

a)	the <request-type> element set to a value of "one-to-one-fd"; and

b)	the <mcdata-calling-user-id> element set to the originating MCData ID;

6)	if the file upload is for group file distribution, shall insert an application/vnd.3gpp.mcdata-info+xml MIME body with:

a)	the <request-type> element set to a value of "group-fd";

b)	the <mcdata-request-uri> element set to the MCData group identity; and

c)	the <mcdata-calling-user-id> element set to the originating MCData ID;

7)	if end-to-end security is required for a one-to-one communication, the MCData client protects the binary data representing the file and prefixes the protected binary data with security parameters as described in 3GPP TS 33.180 [26];

8)	if

i)	end-to-end security is not required for a one-to-one communication, or

ii)	the file upload is for group file distribution;

shall include the binary data representing the file with Content-Type field set to application/octet-stream and Content-Length field set to the file size; and

9)	shall send the HTTP POST request towards the media storage function.

On receipt of a HTTP 201 Created containing a Location header field with a URL identifying the location of the resource where the file has been stored on the media storage function, then the media storage client shall store this information.

[TS 24.282, clause 10.2.4.2.1]

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	if a one-to-one standalone FD message is to be sent shall insert in the SIP MESSAGE request:

a)	an application/resource-lists+xml MIME body with the MCData ID of the target MCData user, according to rules and procedures of IETF RFC 4826 [9]; and

b)	an application/vnd.3gpp.mcdata-info+xml MIME body with a <request-type> element set to a value of "one-to-one-fd";

…

4)	shall generate a standalone FD message as specified in subclause 6.2.2.2; and

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 12.2.1.2]

Upon receipt of a:

"SIP MESSAGE request for SDS disposition notification for terminating MCData client"; or

"SIP MESSAGE request for FD disposition notification for terminating MCData client";

the MCData client:

1)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2)	shall deliver the notification to the user or application.

6.2.3.3	Test description

6.2.3.3.1	Pre-test conditions

System Simulator:

-	SS (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCData client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

-	Test File 1 for CO FD as specified in annex A.2.1 and test File 2 for CO FD as specified in aAnnex A.2.2 are available at the UE for upload.

Preamble:

-	In the MCData Group Configuration document the <mcdata-on-network-max-data-size-auto-recv> shall be set to 0 to indicate non-mandatory download independent from the file size.

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCData Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.2.3.3.2	Test procedure sequence

Table 6.2.3.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) send test file 1 (TS 36.579-7 A.2.1) for CO group FD over HTTP for non-mandatory download and with disposition request "FILE DOWNLOAD COMPLETED UPDATE". (NOTE 1, NOTE 2) | - | - | - | - |
| 2 | Check: Does the UE (MCData client) correctly perform procedure 'Discovery of the absolute URI of the media storage function (one-to-one communication)' as described in TS 36.579-1 [2] Table 5.3C.9.3-1 ? | - | - | 1 | P |
| 3 | Check: Does the UE (MCData client) correctly perform procedure 'FD file upload using HTTP' as described in TS 36.579-1 [2] Table 5.3C.10.3-1? | - | - | 2 | P |
| 3A | Check: Is the content of the uploaded file the same as specified in annex A.2.1? | - | - | 2 | P |
| 4-6 | Void | - | - | - | - |
| 7 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD REQUEST ACCEPTED" for the FD message sent at step 3? | - | - | 3 | P |
| 8 | Void | - | - | - | - |
| 9 | Check: Does the UE (MCData client) notify the user that the remote client has accepted the download? (NOTE 1) | - | - | 3 | P |
| 10 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD COMPLETED" for the FD message sent at step 3? | - | - | 3 | P |
| 11 | Void | - | - | - | - |
| 12 | Check: Does the UE (MCData client) notify the user that the remote client has completed the download? (NOTE 1) | - | - | 3 | P |
| 13 | Make the UE (MCData client) send test file 2 (TS 36.579-7 A.2.2) for CO group FD over HTTP for non-mandatory download and with disposition request "FILE DOWNLOAD COMPLETED UPDATE". (NOTE 1, NOTE 2) | - | - | - | - |
| - | EXCEPTION: Step 14a1 describes behaviour that depends on UE implementation. | - | - | - | - |
| 14a1 | IF the client needs to discover again the absolute URI of the media storage function THEN the UE (MCData client) performs procedure 'Discovery of the absolute URI of the media storage function (one-to-one communication)' as described in TS 36.579-1 [2] Table 5.3C.9.3-1. | - | - | - | - |
| 14a2-14a4 | Void | - | - | - | - |
| 15 | Check: Does the UE (MCData client) correctly perform procedure 'FD file upload using HTTP' as described in TS 36.579-1 [2] Table 5.3C.10.3-1? | - | - | 2 | P |
| 15A | Check: Is the content of the uploaded file the same as specified in annex A.2.2? | - | - | 2 | P |
| 16-18 | Void | - | - | - | - |
| 19 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD REQUEST REJECTED" for the FD message sent at step 15? | - | - | 3 | P |
| 20 | Void | - | - | - | - |
| 21 | Check: Does the UE (MCData client) notify the user that the remote client has rejected the download? (NOTE 1) | - | - | 3 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Test file 1 and 2 for CO FD as specified in annex A.2.1 and A.2.2. |  |  |  |  |  |

6.2.3.3.3	Specific message contents

Table 6.2.1.3.3-1..6: Void

Table 6.2.3.3.3-7: HTTP POST from the UE (steps 3, 15, Table 6.2.3.3.2-1; 
step 2, TS 36.579-1 [2] Table 5.3C.10.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1, condition FD_HTTP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.3.3.3-8 |  |  |  |

Table 6.2.3.3.3-8: MCData-Info (Table 6.2.3.3.3-7)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "group-fd" |  |  |  |
| mcdata-request-uri | px_MCData_Group_A_ID | NOTE: the element is not encrypted |  |  |
| mcdata-calling-user-id | px_MCData_ID_User_A | NOTE: the element is not encrypted |  |  |

Table 6.2.3.3.3-9: Void

Table 6.2.3.3.3-10: HTTP 201 Created from the SS (step 3, Table 6.2.3.3.2-1; 
step 3, TS 36.579-1 [2] Table 5.3C.10.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.7-1, condition FD_HTTP

Table 6.2.3.3.3-10A: HTTP 201 Created from the SS (step 15, Table 6.2.3.3.2-1; 
step 3, TS 36.579-1 [2] Table 5.3C.10.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.7-1, condition FD_HTTP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Location |  |  |  |  |
| absoluteURI | tsc_MCData_MSF_URI & "/file-location-2" |  |  |  |

Table 6.2.3.3.3-11: SIP MESSAGE from the UE (steps 3, 15, Table 6.2.3.3.2-1; 
step 4, TS 36.579-1 [2] Table 5.3C.10.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.3.3.3-12 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD SIGNALLING PAYLOAD as described in Table 6.2.3.3.3-12A |  |  |  |

Table 6.2.3.3.3-12: MCData-Info (Table 6.2.3.3.3-11)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "group-fd" |  |  |  |

Table 6.2.3.3.3-12A: FD SIGNALLING PAYLOAD (Table 6.2.3.3.3-11)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.5-1, condition FD_HTTP

Table 6.2.3.3.3-13: SIP MESSAGE from the SS (step 7, Table 6.2.3.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.1.3.3-14 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.3.3.3-15 |  |  |  |

Table 6.2.3.3.3-14: MCData-Info (Table 6.2.3.3.3-13)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-group-id | Encrypted <mcdata-calling-group-id> with mcdataURI set to px_MCData_Group_A_ID | Encrypted according to TS 36.579-1 [2] Table 5.5.3.2.2-3A |  |  |

Table 6.2.3.3.3-15: FD NOTIFICATION (Table 6.2.3.3.3-13)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.8-1, condition FD_ACCEPTED

Table 6.2.3.3.3-16: SIP MESSAGE from the SS (step 10, Table 6.2.3.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.1.3.3-14 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.3.3.3-17 |  |  |  |

Table 6.2.3.3.3-17: FD NOTIFICATION (Table 6.2.3.3.3-16)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.8-1, condition FD_COMPLETED

Table 6.2.3.3.3-18: SIP MESSAGE from the SS (step 19, Table 6.2.3.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.1.3.3-14 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.3.3.3-19 |  |  |  |

Table 6.2.3.3.3-19: FD NOTIFICATION (Table 6.2.3.3.3-18)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.8-1, condition FD_REJECTED

### 6.2.4	On-network / File Distribution (FD) / FD Using HTTP / Group Standalone FD / Non-Mandatory Download / After TDU2 Timers Expires / FILE DOWNLOAD REQUEST ACCEPTED / FILE DOWNLOAD COMPLETED / FILE DOWNLOAD REQUEST REJECTED / Client Terminated (CT)

6.2.4.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { UE (MCDATA Client) receives a SIP MESSAGE message for a group standalone FD message with a non-mandatory download and with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" }

then { UE (MCDATA Client) responds with a SIP 200 (OK) message and notifies the MCDATA User about the incoming FD request }

}

(2)

with { UE (MCDATA Client) having received a group standalone FD message with a non-mandatory download and with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" }

ensure that {

when { the MCDATA User requests to accept the FD request }

then { UE (MCDATA Client) generates an FD NOTIFICATION indicating acceptance of the FD request and attempts to download the file with an HTTP GET message }

}

(3)

with { MCDATA User having accepted an FD request with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" }

ensure that {

when { UE (MCDATA Client) has successfully downloaded the file }

then { UE (MCDATA Client) notifies the MCDATA User that the file has successfully downloaded and generates an FD NOTIFICATION indicating the successful download of the file }

}

(4)

with { UE (MCDATA Client) having received a group standalone FD message with a non-mandatory download and with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" }

ensure that {

when { the MCDATA User requests to reject the FD request }

then { UE (MCDATA Client) generates an FD NOTIFICATION indicating the rejection of the FD request }

}

(5)

Void

6.2.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 10.2.4.2.2, 10.2.1.2.3, 12.2.1.1, 10.2.3.1. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 10.2.4.2.2]

Upon receipt of a "SIP MESSAGE request for FD using HTTP for terminating MCData client", the MCData client:

1)	may reject the SIP MESSAGE request if there are not enough resources to handle the SIP MESSAGE request;

2)	if the SIP MESSAGE request is rejected in step 1), shall respond towards the participating MCData function with a SIP 480 (Temporarily unavailable) response and skip the rest of the steps of this subclause;

3)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5];

4)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5]; and

5)	shall handle the received message as specified in subclause 10.2.1.2.

[TS 24.282, clause 10.2.1.2.3]

The MCData client:

1)	if the FD SIGNALLING PAYLOAD message does not contain an Application ID IE:

a)	shall determine that the payload contained in the Payload IE in the FD SIGNALLING PAYLOAD message is for user consumption;

b)	shall notify the user about the incoming FD request; and

c)	if the FD SIGNALLING PAYLOAD message contains a Metadata IE, shall deliver the contents of the Metadata IE to the user;

2)	if the FD SIGNALLING PAYLOAD message contains an Application ID IE:

a)	shall determine that the payload contained in the Payload IE in the FD SIGNALLING PAYLOAD message is not for user consumption;

b)	if the Application ID value is unknown, shall discard the FD message and exit this subclause;

c)	if the Application ID value is known, shall notify the application of the incoming FD request; and

NOTE 1:	If FD request is addressed to a non-MCData application that is not running, the MCData client starts the local non-MCData application.

d)	if the FD SIGNALLING PAYLOAD message contains a Metadata IE, shall deliver the contents of the Metadata IE to the application;

3)	shall start a timer TDU2 (FD non-mandatory download timer) with the timer value as specified in subclause F.2.3;

4)	shall wait for the user or application to request to download the file indicated by file URL in the Payload data in the Payload IE in the FD SIGNALLING PAYLOAD message;

5)	if the user or application accepts or rejects or decides to defer the FD request, shall stop timer TDU2 (FD non-mandatory download timer);

6)	if the user deferred the FD request while the timer TDU2 (FD non-mandatory download timer) was running, shall generate an FD NOTIFICATION indicating deferral of the FD request as specified in subclause 12.2.1.1;

NOTE 2:	Once the timer TDU2 (FD non-mandatory download timer) has expired the FD request can only be accepted or rejected with an appropriate action by the MCData client.

NOTE 3:	Once the timer TDU2 (FD non-mandatory download timer) has expired, no action is taken by the MCData client if the FD request is deferred.

7)	if the user or application rejects the FD request, shall generate an FD NOTIFICATION indicating rejection of the FD request as specified in subclause 12.2.1.1 and shall exit this subclause; and

8)	if the user accepts the FD request:

a)	shall generate an FD NOTIFICATION indicating acceptance of the FD request as specified in subclause 12.2.1.1;

b)	if the FD SIGNALLING PAYLOAD message contains a new Conversation ID, shall instantiate a new conversation with the Message ID in the FD SIGNALLING PAYLOAD identifying the first message in the conversation thread;

c)	if the FD SIGNALLING PAYLOAD message contains an existing Conversation ID and:

i)	if the FD SIGNALLING PAYLOAD message does not contain an InReplyTo message ID, shall use the Message ID in the FD SIGNALLING PAYLOAD to identify a new message in the existing conversation thread; and

ii)	if the FD SIGNALLING PAYLOAD message contains an InReplyTo message ID, shall associate the message to an existing message in the conversation thread as identified by the InReplyTo message ID in the FD SIGNALLING PAYLOAD, and use the Message ID in the FD SIGNALLING PAYLOAD to identify the new message;

d)	may store the Conversation ID, Message ID, InReplyTo message ID and Date and time in local storage;

e)	shall attempt to download the file as identified by the file URL in the Payload IE in the FD SIGNALLING PAYLOAD message, as specified in subclause 10.2.3.1; and

f)	if the received FD SIGNALLING PAYLOAD message contains an FD disposition request type IE requesting a file download completed update, then after the file download has been successfully downloaded, shall generate an FD NOTIFICATION by following the procedures in subclause 12.2.1.1.

[TS 24.282, clause 12.2.1.1]

The MCData client shall follow the procedures in this subclause to:

-	indicate to an MCData client that an SDS message was delivered, read or delivered and read when the originating client requested a delivery, read or delivery and read report;

-	indicate to the participating MCData function serving the MCData user that an SDS message was undelivered. The participating MCData function can store the message for later re-delivery;

-	indicate to an MCData client that a request for FD was accepted, deferred or rejected; or

-	indicate to an MCData client that a file download has been completed;

Before sending a disposition notification the MCData client needs to determine:

-	the group identity related to an SDS or FD message request received as part of a group communication. The MCData client determines the group identity from the contents of the <mcdata-calling-group-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request; and

-	the MCData user targeted for the disposition notification. The MCData client determines the targeted MCData user from the contents of the <mcdata-calling-user-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request.

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	shall follow the rules specified in subclause 6.4 for the handling of MIME bodies in a SIP message when processing the remaining steps in this subclause;

3)	shall insert in the SIP MESSAGE request an application/resource-lists+xml MIME body containing the MCData ID of the targeted MCData user, according to rules and procedures of IETF RFC 5366 [18];

4)	void;

5)	if sending a disposition notification in response to an MCData group data request, shall include an <mcdata-calling-group-id> element set to the MCData group identity in the application/vnd.3gpp.mcdata-info+xml MIME body;

6)	if requiring to send an SDS notification, shall generate an SDS NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.1;

7)	if requiring to send an FD notification, shall generate an FD NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.2; and

8)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 10.2.3.1]

The media storage client on the MCData client shall send HTTP requests over a TLS connection as specified for the HTTP client in the UE, in annex A of 3GPP TS 24.482 [24].

NOTE 1:	The HTTP client encodes the MCData ID in the bearer access token of the Authorization header field of an HTTP request as specified in 3GPP TS 24.482 [24].

NOTE 2:	The HTTP client always sends the HTTP requests to an HTTP proxy. Annex A of 3GPP TS 24.482 [24] indicates how the HTTP proxy forwards the HTTP request to the HTTP server.

To download a file from the media storage function on the controlling MCData function, the media storage client on the MCData client:

1)	shall generate an HTTP GET request as specified in IETF RFC 7230 [22] and IETF RFC 7231 [23] with a Request-URI set to an absolute URI identifying the URL of the file being requested from the media storage function on the controlling MCData function; and

2)	shall send the HTTP GET request towards the media storage function on the controlling MCData function.

On receipt of a HTTP 200 OK response containing the requested file, the MCData client shall notify the user or application that the file has been successfully downloaded.

6.2.4.3	Test description

6.2.4.3.1	Pre-test conditions

System Simulator:

-	SS (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCData client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

-	Test files downloaded or received at previous test runs are deleted.

Preamble:

-	In the MCData Group Configuration document the <mcdata-on-network-max-data-size-auto-recv> shall be set to 0 to indicate non-mandatory download independent from the file size.

-	Timer TDU2 (FD non-mandatory download timer) is configured to be 15 seconds.

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCData Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.2.4.3.2	Test procedure sequence

Table 6.2.4.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD message for group file distribution with disposition request "FILE DOWNLOAD COMPLETED UPDATE"? (NOTE 2) | - | - | 1 | P |
| 2 | Void | - | - | - | - |
| 3 | Check: Does the UE (MCData client) notify the user of the incoming FD request? (NOTE 1) | - | - | 1 | P |
| 4 | The SS waits 15s for expiry of timer TDU2. | - | - | - | - |
| 4A | Make the UE (MCData client) accept the FD request and download the file. (NOTE 1) | - | - | - | - |
| 5 | Check: Does the UE (MCData client) correctly perform procedure 'FD file accept and download using HTTP' as described in TS 36.579-1 [2] Table 5.3C.11.3-1 to download test file 1? (NOTE 3) | - | - | 2,3 | P |
| 6-8 | Void | - | - | - | - |
| 9 | Check: Does the UE (MCData client) notify the user of the file download? (NOTE 1) | - | - | 3 | P |
| 9A | Check: Has the UE (MCData client) downloaded test file 1 (TS 36.579-7 A.3.1)? (NOTE 1) | - | - | 2 | P |
| 10-11 | Void | - | - | - | - |
| 12 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD message for group file distribution with disposition request "FILE DOWNLOAD COMPLETED UPDATE"? (NOTE 2) | - | - | 1 | P |
| 13 | Void | - | - | - | - |
| 14 | Check: Does the UE (MCData client) notify the user of the incoming FD request? (NOTE 1) | - | - | 1 | P |
| 15 | The SS waits 15s for expiry of timer TDU2. | - | - | - | - |
| 15A | Make the UE (MCData client) reject the FD request. (NOTE 1) | - | - | - | - |
| 16 | Check: Does the UE (MCData client) correctly perform procedure 'CO SDS or FD message transfer using signalling plane' as described in TS 36.579-1 [2] Table 5.3C.1.3-1 to send an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD REQUEST REJECTED"? | - | - | 4 | P |
| 17 | Void | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Timer TDU2 (FD non-mandatory download timer) starts on reception of the FD request via the SIP MESSAGE request. NOTE 3: Test file 1 for CT FD as specified in annex A.3.1. |  |  |  |  |  |

6.2.4.3.3	Specific message contents

Table 6.2.4.3.3-1: SIP MESSAGE from the SS (steps 1, 12, Table 6.2.4.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.4.3.3-2 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD SIGNALLING PAYLOAD as described in Table 6.2.4.3.3-2A |  |  |  |

Table 6.2.4.3.3-2: MCData-Info (Table 6.2.4.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3, condition MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "group-fd" |  |  |  |

Table 6.2.4.3.3-2A: FD SIGNALLING PAYLOAD (Table 6.2.2.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.6-1, condition FD_HTTP

Table 6.2.4.3.3-3: Void

Table 6.2.4.3.3-4: SIP MESSAGE from the UE (step 5, Table 6.2.4.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.11.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.4.3.3-5 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.4.3.3-6 |  |  |  |

Table 6.2.4.3.3-5: MCData-Info (Table 6.2.4.3.3-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-group-id | Encrypted <mcdata-request-uri> with mcdataURI set to px_MCData_Group_A_ID | Encrypted according to TS 36.579-1 [2] Table 5.5.3.2.1-3A |  |  |

Table 6.2.4.3.3-6: FD NOTIFICATION (Table 6.2.4.3.3-4)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.7-1, condition FD_ACCEPTED

Table 6.2.4.3.3-7: HTTP GET from the UE (step 5, Table 6.2.4.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3C.11.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.2-1, condition FD_HTTP

Table 6.2.4.3.3-8: HTTP 200 (OK) from the SS (step 5, Table 6.2.4.3.2-1;
step 5, TS 36.579-1 [2] Table 5.3C.11.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.6-1, condition FD_HTTP

Table 6.2.4.3.3-9: Void

Table 6.2.4.3.3-10: SIP MESSAGE from the UE (step 5, Table 6.2.4.3.2-1;
step 6a1, TS 36.579-1 [2] Table 5.3C.11.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-headers | MCData-Info as described in Table 6.2.4.3.3-5 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.4.3.3-11 |  |  |  |

Table 6.2.4.3.3-11: FD NOTIFICATION (Table 6.2.4.3.3-10)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.7-1, condition FD_COMPLETED

Table 6.2.4.3.3-12: SIP MESSAGE from the UE (step 16, Table 6.2.4.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-headers | MCData-Info as described in Table 6.2.4.3.3-5 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.4.3.3-13 |  |  |  |

Table 6.2.4.3.3-13: FD NOTIFICATION (Table 6.2.4.3.3-12)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.7-1, condition FD_REJECTED

### 6.2.5	On-network / File Distribution (FD) / FD Using HTTP / One-to-one Standalone FD / Mandatory Download / With Disposition Request / Client Originated (CO)

6.2.5.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a standalone one-to-one FD message with a mandatory download and with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" and the UE (MCDATA Client) is unaware of the URL of the Media Storage Function }

then { UE (MCDATA Client) sends a SIP MESSAGE to find the URL of the Media Storage Function and responds to a SIP MESSAGE that contains the URL of the Media Storage Function with a SIP 200 (OK) message }

}

(2)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a standalone one-to-one FD message with a mandatory download and with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" and the UE (MCDATA Client) is aware of the URL of the Media Storage Function }

then { UE (MCDATA Client) uploads the file to the Media Storage Function via an HTTP POST message and then sends the URL of the file location to the recipient via a SIP MESSAGE message }

}

(3)

with { UE (MCDATA Client) having sent the URL of the file location to the recipient }

ensure that {

when { UE (MCDATA Client) receives a FD notification via a SIP MESSAGE message }

then { UE (MCDATA Client) responds to the SIP MESSAGE message with a SIP 200 (OK) message and delivers the notification to the MCDATA User }

}

6.2.5.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 10.2.1.3.2, 10.2.2.1, 10.2.4.2.1, 12.2.1.2. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 10.2.1.3.2]

To discover the absolute URI of the media storage function, the MCData client shall generate a SIP MESSAGE request towards the participating MCData function, in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	shall follow the rules specified in subclause 6.4 for the handling of MIME bodies in a SIP message when processing the remaining steps in this subclause;

3)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with a <request-type> element containing the value "msf-disc-req";

4)	if the upload of a file is for a group standalone FD request, shall include in an application/vnd.3gpp.mcdata-info+xml MIME body, the <mcdata-calling-group-id> element set to the required MCData group identity; and

NOTE 1:	The absence of a group identity in the <mcdata-calling-group-id> element of the application/vnd.3gpp.mcdata-info+xml MIME body implies that the MCData client intends to upload a file for a one-to-one FD request. In this case, the participating MCData function identifies the MCData ID of the user from the binding between the public user identity and the MCData ID.

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

On receipt of a "SIP MESSAGE request for absolute URI discovery response", the MCData client:

1)	shall store the absolute URI found in the <mcdata-controller-psi> element;

2)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5]; and

3)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 10.2.2.1]

If the media storage client is not aware of the absolute URI of the media storage function, the media storage client shall request the MCData client to discover the absolute URI associated with the media storage function by following the procedures in subclause 10.2.1.3.

The media storage client shall send HTTP requests over a TLS connection as specified for the HTTP client in the UE in annex A of 3GPP TS 24.482 [24].

NOTE 1:	The HTTP client encodes the MCData ID in the bearer access token of the Authorization header field of an HTTP request as specified in 3GPP TS 24.482 [24].

NOTE 2:	The HTTP client always sends the HTTP requests to an HTTP proxy. Annex A of 3GPP TS 24.482 [24] indicates how the HTTP proxy forwards the HTTP request to the HTTP server.

To upload a file to media storage function, the media storage client:

1)	shall generate an HTTP POST request as specified in IETF RFC 7230 [22] and IETF RFC 7231 [23];

2)	shall set the Request-URI to the absolute URI identifying the resource on a media storage function;

3)	shall set the Host header field to a hostname identifying the media storage function;

4)	shall set the Content-Type header field to multipart/mixed and with a boundary delimiter parameter set to any chosen value;

5)	if the file upload is for one-to-one file distribution, shall insert an application/vnd.3gpp.mcdata-info+xml MIME body with:

a)	the <request-type> element set to a value of "one-to-one-fd"; and

b)	the <mcdata-calling-user-id> element set to the originating MCData ID;

6)	if the file upload is for group file distribution, shall insert an application/vnd.3gpp.mcdata-info+xml MIME body with:

a)	the <request-type> element set to a value of "group-fd";

b)	the <mcdata-request-uri> element set to the MCData group identity; and

c)	the <mcdata-calling-user-id> element set to the originating MCData ID;

7)	if end-to-end security is required for a one-to-one communication, the MCData client protects the binary data representing the file and prefixes the protected binary data with security parameters as described in 3GPP TS 33.180 [26];

8)	if

i)	end-to-end security is not required for a one-to-one communication, or

ii)	the file upload is for group file distribution;

shall include the binary data representing the file with Content-Type field set to application/octet-stream and Content-Length field set to the file size; and

9)	shall send the HTTP POST request towards the media storage function.

On receipt of a HTTP 201 Created containing a Location header field with a URL identifying the location of the resource where the file has been stored on the media storage function, then the media storage client shall store this information.

[TS 24.282, clause 10.2.4.2.1]

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	if a one-to-one standalone FD message is to be sent shall insert in the SIP MESSAGE request:

a)	an application/resource-lists+xml MIME body with the MCData ID of the target MCData user, according to rules and procedures of IETF RFC 4826 [9]; and

b)	an application/vnd.3gpp.mcdata-info+xml MIME body with a <request-type> element set to a value of "one-to-one-fd";

…

4)	shall generate a standalone FD message as specified in subclause 6.2.2.2; and

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 12.2.1.2]

Upon receipt of a:

"SIP MESSAGE request for SDS disposition notification for terminating MCData client"; or

"SIP MESSAGE request for FD disposition notification for terminating MCData client";

the MCData client:

1)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2)	shall deliver the notification to the user or application.

6.2.5.3	Test description

6.2.5.3.1	Pre-test conditions

System Simulator:

-	SS (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCData client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

-	Test File 1 for CO FD as specified in annex A.2.1 is available at the UE for upload.

Preamble:

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCData Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.2.5.3.2	Test procedure sequence

Table 6.2.5.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) send test file 1 (TS 36.579-7 A.2.1) for CO one-to-one FD over HTTP for mandatory download and with disposition request "FILE DOWNLOAD COMPLETED UPDATE". (NOTE 1, NOTE 2) | - | - | - | - |
| 2 | Check: Does the UE (MCData client) correctly perform procedure 'Discovery of the absolute URI of the media storage function (one-to-one communication)' as described in TS 36.579-1 [2] Table 5.3C.8.3-1 ? | - | - | 1 | P |
| 3 | Check: Does the UE (MCData client) correctly perform procedure 'FD file upload using HTTP' as described in TS 36.579-1 [2] Table 5.3C.10.3-1? | - | - | 2 | P |
| 3A | Check: Is the content of the uploaded file the same as specified in annex A.2.1? | - | - | 2 | P |
| 4-6 | Void | - | - | - | - |
| 7 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD REQUEST ACCEPTED" for the FD message sent at step 3? | - | - | 3 | P |
| 8 | Void | - | - | - | - |
| 9 | Check: Does the UE (MCData client) notify the user that the remote client has accepted the download? (NOTE 1) | - | - | 3 | P |
| 10 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD COMPLETED" for the FD message sent at step 3? | - | - | 3 | P |
| 11 | Void | - | - | - | - |
| 12 | Check: Does the UE (MCData client) notify the user that the remote client has completed the download? (NOTE 1) | - | - | 3 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Test file 1 for CO FD as specified in annex A.2.1. |  |  |  |  |  |

6.2.5.3.3	Specific message contents

Table 6.2.5.3.3-1..6: Void

Table 6.2.5.3.3-7: HTTP POST from the UE (step 3, Table 6.2.5.3.2-1; 
step 2, TS 36.579-1 [2] Table 5.3C.10.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1, condition FD_HTTP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.1.3.3-8 |  |  |  |

Table 6.2.5.3.3-8: MCData-Info (Table 6.2.5.3.3-7)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "one-to-one-fd" |  |  |  |
| mcdata-calling-user-id | px_MCData_ID_User_A |  |  |  |

Table 6.2.5.3.3-9..10: Void

Table 6.2.5.3.3-11: HTTP 201 Created from the SS (step 3, Table 6.2.5.3.2-1; 
step 3, TS 36.579-1 [2] Table 5.3C.10.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.7-1, condition FD_HTTP

Table 6.2.5.3.3-12: SIP MESSAGE from the UE (step 3, Table 6.2.5.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3C.10.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.5.3.3-13 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD SIGNALLING PAYLOAD as described in Table 6.2.5.3.3-14 |  |  |  |

Table 6.2.5.3.3-13: MCData-Info (Table 6.2.5.3.3-12)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "one-to-one-fd" |  |  |  |

Table 6.2.5.3.3-14: FD Signalling Payload (Table 6.2.5.3.3-12)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.5-1, condition FD_HTTP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Mandatory download | '0001'B | MANDATORY DOWNLOAD | TS 24.282 [31] clause 15.2.16 |  |

Table 6.2.5.3.3-15: SIP MESSAGE from the SS (step 7, Table 6.2.5.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION from the SS as described in Table 6.2.5.3.3-17 |  |  |  |

Table 6.2.5.3.3-16: Void

Table 6.2.5.3.3-17: FD NOTIFICATION (Table 6.2.5.3.3-15)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.8-1, condition FD_ACCEPTED

Table 6.2.5.3.3-18: SIP MESSAGE from the SS (step 10, Table 6.2.5.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION from the SS as described in Table 6.2.5.3.3-19 |  |  |  |

Table 6.2.5.3.3-19: FD NOTIFICATION (Table 6.2.5.3.3-18)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.8-1, condition FD_COMPLETED

### 6.2.6	On-network / File Distribution (FD) / FD Using HTTP / One-to-one Standalone FD / Mandatory Download / With Disposition Request / Client Terminated (CT)

6.2.6.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { UE (MCDATA Client) receives a SIP MESSAGE message for a standalone one-to-one FD message with a mandatory download and with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" }

then { UE (MCDATA Client) responds with a SIP 200 (OK) message and notifies the MCDATA User about the incoming FD request and that the file identified by file URL in the Payload data in the Payload IE will be downloaded automatically and generates an FD NOTIFICATION indicating acceptance of the FD request and attempts to download the file with an HTTP GET message }

}

(2)

with { UE (MCDATA Client) having started to download the file by sending an HTTP GET message }

ensure that {

when { UE (MCDATA Client) has successfully downloaded the file }

then { UE (MCDATA Client) notifies the MCDATA User that the file has successfully downloaded and generates an FD NOTIFICATION indicating the successful download of the file }

}

6.2.6.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 10.2.4.2.2, 10.2.1.2.3, 12.2.1.1, 10.2.3.1. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 10.2.4.2.2]

Upon receipt of a "SIP MESSAGE request for FD using HTTP for terminating MCData client", the MCData client:

1)	may reject the SIP MESSAGE request if there are not enough resources to handle the SIP MESSAGE request;

2)	if the SIP MESSAGE request is rejected in step 1), shall respond towards the participating MCData function with a SIP 480 (Temporarily unavailable) response and skip the rest of the steps of this subclause;

3)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5];

4)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5]; and

5)	shall handle the received message as specified in subclause 10.2.1.2.

[TS 24.282, clause 10.2.1.2.3]

The MCData client:

1)	if the FD SIGNALLING PAYLOAD message does not contain an Application ID IE:

a)	shall determine that the payload contained in the Payload IE in the FD SIGNALLING PAYLOAD message is for user consumption;

b)	shall notify the user about the incoming FD request; and

c)	if the FD SIGNALLING PAYLOAD message contains a Metadata IE, shall deliver the contents of the Metadata IE to the user;

2)	if the FD SIGNALLING PAYLOAD message contains an Application ID IE:

a)	shall determine that the payload contained in the Payload IE in the FD SIGNALLING PAYLOAD message is not for user consumption;

b)	if the Application ID value is unknown, shall discard the FD message and exit this subclause;

c)	if the Application ID value is known, shall notify the application of the incoming FD request; and

NOTE 1:	If FD request is addressed to a non-MCData application that is not running, the MCData client starts the local non-MCData application.

d)	if the FD SIGNALLING PAYLOAD message contains a Metadata IE, shall deliver the contents of the Metadata IE to the application;

3)	shall start a timer TDU2 (FD non-mandatory download timer) with the timer value as specified in subclause F.2.3;

4)	shall wait for the user or application to request to download the file indicated by file URL in the Payload data in the Payload IE in the FD SIGNALLING PAYLOAD message;

5)	if the user or application accepts or rejects or decides to defer the FD request, shall stop timer TDU2 (FD non-mandatory download timer);

6)	if the user deferred the FD request while the timer TDU2 (FD non-mandatory download timer) was running, shall generate an FD NOTIFICATION indicating deferral of the FD request as specified in subclause 12.2.1.1;

NOTE 2:	Once the timer TDU2 (FD non-mandatory download timer) has expired the FD request can only be accepted or rejected with an appropriate action by the MCData client.

NOTE 3:	Once the timer TDU2 (FD non-mandatory download timer) has expired, no action is taken by the MCData client if the FD request is deferred.

7)	if the user or application rejects the FD request, shall generate an FD NOTIFICATION indicating rejection of the FD request as specified in subclause 12.2.1.1 and shall exit this subclause; and

8)	if the user accepts the FD request:

a)	shall generate an FD NOTIFICATION indicating acceptance of the FD request as specified in subclause 12.2.1.1;

b)	if the FD SIGNALLING PAYLOAD message contains a new Conversation ID, shall instantiate a new conversation with the Message ID in the FD SIGNALLING PAYLOAD identifying the first message in the conversation thread;

c)	if the FD SIGNALLING PAYLOAD message contains an existing Conversation ID and:

i)	if the FD SIGNALLING PAYLOAD message does not contain an InReplyTo message ID, shall use the Message ID in the FD SIGNALLING PAYLOAD to identify a new message in the existing conversation thread; and

ii)	if the FD SIGNALLING PAYLOAD message contains an InReplyTo message ID, shall associate the message to an existing message in the conversation thread as identified by the InReplyTo message ID in the FD SIGNALLING PAYLOAD, and use the Message ID in the FD SIGNALLING PAYLOAD to identify the new message;

d)	may store the Conversation ID, Message ID, InReplyTo message ID and Date and time in local storage;

e)	shall attempt to download the file as identified by the file URL in the Payload IE in the FD SIGNALLING PAYLOAD message, as specified in subclause 10.2.3.1; and

f)	if the received FD SIGNALLING PAYLOAD message contains an FD disposition request type IE requesting a file download completed update, then after the file download has been successfully downloaded, shall generate an FD NOTIFICATION by following the procedures in subclause 12.2.1.1.

[TS 24.282, clause 12.2.1.1]

The MCData client shall follow the procedures in this subclause to:

-	indicate to an MCData client that an SDS message was delivered, read or delivered and read when the originating client requested a delivery, read or delivery and read report;

-	indicate to the participating MCData function serving the MCData user that an SDS message was undelivered. The participating MCData function can store the message for later re-delivery;

-	indicate to an MCData client that a request for FD was accepted, deferred or rejected; or

-	indicate to an MCData client that a file download has been completed;

Before sending a disposition notification the MCData client needs to determine:

-	the group identity related to an SDS or FD message request received as part of a group communication. The MCData client determines the group identity from the contents of the <mcdata-calling-group-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request; and

-	the MCData user targeted for the disposition notification. The MCData client determines the targeted MCData user from the contents of the <mcdata-calling-user-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request.

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	shall follow the rules specified in subclause 6.4 for the handling of MIME bodies in a SIP message when processing the remaining steps in this subclause;

3)	shall insert in the SIP MESSAGE request an application/resource-lists+xml MIME body containing the MCData ID of the targeted MCData user, according to rules and procedures of IETF RFC 5366 [18];

4)	void;

5)	if sending a disposition notification in response to an MCData group data request, shall include an <mcdata-calling-group-id> element set to the MCData group identity in the application/vnd.3gpp.mcdata-info+xml MIME body;

6)	if requiring to send an SDS notification, shall generate an SDS NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.1;

7)	if requiring to send an FD notification, shall generate an FD NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.2; and

8)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 10.2.3.1]

The media storage client on the MCData client shall send HTTP requests over a TLS connection as specified for the HTTP client in the UE, in annex A of 3GPP TS 24.482 [24].

NOTE 1:	The HTTP client encodes the MCData ID in the bearer access token of the Authorization header field of an HTTP request as specified in 3GPP TS 24.482 [24].

NOTE 2:	The HTTP client always sends the HTTP requests to an HTTP proxy. Annex A of 3GPP TS 24.482 [24] indicates how the HTTP proxy forwards the HTTP request to the HTTP server.

To download a file from the media storage function on the controlling MCData function, the media storage client on the MCData client:

1)	shall generate an HTTP GET request as specified in IETF RFC 7230 [22] and IETF RFC 7231 [23] with a Request-URI set to an absolute URI identifying the URL of the file being requested from the media storage function on the controlling MCData function; and

2)	shall send the HTTP GET request towards the media storage function on the controlling MCData function.

On receipt of a HTTP 200 OK response containing the requested file, the MCData client shall notify the user or application that the file has been successfully downloaded.

6.2.6.3	Test description

6.2.6.3.1	Pre-test conditions

System Simulator:

-	SS (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCData client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

-	Test files downloaded or received at previous test runs are deleted.

Preamble:

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCData Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.2.6.3.2	Test procedure sequence

Table 6.2.6.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-1B | Check: Does the UE (MCData client) correctly perform steps 1a1-3 of procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD message for one-to-one file distribution with disposition request "FILE DOWNLOAD COMPLETED UPDATE" and Mandatory Download IE? | - | - | 1 | P |
| 2-3 | Void | - | - | - | - |
| 4 | Check: Does the UE (MCData client) correctly perform procedure 'FD file accept and download using HTTP' as described in TS 36.579-1 [2] Table 5.3C.11.3-1 to download test file 1? (NOTE 2) | - | - | 1,2 | P |
| 5-10 | Void | - | - | - | - |
| 11 | Check: Does the UE (MCData client) notify the user of the incoming FD request and the file download? (NOTE 1) | - | - | 1,2 | P |
| 12 | Check: Has the UE (MCData client) downloaded test file 1 (TS 36.579-7 A.3.1)? (NOTE 1) | - | - | 2 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Test file 1 for CT FD as specified in annex A.3.1. |  |  |  |  |  |

6.2.6.3.3	Specific message contents

Table 6.2.6.3.3-1: SIP MESSAGE from the SS (step 1A, Table 6.2.6.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.6.3.3-2 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD SIGNALLING PAYLOAD as described in Table 6.2.6.3.3-3 |  |  |  |

Table 6.2.6.3.3-2: MCData-Info (Table 6.2.6.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3, condition MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "one-to-one-fd" |  |  |  |

Table 6.2.6.3.3-3: FD Signalling Payload (Table 6.2.6.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.6-1, condition FD_HTTP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Mandatory download | '0001'B | MANDATORY DOWNLOAD | TS 24.282 [31] clause 15.2.16 |  |

Table 6.2.6.3.3-4: Void

Table 6.2.6.3.3-5: SIP MESSAGE from the UE (step 4, Table 6.2.6.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.11.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part | not present | MCData Info |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.6.3.3-6 |  |  |  |

Table 6.2.6.3.3-6: FD NOTIFICATION (Table 6.2.6.3.3-5)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.7-1, condition FD_ACCEPTED

Table 6.2.6.3.3-7: HTTP GET from the UE (step 4, Table 6.2.6.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3C.11.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.2-1, condition FD_HTTP

Table 6.2.6.3.3-8: HTTP 200 (OK) from the SS (step 4, Table 6.2.6.3.2-1;
step 5, TS 36.579-1 [2] Table 5.3C.11.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.6-1, condition FD_HTTP

Table 6.2.6.3.3-9..10: Void

Table 6.2.6.3.3-11: SIP MESSAGE from the UE (step 4, Table 6.2.6.3.2-1;
step 6a1, TS 36.579-1 [2] Table 5.3C.11.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part | not present | MCData Info |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.6.3.3-12 |  |  |  |

Table 6.2.6.3.3-12: FD NOTIFICATION (Table 6.2.6.3.3-11)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.7-1, condition FD_COMPLETED

### 6.2.7	On-network / File Distribution (FD) / FD Using HTTP / Group Standalone FD / Mandatory Download / Without Disposition Request / Client Originated (CO)

6.2.7.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a Group Standalone FD message with a mandatory download and without a disposition request and the UE (MCDATA Client) is unaware of the URL of the Media Storage Function }

then { UE (MCDATA Client) sends a SIP MESSAGE to find the URL of the Media Storage Function and responds to a SIP MESSAGE that contains the URL of the Media Storage Function with a SIP 200 (OK) message }

}

(2)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a Group Standalone FD message with a mandatory download and without a disposition request and the UE (MCDATA Client) is aware of the URL of the Media Storage Function }

then { UE (MCDATA Client) uploads the file to the Media Storage Function via an HTTP POST message and then sends the URL of the file location to the recipient via a SIP MESSAGE message }

}

(3)

with { UE (MCDATA Client) having sent the URL of the file location to the recipient }

ensure that {

when { UE (MCDATA Client) receives a FD notification via a SIP MESSAGE message }

then { UE (MCDATA Client) responds to the SIP MESSAGE message with a SIP 200 (OK) message and delivers the notification to the MCDATA User }

}

6.2.7.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 10.2.1.3.2, 10.2.2.1, 10.2.4.2.1, 12.2.1.2. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 10.2.1.3.2]

To discover the absolute URI of the media storage function, the MCData client shall generate a SIP MESSAGE request towards the participating MCData function, in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	shall follow the rules specified in subclause 6.4 for the handling of MIME bodies in a SIP message when processing the remaining steps in this subclause;

3)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with a <request-type> element containing the value "msf-disc-req";

4)	if the upload of a file is for a group standalone FD request, shall include in an application/vnd.3gpp.mcdata-info+xml MIME body, the <mcdata-calling-group-id> element set to the required MCData group identity; and

NOTE 1:	The absence of a group identity in the <mcdata-calling-group-id> element of the application/vnd.3gpp.mcdata-info+xml MIME body implies that the MCData client intends to upload a file for a one-to-one FD request. In this case, the participating MCData function identifies the MCData ID of the user from the binding between the public user identity and the MCData ID.

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

On receipt of a "SIP MESSAGE request for absolute URI discovery response", the MCData client:

1)	shall store the absolute URI found in the <mcdata-controller-psi> element;

2)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5]; and

3)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 10.2.2.1]

If the media storage client is not aware of the absolute URI of the media storage function, the media storage client shall request the MCData client to discover the absolute URI associated with the media storage function by following the procedures in subclause 10.2.1.3.

The media storage client shall send HTTP requests over a TLS connection as specified for the HTTP client in the UE in annex A of 3GPP TS 24.482 [24].

NOTE 1:	The HTTP client encodes the MCData ID in the bearer access token of the Authorization header field of an HTTP request as specified in 3GPP TS 24.482 [24].

NOTE 2:	The HTTP client always sends the HTTP requests to an HTTP proxy. Annex A of 3GPP TS 24.482 [24] indicates how the HTTP proxy forwards the HTTP request to the HTTP server.

To upload a file to media storage function, the media storage client:

1)	shall generate an HTTP POST request as specified in IETF RFC 7230 [22] and IETF RFC 7231 [23];

2)	shall set the Request-URI to the absolute URI identifying the resource on a media storage function;

3)	shall set the Host header field to a hostname identifying the media storage function;

4)	shall set the Content-Type header field to multipart/mixed and with a boundary delimiter parameter set to any chosen value;

5)	if the file upload is for one-to-one file distribution, shall insert an application/vnd.3gpp.mcdata-info+xml MIME body with:

a)	the <request-type> element set to a value of "one-to-one-fd"; and

b)	the <mcdata-calling-user-id> element set to the originating MCData ID;

6)	if the file upload is for group file distribution, shall insert an application/vnd.3gpp.mcdata-info+xml MIME body with:

a)	the <request-type> element set to a value of "group-fd";

b)	the <mcdata-request-uri> element set to the MCData group identity; and

c)	the <mcdata-calling-user-id> element set to the originating MCData ID;

7)	if end-to-end security is required for a one-to-one communication, the MCData client protects the binary data representing the file and prefixes the protected binary data with security parameters as described in 3GPP TS 33.180 [26];

8)	if

i)	end-to-end security is not required for a one-to-one communication, or

ii)	the file upload is for group file distribution;

shall include the binary data representing the file with Content-Type field set to application/octet-stream and Content-Length field set to the file size; and

9)	shall send the HTTP POST request towards the media storage function.

On receipt of a HTTP 201 Created containing a Location header field with a URL identifying the location of the resource where the file has been stored on the media storage function, then the media storage client shall store this information.

[TS 24.282, clause 10.2.4.2.1]

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	if a one-to-one standalone FD message is to be sent shall insert in the SIP MESSAGE request:

a)	an application/resource-lists+xml MIME body with the MCData ID of the target MCData user, according to rules and procedures of IETF RFC 4826 [9]; and

b)	an application/vnd.3gpp.mcdata-info+xml MIME body with a <request-type> element set to a value of "one-to-one-fd";

…

4)	shall generate a standalone FD message as specified in subclause 6.2.2.2; and

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 12.2.1.2]

Upon receipt of a:

"SIP MESSAGE request for SDS disposition notification for terminating MCData client"; or

"SIP MESSAGE request for FD disposition notification for terminating MCData client";

the MCData client:

1)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2)	shall deliver the notification to the user or application.

6.2.7.3	Test description

6.2.7.3.1	Pre-test conditions

System Simulator:

-	SS (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCData client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

-	Test File 1 for CO FD as specified in annex A.2.1 is available at the UE for upload.

Preamble:

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCData Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.2.7.3.2	Test procedure sequence

Table 6.2.7.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) send test file 1 (TS 36.579-7 A.2.1) for CO group FD over HTTP for mandatory download and without disposition request. (NOTE 1, NOTE 2) | - | - | - | - |
| 2 | Check: Does the UE (MCData client) correctly perform procedure 'Discovery of the absolute URI of the media storage function (one-to-one communication)' as described in TS 36.579-1 [2] Table 5.3C.9.3-1 ? | - | - | 1 | P |
| 3 | Check: Does the UE (MCData client) correctly perform procedure 'FD file upload using HTTP' as described in TS 36.579-1 [2] Table 5.3C.10.3-1? | - | - | 2 | P |
| 3A | Check: Is the content of the uploaded file the same as specified in annex A.2.1? | - | - | 2 | P |
| 4-6 | Void | - | - | - | - |
| 7 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD REQUEST ACCEPTED" for the FD message sent at step 3? | - | - | 3 | P |
| 8 | Void | - | - | - | - |
| 9 | Check: Does the UE (MCData client) notify the user that the remote client has accepted the download? (NOTE 1) | - | - | 3 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Test file 1 for CO FD as specified in annex A.2.1. |  |  |  |  |  |

6.2.7.3.3	Specific message contents

Table 6.2.7.3.3-1..6: Void

Table 6.2.7.3.3-7: HTTP POST from the UE (step 3, Table 6.2.7.3.2-1; 
step 2, TS 36.579-1 [2] Table 5.3C.10.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1, condition FD_HTTP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.7.3.3-8 |  |  |  |

Table 6.2.7.3.3-8: MCData-Info (Table 6.2.7.3.3-7)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "group-fd" |  |  |  |
| mcdata-request-uri | px_MCData_Group_A_ID | NOTE: the element is not encrypted |  |  |
| mcdata-calling-user-id | px_MCData_ID_User_A | NOTE: the element is not encrypted |  |  |

Table 6.2.7.3.3-9: Void

Table 6.2.7.3.3-10: HTTP 201 Created from the SS (step 3, Table 6.2.7.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.10.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.7-1, condition FD_HTTP

Table 6.2.7.3.3-11: SIP MESSAGE from the UE (step 3, Table 6.2.7.3.2-1; 
step 4, TS 36.579-1 [2] Table 5.3C.10.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.7.3.3-12 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD SIGNALLING PAYLOAD as described in Table 6.2.7.3.3-13 |  |  |  |

Table 6.2.7.3.3-12: MCData-Info (Table 6.2.7.3.3-11)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "group-fd" |  |  |  |

Table 6.2.7.3.3-13: FD Signalling Payload (Table 6.2.5.3.3-12)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.5-1, condition FD_HTTP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| FD disposition request type | Not present | no disposition request |  |  |
| Mandatory download | '0001'B | MANDATORY DOWNLOAD | TS 24.282 [31] clause 15.2.16 |  |

Table 6.2.7.3.3-14: SIP MESSAGE from the SS (step 7, Table 6.2.7.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.1.3.3-15 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION from the SS as described in Table 6.2.3.3.3-17 |  |  |  |

Table 6.2.7.3.3-15: MCData-Info (Table 6.2.7.3.3-14)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-group-id | Encrypted <mcdata-calling-group-id> with mcdataURI set to px_MCData_Group_A_ID | Encrypted according to TS 36.579-1 [2] Table 5.5.3.2.2-3A |  |  |

Table 6.2.7.3.3-16: FD NOTIFICATION (Table 6.2.7.3.3-14)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.8-1, condition FD_ACCEPTED

### 6.2.8	On-network / File Distribution (FD) / FD Using HTTP / Group Standalone FD / Mandatory Download / Without Disposition Request / Client Terminated (CT)

6.2.8.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { UE (MCDATA Client) receives a SIP MESSAGE message for a group standalone FD message with a mandatory download and without a disposition request }

then { UE (MCDATA Client) responds with a SIP 200 (OK) message and notifies the MCDATA User about the incoming FD request and that the file identified by file URL in the Payload data in the Payload IE will be downloaded automatically and generates an FD NOTIFICATION indicating acceptance of the FD request and attempts to download the file with an HTTP GET message }

}

(2)

with { UE (MCDATA Client) having started to download the file by sending an HTTP GET message }

ensure that {

when { UE (MCDATA Client) has successfully downloaded the file }

then { UE (MCDATA Client) notifies the MCDATA User that the file has successfully downloaded }

}

6.2.8.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 10.2.4.2.2, 10.2.1.2.3, 12.2.1.1, 10.2.3.1. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 10.2.4.2.2]

Upon receipt of a "SIP MESSAGE request for FD using HTTP for terminating MCData client", the MCData client:

1)	may reject the SIP MESSAGE request if there are not enough resources to handle the SIP MESSAGE request;

2)	if the SIP MESSAGE request is rejected in step 1), shall respond towards the participating MCData function with a SIP 480 (Temporarily unavailable) response and skip the rest of the steps of this subclause;

3)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5];

4)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5]; and

5)	shall handle the received message as specified in subclause 10.2.1.2.

[TS 24.282, clause 10.2.1.2.3]

The MCData client:

1)	if the FD SIGNALLING PAYLOAD message does not contain an Application ID IE:

a)	shall determine that the payload contained in the Payload IE in the FD SIGNALLING PAYLOAD message is for user consumption;

b)	shall notify the user about the incoming FD request; and

c)	if the FD SIGNALLING PAYLOAD message contains a Metadata IE, shall deliver the contents of the Metadata IE to the user;

2)	if the FD SIGNALLING PAYLOAD message contains an Application ID IE:

a)	shall determine that the payload contained in the Payload IE in the FD SIGNALLING PAYLOAD message is not for user consumption;

b)	if the Application ID value is unknown, shall discard the FD message and exit this subclause;

c)	if the Application ID value is known, shall notify the application of the incoming FD request; and

NOTE 1:	If FD request is addressed to a non-MCData application that is not running, the MCData client starts the local non-MCData application.

d)	if the FD SIGNALLING PAYLOAD message contains a Metadata IE, shall deliver the contents of the Metadata IE to the application;

3)	shall start a timer TDU2 (FD non-mandatory download timer) with the timer value as specified in subclause F.2.3;

4)	shall wait for the user or application to request to download the file indicated by file URL in the Payload data in the Payload IE in the FD SIGNALLING PAYLOAD message;

5)	if the user or application accepts or rejects or decides to defer the FD request, shall stop timer TDU2 (FD non-mandatory download timer);

6)	if the user deferred the FD request while the timer TDU2 (FD non-mandatory download timer) was running, shall generate an FD NOTIFICATION indicating deferral of the FD request as specified in subclause 12.2.1.1;

NOTE 2:	Once the timer TDU2 (FD non-mandatory download timer) has expired the FD request can only be accepted or rejected with an appropriate action by the MCData client.

NOTE 3:	Once the timer TDU2 (FD non-mandatory download timer) has expired, no action is taken by the MCData client if the FD request is deferred.

7)	if the user or application rejects the FD request, shall generate an FD NOTIFICATION indicating rejection of the FD request as specified in subclause 12.2.1.1 and shall exit this subclause; and

8)	if the user accepts the FD request:

a)	shall generate an FD NOTIFICATION indicating acceptance of the FD request as specified in subclause 12.2.1.1;

b)	if the FD SIGNALLING PAYLOAD message contains a new Conversation ID, shall instantiate a new conversation with the Message ID in the FD SIGNALLING PAYLOAD identifying the first message in the conversation thread;

c)	if the FD SIGNALLING PAYLOAD message contains an existing Conversation ID and:

i)	if the FD SIGNALLING PAYLOAD message does not contain an InReplyTo message ID, shall use the Message ID in the FD SIGNALLING PAYLOAD to identify a new message in the existing conversation thread; and

ii)	if the FD SIGNALLING PAYLOAD message contains an InReplyTo message ID, shall associate the message to an existing message in the conversation thread as identified by the InReplyTo message ID in the FD SIGNALLING PAYLOAD, and use the Message ID in the FD SIGNALLING PAYLOAD to identify the new message;

d)	may store the Conversation ID, Message ID, InReplyTo message ID and Date and time in local storage;

e)	shall attempt to download the file as identified by the file URL in the Payload IE in the FD SIGNALLING PAYLOAD message, as specified in subclause 10.2.3.1; and

f)	if the received FD SIGNALLING PAYLOAD message contains an FD disposition request type IE requesting a file download completed update, then after the file download has been successfully downloaded, shall generate an FD NOTIFICATION by following the procedures in subclause 12.2.1.1.

[TS 24.282, clause 12.2.1.1]

The MCData client shall follow the procedures in this subclause to:

-	indicate to an MCData client that an SDS message was delivered, read or delivered and read when the originating client requested a delivery, read or delivery and read report;

-	indicate to the participating MCData function serving the MCData user that an SDS message was undelivered. The participating MCData function can store the message for later re-delivery;

-	indicate to an MCData client that a request for FD was accepted, deferred or rejected; or

-	indicate to an MCData client that a file download has been completed;

Before sending a disposition notification the MCData client needs to determine:

-	the group identity related to an SDS or FD message request received as part of a group communication. The MCData client determines the group identity from the contents of the <mcdata-calling-group-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request; and

-	the MCData user targeted for the disposition notification. The MCData client determines the targeted MCData user from the contents of the <mcdata-calling-user-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request.

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.8.1;

2)	shall follow the rules specified in subclause 6.4 for the handling of MIME bodies in a SIP message when processing the remaining steps in this subclause;

3)	shall insert in the SIP MESSAGE request an application/resource-lists+xml MIME body containing the MCData ID of the targeted MCData user, according to rules and procedures of IETF RFC 5366 [18];

4)	void;

5)	if sending a disposition notification in response to an MCData group data request, shall include an <mcdata-calling-group-id> element set to the MCData group identity in the application/vnd.3gpp.mcdata-info+xml MIME body;

6)	if requiring to send an SDS notification, shall generate an SDS NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.1;

7)	if requiring to send an FD notification, shall generate an FD NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.2; and

8)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 10.2.3.1]

The media storage client on the MCData client shall send HTTP requests over a TLS connection as specified for the HTTP client in the UE, in annex A of 3GPP TS 24.482 [24].

NOTE 1:	The HTTP client encodes the MCData ID in the bearer access token of the Authorization header field of an HTTP request as specified in 3GPP TS 24.482 [24].

NOTE 2:	The HTTP client always sends the HTTP requests to an HTTP proxy. Annex A of 3GPP TS 24.482 [24] indicates how the HTTP proxy forwards the HTTP request to the HTTP server.

To download a file from the media storage function on the controlling MCData function, the media storage client on the MCData client:

1)	shall generate an HTTP GET request as specified in IETF RFC 7230 [22] and IETF RFC 7231 [23] with a Request-URI set to an absolute URI identifying the URL of the file being requested from the media storage function on the controlling MCData function; and

2)	shall send the HTTP GET request towards the media storage function on the controlling MCData function.

On receipt of a HTTP 200 OK response containing the requested file, the MCData client shall notify the user or application that the file has been successfully downloaded.

6.2.8.3	Test description

6.2.8.3.1	Pre-test conditions

System Simulator:

-	SS (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCData client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

-	Test files downloaded or received at previous test runs are deleted.

Preamble:

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCData Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.2.8.3.2	Test procedure sequence

Table 6.2.8.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-1B | Check: Does the UE (MCData client) correctly perform steps 1a1-3 of the procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD message for group file distribution with disposition request "FILE DOWNLOAD COMPLETED UPDATE" and Mandatory Download IE? | - | - | 1 | P |
| 2-3 | Void | - | - | - | - |
| 4-4C | Check: Does the UE (MCData client) correctly perform steps 2-5 of procedure 'FD file accept and download using HTTP' as described in TS 36.579-1 [2] Table 5.3C.11.3-1 to download test file 1? (NOTE 2) | - | - | 1,2 | P |
| 5-7 | Void | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in step 8 and step 9 the events described in Table 6.2.8.3.2-2 take place. | - | - | - | - |
| 8 | Check: Does the UE (MCData client) notify the user of the incoming FD request and the file download? (NOTE 1) | - | - | 1,2 | P |
| 9 | Check: Has the UE (MCData client) downloaded test file 1 (TS 36.579-7 A.3.1)? (NOTE 1) | - | - | 2 | P |
| 10 | The SS releases the RRC connection | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Test file 1 for CT FD as specified in annex A.3.1. |  |  |  |  |  |

Table 6.2.8.3.22-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCData client) send a SIP MESSAGE request for notification of file download completed in parallel to step 8 and step 9 of Table 6.2.8.3.22-1 or at least for 10s? | --> | SIP MESSAGE | - | F |

6.2.8.3.3	Specific message contents

Table 6.2.8.3.3-1: SIP MESSAGE from the SS (step 1A, Table 6.2.8.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.8.3.3-2 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD SIGNALLING PAYLOAD as described in Table 6.2.8.3.3-3 |  |  |  |

Table 6.2.8.3.3-2: MCData-Info (Table 6.2.8.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3, condition MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "group-fd" |  |  |  |

Table 6.2.8.3.3-3: FD Signalling Payload (Table 6.2.6.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.6-1, condition FD_HTTP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| FD disposition request type | Not present | no disposition request |  |  |
| Mandatory download | '0001'B | MANDATORY DOWNLOAD | TS 24.282 [31] clause 15.2.16 |  |

Table 6.2.8.3.3-4: Void

Table 6.2.8.3.3-5: SIP MESSAGE from the UE (step 4, Table 6.2.8.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.11.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-headers | MCData-Info as described in Table 6.2.8.3.3-6 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.8.3.3-7 |  |  |  |

Table 6.2.8.3.3-6: MCData-Info (Table 6.2.8.3.3-5)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-group-id | Encrypted <mcdata-request-uri> with mcdataURI set to px_MCData_Group_A_ID | Encrypted according to TS 36.579-1 [2] Table 5.5.3.2.1-3A |  |  |

Table 6.2.8.3.3-7: FD NOTIFICATION (Table 6.2.8.3.3-5)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.7-1, condition FD_ACCEPTED

Table 6.2.8.3.3-8: HTTP GET from the UE (step 4, Table 6.2.8.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3C.11.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.2-1, condition FD_HTTP

Table 6.2.8.3.3-9: HTTP 200 (OK) from the SS (step 4, Table 6.2.8.3.2-1;
step 5, TS 36.579-1 [2] Table 5.3C.11.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.6-1, condition FD_HTTP

Table 6.2.8.3.3-10: Void

### 6.2.9	On-network / File Distribution (FD) / FD Using Media Plane / One-to-one Standalone FD / Client Originated (CO)

6.2.9.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a one-to-one standalone FD message using the media plane}

then { UE (MCDATA Client) sends a request to establish an MSRP connection via a SIP INVITE message and then responds to the SIP 200 (OK) message with a SIP ACK message }

}

(2)

with { UE (MCDATA Client) having requested the establishment of a MSRP connection }

ensure that {

when { UE (MCDATA Client) receives a SIP 200 (OK) message with the a=setup attribute set to "passive"  from the SS (MCDATA server) }

then { UE (MCDATA Client) sends a blank MSRP SEND message to bind the MSRP connection and then sends the one-to-one standalone FD message via a MSRP SEND message }

}

(3)

with { UE (MCDATA Client) having sent a one-to-one standalone FD message using the media plane }

ensure that {

when { UE (MCDATA Client receives a MSRP 200 (OK) message in response to the last MSRP SEND message indicating that the standalone FD message has been successfully transferred }

then { UE (MCDATA Client) sends a SIP BYE message }

}

(4)

with { UE (MCDATA Client) having sent a one-to-one standalone FD message using the media plane with a disposition of "FILE DOWNLOAD COMPLETED UPDATE" }

ensure that {

when { UE (MCDATA Client receives a disposition response via a SIP MESSAGE message from the SS (MCDATA Server }

then { UE (MCDATA Client) responds to the SIP MESSAGE message by sending a SIP 200 (OK) message and delivers the notification to the MCDATA User }

}

6.2.9.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 10.2.5.2.3, 6.2.2.3, 12.2.1.2, TS 24.582 clause 7.1.2.1. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 10.2.5.2.3]

The MCData client shall generate a SIP INVITE request in accordance with 3GPP TS 24.229 [5] with the clarifications given below.

The MCData client:

1)	shall include the g.3gpp.mcdata.fd media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

2)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.fd media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

3)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

4)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP INVITE request;

5)	should include the "timer" option tag in the Supported header field;

6)	should include the Session-Expires header field according to IETF RFC 4028 [38]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

7)	shall generate and contain an application/vnd.3gpp.mcdata-signalling MIME body with the FD SIGNALLING PAYLOAD as described in subclause 6.2.2.3;

8)	if a one-to-one file distribution is requested:

a)	shall insert in the SIP INVITE request a MIME resource-lists body with the MCData ID of the invited MCData user, according to rules and procedures of IETF RFC 5366 [18]; and

b)	shall contain an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <request-type> element set to a value of "one-to-one-fd";

c)	if an end-to-end security context needs to be established and the security context does not exist or if the existing security context has expired, then:

i)	if necessary, shall instruct the key management client to request keying material from the key management server as described in 3GPP TS 33.180 [26];

ii)	shall use the keying material to generate a PCK as described in 3GPP TS 33.180 [26];

iii)	shall use the PCK to generate a PCK-ID with the four most significant bits set to "0001" to indicate that the purpose of the PCK is to protect one-to-one communications and with the remaining twenty eight bits being randomly generated as described in 3GPP TS 33.180 [26];

iv)	shall encrypt the PCK to a UID associated to the MCData client using the MCData ID of the invited user and a time related parameter as described in 3GPP TS 33.180 [26];

v)	shall generate a MIKEY-SAKKE I_MESSAGE using the encapsulated PCK and PCK-ID as specified in 3GPP TS 33.180 [26]; and

vi)	shall add the MCData ID of the originating MCData to the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26]; and

vii)	shall sign the MIKEY-SAKKE I_MESSAGE using the originating MCData user's signing key provided in the keying material together with a time related parameter, and add this to the MIKEY-SAKKE payload, as described in 3GPP TS 33.180 [26];

9)	if a group file distribution is requested:

a)	if the "/<x>/<x>/Common/MCData/AllowedFD" leaf node present in the group document of the requested MCData group, configured on the group management client as specified in 3GPP TS 24.483 [42] is set to "false", shall reject the request for FD and not continue with the rest of the steps in this subclause; and

b)	shall contain in an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <request-type> element set to a value of "group-fd";

ii)	the <mcdata-request-uri> element set to the MCData group identity; and

iii)	the <mcdata-client-id> element set to the MCData client ID of the originating MCData client;

NOTE 1:	The MCData client does not include the MCData ID of the originating MCData user in the body, as this will be inserted into the body of the SIP INVITE request that is sent from the originating participating MCData function.

10)	shall set the Request-URI of the SIP INVITE request to the public service identity identifying the participating MCData function serving the MCData user;

NOTE 2:	The MCData client is configured with public service identity identifying the participating MCData function serving the MCData user.

11)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [5];

12)	shall include an SDP offer according to 3GPP TS 24.229 [5] with the clarifications given in subclause 10.2.5.2.1; and

13)	shall send the SIP INVITE request towards the MCData server according to 3GPP TS 24.229 [5].

On receipt of a SIP 2xx response to the SIP INVITE request, the MCData client:

1)	shall send a SIP ACK request as specified in 3GPP TS 24.229 [5];

2)	shall start the SIP Session timer according to rules and procedures of IETF RFC 4028 [38]; and

3)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] subclause 10.2.5.1.1..

[TS 24.282, clause 6.2.2.3]

In order to generate an FD message, the MCData client:

1)	shall generate an FD SIGNALLING PAYLOAD message as specified in subclause 15.1.3; and

2)	shall include in the SIP request, the FD SIGNALLING PAYLOAD message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in subclause E.1.

When generating an FD SIGNALLING PAYLOAD message as specified in subclause 15.1.3, the MCData client:

1)	shall set the Date and time IE to the current time as specified in subclause 15.2.8;

2)	if the filestarts a new conversation, shall set the Conversation ID IE to a newly generated Conversation ID value as specified in subclause 15.2.9;

3)	if the file continues an existing conversation, shall set the Conversation ID IE to the Conversation ID value of the existing conversation as specified in subclause 15.2.9;

4)	shall set the Message ID IE to a newly generated Message ID value as specified in subclause 15.2.10;

5)	if the files in reply to a previously received SDS message or file, shall include the InReplyTo message ID IE with the Message ID value in the previously received SDS message or file;

6)	if the file is for user consumption, shall not include an Application ID IE as specified in subclause 15.2.7;

7)	if the file is intended for an application on the terminating MCData client, shall include an Application ID IE with an Application ID value representing the intended application as specified in subclause 15.2.7;

NOTE:	The value chosen for the Application ID value is decided by the mission critical organisation.

8)	if a file download complete notification is required shall include a FD disposition request type IE set to "FILE DOWNLOAD COMPLETED UPDATE" as specified in subclause 15.2.4; and

9)	shall include and set the Mandatory download IE to "MANDATORY DOWNLOAD" as described in subclause 15.2.16.

[TS 24.282, clause 12.2.1.2]

Upon receipt of a:

"SIP MESSAGE request for SDS disposition notification for terminating MCData client"; or

"SIP MESSAGE request for FD disposition notification for terminating MCData client";

the MCData client:

1)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2)	shall deliver the notification to the user or application.

[TS 24.582, clause 6.1.1.2.1]

Upon receiving an indication to establish MSRP connection for standalone SDS using media plane as the originating client, the MCData client:

1.	shall act as an MSRP client according to IETF RFC 6135 [12];

2.	shall act according to IETF RFC 6135 [12], as:

a.	an "active" endpoint, if a=setup attribute in the received SDP answer is set to "passive"; and

b.	an "passive" endpoint, if a=setup attribute in the received SDP answer is set to "active";

3.	shall establish the MSRP connection according to the MSRP connection parameters in the SDP answer received in the SIP 200 (OK) response according to IETF RFC 4975 [11]; and

4.	if acting as an "active" endpoint, shall send an empty MSRP SEND request to bind the MSRP connection to the MSRP session from the perspective of the passive endpoint according to the rules and procedures of IETF RFC 4975 [11] and IETF RFC 6135 [12].

On receiving MSRP 200 (OK) response to the first MSRP SEND request, the MCData client:

1.	shall generate a SDS SIGNALLING PAYLOAD as specified in subclause 6.1.1.2.2;

2.	shall generate a SDS DATA PAYLOAD as specified in subclause 6.1.1.2.3;

3.	shall include the SDS SIGNALLING PAYLOAD and SDS DATA PAYLOAD in an MSRP SEND request as specified in subclause 6.1.1.2.4; and

4.	shall send the MSRP SEND request on the established MSRP connection.

NOTE:	MSRP chunking, if needed, may affect the number of "Content Type" lines in each MSRP SEND message conveying a chunk, as also specified in subclause 6.1.1.2.4.

If MSRP chunking is not used then on receipt of a 200 (OK) response, the MCData client shall terminate the SIP session as specified in 3GPP TS 24.282 [8].

If MSRP chunking is used, the MCData client:

1.	shall send further MSRP SEND requests as necessary;

2.	shall wait for a 200 (OK) response to each MSRP SEND request sent; and

3.	on receipt of the last 200 (OK) response shall terminate the SIP session as specified in 3GPP TS 24.282 [8].

On receiving a non-200 MSRP response to the MSRP SEND request the MCData client shall handle the error as specified in IETF RFC 4975 [11]. To terminate the MSRP session, the MCData client:

1.	if there are further MSRP chunks to send, shall abort transmission of these further MSRP chunks;

2.	shall indicate to MCData user that the SDS message could not be sent; and

3.	shall terminate the SIP session as specified in 3GPP TS 24.282 [8].

On receiving an indication to terminate the session from the signalling plane, the MCData client:

1.	if there are further MSRP chunks to send, shall abort transmission of these further MSRP chunks and may indicate to MCData user that the SDS message could not be sent.

[TS 24.582, clause 7.1.2.1]

Upon receiving an indication to establish MSRP connection for file distribution as the originating client, the MCData client:

1.	shall act as an MSRP client according to IETF RFC 6135 [12];

2.	shall act according to IETF RFC 6135 [12], as:

a.	an "active" endpoint, if a=setup attribute in the received SDP answer is set to "passive"; and

b.	an "passive" endpoint, if a=setup attribute in the received SDP answer is set to "active";

3.	shall establish the MSRP connection according to the MSRP connection parameters in the SDP answer received in the SIP 200 (OK) response according to IETF RFC 4975 [11]; and

4.	if acting as an "active" endpoint, shall send an empty MSRP SEND request to bind the MSRP connection to the MSRP session from the perspective of the passive endpoint according to the rules and procedures of IETF RFC 4975 [11] and IETF RFC 6135 [12].

On receiving MSRP 200 (OK) response to the first MSRP SEND request, the MCData client can send the file. To send the file, the MCData client:

1.	shall generate MSRP SEND for file distribution request according to IETF RFC 4975 [11]. When generating an MSRP SEND, the MCData client:

a.	shall set To-Path header according to the MSRP URI(s) received in the answer SDP;

b.	shall set the first content type as Content-Type = "application/vnd.3gpp.mcdata-file"; and

c.	shall set the body of the MSRP SEND request with MSRP payload. MSRP payload is set to the file or part of the file.

2.	shall send the MSRP SEND request(s) on the established MSRP connection.

If MSRP chunking is used, the MCData client:

1.	shall send further MSRP SEND requests containing the file as necessary;

2.	shall wait for a 200 (OK) response to each MSRP SEND request sent; and

3.	on receipt of the last 200 (OK) response shall terminate the SIP session as specified in 3GPP TS 24.282 [8].

On receiving a non-200 MSRP response to the MSRP SEND request the MCData client shall handle the error as specified in IETF RFC 4975 [11]. To terminate the MSRP session, the MCData client:

1.	if there are further MSRP chunks to send, shall abort transmission of these further MSRP chunks;

2.	shall indicate to MCData user that the file could not be distributed; and

3.	shall terminate the SIP session as specified in 3GPP TS 24.282 [8].

6.2.9.3	Test description

6.2.9.3.1	Pre-test conditions

System Simulator:

-	SS (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCData client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

-	Test File 1 for CO FD as specified in annex A.2.1 is available at the UE for upload.

Preamble:

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCData Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.2.9.3.2	Test procedure sequence

Table 6.2.9.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) send test file 1 (TS 36.579-7 A.2.1) for CO one-to-one FD over media plane with disposition notification type "FILE DOWNLOAD COMPLETED UPDATE". (NOTE 1, NOTE 2) | - | - | - | - |
| 2 | Check: Does the UE (MCData client) correctly perform procedure 'CO MCData Call Establishment' as described in TS 36.579-1 [2] Table 5.3C.2.3-1? | - | - | 1,2 | P |
| 3-6 | Void | - | - | - | - |
| 7 | Check: Does the UE (MCData client) correctly perform procedure 'CO MSRP message transfer' as described in TS 36.579-1 [2] Table 5.3C.4.3-1 to send an FD message containing test file 1 for CO FD? | - | - | 2 | P |
| 7A | Check: Is the content of the transferred file the same as specified in annex A.2.1? | - | - | 2 | P |
| 8 | Check: Does the UE (MCData client) correctly perform procedure 'CO MCData call release' as described in TS 36.579-1 [2] Table 5.3C.6.3-1? | - | - | 3 | P |
| 9 | Void | - | - | - | - |
| 10 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive the disposition notification for the FD message sent at step 7? | - | - | 4 | P |
| 11 | Void | - | - | - | - |
| 12 | Check: Does the UE (MCData client) provide the disposition notification to the user? (NOTE 1) | - | - | 4 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Test file 1 for CO FD as specified in annex A.2.1. |  |  |  |  |  |

6.2.9.3.3	Specific message contents

Table 6.2.9.3.3-1: SIP INVITE from the UE (step 2, Table 6.2.9.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.1-1, condition MCDATA_FD, MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP message as described in Table 6.2.9.3.3-2 |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.9.3.3-3 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD SIGNALLING PAYLOAD as described in Table 6.2.9.3.3-3A |  |  |  |

Table 6.2.9.3.3-2: SDP for SIP INVITE (Table 6.2.9.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-3, condition MCDATA_FD, SDP_OFFER, MCD_1to1

Table 6.2.9.3.3-3: MCData-Info (Table 6.2.9.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "one-to-one-fd" |  |  |  |

Table 6.2.9.3.3-3A: FD SIGNALLING PAYLOAD (Table 6.2.9.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.5-1, condition FD_MSRP

Table 6.2.9.3.3-4: SIP 200 (OK) from the SS (step 2, Table 6.2.9.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP message | As described in Table 6.2.9.3.3-5 |  |  |  |

Table 6.2.9.3.3-5: SDP for SIP 200 (OK) (Table 6.2.9.3.3-4)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-3, condition MCDATA_FD, SDP_ANSWER

Table 6.2.9.3.3-6: MSRP SEND from the UE (step 7, Table 6.2.9.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-file" |  |  |  |
| data | As specified in table 6.2.9.3.3-8 |  |  |  |

Table 6.2.9.3.3-7: Void

Table 6.2.9.3.3-8: MCData Protected Payload Message (Table 6.2.9.3.3-6)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.10-1, condition PROTECTED_FILE, PCK

Table 6.2.9.3.3-9..10: Void

Table 6.2.9.3.3-11: SIP BYE from the UE (step 8, Table 6.2.9.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.6.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Reason |  |  | RFC 3326 [125] |  |
| reason-value | "SIP" |  |  |  |
| protocol-cause | "cause="200"" |  |  |  |
| reason-text | "text="transmission succeeded"" |  |  |  |

Table 6.2.9.3.3-12: Void

Table 6.2.9.3.3-13: SIP MESSAGE from the SS (step 10, Table 6.2.9.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.9.3.3-15 |  |  |  |

Table 6.2.9.3.3-14: Void

Table 6.2.9.3.3-15: FD NOTIFICATION (Table 6.2.9.3.3-13)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.8-1, condition FD_COMPLETED

Table 6.2.9.3.3-16: Void

### 6.2.10	On-network / File Distribution (FD) / FD Using Media Plane / One-to-one Standalone FD / Client Terminated (CT)

6.2.10.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User receives a SIP INVITE to initiate a standalone one-to-one FD message using the media plane }

then { UE (MCDATA Client) responds by sending a SIP 200 (OK) message }

}

(2)

with { UE (MCDATA Client) having responded to the SIP INVITE message that initiated a standalone one-to-one FD message using the media plane }

ensure that {

when { UE (MCDATA Client) receives an MSRP SEND message from the SS (MCDATA Server) }

then { UE (MCDATA Client) responds with an MSRP 200 (OK) message }

}

(3)

with { UE (MCDATA Client) having finished receiving the file from the SS (MCDATA server) }

ensure that {

when { UE (MCDATA Client) receives a SIP BYE message to release communications }

then { UE (MCDATA Client) responds with a SIP 200 (OK) message and then sends a "FILE DOWNLOAD COMPLETED" disposition via a SIP MESSAGE message }

}

6.2.10.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 10.2.5.2.4, 12.2.1.1, 6.2.3.2, TS 24.582 clauses 7.1.3.1, 7.1.3.2. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 10.2.5.2.4]

Upon receipt of an initial SIP INVITE request, the MCData client shall follow the procedures for termination of multimedia sessions in the IM CN subsystem as specified in 3GPP TS 24.229 [5] with the clarifications below.

The MCData client:

1)	may reject the SIP INVITE request if either of the following conditions are met:

a)	MCData client does not have enough resources to handle the call; or

b)	any other reason outside the scope of this specification;

and skip the rest of the steps after step 2;

2)	if the SIP INVITE request is rejected in step 1), shall respond toward participating MCData function either with appropriate reject code as specified in 3GPP TS 24.229 [5] and warning texts as specified in subclause 4.9 or with SIP 480 (Temporarily unavailable) response not including warning texts if the user is authorised to restrict the reason for failure and skip the rest of the steps of this subclause;

3)	if the SDP offer of the SIP INVITE request contains an "a=key-mgmt" attribute field with a "mikey" attribute value containing a MIKEY-SAKKE I_MESSAGE:

a)	shall extract the MCData ID of the originating MCData user from the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26];

b)	shall convert the MCData ID to a UID as described in 3GPP TS 33.180 [26];

c)	shall use the UID to validate the signature of the MIKEY-SAKKE I_MESSAGE as described in 3GPP TS 33.180 [26];

d)	if authentication verification of the MIKEY-SAKKE I_MESSAGE fails, shall reject the SIP INVITE request with a SIP 488 (Not Acceptable Here) response as specified in IETF RFC 4567 [45], and include warning text set to "136 authentication of the MIKEY-SAKKE I_MESSAGE failed" in a Warning header field as specified in subclause 4.9 and not continue with rest of the steps in this subclause; and

e)	if the signature of the MIKEY-SAKKE I_MESSAGE was successfully validated:

i)	shall extract and decrypt the encapsulated PCK using the terminating user's (KMS provisioned) UID key as described in 3GPP TS 33.180 [26]; and

ii)	shall extract the PCK-ID, from the payload as specified in 3GPP TS 33.180 [26];

NOTE:	With the PCK successfully shared between the originating MCData client and the terminating MCData client, both clients are able to create an end-to-end secure session.

4)	may display to the MCData user the MCData ID of the inviting MCData user;

5)	may display to the MCData user the file meta-data of the incoming file as described by the SDP included in the received SIP INVITE request;

6)	if the Mandatory indication IE of the FD SIGNALLING PAYLOAD contained in the application/vnd.3gpp.mcdata-signalling MIME body received in the SIP INVITE request is set to "MANDATORY", then:

i)	shall accept the SIP INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5];

ii)	shall include the option tag "timer" in a Require header field of the SIP 200 (OK) response;

iii)	shall include the Session-Expires header field in the SIP 200 (OK) response and start the SIP session timer according to IETF RFC 4028 [38]. The "refresher" parameter in the Session-Expires header field shall be set to "uas";

iv)	shall include the g.3gpp.mcdata.fd media feature tag in the Contact header field of the SIP 200 (OK) response;

v)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" in the Contact header field of the SIP 200 (OK) response;

vi)	shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP INVITE request according to 3GPP TS 24.229 [5] with the clarifications given in subclause 10.2.5.2.2; and

vii)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5].

On receipt of an SIP ACK message to the sent SIP 200 (OK) message, the MCData client shall:

1)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] subclause 10.2.5.1.2.

On receipt of an indication from the media plane of the successful download of the file and if the received FD SIGNALLING PAYLOAD message contained an FD disposition request type IE requesting a file download completed update indication, then, the MCData client:

1)	shall follow the procedures described in subclause 12.2.1.1.

[TS 24.282, clause 12.2.1.1]

The MCData client shall follow the procedures in this subclause to:

-	indicate to an MCData client that an SDS message was delivered, read or delivered and read when the originating client requested a delivery, read or delivery and read report;

-	indicate to the participating MCData function serving the MCData user that an SDS message was undelivered. The participating MCData function can store the message for later re-delivery;

-	indicate to an MCData client that a request for FD was accepted, deferred or rejected; or

-	indicate to an MCData client that a file download has been completed;

Before sending a disposition notification the MCData client needs to determine:

-	the controlling MCData function that sent the SDS or FD message request. The MCData client determines the controlling MCData function from the contents of the <mcdata-controller-psi> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request;

-	the group identity related to an SDS or FD message request received as part of a group communication. The MCData client determines the group identity from the contents of the <mcdata-calling-group-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request; and

-	the MCData user targeted for the disposition notification. The MCData client determines the targeted MCData user from the contents of the <mcdata-calling-user-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request.

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	shall follow the rules specified in subclause 6.4 for the handling of MIME bodies in a SIP message when processing the remaining steps in this subclause;

3)	shall insert in the SIP MESSAGE request an application/resource-lists+xml MIME body containing the MCData ID of the targeted MCData user, according to rules and procedures of IETF RFC 5366 [18];

4)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with an <mcdata-controller-psi> element containing the PSI of the controlling MCData function;

5)	if sending a disposition notification in response to an MCData group data request, shall include an <mcdata-calling-group-id> element set to the MCData group identity in the application/vnd.3gpp.mcdata-info+xml MIME body;

6)	if requiring to send an SDS notification, shall generate an SDS NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.1;

7)	if requiring to send an FD notification, shall generate an FD NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.2; and

8)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 6.2.3.2]

In order to generate an FD notification, the MCData client:

1)	shall generate an FD NOTIFICATION message as specified in subclause 15.1.6; and

2)	shall include in the SIP request, the FD NOTIFICATION message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in subclause E.1.

When generating an FD NOTIFICATION message as specified in subclause 15.1.6, the MCData client:

1)	if sending a file download accept notification, shall set the FD disposition notification type IE as "FILE DOWNLOAD REQUEST ACCEPTED" as specified in subclause 15.2.6;

2)	if sending a file download reject notification, shall set the FD disposition notification type IE as "FILE DOWNLOAD REQUEST REJECTED" as specified in subclause 15.2.6;

3)	if sending a file download deferred notification, shall set the FD disposition notification type IE as "FILE DOWNLOAD REQUEST DEFERRED" as specified in subclause 15.2.6;

4)	shall set the Conversation ID to the value of the Conversation ID that was received in the FD message as specified in subclause 15.2.9;

5)	shall set the Date and time IE to the current time as specified in subclause 15.2.8; and

6)	if sending a file download completed notification:

a)	shall set the FD disposition notification type IE as "FILE DOWNLOAD COMPLETED" as specified in subclause 15.2.6;

b)	shall set the Message ID to the value of the Message ID that was received in the FD message as specified in subclause 15.2.10;

c)	if the FD message was destined for the user, shall not include an Application ID IE as specified in subclause 15.2.7; and

d)	if the FD message was destined for an application, shall include an Application ID IE set to the value of the Application ID that was included in the FD message as specified in subclause 15.2.3.

[TS 24.582, clause 7.1.3.1]

Upon receiving an indication to establish MSRP connection for file distribution as the terminating client, the MCData client:

1.	shall act as an MSRP client according to IETF RFC 6135 [12];

2.	shall act either as an active endpoint or as an passive endpoint to open the transport connection, according to IETF RFC 6135 [12];

3.	shall establish the MSRP connection according to the MSRP connection parameters in the SDP offer received in the SIP INVITE request according to IETF RFC 4975 [11];

4.	if acting as an "active" endpoint, shall send an empty MSRP SEND request to bind the MSRP connection to the MSRP session from the perspective of the passive endpoint according to the rules and procedures of IETF RFC 4975 [11] and IETF RFC 6135 [12];

Once the MSRP session is established, the MCData client:

1.	on receipt of an MSRP request in the MSRP session, shall follow the rules and procedures defined in IETF RFC 4975 [11] and in IETF RFC 6714 [13];

2.	If an MSRP SEND request indicates the use of chunking, shall wait until all further MSRP SEND requests for the remaining chunks have been received and shall reassemble the entire set of MSRP requests into the file before delivering the content to the application; and

3.	shall handle the received content as described in subclause 7.1.3.2.

[TS 24.582, clause 7.1.3.2]

Upon receiving a file, the MCData client:

1.	shall decode the contents of the application/vnd.3gpp.mcdata-file MIME body; and

2.	once all the chunks of the file are successfully received, shall indicate to the signalling plane that the file download is completed.

6.2.10.3	Test description

6.2.10.3.1	Pre-test conditions

System Simulator:

-	SS (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCData client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

-	Test files downloaded or received at previous test runs are deleted.

Preamble:

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCData Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.2.10.3.2	Test procedure sequence

Table 6.2.10.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCData client) correctly perform procedure 'CT MCData Call Establishment' as described in TS 36.579-1 [2] Table 5.3C.3.3-1? | - | - | 1,2 | P |
| 2-5 | Void | - | - | - | - |
| 6 | Check: Does the UE (MCData client) correctly perform procedure 'CT MSRP message transfer' as described in TS 36.579-1 [2] Table 5.3C.5.3-1 to receive an FD message containing test file 1? (NOTE 2) | - | - | 2 | P |
| 7 | Void | - | - | - | - |
| 8 | Check: Does the UE (MCData client) correctly perform procedure 'CT MCData call release' as described in TS 36.579-1 [2] Table 5.3C.7.3-1? | - | - | 3 | P |
| 9-10 | Void | - | - | - | - |
| 11 | Check: Does the UE (MCData client) correctly perform procedure 'CO SDS or FD message transfer using signalling plane' as described in TS 36.579-1 [2] Table 5.3C.1.3-1 to send a disposition notification for the file received at step 6? | - | - | 2 | P |
| 12 | Void | - | - | - | - |
| 13 | Check: Has the UE (MCData client) downloaded test file 1 (TS 36.579-7 A.3.1)? (NOTE 1) | - | - | 2 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Test file 1 for CT FD as specified in annex A.3.1. |  |  |  |  |  |

6.2.10.3.3	Specific message contents

Table 6.2.10.3.3-1: SIP INVITE from the SS (step 1, Table 6.2.10.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1, condition MCDATA_FD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP message as described in Table 6.2.10.3.3-2 |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.10.3.3-3 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD SIGNALLING PAYLOAD as described in Table 6.2.10.3.3-3A |  |  |  |

Table 6.2.10.3.3-2: SDP for SIP INVITE (Table 6.2.10.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-3, condition MCDATA_FD, SDP_OFFER, MCD_1to1

Table 6.2.10.3.3-3: MCData-Info (Table 6.2.10.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3, condition MCD_1to1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "one-to-one-fd" |  |  |  |

Table 6.2.10.3.3-3A: FD SIGNALLING PAYLOAD (Table 6.2.10.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.6-1, condition FD_MSRP

Table 6.2.10.3.3-4: SIP 200 (OK) from the UE (step 1, Table 6.2.10.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3C.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.1-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| value | "application/sdp" |  |  |  |
| Message-body |  |  |  |  |
| SDP message | As described in Table 6.2.10.3.3-5 |  |  |  |

Table 6.2.10.3.3-5: SDP for SIP 200 (OK) (Table 6.2.10.3.3-4)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-3, condition MCDATA_FD, SDP_ANSWER

Table 6.2.10.3.3-6: MSRP SEND from the SS (step 6, Table 6.2.10.3.2-1;
step 1. TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-file" |  |  |  |
| data | As specified in table 6.2.10.3.3-8 |  |  |  |

Table 6.2.10.3.3-7: Void

Table 6.2.10.3.3-8: MCData Protected Payload Message (Table 6.2.9.3.3-6)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.10-2, condition PROTECTED_FILE, PCK

Table 6.2.10.3.3-9: SIP BYE from the SS (step 8, Table 6.2.10.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.7.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Reason |  |  |  |  |
| reason-value | "SIP" |  |  |  |
| protocol-cause | "cause="200"" |  |  |  |
| reason-text | "text="transmission succeeded"" |  |  |  |

Table 6.2.10.3.3-10: Void

Table 6.2.10.3.3-11: SIP MESSAGE from the UE (step 11, Table 6.2.10.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part | not present | MCData-Info |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | SDS NOTIFICATION as described in Table 6.2.10.3.3-12 |  |  |  |

Table 6.2.10.3.3-12: FD NOTIFICATION (Table 6.2.2.3.3-11)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.7-1, condition FD_COMPLETED

### 6.2.11	On-network / File Distribution (FD) / FD Using Media Plane / Group Standalone FD / Client Originated (CO)

6.2.11.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a group standalone FD message using the media plane}

then { UE (MCDATA Client) sends a request to establish an MSRP connection via a SIP INVITE message and then responds to the SIP 200 (OK) message with a SIP ACK message }

}

(2)

with { UE (MCDATA Client) having requested the establishment of a MSRP connection }

ensure that {

when { UE (MCDATA Client) receives a SIP 200 (OK) message with the a=setup attribute set to "passive"  from the SS (MCDATA server) }

then { UE (MCDATA Client) sends a blank MSRP SEND message to bind the MSRP connection and then sends the group standalone FD message via a MSRP SEND message }

}

(3)

with { UE (MCDATA Client) having sent a group standalone FD message using the media plane }

ensure that {

when { UE (MCDATA Client receives a MSRP 200 (OK) message in response to the last MSRP SEND message indicating that the standalone FD message has been successfully transferred }

then { UE (MCDATA Client) sends a SIP BYE message }

}

(4)

with { UE (MCDATA Client) having sent a group standalone FD message using the media plane with a disposition of "FILE DOWNLOAD COMPLETED UPDATE" }

ensure that {

when { UE (MCDATA Client receives a disposition response via a SIP MESSAGE message from the SS (MCDATA Server }

then { UE (MCDATA Client) responds to the SIP MESSAGE message by sending a SIP 200 (OK) message and delivers the notification to the MCDATA User }

}

6.2.11.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 10.2.5.2.3, 6.2.2.3, 12.2.1.2, TS 24.582 clause 7.1.2.1. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 10.2.5.2.3]

The MCData client shall generate a SIP INVITE request in accordance with 3GPP TS 24.229 [5] with the clarifications given below.

The MCData client:

1)	shall include the g.3gpp.mcdata.fd media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

2)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.fd media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

3)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

4)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP INVITE request;

5)	should include the "timer" option tag in the Supported header field;

6)	should include the Session-Expires header field according to IETF RFC 4028 [38]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

7)	shall generate and contain an application/vnd.3gpp.mcdata-signalling MIME body with the FD SIGNALLING PAYLOAD as described in subclause 6.2.2.3;

8)	if a one-to-one file distribution is requested:

a)	shall insert in the SIP INVITE request a MIME resource-lists body with the MCData ID of the invited MCData user, according to rules and procedures of IETF RFC 5366 [18]; and

b)	shall contain an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <request-type> element set to a value of "one-to-one-fd";

c)	if an end-to-end security context needs to be established and the security context does not exist or if the existing security context has expired, then:

i)	if necessary, shall instruct the key management client to request keying material from the key management server as described in 3GPP TS 33.180 [26];

ii)	shall use the keying material to generate a PCK as described in 3GPP TS 33.180 [26];

iii)	shall use the PCK to generate a PCK-ID with the four most significant bits set to "0001" to indicate that the purpose of the PCK is to protect one-to-one communications and with the remaining twenty eight bits being randomly generated as described in 3GPP TS 33.180 [26];

iv)	shall encrypt the PCK to a UID associated to the MCData client using the MCData ID of the invited user and a time related parameter as described in 3GPP TS 33.180 [26];

v)	shall generate a MIKEY-SAKKE I_MESSAGE using the encapsulated PCK and PCK-ID as specified in 3GPP TS 33.180 [26]; and

vi)	shall add the MCData ID of the originating MCData to the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26]; and

vii)	shall sign the MIKEY-SAKKE I_MESSAGE using the originating MCData user's signing key provided in the keying material together with a time related parameter, and add this to the MIKEY-SAKKE payload, as described in 3GPP TS 33.180 [26];

9)	if a group file distribution is requested:

a)	if the "/<x>/<x>/Common/MCData/AllowedFD" leaf node present in the group document of the requested MCData group, configured on the group management client as specified in 3GPP TS 24.483 [42] is set to "false", shall reject the request for FD and not continue with the rest of the steps in this subclause; and

b)	shall contain in an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <request-type> element set to a value of "group-fd";

ii)	the <mcdata-request-uri> element set to the MCData group identity; and

iii)	the <mcdata-client-id> element set to the MCData client ID of the originating MCData client;

NOTE 1:	The MCData client does not include the MCData ID of the originating MCData user in the body, as this will be inserted into the body of the SIP INVITE request that is sent from the originating participating MCData function.

10)	shall set the Request-URI of the SIP INVITE request to the public service identity identifying the participating MCData function serving the MCData user;

NOTE 2:	The MCData client is configured with public service identity identifying the participating MCData function serving the MCData user.

11)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [5];

12)	shall include an SDP offer according to 3GPP TS 24.229 [5] with the clarifications given in subclause 10.2.5.2.1; and

13)	shall send the SIP INVITE request towards the MCData server according to 3GPP TS 24.229 [5].

On receipt of a SIP 2xx response to the SIP INVITE request, the MCData client:

1)	shall send a SIP ACK request as specified in 3GPP TS 24.229 [5];

2)	shall start the SIP Session timer according to rules and procedures of IETF RFC 4028 [38]; and

3)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] subclause 10.2.5.1.1..

[TS 24.282, clause 6.2.2.3]

In order to generate an FD message, the MCData client:

1)	shall generate an FD SIGNALLING PAYLOAD message as specified in subclause 15.1.3; and

2)	shall include in the SIP request, the FD SIGNALLING PAYLOAD message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in subclause E.1.

When generating an FD SIGNALLING PAYLOAD message as specified in subclause 15.1.3, the MCData client:

1)	shall set the Date and time IE to the current time as specified in subclause 15.2.8;

2)	if the filestarts a new conversation, shall set the Conversation ID IE to a newly generated Conversation ID value as specified in subclause 15.2.9;

3)	if the file continues an existing conversation, shall set the Conversation ID IE to the Conversation ID value of the existing conversation as specified in subclause 15.2.9;

4)	shall set the Message ID IE to a newly generated Message ID value as specified in subclause 15.2.10;

5)	if the files in reply to a previously received SDS message or file, shall include the InReplyTo message ID IE with the Message ID value in the previously received SDS message or file;

6)	if the file is for user consumption, shall not include an Application ID IE as specified in subclause 15.2.7;

7)	if the file is intended for an application on the terminating MCData client, shall include an Application ID IE with an Application ID value representing the intended application as specified in subclause 15.2.7;

NOTE:	The value chosen for the Application ID value is decided by the mission critical organisation.

8)	if a file download complete notification is required shall include a FD disposition request type IE set to "FILE DOWNLOAD COMPLETED UPDATE" as specified in subclause 15.2.4; and

9)	shall include and set the Mandatory download IE to "MANDATORY DOWNLOAD" as described in subclause 15.2.16.

[TS 24.282, clause 12.2.1.2]

Upon receipt of a:

"SIP MESSAGE request for SDS disposition notification for terminating MCData client"; or

"SIP MESSAGE request for FD disposition notification for terminating MCData client";

the MCData client:

1)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2)	shall deliver the notification to the user or application.

[TS 24.582, clause 6.1.1.2.1]

Upon receiving an indication to establish MSRP connection for standalone SDS using media plane as the originating client, the MCData client:

1.	shall act as an MSRP client according to IETF RFC 6135 [12];

2.	shall act according to IETF RFC 6135 [12], as:

a.	an "active" endpoint, if a=setup attribute in the received SDP answer is set to "passive"; and

b.	an "passive" endpoint, if a=setup attribute in the received SDP answer is set to "active";

3.	shall establish the MSRP connection according to the MSRP connection parameters in the SDP answer received in the SIP 200 (OK) response according to IETF RFC 4975 [11]; and

4.	if acting as an "active" endpoint, shall send an empty MSRP SEND request to bind the MSRP connection to the MSRP session from the perspective of the passive endpoint according to the rules and procedures of IETF RFC 4975 [11] and IETF RFC 6135 [12].

On receiving MSRP 200 (OK) response to the first MSRP SEND request, the MCData client:

1.	shall generate a SDS SIGNALLING PAYLOAD as specified in subclause 6.1.1.2.2;

2.	shall generate a SDS DATA PAYLOAD as specified in subclause 6.1.1.2.3;

3.	shall include the SDS SIGNALLING PAYLOAD and SDS DATA PAYLOAD in an MSRP SEND request as specified in subclause 6.1.1.2.4; and

4.	shall send the MSRP SEND request on the established MSRP connection.

NOTE:	MSRP chunking, if needed, may affect the number of "Content Type" lines in each MSRP SEND message conveying a chunk, as also specified in subclause 6.1.1.2.4.

If MSRP chunking is not used then on receipt of a 200 (OK) response, the MCData client shall terminate the SIP session as specified in 3GPP TS 24.282 [8].

If MSRP chunking is used, the MCData client:

1.	shall send further MSRP SEND requests as necessary;

2.	shall wait for a 200 (OK) response to each MSRP SEND request sent; and

3.	on receipt of the last 200 (OK) response shall terminate the SIP session as specified in 3GPP TS 24.282 [8].

On receiving a non-200 MSRP response to the MSRP SEND request the MCData client shall handle the error as specified in IETF RFC 4975 [11]. To terminate the MSRP session, the MCData client:

1.	if there are further MSRP chunks to send, shall abort transmission of these further MSRP chunks;

2.	shall indicate to MCData user that the SDS message could not be sent; and

3.	shall terminate the SIP session as specified in 3GPP TS 24.282 [8].

On receiving an indication to terminate the session from the signalling plane, the MCData client:

1.	if there are further MSRP chunks to send, shall abort transmission of these further MSRP chunks and may indicate to MCData user that the SDS message could not be sent.

[TS 24.582, clause 7.1.2.1]

Upon receiving an indication to establish MSRP connection for file distribution as the originating client, the MCData client:

1.	shall act as an MSRP client according to IETF RFC 6135 [12];

2.	shall act according to IETF RFC 6135 [12], as:

a.	an "active" endpoint, if a=setup attribute in the received SDP answer is set to "passive"; and

b.	an "passive" endpoint, if a=setup attribute in the received SDP answer is set to "active";

3.	shall establish the MSRP connection according to the MSRP connection parameters in the SDP answer received in the SIP 200 (OK) response according to IETF RFC 4975 [11]; and

4.	if acting as an "active" endpoint, shall send an empty MSRP SEND request to bind the MSRP connection to the MSRP session from the perspective of the passive endpoint according to the rules and procedures of IETF RFC 4975 [11] and IETF RFC 6135 [12].

On receiving MSRP 200 (OK) response to the first MSRP SEND request, the MCData client can send the file. To send the file, the MCData client:

1.	shall generate MSRP SEND for file distribution request according to IETF RFC 4975 [11]. When generating an MSRP SEND, the MCData client:

a.	shall set To-Path header according to the MSRP URI(s) received in the answer SDP;

b.	shall set the first content type as Content-Type = "application/vnd.3gpp.mcdata-file"; and

c.	shall set the body of the MSRP SEND request with MSRP payload. MSRP payload is set to the file or part of the file.

2.	shall send the MSRP SEND request(s) on the established MSRP connection.

If MSRP chunking is used, the MCData client:

1.	shall send further MSRP SEND requests containing the file as necessary;

2.	shall wait for a 200 (OK) response to each MSRP SEND request sent; and

3.	on receipt of the last 200 (OK) response shall terminate the SIP session as specified in 3GPP TS 24.282 [8].

On receiving a non-200 MSRP response to the MSRP SEND request the MCData client shall handle the error as specified in IETF RFC 4975 [11]. To terminate the MSRP session, the MCData client:

1.	if there are further MSRP chunks to send, shall abort transmission of these further MSRP chunks;

2.	shall indicate to MCData user that the file could not be distributed; and

3.	shall terminate the SIP session as specified in 3GPP TS 24.282 [8].

6.2.11.3	Test description

6.2.11.3.1	Pre-test conditions

System Simulator:

-	SS (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCData client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

-	Test File 1 for CO FD as specified in annex A.2.1 is available at the UE for upload.

Preamble:

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCData Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.2.11.3.2	Test procedure sequence

Table 6.2.11.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) send test file 1 (TS 36.579-7 A.2.1) for CO group FD over media plane with disposition notification type "FILE DOWNLOAD COMPLETED UPDATE". (NOTE 1, NOTE 2) | - | - | - | - |
| 2 | Check: Does the UE (MCData client) correctly perform procedure 'CO MCData Call Establishment' as described in TS 36.579-1 [2] Table 5.3C.2.3-1? | - | - | 1,2 | P |
| 3-6 | Void | - | - | - | - |
| 7 | Check: Does the UE (MCData client) correctly perform procedure 'CO MSRP message transfer' as described in TS 36.579-1 [2] Table 5.3C.4.3-1 to send an FD message containing test file 1 for CO FD? | - | - | 2 | P |
| 7A | Check: Is the content of the transferred file the same as specified in annex A.2.1? | - | - | 2 | P |
| 8 | Check: Does the UE (MCData client) correctly perform procedure 'CO MCData call release' as described in TS 36.579-1 [2] Table 5.3C.6.3-1? | - | - | 3 | P |
| 9 | Void | - | - | - | - |
| 10 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive the disposition notification for the FD message sent at step 7? | - | - | 4 | P |
| 11 | Void | - | - | - | - |
| 12 | Check: Does the UE (MCData client) provide the disposition notification to the user? (NOTE 1) | - | - | 4 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Test file 1 for CO FD as specified in annex A.2.1. |  |  |  |  |  |

6.2.11.3.3	Specific message contents

Table 6.2.11.3.3-1: SIP INVITE from the UE (step 2, Table 6.2.11.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.1-1, condition MCDATA_FD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP message as described in Table 6.2.11.3.3-2 |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.11.3.3-3 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD SIGNALLING PAYLOAD as described in Table 6.2.11.3.3-3A |  |  |  |

Table 6.2.11.3.3-2: SDP for SIP INVITE (Table 6.2.11.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-3, condition MCDATA_FD, SDP_OFFER

Table 6.2.11.3.3-3: MCData-Info (Table 6.2.11.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "group-fd" |  |  |  |

Table 6.2.11.3.3-3A: FD SIGNALLING PAYLOAD (Table 6.2.11.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.5-1, condition FD_MSRP

Table 6.2.11.3.3-4: SIP 200 (OK) from the SS (step 2, Table 6.2.11.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP message | As described in Table 6.2.11.3.3-5 |  |  |  |

Table 6.2.11.3.3-5: SDP for SIP 200 (OK) (Table 6.2.11.3.3-4)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-3, condition MCDATA_FD, SDP_ANSWER

Table 6.2.11.3.3-6: MSRP SEND from the UE (step 7, Table 6.2.11.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-file" |  |  |  |
| data | As specified in table 6.2.11.3.3-7 |  |  |  |

Table 6.2.11.3.3-7: MCData Protected Payload Message (Table 6.2.11.3.3-6)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.10-1, condition PROTECTED_FILE, GMK

Table 6.2.11.3.3-8..9: Void

Table 6.2.11.3.3-10: SIP BYE from the UE (step 8, Table 6.2.11.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.6.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Reason |  |  | RFC 3326 [125] |  |
| reason-value | "SIP" |  |  |  |
| protocol-cause | "cause="200"" |  |  |  |
| reason-text | "text="transmission succeeded"" |  |  |  |

Table 6.2.11.3.3-11: Void

Table 6.2.11.3.3-13: SIP MESSAGE from the SS (step 10, Table 6.2.11.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | As described in Table 6.2.11.3.3-14 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.11.3.3-15 |  |  |  |

Table 6.2.11.3.3-14: MCData-Info (Table 6.2.11.3.3-13)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-group-id | Encrypted <mcdata-calling-group-id> with mcdataURI set to px_MCData_Group_A_ID | Encrypted according to TS 36.579-1 [2] Table 5.5.3.2.2-3A |  |  |

Table 6.2.11.3.3-15: FD NOTIFICATION (Table 6.2.11.3.3-13)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.8-1, condition FD_COMPLETED

Table 6.2.11.3.3-16: Void

### 6.2.12	On-network / File Distribution (FD) / FD Using Media Plane / Group Standalone FD / Client Terminated (CT)

6.2.12.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User receives a SIP INVITE to initiate a standalone group FD message using the media plane }

then { UE (MCDATA Client) responds by sending a SIP 200 (OK) message }

}

(2)

with { UE (MCDATA Client) having responded to the SIP INVITE message that initiated a standalone group FD message using the media plane }

ensure that {

when { UE (MCDATA Client) receives an MSRP SEND message from the SS (MCDATA Server) }

then { UE (MCDATA Client) responds with an MSRP 200 (OK) message }

}

(3)

with { UE (MCDATA Client) having finished receiving the file from the SS (MCDATA server) }

ensure that {

when { UE (MCDATA Client) receives a SIP BYE message to release communications }

then { UE (MCDATA Client) responds with a SIP 200 (OK) message and then sends a "FILE DOWNLOAD COMPLETED" disposition via a SIP MESSAGE message }

}

6.2.12.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 10.2.5.2.4, 12.2.1.1, 6.2.3.2, TS 24.582 clauses 7.1.3.1, 7.1.3.2. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 10.2.5.2.4]

Upon receipt of an initial SIP INVITE request, the MCData client shall follow the procedures for termination of multimedia sessions in the IM CN subsystem as specified in 3GPP TS 24.229 [5] with the clarifications below.

The MCData client:

1)	may reject the SIP INVITE request if either of the following conditions are met:

a)	MCData client does not have enough resources to handle the call; or

b)	any other reason outside the scope of this specification;

and skip the rest of the steps after step 2;

2)	if the SIP INVITE request is rejected in step 1), shall respond toward participating MCData function either with appropriate reject code as specified in 3GPP TS 24.229 [5] and warning texts as specified in subclause 4.9 or with SIP 480 (Temporarily unavailable) response not including warning texts if the user is authorised to restrict the reason for failure and skip the rest of the steps of this subclause;

3)	if the SDP offer of the SIP INVITE request contains an "a=key-mgmt" attribute field with a "mikey" attribute value containing a MIKEY-SAKKE I_MESSAGE:

a)	shall extract the MCData ID of the originating MCData user from the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26];

b)	shall convert the MCData ID to a UID as described in 3GPP TS 33.180 [26];

c)	shall use the UID to validate the signature of the MIKEY-SAKKE I_MESSAGE as described in 3GPP TS 33.180 [26];

d)	if authentication verification of the MIKEY-SAKKE I_MESSAGE fails, shall reject the SIP INVITE request with a SIP 488 (Not Acceptable Here) response as specified in IETF RFC 4567 [45], and include warning text set to "136 authentication of the MIKEY-SAKKE I_MESSAGE failed" in a Warning header field as specified in subclause 4.9 and not continue with rest of the steps in this subclause; and

e)	if the signature of the MIKEY-SAKKE I_MESSAGE was successfully validated:

i)	shall extract and decrypt the encapsulated PCK using the terminating user's (KMS provisioned) UID key as described in 3GPP TS 33.180 [26]; and

ii)	shall extract the PCK-ID, from the payload as specified in 3GPP TS 33.180 [26];

NOTE:	With the PCK successfully shared between the originating MCData client and the terminating MCData client, both clients are able to create an end-to-end secure session.

4)	may display to the MCData user the MCData ID of the inviting MCData user;

5)	may display to the MCData user the file meta-data of the incoming file as described by the SDP included in the received SIP INVITE request;

6)	if the Mandatory indication IE of the FD SIGNALLING PAYLOAD contained in the application/vnd.3gpp.mcdata-signalling MIME body received in the SIP INVITE request is set to "MANDATORY", then:

i)	shall accept the SIP INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5];

ii)	shall include the option tag "timer" in a Require header field of the SIP 200 (OK) response;

iii)	shall include the Session-Expires header field in the SIP 200 (OK) response and start the SIP session timer according to IETF RFC 4028 [38]. The "refresher" parameter in the Session-Expires header field shall be set to "uas";

iv)	shall include the g.3gpp.mcdata.fd media feature tag in the Contact header field of the SIP 200 (OK) response;

v)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" in the Contact header field of the SIP 200 (OK) response;

vi)	shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP INVITE request according to 3GPP TS 24.229 [5] with the clarifications given in subclause 10.2.5.2.2; and

vii)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5].

On receipt of an SIP ACK message to the sent SIP 200 (OK) message, the MCData client shall:

1)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] subclause 10.2.5.1.2.

On receipt of an indication from the media plane of the successful download of the file and if the received FD SIGNALLING PAYLOAD message contained an FD disposition request type IE requesting a file download completed update indication, then, the MCData client:

1)	shall follow the procedures described in subclause 12.2.1.1.

[TS 24.282, clause 12.2.1.1]

The MCData client shall follow the procedures in this subclause to:

-	indicate to an MCData client that an SDS message was delivered, read or delivered and read when the originating client requested a delivery, read or delivery and read report;

-	indicate to the participating MCData function serving the MCData user that an SDS message was undelivered. The participating MCData function can store the message for later re-delivery;

-	indicate to an MCData client that a request for FD was accepted, deferred or rejected; or

-	indicate to an MCData client that a file download has been completed;

Before sending a disposition notification the MCData client needs to determine:

-	the controlling MCData function that sent the SDS or FD message request. The MCData client determines the controlling MCData function from the contents of the <mcdata-controller-psi> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request;

-	the group identity related to an SDS or FD message request received as part of a group communication. The MCData client determines the group identity from the contents of the <mcdata-calling-group-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request; and

-	the MCData user targeted for the disposition notification. The MCData client determines the targeted MCData user from the contents of the <mcdata-calling-user-id> element contained in the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SDS or FD message request.

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	shall follow the rules specified in subclause 6.4 for the handling of MIME bodies in a SIP message when processing the remaining steps in this subclause;

3)	shall insert in the SIP MESSAGE request an application/resource-lists+xml MIME body containing the MCData ID of the targeted MCData user, according to rules and procedures of IETF RFC 5366 [18];

4)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with an <mcdata-controller-psi> element containing the PSI of the controlling MCData function;

5)	if sending a disposition notification in response to an MCData group data request, shall include an <mcdata-calling-group-id> element set to the MCData group identity in the application/vnd.3gpp.mcdata-info+xml MIME body;

6)	if requiring to send an SDS notification, shall generate an SDS NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.1;

7)	if requiring to send an FD notification, shall generate an FD NOTIFICATION message and include it in the SIP MESSAGE request as specified in subclause 6.2.3.2; and

8)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 6.2.3.2]

In order to generate an FD notification, the MCData client:

1)	shall generate an FD NOTIFICATION message as specified in subclause 15.1.6; and

2)	shall include in the SIP request, the FD NOTIFICATION message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in subclause E.1.

When generating an FD NOTIFICATION message as specified in subclause 15.1.6, the MCData client:

1)	if sending a file download accept notification, shall set the FD disposition notification type IE as "FILE DOWNLOAD REQUEST ACCEPTED" as specified in subclause 15.2.6;

2)	if sending a file download reject notification, shall set the FD disposition notification type IE as "FILE DOWNLOAD REQUEST REJECTED" as specified in subclause 15.2.6;

3)	if sending a file download deferred notification, shall set the FD disposition notification type IE as "FILE DOWNLOAD REQUEST DEFERRED" as specified in subclause 15.2.6;

4)	shall set the Conversation ID to the value of the Conversation ID that was received in the FD message as specified in subclause 15.2.9;

5)	shall set the Date and time IE to the current time as specified in subclause 15.2.8; and

6)	if sending a file download completed notification:

a)	shall set the FD disposition notification type IE as "FILE DOWNLOAD COMPLETED" as specified in subclause 15.2.6;

b)	shall set the Message ID to the value of the Message ID that was received in the FD message as specified in subclause 15.2.10;

c)	if the FD message was destined for the user, shall not include an Application ID IE as specified in subclause 15.2.7; and

d)	if the FD message was destined for an application, shall include an Application ID IE set to the value of the Application ID that was included in the FD message as specified in subclause 15.2.3.

[TS 24.582, clause 7.1.3.1]

Upon receiving an indication to establish MSRP connection for file distribution as the terminating client, the MCData client:

1.	shall act as an MSRP client according to IETF RFC 6135 [12];

2.	shall act either as an active endpoint or as an passive endpoint to open the transport connection, according to IETF RFC 6135 [12];

3.	shall establish the MSRP connection according to the MSRP connection parameters in the SDP offer received in the SIP INVITE request according to IETF RFC 4975 [11];

4.	if acting as an "active" endpoint, shall send an empty MSRP SEND request to bind the MSRP connection to the MSRP session from the perspective of the passive endpoint according to the rules and procedures of IETF RFC 4975 [11] and IETF RFC 6135 [12];

Once the MSRP session is established, the MCData client:

1.	on receipt of an MSRP request in the MSRP session, shall follow the rules and procedures defined in IETF RFC 4975 [11] and in IETF RFC 6714 [13];

2.	If an MSRP SEND request indicates the use of chunking, shall wait until all further MSRP SEND requests for the remaining chunks have been received and shall reassemble the entire set of MSRP requests into the file before delivering the content to the application; and

3.	shall handle the received content as described in subclause 7.1.3.2.

[TS 24.582, clause 7.1.3.2]

Upon receiving a file, the MCData client:

1.	shall decode the contents of the application/vnd.3gpp.mcdata-file MIME body; and

2.	once all the chunks of the file are successfully received, shall indicate to the signalling plane that the file download is completed.

6.2.12.3	Test description

6.2.12.3.1	Pre-test conditions

System Simulator:

-	SS (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCData client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

-	Test files downloaded or received at previous test runs are deleted.

Preamble:

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCData Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.2.12.3.2	Test procedure sequence

Table 6.2.12.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCData client) correctly perform procedure 'CT MCData Call Establishment' as described in TS 36.579-1 [2] Table 5.3C.3.3-1? | - | - | 1,2 | P |
| 2-5 | Void | - | - | - | - |
| 6 | Check: Does the UE (MCData client) correctly perform procedure 'CT MSRP message transfer' as described in TS 36.579-1 [2] Table 5.3C.5.3-1 to receive an FD message containing test file 1? (NOTE 2) | - | - | 2 | P |
| 7 | Void | - | - | - | - |
| 8 | Check: Does the UE (MCData client) correctly perform procedure 'CT MCData call release' as described in TS 36.579-1 [2] Table 5.3C.7.3-1? | - | - | 3 | P |
| 9-10 | Void | - | - | - | - |
| 11 | Check: Does the UE (MCData client) correctly perform procedure 'CO SDS or FD message transfer using signalling plane' as described in TS 36.579-1 [2] Table 5.3C.1.3-1 to send a disposition notification for the file received at step 6? | - | - | 2 | P |
| 12 | Void | - | - | - | - |
| 13 | Check: Has the UE (MCData client) downloaded test file 1 (TS 36.579-7 A.3.1)? (NOTE 1) | - | - | 2 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Test file 1 for CT FD as specified in annex A.3.1. |  |  |  |  |  |

6.2.12.3.3	Specific message contents

Table 6.2.12.3.3-1: SIP INVITE from the SS (step 1, Table 6.2.12.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1, condition MCDATA_FD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP message as described in Table 6.2.12.3.3-2 |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.12.3.3-3 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD SIGNALLING PAYLOAD as described in Table 6.2.12.3.3-3A |  |  |  |

Table 6.2.12.3.3-2: SDP for SIP INVITE (Table 6.2.12.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-3, condition MCDATA_FD, SDP_OFFER

Table 6.2.12.3.3-3: MCData-Info (Table 6.2.12.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3, condition MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "group-fd" |  |  |  |

Table 6.2.12.3.3-3A: FD SIGNALLING PAYLOAD (Table 6.2.12.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.6-1, condition FD_MSRP

Table 6.2.12.3.3-4: SIP 200 (OK) from the UE (step 1, Table 6.2.12.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3C.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.1-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| value | "application/sdp" |  |  |  |
| Message-body |  |  |  |  |
| SDP message | As described in Table 6.2.12.3.3-5 |  |  |  |

Table 6.2.12.3.3-5: SDP for SIP 200 (OK) (Table 6.2.12.3.3-4)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-3, condition MCDATA_FD, SDP_ANSWER

Table 6.2.12.3.3-6: MSRP SEND from the SS (step 6, Table 6.2.12.3.2-1;
step 1. TS 36.579-1 [2] Table 5.3C.5.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.12.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-file" |  |  |  |
| data | As specified in table 6.2.12.3.3-7 |  |  |  |

Table 6.2.12.3.3-7: MCData Protected Payload Message (Table 6.2.12.3.3-6)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.10-2, condition PROTECTED_FILE, GMK

Table 6.2.12.3.3-8: SIP BYE from the SS (step 8, Table 6.2.12.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3C.7.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Reason |  |  |  |  |
| reason-value | "SIP" |  |  |  |
| protocol-cause | "cause="200"" |  |  |  |
| reason-text | "text="transmission succeeded"" |  |  |  |

Table 6.2.12.3.3-9: Void

Table 6.2.12.3.3-10: SIP MESSAGE from the UE (step 11, Table 6.2.12.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3C.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, RESOURCE_LISTS, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.12.3.3-10A |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | SDS NOTIFICATION as described in Table 6.2.12.3.3-11 |  |  |  |

Table 6.2.12.3.3-10A: MCData-Info (Table 6.2.12.3.3-10)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-group-id | Encrypted <mcdata-request-uri> with mcdataURI set to px_MCData_Group_A_ID | Encrypted according to TS 36.579-1 [2] Table 5.5.3.2.1-3A |  |  |

Table 6.2.12.3.3-11: FD NOTIFICATION (Table 6.2.2.3.3-10)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.7-1, condition FD_COMPLETED

### 6.2.13	On-network / File Distribution (FD) / Accessing list of deferred data group communications / Client Originated (CO)

6.2.13.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a Group Standalone FD message with a non-mandatory download and with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" and the UE (MCDATA Client) is unaware of the URL of the Media Storage Function }

then { UE (MCDATA Client) sends a SIP MESSAGE to find the URL of the Media Storage Function and responds to a SIP MESSAGE that contains the URL of the Media Storage Function with a SIP 200 (OK) message }

}

(2)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a Group Standalone FD message with a non-mandatory download and with a disposition of "FILE DOWNLOAD COMPLETE UPDATE" and the UE (MCDATA Client) is aware of the URL of the Media Storage Function }

then { UE (MCDATA Client) uploads the file to the Media Storage Function via an HTTP POST message and then sends the URL of the file location to the recipient via a SIP MESSAGE message }

}

(3)

with { UE (MCDATA Client) having sent the URL of the file location to the recipient }

ensure that {

when { UE (MCDATA Client) receives a FD notification via a SIP MESSAGE message }

then { UE (MCDATA Client) responds to the SIP MESSAGE message with a SIP 200 (OK) message and delivers the notification to the MCDATA User }

}

(4)

with { UE (MCDATA Client) having received a notification that a sent message was deferred by the recipient }

ensure that {

when { MCData User requests to access the list of deferred group communication }

then { UE (MCDATA Client) sends a SIP MESSAGE message requesting to access the list of deferred communication and responds to a received SIP MESSAGE message with a SIP 200 (OK message and delivers the notification to the MCDATA User }

}

6.2.13.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 10.2.1.3.2, 10.2.2.1, 10.2.4.2.1, 12.2.1.2, 11.3.2.1, 11.3.2.2. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.282, clause 10.2.1.3.2]

To discover the absolute URI of the media storage function, the MCData client shall generate a SIP MESSAGE request towards the participating MCData function, in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	shall follow the rules specified in subclause 6.4 for the handling of MIME bodies in a SIP message when processing the remaining steps in this subclause;

3)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with a <request-type> element containing the value "msf-disc-req";

4)	if the upload of a file is for a group standalone FD request, shall include in an application/vnd.3gpp.mcdata-info+xml MIME body, the <mcdata-calling-group-id> element set to the required MCData group identity; and

NOTE 1:	The absence of a group identity in the <mcdata-calling-group-id> element of the application/vnd.3gpp.mcdata-info+xml MIME body implies that the MCData client intends to upload a file for a one-to-one FD request. In this case, the participating MCData function identifies the MCData ID of the user from the binding between the public user identity and the MCData ID.

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

On receipt of a "SIP MESSAGE request for absolute URI discovery response", the MCData client:

1)	shall store the absolute URI found in the <mcdata-controller-psi> element;

2)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5]; and

3)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 10.2.2.1]

If the media storage client is not aware of the absolute URI of the media storage function, the media storage client shall request the MCData client to discover the absolute URI associated with the media storage function by following the procedures in subclause 10.2.1.3.

The media storage client shall send HTTP requests over a TLS connection as specified for the HTTP client in the UE in annex A of 3GPP TS 24.482 [24].

NOTE 1:	The HTTP client encodes the MCData ID in the bearer access token of the Authorization header field of an HTTP request as specified in 3GPP TS 24.482 [24].

NOTE 2:	The HTTP client always sends the HTTP requests to an HTTP proxy. Annex A of 3GPP TS 24.482 [24] indicates how the HTTP proxy forwards the HTTP request to the HTTP server.

To upload a file to media storage function, the media storage client:

1)	shall generate an HTTP POST request as specified in IETF RFC 7230 [22] and IETF RFC 7231 [23];

2)	shall set the Request-URI to the absolute URI identifying the resource on a media storage function;

3)	shall set the Host header field to a hostname identifying the media storage function;

4)	shall set the Content-Type header field to multipart/mixed and with a boundary delimiter parameter set to any chosen value;

5)	if the file upload is for one-to-one file distribution, shall insert an application/vnd.3gpp.mcdata-info+xml MIME body with:

a)	the <request-type> element set to a value of "one-to-one-fd"; and

b)	the <mcdata-calling-user-id> element set to the originating MCData ID;

6)	if the file upload is for group file distribution, shall insert an application/vnd.3gpp.mcdata-info+xml MIME body with:

a)	the <request-type> element set to a value of "group-fd";

b)	the <mcdata-request-uri> element set to the MCData group identity; and

c)	the <mcdata-calling-user-id> element set to the originating MCData ID;

7)	if end-to-end security is required for a one-to-one communication, the MCData client protects the binary data representing the file and prefixes the protected binary data with security parameters as described in 3GPP TS 33.180 [26];

8)	if

i)	end-to-end security is not required for a one-to-one communication, or

ii)	the file upload is for group file distribution;

shall include the binary data representing the file with Content-Type field set to application/octet-stream and Content-Length field set to the file size; and

9)	shall send the HTTP POST request towards the media storage function.

On receipt of a HTTP 201 Created containing a Location header field with a URL identifying the location of the resource where the file has been stored on the media storage function, then the media storage client shall store this information.

[TS 24.282, clause 10.2.4.2.1]

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	if a one-to-one standalone FD message is to be sent shall insert in the SIP MESSAGE request:

a)	an application/resource-lists+xml MIME body with the MCData ID of the target MCData user, according to rules and procedures of IETF RFC 4826 [9]; and

b)	an application/vnd.3gpp.mcdata-info+xml MIME body with a <request-type> element set to a value of "one-to-one-fd";

…

4)	shall generate a standalone FD message as specified in subclause 6.2.2.2; and

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 12.2.1.2]

Upon receipt of a:

"SIP MESSAGE request for SDS disposition notification for terminating MCData client"; or

"SIP MESSAGE request for FD disposition notification for terminating MCData client";

the MCData client:

1)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body; and

2)	shall deliver the notification to the user or application.

[TS 24.282, clause 11.3.2.1]

Upon receiving a request from the MCData user to access the list of deferred data group communications, the MCData client:

1)	shall build the SIP MESSAGE request as specified in subclause 6.2.4.1;

2)	shall generate DEFERRED DATA REQUEST message as specified in subclause 15.1.11.1;

3)	shall include in the SIP request, the DEFERRED DATA GROUP COMM message in an application/vnd.3gpp.mcdata-signalling MIME body as specified in subclause E.1; and

4)	shall send the SIP MESSAGE request towards the participating MCData function according to rules and procedures of 3GPP TS 24.229 [5].

[TS 24.282, clause 11.3.2.2]

Upon receipt of a "SIP MESSAGE response for the list of deferred group communications request", the MCData client:

1)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5];

2)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5];

3)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body:

a)	if the application/vnd.3gpp.mcdata-signalling MIME body contains DEFERRED DATA RESPONSE message as specified in subclause 15.1.12:

i)	for each payload, if payload type is set to "FILEURL", shall store the payload data; and

4)	shall present to MCData user, the list of file URLs which were deferred.

6.2.13.3	Test description

6.2.13.3.1	Pre-test conditions

System Simulator:

-	SS (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCData client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

-	Test File 1 for CO FD as specified in annex A.2.1 is available at the UE for upload.

Preamble:

-	In the MCData Group Configuration document the <mcdata-on-network-max-data-size-auto-recv> shall be set to 0 to indicate non-mandatory download independent from the file size.

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCData Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.2.13.3.2	Test procedure sequence

Table 6.2.13.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) send test file 1 (TS 36.579-7 A.2.1) for CO group FD over HTTP for non-mandatory download and with disposition request "FILE DOWNLOAD COMPLETED UPDATE". (NOTE 1, NOTE 2) | - | - | - | - |
| 2 | Check: Does the UE (MCData client) correctly perform procedure 'Discovery of the absolute URI of the media storage function (one-to-one communication)' as described in TS 36.579-1 [2] Table 5.3C.9.3-1 ? | - | - | 1 | P |
| 3 | Check: Does the UE (MCData client) correctly perform procedure 'FD file upload using HTTP' as described in TS 36.579-1 [2] Table 5.3C.10.3-1? | - | - | 2 | P |
| 3A | Check: Is the content of the uploaded file the same as specified in annex A.2.1? | - | - | 2 | P |
| 4-6 | Void | - | - | - | - |
| 7 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to receive an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD DEFERRED" for the FD message sent at step 3? | - | - | 4 | P |
| 8 | Void | - | - | - | - |
| 9 | Check: Does the UE (MCData client) notify the user that the remote Client has deferred the acceptance of the download? (NOTE 1) | - | - | 3 | P |
| 10 | Make the UE (MCData client) access the list of deferred data group communications. (NOTE 1) | - | - | - | - |
| 11 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE Request - Accept CO' as described in TS 36.579-1 [2] Table 5.3.30.3-1 to retrieve the list of deferred data group communication? | - | - | 4 | P |
| 12-14 | Void | - | - | - | - |
| 15 | Check: Does the UE (MCData client) provide the list of file URLs which were deferred to the user? (NOTE 1) | - | - | 4 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Test file 1 for CO FD as specified in annex A.2.1. |  |  |  |  |  |

6.2.13.3.3	Specific message contents

Table 6.2.13.3.3-1..6: Void

Table 6.2.13.3.3-7: HTTP POST from the UE (step 3, Table 6.2.13.3.2-1; 
step 2, TS 36.579-1 [2] Table 5.3C.10.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.4.3-1, condition FD_HTTP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.1.3.3-8 |  |  |  |

Table 6.2.13.3.3-8: MCData-Info (Table 6.2.13.3.3-7)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "group-fd" |  |  |  |
| mcdata-request-uri | px_MCData_Group_A_ID | NOTE: the element is not encrypted |  |  |
| mcdata-calling-user-id | px_MCData_ID_User_A | NOTE: the element is not encrypted |  |  |

Table 6.2.13.3.3-9: Void

Table 6.2.13.3.3-10: HTTP 201 Created from the SS (step 3, Table 6.2.13.3.2-1; 
step 3, TS 36.579-1 [2] Table 5.3C.10.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.7-1, condition FD_HTTP

Table 6.2.13.3.3-11: SIP MESSAGE from the UE (step 3, Table 6.2.13.3.2-1; 
step 3, TS 36.579-1 [2] Table 5.3C.10.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.13.3.3-12 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD SIGNALLING PAYLOAD as described in Table 6.2.13.3.3-12A |  |  |  |

Table 6.2.13.3.3-12: MCData-Info (Table 6.2.13.3.3-11)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "group-fd" |  |  |  |

Table 6.2.13.3.3-12A: FD SIGNALLING PAYLOAD (Table 6.2.13.3.3-11)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.5-1, condition FD_HTTP

Table 6.2.13.3.3-13: SIP MESSAGE from the SS (step 7, Table 6.2.13.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA_FD, MCDATA_SIGNALLING |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.2.1.3.3-14 |  |  |  |
| MIME body part |  | MCData Data signalling message |  |  |
| MIME-part-body | MCData Protected Payload Message containing FD NOTIFICATION as described in Table 6.2.13.3.3-15 |  |  |  |

Table 6.2.13.3.3-14: MCData-Info (Table 6.2.13.3.3-13)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-group-id | Encrypted <mcdata-calling-group-id> with mcdataURI set to px_MCData_Group_A_ID | Encrypted according to TS 36.579-1 [2] Table 5.5.3.2.2-3A |  |  |

Table 6.2.13.3.3-15: FD NOTIFICATION (Table 6.2.13.3.3-13)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.8.8-1, condition FD_DEFERRED

Table 6.2.13.3.3-16: SIP MESSAGE from the UE (step 11, Table 6.2.13.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.30.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1, condition MCDATA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| Message-body |  |  |  |  |
| MCData Signalling message | MCData Protected Payload Message containing DEFERRED DATA REQUEST as described in Table 6.2.13.3.3-17 |  |  |  |

Table 6.2.13.3.3-17: DEFERRED DATA REQUEST (Table 6.2.13.3.3-16)

| Derivation Path: TS 24.282 [87], clause 15.1.11 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Deferred data request message identity | '00001011'B | Deferred List Access Request | TS 24.282 [87] clause 15.2.2 |  |

Table 6.2.13.3.3-18: SIP MESSAGE from the SS (step 11, Table 6.2.13.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3.30.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition MCDATA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/vnd.3gpp.mcdata-signalling" |  |  |  |
| Message-body |  |  |  |  |
| MCData Signalling message | MCData Protected Payload Message containing DEFERRED DATA RESPONSE as described in Table 6.2.13.3.3-19 |  |  |  |

Table 6.2.13.3.3-19: DEFERRED DATA RESPONSE (Table 6.2.13.3.3-18)

| Derivation Path: TS 24.282 [87], clause 15.1.12 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Deferred data response message identity | '00001100'B | Deferred List Access Response | TS 24.282 [87] clause 15.2.2 |  |
| Number of payloads | "1" |  | TS 24.282 [87] clause 15.2.12 |  |
| Security parameters and Payload | Not present |  |  |  |
| Payload |  |  | TS 24.282 [87] clauses 11.3.3.2 and 15.2.13 |  |
| Length of Payload contents | Length of the payload contents |  |  |  |
| Payload content type | “00000100” | FILEURL |  |  |
| Payload contents | same URI as assigned by the HTTP 201 (Created) at step 3 |  |  |  |
| MCData group ID | Not present | Rel-17 | TS 24.282 [87] clause 15.2.14 |  |
| Deferred FD signalling payload | Not present | Rel-17 | TS 24.282 [87] clause 15.2.27 |  |