3GPP TSG-RAN5 Meeting #98	R5-230196

# Athens, Greece, 27 February – 3 March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1517 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Spurious emissions band UE co-existence Rel-16 Test requirements for DC_8A_n94A_ULSUP-TDM and DC_20A_n92A_ULSUP-TDM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Inter-band EN-DC two band configurations DC_8A_n94A_ULSUP-TDM and DC_20A_n92A_ULSUP-TDM are specified as UL configuration in 38.101-3 Rel-16 but missing in 38.521-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add requirements for inter-band EN-DC configurations DC_8A_n94A_ULSUP-TDM and DC_20A_n92A_ULSUP-TDM: - Update 6.5B.3.3 Spurious emissions - Table 6.5B.3.3.2.5-2: Test requirements for Rel-16 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spurious emissions requirements for inter-band EN-DC configurations DC_8A_n94A_ULSUP-TDM and DC_20A_n92A_ULSUP-TDM will not be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.3.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0721, 0722 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on RAN4 CRs R4-2300401, R4-2300402, R4-2300403 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

6.5B.3.3.2.5	Test Requirement

The test requirements are in Table 6.5B.3.3.2.5-1 and Table 6.5B.3.3.2.5-2 for Release 15 and Release 16, respectively. For EN-DC only capable devices, in addition to Table 6.5B.3.3.2.5-1 and Table 6.5B.3.3.2.5-2, the test requirements as in clause 6.5.3.2.5 in TS 38.521-1 [8] are also needed.

For inter-band EN_DC with the uplink assigned to one LTE band and one NR band, the test requirements in Table 6.5B.3.3.2.5-1 and Table 6.5B.3.3.2.5-2 can be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur.

-------- Skipped unchanged clauses --------

Table 6.5B.3.3.2.5-2: Requirements for inter-band within FR1 for Rel-16

| EN-DC Configuration |  | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band | Frequency range (MHz) |  |  |  |  |  |  |  |  | Maximum Level (dBm) |  |  |  | MBW (MHz) |  |  |  | NOTE |  |  |  |
| DC_1_n3 |  | E-UTRA Band 1, 11, 21, 32, 43, 50, 65, 74, 75 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR Band 77, 78 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_1_n5 |  | E-UTRA Band 22, 42, 43 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR Band n77, n78, n79 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_1_n7 |  | E-UTRA Band 32, 50, 51, 75, 76 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR Band n79 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_1_n8 |  | E-UTRA Band 43 NR band n77, n78, n79 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_1_n28 |  | E-UTRA Band 7, 31, 41, 72, 73 NR Band n79 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 42, 52 NR Band n77, n78 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | Frequency range | 470 |  |  |  | - |  |  | 694 |  | -42 |  |  |  | 8 |  |  |  | 5, 17 |  |  |  |
|  |  | Frequency range | 470 |  |  |  | - |  |  | 710 |  | -26.2 |  |  |  | 6 |  |  |  | 14 |  |  |  |
| DC_1_n77 |  | E-UTRA Band 1, 3, 11, 21, 34, 65, 74 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range | 1880 |  |  |  | - |  |  | 1895 |  | -40 |  |  |  | 1 |  |  |  | 5, 8 |  |  |  |
|  |  | Frequency range | 1895 |  |  |  | - |  |  | 1915 |  | -15.5 |  |  |  | 5 |  |  |  | 5, 7, 8 |  |  |  |
|  |  | Frequency range | 1915 |  |  |  | - |  |  | 1920 |  | 1.6 |  |  |  | 5 |  |  |  | 5, 7, 8 |  |  |  |
| DC_1_n78 |  | E-UTRA Band 3, 11, 21, 74 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range | 1880 |  |  |  | - |  |  | 1895 |  | -40 |  |  |  | 1 |  |  |  | 5, 8 |  |  |  |
| DC_1_n79 |  | E-UTRA Band 5, 7, 8, 18, 19, 26, 28, 41 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_2_n5 |  | E-UTRA Band 42, 48 NR band n77 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 41, 43 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_2_n41 |  | E-UTRA Band 12, 13, 14, 17, 28, 29, 42, 71 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Bands 2 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 5 |  |  |  |
| DC_2_n66 |  | E-UTRA Band 4, 10, 24, 50, 66, 70, 74 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 2, 25 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 5 |  |  |  |
|  |  | E-UTRA Band 42, 48 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_2_n71 |  | E-UTRA Band 41 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_2_n78 |  | E-UTRA Band 24, 50, 51, 74 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 2, 25 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_3_n1 |  | E-UTRA Band 1, 11, 21, 32, 50, 65, 74, 75 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_3_n5 |  | E-UTRA Band 5, 7, 8, 18,19, 26, 38 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR Band n79 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 42, 52 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | NR Band n77, n78 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_3_n7 |  | E-UTRA Band 5, 8, 20, 26, 27, 28, 44, 67 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 42 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_3_n28 |  | NR band n77 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | E-UTRA Band 7, 41 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_3_n41 |  | E-UTRA Band 5, 8, 20, 26, 27, 28, 44 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 18, 19 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 14, 20 |  |  |  |
|  |  | E-UTRA Band 42, NR Band n77, n78, n79 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_3_n77 |  | E-UTRA Band 1, 3, 28, 34, 39, 40, 65, 74 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range | 1884.5 |  |  |  | - |  |  | 1915.7 |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_3_n78 |  | E-UTRA Band 3, 34, 39 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range | 1884.5 |  |  |  | - |  |  | 1915.7 |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_3_n79 |  | E-UTRA Band 5, 8, 11, 18, 19, 21, 41, 74 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_5_n2 |  | E-UTRA Band 42 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 41, 43 NR Band n77 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_5_n66 |  | E-UTRA Band 5, 6, 7, 8, 38 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 26 | 859 |  |  |  | - |  |  | 869 |  | -27 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 41, 42, 52 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | NR Band n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5_n78 |  | E-UTRA Band 1, 2, 3, 4, 7, 25, 34, 38, 65, 66, 70 |  | FDL_low |  |  |  | - |  |  | FDL_high |  |  | -50 |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 41 |  | FDL_low |  |  |  | - |  |  | FDL_high |  |  | -50 |  |  | 1 |  |  |  | 2, 7 |  |  |  |
| DC_7_n1 |  | E-UTRA Band 32, 50, 51, 75, 76 or NR Band n79 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_7_n3 |  | E-UTRA Band 5, 8, 20, 26, 27, 28, 67, 68 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA band 42, 52 NR Band n77, n78 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_7_n5 |  | E-UTRA Band 5, 26, 28, 42 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 52 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | NR Band n77, n78 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_7_n8 |  | E-UTRA Band 20, 28, 67, 68 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 42, 52 NR band n77, n78 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_7_n28 |  | E-UTRA Band 3 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR band n78 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_7_n66 |  | E-UTRA Band 5, 12, 13, 14, 17, 26, 27, 28, 29, 85 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA band 42 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_7_n78 |  | E-UTRA Band 3, 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 32, 50, 51, 67, 68, 74, 75, 76 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_8_n1 |  | E-UTRA Band 43 or NR Band n77, n78, n79 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_8_n3 |  | E-UTRA Band 20, 28, 38, 44 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 7, 22, 41, 42, 43 or NR Band n77, n78, n79 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | Frequency range | 860 |  |  |  | - |  |  | 890 |  | -40 |  |  |  | 1 |  |  |  | 5, 12 |  |  |  |
| DC_8_n20 |  | E-UTRA Band 67, 68 | 749 |  |  |  | - |  |  | 783 |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 7, 38, 69 | 2570 |  |  |  | - |  |  | 2690 |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | E-UTRA Band 8, 20 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 5 |  |  |  |
|  |  | Frequency range | 758 |  |  |  | - |  |  | 788 |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_8_n28 |  | E-UTRA Band 38, 40 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 41 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | Frequency range | 491 |  |  |  | - |  |  | 616 |  | -42 |  |  |  | 8 |  |  |  | 5, 17 |  |  |  |
|  |  | Frequency range | 491 |  |  |  | - |  |  | 616 |  | -26.2 |  |  |  | 6 |  |  |  | 14 |  |  |  |
| DC_8_n41 |  | E-UTRA Band 28 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA band 3, 42, 52 NR Band n77, n78, n79 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | E-UTRA Band 8 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 5 |  |  |  |
|  |  | Frequency range | 860 |  |  |  | - |  |  | 890 |  | -40 |  |  |  | 1 |  |  |  | 5, 12 |  |  |  |
| DC_8_n77 |  | E-UTRA Band 1, 32, 33, 34, 38, 39, 40,50, 65, 69, 74, 75 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA band 3, 7, 41 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | E-UTRA Band 11, 21 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 12 |  |  |  |
|  |  | Frequency range | 1884.5 |  |  |  | - |  |  | 1915.7 |  | -41 |  |  |  | 0.3 |  |  |  | 3, 12 |  |  |  |
| DC_8_n78 |  | E-UTRA band 34, 39, 40, 74 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 3, 7, 41 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | E-UTRA Band 11, 21 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 12 |  |  |  |
|  |  | Frequency range | 1884.5 |  |  |  | - |  |  | 1915.7 |  | -41 |  |  |  | 0.3 |  |  |  | 3, 12 |  |  |  |
| DC_8_n94_ULSUP-TDM |  | E-UTRA Band 3, 7, 41 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2, 5 |  |  |  |
|  |  | E-UTRA Band 8 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_11_n77 | E-UTRA Band 1, 3, 18, 19, 28, 34, 65 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | Frequency range |  |  |  | 945 | - |  |  | 960 |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | Frequency range |  |  |  | 1884.5 | - |  |  | 1915.7 |  |  |  | -41 |  | 0.3 |  |  |  | 3 |  |  |  |
|  | Frequency range |  |  |  | 2545 | - |  |  | 2575 |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | Frequency range |  |  |  | 2595 | - |  |  | 2645 |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
| DC_11_n78 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | Frequency range |  |  |  | 1884.5 | - |  |  | 1915.7 |  |  |  | -41 |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_11_n79 | E-UTRA Band 1, 3, 34, 42, 65 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | Frequency range |  |  |  | 1884.5 | - |  |  | 1915.7 |  |  |  | -41 |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_12_n78 | E-UTRA Band 2, 7, 25, 41 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | E-UTRA Band 4, 66 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  | 2 |  |  |  |
|  | Frequency range |  |  |  | 1884.5 | - |  |  | 1915.7 |  |  |  | -41 |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_13_n2 | E-UTRA Band 41 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
| DC_13_n66 | E-UTRA Band 41, 53 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | NR Band n77 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  | 2 |  |  |  |
| DC_14_n2 | E-UTRA Band 41 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | NR Band n77 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
| DC_14_n66 | E-UTRA Band 41 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | NR Band n77 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
| DC_19_n77 | E-UTRA Band 1, 3, 34, 40, 65 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  |  |  |  | 1 |  |  |  |  |
|  | Frequency range |  |  |  | 1884.5 | - |  |  | 1915.7 |  |  |  | -41 |  |  |  |  | 0.3 |  |  |  | 3 |
|  | Frequency range |  |  |  | 2545 | - |  |  | 2575 |  |  |  | -50 |  |  |  |  | 1 |  |  |  |  |
|  | Frequency range |  |  |  | 2595 | - |  |  | 2645 |  |  |  | -50 |  |  |  |  | 1 |  |  |  |  |
| DC_19_n78 | E-UTRA Band 1, 3, 34, 65 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | Frequency range |  |  |  | 1884.5 | - |  |  | 1915.7 |  |  |  | -41 |  | 0.3 |  |  |  | 3 |  |  |  |
|  | Frequency range |  |  |  | 2545 | - |  |  | 2575 |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | Frequency range |  |  |  | 2595 | - |  |  | 2645 |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
| DC_19_n79 | E-UTRA Band 42 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  |  |  |  | 1 |  |  |  |  |
| DC_20_n1 | E-UTRA Band 22, 42, 43 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | NR Band 77, 78 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  | 2 |  |  |  |
| DC_20_n3 | E-UTRA Band 7, 8 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | E-UTRA Band 38, 42, 52 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  | 2 |  |  |  |
| DC_20_n7 | E-UTRA Band 68 |  |  |  | 776 | - |  |  | 783 |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | E-UTRA Band 42, 52 |  |  |  | 3332 | - |  |  | 3432 |  |  |  | -50 |  | 1 |  |  |  | 2 |  |  |  |
|  | NR band n77, n78 |  |  |  | 3332 | - |  |  | 3432 |  |  |  | -50 |  | 1 |  |  |  | 2 |  |  |  |
|  | E-UTRA Band 20 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  | 5 |  |  |  |
| DC_20_n8 | E-UTRA Band 28 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | E-UTRA Band 7, 38 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  | 2 |  |  |  |
| DC_20_n28 | E-UTRA Band 8 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
| DC_20_n78 | E-UTRA Band 1, 3, 7, 33, 34, 65 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  |  |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 38, 69 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  |  |  |  | 1 |  |  |  | 2 |
| DC_20_n92_ULSUP-TDM | E-UTRA Band 20 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  |  |  |  | 1 |  |  |  | 5 |
|  | E-UTRA Band 69 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  |  |  |  | 1 |  |  |  | 2 |
| DC_21_n77 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  |  |  |  | 1 |  |  |  |  |
|  | Frequency range |  |  |  | 945 | - |  |  | 960 |  |  |  | -50 |  |  |  |  | 1 |  |  |  |  |
|  | Frequency range |  |  |  | 1884.5 | - |  |  | 1915.7 |  |  |  | -41 |  |  |  |  | 0.3 |  |  |  | 3 |
|  | Frequency range |  |  |  | 2545 | - |  |  | 2575 |  |  |  | -50 |  |  |  |  | 1 |  |  |  |  |
|  | Frequency range |  |  |  | 2595 | - |  |  | 2645 |  |  |  | -50 |  |  |  |  | 1 |  |  |  |  |
| DC_21_n78 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  |  |  |  | 1 |  |  |  |  |
|  | Frequency range |  |  |  | 1884.5 | - |  |  | 1915.7 |  |  |  | -41 |  |  |  |  | 0.3 |  |  |  | 3 |
| DC_21_n79 | E-UTRA Band 3, 21, 34, 42 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  |  |  |  | 1 |  |  |  |  |
|  | Frequency range |  |  |  | 1884.5 | - |  |  | 1915.7 |  |  |  | -41 |  |  |  |  | 0.3 |  |  |  | 3 |
| DC_25_n41 | E-UTRA Band 12, 13, 14, 17, 28, 29, 42, 71 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | NR Band n77 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  | 2 |  |  |  |
| DC_26_n41 | E-UTRA Band 3, 5, 18, 19, 26, 42 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | Frequency range |  |  |  | 799 | - |  |  | 803 |  |  |  | -40 |  | 1 |  |  |  | 5 |  |  |  |
|  | Frequency range |  |  |  | 945 | - |  |  | 960 |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
| DC_26_n77 | E-UTRA Band 1, 3, 34, 39, 40, 65 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | E-UTRA Band 41 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  | 2 |  |  |  |
|  | Frequency range |  |  |  | 1884.5 | - |  |  | 1915.7 |  |  |  | -41 |  | 0.3 |  |  |  | 3 |  |  |  |
|  | Frequency range |  |  |  | 2545 | - |  |  | 2575 |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | Frequency range |  |  |  | 2595 | - |  |  | 2645 |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
| DC_26_n78 | E-UTRA Band 1, 3, 34, 39, 65 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | E-UTRA Band 41 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  | 2 |  |  |  |
|  | Frequency range |  |  |  | 1884.5 | - |  |  | 1915.7 |  |  |  | -41 |  | 0.3 |  |  |  | 3 |  |  |  |
|  | Frequency range |  |  |  | 2545 | - |  |  | 2575 |  |  |  | -50 |  | 1 |  |  |  | 2 |  |  |  |
|  | Frequency range |  |  |  | 2595 | - |  |  | 2645 |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
| DC_28_n3 | NR Band n77 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  | 2 |  |  |  |
|  | E-UTRA Band 7, 41 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
| DC_28_n5 | E-UTRA Band 8, 40, 71 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | E-UTRA Band 24 |  |  |  | 1527 | - |  |  | 1559 |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | E-UTRA Band 30 |  |  |  | 2351 | - |  |  | 2360 |  |  |  | -50 |  | 1 |  |  |  |  |  |  |  |
|  | Frequency range |  |  |  | 557 | - |  |  | 672 |  |  |  | -42 |  | 8 |  |  |  | 5, 17 |  |  |  |
|  | Frequency range |  |  |  | 557 | - |  |  | 672 |  |  |  | -26.2 |  | 6 |  |  |  | 14 |  |  |  |
|  | Frequency range |  |  |  | 662 | - |  |  | 672 |  |  |  | -26.2 |  | 6 |  |  |  | 5 |  |  |  |
| DC_28_n7 | E-UTRA Band 3 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  |  |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 52 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  |  |  |  | 1 |  |  |  | 2 |
|  | NR band n77, n78 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  |  |  |  | 1 |  |  |  | 2 |
| DC_28_n77 | E-UTRA Band 3, 7, 34, 39, 40, 41 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  |  |  |  | 1 |  |  |  |  |
|  | E-UTRA Band 1, 65 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  |  |  |  | 1 |  |  |  | 2 |
|  | E-UTRA Band 1 |  |  |  | FDL_low | - |  |  | FDL_high |  |  |  | -50 |  |  |  |  | 1 |  |  |  | 9, 11 |
|  | Frequency range |  |  |  | 1884.5 | - |  |  | 1915.7 |  |  |  | -41 |  |  |  |  | 0.3 |  |  |  | 3, 9 |
| DC_28_n78 DC_28_n83_ULSUP-TDM_n78 |  | E-UTRA Band 3, 7, 34, 39, 40, 41 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 1, 65 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | E-UTRA Band 1 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 9, 11 |  |  |  |
|  |  | Frequency range | 1884.5 |  |  |  | - |  |  | 1915.7 |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_28_n79 |  | E-UTRA Band 42 |  | FDL_low |  |  |  | - |  |  | FDL_high |  |  | -50 |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_30_n5 |  | E-UTRA Band 71 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR band n77 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_39_n41 |  | E-UTRA Band 42, 44 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR Band n77, n78, n79 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_39_n79 |  | E-UTRA Band 8, 28, 41, 44 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_40_n1 |  | E-UTRA Band 7, 31, 32, 41, 45, 50, 72, 73, 74, 75 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_40_n41 |  | Bands 1, 34, 39, 65 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR Band n79 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  | Frequency range | 1884.5 |  |  |  | - |  |  | 1915.7 |  | -41 |  |  |  | 0.3 |  |  |  | 3, 19 |  |  |  |
| DC_40_n78 |  | E-UTRA Band 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 32, 44, 45, 50, 51, 74, 75, 76 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | NR Band n79 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_40_n79 |  | E-UTRA Band 1, 34, 41, 65 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_41_n77 |  | E-UTRA Band 3, 5, 8, 11, 18, 19, 21, 26, 28, 33, 34, 39, 44, 45, 74 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range | 1884.5 |  |  |  |  |  |  | 1915.7 |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_41_n78 |  | E-UTRA Band 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 44, 45, 74 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range | 1884.5 |  |  |  | - |  |  | 1915.7 |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_41_n79 |  | E-UTRA Band 1, 3, 5, 8, 9, 18, 19, 26, 28, 34, 40, 44, 65 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range | 1884.5 |  |  |  | - |  |  | 1915.7 |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_48_n5 |  | E-UTRA Band 25, 70 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | Frequency range | 1884.5 |  |  |  | - |  |  | 1915.7 |  | -41 |  |  |  | 0.3 |  |  |  | 3 |  |  |  |
| DC_48_n66 |  | E-UTRA Band 2, 25 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| DC_66_n2 |  | E-UTRA Band 4, 10, 22, 24, 50, 66, 70, 74 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 2, 25 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 5 |  |  |  |
|  |  | E-UTRA Band 42, 43 NR Band n77 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_66_n5 |  | E-UTRA Band 5, 6, 7, 8, 26, 38 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 41, 42, 52 NR Band n77 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_66_n41 |  | E-UTRA Band 5, 12, 13, 14, 17, 26, 27, 28, 29, 43, 85 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
|  |  | E-UTRA Band 42, 48, NR band n77 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| DC_66_n78 |  | E-UTRA Band 3, 34, 39 | FDL_low |  |  |  | - |  |  | FDL_high |  | -50 |  |  |  | 1 |  |  |  |  |  |  |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 in TS 36.101 [5] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [5] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 4: Void. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [5] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 19: Void. NOTE 20: Void. NOTE 21: Void. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- End of changes --------