3GPP TSG RAN WG5#98	R5-230251

# Athens, February 26  - March 3 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2073 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of CA_n41A-n66A, RIB,c and sensitivity exception. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New 2RX CA configuration introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of ΔRIB,c in Table 7.3A.0.3.2.1-1 Addition of sensitivity exception due to CBI in new table Table 7.3A.0.6-3. Old tables does not comply with new tables in 38.101! Cleanup of old CBI configuration CA_n71A-n29A in clause 7.3A.0.6, due to RAN 4 changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Tables not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.0.3.2.1, 7.3A.0.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Dependant on CR 230252 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### << Beginning of changes>>

7.3A.0.3.2.1	ΔRIB,c for two bands

Table 7.3A.0.3.2.1-1: ΔRIB,c due to CA (two bands)

| Inter-band CA configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1-n77 | n1 | 0.2 |
|  | n77 | 0.5 |
| CA_n1-n78 | n78 | 0.5 |
| CA_n2-n48 | n2 | 0.2 |
|  | n48 | 0.5 |
| CA_n2-n66 | n2 | 0.3 |
|  | n66 | 0.3 |
| CA_n2-n77 | n2 | 0.2 |
|  | n77 | 0.5 |
| CA_n3-n41 | n41 | 02 |
|  |  | 0.53 |
| CA_n3-n78 | n3 | 0.2 |
|  | n78 | 0.5 |
| CA_n5-n77 | n5 | 0.2 |
|  | n77 | 0.5 |
| CA_n5-n78 | n5 | 0.2 |
|  | n78 | 0.5 |
| CA_n7-n78 | n7 | 0.5 |
|  | n78 | 0.5 |
| CA_n8-n78 | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n24-n48 | n24 | 0.2 |
|  | n48 | 0.5 |
| CA_n24-n77 | n24 | 0.2 |
|  | n77 | 0.5 |
| CA_n28-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n28-n79 | n28 | 0.2 |
|  | n79 | 0.5 |
| CA_n41-n66 | n41 | 0.54 |
|  |  | 15 |
|  | n66 | 0.5 |
| CA_n41-n79 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n48-n66 | n48 | 0.5 |
|  | n66 | 0.2 |
| CA_n48-n70 | n48 | 0.5 |
|  | n70 | 0.2 |
| CA_n66A-n77A | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n71-n77 | n71 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: FFS. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. |  |  |

##### << Unchanged clauses omitted>>

#### 7.3A.0.6	Reference sensitivity exceptions due to cross band isolation for CA

Editor’s Note: Table 7.3A.0.6-1 and  Table 7.3A.0.6-2 format is different from 38.101-1 (V17.7.0) Table 7.3A.6-1. The old format will exist until RAN5 has final solutions on how to adopt RAN4 corresponding table for the minimum requirement of Reference sensitivity exceptions due to CBI, and the resulted possible new format of Table 7.3A.1_1.4.1-1: Test Configuration Table for inter-band 2DL CA exceptions, and Table 7.3A.1_1.5-1: Reference sensitivity requirement for inter band CA.  Table 7.3A.0.6-3 is added to be used for additions of new band configurations.

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part of the same NR CA configuration due to cross band isolation issues. Reference sensitivity exceptions for the victim band are specified in Table 7.3A.0.6-1 with uplink configuration of the aggressor band specified in Table 7.3A.0.6-2.

Table 7.3A.0.6-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n1 | n3 | 3 | 2.2 | 1.9 | 1.7 | 1.6 | 1.5 | 1.4 |  |  |  |  |  |  |
| n3 | n41 |  | 0.7 | 0.7 | 0.7 |  |  | 0.7 | 0.7 | 0.7 |  | 0.7 | 0.7 | 0.7 |
| n41 | n3 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |
| n78 | n71 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |
| n78 | n79 |  |  |  |  |  |  | 2 | 2 | 2 |  | 2 |  | 2 |
| n79 | n78 |  | 2.6 | 2.6 | 2.6 |  |  | 2.6 | 2.6 | 2.6 |  | 2.6 | 2.6 | 2.6 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.0.6-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n3 | 15 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |
| n3 | n41 | 15 |  | 50 | 50 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n41 | n3 | 30 | 160 | 160 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |
| n78 | n7 | 30 | 270 | 270 | 270 | 270 | 270 | 270 | 270 | 270 |  |  |  |  |  |
| n78 | n79 | 30 |  |  |  |  |  |  | 2702 | 2702 | 2702 |  | 2702 |  | 2702 |
| n79 | n78 | 30 |  | 2702 | 2702 | 2702 |  |  | 2702 | 2702 | 2702 |  | 2702 | 2702 | 2702 |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2.3-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2.3-3 applies. NOTE 2: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.0.6-3: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n411 | n66 | 2521 | 50 | 30 | 128 (RBstart=0) | 2197.5 | 5 | 3.5 | >ACLR2 |
| n71 | n29 | 688 | 20 | 15 | 20 (RBstart=86) | 719.5 | 5 | 17.5 | ACLR2 |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2.3-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2.3-3 applies. NOTE 2: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3.A.

##### << End of changes>>