3GPP TSG-RAN5 Meeting #98	R5-230405

# Athens, Greece, Feb 27th - Mar 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-2 | CR | 1391 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NTN freq bands TC A.4.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NTN frequency bands need to be added into TC A.4.3.1 RF Baseline Implementation Capabilities as per WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NTN frequency bands has been added into TC A.4.3.1 RF Baseline Implementation Capabilities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work item cannot be finalized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

4.3.1	RF Baseline Implementation Capabilities

NOTE:	The values indicated in column "Release" in tables A.4.3.1-1 and A.4.3.1-2 below are to be understood as the specifications release version in which a band was introduced and not as a mandate that a UE conforming to particular release shall support a particular band. For further guidance to release independent bands see TS 36.307 [30].

Table A.4.3.1-1: FDD RF Baseline Implementation Capabilities

| Item | FDD (DS) RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Frequency band: 1920-1980, 2110-2170 MHz | 36.101, 5.5 | Rel-8 | pc_eBand1_Supp | Band 1 |
| 2 | Frequency band: 1850-1910, 1930-1990 MHz | 36.101, 5.5 | Rel-8 | pc_eBand2_Supp | Band 2 |
| 3 | Frequency band: 1710-1785, 1805-1880 MHz | 36.101, 5.5 | Rel-8 | pc_eBand3_Supp | Band 3 |
| 4 | Frequency band: 1710-1755, 2110-2155 MHz | 36.101, 5.5 | Rel8 | pc_eBand4_Supp | Band 4 |
| 5 | Frequency band: 824-849, 869-894 MHz | 36.101, 5.5 | Rel-8 | pc_eBand5_Supp | Band 5 |
| 6 | Frequency band: 830-840, 875-885 MHz | 36.101, 5.5 | Rel-8 | pc_eBand6_Supp | Band 6 |
| 7 | Frequency band: 2500-2570, 2620-2690 MHz | 36.101, 5.5 | Rel-8 | pc_eBand7_Supp | Band 7 |
| 8 | Frequency band: 880-915, 925-960 MHz | 36.101, 5.5 | Rel-8 | pc_eBand8_Supp | Band 8 |
| 9 | Frequency band: 1749.9-1784.9, 1844.9-1879.9 MHz | 36.101, 5.5 | Rel-8 | pc_eBand9_Supp | Band 9 |
| 10 | Frequency band: 1710-1770, 2110-2170 MHz | 36.101, 5.5 | Rel-8 | pc_eBand10_Supp | Band 10 |
| 11 | Frequency band: 1427.9-1452.9, 1475.9-1500.9 MHz | 36.101, 5.5 | Rel-8 | pc_eBand11_Supp | Band 11 |
| 12 | Frequency band: 699-716, 729-746 MHz | 36.101, 5.5 | Rel-8 | pc_eBand12_Supp | Band 12 |
| 13 | Frequency band: 777-787, 746-756 MHz | 36.101, 5.5 | Rel-8 | pc_eBand13_Supp | Band 13 |
| 14 | Frequency band: 788-798, 758-768 MHz | 36.101, 5.5 | Rel-8 | pc_eBand14_Supp | Band 14 |
| 15 | Reserved |  |  |  |  |
| 16 | Reserved |  |  |  |  |
| 17 | Frequency band: 704-716, 734-746 MHz | 36.101, 5.5 | Rel-8 | pc_eBand17_Supp | Band 17 |
| 18 | Frequency band: 815-830, 860-875 MHz | 36.101, 5.5 | Rel-9 | pc_eBand18_Supp | Band 18 |
| 19 | Frequency band: 830-845, 875-890 MHz | 36.101, 5.5 | Rel-9 | pc_eBand19_Supp | Band 19 |
| 20 | Frequency band: 832-862, 791-821 MHz | 36.101, 5.5 | Rel-9 | pc_eBand20_Supp | Band 20 |
| 21 | Frequency band: 1447.9-1462.9, 1495.9-1510.9 MHz | 36.101, 5.5 | Rel-9 | pc_eBand21_Supp | Band 21 |
| 22 | Frequency band: 3410-3490, 3510-3590 MHz | 36.101, 5.5 | Rel-10 | pc_eBand22_Supp | Band 22 |
| 23 | Frequency band: 2000-2020, 2180-2200 MHz | 36.101, 5.5 | Rel-10 | pc_eBand23_Supp | Band 23 |
| 24 | Frequency band: 1626.5-1660.5, 1525-1559 MHz | 36.101, 5.5 | Rel-10 | pc_eBand24_Supp | Band 24 |
| 25 | Frequency band: 1850-1915, 1930-1995 MHz | 36.101, 5.5 | Rel-10 | pc_eBand25_Supp | Band 25 |
| 26 | Frequency band: 814-849, 859-894 MHz | 36.101, 5.5 | Rel-11 | pc_eBand26_Supp | Band 26 |
| 27 | Frequency band: 807-824, 852-869 MHz | 36.101, 5.5 | Rel-11 | pc_eBand27_Supp | Band 27 |
| 28 | Frequency band: 703-748, 758-803 MHz | 36.101, 5.5 | Rel-11 | pc_eBand28_Supp | Band 28 |
| 29 | Frequency band: N/A, 717-728 MHz | 36.101, 5.5 | Rel-11 | pc_eBand29_Supp | Band 29 |
| 30 | Frequency band: 2305-2315, 2350-2360 MHz | 36.101, 5.5 | Rel-12 | pc_eBand30_Supp | Band 30 |
| 31 | Frequency band: 452.5-457.5, 462.5-467.5 MHz | 36.101, 5.5 | Rel-12 | pc_eBand31_Supp | Band 31 |
| 32 | Frequency band: N/A, 1452-1496 MHz | 36.101, 5.5 | Rel-12 | pc_eBand32_Supp | Band 32 |
| 33 | Frequency band: 1920-2010, 2110-2200 MHz | 36.101, 5.5 | Rel-13 | pc_eBand65_Supp | Band 65 |
| 34 | Frequency band: 1710-1780, 2110-2200 MHz | 36.101, 5.5 | Rel-13 | pc_eBand66_Supp | Band 66 |
| … |  |  |  |  |  |
| 36 | Frequency band: 698-728, 753-783 MHz | 36.101, 5.5 | Rel-15 | pc_eBand68_Supp | Band 68 |
| 37 | Frequency band: N/A, 2570-2620 MHz | 36.101, 5.5 | Rel-14 | pc_eBand69_Supp | Band 69 |
| 38 | Frequency band: 1695-1710, 1995-2020 MHz | 36.101, 5.5 | Rel-14 | pc_eBand70_Supp | Band 70 |
| 39 | Frequency band: 663-698, 614-652 MHz | 36.101, 5.5 | Rel-15 | pc_eBand71_Supp | Band 71 |
| 40 | Frequency band: 451-456, 461-466 MHz | 36.101, 5.5 | Rel-15 | pc_eBand72_Supp | Band 72 |
| 41 | Frequency band: 450-455, 460-465 MHz | 36.101, 5.5 | Rel-15 | pc_eBand73_Supp | Band 73 |
| 42 | Frequency band: 1427-1470, 1475-1518 MHz | 36.101, 5.5 | Rel-15 | pc_eBand74_Supp | Band 74 |
| ... |  |  |  |  |  |
| 85 | Frequency band: 698-716, 728-746 MHz | 36.101, 5.5 | Rel-15 | pc_eBand85_Supp | Band 85 |
| ... |  |  |  |  |  |
| 87 | Frequency band: 410-415, 420-425 MHz | 36.101, 5.5 | Rel-16 | pc_eBand87_Supp | Band 87 |
| 88 | Frequency band: 412-417, 422-427 MHz | 36.101, 5.5 | Rel-16 | pc_eBand88_Supp | Band 88 |
| ... |  |  |  |  |  |
| 255 | Frequency band: 1626.5-1660.5, 1525-1559 MHz | 36.102, 5.2 | Rel-18 | pc_eBand255_Supp | Band 255 |
| 266 | Frequency band: 1980-2010, 2170-2200 MHz | 36.102, 5.2 | Rel-18 | pc_eBand256_Supp | Band 256 |

<< END OF CHANGES >>