3GPP TSG-RAN5 Meeting #98	R5-230456

# Athens, Greece, Feburary 27th - March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2169 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of pass fail limits for CBR test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The pass/fail limit for RRM testing (specified in 38.533 Annex G.2) is designed based on the error ratio =10% (corresponding to rate of correct events observed during repeated tests = 90% required by 38.133). However, according to 38.133 A.9.1.5, The rate of correct events observed during repeated tests in CBR test cases 38.533 9.1.5.1 and 9.1.5.2 shall be at least 98%, i.e. the error ratio in CBR test cases is only 2% rather than 10%. Therefore, the existing pass/fail limits in 38.533 Annex G.2 are not applicable. Designing a new set of pass/fail limit for CBR test cases is needed. 38.133 clause A.9.1.5 For UEs that support NR Uu and sidelink operation, the UEs shall not send event C1 triggered measurement reports during T1 and shall send event C1 triggered measurement reports during T2. For UEs that support sidelink operation only, the UE channel occupancy ratio shall be larger than 0.001 during T1, and the UE channel occupancy ratio shall be no larger than 0.001 during T2. The rate of correct events observed during repeated tests shall be at least 98%.. |
| Summary of change: |  | - Pass/fail limit for CBR test is added. The method to design limit and the method to calculate risk probability is also added as informative clause for information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

G.3	Statistical testing of NR sidelink CBR measurement tests

G.3.1	General

The CBR measurement tests are of statistical nature. The pass fail limits in tests of statistical nature are expressed as a limit (e.g. delay limit) and a success ratio applicable for the limit. The success ratio is 98% (the complement is the error ratio ER = 2%).

G.3.2	Design of the test

The test is defined by the following design principles (see TS 36.521-1 clause G.7, Theory):

- The early decision concept is applied.

- A second limit is introduced: bad DUT factor M>1

To decide the test pass:

- Supplier risk is applied based on the bad DUT quality

To decide the test fails

- Customer risk is applied based on the specified DUT quality

The test is defined by the following parameters:

- Limit ER = 0.02 (success ratio = 98%)

- Bad DUT factor M=1.5 (selectivity)

- Confidence level CL = 95% (for specified DUT and bad DUT-quality)

G.3.3	Numerical definition of the pass fail limits

Table G.3.3-1: pass fail limits for ER = 0.02

| ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 206 | N/A | 43 | 1970 | 1389 | 86 | 6370 | 3300 | 129 | 6413 | 5409 |
| 1 | 287 | N/A | 44 | 2005 | 1431 | 87 | 6371 | 3348 | 130 | 6414 | 5459 |
| 2 | 345 | N/A | 45 | 2041 | 1468 | 88 | 6372 | 3396 | 131 | 6415 | 5511 |
| 3 | 398 | 10 | 46 | 2078 | 1511 | 89 | 6373 | 3443 | 132 | 6416 | 5560 |
| 4 | 447 | 23 | 47 | 2113 | 1555 | 90 | 6374 | 3492 | 133 | 6417 | 5613 |
| 5 | 494 | 39 | 48 | 2149 | 1600 | 91 | 6375 | 3537 | 134 | 6418 | 5660 |
| 6 | 540 | 57 | 49 | 6333 | 1637 | 92 | 6376 | 3585 | 135 | 6419 | 5712 |
| 7 | 585 | 78 | 50 | 6334 | 1683 | 93 | 6377 | 3631 | 136 | 6420 | 5765 |
| 8 | 629 | 102 | 51 | 6335 | 1728 | 94 | 6378 | 3683 | 137 | 6421 | 5811 |
| 9 | 671 | 130 | 52 | 6336 | 1768 | 95 | 6379 | 3728 | 138 | 6422 | 5866 |
| 10 | 714 | 155 | 53 | 6337 | 1813 | 96 | 6380 | 3778 | 139 | 6423 | 5915 |
| 11 | 756 | 182 | 54 | 6338 | 1857 | 97 | 6381 | 3828 | 140 | 6424 | 5970 |
| 12 | 797 | 212 | 55 | 6339 | 1900 | 98 | 6382 | 3874 | 141 | 6425 | 6020 |
| 13 | 838 | 243 | 56 | 6340 | 1938 | 99 | 6383 | 3922 | 142 | 6426 | 6069 |
| 14 | 879 | 276 | 57 | 6341 | 1982 | 100 | 6384 | 3971 | 143 | 6427 | 6124 |
| 15 | 918 | 306 | 58 | 6342 | 2028 | 101 | 6385 | 4018 | 144 | N/A | 6427 |
| 16 | 959 | 342 | 59 | 6343 | 2072 | 102 | 6386 | 4070 | 145 |  |  |
| 17 | 998 | 375 | 60 | 6344 | 2116 | 103 | 6387 | 4118 | 146 |  |  |
| 18 | 1037 | 408 | 61 | 6345 | 2161 | 104 | 6388 | 4169 | 147 |  |  |
| 19 | 1077 | 442 | 62 | 6346 | 2203 | 105 | 6389 | 4216 | 148 |  |  |
| 20 | 1115 | 480 | 63 | 6347 | 2246 | 106 | 6390 | 4264 | 149 |  |  |
| 21 | 1154 | 517 | 64 | 6348 | 2292 | 107 | 6391 | 4314 | 150 |  |  |
| 22 | 1192 | 553 | 65 | 6349 | 2338 | 108 | 6392 | 4358 | 151 |  |  |
| 23 | 1230 | 591 | 66 | 6350 | 2382 | 109 | 6393 | 4413 | 152 |  |  |
| 24 | 1269 | 629 | 67 | 6351 | 2427 | 110 | 6394 | 4462 | 153 |  |  |
| 25 | 1306 | 666 | 68 | 6352 | 2473 | 111 | 6395 | 4506 | 154 |  |  |
| 26 | 1344 | 706 | 69 | 6353 | 2516 | 112 | 6396 | 4561 | 155 |  |  |
| 27 | 1382 | 741 | 70 | 6354 | 2559 | 113 | 6397 | 4611 | 156 |  |  |
| 28 | 1420 | 777 | 71 | 6355 | 2608 | 114 | 6398 | 4661 | 157 |  |  |
| 29 | 1457 | 815 | 72 | 6356 | 2652 | 115 | 6399 | 4712 | 158 |  |  |
| 30 | 1494 | 855 | 73 | 6357 | 2694 | 116 | 6400 | 4760 | 159 |  |  |
| 31 | 1531 | 898 | 74 | 6358 | 2741 | 117 | 6401 | 4807 | 160 |  |  |
| 32 | 1568 | 936 | 75 | 6359 | 2789 | 118 | 6402 | 4854 | 161 |  |  |
| 33 | 1605 | 976 | 76 | 6360 | 2836 | 119 | 6403 | 4909 | 162 |  |  |
| 34 | 1642 | 1015 | 77 | 6361 | 2883 | 120 | 6404 | 4960 | 163 |  |  |
| 35 | 1679 | 1059 | 78 | 6362 | 2928 | 121 | 6405 | 5010 | 164 |  |  |
| 36 | 1715 | 1098 | 79 | 6363 | 2971 | 122 | 6406 | 5062 | 165 |  |  |
| 37 | 1752 | 1138 | 80 | 6364 | 3018 | 123 | 6407 | 5110 | 166 |  |  |
| 38 | 1788 | 1180 | 81 | 6365 | 3068 | 124 | 6408 | 5158 | 167 |  |  |
| 39 | 1825 | 1220 | 82 | 6366 | 3115 | 125 | 6409 | 5204 | 168 |  |  |
| 40 | 1861 | 1263 | 83 | 6367 | 3162 | 126 | 6410 | 5256 | 169 |  |  |
| 41 | 1897 | 1302 | 84 | 6368 | 3203 | 127 | 6411 | 5305 |  |  |  |
| 42 | 1933 | 1344 | 85 | 6369 | 3251 | 128 | 6412 | 5358 |  |  |  |

The first column is the number of errors (ne = number of exceeded delays or number of wrong reports)

The second column is the number of samples for the pass limit (nsp , ns=Number of samples= number of successes + number of wrong reports)

The third column is the number of samples for the fail limit (nsf)

G.3.4	Pass fail decision rules

The pass fail decision rules apply for a single test, comprising one component in the test vector. The over all Pass /Fail conditions are defined in clause G.2.6

Having observed 0 errors, pass the test at 206+ samples, otherwise continue

Having observed 1 error, pass the test at 287+ samples, otherwise continue

Having observed 2 errors, pass the test at 345+ samples, otherwise continue

Having observed 3 errors, pass the test at 398+ samples, fail the test at 10 samples, otherwise continue

…

Having observed 143 errors, pass the test at 6427+ samples, fail the test at 6124- samples, otherwise continue

Having observed 144 errors, fail the test at 6427- samples,

Where x+ means: x or more, x- means x or less

NOTE 1: an ideal DUT passes after 260 samples. The maximum test time is 6427 samples.

G.4	Theory to derive the numbers in Table G.2.3-1 (informative)

TS 36.521-1 Annex G.7 applies.

G.5	Theory to derive the numbers in Table G.3.3-1 (informative)

TS 36.521-1 Annex G.7 applies except that:

-	36.521-1 Annex G.7.9 is replaced by G.5.1 and G.5.2.

G.5.1	Algorithm to calculate pass and fail probabilities

Following basic assumptions are introduced for statistical testing are assumed in this clause:

- Result of single iteration of test procedure loop obeys Bernoulli distribution with error ratio p, i.e. the probability that the TE observes expected UE behaviour in single iteration (counted as a success) is 1-p and the probability that the TE doesn't observe expected UE behaviour in single iteration (counted as a fail) is p.

- Results of different iterations are independent and identically distributed (i.i.d).

- Early decision concept is applied, i.e., the TE repeats multiple iterations of the test procedure loop. For each iteration the TE records its result (count as a success if the TE observes the expected behavior, count as a fail otherwise) and its index in all iterations. When the i-th fail is observed in ei-th iteration, the TE compares ei with corresponding pass limit Np,i and fail limit Nf,i. The TE shall:

-	terminate the test and decide the DUT passes the test if ei>Np,i

-	terminate the test and decide the DUT fails the test if ei≤Nf,i,

-	continue the test and make no decision if Nf,i<ei≤Np,i.

For a more intuitive understanding, we use the state transition diagram shown in Figure G.5.1-1 to describe the entire process of statistical testing. In Figure 1, X-axis and Y-axis denote the success count and fail count in iterations already executed respectively. And the point with coordinate i,j (referred as state i,j thereafter) is used to indicates that the DUT has executed i+j iterations which includes i successes and j fails, where i,j≥0.

Figure G.5.1-1: Visualization of early decision statistical testing process. In this example, the TE verdicts that the DUT passes the test after executing 15 iterations of test procedure loop. The results of these 15 iterations are “s”, “f”, “f”, “s”, “s”, “s”, “f”, “s”, “s”, “s”, “f”, “s”, “s” and “s” in order (“s” = “success” and “f” = “fail”).

From Figure G.5.1-1 it’s straightforward to get following observations:

- The DUT is always in state 0,0 at the beginning of statistical test since no iteration is executed at that moment.

- Assume the DUT is in state i,j after executing i+j iterations. Then,

-	if the i+j+1-th iteration counts a success (with probability 1-p), the success count shall increase 1 and fail count shall remain unchanged after the i+j+1-th iteration. As a result, the DUT will be in state i+1,j after the i+j+1-th iteration. We call that the DUT moves from state i,j to state i+1,j, depicted as moving right by one in the diagram.

  - -	if the i+j+1-th iteration counts a fail (with probability p), the success count remains unchanged and the fail count increases 1 after the i+j+1-th iteration. Then the DUT will be in state i,j+1 after the i+j+1-th iteration. We call that the DUT moves from state i,j to state i,j+1, depicted as moving up by one in the diagram.

- The TE shall evaluate whether pass criteria or fail criteria is satisfied every time the DUT has executed a new iteration, i.e. the DUT moves to a new state. Specifically speaking,

-	If pass criterion is satisfied when DUT is in state i,j, the TE makes the decision that the DUT passes the test. Correspondingly, state i,j is referred as a passing state (depicted as green points in Figure G.5.1-1);

-	If fail criterion is satisfied when DUT is in state i,j, the TE makes the decision that the DUT fails the test. Correspondingly, state i,j is referred as a failure state (depicted as red points in Figure G.5.1-1).

-	If neither pass criterion nor fail criterion is satisfied when DUT is in state i,j, the TE makes no decision and continues the test. Correspondingly, state i,j is referred as a non-decidable state (depicted as hollow points in Figure G.5.1-1).

In this way, the statistical testing is represented as a process in which the DUT starts from state 0,0, moves along certain path in the diagram, and finally stops in certain passing state or failure state.

Obviously, there must be a close relationship between the pass/fail states and the pass/fail limits used for the test. The following two conclusions precisely describes the mapping between pass/fail limits and pass/fail states.

Conclusion 1: The DUT is decided to pass the test if and only if the DUT moves from state 0,0 to a state belongs to passing state set P=Np,i+1-i,i: i=0,1,2,⋯.

Proof: We prove sufficiency first, and then necessity.

- Sufficiency: If the DUT is in state Np,i+1-i,i, it means that the DUT has executed Np,i+1 iterations which includes i success and Np,i+1-i fails. Clearly, the TE can only get the i+1-th fail as early as the Np,i+1+1-th iteration, then we have ei+1>Np,i+1. This is just the pass criterion when the TE collecting the i+1-th fail.

- Necessity: If the TE verdicts the DUT to pass when it collects the i+1-th fail, following inequations must have been satisfied according to the pass criterion:

e1≤Np,1, e2≤Np,2,⋯, ei≤Np,i,ei+1>Np,i+1.

The DUT is in state ei-i, i when the i-th fail happens in the ei-th iteration. Then the next Np,i+1-ei iterations after the ei-th iteration must be counted as success since ei+1>Np,i+1. Then the DUT will be in state Np,i+1-i,i after executing these Np,i+1-ei iterations.

Conclusion 2: The DUT is decided to fail the test if and only if the DUT moves from state 0,0 to a state belongs to failure set F=j,i+1: Nf,i-i<j≤Nf,i+1-i+1, i=0,1,2,⋯, where Nf,0≜-1 for sake of simplify.

Proof: Proof of conclusion 2 is quite similar as the proof of conclusion 1 and is omitted here.

Conclusion 1 and 2 have shown the equivalence between pass/fail limits and pass/fail state sets. As a result, the probability that the DUT pass the test for given error ratio p, passing state set P and failure state set F, denoted as PpP,F, is given by

PpP,F=ProbDUT moves from state0,0 to state i,j which belongs to P.

Furthermore, it’s easy to prove that any path from state0,0 to passing state i,j won’t pass through any passing state or failure state except the ending state. This is because if the path contains a passing state or failure state i',j' which is not the ending state, the test will be terminated immediately when the DUT is in state i',j'. Then i',j' shall be the ending state, i.e. i',j'=i,j. This contradicts the assumption that state i',j' is not the ending state. This also means that two events “DUT moves from state (0,0) to passing state (i,j)” and “DUT moves from state (0,0) to passing state (i',j')” are disjoint if i',j'≠i,j. Then we have,

PpP,F=i,j∈PProbDUT moves from state0,0 to passing state i,j=i,j∈PTi,jpP,F,

where Ti,jpP,F is defined as the transition probability from state 0,0 to state i,j, i.e.

Ti,jpP,F≜ProbDUT moves from state0,0 to state i,j.

Similarly, the probability that the DUT fail the test for given error ratio p, passing state set P and failure state set F, denoted as FpP,F, is derived as

FpP,F=i,j∈FTi,jpP,F

From the discussion above we can observe that the calculation of the PpP,F and FpP,F is finally transformed into the calculation of the transition probability Ti,jpP,F. Assuming DUT is in state i,j after the i+j-th iteration, i,j≥0. Considering that the i+j-th iteration can only have two possible results: success or fail, we can derive that the DUT can only be in state i-1,j or state i,j-1 after executing i+j-1 iterations. Then we have:

Ti,jpP,F=ProbDUT moves from state0,0 to state i,j, passing i-1,j+ProbDUT moves from state0,0 to state i,j, passing state i,j-1=ProbDUT moves from state0,0 to i-1,jProbDUT moves from state i-1,j to i,j+ProbDUT moves from state0,0 to i,j-1ProbDUT moves from state i,j-1 to i,j

The first equation above holds because the event “DUT moves from state (0,0) to state (i,j), passing state (i,j-1)” and event “DUT moves from state (0,0) to state (i,j), passing state (i-1,j)” are disjoint. And the second equation above holds because results of different iterations are independent.

Furthermore, we have following obvious observations:

If i=0, obviously the event “DUT moves from state 0,0 to state i-1,j” won’t happen since state i-1,j doesn’t exist in this case. Otherwise, the probability that this event occurs equals to  Ti-1,jpP,F according to the definition of transition probability. Then we have

ProbDUT moves from state0,0 to i-1,j=Ti-1,jpP,FIi>0,

Similarly, we can also derive that

ProbDUT moves from state0,0 to i,j-1=Ti,j-1pP,FIj>0,

Where, Ii>0 and Ij>0 are indicator functions,

Ii>0=1,if i>00,otherwise,  Ij>0=1,if j>00,otherwise

If i-1,j∈P∪F, the event “DUT moves from state i-1,j to state i,j” won’t happen since the test will be terminated when DUT reaches state i-1,j. Otherwise, this event is equivalent to the event “result of the i+j-th iteration is a success”. Then we have

ProbDUT moves from state i-1,j to i,j=1-pIi-1,j∉P∪F

Similarly, we can also derive that

ProbDUT moves from state i,j-1 to i,j=pIi,j-1∉P∪F.

where,

Ii-1,j∉P∪F=1,i-1,j∉P∪F0,otherwise,  Ii,j-1∉P∪F=1,if i,j-1∉P∪F0,otherwise

Finally, we have shown that following conclusion holds.

Conclusion 3: Transition probability Ti,jpP,F can be calculated by

Ti,jpP,F=Ti-1,jpP,F1-pIi>0Ii-1,j∉P∪F+Ti,j-1pP,FpIj>0Ii,j-1∉P∪F.

Proof: See above.

The conclusion above shows that Ti,jpP,F can be calculated in a dynamic programming manner: Clearly, we can calculate Ti,jpP,F using the above equation if we know the value of Ti-1,jpP,F and Ti,j-1pP,F; For the same reason, we can calculate Ti-1,jpP,F and Ti,j-1pP,F using the equation above if we know the value of  Ti-2,jpP,F, Ti-1,j-1pP,F and Ti,j-2pP,F; Repeating the recursion above, at last all we need to know is only T0,0pP,F. And we know T0,0pP,F=1 since UE always is in state 0,0 when test starts.

Using the recursion above, we can finally obtain Ti,jpP,F and further PpP,F and FpP,F for given p, P and F.

Assume the limit-risk mapping given in 36.521-1 is still used. Then the customer risk Rc and supplier risk Rs for given passing state set P and failure state set F is the probability that baseline bad DUT (error ratio = Mp, where M is the bad DUT factor) passes the test and the probability that baseline good DUT (error ratio = mp, where m is the bad DUT factor) fails the test respectively. Then we have

Rc=i,j∈PTi,jMpP,F,Rs=i,j∈FTi,jmpP,F.

G.5.2	Method of designing pass/fail limit to approach given risk probability

For given error ratio p, confidence level PCL, good DUT factor m and bad DUT factor M, pass limit Np,i,i=1,2,⋯ and fail limit Nf,i:i=1,2,⋯ can be obtained by using the method listed below. Note that the limit-risk mapping in 36.521-1 is assumed.

Step 1: Initialize as follows:

- Set P and F, passing state set and failure state set respectively, as P, F=∅.

- Set Rc and Rs, customer risk and supplier risk respectively, as Rc,Rs=0;

- Set n, number of executed iterations, as n=0;

- Set S, non-decidable state set after n-th iteration, as S=0,0;

- Set Ti,jmpP,F and T0,0MpP,F, transition probability to state 0,0 for baseline good DUT and for baseline bad DUT respectively, as Ti,jmpP,F=1, T0,0MpP,F=1.

- Select Δc and Δs, step size for customer risk and supplier risk respectively, as Δc≤1-PCL, Δs≤1-PCL.

Figure G.5.2-1: Demonstration of step 1

Figure G.5.2-1 is the demonstration of step 1. The reason to initialize =0,0, Ti,jmpP,F=1, T0,0MpP,F=1 is because the DUT is always in state 0,0 before executing any iteration. And we have P, F=∅ since no passing state or failure state is selected yet.

Step 2: Calculate set S', the set of the state can be reached after the n+1-th iteration, as S'=i,j:i-1,j∈ S or i,j-1∈S. Then for all state i,j∈S', calculate Ti,jMpP,F and Ti,jmpP,F as follows:

Ti,jMpP,F=Ti-1,jMpP,F1-MpIi>0Ii-1,j∉P∪F+Ti,j-1MpP,FMpIj>0Ii,j-1∉P∪F,

Ti,jmpP,F=Ti-1,jmpP,F1-mpIi>0Ii-1,j∉P∪F+Ti,j-1mpP,FmpIj>0Ii,j-1∉P∪F.

Figure G.5.2-2: Demonstration of step 2

See Figure G.5.2-2 for better understanding of step 2. If DUT doesn’t pass or fail the test after the n-th iteration then DUT must be in one state in S. It’s easy to observe that the DUT must be in one state in S'. Then the transition probability to state in S' can be calculated by using Conclusion 3. For example, the only way to move to state D in Figure 2 is to first move to state A then count a fail in the n+1-th iteration, then we have TDp=TApp. On the other hand, to move to state E, the DUT can first move to state B then count a success in the n+1-th iteration or first move to state C then count a fail in the n+1-the iteration, i.e. TEp=TBp1-p+TCpp.

Step 3: First, try finding an integer ne which satisfies n+1-ne,ne∈S', Rc+i,j∈S'j≤neTi,jMpP,F≤1-PCL and Rs+i,j∈S'j>neTi,jmpP,F≤1-PCL. If such a ne exists, let

Rc=Rc+i,j∈S',j≤neTi,jMpP,F,Rs=Rs+i,j∈S',j>neTi,jmpP,F,

P=P∪i,j:i,j∈S',j≤ne, F=F∪i,j:i,j∈S',j>ne,

If such a ne doesn’t exist, instead try finding integers np and nf which satisfy

n+1-np,np∈S',n+1-nf,nf∈S',

minicΔc,1-PCL<Rc+i,j∈S',j≤npTi,jMpP,F≤minic+1Δc,1-PCL,

minisΔs,1-PCL<Rs+i,j∈S',j≥nfTi,jmpP,F≤minis+1Δs,1-PCL.

Then,

- if such a np exists, set

Rc=Rc+i,j∈S',j≤npTi,jMpP,F,  P=P∪i,j:i,j∈S',j≤np,  ic=ic+1.

- if such a nf exists, set

Rs=Rs+i,j∈S',j≥nfTi,jmpP,F,  F=F∪i,j:i,j∈S',j≥nf,  is=is+1.

Two possible cases in step 3 are depicted in figure G.5.2-3 and figure G.5.2-4 respectively. If ne exists, even we add all states i,j∈S' with j≤ne to P and all states i,j∈S' with j>ne to F, the customer risk and supplier risk are still no larger than 1-PCL (the increment of the customer risk and supplier risk, are denoted as blue and green triangle in figure 4 respectively).

Figure G.5.2-3: Demonstration of step 3 (ne exists).

If ne doesn’t exist, instead we try to find state set i,j:i,j∈S',j≤np which satisfies that the total transition probability to any state in the set is approximately Δc. If such a set exists we can add states in this set to P without breaking customer risk threshold. Similarly, we can also try to a state seti,j:i,j∈S',j≥nf which satisfies that the total transition probability to any state in the set is approximately Δs and add states in this set to F.

Figure G.5.2-4: Demonstration of step 3 (ne doesn’t exist).

Step 4: Let S=S'\P∪F, n=n+1. Then go back to Step 2 if S≠∅, otherwise go to Step 5.

If a state i,j∈S' is selected as a passing state or failing state then it’s no longer a non-decidable state. Then we need to exclude it from non-decidable state set after n+1-the iteration. If all states in S' are excluded, clearly the test surely will terminate after the n+1-the iteration and the designing is finished. Otherwise, we need to go back to step 2 and repeat procedures above.

Step 5: Sort all states i,j in P in the ascending order of j and all states i,j in F in the ascending order of i, i.e. we denote

P=A0,0, A1,1, A2,2, A3,3⋯,  F=0,B0, 1,B1,2,B2,3,B3,⋯

Then construct passing state set P' and failure state set F' as

P'=Aj',j:j=0,1,2,⋯,  F'=i,Bi':i=0,1,2⋯

Where Aj'=mink≥jAk,  Bi'=mink≥iBi

Purpose of step 5 is to further optimize the passing state and failure state obtained in step 2 to 4. To understand step 5, we only need to note that all path to passing state A,B must also pass state C,D. On the other hand, if a path passes state D or E, it must eventually reach one passing state in A, B and C. Then one can derive that

i,j∈A,B,CTi,jpP,F=i,j∈A,D,ETi,jpP,F

In other words, if P=A,B,C is a passing set which satisfies customer risk requirements, then P'=A,D,E is also a passing set satisfying customer risk requirements since it has exactly the same customer risk as P. But P' is a better choice than P since it requires fewer iterations to make decision.

For the same reason, we can also derive that failure set F'=F,G,L,M,N,K has same supplier risk with F=F,G,H,I,J,K but requires fewer iterations to fail the test.

Figure G.5.2-5: Demonstration of step 3 (ne doesn’t exist).

Step 6: Construct pass limit Np,i and fail limit Nf,i as follows:

Np,1=A0',Np,2=A1'+1,Np,3=A2'+2,⋯

Nf,1=maxBi=1i+Bi,Nf,2=maxBi=2i+Bi,Nf,3=maxBi=3i+Bi,⋯

At last, we can convert pass/fail state sets to pass/fail limits by using conclusion 3 in Annex G.5.1.

<End of modified section 1>