3GPP TSG-RAN5 Meeting #98	R5-230483

# Athens, Greece, Feburary 27th - March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2194 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RedCap RRM TC 16.6.1.5 - intra gap based non-DRX 1Rx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To add RedCap RRM TC 16.6.1.5 - NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RedCap RRM TC 16.6.1.5 - NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.6.1.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0244 TS 38.533 CR 2225,2226 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

16.6.1.5	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE

Editor’s note: This test cases is incomplete in following aspects:

-	TT analysis is still missing

16.6.1.5.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in 38.133 [6] clause 9.2B.6.1 and 9.2B.6.2.

16.6.1.5.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards supporting CSI-RS based RLM and BWP operation without bandwidth restriction.

16.6.1.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.1.0.1 and 16.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.1.5.

16.6.1.5.4	Test description

16.6.1.5.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.1.3.4.1 with following exceptions:

-	Table 6.6.1.3.4.1-1 is replaced by Table 16.6.1.5.4.1-1.

-	Table 6.6.1.3.4.1-2 is replaced by Table 16.6.1.5.4.1-2.

-	Table 6.6.1.3.4.1-3 is replaced by Table 16.6.1.5.4.1-3.

Table 16.6.1.5.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.1.5-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.1.5-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.1.5-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.1.5-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.6.1.5.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.1.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.1.5.4.1-3: General test parameters

| Parameter | Unit | Test config. | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1,2,3,4 | Cell 1 |  |
| Neighbour cell |  | 1,2,3,4 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1,2,3,4 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1,2,3,4 | Per-UE gaps |  |
| Measurement gap repetition periodicity | ms | 1,2,3,4 | 40 |  |
| Measurement gap length | ms | 1,2,3,4 | 6 |  |
| Measurement gap offset | ms | 1,2,3,4 | 39 |  |
| SSB configuration |  | 1,2,4 | SSB.1 FR1 |  |
|  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  | 1,4 | SMTC.2 |  |
|  |  | 2,3 | SMTC.1 |  |
| CSI-RS parameters |  | 1,4 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1,2,3,4 | -4.5 |  |
| CP length |  | 1,2,3,4 | Normal |  |
| Hysteresis | dB | 1,2,3,4 | 0 |  |
| Time To Trigger | s | 1,2,3,4 | 0 |  |
| Filter coefficient |  | 1,2,3,4 | 0 | L3 filtering is not used |
| DRX | ms | 1,2,3,4 | N/A | OFF |
| Time offset between serving and neighbour cells |  | 1,4 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2,3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

16.6.1.5.4.2	Test procedure

Same as the test procedure given in clause 6.6.1.3.4.2.

16.6.1.5.4.3	Message contents

Same as the message contents given in clause 6.6.1.3.4.3 with the following exception:

-	For test configuration 16.6.1.5-1 and 16.6.1.5-4,

-	Table H.3.1-3 is used with condition INTRA-FREQ MO.

-	Table 7.3.1-3 specified in 38.508-1 [14] is used with condition SMTC.2.

-	For test configuration 16.6.1.5-2 and 16.6.1.5-3,

-	Table H.3.1-3 is used with condition INTRA-FREQ MO and Synchronous cells.

-	Table 7.3.1-3 specified in 38.508-1 [14] is used with condition SMTC.1.

16.6.1.5.5	Test requirement

Same as the test requirements given in clause 6.6.1.3.5 with following exceptions:

-	Table 6.6.1.3.5-1 is replaced by Table 16.6.1.5.5-1.

Table 16.6.1.5.5-1: Cell specific test parameters

| Parameter | Unit | Test config. | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1,4 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1,4 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1,4 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1,4 | CCR.1.2 FDD |  | N/A |  |
|  |  | 2 | CCR.1.2 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1,2,3,4 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1,4 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1,2,3,4 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1,2,3,4 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1,2,3,4 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1,2,3,4 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1,2,4 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 KHz | 1,2,3,4 | -98 |  |  |  |
|  | dB | 1,2,3,4 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1,2,3,4 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS | 1,2,4 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1,2,4 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/18.36 MHz | 3 | -61.68 | -59.34 | -61.68 | -59.34 |
| Propagation Condition |  | 1,2,3,4 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

<End of modified section 1>