3GPP TSG-RAN5 Meeting #98	R5-230490

# Athens, Greece, Feburary 27th - March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2201 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RedCap RRM TC 16.6.4.8 - CSI-RS L1-RSRP DRX 2Rx with TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To add RedCap RRM TC 16.6.4.8 - NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RedCap RRM TC 16.6.4.8 - NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.6.4.8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0244 TS 38.533 CR 2225,2226 TS 38.903 CR 0480 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

16.6.4.8	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE

16.6.4.8.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5B.4.2.

16.6.4.8.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards supporting long DRX cycle.

16.6.4.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.4.0.1 and 16.6.4.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.4.8.

16.6.4.8.4	Test description

16.6.4.8.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.4.3.4.1 with following exceptions:

-	Table 6.6.4.3.4.1-1 is replaced by Table 16.6.4.8.4.1-1.

-	Table 6.6.4.3.4.1-2 is replaced by Table 16.6.4.8.4.1-2.

-	Table 6.6.4.3.4.1-3 is replaced by Table 16.6.4.8.4.1-3.

Table 16.6.4.8.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.4.8-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.4.8-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.4.8-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.4.8-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 16.6.4.8.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.4.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.4.8.4.1-3: General test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN |  |  | freq1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2,3 |  | TDD |
|  | Config 4 |  | HD-FDD |
| TDD Configuration | Config 1,4 |  | N/A |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| BWchannel | Config 1,2,4 | MHz | 10: NRB,c = 52 |
|  | Config 3 |  | 20: NRB,c = 51 |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |
|  | Config 2 |  | SR.1.1 TDD |
|  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1,4 |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.2.1 TDD |
| SSB configuration | Config 1,2,4 |  | SSB.3 FR1 |
|  | Config 3 |  | SSB.2 Redcap FR1 |
| CSI-RS configuration | Config 1,4 |  | CSI-RS 1.3 FDD |
|  | Config 2 |  | CSI-RS 1.3 TDD |
|  | Config 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns |  |  | OP.1 |
| TRS Configuration | Config 1,4 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  |  | SMTC.1 RedCap |
| DRX configuration |  |  | DRX.3 |
| reportConfigType |  |  | aperiodic |
| reportQuantity |  |  | cri-RSRP |
| Number of reported RS |  |  | 2 |
| qcl-Info |  |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList |  | slots | 8 |
| T1 |  | s | 5 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

16.6.4.8.4.2	Test procedure

Same as the test procedure given in clause 6.6.4.3.4.2.

16.6.4.8.4.3	Message contents

Same as the message contents given in clause 6.6.4.3.4.3.

16.6.4.8.5	Test requirement

Same as the test requirements given in clause 6.6.4.3.5 with following exceptions:

-	Table 6.6.4.3.5-1 is replaced by Table 16.6.4.8.5-1.

Table 16.6.4.8.5-1: Cell specific test parameters

| Parameter |  | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 |  | dBm/15kHz | -94.65 |  |
| Note1 | Config 1,2,4 | dBm/SCS | -94.65 |  |
|  | Config 3 |  | -91.65 |  |
|  |  | dB | 0 | 3.5 |
| CSI-RS RSRP Note2 | Config 1,2,4 | dBm/SSB SCS | -94.65 | -91.15 |
|  | Config 3 |  | -91.65 | -88.14 |
| Io Note2 | Config 1,2,4 | dBm/9.36 MHz | -63.69 | -61.59 |
|  | Config 3 | dBm/18.36 MHz | -60.77 | -58.67 |
|  |  | dB | 0 | 3.5 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

<End of modified section 1>