3GPP TSG-RAN5 Meeting #98	R5-230513

# Athens, Greece, Feburary 27th - March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2224 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Annex A for RedCap RRM TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To update RedCap RMCs to align with core specs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RedCap RMCs are updated to align with core specs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3B, A.4A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

A.3B	Reference SSB configuration for RedCap

A.3B.1	SSB configuration for FR1

Table A.3B.1-1: SSB allocation for FR1

| SSB Parameters |  | Unit | Value |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Pattern |  |  | SSB.1 RedCap FR1 |  | SSB.2 RedCap FR1 | SSB.3 RedCap FR1 | SSB.4 RedCap FR1 |  | SSB.5 RedCap FR1 |  | SSB.6 RedCap FR1 |  | SSB.7 RedCap FR1 |
| Channel bandwidth |  | MHz | 20 |  | 20 | 20 | 10 |  | 20 |  | 10 |  | 20 |
| SSB SCS |  | kHz | 30 |  | 30 | 30 | 15 |  | 30 |  | 15 |  | 30 |
| SSB periodicity (TSSB) |  | ms | 20 |  | 20 | 20 | 20 |  | 20 |  | 80 |  | 80 |
| Number of SSBs per SS-burst |  |  | 1 |  | 2 | 1 | 1 |  | 1 |  | 1 |  | 1 |
| Symbol numbers containing SSB Note 4 | SSB #0 |  | 4-7 or 2-5 Note 3 |  | 4-7 or 2-5 Note 3 | 4-7 or 2-5 Note 3 | 2-5 |  | 4-7 or 2-5 Note 3 |  | 2-5 |  | 4-7 or 2-5 Note 3 |
|  | SSB #1 |  | N/A |  | 8-11 | N/A | N/A |  | N/A |  | N/A |  | N/A |
| Slot numbers containing SSB Note 4 |  |  | 0 |  |  |  |  |  |  |  | 5 | 10 |  |
| SFN containing SSB |  |  | SFN mod (TSSB /10ms) = 0 | SFN mod (TSSB /10ms) = 0 |  | SFN mod (TSSB /10ms) = 1 |  | SFN mod (TSSB /10ms) = 0 |  | SFN mod (TSSB /10ms) = 0 | SFN mod (TSSB /10ms) = 0 |  | SFN mod (TSSB /10ms) = 0 |
| RB numbers containing SSB within channel BW |  |  | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |  |  |  |  |  |  | (RBJ, RBJ+1,.…, RBJ+19)Note 2 |  |  |
| NOTE 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [28]. NOTE 2: RBs containing SSB can be configured in any frequency location within the associated bandwidth part except the RBs for allowed synchronization raster defined in TS 38.104 [28]. NOTE 3: Symbols 4-7 are chosen if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [28]; Otherwise, symbols 2-5 are chosen. NOTE 4: These values have been derived from other parameters for information purposes (as per TS 38.213). They are not settable parameters themselves |  |  |  |  |  |  |  |  |  |  |  |  |  |

A.3B.2	SSB configuration for FR2

Table A.3B.2-1: SSB allocation for FR2

| SSB Parameters |  | Unit |  | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Pattern |  |  |  | SSB.1 RedCap FR2 |  | SSB.2 RedCap FR2 |  | SSB.3 RedCap FR2 | SSB.4 RedCap FR2 |  | SSB.5 RedCap FR2 |
| Channel bandwidth |  | MHz |  | 100 |  | 100 |  | 100 | 100 |  | 100 |
| SSB SCS |  | kHz |  | 120 |  | 120 |  | 120 | 240 |  | 240 |
| SSB periodicity (TSSB) |  | ms |  | 20 |  | 40 |  | 80 | 40 |  | 80 |
| Number of SSBs per SS-burst |  |  |  | 1 |  | 1 |  | 1 | 1 |  | 1 |
| Symbol numbers containing SSB Note 3 | SSB #0 |  |  | 4-7 |  | 4-7 |  | 4-7 | 8-11 |  | 8-11 |
|  | SSB #1 |  |  | N/A |  | N/A |  | N/A | N/A |  | N/A |
| Indices of slots containing SSB Note 3 |  |  |  | 0 |  |  |  | 40 | 0 |  | 80 |
| Indices of SFN containing SSB |  |  |  | SFN mod (TSSB /10ms) = 0 |  | SFN mod (TSSB /10ms) = 0 |  | SFN mod (TSSB /10ms) = 0 | SFN mod (TSSB /10ms) = 0 |  | SFN mod (TSSB /10ms) = 0 |
| RB numbers containing SSB within channel BW |  |  |  | (RBJ,RBJ+1,.…,RBJ+19)Note 1 |  |  |  | (RBJ,RBJ+1,.…,RBJ+19)Note 2 | (RBJ,RBJ+1,.…,RBJ+19)Note 1 |  | (RBJ,RBJ+1,.…,RBJ+19)Note 2 |
| NOTE 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [28]. NOTE 2: RBs containing SSB can be configured in any frequency location within the associated bandwidth part except the RBs for allowed synchronization raster defined in TS 38.104 [28]. NOTE 3: These values have been derived from other parameters for information purposes (as per TS 38.213). They are not settable parameters themselves |  |  |  |  |  |  |  |  |  |  |  |

<End of modified section 1>

<Start of modified section 2>

A.4A	Reference SMTC configuration for RedCap

Table A.4A-1: SMTC configurations

| SMTC Parameters | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SMTC Pattern |  | SMTC.1 RedCap | SMTC.2 RedCap | SMTC.3 RedCap | SMTC.4 RedCap |  |  |
| SMTC periodicity | ms | 40 | 80 | 40 | 80 |  |  |
| SMTC offset | ms | 0 | 5 | 20 | 5 |  |  |
| SMTC duration | ms | 1 | 1 | 1 | 5 |  |  |

<End of modified section 2>