3GPP TSG-RAN5 Meeting #98	R5-230675

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2713 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Test Procedures for Switch off/Power off |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current Prose in table 4.9.6.1-1, after Step 1a4Aa5 the PDU Session Release is being handled. The current Prose does not handle the case whre a UE may transmit more than one PDU Session Release REQ (Multi-PDU case). This needs to be accommodated. Same needs to be done for Tables 4.9.6.2.1-1 and 4.9.6.3-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added an exception before Step 1a4Aa5a1 which indicates that Steps 1a4Aa5a1 - 1a4Aa5a2 may be repeated for all active PDU sessions. Similarly added exceptions in Tables 4.9.6.2.1-1 and 4.9.6.3-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | R5s230022, R5s221335 need this correction. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.9.6	Test procedures for Switch off / Power off UE

#### 4.9.6.1	Switch off / Power off procedure in RRC_IDLE

Table 4.9.6.1-1: Switch off procedure in RRC_IDLE

| Step |  | Procedure |  | Message Sequence |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | U - S |  | Message |  |
| - |  | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [36] pc_SwitchOnOff or [37] pc_USIM_Removal is supported |  | - |  | - |  |  |
| 1a1 |  | IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM (Note 1) |  | - |  | - |  |  |
| 1a2 |  | UE transmits an RRCSetupRequest message. |  | --> |  | RRC: RRCSetupRequest |  |  |
| 1a3 |  | SS transmit an RRCSetup message. |  | <-- |  | RRC: RRCSetup |  |  |
| - |  | EXCEPTION: Steps 1a4Aa1 to 1a4Aa5b1 specify optional behaviour if the UE has previously performed IMS registration |  | - |  | - |  |  |
| 1a4Aa1 |  | The UE transmits an RRCSetupComplete message to confirm the successful completion of the connection establishment and to initiate the IMS signalling procedure by including the SERVICE REQUEST message. |  | --> |  | RRC: RRCSetupComplete 5GMM: SERVICE REQUEST |  |  |
| 1a4Aa2 |  | The SS transmits a SecurityModeCommand message to activate AS security. |  | <-- |  | RRC: SecurityModeCommand |  |  |
| 1a4Aa3 |  | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. |  | --> |  | RRC: SecurityModeComplete |  |  |
| 1a4Aa4 |  | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB. |  | <-- |  | RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT |  |  |
| - |  | EXCEPTION: In parallel to the event described in step 1a4Aa5 below, the generic test procedure in TS 34.229-5 [47] Annex A.11 may be performed. |  | - |  | - |  |  |
| 1a4Aa5 |  | The UE transmits an RRCReconfigurationComplete message. |  | --> |  | RRC: RRCReconfigurationComplete |  |  |
| - |  | EXCEPTION: Steps 1a4Aa5a1 - 1a4Aa5b1 may be performed depending on UE implementation |  | - |  | - |  |  |
| - |  | EXCEPTION: Steps 1a4Aa5a1 - 1a4Aa5a2 may be repeated for all active PDU sessions |  | - |  | - |  |  |
| 1a4Aa5a1 |  | The UE transmits a PDU SESSION RELEASE REQUEST message. |  | --> |  | PDU SESSION RELEASE REQUEST |  |  |
| 1a4Aa5a2 |  | PDU session release procedure defined in clause 4.9.21 of TS 38.508-1 [4] is performed |  | - |  | - |  |  |
| 1a4Aa5a3 |  | The UE transmits a DEREGISTRATION REQUEST message. |  | --> |  | 5GMM: DEREGISTRATION REQUEST |  |  |
| 1a4Aa5b1 |  | The UE transmits a DEREGISTRATION REQUEST message |  | - |  | 5GMM: DEREGISTRATION REQUEST |  |  |
| - |  | EXCEPTION: Step 1a4Ab1 below specifies the behaviour if the UE has not previously performed IMS registration |  | - |  | - |  |  |
| 1a4Ab1 |  | The UE transmits an RRCSetupComplete message including the DEREGISTRATION REQUEST message. |  | --> |  | RRC: RRCSetupComplete 5GMM: DEREGISTRATION REQUEST |  |  |
| 1a4 |  | Void |  | - |  | - |  |  |
| 1a5 |  | The SS transmits an RRCRelease message |  | <-- |  | RRC: RRCRelease |  |  |
| 1b1 |  | ELSE power off UE (Note 2) |  | - |  | - |  |  |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |  |  |  |  |  |

#### 4.9.6.2	Switch off / Power off procedure in RRC_INACTIVE

4.9.6.2.1	Procedure

Table 4.9.6.2.1-1: Switch off procedure in RRC_INACTIVE

| Step |  | Procedure |  | Message Sequence |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | U - S |  | Message |  |
| - |  | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [36] pc_SwitchOnOff or [37] pc_USIM_Removal is supported |  | - |  | - |  |  |
| 1a1 |  | IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM (Note 1) |  | - |  | - |  |  |
| 1a2 |  | UE transmits an RRCResumeRequest message. |  | --> |  | NR RRC: RRCResumeRequest |  |  |
| 1a3 |  | SS transmit an RRCResume message. |  | <-- |  | NR RRC: RRCResume |  |  |
| - |  | EXCEPTION: Steps 1a4Aa1 to 1a4Aa5b1 specify optional behaviour if the UE has previously performed IMS registration |  | - |  | - |  |  |
| 1a4Aa1 |  | The UE transmits an RRCResumeComplete message to confirm the successful completion of the connection establishment and to initiate the IMS signalling procedure by including the SERVICE REQUEST message. |  | --> |  | RRC: RRCResumeComplete 5GMM: SERVICE REQUEST |  |  |
| 1a4Aa2 |  | The SS transmits a SecurityModeCommand message to activate AS security. |  | <-- |  | RRC: SecurityModeCommand |  |  |
| 1a4Aa3 |  | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. |  | --> |  | RRC: SecurityModeComplete |  |  |
| 1a4Aa4 |  | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB. |  | <-- |  | RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT |  |  |
| - |  | EXCEPTION: In parallel to the event described in step 1a4Aa5 below, the generic test procedure in TS 34.229-5 [47] Annex A.11 may be performed. |  | - |  | - |  |  |
| 1a4Aa5 |  | The UE transmits an RRCReconfigurationComplete message. |  | --> |  | RRC: RRCReconfigurationComplete |  |  |
| - |  | EXCEPTION: Steps 1a4Aa5a1 - 1a4Aa5b1 may be performed depending on UE implementation |  | - |  | - |  |  |
| - |  | EXCEPTION: Steps 1a4Aa5a1 - 1a4Aa5a2 may be repeated for all active PDU sessions |  | - |  | - |  |  |
| 1a4Aa5a1 |  | The UE transmits a PDU SESSION RELEASE REQUEST message. |  | --> |  | PDU SESSION RELEASE REQUEST |  |  |
| 1a4Aa5a2 |  | PDU session release procedure defined in clause 4.9.21 of TS 38.508-1 [4] is performed |  | - |  | - |  |  |
| 1a4Aa5a3 |  | The UE transmits a DEREGISTRATION REQUEST message. |  | --> |  | 5GMM: DEREGISTRATION REQUEST |  |  |
| 1a4Aa5b1 |  | The UE transmits a DEREGISTRATION REQUEST message |  | - |  | 5GMM: DEREGISTRATION REQUEST |  |  |
| - |  | EXCEPTION: Step 1a4Ab1 below specifies the behaviour if the UE has not previously performed IMS registration |  | - |  | - |  |  |
| 1a4 |  | Void |  | - |  | - |  |  |
| 1a5 |  | The SS transmits an RRCRelease message |  | <-- |  | NR RRC: RRCRelease |  |  |
| 1b1 |  | ELSE power off UE (Note 2) |  | - |  | - |  |  |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |  |  |  |  |  |

4.9.6.2.2	Specific Message contents

Table 4.9.6.2.2-1: RRCResumeRequest

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |  |
| resumeIdentity |  | Not checked |  |  |
| resumeMAC-I |  | Not checked |  |  |
| resumeCause |  | Not checked |  |  |
| spare |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 4.9.6.3	Switch off / Power off procedure in RRC_CONNECTED

Table 4.9.6.3-1: Switch off procedure in RRC_CONNECTED

| Step |  | Procedure |  | Message Sequence |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | U - S |  | Message |  |
| 1a1-1a3 |  | Void |  | - |  | - |  |  |
| 2-4 |  | Void |  | - |  | - |  |  |
| - |  | EXCEPTION: Steps 5a1 to 5b1 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if a particular implementation is under test. |  | - |  | - |  |  |
| 5a1 |  | IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM (Note 1) |  |  |  |  |  |  |
| - |  | EXCEPTION : Step 5a1Aa1 to 5a2Ab1 below specifies optional behaviour if the UE has previously performed IMS registration |  | - |  | - |  |  |
| 5a1Aa1 |  | The UE may perform the generic test procedure described in TS 34.229-5 [47] Annex A.11 |  | - |  | - |  |  |
| - |  | EXCEPTION: Steps 5a2Aa1a1 – 5a2Ab1 may be performed depending on UE implementation |  | - |  | - |  |  |
| - |  | EXCEPTION: Steps 5a2Aa1a1 - 5a2Aa1a2 may be repeated for all active PDU sessions |  | - |  | - |  |  |
| 5a2Aa1a1 |  | The UE transmits a PDU SESSION RELEASE REQUEST message. |  | --> |  | PDU SESSION RELEASE REQUEST |  |  |
| 5a2Aa1a2 |  | PDU session release procedure defined in clause 4.9.21 of TS 38.508-1 [4] is performed |  | - |  | - |  |  |
| 5a2Aa1a3 |  | The UE transmits a DEREGISTRATION REQUEST message. |  | --> |  | 5GMM: DEREGISTRATION REQUEST |  |  |
| 5a2Ab1 |  | The UE transmits a DEREGISTRATION REQUEST message. |  | --> |  | 5GMM: DEREGISTRATION REQUEST |  |  |
| 5a3 |  | The SS transmits an RRCRelease message. |  | <-- |  | NR RRC: RRCRelease |  |  |
| 5b1 |  | ELSE power off UE (Note 2) |  | - |  | - |  |  |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |  |  |  |  |  |

#### 4.9.6.3A	Switch off / Power off procedure in RRC_CONNECTED with T3540 started

Table 4.9.6.3A-1: Switch off procedure in RRC_CONNECTED with T3540 started

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |  |
| 1 | SS starts timer1 = T3540 (10 sec). | - | - |
| 2 | The SS locally releases the RRC connection. | - | - |
| 3 | SS waits for Timer1 expires. NOTE: On expiry of T3540 UE shall locally release the established N1 NAS signalling connection | - | - |
| 4 | The test procedure as described in Table 4.9.6.1-1: Switch off procedure in RRC_IDLE take place. | - | - |

#### 4.9.6.4	Switch off / Power off procedure in State DEREGISTERED

Table 4.9.6.4-1: Switch off procedure in State DEREGISTERED

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |  |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [36] pc_SwitchOnOff or [37] pc_USIM_Removal is supported | - | - |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE (Note 1) | - | - |
| 1b1 | ELSE power off UE (Note 2) | - | - |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal. |  |  |  |

#### 4.9.6.5	Switch off / Power off procedure in WLAN Ipsec_SA_Established

Table 4.9.6.5-1: Switch off procedure in Ipsec_SA_Established

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |  |
| - | EXCEPTION: Steps 1a1 to 1b1 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if [30] pc_SwitchOnOff or [31] pc_USIM_Removal is supported | - | - |
| 1a1 | IF pc_SwitchOnOff THEN switch off UE (Note 1) | - | - |
| 1a2 | The UE transmits a DEREGISTRATION REQUEST message. | --> | 5GMM: DEREGISTRATION REQUEST |
| 1a3 | The generic procedure for SS-requested IPsec Secure tunnel disconnection, specified in subclause 4.5A.5, takes place performing disconnection of security association | - | - |
| 1b1 | ELSE power off UE (Note 2) | - | - |
| Note 1: USIM removal is a feasible alternative to switch off UE. Note 2: Power off is used when UE don’t support switch off or USIM removal, in which case no UE originated deregistration procedure is expected. |  |  |  |