3GPP TSG RAN WG5#98	R5-230683

# Athens, Greece, 27 February - 3 March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0628 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Minimum test time for HD-FDD RMCs for RedCap test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Minimum test time is missing in Annex G for some HD-FDD RMCs for RedCap |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added minimum test time in table G.1.5-1 15 allocated slots per 2 frames, SCS 15 kHz => scaling factor = 20/15 = 1.3333 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RedCap WI cannot be finalized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### << Start of changes >>

## G.1.5	Minimum Test time

Editor's Note: Simulation method to derive minimum test time for FR2 needs to be evaluated.

If a pass fail decision in clause G.1.4 can be achieved earlier than the minimum test time, then the test shall not be decided, but continued until the minimum test time is elapsed.

The tables below contain the minimum number of slots for FDD and TDD.

By simulations the minimum number of active subframes (carrying DL payload) was derived (MNAS), then adding inactive subframes to the active ones. (for TDD additional subframes contain no DL payload), then rounding up to full thousand.

Simulation method to derive minimum test time:

With a level, corresponding a throughput at the test limit (here 30 % or 70 % of the max. throughput) the preliminary throughput versus time converges towards the final throughput. The allowance of ± 0.2 dB around the above mentioned level is predefined by RAN5 to find the minimum test time. The allowance of ±0.2 dB maps through the function "final throughput versus level" into a throughput corridor. The minimum test time is achieved when the preliminary throughput escapes the corridor the last time. The two functions "final throughput versus level" and "preliminary throughput versus time" are simulation results, which are done individual for each demodulation scenario.

Figure G.1.5-1: Simulation method to derive minimum test time

Table G.1.5-1: Minimum Test time for PDSCH demodulation

| TDD UL-DL pattern |  | Reference Channel |  | Propagation condition |  | Demodulation scenario (doppler speed) |  | Minimum number of active subframes (MNAS) |  | MNAS to MNS Scaling factor (Note 3) |  | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NA |  | R.PDSCH.1-8.1 FDD |  | HST-750 |  | 750 Hz |  | 6000 (Note 1) |  | 1.0526 |  | 6400 |  |
| NA |  | R.PDSCH.1-8.2 FDD |  | HST-972 |  | 972 Hz |  | 6000 (Note 1) |  | 1.0526 |  | 6400 |  |
| NA |  | R.PDSCH.1-8.1 FDD |  | TDLC300-600 |  | 600 Hz |  | 8000 (Note 1) |  | 1.0526 |  | 9000 |  |
| NA |  | R.PDSCH.1-1.1 FDD |  | TDLB100-400 |  | 400 Hz |  | 10000 (Note 1) |  | 1.0526 |  | 11000 |  |
| NA |  | R.PDSCH.1-1.1 HD-FDD |  | TDLB100-400 |  | 400 Hz |  | 10000 (Note 1) |  | 1.3333 |  | 14000 |  |
| NA |  | R.PDSCH.1-1.2 FDD, R.PDSCH.1-2.1 FDD, R.PDSCH.1-5.1 FDD |  | TDLC300-100 |  | 100 Hz |  | 20000 (Note 1) |  | 1.0526 |  | 22000 |  |
| NA |  | R.PDSCH.1-1.2 HD-FDD |  | TDLC300-100 |  | 100 Hz |  | 20000 (Note 1) |  | 1.3333 |  | 27000 |  |
| NA |  | R.PDSCH.1-1.3 FDD, R.PDSCH.1-2.2 FDD, R.PDSCH.1-2.3 FDD, R.PDSCH.1-2.4 FDD, R.PDSCH.1-2.5 FDD, R.PDSCH.1-3.1 FDD, R.PDSCH.1-3.2 FDD, R.PDSCH.1-3.3 FDD, R.PDSCH.1-3.4 FDD, R.PDSCH.1-4.1 FDD, R.PDSCH.1-12.1 FDD, R.PDSCH.2-1.1 FDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.0526 |  | 79000 |  |
| NA |  | R.PDSCH.1-1.3 HD-FDD, R.PDSCH.1-1.4 HD-FDD, R.PDSCH.1-1.5 HD-FDD, R.PDSCH.1-2.1 HD-FDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.3333 |  | 100000 |  |
| NA |  | R.PDSCH.1-7.1 FDD, R.PDSCH.1-7.2 FDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.25 |  | 94000 |  |
| NA |  | R.PDSCH.1-8.3 FDD, R.PDSCH.1-13.1 FDD, R.PDSCH.1-13.2 FDD, R.PDSCH.1-13.3 FDD, R.PDSCH.1-13.4 FDD, R.PDSCH.1-13.5 FDD, R.PDSCH.1-14.1 FDD, R.PDSCH.1-14.2 FDD, R.PDSCH.1-14.3 FDD, R.PDSCH.1-14.4 FDD |  | HST-SFN |  | 870 Hz |  | 30000 (Note 1) |  | 1.0526 |  | 32000 |  |
| NA |  | R.PDSCH.1-8.4 FDD, R.PDSCH.1-15.1 FDD, R.PDSCH.1-15.2 FDD, R.PDSCH.1-15.3 FDD, R.PDSCH.1-15.4 FDD, R.PDSCH.1-15.5 FDD, R.PDSCH.1-16.1 FDD, R.PDSCH.1-16.2 FDD, R.PDSCH.1-16.3 FDD, R.PDSCH.1-16.4 FDD |  | HST-DPS |  | 870 Hz |  | 30000 (Note 1) |  | 1.0526 |  | 32000 |  |
| FR1.15-1 |  | R.PDSCH.1-1.1 TDD, R.PDSCH.1-1.2 TDD |  | TDLA30-10 |  | 10Hz |  | 75000 (Note 1) |  | 2.8571 |  | 215000 |  |
| FR1.30-1A |  | R.PDSCH.2-1.1 TDD, R.PDSCH.2-1.5 TDD |  | TDLB100-400 |  | 400 Hz |  | 10000 (Note 1) |  | 1.2903 |  | 13000 |  |
| FR1.30-1 |  | R.PDSCH.2-1.2 TDD, R.PDSCH.2-2.1 TDD, R.PDSCH.2-7.1 TDD, R.PDSCH.2-26.1 TDD |  | TDLC300-100 |  | 100 Hz |  | 20000 (Note 1) |  | 1.2903 |  | 26000 |  |
| FR1.30-1 |  | R.PDSCH.2-2.2 TDD, R.PDSCH.2-2.3 TDD, R.PDSCH.2-2.4 TDD, R.PDSCH.2-2.5 TDD, R.PDSCH.2-3.1 TDD, R.PDSCH.2-3.2 TDD, R.PDSCH.2-3.3 TDD, R.PDSCH.2-3.4 TDD, R.PDSCH.2-3.5 TDD, R.PDSCH.2-4.1 TDD, R.PDSCH.2-4.3 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.2903 |  | 97000 |  |
| FR1.30-1 |  | R.PDSCH.2-1.3 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.4815 |  | 112000 |  |
| FR1.30-2 |  | R.PDSCH.2-5.1 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.2903 |  | 97000 |  |
| FR1.30-2 |  | R.PDSCH.2-17.1 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 5 |  | 375000 |  |
| FR1.30-3 |  | R.PDSCH.2-6.1 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.4815 |  | 112000 |  |
| FR1.30-4 |  | R.PDSCH.2-9.1 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.2903 |  | 97000 |  |
| FR1.30-5 |  | R.PDSCH.2-11.1 TDD |  | TDLB100-400 |  | 400Hz |  | 10000 (Note 1) |  | 1.2903 |  | 13000 |  |
| FR1.30-6 |  | R.PDSCH.2-12.1 TDD |  | TDLB100-400 |  | 400Hz |  | 10000 (Note 1) |  | 1.2903 |  | 13000 |  |
| FR1.30-1 |  | R.PDSCH.2-10.1 TDD |  | HST-1000 |  | 1000 Hz |  | 15000 (Note 1) |  | 1.4815 |  | 23000 |  |
| FR1.30-1 |  | R.PDSCH.2-10.1 TDD |  | HST-1667 |  | 1667 Hz |  | 15000 (Note 1) |  | 1.4815 |  | 23000 |  |
| FR1.30-1 |  | R.PDSCH.2-10.4 TDD, R.PDSCH.2-19.1 TDD, R.PDSCH.2-19.2 TDD, R.PDSCH.2-19.3 TDD, R.PDSCH.2-19.4 TDD, R.PDSCH.2-19.5 TDD, R.PDSCH.2-20.1 TDD, R.PDSCH.2-20.2 TDD, R.PDSCH.2-20.3 TDD, R.PDSCH.2-20.4 TDD, R.PDSCH.2-20.5 TDD, R.PDSCH.2-21.1 TDD |  | HST-SFN |  | 1667 Hz |  | 30000 (Note 1) |  | 1.4815 |  | 45000 |  |
| FR1.30-1 |  | R.PDSCH.2-10.5 TDD, R.PDSCH.2-22.1 TDD, R.PDSCH.2-22.2 TDD, R.PDSCH.2-22.3 TDD, R.PDSCH.2-22.4 TDD, R.PDSCH.2-22.5 TDD, R.PDSCH.2-23.1 TDD, R.PDSCH.2-23.2 TDD, R.PDSCH.2-23.3 TDD, R.PDSCH.2-23.4 TDD, R.PDSCH.2-23.5 TDD, R.PDSCH.2-24.1 TDD |  | HST-DPS |  | 1667 Hz |  | 30000 (Note 1) |  | 1.4815 |  | 45000 |  |
| FR2.60-1 |  | R.PDSCH.4-1.1 TDD |  | TDLA30-75 |  | 75 Hz |  | 20000 (Note 2) |  | 1.33 |  | 27000 |  |
| FR2.120-1A |  | R.PDSCH.5-1.1 TDD |  | TDLC60-300 |  | 300 Hz |  | 10000 (Note 2) |  | 1.25 |  | 13000 |  |
| FR2.120-1 |  | R.PDSCH.5-2.1 TDD, R.PDSCH.5-2.2 TDD, R.PDSCH.5-2.3 TDD, R.PDSCH.5-3.1 TDD |  | TDLA30-300 |  | 300 Hz |  | 10000 (Note 2) |  | 1.25 |  | 13000 |  |
| FR2.120-1 |  | R.PDSCH.5-1.2 TDD |  | TDLA30-75 |  | 75 Hz |  | 20000 (Note 2) |  | 1.25 |  | 25000 |  |
| FR2.120-2 |  | R.PDSCH.5-4.1 TDD, R.PDSCH.5-5.1 TDD, R.PDSCH.5-5.2 TDD, R.PDSCH.5-6.1 TDD |  | TDLA30-75 |  | 75 Hz |  | 20000 (Note 2) |  | 1.33 |  | 27000 |  |
| FR2.120-1 |  | R.PDSCH.5-10.1 TDD |  | TDLD30-75 |  | 75 Hz |  | 20000 (Note 2) |  | 1.26 |  | 26000 |  |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table G.1.5-1a: Minimum Test time for PDSCH demodulation with 1% BLER

| TDD UL-DL pattern | Reference Channel | Propagation condition | Demodulation scenario (doppler speed) | Minimum number of active subframes (MNAS) | MNAS to MNS Scaling factor (Note 3) | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |
| --- | --- | --- | --- | --- | --- | --- |
| NA | R.PDSCH.1-11.1 FDD R.PDSCH.1-11.2 FDD | TDLA30-10 | 10 Hz | [200000] (Note 1) | 1.1111 | [223000] |
| FR1.30-1 | R.PDSCH.2-16.1 TDD R.PDSCH.2-16.2 TDD | TDLA30-10 | 10 Hz | [200000] (Note 1) | 1.6667 | [334000] |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed). Note 3: MNS/MNAS ratio decided by scheduling pattern (how much time is required to collect required number of active DL SFs). |  |  |  |  |  |  |

Table G.1.5-2: Minimum Test time for PDCCH demodulation

| Reference Channel | Demodulation scenario (doppler speed) | Minimum number of active subframes (MNAS) | MNAS to MNS Scaling factor (Note 3) | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |
| --- | --- | --- | --- | --- |
| R.PDCCH 1-1.1 FDD, R.PDCCH.1-1.3 FDD, R.PDCCH.1-2.1 FDD, R.PDCCH.1-2.2 FDD, R.PDCCH.1-2.3 FDD, R.PDCCH.1-2.4 FDD, R.PDCCH.1-2.5 FDD, R.PDCCH.1-2.6 FDD | 10, 100, 400 Hz | 100000 (Note 1) | 1.0526 | 106000 |
| R.PDCCH.2-1.1 TDD, R.PDCCH.2-1.2 TDD, R.PDCCH.2-2.1 TDD, R.PDCCH.2-1.3 TDD | 10, 100, 400 Hz | 100000 (Note 1) | 1.2903 | 130000 |
| R.PDCCH.5-1.1 TDD, R.PDCCH.5-1.2 TDD, R.PDCCH.5-1.3 TDD, R.PDCCH.5-2.1 TDD | 75, 300 Hz | 100000 (Note 2) | 1.25 | 130000 |
| R.PDCCH.1-2.4 FDD+ R.PDCCH.1-2.7 FDD | 10Hz | 100000 (Note 1) | 2 (Note 4) | 200000 |
| R.PDCCH.2-1.2 TDD+ R.PDCCH.2-1.4 TDD | 100 Hz | 100000 (Note 1) | 2.666 (Note 4) | 267000 |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots. Note 4: For power saving, MNS/MNAS ratio decided by scheduling pattern and DRX configuration, and is ratio of all slots in both DRX active time and DRX inactive time to DL slots in DRX active time. |  |  |  |  |

#### << End of changes >>