3GPP TSG-RAN5 Meeting #98	R5-230687

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3536 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of testcase 7.1.1.3.16.1 Correct Handling of UL grant DRB configured with survival time on split DRB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To verify UE conformance for services with survival time in case of split DRB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new subclause 7.1.1.3.16 and testcase 7.1.1.3.16.1 under the new subclause. Testcase 7.1.13.16.1 verifies triggering of the PDCP duplication in case for DRB configured with survival time in case of split DRB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE conformance for services with survival time in case of split DRB is not ensured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0320 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### Start of change

#### 7.1.1.3.16	Correct Handling of UL grant / DRB configured with survival time

##### 7.1.1.3.16.1	Correct Handling of UL grant / DRB configured with survival time / Split DRB

7.1.1.3.16.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state to a serving cell with running timeAlignmentTimer and having split DRB configured with survivalTimeStateSupport and PDCP duplication configured }

ensure that {

when { UE receives uplink grant addressed to CS-RNTI with NDI=1 for a logical channel associated with the split DRB }

then { UE activates PDCP duplication for all RLC entities of the split DRB }

}

7.1.1.3.16.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.321 clause 5.4.1 and 5.10. Unless otherwise stated, these are Rel-17 features.

[TS 38.321, clause 5.4.1]

If the MAC entity has a C-RNTI, a Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer or a running cg-SDT-TimeAlignmentTimer and for each grant received for this PDCCH occasion:

1>	else if an uplink grant for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	if a logical channel associated with a DRB configured with survivalTimeStateSupport is multiplexed in the MAC PDU stored in the HARQ buffer for the corresponding HARQ process:

4>	trigger activation of PDCP duplication for all configured RLC entities of the DRB.

[TS 38.321, clause 5.10]

If one or more DRBs are configured with PDCP duplication, the network may activate and deactivate the PDCP duplication for all or a subset of associated RLC entities for the configured DRB(s).

The PDCP duplication for all associated RLC entities for the configured DRB(s) is activated by:

-	receiving an uplink grant addressed to CS-RNTI with NDI=1 for a logical channel associated with the DRB configured with survivalTimeStateSupport, described in clause 5.4.1.

The MAC entity shall for each DRB configured with PDCP duplication:

1>	if activation of a PDCP duplication for all configured RLC entities is triggered for the DRB as specified in clause 5.4.1:

2>	indicate the activation of PDCP duplication for all configured RLC entities of the DRB to upper layers.

7.1.1.3.16.1.3	Test Description

7.1.1.3.16.1.3.1	Pre-test conditions

Pre test conditions as in clause 7.1.3.0 with execution conditions pc_NG_RAN_NR and Connectivity(NR-DC) with following exeptions.

System Simulator:

-	None.

UE:

-	None.

Preamble:

-	Generic procedure parameters: Bearers(MCG(s) and split).

7.1.1.3.16.1.3.2	Test procedure sequence

Table 7.1.1.3.16.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity. | <-- | PDCP DATA PDU | - | - |
| 2 | SS allocates an UL Grant with the CS-RNTI assigned to the UE with NDI=1 for the AM RLC primary entity. The SS UL grant allocation for the AM RLC secondary entity is not impacted. | <-- | (UL Grant (CS-RNTI and NDI=1)) | - | - |
| - | EXCEPTION: Steps 3 and 4 below may occur in any sequence | - | - | - | - |
| 3 | Check: Does UE transmit a PDCP Data PDU on the AM RLC primary entity? | --> | PDCP DATA PDU | 1 | P |
| 4 | Check: Does UE transmit a PDCP Data PDU on the AM RLC secondary entity? | --> | PDCP DATA PDU | 1 | P |

7.1.1.3.16.1.3.3	Specific message contents

Table 7.1.1.3.16.1.3.3-1: PDCP-Config (Preamble)

| Derivation Path: 38.508-1 [4], Table 4.6.3-99 with condition NR_split |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| survivalTimeStateSupport-r17 | true |  |  |
| } |  |  |  |

#### End of change