3GPP TSG-RAN5 Meeting #98	R5-230733

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1412 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of condition description in Interworking_with_5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As PCO and ePCO IEs are to be included by UE in ESM INFORMATION RESPONSE and PDN CONNECTIVITY REQUEST messages, the description of Interworking_with_5GS needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SS was replaced by UE to include the PCO and ePCO IEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity will exist in spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

4.7.3	Contents of ESM messages

-	ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT

[Skip unchanged text]

-	ESM INFORMATION RESPONSE

This message is sent by the UE to the SS.

Table 4.7.3-14: ESM INFORMATION RESPONSE

| Derivation Path: 24.301 clause 8.3.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | '0000'B | No EPS bearer identity assigned |  |
| Procedure transaction identity |  | The same value as the value set in ESM INFORMATION REQUEST message. |  |  |
| ESM information response message identity |  | '1101 1010'B | ESM information response |  |
| Access point name |  | Not present or any allowed value |  | Internet_APN |
| Access point name |  | Not present |  | IMS_APN_Default |
| Access point name |  | Any allowed value |  | IMS_APN_Provided |
| Protocol configuration options |  | Not present or any allowed value |  | NOT (pc_NB or pc_NonIP_PDN) |
| Protocol configuration options |  | If present including at least the following container |  | Interworking_with_5GS |
| Container ID n |  | ‘001A’H | n assigned to next available number |  |
| Length of container ID n contents |  |  | 1 octet |  |
| Container ID n contents |  | PDU session ID | PDU session ID assigned by the UE |  |
| Protocol configuration options |  | Not present |  | pc_NB or pc_NonIP_PDN |
| Extended protocol configuration options |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | If present including at least the following container |  | Interworking_with_5GS |
| Container ID n |  | ‘001A’H | n assigned to next available number |  |
| Length of container ID n contents |  |  | 1 octet |  |
| Container ID n contents |  | PDU session ID | PDU session ID assigned by the UE |  |

| Condition | Expression | Explanation |
| --- | --- | --- |
| IMS_APN_Provided | pc_IMS AND NOT pc_Provide_IMS_as_second_APN AND pc_Provide_IMS_APN | IMS APN shall be provided by the UE: |
| IMS_APN_Default | pc_IMS AND NOT pc_Provide_IMS_as_second_APN AND NOT pc_Provide_IMS_APN | No IMS APN shall be provided by the UE; IMS APN is default APN |
| Internet_APN | NOT pc_IMS OR pc_Provide_IMS_as_second_APN |  |
| Interworking_with_5GS |  | If the UE has indicated support of N1, then the UE shall include this IE to provide details for the interworking with 5GS. Applicable to 5G test cases defined in TS 38.523-1 [71]. Note: The PDU session ID container shall be sent in either the Protocol configuration options or the Extended protocol configuration options IE. |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

[Skip unchanged text]

-	PDN CONNECTIVITY REQUEST

This message is sent by the UE to the SS.

Table 4.7.3-20: PDN CONNECTIVITY REQUEST

| Derivation Path: 24.301 clause 8.3.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | '0000'B | No EPS bearer identity assigned |  |
| Procedure transaction identity |  | Any value from 1 to 254 |  |  |
| PDN connectivity request message identity |  | '1101 0000'B | PDN connectivity request |  |
| Request type |  | '0001'B | initial request |  |
| PDN type |  | Any value between '001'B, '010'B, '011'B and '100'B | The allowed values are respectively IPv4, IPv6, IPv4v6 and "unused but interpreted as IPv6 by the network" |  |
| PDN type |  | ‘101’B | Non-IP | CIoT_CP_NON_IP |
| ESM information transfer flag |  | Not present or any allowed value |  |  |
| Access point name |  | Not present | The value is mandatory when the PDN CONNECTIVITY REQUEST message is sent together with an ATTACH REQUEST message. |  |
| Protocol configuration options |  | Not present or any allowed value | The value received from the UE does not affect the possible verdict associated with the message when received by the SS. The SS shall remember if this IE is present and its contents because this affects subsequent SS behaviour, e.g. coding of ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST. | NOT (pc_NB or pc_NonIP_PDN) |
| Protocol configuration options |  | Not present |  | pc_NB or pc_NonIP_PDN |
| Protocol configuration options |  | If present including at least the following container |  | Interworking_with_5GS |
| Container ID n |  | ‘001A’H | n assigned to next available number |  |
| Length of container ID n contents |  |  | 1 octet |  |
| Container ID n contents |  | PDU session ID | PDU session ID assigned by the UE |  |
| NBIFOM container |  | Not present or any allowed value |  |  |
| Header compression configuration |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | If present including at least the following container |  | Interworking_with_5GS |
| Container ID n |  | ‘001A’H | n assigned to next available number |  |
| Length of container ID n contents |  |  | 1 octet |  |
| Container ID n contents |  | PDU session ID | PDU session ID assigned by the UE |  |

| Condition | Explanation |
| --- | --- |
| CIoT_CP_NON_IP | If the UE requests a Non-IP type PDN |
| Interworking_with_5GS | If the UE has indicated support of N1, then the UE shall include this IE to provide details for the interworking with 5GS. Applicable to 5G test cases defined in TS 38.523-1 [71]. Note: The PDU session ID container shall be sent in either the Protocol configuration options or the Extended protocol configuration options IE. |

NOTE:	This message is sent without integrity protection before NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED NAS MESSAGE message after NAS security mode control procedure has been successfully completed.

[End of change]