3GPP TSG-RAN5 Meeting #98	R5-230742

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0436 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Handover Capabilities for 5GC-N3IWF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New Mobility Capabilities is needed for some 5GC-N3IWF test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Mobility Capabilities for 5GC-N3IWF test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Mobility Capabilities for 5GC-N3IWF test cases will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.523-2 R5-230613 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

A.4.3.8	Mobility Capabilities

Table A.4.3.8-1: UE Mobility Capabilities

| Item |  |  | UE Mobility Capabilities |  |  | Ref. |  |  | Release |  |  | Mnemonic |  |  | M |  |  | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release |  |  | Comments |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | Support inter-RAT Handover to EUTRA connected to EPC |  |  | 38.306, 4.2.9 |  |  | Rel-15 |  |  | pc_interRAT_EUTRA_Handover |  |  | Yes |  |  |  |  |  |  |  |  |
| 2 |  |  | Support inter-frequency Handover from the corresponding duplex mode or from the corresponding frequency range. |  |  | 38.306, 4.2.9 |  |  | Rel-15 |  |  | pc_handoverInterF |  |  | Yes |  |  |  |  |  |  |  |  |
| 3 |  |  | Support Handover between FR1 and FR2 |  |  | 38.306, 4.2.9 |  |  | Rel-15 |  |  | pc_FR1toFR2_Handover |  |  | Yes |  |  |  |  |  |  |  |  |
| 4 |  |  | Support Handover between FDD and TDD |  |  | 38.306, 4.2.9 |  |  | Rel-15 |  |  | pc_FDDtoTDD_Handover |  |  | Yes |  |  |  |  |  |  |  |  |
| 5 |  |  | Support inter-RAT Handover to E-UTRA connected to 5GC |  |  | 38.306, 4.2.9 |  |  | Rel-15 |  |  | pc_interRAT_eLTE_Handover |  |  | Yes |  |  |  |  |  |  |  |  |
| 6 |  |  | Support inter-RAT Handover to NR FR1 TDD from EUTRA connected to EPC |  |  | 36.306, 4.3.34.9 |  |  | Rel-15 |  |  | pc_eutra_EPC_HO_ToNR_TDD_FR1_r15 |  |  | Yes |  |  |  |  |  |  |  |  |
| 7 |  |  | Support inter-RAT Handover to NR FR1 FDD from EUTRA connected to EPC |  |  | 36.306, 4.3.34.8 |  |  | Rel-15 |  |  | pc_eutra_EPC_HO_ToNR_FDD_FR1_r15 |  |  | Yes |  |  |  |  |  |  |  |  |
| 8 |  |  | Support inter-RAT Handover to NR FR2 TDD from EUTRA connected to EPC |  |  | 36.306, 4.3.34.11 |  |  | Rel-15 |  |  | pc_eutra_EPC_HO_ToNR_TDD_FR2_r15 |  |  | Yes |  |  |  |  |  |  |  |  |
| 9 |  |  | Support intra-frequency DAPS handover |  |  | 38.306, 4.2.7.5 |  |  | Rel-16 |  |  | pc_intraFreqDAPS_r16 |  |  | No |  |  |  |  |  | It is mandated if the UE supports asynchronous intra-frequency DAPS handover |  |  |
| 10 |  |  | Support inter-RAT Handover from NR to EN-DC |  |  | 38.306, 4.2.10 |  |  | Rel-16 |  |  | pc_interRAT_NR_ToENDC |  |  | CY |  |  |  |  |  | It is mandated if the UE supports EN-DC. |  |  |
| 11 |  |  | Support conditional handover |  |  | 38.306, 4.2.7.2 |  |  | Rel-16 |  |  | pc_condHandover_r16 |  |  | No |  |  |  |  |  |  |  |  |
| 12 |  |  | Support conditional handover during re-establishment procedure when the selected cell is configured as candidate cell for condition handover |  |  | 38.306, 4.2.7.2 |  |  | Rel-16 |  |  | pc_condHandoverFailure_r16 |  |  | No |  |  |  |  |  |  |  |  |
| 13 |  |  | Support 2 trigger events for same execution condition of conditional handover |  |  | 38.306, 4.2.7.2 |  |  | Rel-16 |  |  | pc_condHandoverTwoTriggerEvents_r16 |  |  | CY |  |  |  |  |  | It is mandated if the UE supports condHandover-r16. |  |  |
| 14 |  |  | Support inter-RAT Handover from NR to UTRA-FDD CELL_DCH CS |  |  | 38.306, 4.2.9 |  |  | Rel-16 |  |  | pc_handoverUTRA_FDD_r16 |  |  | No |  |  |  |  |  |  |  |  |
| 15 |  |  | Support inter-frequency DAPS handover |  |  | 38.306, 4.2.7.4 |  |  | Rel-16 |  |  | pc_interFreqDAPS_r16 |  |  | No |  |  |  |  |  | It is mandated if the UE supports asynchronous inter-frequency DAPS handover or supports different SCSs in source PCell and inter-frequency target PCell in DAPS handover |  |  |
| 16 |  |  | UE supports asynchronous intra-frequency DAPS handover |  |  | 38.306, 4.2.7.5 |  |  | Rel-16 |  |  | pc_intraFreqAsyncDAPS_r16 |  |  | No |  |  |  |  |  |  |  |  |
| 17 |  |  | UE supports asynchronous inter-frequency DAPS handover |  |  | 38.306, 4.2.7.5 |  |  | Rel-16 |  |  | pc_interFreqAsyncDAPS_r16 |  |  | No |  |  |  |  |  |  |  |  |
| 18 |  |  | UE supports different SCSs in source PCell and inter-frequency target PCell in DAPS handover |  |  | 38.306, 4.2.7.5 |  |  | Rel-16 |  |  | pc_ interFreqDiffSCS-DAPS_r16 |  |  | No |  |  |  |  |  |  |  |  |
| 19 |  |  | Support conditional PSCell change |  |  | 38.306, 4.2.7.2 |  |  | Rel-16 |  |  | pc_condPSCellChange_r16 |  |  | No |  |  |  |  |  |  |  |  |
| 20 |  |  | Support handover from 5GS to EPC/ePDG |  |  | 24.302, 7.2.2.1 |  |  | Rel-15 |  |  | pc_HO_from_5GS_to_EPC_ePDG |  |  | No |  |  |  |  |  |  |  |  |
| 21 |  |  | Support handover from EPC/ePDG to 5GS |  |  | 23.502, 4.11.4.1 |  |  | Rel-15 |  |  | pc_HO_from_EPC_ePDG_to_5GS |  |  | No |  |  |  |  |  |  |  |  |
| xx |  |  | Support Handover from EPS to 5GC-N3IWF |  |  | 23.502, 4.11.3.1 |  |  | Rel-15 |  |  | pc_HO_from_EPS_to_5GC-N3IWF |  |  | No |  |  |  |  |  |  |  |  |
| xy |  |  | Support Handover from 5GC-N3IWF to EPS |  |  | 23.502, 4.11.3.2 |  |  | Rel-15 |  |  | pc_HO_from_5GC-N3IWF _to_EPS |  |  | No |  |  |  |  |  |  |  |  |
| yx |  |  | Support Handover of a PDU Session procedure from untrusted non-3GPP to 3GPP access |  |  | 23.502, 4.9.2.1 |  |  | Rel-15 |  |  | pc_HO_from_5GC-N3IWF _to_5GC |  |  | No |  |  |  |  |  |  |  |  |
| yy |  |  | Support Handover of a PDU Session procedure from 3GPP to untrusted non-3GPP access |  |  | 23.502, 4.9.2.2 |  |  | Rel-15 |  |  | pc_HO_from_5GC _to_5GC-N3IWF |  |  | No |  |  |  |  |  |  |  |  |