3GPP TSG-RAN5 Meeting #98	R5-230787

# Athens, Greece, 27th Feb 2023 – 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2243 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of CG-SDT test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CG-SDT test case 6.2.1 is missing from the test specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added minimum requirement section 6.2.0. - Added CG-SDT test 6.2.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case would be missing from test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.0, 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

## 6.2	RRC_INACTIVE state mobility

### 6.2.0	General

The requirements in this clause are applicable when the UE is configured with timing alignment (TA) validation using cg-SDT-RSRP-ChangeThreshold for transmitting in uplink using CG-SDT as specified in TS 38.331 [13].

The requirements in this clause are applicable for the UE performing small data tranmissions using configured resources as per TS 38.331 [13].

The UE is allowed to transmit using the configured uplink resources provided that the UE is synchronized towards (i.e. using the timing derived using the latest available NTA value as specified in 38.133 [6] clause 7.1.2) the serving cell prior to transmission. If the UE is not able to obtain the synchronization towards the serving cell then the UE shall drop the small data transmission. The UE determines the small data transmission occasion according to the received CG-SDT configuration TS 38.331 [13].

#### The normative reference for this requirement is TS 38.133 [6] clauses 5.5.1 and 5.5.2.

#### 6.2.0.1	Minimum conformance requirements for TA validation using CG-SDT

When cg-SDT-RSRP-ChangeThreshold is configured for TA validation based on the RSRP change criterion according to clause 5.8.2 in TS 38.321 [12], the UE is allowed to transmit using CG-SDT using the timing derived using the latest available NTA value as specified in TS 38.133 [6] clause 7.1 provided that

-	the first RSRP (RSRP1) measurement and the second RSRP (RSRP2) measurements used in the TA validation are valid measurements and,

-	timing alignment validation for transmission using CG-SDT is valid according to the validation criteria in clause 5.8.2 in TS 38.321 [12].

RSRP1 and RSRP2 are considered valid provided that the conditions in Table 6.2.0.1-1 are met for FR1.

Table 6.2.0-1 Valid measurement for FR1

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(640ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(640ms, M1*TDRX)) |
| RSRP2 | (T2 – min(640ms, M1*TDRX)) ≤ T2’ ≤ T2 |

If at least one of RSRP1 and RSRP2 is considered to be invalid based on the above conditions, then the UE shall not validate the CG-SDT using RSRP1 and RSRP2 and shall not transmit using CG-SDT. Additionally, the UE shall not transmit in an CG-SDT occasion that occurs more than 640 ms after T2.

Where:

-	T1 is the time when

-	RRCRelease with CG-SDT configuration (TS 38.331 [13]) is received

-	the latest TA is received if TA is received while in RRC_INACITVE state.

-	T1’ is the time when the UE has completed RSRP1.

-	T2 is the time when the UE performs TA validation as defined in clause 5.27.2 in (TS 38.321 [12]) for transmission using CG-SDT.

-	T2’ is the time when the UE has completed RSRP2.

-	TDRX is the DRX cycle length in ms.

-	M1 the scaling factor as defined in TS 38.133 [6] clause 4.2.2.2.

The normative reference for this requirement is TS 38.133 [6] clause 5.5.3.

### 6.2.1	NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

Editor's Note:

- MU and TT analysis is incomplete

- Message contents are either FFS or need to be updated

- Multiple aspects of call setup and test procedure need to be updated

- Applicability needs to be updated

- Some test parameters are still in brackets

#### 6.2.1.1	Test Purpose

The purpose of this test is to verify that the UE properly perform TA validation for CG-SDT transmission in clause 6.2.0.1.

#### 6.2.1.2	Test Applicability

This test applies to all types of NR UE [release 17 and forward], supporting NR SA and [CG-SDT].

#### 6.2.1.3	Minimum Conformance Requirement

The minimum requirements are specified in clause 6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.2.1.

#### 6.2.1.4	Test Description

In the test, UE is configured two CG-SDT configurations at different time. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 repectively. There is one cell, which is the active NR cell in FR1.  Figure 6.2.1.4-1 shows the variation of the RSRP per time. Prior to the time point TA, the UE shall be fully synchronized to PCell (Cell 1).

Figure 6.2.1.4-1: RSRP variation model for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

Before starting the test at time point TA, test equipment configures RSRP to P0.

At time point TB, RSRP is changed from P0 to P1.

At time point TC, UE expect to receive RRC release with CG SDT configuration and RRC status is changed to INACTIVE status.

At time point TD, RSRP is changed from P1 to P0.

At time point TE, RSRP is changed from P0 to P2.

Test equipment triggers UL data arrival at UE lower layer at time point TF.

At time point TH, UE expect to receive a RRC release with CG-SDT configurations.

At time point TI, RSRP is changed from P2 to P3.

Test equipment triggers UL data arrival at UE lower layer at time point TJ.

6.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.2.1.4.1-1.

Table 6.2.1.4.1-1: NR configuration for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Config | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.2.1.4.1-2.

Table 6.2.1.4.1-2: Initial conditions for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.2.1.4.3.

2.	The power levels and settings for Cell 1 are set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 6.2.1.4.1-3. Supplementary timeline events are given in Table 6.2.1.4.1-4.

Table 6.2.1.4.1-3 : General test parameters for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| TDD Configuration |  | 1 | N/A |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| BWchannel | MHz |  | 10: NRB,c = 52 |  |
| PDSCH Reference measurement channel |  | 1 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | 1 | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| SSB configuration |  | 1,2,3 | SSB.1 FR1 |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  | 1,2,3 | SMTC.1 |  |
| DRX configuration |  | 1,2,3 | DRX.7 |  |
| T1 | s | 1,2,3 | [1] |  |
| T2 | s | 1,2,3 | [1.28] |  |
| T3 | s | 1,2,3 | [2.36] |  |
| T4 | s | 1,2,3 | [2.56] |  |
| T5 | s | 1,2,3 | [2.36] |  |
| cg-SDT-RSRP-ChangeThreshold | dB | 1,2,3 | [8] |  |
| cg-SDT-TimeAlignmentTime |  | 1,2,3 | infinity |  |
| CG-SDT resource period | ms |  | 640 | configuredGrantConfig Peridocity at 38.331 [13] Config 1,2 : sym640x14, config 3 : sym1024x14 |
| Propagation condition |  | 1,2,3 | AWGN |  |

Table 6.2.1.4.1-4 : Supplementary timeline event derivations for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Time Period |  | Unit |  |  | Test configuration |  |  | Value |  |  | Derivation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T_delay_modeB | ms |  |  | 1,2,3 |  |  | [4000] |  |  |  | Test configuration |
| W1 | ms |  |  | 1,2,3 |  |  | [640] |  |  |  | W1 = T2/2 (see Table 6.2.0-1) |
| TAtoTB |  | ms |  |  | 1,2,3 |  |  | [1000] |  |  | TAtoTB = T1 |
| TBtoTC |  | ms |  |  | 1,2,3 |  |  | [640] |  |  | TBtoTC = W1 |
| TCtoTD |  | ms |  |  | 1,2,3 |  |  | [640] |  |  | TCtoTD = W1 |
| TDtoTE |  | ms |  |  | 1,2,3 |  |  | [2360] |  |  | TDtoTE = T3 |
| TEtoTF |  | ms |  |  | 1,2,3 |  |  | [1000] |  |  | TEtoTF = T_delay_modeB – TDtoTE – TCtoTD = T_delay_modeB – T3 – W1 |
| TFtoTG |  |  | ms |  |  | 1,2,3 |  |  | [480] |  | TFtoTG = 640*k – T_delay_modeB, where k = ceil(argmin(640*k > T_delay_modeB)) |
| TGtoTH |  | ms |  |  | 1,2,3 |  |  | [440] |  |  | TGtoTH = T4 – TEtoTF – TFtoTG – W1 |
| THtoTI | ms |  |  | 1,2,3 |  |  | [640] |  |  | THtoTI = W1 |  |
| TItoTJ | ms |  |  | 1,2,3 |  |  | >[1720] |  |  | TItoTK = T5 – TJtoTK |  |
| TJtoTK | ms |  |  | 1,2,3 |  |  | <640 |  |  | TJtoTK < CG-SDT resource periodicity = 640ms |  |
| Note 1: These timeline derivations are useful to understand different timeline events other than T1, T2, T3, T4 and T5. Calculations are informative. |  |  |  |  |  |  |  |  |  |  |  |

6.2.1.4.2	Test Procedure

There is one cell in the test, the FR1 PCell, which is the active NR cell. The test parameters for the PCell are given in Table 6.2.1.4.1-1, Table 6.2.1.4.1-3, and Table 6.2.1.5-1. Prior to the start of the test, the UE shall be fully synchronized to PCell (Cell 1). According to Table 6.2.1.5-1, in time intervals T1 to T5 the SS configures the SS-RSRP by fixing Noc and adjusting SNR correspondingly.

1.	Ensure the UE is in [State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On] [and UE test loop mode B is activated and closed] according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

2.	Set the parameters of NR Cell according to T1 in Table 6.2.1.5-1. Propagation conditions are set according to clause C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 6.2.1.5-1. T2 starts.

4.	W1=[640]ms seconds after the start of T2 (time event TC), the SS shall transmit [2 PDCP SDUs (which will get buffered at the UE)] immediately followed by an RRCRelease message containing CG-SDT configuration for the UE to change its RRC state to RRC_INACTIVE, according to section 6.2.1.4.3.

5.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 6.2.1.5-1. T3 starts.

6.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 6.2.1.5-1. T4 starts.

7.	T_delay_modeB expires TEtoTF seconds after the start of T4 (time event TF). If the UE [changes its RRC state to RRC_CONNECTED] and transmits PUSCH at the next CG-SDT resource within 640ms + Z after T_delay_modeB expiration (time event TG), then the number of successful tests for CG-SDT transmit event is increased by one. Otherwise, the number of failed tests for CG-SDT transmit event is increased by one and proceed with step 11.

8.	W1=[640]ms before the end of T4 (time event TH), the SS shall send an RRCRelease message to the UE with CG-SDT configuration for the UE to change its RRC state to RRC_INACTIVE, according to section 6.2.1.4.3.

9.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 6.2.1.5-1. T5 starts.

10.	TItoTJ seconds after the start of T5 [the second PDU becomes eligible for transmission]. If the UE [changes its RRC state to RRC_CONNECTED] and transmits PUSCH at CG-SDT resource [between TJ and until T5 expires] (time event TK), then the number of failed tests for CG-SDT don’t transmit event is increased by one and proceed with step 11. Otherwise, the number of successful tests for CG-SDT don’t transmit event is increased by one and proceed with step 12.

11.	The SS shall switch off and then on the UE and then proceed with step 12.

12.	Repeat steps 1-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.2.1.4.3	Message Contents

Editor's Note: Message contents specific to CG-SDT are still FFS

Message contents are according to TS 38.508-1 [14] clauses 4.6.1 and 7.3 with the following exceptions:

Table 6.2.1.4.3-1: Common Exception messages for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | 38.508-1 [14] Table 4.6.1-16 with conditions NR_RRC_INACTIVE and SDT |

Table 6.2.1.4.3-2: SDT-MAC-PHY-CG-Config-r17 for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Derivation Path: TS 38.508-1 [6], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark |  |  | Comment | Condition |  |
| SDT-MAC-PHY-CG-Config-r17::= SEQUENCE { |  |  |  |  |  |  |
| cg-SDT-ConfigLCH-RestrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF CG-SDT-Config-LCH-restriction-r17 { |  | [FFS] |  |
| logicalChannelIdentity-r17 | LogicalChannelIdentity with condition DRB1 |  |  |  |  |  |
| configuredGrantType1Allowed-r17 | true |  |  |  |  |  |
| allowedCG-List-r17 | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 CHOICE { |  |  |  |
| setup |  | BWP-UplinkDedicatedSDT-r17 |  |  |  |  |
| } |  |  |  |  |  |  |
| cg-SDT-ConfigInitialBWP-SUL-r17 | Not present |  |  |  |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 |  |  |  |  |  |  |
| setup | BWP-DownlinkDedicatedSDT-r17 |  |  |  |  |  |
| } |  |  |  |  |  |  |
| cg-SDT-TimeAlignmentTimer-r17 | infinity |  |  |  |  |  |
| cg-SDT-RSRP-ThresholdSSB-r17 | FFS |  |  |  |  |  |
| cg-SDT-TA-ValiditationConfig-r17 SEQUENCE { |  |  |  |  |  |  |
| cg-SDT-RSRP-ChangeThreshold-r17 | dB8 |  |  |  |  |  |
| } |  |  |  |  |  |  |
| cg-SDT-CS-RNTI-r17 | FFS |  |  |  |  |  |
| } |  |  |  |  |  |  |

Table 6.2.1.4.3-3: BWP-UplinkDedicatedSDT-r17 for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicatedSDT-r17::= SEQUENCE { |  |  |  |
| pusch-Config-r17 CHOICE { |  |  |  |
| setup | PUSCH-Config |  |  |
| } |  |  |  |
| configuredGrantConfigToAddModList-r17 SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfig | 1 entry |  |  |
| } |  |  |  |

Table 6.2.1.4.3-4: ConfiguredGrantConfig for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| periodicity | sym640x14 |  | Config 1,2 |
|  | sym1040640x14 |  | Config 3 |
| } |  |  |  |

#### 6.2.1.5	Test Requirement

Table 6.2.1.5-1 defines the cell specific primary level settings including test tolerances for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT).

Table 6.2.1.5-1: Cell-specific test parameters for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

| Parameter | Config | Unit | SSB#0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
|  | 1,2,3 | dBm/15kHz | -100+TT |  |  |  |  |
|  | 1,2 | dBm/SSB SCS | -100+TT |  |  |  |  |
|  | 3 | dBm/SSB SCS | -103+TT |  |  |  |  |
|  | 1,2,3 | dB | [0]+TT | [10.35]+TT | [0]+TT | [15.25]+TT | [6]+TT |
| SS RSRP | 1,2,3 | dBm/SSB SCS | [-100]+TT | [-89.65]+TT | [-100]+TT | [-84.75]+TT | [-96]+TT |
| Io | 1,2 | dBm/9.36MHz | [-69.04]+TT | [-61.31]+TT | [-69.04]+TT | [-56.67]+TT | [-65.07]+TT |
|  | 3 | dBm/38.16MHz | [-69.04]+TT | [-61.31]+TT | [-69.04]+TT | [-56.67]+TT | [-65.07]+TT |
|  | 1,2,3 | dB | [0]+TT | [10.35]+TT | [0]+TT | [15.25]+TT | [6]+TT |

The UE behaviour in each test during time durations shall be as follows:

During T4, UE shall transmit PUSCH at CG-SDT resource within 640ms + Z after time point TF.

During T5, UE shall not transmit PUSCH at CG-SDT resources after TJ until the end of the test at time point TK.

The rate of correct events observed during repeated tests shall be at least 90%.

<< End of Changes >>