3GPP TSG-RAN5 Meeting #98	R5-230811

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0892 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to test applicability of MPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh-UEConTest |  |  |  |  |  | Date: |  |  |  |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During RAN5#95-e, an LS was sent to RAN4 to seek for clarification of enhance MPR applicability. An reply LS R5-225920 was received in RAN5#96. In this meeing, an following up LS R5-230022 was received. This CR updates the applicability of MPR and enhance MPR testing to reflect the RAN4 agreements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The capability doesn’t apply to PC2, 4 UEs. For Rel-15 PC2, 4 UEs, default MPR requirements apply. For Rel-16 and forward PC1, 2, 4 UEs, enhanced MPR requirements apply. The capability apply to Rel-15 and Rel-16 PC3 UEs. For Rel-15/16 PC3 UEs, default MPR requirements if UE doesn’t support modifiedMPRbehaviour bit 0; otherwise the enhanced MPR requirements apply. The capability doesn’t apply to Rel-17 PC3 UEs. For Rel-17 and forward PC3 UEs, enhanced MPR requirements apply. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The applicability of MPR requirements remain unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.2.2_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2.2	UE maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for PC1, PC2 and PC4.

- Measurement grid for PC2/4 in Annex M.4 is FFS.

- Once test case 6.2.2_1 is available for certification, the test applicability needs an update to exclude Rel15 UE supporting modifiedMPR bit 0 and Rel16 and later UE.

6.2.2.0	General

The requirements in section 6.2.2 only apply when both UL and DL of a UE are configured for single CC operation, and they are of the same bandwidth. A UE may reduce its maximum output power due to modulation orders, transmit bandwidth configurations, waveform types and narrow allocations. This Maximum Power Reduction (MPR) is defined in subclauses below. The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation. When the maximum output power of a UE is modified by MPR, the power limits specified in subclause 6.2.4 apply.

For a UE that is configured for single CC operation with different channel bandwidths in UL and DL, the requirements in section 6.2A.2 apply.

For all power classes, the waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition.

6.2.2.1	Test purpose

The number of RB identified in 6.2.2.3 is based on meeting the requirements for the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2.2.2	Test applicability

The requirements of this test apply to all types of NR Power Class 1 UE release 15 and forward.

The requirements of this test apply to all types of NR Power Class 2 and Power Class 4 UE release 15.

The requirements of this test apply to all types of NR Power Class 3 UE release 15 and release 16 which doesn’t supports modifiedMPRbehaviour bit 0 capability (according to Annex P.1).

6.2.2.3	Minimum conformance requirements

6.2.2.3.1	UE maximum output power reduction for power class 1

For power class 1, MPR for contiguous allocations is defined as:

MPR = max(MPRWT, MPRnarrow)

Where,

MPRnarrow = 14.4 dB, when BWalloc,RB ≤ 1.44 MHz, MPRnarrow = 10 dB, when 1.44 MHz < BWalloc,RB ≤ 10.8 MHz, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined in Tables 6.2.2.3.1-1 and 6.2.2.3.1-2.

<Unchanged Text Skipped>

6.2.2_1	UE maximum output power reduction enhancements

6.2.2_1.0	General

The requirements in section 6.2.2_1 only apply when both UL and DL of a UE are configured for single CC operation, and they are of the same bandwidth. A UE may reduce its maximum output power due to modulation orders, transmit bandwidth configurations, waveform types and narrow allocations. This Maximum Power Reduction (MPR) is defined in subclauses below. The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation. When the maximum output power of a UE is modified by MPR, the power limits specified in subclause 6.2.4 apply.

For a UE that is configured for single CC operation with different channel bandwidths in UL and DL, the requirements in section 6.2A.2 apply.

For all power classes, the waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition.

6.2.2_1.1	Test purpose

The number of RB identified in 6.2.2_1.3 is based on meeting the requirements for the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2.2_1.2	Test applicability

The requirements of this test apply to all types of NR Power Class 2 and Power Class 4 UE release 16 and forward.

The requirements of this test apply to all types of NR Power Class 3 UE release 15 and release 16 which supports modifiedMPRbehaviour bit 0 capability (according to Annex P.1).

The requirements of this test apply to all types of NR Power Class 3 UE release 17 and forward.

6.2.2_1.3	Minimum conformance requirements

6.2.2_1.3.1	Void

6.2.2_1.3.2	UE maximum output power reduction for power class 2

For power class 2, MPR for FR2-1 and FR2-2 as specified in clause 6.2.2_1.3.3 applies.

6.2.2_1.3.3	UE maximum output power reduction for power class 3

For transmission bandwidth configuration less than or equal to 200MHz, and 0 ≤ RBstart < Ceil(1/3 NRB) or Ceil((2/3NRB) LCRB) < RBstart ≤ NRB-LCRB:

-	MPRnarrow = 2.5 dB, when BWalloc,RB is less than or equal to 1.44 MHz,

-	MPRnarrow = 2.0 dB, when 1.44 MHz < BWalloc,RB <= 4.32 MHz,

-	otherwise MPRnarrow = 0 dB.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined in Table 6.2.2_1.3.3-1.

Table 6.2.2_1.3.3-1 MPRWT for power class 3, BWchannel ≤ 200 MHz

| Modulation |  | MPRWT, BWchannel ≤ 200 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 2.0 |
|  | QPSK | 0.0 | ≤ 2.0 |
|  | 16 QAM | ≤ 3.0 | ≤ 3.5 |
|  | 64 QAM | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 5.0 | ≤ 5.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 7.5 |

Where the following parameters are defined to specify valid RB allocation ranges for RB allocations in Table 6.2.2_1.3.3-1:

-	RBStart,Low = max(1, LCRB), where max() indicates the largest value of all arguments.

-	RBStart,High = NRB – RBStart,Low – LCRB,

An RB allocation belonging to table 6.2.2_1.3.3-1 is a Region 1 inner RB allocation if:

-	RBStart,Low  ≤  RBStart  ≤  RBStart,High, and LCRB  ≤  ceil(NRB/3), where ceil(x) is the smallest integer greater than or equal to x.

For transmission bandwidth configuration equal to 400MHz,

MPRnarrow = 2.5 dB, when BWalloc,RB is less than or equal to 1.44 MHz, and 0 ≤ RBstart < Ceil(1/3 NRB) or Ceil(2/3NRB) ≤ RBstart ≤ NRB-LCRB, where BWalloc,RB is the bandwidth of the RB allocation size.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined in Table 6.2.2_1.3.3-2.

Table 6.2.2_1.3.3-2 MPRWT for power class 3, BWchannel = 400 MHz

| Modulation |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 3.0 |
|  | QPSK | 0.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 9.0 |

Where the following parameters are defined to specify valid RB allocation ranges for RB allocations in Table 6.2.2_1.3.3-2:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBend = RBStart + LCRB - 1

An RB allocation belonging to table 6.2.2_1.3.3-2 is a Region 1 inner RB allocation if

RBstart ≥ Ceil(1/4 NRB) AND RBend < Ceil(3/4 NRB) AND LCRB ≤ Ceil(1/4 NRB)

For all transmission bandwidth configurations, an RB allocation is an Edge allocation if it is NOT a Region 1 inner allocation.

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.2.3

6.2.2_1.3.4	UE maximum output power reduction for power class 4

For power class 4, MPR specified in sub-clause 6.2.2.3 applies.

6.2.2_1.4	Test description

Same as in clause 6.2.2.1.4.1 with following exceptions: Instead of Tables 6.2.2.1.4.1-1 to 6.2.2.1.4.1-9 use Tables 6.2.2_1.1.4.1-1 and 6.2.2_1.1.4.1-4

Table 6.2.2_1.4.1-1: Test Configuration Table (Power Class 2, 3 and 4, MPRnarrow, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest supported channel bandwidth that ≤ 200 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right |
| 3 | Low |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Left |
| 4 | High |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| 5 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial2_Left |
| 6 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial2_Right |
| 7 | Low |  |  |  | DFT-s-OFDM QPSK | Inner_Partial2_Left |
| 8 | High |  |  |  | DFT-s-OFDM QPSK | Inner_Partial2_Right |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2.2_1.4.1-2: Test Configuration Table (Power Class 2, 3 and 4, MPRWT, BWchannel ≤ 200 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest and Highest supported channel bandwidth that ≤ 200 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 2 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 3 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Inner_Full |
| 4 | Low |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Right |
| 6 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 7 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Inner_Full |
| 8 | Low |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Left |
| 9 | High |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Right |
| 10 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 11 | Mid |  |  |  | CP-OFDM QPSK | Inner_Full |
| 12 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 13 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 14 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 15 | Low |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Left |
| 16 | High |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Right |
| 17 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 18 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 19 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 20 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2.2_1.4.1-3: Test Configuration Table (Power Class 2, 3 and 4, MPRnarrow, BWchannel = 400 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | 400 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120 kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_1RB_Right |
| 3 | Low |  |  |  | DFT-s-OFDM QPSK | Inner_1RB_Left |
| 4 | High |  |  |  | DFT-s-OFDM QPSK | Inner_1RB_Right |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

Table 6.2.2_1.4.1-4: Test Configuration Table (Power Class 2, 3 and 4, MPRWT, BWchannel = 400 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | 400 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | - | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| 6 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Right |
| 9 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Right |
| 12 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 13 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 14 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 15 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 16 | Low |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Left |
| 17 | High |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Right |
| 18 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 19 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 20 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 21 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

6.2.2_1.5	Test requirement

The maximum output power, derived in step 5 shall be within the range prescribed by the nominal maximum output power and tolerance in following tables.

Table 6.2.2_1.5-1: UE Power Class test requirements for Power Class 3 (n257, 258, 261)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| 6.2.2_1.4.1-1 | 1 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 2 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 3 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 4 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 5 | 22.4 | 2 | 1.5 | 18.9-TT-ΔMBP,n | 43 |
|  | 6 | 22.4 | 2 | 1.5 | 18.9-TT-ΔMBP,n | 43 |
|  | 7 | 22.4 | 2 | 1.5 | 18.9-TT-ΔMBP,n | 43 |
|  | 8 | 22.4 | 2 | 1.5 | 18.9-TT-ΔMBP,n | 43 |
| 6.2.2_1.4.1-2 | 1 | 22.4 | 2 | 1.5 | 18.9-TT-ΔMBP,n | 43 |
|  | 2 | 22.4 | 2 | 1.5 | 18.9-TT-ΔMBP,n | 43 |
|  | 3 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 4 | 22.4 | 3.5 | 3 | 15.9-TT-ΔMBP,n | 43 |
|  | 5 | 22.4 | 3.5 | 3 | 15.9-TT-ΔMBP,n | 43 |
|  | 6 | 22.4 | 3.5 | 3 | 15.9-TT-ΔMBP,n | 43 |
|  | 7 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 8 | 22.4 | 5.5 | 5 | 11.9-TT-ΔMBP,n | 43 |
|  | 9 | 22.4 | 5.5 | 5 | 11.9-TT-ΔMBP,n | 43 |
|  | 10 | 22.4 | 5.5 | 5 | 11.9-TT-ΔMBP,n | 43 |
|  | 11 | 22.4 | 3.5 | 3 | 15.9-TT-ΔMBP,n | 43 |
|  | 12 | 22.4 | 4 | 3 | 15.4-TT-ΔMBP,n | 43 |
|  | 13 | 22.4 | 4 | 3 | 15.4-TT-ΔMBP,n | 43 |
|  | 14 | 22.4 | 4 | 3 | 15.4-TT-ΔMBP,n | 43 |
|  | 15 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 16 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 17 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 18 | 22.4 | 7.5 | 5 | 9.9-TT-ΔMBP,n | 43 |
|  | 19 | 22.4 | 7.5 | 5 | 9.9-TT-ΔMBP,n | 43 |
|  | 20 | 22.4 | 7.5 | 5 | 9.9-TT-ΔMBP,n | 43 |
| 6.2.2_1.4.1-3 | 1 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 2 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 3 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
|  | 4 | 22.4 | 2.5 | 2 | 17.9-TT-ΔMBP,n | 43 |
| 6.2.2_1.4.1-4 | 1 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 2 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 3 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 4 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 5 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 6 | 22.4 | 3 | 2 | 17.4-TT-ΔMBP,n | 43 |
|  | 7 | 22.4 | 4.5 | 4 | 13.9-TT-ΔMBP,n | 43 |
|  | 8 | 22.4 | 4.5 | 4 | 13.9-TT-ΔMBP,n | 43 |
|  | 9 | 22.4 | 4.5 | 4 | 13.9-TT-ΔMBP,n | 43 |
|  | 10 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 11 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 12 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 13 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 14 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 15 | 22.4 | 5 | 4 | 13.4-TT-ΔMBP,n | 43 |
|  | 16 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 17 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 18 | 22.4 | 6.5 | 5 | 10.9-TT-ΔMBP,n | 43 |
|  | 19 | 22.4 | 9 | 5 | 8.4-TT-ΔMBP,n | 43 |
|  | 20 | 22.4 | 9 | 5 | 8.4-TT-ΔMBP,n | 43 |
|  | 21 | 22.4 | 9 | 5 | 8.4-TT-ΔMBP,n | 43 |
| Note 1: ΔMBP,n is the Multiband Relaxation factor declared by the UE for the tested band in table A.4.3.9-2 of TS38.508-2. This declaration shall fulfil the requirements in clause 6.2.1.1.3.3. Note 2: All UE supported bands needs to be tested to ensure the multiband relaxation declaration is compliant. Note 3: Max allowed sum of ΔMBP,n over all supported FR2 bands as defined in clause 6.2.1.1.3.3. Note 4: ΔMBP,n is 0 for single band UE. |  |  |  |  |  |  |

Table 6.2.2_1.5-2: UE Power Class test requirements for Power Class 3 (n260)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| 6.2.2_1.4.1-1 | 1 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 2 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 3 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 4 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 5 | 20.6 | 2 | 1.5 | 17.1-TT-ΔMBP,n | 43 |
|  | 6 | 20.6 | 2 | 1.5 | 17.1-TT-ΔMBP,n | 43 |
|  | 7 | 20.6 | 2 | 1.5 | 17.1-TT-ΔMBP,n | 43 |
|  | 8 | 20.6 | 2 | 1.5 | 17.1-TT-ΔMBP,n | 43 |
| 6.2.2_1.4.1-2 | 1 | 20.6 | 2 | 1.5 | 17.1-TT-ΔMBP,n | 43 |
|  | 2 | 20.6 | 2 | 1.5 | 17.1-TT-ΔMBP,n | 43 |
|  | 3 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 4 | 20.6 | 3.5 | 3 | 14.1-TT-ΔMBP,n | 43 |
|  | 5 | 20.6 | 3.5 | 3 | 14.1-TT-ΔMBP,n | 43 |
|  | 6 | 20.6 | 3.5 | 3 | 14.1-TT-ΔMBP,n | 43 |
|  | 7 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 8 | 20.6 | 5.5 | 5 | 10.1-TT-ΔMBP,n | 43 |
|  | 9 | 20.6 | 5.5 | 5 | 10.1-TT-ΔMBP,n | 43 |
|  | 10 | 20.6 | 5.5 | 5 | 10.1-TT-ΔMBP,n | 43 |
|  | 11 | 20.6 | 3.5 | 3 | 14.1-TT-ΔMBP,n | 43 |
|  | 12 | 20.6 | 4 | 3 | 13.6-TT-ΔMBP,n | 43 |
|  | 13 | 20.6 | 4 | 3 | 13.6-TT-ΔMBP,n | 43 |
|  | 14 | 20.6 | 4 | 3 | 13.6-TT-ΔMBP,n | 43 |
|  | 15 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 16 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 17 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 18 | 20.6 | 7.5 | 5 | 8.1-TT-ΔMBP,n | 43 |
|  | 19 | 20.6 | 7.5 | 5 | 8.1-TT-ΔMBP,n | 43 |
|  | 20 | 20.6 | 7.5 | 5 | 8.1-TT-ΔMBP,n | 43 |
| 6.2.2_1.4.1-3 | 1 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 2 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 3 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
|  | 4 | 20.6 | 2.5 | 2 | 16.1-TT-ΔMBP,n | 43 |
| 6.2.2_1.4.1-4 | 1 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 2 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 3 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 4 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 5 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 6 | 20.6 | 3 | 2 | 15.6-TT-ΔMBP,n | 43 |
|  | 7 | 20.6 | 4.5 | 4 | 12.1-TT-ΔMBP,n | 43 |
|  | 8 | 20.6 | 4.5 | 4 | 12.1-TT-ΔMBP,n | 43 |
|  | 9 | 20.6 | 4.5 | 4 | 12.1-TT-ΔMBP,n | 43 |
|  | 10 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 11 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 12 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 13 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 14 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 15 | 20.6 | 5 | 4 | 11.6-TT-ΔMBP,n | 43 |
|  | 16 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 17 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 18 | 20.6 | 6.5 | 5 | 9.1-TT-ΔMBP,n | 43 |
|  | 19 | 20.6 | 9 | 5 | 6.6-TT-ΔMBP,n | 43 |
|  | 20 | 20.6 | 9 | 5 | 6.6-TT-ΔMBP,n | 43 |
|  | 21 | 20.6 | 9 | 5 | 6.6-TT-ΔMBP,n | 43 |
| Note 1: ΔMBP,n is the Multiband Relaxation factor declared by the UE for the tested band in table A.4.3.9-2 of TS38.508-2. This declaration shall fulfil the requirements in clause 6.2.1.1.3.3. Note 2: All UE supported bands needs to be tested to ensure the multiband relaxation declaration is compliant. Note 3: Max allowed sum of ΔMBP,n over all supported FR2 bands as defined in clause 6.2.1.1.3.3. Note 4: ΔMBP,n is 0 for single band UE. |  |  |  |  |  |  |

Table 6.2.2_1.5-3: Test Tolerance (Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 3.11 dB | 3.11 dB |

Table 6.2.2_1.5-4: UE Power Class test requirements for Power Class 2 (n257, 258, 261)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| 6.2.2_1.4.1-1 | 1 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 2 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 3 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 4 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 5 | 29 | 2 | 1.5 | 25.5-TT | 43 |
|  | 6 | 29 | 2 | 1.5 | 25.5-TT | 43 |
|  | 7 | 29 | 2 | 1.5 | 25.5-TT | 43 |
|  | 8 | 29 | 2 | 1.5 | 25.5-TT | 43 |
| 6.2.2_1.4.1-2 | 1 | 29 | 2 | 1.5 | 25.5-TT | 43 |
|  | 2 | 29 | 2 | 1.5 | 25.5-TT | 43 |
|  | 3 | 29 | 3 | 2 | 24-TT | 43 |
|  | 4 | 29 | 3.5 | 3 | 22.5-TT | 43 |
|  | 5 | 29 | 3.5 | 3 | 22.5-TT | 43 |
|  | 6 | 29 | 3.5 | 3 | 22.5-TT | 43 |
|  | 7 | 29 | 5 | 4 | 20-TT | 43 |
|  | 8 | 29 | 5.5 | 5 | 18.5-TT | 43 |
|  | 9 | 29 | 5.5 | 5 | 18.5-TT | 43 |
|  | 10 | 29 | 5.5 | 5 | 18.5-TT | 43 |
|  | 11 | 29 | 3.5 | 3 | 22.5-TT | 43 |
|  | 12 | 29 | 4 | 3 | 22-TT | 43 |
|  | 13 | 29 | 4 | 3 | 22-TT | 43 |
|  | 14 | 29 | 4 | 3 | 22-TT | 43 |
|  | 15 | 29 | 5 | 4 | 20-TT | 43 |
|  | 16 | 29 | 5 | 4 | 20-TT | 43 |
|  | 17 | 29 | 5 | 4 | 20-TT | 43 |
|  | 18 | 29 | 7.5 | 5 | 16.5-TT | 43 |
|  | 19 | 29 | 7.5 | 5 | 16.5-TT | 43 |
|  | 20 | 29 | 7.5 | 5 | 16.5-TT | 43 |
| 6.2.2_1.4.1-3 | 1 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 2 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 3 | 29 | 2.5 | 2 | 24.5-TT | 43 |
|  | 4 | 29 | 2.5 | 2 | 24.5-TT | 43 |
| 6.2.2_1.4.1-4 | 1 | 29 | 3 | 2 | 24-TT | 43 |
|  | 2 | 29 | 3 | 2 | 24-TT | 43 |
|  | 3 | 29 | 3 | 2 | 24-TT | 43 |
|  | 4 | 29 | 3 | 2 | 24-TT | 43 |
|  | 5 | 29 | 3 | 2 | 24-TT | 43 |
|  | 6 | 29 | 3 | 2 | 24-TT | 43 |
|  | 7 | 29 | 4.5 | 4 | 20.5-TT | 43 |
|  | 8 | 29 | 4.5 | 4 | 20.5-TT | 43 |
|  | 9 | 29 | 4.5 | 4 | 20.5-TT | 43 |
|  | 10 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 11 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 12 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 13 | 29 | 5 | 4 | 20-TT | 43 |
|  | 14 | 29 | 5 | 4 | 20-TT | 43 |
|  | 15 | 29 | 5 | 4 | 20-TT | 43 |
|  | 16 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 17 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 18 | 29 | 6.5 | 5 | 17.5-TT | 43 |
|  | 19 | 29 | 9 | 5 | 15-TT | 43 |
|  | 20 | 29 | 9 | 5 | 15-TT | 43 |
|  | 21 | 29 | 9 | 5 | 15-TT | 43 |

Table 6.2.2_1.5-5: UE Power Class test requirements for Power Class 4 (n257, 258, 261)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| 6.2.2_1.4.1-1 | 1 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 2 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 3 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 4 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 5 | 34 | 2 | 1.5 | 30.5-TT | 43 |
|  | 6 | 34 | 2 | 1.5 | 30.5-TT | 43 |
|  | 7 | 34 | 2 | 1.5 | 30.5-TT | 43 |
|  | 8 | 34 | 2 | 1.5 | 30.5-TT | 43 |
| 6.2.2_1.4.1-2 | 1 | 34 | 2 | 1.5 | 30.5-TT | 43 |
|  | 2 | 34 | 2 | 1.5 | 30.5-TT | 43 |
|  | 3 | 34 | 3 | 2 | 29-TT | 43 |
|  | 4 | 34 | 3.5 | 3 | 27.5-TT | 43 |
|  | 5 | 34 | 3.5 | 3 | 27.5-TT | 43 |
|  | 6 | 34 | 3.5 | 3 | 27.5-TT | 43 |
|  | 7 | 34 | 5 | 4 | 25-TT | 43 |
|  | 8 | 34 | 5.5 | 5 | 23.5-TT | 43 |
|  | 9 | 34 | 5.5 | 5 | 23.5-TT | 43 |
|  | 10 | 34 | 5.5 | 5 | 23.5-TT | 43 |
|  | 11 | 34 | 3.5 | 3 | 27.5-TT | 43 |
|  | 12 | 34 | 4 | 3 | 27-TT | 43 |
|  | 13 | 34 | 4 | 3 | 27-TT | 43 |
|  | 14 | 34 | 4 | 3 | 27-TT | 43 |
|  | 15 | 34 | 5 | 4 | 25-TT | 43 |
|  | 16 | 34 | 5 | 4 | 25-TT | 43 |
|  | 17 | 34 | 5 | 4 | 25-TT | 43 |
|  | 18 | 34 | 7.5 | 5 | 21.5-TT | 43 |
|  | 19 | 34 | 7.5 | 5 | 21.5-TT | 43 |
|  | 20 | 34 | 7.5 | 5 | 21.5-TT | 43 |
| 6.2.2_1.4.1-3 | 1 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 2 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 3 | 34 | 2.5 | 2 | 29.5-TT | 43 |
|  | 4 | 34 | 2.5 | 2 | 29.5-TT | 43 |
| 6.2.2_1.4.1-4 | 1 | 34 | 3 | 2 | 29-TT | 43 |
|  | 2 | 34 | 3 | 2 | 29-TT | 43 |
|  | 3 | 34 | 3 | 2 | 29-TT | 43 |
|  | 4 | 34 | 3 | 2 | 29-TT | 43 |
|  | 5 | 34 | 3 | 2 | 29-TT | 43 |
|  | 6 | 34 | 3 | 2 | 29-TT | 43 |
|  | 7 | 34 | 4.5 | 4 | 25.5-TT | 43 |
|  | 8 | 34 | 4.5 | 4 | 25.5-TT | 43 |
|  | 9 | 34 | 4.5 | 4 | 25.5-TT | 43 |
|  | 10 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 11 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 12 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 13 | 34 | 5 | 4 | 25-TT | 43 |
|  | 14 | 34 | 5 | 4 | 25-TT | 43 |
|  | 15 | 34 | 5 | 4 | 25-TT | 43 |
|  | 16 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 17 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 18 | 34 | 6.5 | 5 | 22.5-TT | 43 |
|  | 19 | 34 | 9 | 5 | 20-TT | 43 |
|  | 20 | 34 | 9 | 5 | 20-TT | 43 |
|  | 21 | 34 | 9 | 5 | 20-TT | 43 |

Table 6.2.2_1.5-6: UE Power Class test requirements for Power Class 4 (n260)

| Test Configuration Table | Test ID | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| 6.2.2_1.4.1-1 | 1 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 2 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 3 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 4 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 5 | 31 | 2 | 1.5 | 27.5-TT | 43 |
|  | 6 | 31 | 2 | 1.5 | 27.5-TT | 43 |
|  | 7 | 31 | 2 | 1.5 | 27.5-TT | 43 |
|  | 8 | 31 | 2 | 1.5 | 27.5-TT | 43 |
| 6.2.2_1.4.1-2 | 1 | 31 | 2 | 1.5 | 27.5-TT | 43 |
|  | 2 | 31 | 2 | 1.5 | 27.5-TT | 43 |
|  | 3 | 31 | 3 | 2 | 26-TT | 43 |
|  | 4 | 31 | 3.5 | 3 | 24.5-TT | 43 |
|  | 5 | 31 | 3.5 | 3 | 24.5-TT | 43 |
|  | 6 | 31 | 3.5 | 3 | 24.5-TT | 43 |
|  | 7 | 31 | 5 | 4 | 22-TT | 43 |
|  | 8 | 31 | 5.5 | 5 | 19.5-TT | 43 |
|  | 9 | 31 | 5.5 | 5 | 19.5-TT | 43 |
|  | 10 | 31 | 5.5 | 5 | 19.5-TT | 43 |
|  | 11 | 31 | 3.5 | 3 | 24.5-TT | 43 |
|  | 12 | 31 | 4 | 3 | 24-TT | 43 |
|  | 13 | 31 | 4 | 3 | 24-TT | 43 |
|  | 14 | 31 | 4 | 3 | 24-TT | 43 |
|  | 15 | 31 | 5 | 4 | 22-TT | 43 |
|  | 16 | 31 | 5 | 4 | 22-TT | 43 |
|  | 17 | 31 | 5 | 4 | 22-TT | 43 |
|  | 18 | 31 | 7.5 | 5 | 18.5-TT | 43 |
|  | 19 | 31 | 7.5 | 5 | 18.5-TT | 43 |
|  | 20 | 31 | 7.5 | 5 | 18.5-TT | 43 |
| 6.2.2_1.4.1-3 | 1 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 2 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 3 | 31 | 2.5 | 2 | 26.5-TT | 43 |
|  | 4 | 31 | 2.5 | 2 | 26.5-TT | 43 |
| 6.2.2_1.4.1-4 | 1 | 31 | 3 | 2 | 26-TT | 43 |
|  | 2 | 31 | 3 | 2 | 26-TT | 43 |
|  | 3 | 31 | 3 | 2 | 26-TT | 43 |
|  | 4 | 31 | 3 | 2 | 26-TT | 43 |
|  | 5 | 31 | 3 | 2 | 26-TT | 43 |
|  | 6 | 31 | 3 | 2 | 26-TT | 43 |
|  | 7 | 31 | 4.5 | 4 | 22.5-TT | 43 |
|  | 8 | 31 | 4.5 | 4 | 22.5-TT | 43 |
|  | 9 | 31 | 4.5 | 4 | 22.5-TT | 43 |
|  | 10 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 11 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 12 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 13 | 31 | 5 | 4 | 22-TT | 43 |
|  | 14 | 31 | 5 | 4 | 22-TT | 43 |
|  | 15 | 31 | 5 | 4 | 22-TT | 43 |
|  | 16 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 17 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 18 | 31 | 6.5 | 5 | 19.5-TT | 43 |
|  | 19 | 31 | 9 | 5 | 17-TT | 43 |
|  | 20 | 31 | 9 | 5 | 17-TT | 43 |
|  | 21 | 31 | 9 | 5 | 17-TT | 43 |

< End of Changes >