3GPP TSG-RAN5 Meeting #98	R5-230937

# Athens, GR, 27 Feb 2023 - 03 Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2727 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of the contents of RRC messages for L2 U2N relay related operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The contents of RRC messages for L2 U2N relay related operation should be clarified |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the contents of RRC messages |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases may not be implementable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1, 4.6.2, 4.6.3,4.6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

4.6.1	Contents of RRC messages

<Unchanged Skip>

#### –	RRCReestablishment

Table 4.6.1-10: RRCReestablishment

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReestablishment ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReestablishment SEQUENCE { |  |  |  |  |
| nextHopChainingCount |  | NextHopChainingCount |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | L2RemoteUE |
| sl-L2RemoteUE-Config-r17 CHOICE { |  |  |  |  |
| setup |  | SL-L2RemoteUE-Config-r17 |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| L2RemoteUE | For L2 U2N Remote UE test cases. |

<Unchanged Skip>

–	RRCReconfiguration

Table 4.6.1-13: RRCReconfiguration

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |
|  |  | RadioBearerConfig with conditions SRB2 and DRB1 |  | NR, NR-DC |
|  |  | RadioBearerConfig with conditions SRB2 and REEST |  | REEST |
| secondaryCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
|  |  | CellGroupConfig with condition NR-DC_SCG | OCTET STRING (CONTAINING CellGroupConfig) | NR-DC_SCG |
|  |  | CellGroupConfig with conditions EN-DC and PSCell_change | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_HO |
|  |  | CellGroupConfig with condition MEAS | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_MEAS |
|  |  | CellGroupConfig with condition SCell_add | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_SCell_add |
|  |  | Not present |  |  |
| measConfig |  | Not present |  |  |
|  |  | MeasConfig | Measurements configuration | NR_MEAS, EN-DC_MEAS, IRAT_MEAS |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | NR, SCell_add, NR_MEAS, NR-DC, SIDELINK, DAPS_HO_ReleaseSource, CHO, CPC, REEST, L2RelayUE, L2RemoteUE |
| masterCellGroup |  | CellGroupConfig with condition SRB2_DRB1 | OCTET STRING (CONTAINING CellGroupConfig) | NR, NR-DC |
|  |  | CellGroupConfig with condition SCell_add | OCTET STRING (CONTAINING CellGroupConfig) | SCell_add |
|  |  | CellGroupConfig with condition MEAS | OCTET STRING (CONTAINING CellGroupConfig) | NR_MEAS |
|  |  | CellGroupConfig with condition REEST | OCTET STRING (CONTAINING CellGroupConfig) | REEST |
|  |  | CellGroupConfig with condition RELAY | OCTET STRING (CONTAINING CellGroupConfig) | RELAY |
| fullConfig |  | Not present |  |  |
| dedicatedNAS-MessageList |  | Not present |  |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message { |  | 1 entry |  | NR |
| DedicatedNAS-Message[1] |  | DedicatedNAS-Message | entry 1 A sequence of OCTET STRING (s) containing one or more DedicatedNAS-Message(s) |  |
| } |  |  |  |  |
| masterKeyUpdate |  | Not present |  |  |
| masterKeyUpdate SEQUENCE { |  |  |  | MasterKeyChange |
| keySetChangeIndicator |  | true |  |  |
| nextHopChainingCount |  | NextHopChainingCount |  |  |
| nas-Container |  | Not present |  |  |
|  |  | OCTET STRING including the 10 Octets value generated according to TS 24.501 [28] clause 9.11.2.9 |  | Inter_Sys_HO |
| } |  |  |  |  |
| dedicatedSIB1-Delivery |  | Not present |  |  |
| dedicatedSystemInformationDelivery |  | Not present |  |  |
| otherConfig |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | NR-DC, SIDELINK, DAPS_HO_ReleaseSource, CHO, CPC, NE-DC,L2RelayUE, L2RemoteUE |
| otherConfig-v1540 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| mrdc-SecondaryCellGroupConfig |  | Not present |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  | NR-DC, NE-DC |
| setup SEQUENCE { |  |  |  |  |
| mrdc-ReleaseAndAdd |  | Not present |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |  |
| nr-SCG |  | RRCReconfiguration with condition NR-DC_SCG | OCTET STRING (CONTAINING RRCReconfiguration) | NR-DC |
| eutra-SCG |  | RRCConnectionReconfiguration according TS 36.508 [2], table 4.6.1-8 with condition NE-DC | OCTET STRING (CONTAINING RRCConnectionReconfiguration) | NE-DC |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| radioBearerConfig2 |  | Not present |  | NE-DC |
|  |  | RadioBearerConfig with condition DRBn and SecondaryKeys | OCTET STRING (CONTAINING RadioBearerConfig) | NR-DC |
| sk-Counter |  | SK-Counter |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | SIDELINK, DAPS_HO_ReleaseSource, CHO, CPC,L2RelayUE, L2RemoteUE |
| otherConfig-v1610 |  | Not present |  |  |
| bap-Config-r16 |  | Not present |  |  |
| iab-IP-AddressConfigurationList-r16 |  | Not present |  |  |
| conditionalReconfiguration-r16 |  | Not present |  |  |
|  |  | ConditionalReconfiguration |  | CHO, CPC |
| daps-SourceRelease-r16 |  | Not present |  |  |
|  |  | true |  | DAPS_HO_ReleaseSource |
| t316-r16 |  | Not present |  |  |
| needForGapsConfigNR-r16 |  | Not present |  |  |
| onDemandSIB-Request-r16 |  | Not present |  |  |
| dedicatedPosSysInfoDelivery-r16 |  | Not present |  |  |
| sl-ConfigDedicatedNR-r16 |  | Not present |  |  |
| sl-ConfigDedicatedNR-r16 CHOICE { |  |  |  | SIDELINK |
| setup |  | SL-ConfigDedicatedNR-r16 |  |  |
| } |  |  |  |  |
| sl-ConfigDedicatedEUTRA-Info-r16 |  | Not present |  |  |
| smtc-r16 |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | L2RelayUE, L2RemoteUE |
| otherConfig-v1700 |  | Not present |  |  |
| sl-L2RelayUE-Config-r17 |  | Not present |  |  |
| sl-L2RelayUE-Config-r17 CHOICE { |  |  |  |  |
| setup |  | SL-L2RelayUE-Config-r17 |  | L2RelayUE |
| } |  |  |  |  |
| sl-L2RemoteUE-Config-r17 |  | Not present |  |  |
| sl-L2RemoteUE-Config-r17 CHOICE { |  |  |  | L2RemoteUE |
| setup |  | SL-L2RemoteUE-Config-r17 |  |  |
| } |  |  |  |  |
| dedicatedPagingDelivery-r17 |  | Not present |  |  |
| needForGapNCSG-ConfigNR-r17 |  | Not present |  |  |
| needForGapNCSG-ConfigEUTRA-r17 |  | Not present |  |  |
| musim-GapConfig-r17 |  | Not present |  |  |
| ul-GapFR2-Config-r17 |  | Not present |  |  |
| scg-State-r17 |  | Not present |  |  |
| appLayerMeasConfig-r17 |  | Not present |  |  |
| ue-TxTEG-RequestUL-TDOA-Config-r17 |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |
| EN-DC_MEAS | An EN-DC measurement is configured |
| IRAT_MEAS | An IRAT measurement is configured in the NR PCell |
| NR_MEAS | A NR measurement is configured |
| NR | NR connected to 5GC |
| NR-DC | NR-NR Dual Connectivity is configured |
| NR-DC_SCG | Add SCG side configuration (NR-DC) |
| EN-DC_HO | NR PSCell change (EN-DC) |
| SCell_add | Add SCell |
| EN-DC_SCell_add | Add Scell (EN-DC) |
| MasterKeyChange | MasterKeyUpdate when peforming ReconfigurationWithSync and indicating a change of the AS security algorithms associated to the master key |
| Inter_Sys_HO | Used during inter-system handover to NR procedure |
| REEST | The first RRCReconfiguration message after successful completion of the RRC re-establishment procedure |
| SIDELINK | For NR sidelink dedicated configuration |
| DAPS_HO_ReleaseSource | The source cell part of DAPS operation is to be stopped and the source cell part of DAPS configuration is to be released. |
| CHO | Conditional handover |
| CPC | Conditional PSCell change |
| NE-DC | NR E-UTRA Dual Connectivity |
| L2RelayUE | For L2 U2N Relay UE test cases. |
| L2RemoteUE | For L2 U2N Remote UE test cases. |
| RELAY | Add Uu Relay RLC channel configuration |

<Unchanged Skip>

#### –	RRCRelease

Table 4.6.1-16: RRCRelease

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |  |
| redirectedCarrierInfo |  | Not present |  |  |
| cellReselectionPriorities |  | Not present |  |  |
| suspendConfig |  | Not present |  |  |
| suspendConfig SEQUENCE { |  |  |  | NR_RRC_INACTIVE |
| fullI-RNTI |  | I-RNTI-Value |  |  |
| shortI-RNTI |  | ShortI-RNTI-Value |  |  |
| ran-PagingCycle |  | rf32 |  |  |
| ran-NotificationAreaInfo CHOICE { |  |  |  |  |
| cellList SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell { |  | 1 entry |  |  |
| PLMN-RAN-AreaCellList[1] SEQUENCE { |  |  | entry 1 |  |
| plmn-Identity |  | Not present |  |  |
| ran-AreaCells SEQUENCE (SIZE (1..32)) OF CellIdentity { |  | 1 entry |  |  |
| CellIdentity[1] |  | CellIdentity | entry 1 Cellidentity for the used cell. |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| t380 |  | Not present |  |  |
| nextHopChainingCount |  | NextHopChainingCount |  |  |
| sl-UEIdentityRemote-r17 |  | Not present |  |  |
| sl-UEIdentityRemote-r17 |  | RNTI-Value |  | L2RemoteUE |
| sdt-Config-r17 |  | Not present |  |  |
| sdt-Config-r17 CHOICE { |  |  |  | SDT |
| setup SEQUENCE { |  |  |  |  |
| sdt-DRB-List-r17 SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity { |  | 1 entry |  |  |
| DRB-Identity |  | DRB-Identity using condition DRB1 |  |  |
| } |  |  |  |  |
| sdt-SRB2-Indication-r17 |  | Not present |  |  |
|  |  | allowed |  | pc_srb_SDT_r17 |
| sdt-MAC-PHY-CG-Config-r17 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| cg-SDT-ConfigLCH-RestrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF CG-SDT-Config-LCH-restriction-r17 { |  | 1 entry |  |  |
| logicalChannelIdentity-r17 |  | LogicalChannelIdentity with condition DRB1 |  |  |
| configuredGrantType1Allowed-r17 |  | true |  |  |
| allowedCG-List-r17 |  | Not present |  |  |
| } |  |  |  |  |
| cg-SDT-ConfigLCH-RestrictionToReleaseList-r17 |  | Not present |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 |  | Not present |  |  |
| cg-SDT-ConfigInitialBWP-SUL-r17 |  | Not present |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 |  | Not present |  |  |
| cg-SDT-TimeAlignmentTimer-r17 |  | ms750 |  |  |
| cg-SDT-RSRP-ThresholdSSB-r17 |  | 66 |  |  |
| cg-SDT-TA-ValiditationConfig-r17 |  | Not present |  |  |
| cg-SDT-CS-RNTI-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sdt-DRB-ContinueROHC-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| srs-PosRRC-Inactive-r17 |  | Not present |  |  |
| ran-ExtendedPagingCycle-r17 |  | Not present |  |  |
| } |  |  |  |  |
| deprioritisationReq |  | Not present |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| NR_RRC_INACTIVE | NR RRC state RRC_INACTIVE |
| SDT | For SDT test cases |
| L2RemoteUE | For L2 U2N Remote UE test cases. |

<Unchanged Skip>

#### –	RRCResume

Table 4.6.1-17: RRCResume

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResume ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcResume SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition RESUME |  |  |
| masterCellGroup |  | CellGroupConfig with condition RESUME | OCTET STRING (CONTAINING CellGroupConfig) |  |
| measConfig |  | Not present |  |  |
| fullConfig |  | Not present |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | L2RemoteUE |
| radioBearerConfig2 |  | Not present |  |  |
| sk-Counter |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| idleModeMeasurementReq-r16 |  | Not present |  |  |
| restoreMCG-SCells-r16 |  | Not present |  |  |
| restoreSCG-r16 |  | Not present |  |  |
| mrdc-SecondaryCellGroup-r16 |  | Not present |  |  |
| needForGapsConfigNR-r16 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| sl-ConfigDedicatedNR-r17 |  | Not present |  |  |
| sl-L2RemoteUE-Config-r17 CHOICE { |  |  |  |  |
| setup |  | SL-L2RemoteUE-Config-r17 |  |  |
| } |  |  |  |  |
| needForGapNCSG-ConfigNR-r17 |  | Not present |  |  |
| needForGapNCSG-ConfigEUTRA-r17 |  | Not present |  |  |
| scg-State-r17 |  | Not present |  |  |
| appLayerMeasConfig-r17 |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| L2RemoteUE | For L2 U2N Remote UE test cases. |

<Unchanged Skip>

–	RRCSetup

Table 4.6.1-21: RRCSetup

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetup ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetup SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition SRB1 |  |  |
| masterCellGroup |  | CellGroupConfig with condition SRB1 | OCTET STRING (CONTAINING CellGroupConfig) |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | L2RemoteUE |
| sl-ConfigDedicatedNR-r17 |  | Not present |  |  |
| sl-L2RemoteUE-Config-r17 |  | SL-L2RemoteUE-Config |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| L2RemoteUE | For L2 U2N Remote UE test cases. |

<Unchanged Skip>

–	SidelinkUEInformationNR

Table 4.6.1-28A: SidelinkUEInformationNR

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SidelinkUEInformationNR-r16 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| sidelinkUEInformationNR-r16 SEQUENCE { |  |  |  |  |
| sl-RxInterestedFreqList-r16 |  | Not present |  |  |
| sl-RxInterestedFreqList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER { |  | 1 entry |  | SIDELINK_RX |
| INTEGER[1] |  | 1 | entry 1 |  |
| } |  |  |  |  |
| sl-TxResourceReqList-r16 |  | Not present |  |  |
| sl-TxResourceReqList-r16 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReq-r16 { |  | 1 entry |  | SIDELINK_TX |
| SL-TxResourceReq-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-DestinationIdentity-r16 |  | SL-DestinationIdentity-r16 |  |  |
| sl-CastType-r16 |  | unicast |  |  |
| sl-RLC-ModeIndicationList-r16 |  | Not present |  |  |
| sl-QoS-InfoList-r16 SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16 { |  | 1 entry |  |  |
| SL-QoS-Info-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-QoS-FlowIdentity-r16 |  | SL-QoS-FlowIdentity-r16 |  |  |
| sl-QoS-Profile-r16 |  | SL-QoS-Profile-r16 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-TypeTxSyncList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16 { |  | 1 entry |  |  |
| SL-TypeTxSync-r16[1] |  | SL-TypeTxSync-r16 | entry 1 |  |
| } |  |  |  |  |
| sl-TxInterestedFreqList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER { |  | 1 entry |  |  |
| INTEGER[1] |  | 1 | entry 1 |  |
| } |  |  |  |  |
| sl-CapabilityInformationSidelink-r16[1] |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-FailureList-r16 |  | Not present |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | L2RelayUE |
| sl-TxResourceReqList-v1700 |  | Not checked |  |  |
| sl-RxDRX-ReportList-v1700 |  | Not checked |  |  |
| sl-RxInterestedGC-BC-DestList-r17 |  | Not checked |  |  |
| sl-RxInterestedFreqListDisc-r17 |  | Not checked |  |  |
| sl-TxResourceReqListDisc-r17 |  | Not checked |  |  |
| sl-TxResourceReqListCommRelay-r17 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReqCommRelayInfo-r17 { |  |  |  |  |
| SL-TxResourceReqCommRelayInfo-r17 [1] SEQUENCE { |  |  | entry 1 |  |
| sl-RelayDRXConfig-r17 |  | Not checked |  |  |
| sl-TxResourceReqCommRelay-r17 CHOICE { |  |  |  |  |
| sl-TxResourceReqL2U2N-Relay-r17 SEQUENCE { |  |  |  |  |
| sl-DestinationIdentityL2U2N-r17 |  | SL-DestinationIdentity-r16 |  |  |
| sl-TxInterestedFreqListL2U2N-r17 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER { |  |  |  |  |
| INTEGER[1] |  | 1 |  |  |
| } |  |  |  |  |
| sl-TypeTxSyncListL2U2N-r17 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16 { |  |  |  |  |
| SL-TypeTxSync-r16[1] |  | SL-TypeTxSync-r16 | entry 1 |  |
| } |  |  |  |  |
| sl-LocalID-Request-r17 |  | true |  |  |
| sl-PagingIdentityRemoteUE-r17 SEQUENCE { |  |  |  |  |
| ng-5G-S-TMSI-r17 |  | NG-5G-S-TMSI |  |  |
| fullI-RNTI-r17 |  | Not checked |  |  |
| } |  |  |  |  |
| sl-CapabilityInformationSidelink-r17 |  | OCTET STRING containing UECapabilityInformationSidelink specified in Table 4.6.1A-7 with condition RX |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| ue-Type-r17 |  | relayUE |  |  |
| sl-SourceIdentityRemoteUE-r17 |  | Not checked |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SIDELINK_TX | Used when UE indicates its interest on sidelink transmission. |
| SIDELINK_RX | Used when UE indicates its interest on sidelink reception. |
| L2RelayUE | For L2 U2N Relay UE test cases. |

<Unchanged Skip>

4.6.2	System information blocks

<Unchanged Skip>

Table 4.6.2-14A: SIB12-IEs

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB12-IEs-r16 ::= SEQUENCE { |  |  |  |
| sl-ConfigCommonNR-r16 SEQUENCE { |  |  |  |
| sl-FreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16 { | 1 entry |  |  |
| SL-FreqConfigCommon-r16[1] | SL-FreqConfigCommon | entry 1 |  |
| } |  |  |  |
| sl-UE-SelectedConfig-r16 | SL-UE-SelectedConfig |  |  |
| sl-NR-AnchorCarrierFreqList-r16 | Not present |  |  |
| sl-EUTRA-AnchorCarrierFreqList-r16 | Not present |  |  |
| sl-RadioBearerConfigList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16 { | 1 entry |  |  |
| SL-RadioBearerConfig-r16[1] | SL-RadioBearerConfig | entry 1 |  |
| } |  |  |  |
| sl-RLC-BearerConfigList-r16 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16 { | 1 entry |  |  |
| SL-RLC-BearerConfig-r16[1] | SL-RLC-BearerConfig | entry 1 |  |
| } |  |  |  |
| sl-MeasConfigCommon-r16 | SL-MeasConfigCommon |  |  |
| sl-CSI-Acquisition-r16 | Not present |  |  |
| sl-OffsetDFN-r16 | Not present |  |  |
| t400-r16 | ms1000 |  |  |
| sl-MaxNumConsecutiveDTX-r16 | Not present |  |  |
| sl-SSB-PriorityNR-r16 | 1 |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| sl-DRX-ConfigCommonGC-BC-r17 | Not present |  |  |
| sl-DiscConfigCommon-r17 SEQUENCE { |  |  |  |
| sl-RelayUE-ConfigCommon-r17 | SL-RelayUE-Config |  |  |
| sl-RemoteUE-ConfigCommon-r17 | SL-RemoteUE-Config |  |  |
| } |  |  |  |
| sl-L2U2N-Relay-r17 | enabled |  |  |
| sl-NonRelayDiscovery-r17 | Not present |  |  |
| sl-L3U2N-RelayDiscovery-r17 | Not present |  |  |
| sl-TimersAndConstantsRemoteUE-r17 | Not present |  |  |
| } |  |  |  |

<Unchanged Skip>

4.6.3	Radio resource control information elements

<Unchanged Skip>

#### –	CellGroupConfig

Table 4.6.3-19: CellGroupConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| cellGroupId | CellGroupId |  |  |
|  | CellGroupId condition NR-DC_SCG |  | NR-DC_SCG |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | EN-DC |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and DRB2 | entry 1 |  |
|  | RLC-BearerConfig with conditions AM and DRB2 and Re-establish_RLC |  | PSCell_change |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | SRB1 |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB1 | entry 1 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | SRB2_DRB1 |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRB1 | entry 2 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | SRB2_DRB2 |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRB2 | entry 2 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | DRBn, NR-DC_SCG |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and DRBn | entry 1 DRBn is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3 entries |  | PCell_change |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions SRB1 and Re-establish_RLC | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions SRB2 and Re-establish_RLC | entry 2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM, DRB1 and Re-establish_RLC | entry 3 |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1+n entries | n is the number of DRBs established before RRC resume | RESUME |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 and RESUME | entry 1 |  |
| RLC-BearerConfig[k+1, k=1..n] | RLC-BearerConfig with condition DRBk and RESUME | entry [k+1, k=1..n] |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2+n entries | n is the number of DRBs established before RRC re-establishement | REEST |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB1 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with condition SRB2 and Re-establish_RLC | entry 2 |  |
| RLC-BearerConfig[k+2, k=1..n] | RLC-BearerConfig with condition DRBk and Re-establish_RLC | entry [k+2, k=1..n] |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | MRBm AND (AM_PTP OR UM_bi_PTP OR UM_PTP OR UM_PTM) |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and PTP | entry 1 | AM_PTP |
|  | RLC-BearerConfig with conditions UM and PTP |  | UM_bi_PTP |
|  | RLC-BearerConfig with conditions UM_DLonly and PTP |  | UM_PTP |
|  | RLC-BearerConfig with conditions UM_DLonly and PTM |  | UM_PTM |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | MRBm_DRBn AND (AM_PTP OR UM_bi_PTP OR UM_PTP OR UM_PTM) |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and PTP | entry 1 | AM_PTP |
|  | RLC-BearerConfig with conditions UM and PTP |  | UM_bi_PTP |
|  | RLC-BearerConfig with conditions UM_DLonly and PTP |  | UM_PTP |
|  | RLC-BearerConfig with conditions UM_DLonly and PTM |  | UM_PTM |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRBn | entry 2 DRBn is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  | MRBm AND (AMPTP_UMPTM OR UMPTP_UMPTM) |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and PTP | entry 1 | AMPTP_UMPTM |
|  | RLC-BearerConfig with conditions UM_DLonly and PTP |  | UMPTP_UMPTM |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions UM_DLonly and PTM | entry 2 | AM_PTP_UM_PTM, UMPTP_UMPTM |
| } |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3 entries |  | MRBm_DRBn AND (AMPTP_UMPTM OR UMPTP_UMPTM) |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and PTP | entry 1 | AMPTP_UMPTM |
|  | RLC-BearerConfig with conditions UM_DLonly and PTP |  | UMPTP_UMPTM |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions UM_DLonly and PTM | entry 2 | AM_PTP_UM_PTM, UMPTP_UMPTM |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM and DRBn | entry 3 DRBn is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| rlc-BearerToReleaseList | Not present |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig |  |  |
|  | MAC-CellGroupConfig with condition MBS_Multicast |  | AMPTP_UMPTM, UMPTP_UMPTM, UM_PTM |
|  | Not present |  | SRB2_DRB1, MEAS, SRB2_DRB2, SCell_add |
| physicalCellGroupConfig | PhysicalCellGroupConfig |  |  |
|  | Not present |  | SRB2_DRB1, MEAS, SRB2_DRB2, SCell_add, MRBm, MRBm_DRBn |
| spCellConfig | Not present |  | SRB2_DRB1, SRB2_DRB2, SCell_add, AM_PTP, UM_bi_PTP, UM_PTP |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  |  |
|  | ServCellIndex |  | EN-DC, EN-DC AND MEAS |
|  | ServCellIndex with condition NR-DC_SCG |  | NR-DC_SCG |
| reconfigurationWithSync | Not present |  |  |
| reconfigurationWithSync SEQUENCE { |  |  | EN-DC, PCell_change, PSCell_change, NR-DC_SCG |
| spCellConfigCommon | ServingCellConfigCommon |  |  |
| newUE-Identity | RNTI-Value |  |  |
|  | RNTI-Value with condition NR-DC_SCG |  | NR-DC_SCG |
| t304 | ms1000 |  |  |
| rach-ConfigDedicated | Not present |  |  |
| rach-ConfigDedicated CHOICE { |  |  | CFRA |
| uplink | RACH-ConfigDedicated |  |  |
| supplementaryUplink | RACH-ConfigDedicated |  | SUL AND SIG |
| } |  |  |  |
| } |  |  |  |
| rlf-TimersAndConstants CHOICE { |  |  |  |
| setup | RLF-TimersAndConstants |  |  |
| } |  |  |  |
| rlf-TimersAndConstants | Not present |  | MEAS, RESUME |
| rlmInSyncOutOfSyncThreshold | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig |  | EN-DC, SRB1, PCell_change, PSCell_change, NR-DC_SCG, REEST |
|  | Not present |  |  |
|  | ServingCellConfig with condition MEAS |  | MEAS |
|  | ServingCellConfig with condition RESUME |  | RESUME |
|  | ServingCellConfig with condition MBS_Multicast |  | AMPTP_UMPTM, UMPTP_UMPTM, UM_PTM |
| } |  |  |  |
| sCellToAddModList | Not present |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  | SCell_add |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | SCellIndex |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition No_UL |  |  |
|  | ServingCellConfigCommon |  | RF AND UL_CA |
| sCellConfigDedicated | ServingCellConfig with condition No_UL and Scell_Add |  |  |
|  | ServingCellConfig with condition Scell_Add |  | RF AND UL_CA |
| } |  |  |  |
| } |  |  |  |
| sCellToReleaseList | Not present |  |  |
| reportUplinkTxDirectCurrent | Not present |  |  |
| bap-Address-r16 | Not present |  |  |
| bh-RLC-ChannelToAddModList-r16 | Not present |  |  |
| bh-RLC-ChannelToReleaseList-r16 | Not present |  |  |
| f1c-TransferPath-r16 | Not present |  |  |
| simultaneousTCI-UpdateList1-r16 | Not present |  |  |
| simultaneousTCI-UpdateList2-r16 | Not present |  |  |
| simultaneousSpatial-UpdatedList1-r16 | Not present |  |  |
| simultaneousSpatial-UpdatedList2-r16 | Not present |  |  |
| uplinkTxSwitchingOption-r16 | Not present |  |  |
| uplinkTxSwitchingPowerBoosting-r16 | Not present |  |  |
| reportUplinkTxDirectCurrentTwoCarrier-r16 | Not present |  |  |
| f1c-TransferPathNRDC-r17 | Not present |  |  |
| uplinkTxSwitching-2T-Mode-r17 | Not present |  |  |
| uplinkTxSwitching-DualUL-TxState-r17 | Not present |  |  |
| uu-RelayRLC-ChannelToAddModList-r17 | Not present |  |  |
| uu-RelayRLC-ChannelToAddModList-r17 SEQUENCE (SIZE (1.. maxUu-RelayRLC-ChannelID-r17)) OF Uu-RelayRLC-ChannelConfig-r17{ |  |  | RELAY |
| Uu-RelayRLC-ChannelConfig-r17[1] | Uu-RelayRLC-ChannelConfig |  |  |
| } |  |  |  |
| uu-RelayRLC-ChannelToReleaseList-r17 | Not present |  |  |
| simultaneousU-TCI-UpdateList1-r17 | Not present |  |  |
| simultaneousU-TCI-UpdateList2-r17 | Not present |  |  |
| simultaneousU-TCI-UpdateList3-r17 | Not present |  |  |
| simultaneousU-TCI-UpdateList4-r17 | Not present |  |  |
| rlc-BearerToReleaseListExt-r17 | Not present |  |  |
| iab-ResourceConfigToAddModList-r17 | Not present |  |  |
| iab-ResourceConfigToReleaseList-r17 | Not present |  |  |
| reportUplinkTxDirectCurrentMoreCarrier-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |
| CFRA | This condition applies when CFRA is configured |
| SUL | Supplementary Uplink |
| SRB1 | Establishment of SRB1 |
| SRB2_DRB1 | Establishment of SRB2 and DRB1 |
| SRB2_DRB2 | Establishment of SRB2 and DRB2 |
| DRBn | Establishment of DRBn |
| PCell_change | Intra-NR PCell change NR) |
| PSCell_change | NR PSCell change (EN-DC) |
| SCell_add | Add SCell |
| MEAS | A NR or EN-DC measurement is configured |
| NR-DC_SCG | Add SCG (NR-DC) |
| RESUME | Used in RRCResume Message |
| REEST | The first RRCReconfiguration message after successful completion of the RRC re-establishment procedure. |
| UL_CA | Uplink CA |
| MRBm | Establishment of MRBm |
| MRBm_DRBn | Establishment of MRBm and DRBn |
| AM_PTP | Multicast MRB with RLC-AM entity configuration for receiving PTP transmission |
| UM_bi_PTP | Multicast MRB with bidirectional RLC-UM configuration for receiving PTP transmission |
| UM_PTP | Multicast MRB with DL only RLC-UM configuration for receiving PTP transmission |
| UM_PTM | Multicast MRB with DL only RLC-UM entity for receiving PTM transmission |
| AMPTP_UMPTM | Multicast MRB with two RLC entities, one RLC-AM entity for receiving PTP transmission and the other DL only RLC-UM entity for receiving PTM transmission |
| UMPTP_UMPTM | Multicast MRB with two RLC-UM entities, one DL only RLC-UM entity for receiving PTP transmission and the other DL only RLC-UM entity for receiving PTM transmission |
| RELAY | This condition applies when Relay UE is configured |

<Unchanged Skip>

4.6.6	Sidelink information elements

<Unchanged Skip>

–	SL-LogicalChannelConfig

Table 4.6.6-12: SL-LogicalChannelConfig

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-LogicalChannelConfig-r16 ::= SEQUENCE { |  |  |  |  |
| sl-Priority-r16 |  | 1 |  |  |
| sl-PrioritisedBitRate-r16 |  | infinity |  |  |
| sl-BucketSizeDuration-r16 |  | ms50 |  |  |
| sl-ConfiguredGrantType1Allowed-r16 |  | Not present |  |  |
| sl-HARQ-FeedbackEnabled-r16 |  | enabled |  |  |
| sl-AllowedCG-List-r16 |  | Not present |  |  |
| sl-AllowedSCS-List-r16 |  | Not present |  |  |
| sl-MaxPUSCH-Duration-r16 |  | Not present |  |  |
| sl-LogicalChannelGroup-r16 |  | 1 | Avoiding using LCG #0, which is reserved for SL SRB |  |
|  |  | 5 | Avoiding using LCG #0, which s reserved for SL SRB | LO |
| sl-SchedulingRequestId-r16 |  | SchedulingRequestId |  |  |
| sl-LogicalChannelSR-DelayTimerApplied-r16 |  | Not present |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| LO | LCG of the lower priority logical channels mapped to SL DRBs |

–	SL-L2RelayUE-Config

Table 4.6.6-12A: SL-L2RelayUE-Config

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-L2RelayUE-Config -r17 ::= SEQUENCE { |  |  |  |  |
| sl-RemoteUE-ToAddModList-r17 |  | FFS |  |  |
| sl-RemoteUE-ToReleaseList-r17 |  | FFS |  |  |
| } |  |  |  |  |

–	SL-L2RemoteUE-Config

Table 4.6.6-12B: SL-L2RemoteUE-Config

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-L2RemoteUE-Config -r17 ::= SEQUENCE { |  |  |  |  |
| sl-SRAP-ConfigRemote-r17 |  | FFS |  |  |
| sl-UEIdentityRemote-r17 |  | FFS |  |  |
| } |  |  |  |  |

<Unchanged Skip>

–	SL-RLC-BearerConfig

Table 4.6.6-26: SL-RLC-BearerConfig

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-RLC-BearerConfig-r16 ::= SEQUENCE { |  |  |  |  |
| sl-RLC-BearerConfigIndex-r16 |  | SL-RLC-BearerConfigIndex |  |  |
| sl-ServedRadioBearer-r16 |  | SLRB-Uu-ConfigIndex |  |  |
| sl-RLC-Config-r16 |  | SL-RLC-Config with condition AM |  |  |
| sl-MAC-LogicalChannelConfig-r16 |  | SL-LogicalChannelConfig |  |  |
| } |  |  |  |  |

–	SL-RelayUE-Config

Table 4.6.6-26A: SL-RelayUE-Config

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-RelayUE-Config-r17 ::= SEQUENCE { |  |  |  |  |
| threshHighRelay-r1 |  | Not present |  |  |
| threshLowRelay-r17 |  | RSRP-Range |  |  |
| hystMaxRelay-r17 |  | Not present |  |  |
| hystMinRelay-r17 |  | Not present |  |  |
| } |  |  |  |  |

–	SL-RemoteUE-Config

Table 4.6.6-26B: SL-RemoteUE-Config

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-RemoteUE-Config-r17 ::= SEQUENCE { |  |  |  |  |
| threshHighRemote-r17 |  | 36 |  |  |
| hystMaxRemote-r17 |  | Not present |  |  |
| sl-ReselectionConfig-r17 SEQUENCE { |  |  |  |  |
| sl-RSRP-Thresh-r17 |  | 4 |  |  |
| sl-FilterCoefficientRSRP-r17 |  | Not present |  |  |
| sl-HystMin-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<End of Changed Section>