3GPP TSG-RAN5 Meeting #98	R5-231023

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2733 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of default RRC message configuration for measurement gap enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add RRC IEs for Rel-17 measurement gap enhancement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MeasGapConfig was updated to include measurement gap enhancement according to TS 38.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Default message configuration will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

4.6.3	Radio resource control information elements

–	AdditionalSpectrumEmission

[Skip unchanged text]

–	MeasGapConfig

Table 4.6.3-70: MeasGapConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapFR2 | Not present |  |  |
| gapFR2 CHOICE { |  |  | GAP_FR2 |
| setup SEQUENCE { |  |  |  |
| gapOffset | 159 |  |  |
| mgl | ms3dot5 |  |  |
| mgrp | ms160 |  |  |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| gapFR1 | Not present |  |  |
| gapFR1 CHOICE { |  |  | GAP_FR1 |
| setup SEQUENCE { |  |  |  |
| gapOffset | 39 |  |  |
|  | 9 |  | SIG AND INTER-FREQ_ODD |
| mgl | ms6 |  |  |
| mgrp | ms40 |  |  |
| mgta | ms0 |  |  |
| refServCellIndicator | Not present |  |  |
| refFR2ServCellAsyncCA-r16 | Not present |  |  |
| mgl-r16 | Not present |  |  |
|  | ms10 |  | PRS |
| } |  |  |  |
| gapUE | Not present |  | GAP_FR1 OR GAP_FR2 |
| gapUE CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset | 39 |  |  |
|  | 9 |  | SIG AND INTER-FREQ_ODD |
| mgl | ms6 |  |  |
| mgrp | ms40 |  |  |
| mgta | ms0 |  |  |
| refServCellIndicator | Not present |  |  |
| refFR2ServCellAsyncCA-r16 | Not present |  |  |
| mgl-r16 | Not present |  |  |
|  | ms10 |  | PRS |
| } |  |  |  |
| } |  |  |  |
| gapToAddModList-r17 | Not present |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 { | 1 entry |  | GAP_ADD |
| GapConfig-r17[1] SEQUENCE { |  |  | entry 1 |
| measGapId-r17 | MeasGapId |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 39 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms40 |  |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
|  | true |  | Pre-config |
| ncsgInd-r17 | Not present |  |  |
|  | true |  | NCSG |
| gapAssociationPRS-r17 | Not present |  |  |
|  | true |  | PRS |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| gapToReleaseList-r17 | Not present |  |  |
| gapToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF MeasGapId-r17 { | 1 entry |  | GAP_RELEASE |
| MeasGapId-r17[1] | 1 |  | entry 1 |
| } |  |  |  |
| posMeasGapPreConfigToAddModList-r17 | Not present |  |  |
| posMeasGapPreConfigToAddModList-r17 SEQUENCE (SIZE (1..maxNrofPreConfigPosGapId-r17)) OF PosGapConfig-r17 { | 1 entry |  | PRS_GAP_ADD |
| PosGapConfig-r17[1] SEQUENCE { |  |  | entry 1 |
| measPosPreConfigGapId-r17 | 1 |  |  |
| gapOffset-r17 | 39 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms40 |  |  |
| mgta-r17 | ms0 |  |  |
| gapType-r17 | perUE |  |  |
| } |  |  |  |
| posMeasGapPreConfigToReleaseList-r17 | Not present |  |  |
| posMeasGapPreConfigToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofPreConfigPosGapId-r17)) OF MeasPosPreConfigGapId-r17 { | 1 entry |  | PRS_GAP_RELEASE |
| MeasPosPreConfigGapId-r17[1] | 1 |  | entry 1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GAP_FR1 | Configuration for FR1 per-FR gaps |
| GAP_FR2 | Configuration for FR2 per-FR gaps |
| INTER-FREQ_ODD | When the SFNoffset of inter frequency neighbour cell is odd number. SFNoffset is defined in TS 38.523-3 [23]Table 7.1.5.2-1 |
| GAP_ADD | Addition or modification of measurement gaps |
| GAP_RELEASE | Release of measurement gaps |
| PRS | PRS measurement |
| PRS_GAP_ADD | Addition or modification of preconfigured measurement gap for positioning |
| PRS_GAP_RELEASE | Addition or modification of preconfigured measurement gap for positioning |

–	MeasGapId

Table 4.6.3-70A: MeasGapId

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapId-r17 | 1 |  |  |

–	MeasGapSharingConfig

Table 4.6.3-71: MeasGapSharingConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapSharingConfig ::= SEQUENCE { |  |  |  |
| gapSharingFR2 | Not present |  |  |
| } |  |  |  |

[End of change]