# Title:						Test Tolerances for cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion

# Source:					Huawei, HiSilicon, Ericsson

# 1.	Introduction

This document relates to the Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion test case in TS 38.533 and is also applicable to the 2 Rx RedCap UE test case 16.1.1.8.

The test cases in question are:

- 6.1.1.5, NR SA FR1- FR1 cell re-selection,

- 16.1.1.8, NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Prototype test case in Annex A of TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v16.9.0 Annex A.

#### A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion

##### A.6.1.1.5.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements specified in clause 4.2.2.10.2, for UE fulfilling low mobility relaxed measurement criterion.

##### A.6.1.1.5.2	Test Parameters

The test scenario comprises of 2 cells on 2 different NR carriers respectively as given in tables A.6.1.1.5.2-1, A.6.1.1.5.2-2 and A.6.1.1.5.2-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1.

As specified in the Test Purpose, the UE is configured with the relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1]. So, Cell 2 and Cell 1 configure the UE as follows:

lowMobilityEvalutation [2] criterion is configured according to the parameters listed in Table A.6.1.1.5.2-3;

cellEdgeEvaluation [2] criterion is not configured;

combineRelaxedMeasCondition [2] is not configured;

Table A.6.1.1.5.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.1.5.2-2: General test parameters for FR1 inter frequency NR cell re-selection test case for UE fulfilling low mobility criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell2 | The UE camps on cell 2 in the initial phase, it fulfills Low Mobility relaxation measurements criterion, and during T1 period the UE reselects to cell 1 |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| T1 end condition | Active cell |  | 1, 2, 3 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2, 3 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T2 |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB Configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 | Configured in SIB4 of Cell 1 |
|  |  |  |  | SMTC pattern 6 | Configured in SIB4 of Cell 2 |
|  |  |  | 2 | SMTC pattern 1 |  |
|  |  |  | 3 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 25 s | T1 is defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3 | 25 s | T2 is defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.5.2-3: Cell specific test parameters for FR1 inter frequency NR cell re-selection test case in AWGN for UE fulfilling low mobility criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  | -140 |  |  |
|  |  | 3 | -137 |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  | 0 |  |  |
| Qhysts | dB | 1, 2, 3 | 0 |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | -4 | 12 |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1 | 14 | 14 | -4 |  | 12 |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -84 | -84 | -102 |  | -86 |
|  |  | 2 | -84 | -84 | -102 |  | -86 |
|  |  | 3 | -81 | -81 | -99 |  | -83 |
| Io | dBm/9.36 MHz | 1 | -55.88 | -55.88 | -68.60 |  | -57.78 |
|  | dBm/9.36 MHz | 2 | -55.88 | -55.88 | -68.60 |  | -57.78 |
|  | dBm/38.16 MHz | 3 | -49.79 | -49.79 | -62.50 |  | -51.69 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 |  | 0 |
| SnonintrasearchP | dB | 1, 2, 3 | Not sent |  | Not sent |  |  |
| Threshx, highP | dB | 1, 2, 3 | 48 |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3 | 44 |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3 | 50 |  | 50 |  |  |
| SSearchDeltaP | dB | 1, 2, 3 | 3 |  | 3 |  |  |
| TSearchDeltaP | s | 1, 2, 3 | 5 |  | 5 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.6.1.1.5.3	Test Requirements

The cell reselection delay to an already detected lower priority cell for UE fulfilling low mobility relaxed measurements is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to a lower priority cell for UE fulfilling low mobility relaxed measurements shall be less than 17 s.

The cell reselection delay to an already detected higher priority cell for UE fulfilling low mobility relaxed measurements is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to an already detected higher priority cell for UE fulfilling low mobility relaxed measurements shall be less than 17 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a known lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR,

Where:

Tevaluate, NR_ inter	See Table 4.2.2.10.2-1 in clause 4.2.2.10.2

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 16.64 s, allow 17s  for the cell re-selection delay to an already detected lower priority cell and 16.64s for the cell re-selection delay to an already detected higher priority cell, which we allow 17s for UE fulfilling low mobility relaxed measurements in the test case.

# 3. Discussion

The test case has 2 time periods: T1 and T2.

3.1 Selection criterion

The selection criterion is satisfied according to the equation from TS 38.304 v15.3.0 clause 5.2.3.2:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset ) – Pcompensation - Qoffsettemp

The values in this equation are derived as follows:

- Srxlev is evaluated for each cell at T1 and T2.

- Qrxlevmeas is the Measured cell RX level value (SS-RSRP) as given in table A.6.1.1.5.2-3

- Qrxlevmin is -140dBm under Config 1 as given in table table A.6.1.1.5.2-3.

- Qrxlevminoffset is only taken into account as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN, and is therefore not applicable here.

- Pcompensation = 0 dB as given in table A.6.1.1.5.2-3.

- Qoffsettemp = 0 dB as given in table A.6.1.1.5.2-3.

The equation therefore reduces to:

Srxlev = Qrxlevmeas – Qrxlevmin

3.2 Thresholds

Following thresholds are used：

- ThreshX, HighP = 48 dB according to 38.133 Table A.6.1.1.5.2-3.

- ThreshX, LowP = 50 dB according to 38.133 Table A.6.1.1.5.2-3.

- ThreshServing, LowP = 44 dB according to 38.133 Table A.6.1.1.5.2-3.

- SnonIntraSearchP is not configured according to 38.133 Table A.6.1.1.5.2-3. So UE shall use the default value of infinity.

- SSearchDeltaP = 3 dB according to 38.133 Table A.6.1.1.5.2-3.

The thresholds are defined in TS 38.304 v15.3.0 clause 5.2.4.7:

ThreshX, HighP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold.

ThreshX, LowP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of NR and E-UTRAN might have a specific threshold.

ThreshServing, LowP

This specifies the Srxlev threshold (in dB) used by the UE on the serving cell when reselecting towards a lower priority RAT/ frequency.

SnonIntraSearchP

This specifies the Srxlev threshold (in dB) for NR inter-frequency and inter-RAT measurements.

SSearchDeltaP

This specifies the threshold (in dB) on Srxlev variation for relaxed measurement.

Details about SnonIntraSearchP is given in TS 38.304 clause 5.2.4.2:

#### 5.2.4.2	Measurement rules for cell re-selection

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for NR inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For a NR inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency, the UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8].

-	For a NR inter-frequency with an equal or lower reselection priority than the reselection priority of the current NR frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current NR frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority;

-	Otherwise, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8].

-	If the UE supports relaxed measurement and relaxedMeasurement is present in SIB2, the UE may further relax the needed measurements, as specified in clause 5.2.4.9.

Details about ThreshX, HighP, ThreshX, LowP, ThreshServing, LowP are given in TS 38.304 clause 5.2.4.5:

#### 5.2.4.5	NR Inter-frequency and inter-RAT Cell Reselection criteria

If threshServingLowQ is broadcast in system information and more than 1 second has elapsed since the UE camped on the current serving cell, cell reselection to a cell on a higher priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority NR or EUTRAN RAT/frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT

Otherwise, cell reselection to a cell on a higher priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority NR frequency shall be based on ranking for intra-frequency cell reselection as defined in clause 5.2.4.6.

If threshServingLowQ is broadcast in system information and more than 1 second has elapsed since the UE camped on the current serving cell, cell reselection to a cell on a lower priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority NR or E-UTRAN RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT.

Otherwise, cell reselection to a cell on a lower priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/frequency shall take precedence over a lower priority RAT/frequency if multiple cells of different priorities fulfil the cell reselection criteria.

If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an NR frequency, the highest ranked cell among the cells on the highest priority frequency(ies) meeting the criteria according to clause 5.2.4.6;

-	If the highest-priority frequency is from another RAT, the strongest cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

Details about SSearchDeltaP are given in TS 38.304 clause 5.2.4.9.1:

##### 5.2.4.9.1	Relaxed measurement criterion for UE with low mobility

The relaxed measurement criterion for UE with low mobility is fulfilled when:

-	(SrxlevRef – Srxlev) < SSearchDeltaP,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows:

-	After selecting or reselecting a new cell, or

-	If (Srxlev - SrxlevRef) > 0, or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP:

-	The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell.

3.3 Test case verdict

The design of the test case relies on the UE being able to measure SS-RSRP during T1 and T2, and correctly compare it to the signalled thresholds.

A detailed analysis of the test case is given in section 5 of this document.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Table A.6.1.1.5.2-3 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test. Some signalled parameters Qrxlevmin, Snonintrasearch, ThreshX, HighP, ThreshServing, LowP, and ThreshX, LowP are also copied, although these are not subject to uncertainty.

The key parameters appear in section a) of the accompanying spreadsheet. The layout for Cell1 and Cell 2 is similar to Table A.6.1.1.5.2-3 in TS 38.133, but the AWGN is given a separate set of column. This allows the spreadsheet calculations to be done in a consistent way.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet. The UE compares Srxlev values to thresholds, so the SS-RSRP values need to be converted to Srxlev.

In this test case the UE compares Srxlev values to thresholds, so a further step is done to calculate the difference between Srxlev and the thresholds:

- (Cell 1 Srxlev - Thresh_X_LowP)

- (Cell 1 Srxlev - SnonintrasearchP)

- (Cell 2 Srxlev - Thresh_Serving_LowP)

- (Cell 2 Srxlev - Thresh_X_HighP)

- (Cell 1 SrxlevRef – Cell 1Srxlev)

- (Cell 2 SrxlevRef – Cell 2Srxlev)

c) Uncertainties

The SS provides 2 cells on different frequencies, each with AWGN. We propose to control the following parameters:

- AWGN absolute power on cell 1 frequency, Noc1 ±1.5 dB

- Ratio of NR Cell 1 signal / noise, Ês1 / Noc1 ±0.3 dB

- AWGN absolute power on cell 2 frequency, Noc2 ±1.5 dB

- Ratio of NR Cell 2 signal / noise, Ês2 / Noc2 ±0.3 dB

This choice forms a minimum set, so the superposition principle can be applied if necessary. For these test cases the signals are not faded, and the values for Ês / Noc are chosen to be similar to equivalent parameters in E-UTRAN. The absolute level uncertainties of Noc are chosen to ensure the analysis and Test Requirements are valid for frequency bands up to 7.125 GHz.

For inter-frequency reselection, the UE ranking requirement points to the side conditions for reselection measurements in the following extract from TS 38.133:

#### 4.2.2.4	Measurements of inter-frequency NR cells

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS38.304 [1] within Kcarrier * Tdetect,NR_Inter  if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least 5 dB in FR1 or 6.5dB in FR2 for reselections based on ranking or 6dB in FR1 or 7.5dB in FR2 for SS-RSRP reselections based on absolute priorities or 4dB in FR1 and 4dB in FR2 for SS-RSRQ reselections based on absolute priorities.

The parameter Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell. The parameter Kcarrier for a UE configured with idle mode CA measurements (while T331 is running), is the combined number of NR inter-frequency carriers indicated by the serving cell and the number of NR inter-frequency carriers configured for idle mode CA measurements.

Note:	combined total number means that if a carrier is an inter-frequency carrier indicated by the serving cell for mobility and additionally a carrier configured for idle mode CA measurements, it only counts as one carrier.

An inter-frequency cell is considered to be detectable according to the conditions defined in Annex B.1.3 for a corresponding Band.

In TS 38.133, clause B.1.3 for Inter-frequency refers to Table B.1.2-1 Intra-frequency.

## B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection

This clause defines the following conditions for NR inter-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions defined in Table B.1.2-1 for FR1 NR intra-frequency cell re-selection shall also apply for FR1 NR inter-frequency cells in this clause.

The conditions defined in Table B.1.2-2 for FR2 NR intra-frequency cell re-selection shall also apply for FR2 NR inter-frequency cells in this clause.

## B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection

This section defines the following conditions for NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.1.2-1 for FR1 NR cells.

Table B.1.2-1: Conditions for intra-frequency cell re-selection in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -124 | -121 | -4 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.2. |  |  |  |  |

In summary the SSB Es/Iot of both NR Cell 1 and Cell 2 shall be above -4dB, the cell should have at least 6 dB better ranked and the tightest requirement for SSB_RP shall be above -120.5dBm/15kHz or -117.5dBm/30kHz for band group NR_FDD_FR1_H.

d) Controlled parameters critical to verdict

From 38.533 clause 6.1.1.5.4.2, Cell 2 is of higher priority than Cell 1.

- Before the start of T1 the UE has selected Cell 2 (high priority), and is required to reselect to Cell 1 (low priority) during T1. The conditions for the reselection towards lower priority are:

- Threshx, HighP is not applicable during T1

- The low priority Cell 1 must be good enough, expressed as Cell 1 Srxlev ≥ ThreshX, LowP with a margin of at least 6dB

- The high priority Cell 2 must be bad enough, expressed as Cell 2 Srxlev ≤. ThreshServing, LowP with a margin of at least 6dB.

- SnonIntraSearchP is not configured in TC. UE shall use the value of infinity by default, which means that measurement restriction rule doesn't work during the test. UE shall always perform inter-frequency measurement on neighbour cells.

- low mobility criterion must be satisfied. expressed as Cell 2 SrxlevRef – Cell 2 Srxlev < SSearchDeltaP .According to 38.304 cl.5.2.4.9.1 UE will update SrxlevRef to current Srxlev when it reselects a cell. Considering that UE reselects Cell 2 during T2, Cell 2 SrxlevRef should be Cell 2 Srxlev during T2.

- Both cells must meet the SSB_RP and SSB Es/Iot side conditions in TS 38.133 clause 4.2.2.4

- During T2 the UE is required to reselect to Cell 2 (high priority). The conditions for the reselection towards higher priority are:

- The high priority Cell 2 must be good enough, expressed as Cell 1 Srxlev ≥. ThreshX, HighP, with a margin of at least 6dB.

- ThreshX, LowP and ThreshServing, LowP are not applicable during T2.

- SnonIntraSearchP is not configured in TC. UE shall use the value of infinity by default, which means that measurement restriction rule doesn't work during the test. UE shall always perform inter-frequency measurement on neighbour cells.

- low mobility criterion must be satisfied. expressed as Cell 1 SrxlevRef – Cell 1 Srxlev < SSearchDeltaP .According to 38.304 cl.5.2.4.9.1 UE will update SrxlevRef to current Srxlev when it reselects a cell. Considering that UE reselects Cell 1 during T1, Cell 1 SrxlevRef should be Cell 1 Srxlev during T1.

- Both cells must meet the SSB_RP and SSB Es/Iot side conditions in TS 38.133 clause 4.2.2.4

The critical conditions are related to the controlled parameters listed in section d) of the accompanying spreadsheet. They have been derived by study of the test case diagram and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given later in the “Comment” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. The sensitivity factors are the ratio (effect on a critical parameter y / a test equipment uncertainty x), and are usually in dB/dB. In this RRM test case there is a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict. The sensitivity factors are therefore derived by inspection as one or zero.

For example, an error of 1dB in the absolute AWGN level Noc1 would cause 1dB error in the Cell 1 SS-RSRP, so the sensitivity factor is 1.000 during T1, T2 and T3. It would also cause 1dB error in the derived parameter (Cell 1 Srxlev - Thresh_X_LowP), so the sensitivity factor is also 1.000 during T1, T2 and T3. However the same error of 1dB in the absolute AWGN level Noc1 would cause no change to Cell 1 Es/Iot, because all other powers are specified relative to Noc1, so the sensitivity factor is zero.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903. Uncertainties are calculated separately for T1, T2 and T3.

The normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

e) Determine parameters to offset

During T1,

- Cell 2 Ês/Iot just meeting the side condition of Ês/Iot   -4 dB in TS 38.133 clause 4.2.2.4. The Es/Noc of cell 2 is therefore increased by the amount of its uncertainty, to ensure it does not fall below the minimum value of -4dB.

- the Srxlev of high priority Cell 2 shall be set to be below ThreshServing, LowP with a margin of 6dB. Then ThreshServing, LowP is therefore decreased by the (Cell 2 Srxlev - ThreshServing_LowP) uncertainty. This ensures that the margin of 6dB is maintained.

- Cell 2 Es/Noc during T2 shall be set to ensure Cell 2 SrxlevRef – Cell 2 Srxlev < SSearchDeltaP, Then SSearchDeltaP  for Cell 2 is increased and Cell 2 Es/Noc is decreased by the (Cell 2 SrxlevRef – Cell 2 Srxlev) uncertainty.

- the Srxlev of low priority Cell 1 shall be set to be above ThreshX, LowP with a margin of 6dB. Then ThreshX_LowP is therefore decreased by the (Cell 1 Srxlev - ThreshX_LowP) uncertainty. This ensures that the margin of 6dB is maintained.

During T2,

- the Srxlev of high priority Cell 2 shall be set to be above ThreshX, HighP with a margin of 6dB. Then ThreshX, HighP is therefore decreased by the (Cell 2 Srxlev - Thresh_X_HighP) uncertainty. This ensures that the margin of 6dB is maintained.

The check that all other controlled parameters meet their required range is done in step g). In theory it is possible for steps e) to g) to be iterative, or possibly even steps c) to g) to be iterative, but this test case can be done without such iteration.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets). For example, during T1, T2 and T3 Es/Noc on frequency 1 is increased by just enough to cover the variability from uncertainties.

Re-derived parameters are calculated using the same methods as were used in step b).

- Cell 2 Es/Noc during T1 is increased by 0.3dB, and during T2 is decreased by 2.25dB;

- ThreshX,LowP is decreased by 2dB;

- ThreshX,HighP is decreased by 4dB;

- ThreshServing, LowP is increased by 2dB;

- SSearchDeltaP for Cell 2 is increased by 12 dB since its maximum value is 15dB.

This value is found empirically by observing the minimum/maximum parameter values in step g).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated, as at least some will have changed from the original due to the application of the Test Tolerances in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (Required margin relative to threshold, Es/Iot range, SS-RSRP level). It is not necessary to calculate all parameters during each time interval T1 and T2, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit (in these test cases, all limits are one-sided) are identified by turquoise cells in the spreadsheet. If all requirements are met, then the exercise is complete.

It can be seen that with the uncertainty values and Test Tolerances proposed, all the requirements are met.

# 5. Treatment of other configurations within the same test case

TS 38.533 test case 6.1.1.5 also contains configurations with different SCSs and different channel bandwidths. As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed:

- Subcarrier spacing, section a)

- Channel bandwidth, section a)

- RBs in Channel bandwidth, section a) and associated comment

- NR Cell 1 and NR Cell 2 SSB_RP minimum requirement, section g) and associated Source/Reference

# 6. Treatment of 2Rx RedCap test case A.16.1.1.8

TS 38.533 also contains similar test case 16.1.1.8 for 2 Rx RedCap UEs in clause 16.1.1. Same level of uncertainties applies to these test cases since Rel-15 legacy UE and Rel-17 RedCap UE with 2 Rx share the same requirements according to 38.133. For these 2 Rx RedCap test cases:

- Test parameters for config 1/2/4/ are identical with that of 6.1.1.5 config 1/2 with the exception that HD-FDD duplex mode is used in config 4. Then the same test tolerances for 6.1.1.5 config 1/2/4 apply considering duplex mode bringing no any additional measurement uncertainties.

- Test parameters for config 3 are identical with that of 6.1.1.5 config 3 with the exception that the BW is only 20MHz (51 RBs). As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed comparing with 6.1.1.5 config 3:

  - Channel bandwidth and RBs in Channel bandwidth, section a) and associated comment

From the analysis one can observe that the only difference is that Io value is 10*(log106/51) = 3.18dB lower. Then the same test tolerances for 6.1.1.3 config 3 apply considering that for this test case there are no minimum requirements for Io (Io is not listed in the d) Controlled parameters critical to verdict).