3GPP TSG-RAN5 Meeting #98	R5-231168

# Athens, 27th  Feb 2023- 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3590 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the eCall TC 11.5.1-T3444 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the call release procedure, the test case doesn’t handle the NAS PDU session and RRC actions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the call release procedure steps to call the generic procedure specified in TS 38.508-1, with the process of NAS PDU session and RRC layer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformance UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

11.5.1	eCall Only mode / T3444 / eCall inactivity procedure / Removal of eCall only restriction after an eCall over IMS / 5GS to EPS

11.5.1.1	Test Purpose (TP)

(1)

with { UE is switched ON with eCall only enabled USIM }

ensure that {

when { UE reads ims-EmergencySupport and eCallOverIMS-Support from SIB1 }

then { UE enters substate 5GMM-DEREGISTERED.eCALL-INACTIVE and shall not start registration procedure }

}

(2)

with { The UE is in the state 5GMM-DEREGISTERED.eCALL-INACTIVE }

ensure that {

when { UE is requested to make a manual eCall }

then { UE sends REGISTRATION REQUEST message with 5GS registration type set to 'initial registration' }

}

(3)

with { UE receives REGISTRATION ACCEPT message and IMS voice over PS session is supported over 3GPP access }

ensure that {

when { UE is in 5GMM-REGISTERED.NORMAL-SERVICE state and an initial IMS registration is performed }

then { the UE establishes a New emergency PDU session by sending an UL NAS TRANSPORT message with Request type set to "initial emergency request" and a PDU SESSION ESTABLISHMENT REQUEST }

}

(4)

with { UE receives an RRCRelease message and enters RRC_IDLE state }

ensure that {

when { UE receives a Paging message with MT MMTEL voice call }

then {the UE answers the paging request for MT MMTEL voice call }

}

(5)

with { UE receives an RRCRelease message and enters RRC_IDLE state }

ensure that {

when { UE starts T3444 for 12Hours and the periodic registration update timer T3512 expires }

then { the UE initiates the registration procedure for mobility and periodic registration update

and indicates "periodic registration updating" in the 5GS registration type IE }

}

(6)

with { UE in state 5GMM-REGISTERED and 5GMM-IDLE on a 5GC NR cell }

ensure that {

when { UE detects a suitable EPC E-UTRA cell after the serving NGC cell becomes not suitable when eCall Inactivity timer T3444 is running }

then { UE performs a Inter-system change from N1 mode to S1 mode by initiating and successfully
completing a TAU procedure }

}

(7)

with { UE in state EMM-REGISTERED and EMM-IDLE on an E-UTRA cell  }

ensure that {

when { UE detects a suitable NGC cell after the serving E-UTRA cell becomes not suitable when eCall Inactivity timer T3444 is running }

then { UE performs a Inter-system change from S1 mode to N1 mode by initiating and successfully
completing a mobility and periodic registration update procedure }

}

(8)

with { UE is in RRC_IDLE state }

ensure that {

when { eCall Inactivity timer T3444 expires }

then { The UE performs Deregistration procedure }

}

11.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in TS 24.501, clauses 5.1.3.2.1.3.8, 5.3.1.3, 5.5.3, TS 24.301, clauses 10.2 and TS 38.331, clauses 5.2.2.4.2, 6.2.2. Unless otherwise stated these are Rel-16 requirements.

[TS 24.501, clause 5.1.3.2.1.3.8]

The substate 5GMM-DEREGISTERED.eCALL-INACTIVE is chosen in the UE when:

a)	the UE is configured for eCall only mode as specified in 3GPP TS 31.102 [22];

b)	timer T3444 and timer T3445 have expired or are not running;

c)	a PLMN has been selected as specified in 3GPP TS 23.122 [5];

d)	the UE does not need to perform an eCall over IMS; and

e)	the UE does not need to perform a call to a non-emergency MSISDN or URI for test or terminal reconfiguration service.

In this substate, the UE shall not initiate any signalling towards the network, except to originate an eCall over IMS, or a call to a non-emergency MSISDN or URI for test or terminal reconfiguration service.

[TS 24.501, clause 5.3.1.3]

The signalling procedure for the release of the N1 NAS signalling connection is initiated by the network.

In N1 mode, upon indication from lower layers that the access stratum connection has been released, the UE shall enter 5GMM-IDLE mode and consider the N1 NAS signalling connection released.

If the UE in 3GPP access is configured for eCall only mode as specified in 3GPP TS 31.102 [22] then:

-	if the N1 NAS signalling connection that was released had been established for eCall over IMS, the UE shall start timer T3444; and

-	if the N1 NAS signalling connection that was released had been established for a call to an HPLMN designated non-emergency MSISDN or URI for test or terminal reconfiguration service, the UE shall start timer T3445.

[TS 24.501, clause 5.5.3]

The eCall inactivity procedure is performed only in 3GPP access and applicable only to a UE configured for eCall only mode as specified in 3GPP TS 31.102 [22]. The procedure shall be started when:

a)	the UE is in any 5GMM-REGISTERED substate except substates 5GMM-REGISTERED.PLMN-SEARCH or 5GMM-REGISTERED.NO-CELL-AVAILABLE;

b)	the UE is in 5GMM-IDLE mode; and

c)	one of the following conditions applies:

1)	timer T3444 expires or is found to have already expired and timer T3445 is not running;

2)	timer T3445 expires or is found to have already expired and timer T3444 is not running; or

3)	timers T3444 and T3445 expire or are found to have already expired.

The UE shall then perform the following actions:

a)	stop other running timers (e.g. T3511, T3512);

b)	if the UE is currently registered to the network for 5GS services, perform a de-registration procedure;

c)	delete any 5G-GUTI, TAI list, last visited registered TAI, list of equivalent PLMNs, and ngKSI; and

d)	enter 5GMM-DEREGISTERED.eCALL-INACTIVE state.

[TS 24.301, clause 10.2]

| T3444 | NOTE 11 | All except EMM-NULL and 5GMM-NULL (defined in 3GPP TS 24.501 [54]) | - UE configured for eCall only mode enters EMM-IDLE mode after an eCall over IMS - UE configured for eCall only mode moves from GERAN/UTRAN to E-UTRAN with timer T3242 (see 3GPP TS 24.008 [13]) running - UE configured for eCall only mode enters 5GMM-IDLE mode (defined in 3GPP TS 24.501 [54]) after an eCall over IMS | - Removal of eCall only restriction - Intersystem change from S1 mode to A/Gb or Iu mode | Perform eCall inactivity procedure in EPS as described in clause 5.5.4. Perform eCall inactivity procedure in 5GS as described in 3GPP TS 24.501 [54]. |
| --- | --- | --- | --- | --- | --- |
| T3445 | NOTE 12 | All except EMM-NULL and 5GMM-NULL (defined in 3GPP TS 24.501 [54]) | - UE configured for eCall only mode enters EMM-IDLE mode after a call to a non-emergency MSISDN or URI for test or terminal reconfiguration service - UE configured for eCall only mode moves from GERAN/UTRAN to E-UTRAN with timer T3243 (see 3GPP TS 24.008 [13]) running - UE configured for eCall only mode enters 5GMM-IDLE mode (defined in 3GPP TS 24.501 [54]) after a call to a non-emergency MSISDN or URI for test or terminal reconfiguration service | Removal of eCall only restriction - Intersystem change from S1 mode to A/Gb or Iu mode | Perform eCall inactivity procedure in EPS as described in clause 5.5.4. Perform eCall inactivity procedure in 5GS as described in 3GPP TS 24.501 [54]. |

[TS 38.331, clause 5.2.2.4.2]

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

…

4>	forward the eCallOverIMS-Support to upper layers, if present;

[TS 38.331, clause 6.2.2]

eCallOverIMS-Support

Indicates whether the cell supports eCall over IMS services as defined in TS 23.501 [32]. If absent, eCall over IMS is not supported by the network in the cell.

11.5.1.3	Test description

11.5.1.3.1	Pre-test conditions

System Simulator:

-	2 cells

-	NR Cell 1 as defined in TS 38.508-1 [4] Table 4.4.2-3. System information combination NR-6 as defined in TS 38.508-1 [4], sub-clause 4.4.3.1.2.

-	E-UTRA Cell 1 as defined in TS 36.508 [7] Table 4.4.2-2. System information combination 31 as defined in TS 36.508 [7], sub-clause 4.4.3.1.1.

UE:

-	the eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 38.508-1 [4] Table 6.4.1-24.

Preamble:

-	 The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2.

11.5.1.3.2	Test procedure sequence

Table 11.5.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - NR Cell 1 as "Serving cell" - E-UTRA Cell 1 as "Non-suitable "off" cell". | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | Check: Does the UE send an RRCSetupRequest on NR Cell 1 within 120 seconds? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 4 | A manual eCall is initiated. (Note 1) | - | - | - | - |
| 5 | Check: Does the UE send REGISTRATION REQUEST message with 5GS registration type set to 'initial registration'? | --> | 5GMM: REGISTRATION REQUEST | 2 | P |
| 6-14 | Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 15 | The SS transmits a REGISTRATION ACCEPT message and assigns value of 7 hours for the timer T3512. | <-- | 5GMM: REGISTRATION ACCEPT | - | - |
| 16-20a1 | Steps 15-19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 21 | Check: The UE transmits an ULInformationTransfer message with Request type set to "initial emergency request" and a PDU SESSION ESTABLISHMENT REQUEST message? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | 3 | P |
| 22-26 | Steps 22-26 of Table 4.9.29.2.2-1 of the test procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 27-28 | Release eCall over IMS using the generic procedure described in TS 38.508-1 [4] subclause 4.9.12B. | - | - | - | - |
| 29 | The SS releases the RRC connection. (Note 2) | <-- | NR RRC: RRCRelease | - | - |
| 30 | The SS waits 1 minute. | - | - | - | - |
| 31-43 | Steps 1 to 13 of the generic test procedure for IMS MT speech call establishment in 5GC specified in TS 38.508-1 [4] subclause 4.9.16 are performed. | - | - | 4 | P |
| 44-45 | Generic test procedure for IMS MT call release in 5GS described in TS 38.508-1 [4] subclause 4.9.18 takes place. | - | - | - | - |
| 46 | The SS releases the RRC connection. (Note 3) | <-- | NR RRC: RRCRelease | - | - |
| 47 | Check: Does the UE transmit a REGISTRATION REQUEST message at the expiry of T3512? | --> | 5GMM: REGISTRATION REQUEST | 5 | P |
| 48 | The SS transmits a REGISTRATION ACCEPT message including T3512 value IE. | <-- | 5GMM: REGISTRATION ACCEPT | - | - |
| 49 | The UE transmits a REGISTRATION COMPLETE message | --> | 5GMM: REGISTRATION COMPLETE |  |  |
| 50 | The SS releases the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 51 | The SS configures: - NR Cell 1 as " Non-suitable "off" cell" - E-UTRA Cell 1 as "Serving cell". | - | - | - | - |
| 52 | Check: Does the UE perform on the E-UTRA Cell 1 the TAU procedure for Inter-system change from N1 mode to S1 mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1? | - | - | 6 | P |
| 53 | The SS configures: - NR Cell 1 as "Serving cell" - E-UTRA Cell 1 as "Non-suitable "off" cell". | - | - | - | - |
| 54 | Check: Does the UE perform on the NR Cell 1 the Test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1? | - | - | 7 | P |
| 55 | Check: Does the UE transmit a DEREGISTRATION REQUEST message at expiry of T3444? | --> | 5GMM: DEREGISTRATION REQUEST | 8 | P |
| Note 1: The request to originate a manual eCall may be performed by MMI or AT command. Note 2: Timer T3444 of 12hours starts. Note 3: Timer T3512 of 7hours starts. |  |  |  |  |  |

11.5.1.3.3	Specific message contents

Table 11.5.1.3.3-1: SIB1 for NR Cell 1 (All steps, Table 11.5.1.3.2-1)

Derivation path: TS 38.508-1 [4] table 4.6.1-28 Condition eCalloverIMSforNR

Table 11.5.1.3.3-2: REGISTRATION ACCEPT (step 15, 48, 54 Table 11.5.1.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| T3512 value |  |  |  |
| Unit | '001'B | value is incremented in multiples of 1 hour |  |
| Timer value | '0 0111'B | 7 hours |  |

Table 11.5.1.3.3-3: UL NAS TRANSPORT (step 21, Table 11.5.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.7.1-10, condition INITIAL_PDU_REQUEST. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Request type |  | ‘011’B | initial emergency request |  |
| S-NSSAI |  | Not Present |  |  |
| DNN |  | Not Present |  |  |

Table 11.5.1.3.3-4: REGISTRATION REQUEST (step 47, Table 11.5.1.3.2-1)

| Derivation path: TS 38.508-1 [4] table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type |  |  |  |  |
| 5GS registration type value |  | ‘011’B |  | PERIODIC |

Table 11.5.1.3.3-5: TRACKING AREA UPDATE REQUEST (Step 52, Table 11.5.1.3.2-1; step 3, TS 38.508-1 [4] Table 4.9.7.2.2-1)

Derivation Path: TS 38.508-1 [4] table 4.9.7.2.3-1 with condition First-N1-to-S1 = TRUE

Table 11.5.1.3.3-6: DEREGISTRATION REQUEST (step 55, Table 11.5.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| De-registration type |  |  |  |
| Switch off | ‘0’B | Normal de-registration |  |

<End of modified section 1>