3GPP TSG-RAN5 Meeting #98	R5-231208

Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5199 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EIEI test case 11.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In this test case, eCall establishment is happening (exception before step 29 referring steps 2-3 of eCall establishment prcoedure defined in TS 34.229-1 Annext C.47) followed by dediated bearers establishment (steps 29-31) as both these sequences are specified as parallel events. Ideally, dedicated bearers shall be established followed by confirmation for call setup (200 OK for INVITE message). |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Added steps 31A-31B for eCall IMS signaling sequence after dedicated bearer establishment in Table 11.3.2.3.2-1. 2. Fixed formatting issues as found. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test procedure remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

### 11.3.2	eCall Only mode / T3445 / eCall inactivity procedure / Removal of eCall only restriction after a call to URI for test service

#### 11.3.2.1	Test Purpose (TP)

(1)

with { UE is in the state EMM-DEREGISTERED.eCALL-INACTIVE}

ensure that {

when { UE is requested to make an eCall to URI for test service}

then { UE sends ATTACH REQUEST message with EPS attach type IE 'combined EPS/IMSI attach'}

}

(2)

with { UE receives ATTACH ACCEPT message with EPS attach result 'combined EPS/IMSI attach' and

IMS voice over PS sessions is supported }

ensure that {

when { UE is in the EMM-REGISTERED.NORMAL-SERVICE state and an initial IMS registration is performed }

then { UE transmits an initial INVITE request for ecall to uri for test services }

}

(3)

with { UE receives an RRCConnectionRelease message and enters in RRC_IDLE state }

ensure that {

when { SS sends paging request for IMS MT speech call }

then {the UE answers the paging request for IMS MT speech call }

}

(4)

with { UE receives an RRCConnectionRelease message and enters in RRC_IDLE state }

ensure that {

when { UE starts T3445 for 12Hours and the periodic tracking area updating timer T3412 expires }

then { UE sends TRACKING AREA UPDATE REQUEST message with EPS update type = ’Periodic updating' }

}

(5)

with { UE is in RRC_IDLE state }

ensure that {

when { eCall Inactivity timer T3445 expires }

then { The UE performs Detach procedure }

}

#### 11.3.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 5.3.1.2.1, 5.5.4 and 10.2

[TS 24.301 clause 5.3.1.2.1]

The signalling procedure for the release of the NAS signalling connection is initiated by the network.

In S1 mode, when the RRC connection has been released, the UE shall enter EMM-IDLE mode and consider the NAS signalling connection released.

If the UE is configured for eCall only mode as specified in 3GPP TS 31.102 [17] then:

-	if the NAS signalling connection that was released had been established for eCall over IMS, the UE shall start timer T3444; and

-	if the NAS signalling connection that was released had been established for a call to an HPLMN designated non-emergency MSISDN or URI for test or terminal reconfiguration service, the UE shall start timer T3445.

[TS 24.301 clause 5.5.4]

The eCall inactivity procedure is applicable only to a UE configured for eCall only mode as specified in 3GPP TS 31.102 [17]. The procedure shall be started when timer T3444 or timer T3445 expires or is found to have already expired in any EMM-REGISTERED substate except substates EMM-REGISTERED.PLMN-SEARCH or EMM-REGISTERED.NO-CELL-AVAILABLE. The UE shall then perform the following actions:

-	stop other running timers (e.g. T3411, T3412);

-	if the UE is currently registered to EPS services only, perform a detach procedure for EPS services only;

-	if the UE is currently registered for both EPS services and non-EPS services, perform a combined detach procedure for EPS services and non-EPS services;

-	delete any GUTI, TAI list, last visited registered TAI, list of equivalent PLMNs, and KSI; and

-	enter EMM-DEREGISTERED.eCALL-INACTIVE state.

[TS 24.301 clause 10.2]

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| T3444 | 12 hours | All except EMM-NULL | UE configured for eCall only mode enters EMM-IDLE mode after an eCall over IMS | Removal of eCall only restriction | Perform eCall inactivity procedure as described in subclause 5.5.4 |
| T3445 | 12 hours | All except EMM-NULL | UE configured for eCall only mode enters EMM-IDLE mode after a call to a non-emergency MSISDN or URI for test or terminal reconfiguration service | Removal of eCall only restriction | Perform eCall inactivity procedure as described in subclause 5.5.4 |

#### 11.3.2.3	Test description

11.3.2.3.1	Pre-test conditions

System Simulator:

-	Cell A is configured according to Table 6.3.2.2-1 in [18] and is the serving cell.

UE:

-	the eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 36.508 [18] Table 4.9.3.5-2.

Preamble:

-	the UE is in state Switched OFF (state 1) according to [18].

11.3.2.3.2	Test procedure sequence

Table 11.3.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Wait for 60s for the UE to enter EMM-DEREGISTERED.eCALL-INACTIVE state | - | - | - | - |
| 3 | An eCall to URI for test service is initiated by dialling the eCall Test Number in the EFFDNURI field of the USIM (tel:123456). (See Note 1) | - | - | - | - |
| 3A | Check: Does UE transmits a RRCConnectionRequest message with ' establishmentCause' set to ' mo-Signalling ' | ---> | -RRCConnectionRequest | -1 | -P |
| 3B | SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup | - | - |
| 4 | Check: Does the UE sends an ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN. EPS attach type = "combined EPS/IMSI attach"? | --> | ATTACH REQUEST | 1 | P |
| 5-13 | Steps 5-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 14 | The SS transmits an ATTACH ACCEPT message with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message and SS assigns value of 186 minutes for the timer T3412. | <-- | ATTACH ACCEPT | - | - |
| 15 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| - | EXCEPTION: In parallel to the event described in step 16 below, if initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - | - | - |
| - | EXCEPTION: IF PDN1_IMS THEN in parallel to the event described in step 16 below the behaviour in table 11.3.2.3.2-2 occurs. | - | - | - | - |
| 16 | This message includes the ATTACH COMPLETE message. The ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message is piggybacked in ATTACH COMPLETE. | --> | RRC: ULInformationTransfer NAS: ATTACH COMPLETE NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | - | - |
| - | EXCEPTION: Steps 17A1 to 17A5b1 describe behaviour that depends on test case requirements and UE behaviour; the "lower case letter" identifies the different branches. | - | - | - | - |
| 17A1 – 17A3 | IF MULTI_PDN = TRUE THEN Steps 1 – 3 of the generic procedure for UE establishing additional PDN connectivity as specified subclause 4.5A.16 takes place | - | - | - | - |
| - | EXCEPTION: In parallel to the event described in step 17A4 below, if initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - | - | - |
| - | EXCEPTION: IF ADD_IMS THEN in parallel to the event described in step 17A4 below the behaviour in table 11.3.2.3.2-2 occurs. | - | - | - | - |
| 17A4 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | - | - |
| - | EXCEPTION: Steps17A5a1 to 17A5b1 describe behaviour that depends on UE capabilities/configuration; the "lower case letter" identifies a step sequence that takes place if one of those is supported/configured. | - | - | - | - |
| 17A5a1 | IF pc_UE_supports_user_initiated_PDN_disconnect THEN the non-IMS PDN shall be released as specified in steps 9 – 12 of TS 36.508 clause 4.5A.17 | - | - | - | - |
| 17A5b1 | ELSE the non-IMS PDN shall be released as specified in steps 10 -12 of TS 36.508 clause 4.5A.18 | - | - | - | - |
| 18-28 | Void | - | - | - | - |
| 29-31 | The UE performs Steps 23-25 of the generic test procedure as described in TS 36.508 sub clause 4.5A.27.3 | - | - | - | - |
| 31A-31B | Steps 2 – 3 of generic procedure for eCall establishment as defined in TS 34.229-1 [35] annex C.47 take place. | - | - | - | - |
| 32-35 | Release eCall to URI for test service using the generic procedure described in TS 34.229-1 [35] sub clause C.33. | - | - | - | - |
| 36 | Void | - | - | - | - |
| 37 | The SS releases the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 38 | The SS waits 1 minute. | - | - | - | - |
| 39-62 | Steps 1 to 24 of the generic test procedure for IMS MT speech call specified in TS 36.508 sub clause 4.5A.7.3. | - | - | 3 | P |
| 63-66 | Generic test procedure for MT release of IMS call as described in annex C.33 of TS 34.229-1 [35] takes place. | - | - | - | - |
| 67 | The SS releases the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 68 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message at the expiry of T3412 periodically for every 186 minutes and remains registered until T3445 expires? (See Note 2) | --> | TRACKING AREA UPDATE REQUEST | 4 | P |
| 69 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 70 | The UE sends TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 71 | The SS releases the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 72 | Check: Does the UE transmit a DETACH REQUEST message at expiry of T3445? | --> | DETACH REQUEST | 5 | P |
| 73 | The SS responds with DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| 74 | The SS releases the RRC connection. | <-- | RRCConnectionRelease | - | - |
| Note 1: The request to originate an eCall to URI for test service may be performed by MMI or AT command. Note 2: Timer T3412 is reset and started with its initial value, when the UE changes from EMM-CONNECTED to EMM-IDLE mode so the timer T3412 should be started after step 67 and T3445 should be started after step 37. |  |  |  |  |  |

Table 11.3.2.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Steps 4-7 of the generic procedure for IMS registration as defined in TS 34.229-1 [35] annex C.2 take place. | - | - | - | - |
| - | EXCEPTION: In parallel to the event described in step 2 below the behaviour in table 11.3.2.3.2-3 occurs. | - | - | - | - |
| 2 | Steps 8-11 of the generic procedure for IMS registration as defined in TS 34.229-1 [35] annex C.2 take place. | - | - | - | - |

Table 11.3.2.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an INVITE request message as defined in step 1 of TS 34.229-1 [35] annex C.47. | --> | INVITE | 2 | P |

11.3.2.3.3	Specific message contents

Table 11.3.2.3.3-1: Message SystemInformationBlockType1 (Preamble)

Derivation path: 36.508 table 4.4.3.2-3 Condition eCalloverIMS

Table 11.3.2.3.3-2: Message ATTACH ACCEPT (step 14, Table 11.3.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS attach result | '010'B | ''Combined EPS/IMSI attach'' |  |
| T3412 value |  |  |  |
| Unit | '010' | ''value is incremented in multiples of decihours'' |  |
| Timer value | '11111' | ''186 minutes'' |  |
| GUTI | GUTI-1 |  |  |
| EPS network feature support | '0000 0011'B | IMS voice over PS session in S1 mode is supported emergency bearer services in S1 mode supported |  |

Table 11.3.2.3.3-3: Void

Table 11.3.2.3.3-3A: INVITE (Step 1 Table 11.3.2.3.2-3)

| Derivation path: 34.229-1 [35] table in annex A.2.1 Condition A25 |  |
| --- | --- |
| Header/param | Value/remark |
| Call-Info url purpose | Any URL EmergencyCallData.eCall.MSD |
| Message-body | The following SDP types and values. Session description: - - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) [Note 2] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Note 1: At least one "c=" field shall be present. Note 2: AMR codec (AMR/8000 and/or AMR-WB/16000) shall be present in the media attributes, optionally including channel number "/1". -boundary value (as provided in SIP hdr Content-Type) Content-Type: application/EmergencyCallData.eCall.MSD Content-ID: same URL as in Call-Info header Content-Disposition: by-reference;handling=optional MSD in ASN.1 PER encoding -boundary value (as provided in SIP hdr Content-Type) |

Table 11.3.2.3.3-3B: 200 OK for INVITE (Step 2 TS 34.229-1 [35] annex C.47, in parallel to steps 29-31 Table 11.3.2.3.2-1)

| Derivation path: TS 34.229-1 [35] table in annex A.3.1 Condition A12 |  |
| --- | --- |
| Header/param | Value/remark |
| Message-body | --boundary1 Content-Type: application/EmergencyCallData.eCall.Control+xml Content-ID: <psap@3gpp.org> Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <EmergencyCallData.control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack received="true" ref=" addr-spec of the Content-ID of MIME body part containing the MSD sent by the UE in INVITE"/> </EmergencyCallData.control> --boundary1 The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:37 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 1] - b=AS:37 - b=RS:0 - b=RR:0 Attributes for media: - a=rtpmap: (payload type) AMR/8000/1 or AMR-WB/16000/1 [Note 1] [Note2] - a=fmtp: (format) mode-change-capability=2; max-red=220 - a=ptime:20 - a=maxptime:240 Note 1: The value for fmt, payload type and format is copied from step 1. Note 2: If UE included AMR-WB/16000 in step 1, SS uses AMR-WB/16000/1. Otherwise SS uses AMR/8000/1. |

Table 11.3.2.3.3-4: Message TRACKING AREA UPDATE REQUEST (step 68, Table 11.3.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type value | '011'B | ''Periodic updating'' |  |
| Old GUTI | GUTI-1 |  |  |

Table 11.3.2.3.3-4A: Message TRACKING AREA UPDATE ACCEPT (step 69, Table 11.3.2.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| T3412 value |  |  |  |
| Unit | '010' | ''value is incremented in multiples of decihours'' |  |
| Timer value | '11111' | ''186 minutes'' |  |
| EPS network feature support | '0000 0011'B | IMS voice over PS session in S1 mode is supported emergency bearer services in S1 mode supported |  |

Table 11.3.2.3.3-5: Message DETACH REQUEST (Step 72, Table 11.3.2.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Detach type |  |  |  |
| Type of detach | 001 | combined EPS/IMSI detach |  |
| Switch off | 0 | normal detach |  |
| GUTI or IMSI | GUTI-1 |  |  |

<End of modified section>