3GPP TSG-RAN5 Meeting # 98	R5-231243

# Athens, Greece, 27th February 2023 – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2747 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test frequencies update for bands n8 and n25 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Uplink test frequencies are defined for frequency band n8 with channel bandwidth 35 MHz and frequency band n25 with channel bandwidth 45 MHz. However, these channel bandwidth are only applicable for DL according to 3GPP TS 38.101-1 Table 5.3.5-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete uplink test frequencies for frequency band n8 with channel bandwidth 35 MHz and frequency band n25 with channel bandwidth 45 MHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Uplink frequencies for band n8 and n25 could be misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.8, 4.3.1.1.1.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

###### 4.3.1.1.1.8	Reference test frequencies for NR operating band n8

Table 4.3.1.1.1.8-1: Test frequencies for NR operating band n8 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 927.5 | 185500 | 925.25 | 185050 | 0 | 15 | 2318 | 185530 | 4 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 921.89 | 184378 | 102 |  | 2354 | 188410 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 957.5 | 191500 | 864.53 | 172906 | 504 |  | 2393 | 191530 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 930 | 186000 | 925.32 | 185064 | 0 | 15 | 2319 | 185550 | 6 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 919.46 | 183892 | 102 |  | 2348 | 187930 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 955 | 191000 | 859.6 | 171920 | 504 |  | 2383 | 190610 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 932.5 | 186500 | 925.39 | 185078 | 0 | 15 | 2320 | 185570 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 917.03 | 183406 | 102 |  | 2342 | 187450 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 952.5 | 190500 | 854.67 | 170934 | 504 |  | 2370 | 189630 | 4 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.39 | 176078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 799.67 | 159934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 899.31 | 179862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 935 | 187000 | 925.46 | 185092 | 0 | 15 | 2318 | 185530 | 2 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 914.6 | 182920 | 102 |  | 2336 | 186970 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 950 | 190000 | 849.74 | 169948 | 504 |  | 2357 | 188650 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 890 | 178000 | 880.46 | 176092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 797.24 | 159448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 894.38 | 178876 | 6 |  | - | - | - | - | - | - |
| 35 | 188 | Downlink | Low | 942.5 | 188500 | 925.58 | 185116 | 0 | 15 | 2318 | 185530 | 6 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 942.5 | 188500 | 907.22 | 185116 | 0 | 15 | 2318 | 185530 | 6 | 1 | 0 (0) | 1 |
|  |  |  | High | 942.5 | 188500 | 834.86 | 185116 | 0 | 15 | 2318 | 185530 | 6 | 1 | 0 (0) | 1 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.8-2: Test frequencies for NR operating band n8 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 930 | 186000 | 925.68 | 185136 | 0 | 15 | 2325 | 186030 | 10 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 901.46 | 180292 | 102 |  | 2354 | 188410 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 955 | 191000 | 769.24 | 153848 | 504 |  | 2389 | 191090 | 6 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 932.5 | 186500 | 925.66 | 185132 | 0 | 15 | 2326 | 186050 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 898.94 | 179788 | 102 |  | 2348 | 187930 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 952.5 | 190500 | 764.22 | 152844 | 504 |  | 2373 | 189870 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.66 | 176132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 709.22 | 141844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 898.5 | 179700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 935 | 187000 | 925.82 | 185164 | 0 | 15 | 2324 | 186010 | 18 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 942.5 | 188500 | 896.6 | 179320 | 102 |  | 2342 | 187450 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 950 | 190000 | 759.38 | 151876 | 504 |  | 2360 | 188890 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 890 | 178000 | 880.82 | 176164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 706.88 | 141376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 893.66 | 178732 | 6 |  | - | - | - | - | - | - |
| 35 | 92 | Downlink | Low | 942.5 | 188500 | 925.94 | 185188 | 0 | 15 | 2324 | 186010 | 10 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 942.5 | 188500 | 889.22 | 185188 | 0 | 15 | 2324 | 186010 | 10 | 0 | 1 (6) | 12 |
|  |  |  | High | 942.5 | 188500 | 744.5 | 185188 | 0 | 15 | 2324 | 186010 | 10 | 0 | 1 (6) | 12 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES 1>>>

## <<< START OF CHANGES 2>>>

###### 4.3.1.1.1.25	Reference test frequencies for NR operating band n25

Table 4.3.1.1.1.25-1: Test frequencies for NR operating band n25 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1932.5 | 386500 | 1930.25 | 386050 | 0 | 15 | 4829 | 386410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1962.5 | 392500 | 1941.89 | 388378 | 102 |  | 4904 | 392410 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 1992.5 | 398500 | 1899.53 | 379906 | 504 |  | 4979 | 398410 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1852.5 | 370500 | 1850.25 | 370050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1789.53 | 357906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1912.5 | 382500 | 1909.17 | 381834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 1935 | 387000 | 1930.32 | 386064 | 0 | 15 | 4830 | 386430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1962.5 | 392500 | 1939.46 | 387892 | 102 |  | 4898 | 391930 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 1990 | 398000 | 1894.6 | 378920 | 504 |  | 4969 | 397490 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1855 | 371000 | 1850.32 | 370064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1787.1 | 357420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1910 | 382000 | 1904.24 | 380848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 1937.5 | 387500 | 1930.39 | 386078 | 0 | 15 | 4831 | 386450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1962.5 | 392500 | 1937.03 | 387406 | 102 |  | 4892 | 391450 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 1987.5 | 397500 | 1889.67 | 377934 | 504 |  | 4956 | 396510 | 0 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1850.39 | 370078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1784.67 | 356934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1907.5 | 381500 | 1899.31 | 379862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 1940 | 388000 | 1930.46 | 386092 | 0 | 15 | 4832 | 386650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1934.6 | 386920 | 102 |  | 4886 | 390970 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 1985 | 397000 | 1884.74 | 376948 | 504 |  | 4943 | 395530 | 2 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1860 | 372000 | 1850.46 | 370092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1782.24 | 356448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1905 | 381000 | 1894.38 | 378876 | 6 |  | - | - | - | - | - | - |
| 25 | 133 | Downlink | Low | 1942.5 | 388500 | 1930.53 | 386106 | 0 | 15 | 4833 | 386670 | 8 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1932.17 | 386434 | 102 |  | 4880 | 390490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 1982.5 | 396500 | 1879.81 | 375962 | 504 |  | 4933 | 394610 | 0 | 0 | 2 (4) | 508 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1850.53 | 370106 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1779.81 | 355962 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1902.5 | 380500 | 1889.45 | 377890 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low | 1945 | 389000 | 1930.6 | 386120 | 0 | 15 | 4834 | 386690 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1929.74 | 385948 | 102 |  | 4874 | 390010 | 10 | 0 | 0 (0) | 102 |
|  |  |  | High | 1980 | 396000 | 1874.88 | 374976 | 504 |  | 4920 | 393630 | 2 | 0 | 2 (4) | 508 |
|  |  | Uplink | Low | 1865 | 373000 | 1850.6 | 370120 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1777.38 | 355476 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1900 | 380000 | 1884.52 | 376904 | 6 |  | - | - | - | - | - | - |
| 40 | 216 | Downlink | Low | 1950 | 390000 | 1930.56 | 386112 | 0 | 15 | 4833 | 386670 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1924.7 | 384940 | 102 |  | 4862 | 389050 | 2 | 0 | 1 (2) | 104 |
|  |  |  | High | 1975 | 395000 | 1864.84 | 372968 | 504 |  | 4894 | 391490 | 6 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1870 | 374000 | 1850.56 | 370112 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1772.34 | 354468 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1895 | 379000 | 1874.48 | 374896 | 6 |  | - | - | - | - | - | - |
| 45 | 242 | Downlink | Low | 1952.5 | 390500 | 1930.72 | 386144 | 0 | 15 | 4834 | 386690 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1922.36 | 384472 | 102 | 15 | 4856 | 388570 | 10 | 1 | 0 (0) | 103 |
|  |  |  | High | 1972.5 | 394500 | 1860 | 372000 | 504 | 15 | 4881 | 390510 | 2 | 0 | 0 (0) | 504 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.25-2: Test frequencies for NR operating band n25 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1935 | 387000 | 1930.68 | 386136 | 0 | 15 | 4836 | 386910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1921.46 | 384292 | 102 |  | 4904 | 392410 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 1990 | 398000 | 1804.24 | 360848 | 504 |  | 4975 | 397970 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1855 | 371000 | 1850.68 | 370136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1696.74 | 339348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1910 | 382000 | 1903.52 | 380704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 1937.5 | 387500 | 1930.66 | 386132 | 0 | 15 | 4837 | 386930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1962.5 | 392500 | 1918.94 | 383788 | 102 |  | 4898 | 391930 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 1987.5 | 397500 | 1799.22 | 359844 | 504 |  | 4962 | 396990 | 22 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1850.66 | 370132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1694.22 | 338844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1907.5 | 381500 | 1898.5 | 379700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 1940 | 388000 | 1930.82 | 386164 | 0 | 15 | 4835 | 386890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1916.6 | 383320 | 102 |  | 4892 | 391450 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 1985 | 397000 | 1794.38 | 358876 | 504 |  | 4949 | 396010 | 18 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1860 | 372000 | 1850.82 | 370164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1691.88 | 338376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1905 | 381000 | 1893.66 | 378732 | 6 |  | - | - | - | - | - | - |
| 25 | 65 | Downlink | Low | 1942.5 | 388500 | 1930.8 | 386160 | 0 | 15 | 4836 | 386910 | 10 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1914.08 | 382816 | 102 |  | 4886 | 390970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 1982.5 | 396500 | 1789.36 | 357872 | 504 |  | 4939 | 395090 | 22 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1850.8 | 370160 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1689.36 | 337872 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1902.5 | 380500 | 1888.64 | 377728 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low | 1945 | 389000 | 1930.96 | 386192 | 0 | 15 | 4837 | 386930 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1911.74 | 382348 | 102 |  | 4880 | 390490 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 1980 | 396000 | 1784.52 | 356904 | 504 |  | 4926 | 394110 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1865 | 373000 | 1850.96 | 370192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1687.02 | 337404 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1900 | 380000 | 1883.8 | 376760 | 6 |  | - | - | - | - | - | - |
| 40 | 106 | Downlink | Low | 1950 | 390000 | 1930.92 | 386184 | 0 | 15 | 4836 | 386910 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1906.7 | 381340 | 102 |  | 4868 | 389530 | 18 | 0 | 1 (6) | 216 |
|  |  |  | High | 1975 | 395000 | 1774.48 | 354896 | 504 |  | 4900 | 391970 | 22 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1870 | 374000 | 1850.92 | 370184 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1681.98 | 336396 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1895 | 379000 | 1873.76 | 374752 | 6 |  | - | - | - | - | - | - |
| 45 | 119 | Downlink | Low | 1972.5 | 394500 | 1769.64 | 353928 | 504 | 15 | 4887 | 390990 | 18 | 0 | 0 (5) | 1018 |
|  |  |  | Mid | 1872.5 | 374500 | 1851.08 | 370216 | 0 | - | - | - | - | - | - | - |
|  |  |  | High | 1882.5 | 376500 | 1679.64 | 335928 | 504 | - | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.25-3: Test frequencies for NR operating band n25 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1935 | 387000 | 1931.04 | 386208 | 0 | 15 | - | 386568 |
|  |  |  | Mid | 1962.5 | 392500 | 1885.1 | 377020 | 102 |  | - | 392068 |
|  |  |  | High | 1990 | 398000 | 1623.16 | 324632 | 504 |  | - | 397568 |
|  |  | Uplink | Low | 1855 | 371000 | 1851.04 | 370208 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1515.66 | 303132 | 504 |  | - | - |
|  |  |  | High | 1910 | 382000 | 1901.72 | 380344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 1937.5 | 387500 | 1931.02 | 386204 | 0 | 15 | - | 386564 |
|  |  |  | Mid | 1962.5 | 392500 | 1882.58 | 376516 | 102 |  | - | 391564 |
|  |  |  | High | 1987.5 | 397500 | 1618.14 | 323628 | 504 |  | - | 396564 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1851.02 | 370204 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1513.14 | 302628 | 504 |  | - | - |
|  |  |  | High | 1907.5 | 381500 | 1896.7 | 379340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 1940 | 388000 | 1931.36 | 386272 | 0 | 15 | - | 386632 |
|  |  |  | Mid | 1962.5 | 392500 | 1880.42 | 376084 | 102 |  | - | 391132 |
|  |  |  | High | 1985 | 397000 | 1613.48 | 322696 | 504 |  | - | 395632 |
|  |  | Uplink | Low | 1860 | 372000 | 1851.36 | 370272 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1510.98 | 302196 | 504 |  | - | - |
|  |  |  | High | 1905 | 381000 | 1892.04 | 378408 | 6 |  | - | - |
| 25 | 31 | Downlink | Low | 1942.5 | 388500 | 1931.34 | 386268 | 0 | 15 | - | 386628 |
|  |  |  | Mid | 1962.5 | 392500 | 1877.9 | 375580 | 102 |  | - | 390628 |
|  |  |  | High | 1982.5 | 396500 | 1608.46 | 321692 | 504 |  | - | 394628 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1851.34 | 370268 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1508.46 | 301692 | 504 |  | - | - |
|  |  |  | High | 1902.5 | 380500 | 1887.02 | 377404 | 6 |  | - | - |
| 30 | 38 | Downlink | Low | 1945 | 389000 | 1931.32 | 386264 | 0 | 15 | - | 386624 |
|  |  |  | Mid | 1962.5 | 392500 | 1875.38 | 375076 | 102 |  | - | 390124 |
|  |  |  | High | 1980 | 396000 | 1603.44 | 320688 | 504 |  | - | 393624 |
|  |  | Uplink | Low | 1865 | 373000 | 1851.32 | 370264 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1505.94 | 301188 | 504 |  | - | - |
|  |  |  | High | 1900 | 380000 | 1882 | 376400 | 6 |  | - | - |
| 40 | 51 | Downlink | Low | 1950 | 390000 | 1931.64 | 386328 | 0 | 15 | - | 386688 |
|  |  |  | Mid | 1962.5 | 392500 | 1870.7 | 374140 | 102 |  | - | 389188 |
|  |  |  | High | 1975 | 395000 | 1593.76 | 318752 | 504 |  | - | 391688 |
|  |  | Uplink | Low | 1870 | 374000 | 1851.64 | 370328 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1501.26 | 300252 | 504 |  | - | - |
|  |  |  | High | 1895 | 379000 | 1872.32 | 374464 | 6 |  | - | - |
| 45 | 58 | Downlink | Low | 1952.5 | 390500 | 1931.62 | 386324 | 0 | 15 | - | 386684 |
|  |  |  | Mid | 1962.5 | 392500 | 1868.18 | 373636 | 102 | 15 | - | 388684 |
|  |  |  | High | 1972.5 | 394500 | 1588.74 | 317748 | 504 | 15 | - | 390684 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES 2>>>