3GPP TSG-RAN5 Meeting #98	R5-231276
Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2332 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction in Measurement uncertainty table Annex F |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Table F.1.1.2-1 includes two entries for AWGN and signal flatness uncertainty. However, AWGN and signal flatness entry for 3.6dB corresponds with FR2 uncertainty in Table F.1.1.2-2 as agreed CR R5-226374 at RAN5#97 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed AWGN and signal flatness entry for 3.6dB in Table F.1.1.2-1 Added AWGN and signal flatness entry for 3.6dB in Table F.1.1.2-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AWGN and signal flatness entry for FR2 will remain missing in in Table F.1.1.2-2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex F.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### F.1.1.2	Measurement of RRM requirements

This clause defines the maximum test system uncertainty for the RRM requirements. The maximum uncertainty values allowed for the typical RRM measurement uncertainty contributors is defined in Table F.1.1.2-1 and Table F.1.1.2-2. Unless explicitly stated for a particular test case, these maximum uncertainty values should be used as starting point to perform the test tolerance analysis in TR 38.903 [22] for each of the test cases. Specific test cases might require a tighter measurement uncertainty value for some of the contributors. Exceptions to the general values in Table F.1.1.2-1 and Table F.1.1.2-2 shall be handled case by case.

Table F.1.1.2-1: Maximum allowed measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Cell BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc averaged over BWConfig | dB | ±1.5 |  |
| AWGN absolute power, Noc for Measurement PRB | dB | ±1.5 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB105 for CSI-RS-based |
| Ratio of cell X signal / AWGN, Êsx / Noc averaged over BWConfig | dB | ±0.3 | Same as in LTE |
| Ratio of cell X signal / AWGN, Êsx / Noc for Measurement PRB | dB | ±0.3 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB105 for CSI-RS-based |
| Fading profile uncertainty | dB | ±0.5 for 1 Tx ±0.7 for 2 Tx | Depends on fading profile, can be referenced from TS 38.101-4 [20] |
| AWGN and signal flatness* | dB | ±2.0 | Same as in LTE, can be referenced from TS 38.101-4 [20] |
| Uplink absolute power measurement | dB | ±1.5 | Same as in TS 38.521-1 [17] |
| Uplink relative power measurement | dB | ±0.7 | Same as in TS 38.521-1 [17] |
| Uplink signal transmit timing relative to downlink | Tc | ±112 |  |
| Relative transmit timing accuracy during UE timing adjustment | Tc | ±88 |  |
| Timing Advance Adjustment accuracy | Tc | ±88 |  |
| Note 1: The values in this table are specified per cell. Multi-cell test cases need to combined these values in the TT analysis in TR 38.903 Note 2:These values apply for cell BW ≤ 40 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. Note 3: Void |  |  |  |

Table F.1.1.2-2: Maximum allowed measurement uncertainty values for the test system for FR2 (up to 40GHz) and Cell BW ≤ 400 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc averaged over BWConfig | dB | PC3: ±5.654,5 PC1: FFS |  |
| AWGN absolute power, Noc for Measurement PRB | dB | PC3: ±5.654,5 PC1: FFS | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based |
| Ratio of cell X signal / AWGN, Êsx / Noc averaged over BWConfig | dB | ±0.3 |  |
| Ratio of cell X signal / AWGN, Êsx / Noc for Measurement PRB | dB | ±0.3 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based |
| ÊsxNote 3 absolute power, Noc averaged over BWConfig | dB | PC3: ±5.654,5 PC1: FFS |  |
| ÊsxNote 3 absolute power, Noc for Measurement PRB | dB | PC3: ±5.654,5 PC1: FFS | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based |
| Relative power between any two Ês levels at the same frequency | dB | ±2.0 |  |
| Fading profile uncertainty | dB | ±0.5 for 1 Tx ±0.7 for 2 Tx |  |
| AWGN and signal flatness* | dB | ±3.6 | Same as in Demod Performance test cases, can be referenced from TS 38.521-4 [20] |
| Uplink absolute power measurement | dB | TBD |  |
| Uplink relative power measurement | dB | TBD |  |
| Uplink signal transmit timing relative to downlink | Tc | ±48 |  |
| Relative transmit timing during UE timing adjustment | Tc | ±40 |  |
| Timing Advance Adjustment | Tc | ±40 |  |
| NOTE 1: The values in this table are specified per cell. Multi-cell test cases need to combine these values in the TT analysis in TR 38.903 NOTE 2: These values apply for cell BW ≤ 400 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. NOTE 3: Applies for test cases that use Ês without AWGN NOTE 4: Total Expanded MU for IFF for Max device size ≤ 30cm in TR 38.903 [22] Table E.3.1.1-2 for PC3 UEs and in Table FFS for PC1 UEs. NOTE 5: If the TT analysis for a specific test case based on this MU value results in an unsolvable conflict, making the test case untestable, even after the alternative solutions listed in TR 38.903 [22] clause A.4 have been considered for the test case in TS 38.133 [6] Annex A, the TT analysis shall be repeated using a lower MU value, taking into account the values defined in clause E.3.1 TR 38.903 [22]. The test case will be applicable for the subset of the test systems meeting this reduced MU Threshold. |  |  |  |

The maximum test system uncertainty for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.1.2-3.

The maximum test system uncertainty for the NR/5GC FR1 test cases in chapter 6 is defined in Table F.1.1.2-4.

The maximum test system uncertainty for the EN-DC FR2 test cases in chapter 5 is defined in Table F.1.1.2-5.

The maximum test system uncertainty for the NR/5GC FR2 test cases in chapter 7 is defined in Table F.1.1.2-6.

The maximum test system uncertainty for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases in chapter 8 is defined in Table F.1.1.2-7.

The maximum test system uncertainty for the FR1 NR sidelink test cases in chapter 9 is defined in Table F.1.1.2-8.

The maximum test system uncertainty for the NR/5GC FR1 test cases for RedCap in chapter 16 is defined in Table F.1.1.2-9.

The maximum test system uncertainty for the NR/5GC FR2 test cases for RedCap in chapter 17 is defined in Table F.1.1.2-10.

The maximum test system uncertainty for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases for RedCap in chapter 18 is defined in Table F.1.1.2-11.

## <<< Skip unchanged sections >>>

## <<< END OF CHANGES >>>