3GPP TSG-RAN5 Meeting #98	R5-231373

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0912 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to FR2 RF test case 6.2.5 for EIRP with UL-Gaps |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 core specifications have defined core requirements for UL-Gaps based RF performance. The conformance test cases have to be updated and completed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the FR2 RF test case for EIRP with UL-Gaps enabled |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete test case in RAN5 test specs for defined core requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

### 6.2.5	UE Maximum Output Power – EIRP with UL Gaps

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	Same as in 6.2.1.1 and in addition below items are FFS

-	Test Procedure and Test Config Table is FFS

-	Test Requirements are FFS

-	MU and TT are pending further analysis

6.2.5.1	Test purpose

The objective of this test is to determine the impact of UL-gaps on TX power management by measuring the EIRP with and without UL-Gaps configured

6.2.5.2	Test applicability

This test case applies to all types of NR UEs release 17 and forward supporting [IE UL-Gaps]

6.2.5.3	Minimum conformance requirements

The difference of the measured peak EIRP PUMAX,f,c_GAP_ON when UL gap for TX power management is configured and activated, and the measured peak EIRP PUMAX,f,c_GAP_OFF when UL gap is not configured or de-activated, shall meet the following requirement:

PUMAX,f,c_GAP_ON - PUMAX,f,c_GAP_OFF max((EIRPmeas_peak – 23) + 10 * log10(Z/20), 3)dB

where EIRPmeas_peak  is the measured UE peak EIRP with zero MPR/A-MPR/P-MPR as specified in clause 6.2.1 for the corresponding power class, and Z% is duty cycle of the reference measurement channel. PUMAX,f,c_GAP_ON shall be measured outside of the UL gap symbol(s). The period of measurement shall be at least 4s. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle) and in the test Z is set to 20 when maxUplinkDutyCycle-FR2 is less than 20 or not reported, and should be larger than maxUplinkDutyCycle-FR2 when maxUplinkDutyCycle-FR2 is equal to or greater than 20. The reference measurement channel is specified in Annex A.2.3.

When UL gap for Tx power management is configured and activated, the reported P-MPRf,c shall be less than 3dB. When UL gap for Tx power management is not configured and activated, UE shall set the P bit in PHR to 1 in the test when PHR is configured.

6.2.5.4	Test description

6.2.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 38.521-1 [2] Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 [2] Annex C.2.

Table 6.2.5.4.1-1: Test Configuration Table for power class 3

FFS

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [TBD] Annex A, Figure A.3.1.2.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [TBD] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 38.521-1 [2] Annex C.0, C.1, C.2, and uplink signals according to TS 38.521-1 [2] Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2.2.4.1-1.

5.	Propagation conditions are set according to TS 38.521-1 [2] Annex B.0.

6.	Ensure the UE is in State [TBD] with generic procedure parameters [TBD], Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [TBD] clause 4.5. Message contents are defined in clause 6.2.2.4.3.

6.2.5.4.2	Test procedure

FFS

6.2.5.4.3	Message contents

FFS

6.2.5.5	Test requirement

FFS

<End of Changes>