3GPP TSG-RAN5 Meeting #98	R5-231428

# Athens, Greece, Feburary 27th - March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3480 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR SL SIG TC 12.2.2.1 - SyncRef Reselect Con-current |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To fix following issues according to comments received: - Typos in test purpose, test procedure; - Changing “SyncRef UE” to “NR-SS-UE” - Change all “S-SSB” and “SL SSB” to “SL-SSB”. - Correct wrong message in test procedure. - Configuring cast type in SDAP entity to broadcast. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Issues mentioned above are fixed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case is incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-230539r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

12.2.2.1	Inter-carrier concurrent operation / Sidelink synchronization related procedure / Synchonization reference source (re-)selection

12.2.2.1.1	Test Purpose (TP)

(1)

with { UE in connected state. UE configured by upper layer to perform sidelink transmission and configured with sl-SyncPriority = gnss }

ensure that {

when { GNSS signal is reliable and a SyncRef UE directly synchronized to GNSS is detected }

then { UE selects GNSS as synchonization reference source }

}

(2)

with { UE in connected state. UE configured by upper layer to perform sidelink transmission and configured with sl-SyncPriority = gnss }

ensure that {

when { two SyncRef UEs, one directly synchronized to GNSS and the other indirectly synchronized to GNSS, are detected }

then { UE selects the SyncRef UE directly synchronized to GNSS as synchonization reference source }

}

(3)

with { UE in connected state. UE configured by upper layer to perform sidelink transmission and configured with sl-SyncPriority = gnss }

ensure that {

when { one SyncRef UE indirectly synchronized to GNSS is detected }

then { UE selects the SyncRef UE indirectly synchronized to GNSS as synchonization reference source }

}

(4)

with { UE in connected state. UE configured by upper layer to perform sidelink transmission and configured with sl-SyncPriority = gnbEnb }

ensure that {

when { a SyncRef UE directly synchronized to gNB is detected }

then { UE selects serving cell as synchonization reference source }

}

(5)

with { UE configured by upper layer to perform sidelink transmission and configured with sl-SyncPriority = gnbEnb in pre-configuration. }

ensure that {

when { two SyncRef UEs, one directly synchronized to gNB and the other indirectly synchronized to gNB, are detected }

then { UE selects the SyncRef UE directly synchrinized to gNB as synchonization reference source }

}

(6)

with { UE configured by upper layer to perform sidelink transmission and configured with sl-SyncPriority = gnbEnb in pre-configuration }

ensure that {

when { GNSS is reliable and one SyncRef UE indirectly synchronized to gNB is detected }

then { UE selects the SyncRef UE indirectly synchrinized to gNB as synchonization reference source }

}

(7)

with { UE configured by upper layer to perform sidelink transmission and configured with sl-SyncPriority = gnbEnb in pre-configuration }

ensure that {

when { GNSS is reliable and one SyncRef UE directly synchronized to GNSS is detected }

then { UE selects GNSS as synchonization reference source }

}

(8)

with { UE configured by upper layer to perform sidelink transmission and configured with sl-SyncPriority = gnbEnb in pre-configuration }

ensure that {

when { two SyncRef UEs, one directly synchronized to GNSS and the other indirectly synchronized to GNSS, are detected }

then { UE selects the SyncRef UE directly synchronized to GNSS as synchonization reference source }

}

(9)

with { UE configured by upper layer to perform sidelink transmission and configured with sl-SyncPriority = gnbEnb in pre-configuration. }

ensure that {

when { two SyncRef UEs, one indirectly synchronized to GNSS and the other one neither directly nor indirectly synchronized to GNSS or gNB, are detected }

then { UE selects the SyncRef UE indirectly synchronized to GNSS as synchonization reference source }

}

(10)

with { UE iconfigured by upper layer to perform sidelink transmission and configured with sl-SyncPriority = gnbEnb in pre-configuration }

ensure that {

when { a SyncRef UE which neither directly nor indirectly synchronized to GNSS or gNB is detected }

then { UE selects the SyncRef UE which neither directly nor indirectly synchronized to GNSS or gNB as synchonization reference source }

}

12.2.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 [22], subclause 5.8.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.8.2]

The UE shall perform NR sidelink communication operation only if the conditions defined in this clause are met:

1>	if the UE's serving cell is suitable (RRC_IDLE or RRC_INACTIVE or RRC_CONNECTED); and if either the selected cell on the frequency used for NR sidelink communication operation belongs to the registered or equivalent PLMN as specified in TS 24.587 [57] or the UE is out of coverage on the frequency used for NR sidelink communication operation as defined in TS 38.304 [20] and TS 36.304 [27]; or

…

1>	if the UE has no serving cell (RRC_IDLE);

[TS 38.331, clause 5.8.5.1]

Figure 5.8.5.1-1: Synchronisation information transmission for NR sidelink communication, in (partial) coverage

Figure 5.8.5.1-2: Synchronisation information transmission for NR sidelink communication, out of coverage

The purpose of this procedure is to provide synchronisation information to a UE.

[TS 38.331, clause 5.8.5.2]

A UE capable of NR sidelink communication and SLSS/PSBCH transmission shall, when transmitting NR sidelink communication, and if the conditions for NR sidelink communication operation are met and when the following conditions are met:

…

1>	if out of coverage on the frequency used for NR sidelink communication, and the frequency used to transmit NR sidelink communication is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-FreqInfoList within SIB12; and has selected GNSS or the cell as synchronization reference as defined in 5.8.6.3:

2>	if in RRC_CONNECTED; and if networkControlledSyncTx is configured and set to on; or

…

3>	transmit sidelink SSB on the frequency used for NR sidelink communication in accordance with 5.8.5.3 and TS 38.211 [16], including the transmission of SLSS as specified in 5.8.5.3 and transmission of MasterInformationBlockSidelink as specified in 5.8.9.4.3;

1>	else:

2>	for the frequency used for NR sidelink communication, if syncTxThreshOoC is included in SidelinkPreconfigNR; and the UE is not directly synchronized to GNSS, and the UE has no selected SyncRef UE or the PSBCH-RSRP measurement result of the selected SyncRef UE is below the value of syncTxThreshOoC; or

2>	for the frequency used for NR sidelink communication, if the UE selects GNSS as the synchronization reference source:

3>	transmit sidelink SSB on the frequency used for NR sidelink communication in accordance with TS 38.211 [16], including the transmission of SLSS as specified in 5.8.5.3 and transmission of MasterInformationBlockSidelink as specified in 5.8.9.4.3;

[TS 38.331, clause 5.8.5.3]

The UE shall select the SLSSID and the slot in which to transmit SLSS as follows:

…

1>	if triggered by NR sidelink communication, and out of coverage on the frequency used for NR sidelink communication, and the concerned frequency is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-FreqInfoList within SIB12:

2>	if the UE has selected GNSS as synchronization reference in accordance with 5.8.6.2:

3>	select SLSSID 0;

3>	use sl-SSB-TimeAllocation1 included in the entry of configured sl-SyncConfigList corresponding to the concerned frequency, that includes txParameters and gnss-Sync;

3>	select the slot(s) indicated by sl-SSB-TimeAllocation1;

2>	if the UE has selected a cell as synchronization reference in accordance with 5.8.6.2:

3>	select the SLSSID included in the entry of configured sl-SyncConfigList corresponding to the concerned frequency, that includes txParameters and does not include gnss-Sync;

3>	select the slot(s) indicated by sl-SSB-TimeAllocation1;

1>	else if triggered by NR sidelink communication and the UE has GNSS as the synchronization reference:

2>	select SLSSID 0;

…

2>	else:

3>	select the slot(s) indicated by sl-SSB-TimeAllocation1;

1>	else:

2>	select the synchronisation reference UE (i.e. SyncRef UE) as defined in 5.8.6;

2>	if the UE has a selected SyncRef UE and inCoverage in the MasterInformationBlockSidelink message received from this UE is set to true; or

2>	if the UE has a selected SyncRef UE and inCoverage in the MasterInformationBlockSidelink message received from this UE is set to false while the SLSS from this UE is part of the set defined for out of coverage, see TS 38.211 [16]:

3>	select the same SLSSID as the SLSSID of the selected SyncRef UE;

3>	select the slot in which to transmit the SLSS according to the sl-SSB-TimeAllocation1 or sl-SSB-TimeAllocation2 included in the preconfigured sidelink parameters corresponding to the concerned frequency, such that the timing is different from the SLSS of the selected SyncRef UE;

…

2>	else if the UE has a selected SyncRef UE:

3>	select the SLSSID from the set defined for out of coverage having an index that is 336 more than the index of the SLSSID of the selected SyncRef UE, see TS 38.211 [16];

3>	select the slot in which to transmit the SLSS according to sl-SSB-TimeAllocation1 or sl-SSB-TimeAllocation2 included in the preconfigured sidelink parameters corresponding to the concerned frequency, such that the timing is different from the SLSS of the selected SyncRef UE;

…

[TS 38.331, clause 5.8.6.2]

The UE shall:

1>	if the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12, and sl-SyncPriority is configured for the concerned frequency and set to gnbEnb:

2>	select a cell as the synchronization reference source as defined in 5.8.6.3:

1>	else if the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12, and sl-SyncPriority for the concerned frequency is not configured or is set to gnss, and GNSS is reliable in accordance with TS 38.101-1 [15] and TS 38.133 [14]:

2>	select GNSS as the synchronization reference source;

…

1>	else:

2>	perform a full search (i.e. covering all subframes and all possible SLSSIDs) to detect candidate SLSS, in accordance with TS 38.133 [14]

2>	when evaluating the one or more detected SLSSIDs, apply layer 3 filtering as specified in 5.5.3.2 using the preconfigured sl-filterCoefficient, before using the PSBCH-RSRP measurement results;

2>	if the UE has selected a SyncRef UE:

…

3>	if the PSBCH-RSRP of the current SyncRef UE is less than the minimum requirement defined in TS 38.133 [14]:

4>	consider no SyncRef UE to be selected;

…

2>	if the UE has selected cell as the synchronization reference for NR sidelink communication:

3>	if the PSBCH-RSRP of the candidate SyncRef UE exceeds the minimum requirement defined in TS 38.133 [14] by sl-SyncRefMinHyst and the candidate SyncRef UE belongs to a higher priority group than gNB/eNB; or

3>	if the selected cell is not detected:

4>	consider the cell not to be selected;

2>	if the UE has not selected any synchronization reference:

3>	if the UE detects one or more SLSSIDs for which the PSBCH-RSRP exceeds the minimum requirement defined in TS 38.133 [14] by sl-SyncRefMinHyst and for which the UE received the corresponding MasterInformationBlockSidelink message (candidate SyncRef UEs), or if the UE detects GNSS that is reliable in accordance with TS 38.101-1 [15] and TS 38.133 [14], or if the UE detects a cell, select a synchronization reference according to the following priority group order:

4>	if sl-SyncPriority corresponding to the concerned frequency is set to gnbEnb:

5>	UEs of which SLSSID is part of the set defined for in coverage, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to true, starting with the UE with the highest PSBCH-RSRP result (priority group 1);

5>	UE of which SLSSID is part of the set defined for in coverage, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to false, starting with the UE with the highest PSBCH-RSRP result (priority group 2);

5>	GNSS that is reliable in accordance with TS 38.101-1 [15] and TS 38.133 [14] (priority group 3);

5>	UEs of which SLSSID is 0, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to true, or of which SLSSID is 0 and SLSS is transmitted on slot(s) indicated by sl-SSB-TimeAllocation3, starting with the UE with the highest PSBCH-RSRP result (priority group 4);

5>	UEs of which SLSSID is 0 and SLSS is not transmitted on slot(s) indicated by sl-SSB-TimeAllocation3, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to false, starting with the UE with the highest PSBCH-RSRP result (priority group 5);

5>	UEs of which SLSSID is 337 and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to false, starting with the UE with the highest PSBCH-RSRP result (priority group 5);

5>	Other UEs, starting with the UE with the highest PSBCH-RSRP result (priority group 6);

4>	if sl-SyncPriority corresponding to the concerned frequency is set to gnss, and sl-NbAsSync is set to true:

5>	UEs of which SLSSID is 0, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to true, or of which SLSSID is 0 and SLSS is transmitted on slot(s) indicated by sl-SSB-TimeAllocation3, starting with the UE with the highest PSBCH-RSRP result (priority group 1);

5>	UEs of which SLSSID is 0 and SLSS is not transmitted on slot(s) indicated by sl-SSB-TimeAllocation3, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to false, starting with the UE with the highest PSBCHS-RSRP result (priority group 2);

5>	UEs of which SLSSID is 337 and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to false, starting with the UE with the highest PSBCH-RSRP result (priority group 2);

5>	the cell detected by the UE as defined in 5.8.6.3 (priority group 3);

5>	UEs of which SLSSID is part of the set defined for in coverage, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to true, starting with the UE with the highest PSBCH-RSRP result (priority group 4);

5>	UE of which SLSSID is part of the set defined for in coverage, and inCoverage, included in the MasterInformationBlockSidelink message received from this UE, is set to false, starting with the UE with the highest PSBCH-RSRP result (priority group 5);

5>	Other UEs, starting with the UE with the highest S-RSRP result (priority group 6);

…

[TS 38.331, clause 5.8.9.4.3]

The UE shall set the contents of the MasterInformationBlockSidelink message as follows:

…

1>	else if out of coverage on the frequency used for NR sidelink communication as defined in TS 38.304 [20]; and the concerned frequency is included in sl-FreqInfoToAddModList in RRCReconfiguration or in sl-FreqInfoList within SIB12:

2>	set inCoverage to true;

2>	set reservedBits to the value of the corresponding field included in the preconfigured sidelink parameters (i.e. sl-PreconfigGeneral in SidelinkPreconfigNR defined in 9.3);

2>	set sl-TDD-Config to the value representing the same meaning as that is included in the corresponding field included in the preconfigured sidelink parameters (i.e. sl-PreconfigGeneral in SL-PreconfigurationNR defined in 9.3) as described in TS 38.213, clause 16.1 [13];

1>	else if out of coverage on the frequency used for NR sidelink communication as defined in TS 38.304 [20]; and the UE selects GNSS as the synchronization reference and sl-SSB-TimeAllocation3 is not configured for the frequency used in SidelinkPreconfigNR:

2>	set inCoverage to true;

2>	set reservedBits to the value of the corresponding field included in the preconfigured sidelink parameters (i.e. sl-PreconfigGeneral in SidelinkPreconfigNR defined in 9.3);

2>	set sl-TDD-Config to the value representing the same meaning as that is included in the corresponding field included in the preconfigured sidelink parameters (i.e. sl-PreconfigGeneral in SL-PreconfigurationNR defined in 9.3) as described in TS 38.213, clause 16.1 [13];

1>	else if the UE has a selected SyncRef UE (as defined in 5.8.6):

2>	set inCoverage to false;

2>	set sl-TDD-Config and reservedBits to the value of the corresponding field included in the received MasterInformationBlockSidelink;

1>	else:

2>	set inCoverage to false;

2>	set reservedBits to the value of the corresponding field included in the preconfigured sidelink parameters (i.e. sl-PreconfigGeneral in SidelinkPreconfigNR defined in 9.3);

2> set sl-TDD-Config to the value representing the same meaning as that is included in the corresponding field included in the preconfigured sidelink parameters (i.e. sl-PreconfigGeneral in SL-PreconfigurationNR defined in 9.3) as described in TS 38.213, clause 16.1 [13];

1>	set directFrameNumber and slotIndex according to the slot used to transmit the SLSS, as specified in 5.8.5.3;

1>	submit the MasterInformationBlockSidelink to lower layers for transmission upon which the procedure ends;

[TS 38.331, clause 5.8.12]

When the UE selects GNSS as the synchronization reference source, the DFN, the subframe number within a frame and slot number within a frame used for NR sidelink communication are derived from the current UTC time, by the following formulae:

DFN= Floor (0.1*(Tcurrent –Tref–OffsetDFN)) mod 1024

SubframeNumber= Floor (Tcurrent –Tref–OffsetDFN) mod 10

SlotNumber= Floor ((Tcurrent –Tref–OffsetDFN)*2μ) mod (10*2μ)

Where:

Tcurrent is the current UTC time obtained from GNSS. This value is expressed in milliseconds;

Tref is the reference UTC time 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Thursday, December 31, 1899 and Friday, January 1, 1900). This value is expressed in milliseconds;

OffsetDFN is the value sl-OffsetDFN if configured, otherwise it is zero. This value is expressed in milliseconds.

μ=0/1/2/3 corresponding to the 15/30/60/120 kHz of SCS for SL, respectively.

NOTE 1:	In case of leap second change event, how UE obtains the scheduled time of leap second change to adjust Tcurrent correspondingly is left to UE implementation. How UE handles to avoid the sudden discontinuity of DFN is left to UE implementation.

NOTE 2:	Void.

12.2.2.1.3	Test description

12.2.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell

-	NR Cell 1 is the serving cell.

-	System information combination NR-14 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

-	NR-SS-UE

-	NR-SS-UE 1, 2, 3, 4 and 5 operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE 1 transmits SL-SSB with SLSSID = 3, inCoverage = true in slots determined by sl-SSB-TimeAllocation1 and NR Cell 1 timing.

-	NR-SS-UE 2 transmits SL-SSB with SLSSID = 4, inCoverage = false in slots determined by sl-SSB-TimeAllocation2 and NR Cell 1 timing.

-	NR-SS-UE 3 transmits SL-SSB with SLSSID = 0, inCoverage = true in slots determined by sl-SSB-TimeAllocation1 and GNSS timing.

-	NR-SS-UE 4 transmits SL-SSB with SLSSID = 0, inCoverage = false in slots determined by sl-SSB-TimeAllocation2 and GNSS timing.

-	NR-SS-UE 5 transmits SL-SSB with SLSSID = 338, inCoverage = false in slots determined by sl-SSB-TimeAllocation1 and arbitrary timing.

-	GNSS simulator

- The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	The UE is equipped with below information in UE or in a USIM containing default values (as per TS 38.508-1 [4]) except for those listed in Table 12.2.2.1.3.1-1.

Table 12.2.2.1.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | As per TS 36.508 [18] clause 4.9.3.4 |  |
| EFVST |  | Service n°119 is "available" |  |
| EFV2XP_PC5 |  | As per TS 38.508-1[4] clause 4.8.3.3.3 SL-PreconfigurationNR included in V2X data policy over PC5 is defined in Table 12.2.2.1.3.3-1, Table 12.2.2.1.3.3-1A and Table 12.2.2.1.3.3-2 |  |

Preamble:

-	The UE is in state 3N-B as defined in TS 38.508-1 [4], subclause 4.4A, using generic procedure parameter Sidelink (On), GNSS Sync (On) and Test Mode (On) as defined in TS 38.508-1 [4], subclause 4.5.1.

12.2.2.1.3.2	Test procedure sequence

Table 12.2.2.1.3.2-1 illustrates the sidelink power levels to be applied for NR-SS-UE 1, 2, 3, 4 and 5 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1" to "T7", are applied at the point indicated in the Main behaviour description in Table 12.2.2.1.3.2-2.

Table 12.2.2.1.3.2-1: Time instances of NR-SS-UE power level and parameter changes in conducted test environment

|  | Parameter | Unit | NR-SS-UE 1 | NR-SS-UE 2 | NR-SS-UE 3 | NR-SS-UE 4 | NR-SS-UE 5 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | NR-SS-UE power | dBm/ SCS | OFF | OFF | -85 | OFF | OFF | Priority of NR-SS-UE 3 is lower than GNSS |
|  | EPRE ratio of S-SSS to NR-SS-UE power | dB | - | - | 0 | - | - |  |
| T1 | NR-SS-UE power | dBm/ SCS | OFF | OFF | -85 | -85 | OFF | Priority of NR-SS-UE 4 is lower than priority of NR-SS-UE 3 |
|  | EPRE ratio of S-SSS to NR-SS-UE power | dB | - | - | 0 | 0 | - |  |
| T2 | NR-SS-UE power | dBm/ SCS | OFF | OFF | OFF | -85 | OFF | Priority of NR Cell is lower than priority of NR-SS-UE 3 when GNSS has the highest priority |
|  | EPRE ratio of S-SSS to NR-SS-UE power | dB | - | - | - | 0 | - |  |
| T3 | NR-SS-UE power | dBm/ SCS | -85 | OFF | OFF | OFF | OFF | Priority of NR-SS-UE 1 is lower than NR Cell. |
|  | EPRE ratio of S-SSS to NR-SS-UE power | dB | 0 | - | - | - | - |  |
| T4 | NR-SS-UE power | dBm/ SCS | -85 | -85 | OFF | OFF | OFF | Priority of NR-SS-UE 2 is lower than priority of NR-SS-UE 1 |
|  | EPRE ratio of S-SSS to NR-SS-UE power | dB | 0 | 0 | - | - | - |  |
| T5 | NR-SS-UE power | dBm/ SCS | OFF | -85 | OFF | OFF | OFF | Priority of GNSS is lower than priority of NR-SS-UE 2 when gNB has the highest priority |
|  | EPRE ratio of S-SSS to NR-SS-UE power | dB | - | 0 | - | - | - |  |
| T6 | NR-SS-UE power | dBm/ SCS | OFF | OFF | OFF | -85 | -85 | Priority of NR-SS-UE 5 is lower than priority of NR-SS-UE 4 |
|  | EPRE ratio of S-SSS to NR-SS-UE power | dB | - | - | - | 0 | 0 |  |
| T7 | NR-SS-UE power | dBm/ SCS | OFF | OFF | OFF | OFF | -85 | Priority of UE internal clock is lower than NR-SS-UE 5 |
|  | EPRE ratio of S-SSS to NR-SS-UE power | dB | - | - | - | - | 0 |  |

Table 12.2.2.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to configure sl-SyncPriority = gnss | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS transmits a CLOSE UE TEST LOOP message to close UE test loop mode E (Transmit Mode). | <-- | NR RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 4 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message | --> | NR RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 4A | The UE starts broadcasting continuously | - | - | - | - |
| 5 | The SS waits 10 seconds | - | - | - | - |
| 6 | Check: Does the UE transmit SL-SSBs which satisfy all following conditions? - SLSSID = 0; - inCoverage = true in SL-MIB; - slotIndex and directFrameNumber in SL-MIB are consistent with the slot index and DFN calculated based on the UTC time obtained from GNSS as specified in TS 38.331 [22] clause 5.8.12; - transmitted in slots determined by sl-SSB-TimeAllocation1 and GNSS timing; - reserveBits in SL-MIB is consistent with reserveBits in pre-configuration. | - | - | 1 | P |
| 7 | The SS powers off GNSS simulator. | - | - | - | - |
| 8 | The SS re-adjusts the NR-SS-UE power level according to row "T1" in table 12.2.2.1.3.2-1. | - | - | - | - |
| 9 | The SS waits 10 seconds | - | - | - | - |
| 10 | Check: Does the UE transmit SL-SSBs which satisfy all following conditions? - SLSSID = 0; - inCoverage = false in SL-MIB; - slotIndex and directFrameNumber in SL-MIB are consistent with the slot index and DFN of NR-SS-UE 3; - transmitted in slots determined by sl-SSB-TimeAllocation2 and the NR-SS-UE 3 timing; - reserveBits in SL-MIB is consistent with reserveBits in SL-MIB of NR-SS-UE 3. | - | - | 2 | P |
| 11 | The SS re-adjusts the NR-SS-UE power level according to row "T2" in table 12.2.2.1.3.2-1. | - | - | - | - |
| 12 | The SS waits 10 seconds | - | - | - | - |
| 13 | Check: Does the UE transmit SL-SSBs which satisfy all following conditions? - SLSSID = 336; - inCoverage = false in SL-MIB; - slotIndex and directFrameNumber in SL-MIB are consistent with the slot index and DFN of NR-SS-UE 4. - transmitted in slots determined by sl-SSB-TimeAllocation1 and the NR-SS-UE 4 timing? - reserveBits in SL-MIB is consistent with reserveBits in SL-MIB of NR-SS-UE 4. | - | - | 3 | P |
| 14 | The SS transmits an RRCReconfiguration message to configure sl-SyncPriority = gnbEnb | <-- | NR RRC: RRCReconfiguration | - | - |
| 15 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 16 | The SS re-adjusts the NR-SS-UE power level according to row "T3" in table 12.2.2.1.3.2-1. | - | - | - | - |
| 17 | The SS waits 10 seconds | - | - | - | - |
| 18 | Check: Does the UE transmit SL-SSBs which satisfy all following conditions? - SLSSID is consistent with SLSSID in sl-ConfigDedicatedNR; - inCoverage = true in SL-MIB; - slotIndex and directFrameNumber in SL-MIB are consistent with the slot index and SFN of NR Cell 1. - transmitted in slots determined by sl-SSB-TimeAllocation1 and the NR Cell 1 timing? - reserveBits in SL-MIB is consistent with reserveBits in pre-configuration. | - | - | 4 | P |
| 19 | The SS sends an RRCRelease message and powers off NR Cell 1. | <-- | NR RRC: RRCRelease | - | - |
| 20 | The SS re-adjusts the NR-SS-UE power level according to row "T4" in table 12.2.2.1.3.2-1. | - | - | - | - |
| 21 | The SS waits 10 seconds | - | - | - | - |
| 22 | Check: Does the UE transmit SL-SSBs which satisfy all following conditions? - SLSSID is consistent with SLSSID of NR-SS-UE 1; - inCoverage = false in SL-MIB; - slotIndex and directFrameNumber in SL-MIB are consistent with the slot index and DFN of NR-SS-UE 1; - transmitted in slots determined by sl-SSB-TimeAllocation2 and NR-SS-UE 1 timing; - reserveBits in SL-MIB is consistent with reserveBits in SL-MIB of NR-SS-UE 1. | - | - | 5 | P |
| 23 | The SS powers on GNSS simulator. | - | - | - | - |
| 24 | The SS re-adjusts the NR-SS-UE power level according to row "T5" in table 12.2.2.1.3.2-1. | - | - | - | - |
| 25 | The SS waits 10 seconds | - | - | - | - |
| 26 | Check: Does the UE transmit SL-SSBs which satisfy all following conditions? - SLSSID equals to SLSSID of NR-SS-UE 2 plus 336; - inCoverage = false in SL-MIB; - slotIndex and directFrameNumber in SL-MIB are consistent with the slot index and DFN of NR-SS-UE 2; - transmitted in slots determined by sl-SSB-TimeAllocation1 and NR-SS-UE 2 timing; - reserveBits in SL-MIB is consistent with reserveBits in SL-MIB of NR-SS-UE 2. | - | - | 6 | P |
| 27 | The SS re-adjusts the NR-SS-UE power level according to row "T0" in table 12.2.2.1.3.2-1. | - | - | - | - |
| 28 | The SS waits 10 seconds | - | - | - | - |
| 29 | Check: Does the UE transmit SL-SSBs which satisfy all following conditions? - SLSSID = 0; - inCoverage = true in SL-MIB; - slotIndex and directFrameNumber in SL-MIB are consistent with the slot index and DFN calculated based on the UTC time obtained from GNSS as specified in TS 38.331 [22] clause 5.8.12; - transmitted in slots determined by sl-SSB-TimeAllocation1 and GNSS timing; - reserveBits in SL-MIB is consistent with reserveBits in pre-configuration. | - | - | 7 | P |
| 30 | The SS powers off GNSS simulator. | - | - | - | - |
| 31 | The SS re-adjusts the NR-SS-UE power level according to row "T1" in table 12.2.2.1.3.2-1. | - | - | - | - |
| 32 | The SS waits 10 seconds | - | - | - | - |
| 33 | Check: Does the UE transmit SL-SSBs which satisfy all following conditions? - SLSSID = 0; - inCoverage = false in SL-MIB; - slotIndex and directFrameNumber in SL-MIB are consistent with the slot index and DFN of NR-SS-UE 3; - transmitted in slots determined by sl-SSB-TimeAllocation2 and NR-SS-UE 3 timing; - reserveBits in SL-MIB is consistent with reserveBits in SL-MIB of NR-SS-UE 3. | - | - | 8 | P |
| 34 | The SS re-adjusts the NR-SS-UE power level according to row "T6" in table 12.2.2.1.3.2-1. | - | - | - | - |
| 35 | The SS waits 10 seconds | - | - | - | - |
| 36 | Check: Does the UE transmit SL-SSBs which satisfy all following conditions? - SLSSID = 336; - inCoverage = false in SL-MIB; - slotIndex and directFrameNumber in SL-MIB are consistent with the slot index and DFN of NR-SS-UE 4; - transmitted in slots determined by sl-SSB-TimeAllocation1 and NR-SS-UE 4 timing; - reserveBits in SL-MIB is consistent with reserveBits in SL-MIB of NR-SS-UE 4. | - | - | 9 | P |
| 37 | The SS re-adjusts the NR-SS-UE power level according to row "T7" in table 12.2.2.1.3.2-1. | - | - | - | - |
| 38 | The SS waits 10 seconds | - | - | - | - |
| 39 | Check: Does the UE transmit SL-SSBs which satisfy all following conditions? - SLSSID is consistent with SLSSID of NR-SS-UE 5; - inCoverage = false in SL-MIB; - slotIndex and directFrameNumber in SL-MIB are consistent with the slot index and DFN of NR-SS-UE 5; - transmitted in slots determined by sl-SSB-TimeAllocation2 and NR-SS-UE 5 timing; - reserveBits in SL-MIB is consistent with reserveBits in SL-MIB of NR-SS-UE 5. | - | - | 10 | P |
| 40 | The SS triggers UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 41 | Void | - | - | - | - |

12.2.2.1.3.3	Specific message contents

Table 12.2.2.1.3.3-1: V2X service identifier to default mode of communication mapping rule (Pre-configuration, UE under test)

| Derivation Path: TS 38.508-1 [4] Table 4.7.5.5-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DMC |  | '10'B | Default mode of communication is set to broadcast |  |

Table 12.2.2.1.3.3-1A: SL-SDAP-Config (Pre-configuration, UE under test)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-SDAP-Config-r16 ::= SEQUENCE { |  |  |  |  |
| sl-CastType-r16 |  | broadcast |  |  |
| } |  |  |  |  |

Table 12.2.2.1.3.3-2: SL-SyncConfig (Pre-configuration, UE under test)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-31 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-SyncConfig-r16 ::= SEQUENCE { |  |  |  |
| sl-SSID-r16 | Not present |  |  |  |
| txParameters-r16 SEQUENCE { |  |  |  |  |
| syncTxThreshOoC-r16 | 13 | actual threshold is +infinity |  |  |
| } |  |  |  |  |
| } |  |  |  |

Table 12.2.2.1.3.3-3:  RRCReconfiguration (Table 12.2.2.1.3.2-2, Step 1 and 14)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition SIDELINK |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| sl-ConfigDedicatedNR-r16 CHOICE { |  |  |  |  |
| setup |  | SL-ConfigDedicatedNR-r16 | Table 12.2.2.1.3.3-4 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.2.2.1.3.3-4: SL-ConfigDedicatedNR (Table 12.2.2.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-7 with condition SELECTED |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ConfigDedicatedNR-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PHY-MAC-RLC-Config-r16 SEQUENCE { |  |  |  |  |
| sl-FreqInfoToAddModList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfig-r16 { |  | 1 entry |  |  |
| SL-FreqConfig-r16[1] SEQUENCE { |  | SL-FreqConfig-r16 | entry 1 |  |
| sl-SyncConfigList-r16 SEQUENCE (SIZE (1..maxSL-SyncConfig-r16)) OF SL-SyncConfig-r16 { |  | 1 entry |  |  |
| SL-SyncConfig-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-SSID-r16 |  | 2 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-SyncPriority-r16 |  | gnss |  | Step 1 |
|  |  | gnbEnb |  | Step 14 |
| } |  |  |  |  |
| } |  |  |  |  |
| networkControlledSyncTx-r16 |  | on |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.2.2.1.3.3-5: CLOSE UE TEST LOOP (Table 12.2.2.1.3.2-2, Step 3)

| Derivation Path: 36.508 [7] Table 4.7A-3 with condition UE TEST LOOP MODE E(V2X Transmission) |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE test loop mode E LB setup |  |  |  |  |
| Communication Transmit or Receive |  | 0 0 0 0 0 0 0 1 | ‘01’ indicates V2X UE triggered to transmit NR sidelink communication with single spatial layer. |  |

Table 12.2.2.1.3.3-6: MasterInformationBlockSidelink (NR-SS-UE 1, 2, 3, 4 and 5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1A-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MasterInformationBlockSidelink ::= SEQUENCE { |  |  |  |  |
| inCoverage-r16 | true |  | NR-SS-UE 1, 3 |
|  | false |  | NR-SS-UE 2, 4, 5 |
| directFrameNumber-r16 | DFN determined based on the formula given in 38.331 [22] clause 5.8.12. |  | NR-SS-UE 3, 4 |
|  | SFN of NR Cell 1 |  | NR-SS-UE 1, 2, 5 |
| slotIndex-r16 | slot index determined based on the formula given in 38.331 [22] clause 5.8.12. |  | NR-SS-UE 3, 4 |
|  | slot index of NR Cell 1 |  | NR-SS-UE 1, 2, 5 |
| reservedBits-r16 | 01 |  | NR-SS-UE 1, 3 |
|  | 10 |  | NR-SS-UE 2, 4 |
|  | 11 |  | NR-SS-UE 5 |
| } |  |  |  |  |

Table 12.2.2.1.3.3-7: MasterInformationBlockSidelink (Table 12.2.2.1.3.2-2, step 6, 10, 13, 18, 22, 26, 29, 33, 36 and 39 UE under test)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1A-1 with condition TX |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MasterInformationBlockSidelink ::= SEQUENCE { |  |  |  |  |
| inCoverage-r16 | true |  | Step 6, 18, 29 |
|  | false |  | Step 10, 13, 22, 26, 33, 36, 39 |
| directFrameNumber-r16 | DFN determined based on the formula given in 38.331 [22] clause 5.8.12 |  | Step 6, 10, 13, 29, 33, 36, |
|  | SFN of NR Cell 1 |  | Step 18, 22, 26, 39 |
| slotIndex-r16 | slot index determined based on the formula given in 38.331 [22] clause 5.8.12 |  | Step 6, 10, 13, 29, 33, 36, |
|  | Slot index of NR Cell 1 |  | Step 18, 22, 26, 39 |
| reservedBits-r16 | 00 | Same as pre-configuration | Step 6, 18, 29 |
|  | 01 | Same as NR-SS-UE 1 | Step 10, 22, 33 |
|  | 10 | Same as NR-SS-UE 2 | Step 13, 26, 36 |
|  | 11 | Same as NR-SS-UE 3 | Step 39 |
| } |  |  |  |  |

<End of modified section 1>