3GPP TSG-RAN5 Meeting #98	R5-231519

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1410 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test procedure for registration of a MUSIM UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Most of the test cases in MUSIM WP needs UE to perform registration on both the SIM cards and subsequent test procedure starts. Currently TTCN may not be able to handle simulataneous registrations from both the SIM cards, so there is a need to come up with deterministic way for registration with both the SIMs in a MUSIM UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test procedure for registration of a MUSIM UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Initial registration procedure needs to be unnecessarily repeated for MUSIM test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.2E (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-1 CR 5176 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-231519 is the final version of R5-230549 with below changes: 1. Resolved 4.5.XX to 4.5.2E. 2. Updated Specific message content Table 4.5.2E.4-2 for Attach Accept message. |  |  |  |  |  |  |  |  |

<Start of modified section>

### 4.5.2E	MUSIM UE Registration

The same assumptions and definitions apply as in clause 4.5.2.

#### 4.5.2E.1	Initial conditions

System Simulator:

-	Cell A (home PLMN1) and Cell G (home PLMN2) are configured according to table 6.3.2.2-1 and table 6.3.2.2-3. Any change in the cells and their configurations shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

-	The UE is a MUSIM UE.

-	The UE is equipped with two USIMs with configuration as defined in below Tables 4.5.2E.1-1 and 4.5.2E.1-2.

Table 4.5.2E.1-1: USIM A configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to 001-01. |  |
| EFFPLMN |  | 001-02 |  |

Table 4.5.2E.1-2: USIM B configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to 001-02. |  |
| EFFPLMN |  | 001-01 |  |

#### 4.5.2E.2	Definition of system information messages

The default system information messages are used.

#### 4.5.2E.3	Procedure

Table 4.5.2E.3-1: MUSIM UE registration procedure

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - Cell A as the "Serving cell". - Cell G as a "Non-Suitable Off cell". | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| - | The following messages are to be observed on Cell A unless explicitly stated otherwise. | - | - | - | - |
| 3 | Check: Does UE with USIM A configuration transmit ATTACH REQUEST message including UE network capability with at least one MUSIM feature set to “supported”? | --> | ATTACH REQUEST | - | P |
| 4-16 | Steps 5 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 17 | The SS configures: - Cell G as the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on Cell G unless explicitly stated otherwise. | - | - | - | - |
| 18 | Check: Does UE with USIM B configuration transmit ATTACH REQUEST message including UE network capability with at least one MUSIM feature set to “supported”? | --> | ATTACH REQUEST | - | P |
| 19-31 | Steps 5 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |

#### 4.5.2E.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the exceptions below.

Table 4.5.2E.4-1: ATTACH REQUEST (Steps 3 and 18, Table 4.5.2E.3-1)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE network capability |  |  | pc_EPC_MUSIM |
| All octets with the exception of octet 10, bit 6 to bit 8 and octet 11,bit 1 to 2 | Any allowed value |  |  |
| NAS signalling connection release (NCR) (octet 10, bit 6) | ‘1’B | NAS signalling connection release supported | pc_EPC_MUSIM_NCR |
| Paging indication for voice services (PIV) (octet 10, bit 7) | ‘1’B | paging indication for voice services supported | pc_EPC_MUSIM_PIV |
| Reject paging request (RPR) (octet 10, bit 8) | ‘1’B | reject paging request supported | pc_EPC_MUSIM_RPR |
| Paging restriction (PR) (octet 11, bit 1) | ‘1’B | paging restriction supported | pc_EPC_MUSIM_PR |
| Paging timing collision control (PTCC) (octet 11, bit 2) | ‘1’B | paging timing collision control supported | pc_EPC_MUSIM_PTCC |

Table 4.5.2E.4-2: Message ATTACH ACCEPT (Steps 13 and 28, Table 4.5.2E.3-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS network feature support | '0000 0011 0000 0000 0001 1111'B |  |  |

<End of modifed section>