3GPP TSG-RAN5 Meeting #98	R5-231580

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3618 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test case for RRC downlink segmentation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At RAN5#90-e it was agreed that that test cases are required to test RRC DL segmentation(R5-210581) and at RAN5#95(R5-223261) the test method was agreed. No test case has been added yet for DL segmeantation of the RRCResume message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test case for the DL segmeantation of the RRCResume message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will fail conformant UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.9.3 [New] |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-2 CR 0333 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is an update to R5-231348 and the outcome of 1 revision of this. |  |  |  |  |  |  |  |  |

##### 8.1.5.9.3	RRC resume / DL segment transfer

8.1.5.9.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state, having sent an RRCResumeRequest message with resumeIdentity set to the stored shortI-RNTI value }

ensure that {

when { SS initiates the DL Dedicated Message Segment transfer procedure to send an RRCResume message }

then { UE sucessfully re-establishes the DRB and SRB}

(2)

with { UE in NR RRC_INACTIVE state, having sent an RRCResumeRequest message with resumeIdentity set to the stored fullI-RNTI value }

ensure that {

when { SS initiates the DL Dedicated Message Segment transfer procedure to send an RRCResume message }

then { UE sucessfully re-establishes the DRB and SRB}

8.1.5.9.3.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.7.6.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.7.6.3]

Upon receiving DLDedicatedMessageSegment message, the UE shall:

1>	store the segment included in rrc-MessageSegmentContainer;

1>	if all segments of the message have been received:

2>	assemble the message from the received segments and process the message according to 5.3.5.3 for the RRCReconfiguration message or 5.3.13.4 for the RRCResume message;

2>	discard all segments.

8.1.5.9.3.3	Test Description

8.1.5.9.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used.

UE:

-	None.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 2N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

-	Else, the UE is in 5GS state 2N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

8.1.5.9.3.3.2	Test procedure sequence

Table 8.1.5.9.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message including a matched identity (correct fullI-RNTI). | <-- | NR RRC: Paging | - | - |
| 2 | The UE transmit an RRCResumeRequest message by setting resumeIdentity to the stored shortI-RNTI value. | --> | NR RRC: RRCResumeRequest | - | - |
| 3 | The SS transmits a DLDedicatedMessageSegment containing the first segment of the 3 segment RRCResume message. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 4 | The SS transmits a DLDedicatedMessageSegment containing the second segment of the 3 segment RRCResume message. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 5 | The SS transmits a DLDedicatedMessageSegment containing the third (last) segment of the 3 segment RRCResume message. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 6 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 7 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with the first PDU session? | - | - | 1 | - |
| 8 | The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 9 | The SS changes the SIB1 of NR Cell 1 to set the useFullResumeID to True. | - | - | - | - |
| 10 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 11 | Wait for 2.1* modification period second for the UE to receive new system information. (Note 1) | - | - | - | - |
| 12 | The SS transmits a Paging message including a matched identity (correct fullI-RNTI). | <-- | NR RRC: Paging | - | - |
| 12 | The UE transmit an RRCResumeRequest1 message by setting resumeIdentity to the stored fullI-RNTI value? | --> | NR RRC: RRCResumeRequest1 | - | - |
| 13 | The SS transmits a DLDedicatedMessageSegment containing the first segment of the 2 segment RRCResume message. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 14 | The SS transmits a DLDedicatedMessageSegment containing the second/last segment of the 2 segment RRCResume message. | <-- | NR RRC: DLDedicatedMessageSegment |  |  |
| 15 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 16 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with the first PDU session? |  |  | 2 |  |
| 17 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| Note 1: The modification period, expressed in number of radio frames = modificationPeriodCoeff * defaultPagingCycle. |  |  |  |  |  |

8.1.5.9.3.3.3	Specific message contents

Table 8.1.5.9.3.3.3-1: Paging (step 1, Table 8.1.5.9.3.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-9 with condition NR_RRC_RESUME

Table 8.1.5.9.3.3.3-2: DLDedicatedMessageSegment (Step 3, 4, 5, 13 and 14, Table 8.1.5.9.3.3.2-1)

| Derivation Path: TS 38.508-1[4], table 4.6.1-2B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dlDedicatedMessageSegment-r16 SEQUENCE { |  |  |  |  |
| segmentNumber-r16 |  | 0 |  | Step 3, Step 13 |
| segmentNumber-r16 |  | 1 |  | Step 4, Step 14 |
| segmentNumber-r16 |  | 2 |  | Step 5 |
| rrc-MessageSegmentContainer-r16 |  | OCTET STRING (CONTAINING First segment of the RRCResume message) | Table 8.1.5.9.3.3.3-3 | Step 3, Step 13 |
| rrc-MessageSegmentContainer-r16 |  | OCTET STRING (CONTAINING Second segment of the RRCResume message) | Table 8.1.5.9.3.3.3-3 | Step 4, Step 14 |
| rrc-MessageSegmentContainer-r16 |  | OCTET STRING (CONTAINING Third/Last segment of the RRCResume message) | Table 8.1.5.9.3.3.3-3 | Step 5 |
| rrc-MessageSegmentType-r16 |  | notLastSegment |  | Step 3, Step 4, Step 13 |
| rrc-MessageSegmentType-r16 |  | LastSegment |  | Step 5, Step 14 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.5.9.3.3.3-3: RRCResume (Table 8.1.5.9.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-17 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResume ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcResume SEQUENCE { |  |  |  |
| lateNonCriticalExtension | '00,,,00'O | Add enough padding so that: 2 x maximum PDCP SDU size < encoded DLDedicatedMessageSegment size < 3 x maximum PDCP SDU size | Step 3, Step 4, Step 5 |
| lateNonCriticalExtension | '00,,,00'O | Add enough padding so that: maximum PDCP SDU size < encoded DLDedicatedMessageSegment size < 2 x maximum PDCP SDU size | Step 13, Step 14 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.3.3.3-4: RRCRelease (step 8, Table 8.1.5.9.3.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with condition NR_RRC_INACTIVE

Table 8.1.5.9.3.3.3-5: SIB1 (step 9, Table 8.1.5.9.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| useFullResumeID | true |  |  |
| } |  |  |  |

Table 8.1.5.9.3.3.3-6: Paging (step 12, Table 8.1.5.9.3.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-9 with condition NR_RRC_RESUME