3GPP TSG-RAN5 Meeting #98	R5-231633

# Athens, February 27  - March 3 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0749 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of CA_n41A-n71A. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CA_n41-n71 config has exceptions in reference sensitivity |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 4.1.3.1-2 updated with CA_n41-n71 Table 4.1.3.1-6 updated with CA_n41-n71 Note added also for CA_n41-n66 in Table 4.1.3.1-2 for clarity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case config will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.521-1 CR 2040 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Final version, originates from R5-231225r2 |  |  |  |  |  |  |  |  |

##### << Beginning of changes>>

Table 4.1.3.1-2: Reference Sensitivity test cases per CA configuration (two bands)

| CA config | Two band refsens exception, victim band (Note 1) | Requirement coverage (Note 1) | Test point analysis updated |
| --- | --- | --- | --- |
| CA_n1A-n3A | n1 (IMD3) n3(CBI) | IMD,CBI | RAN5#95-e |
| CA_n1A-n8A | n1 (IMD4) | IMD | RAN5#95-e |
| CA_n1A-n77A | n77 (HD2) | HD | RAN5#89-e |
| CA_n1A-n78A | n1 (IMD4) | IMD | RAN5#94 |
| CA_n2A-n48A | n48 (HD2), n2(IMD4) | HD, IMD | RAN5#96-e |
| CA_n2A-n5A | - | NE | RAN5#97 |
| CA_n2A-n66A | n2(IMD3), n66(IMD5) | IMD | RAN5#96-e |
| CA_n2A-n77A | n77 (HD2), n2 (HM), n2(IMD2), n2(IMD4), n2(IMD5), n2(IMD7) | HD, HM, IMD | RAN5#96-e |
| CA_n3A-n5A | n3 (IMD4 ¦2*fn3-2*fn5¦) n5 (IMD2 ¦fn3-fn5¦) | IMD | RAN5#95-e |
| CA_n3A-n77A | n78 (HD2) | HD | RAN5#84 |
| CA_n3A-n78A | n3 (IMD2 ¦fn78-fn3¦) n3 (IMD4 ¦fn78-3*fn3¦) n78 (HD2) | HD, IMD | RAN5#87-e |
| CA_n5A-n66A | n5(IMD2) | IMD | RAN5#96-e |
| CA_n5A-n77A | n77 (HD4), n77 (HD5), n5 (HM), n5(IMD4), n5(IMD5) | HD, HM,, IMD | RAN5#96-e |
| CA_n5A-n78A | n78 (HD2) | HD | RAN5#94-e |
| CA_n7A-n78A | n7 and n78 (CBI) | CBI | RAN5#94-e |
| CA_n8A-n78A | n8 (IMD4 ¦fn78-3*fn8¦) n78 (HD4) | HD, IMD | RAN5#87-e |
| CA_n26A-n66A | n26 (IMD2) | IMD | RAN5#94-e |
| CA_n26A-n70A | n26 (IMD2) | IMD | RAN5#94-e |
| CA_n28A-n41A |  | NE | RAN5#91-e |
| CA_n28A-n79A |  | NE | RAN5#91-e |
| CA_n29A-n66A |  | NE | RAN5#94-e |
| CA_n29A-n70A |  | NE | RAN5#94-e |
| CA_n29A-n71A | n29 (CBI) | CBI | RAN5#95-e |
| CA_n41A-n66A3 | n66 (CBI), n66 (IMD5) | CBI | RAN#97 |
| CA_n41A-n71A 3 | n41(HD4), n71 (IMD4) | HD | RAN5#98 |
| CA_n41A-n79A |  | NE | RAN5#83 |
| CA_n48A-n66A | n48 (HD2), n66 (IMD5) | HD, IMD | RAN5#94-e |
| CA_n48A-n66(2A) | (NOTE 2) | NE | RAN5#95-e |
| CA_n48A-n70A | n70 (IMD2) | IMD | RAN5#94-e |
| CA_n48A-n71A |  | NE | RAN5#94-e |
| CA_n48A-n71(2A) | - | NE | RAN5#95-e |
| CA_n48A-n77A | - | NE | RAN5#97-e |
| CA_n48B-n66A | (NOTE 2) | NE | RAN5#95-e |
| CA_n48B-n70A | (NOTE 2) | NE | RAN5#95-e |
| CA_n48B-n71A | - | NE | RAN5#95-e |
| CA_n48(2A)-n66A | (NOTE 2) | NE | RAN5#95-e |
| CA_n48(2A)-n66(2A) | (NOTE 2) | NE | RAN5#96-e |
| CA_n48(2A)-n70A | (NOTE 2) | NE | RAN5#95-e |
| CA_n48(2A)-n71A | - | NE | RAN5#95-e |
| CA_n48(2A)-n71(2A) | - | NE | RAN5#96-e |
| CA_n66A-n70A |  | NE | RAN5#94-e |
| CA_n66A-n71A | n66 (IMD4) | IMD | RAN5#87-e |
| CA_n66A-n77A | n77 (HD2), n66(IMD2), n66(IMD5) | HD, IMD | RAN5#97 |
| CA_n70A-n71A | n70 (HD3), n70 (IMD4) | HD, IMD | RAN5#85 |
| CA_n71A-n77A | n71 (IMD5) | IMD | RAN5#97-e |
| NOTE 1: Notations used NE: Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-1 [5], 7.3A.4 HM: RX Harmonic Mixing, as defined in TS 38.101-1 [5], 7.3A.4 IMD: Intermodulation Distortion, as defined in TS 38.101-1 [5], 7.3A.5 CBI: Cross Band Isolation, as defined in TS 38.101-1 [5], 7.3A.6 NOTE 2: Exception for this band is tested with two carrier case. NOTE 3: Only single uplink analysed. IMD exception not yet covered. |  |  |  |

##### << Unchanged clauses omitted>>

Table 4.1.3.1-6: Test frequency selection per band pair for UL harmonics exception

| Band pair (Note 2) | Frequency | Channel BW [MHz] | Comment |
| --- | --- | --- | --- |
| n1A / n77A | Mid / Low | Highest/Highest | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |
|  | Mid / 3900 | 20/ Highest | Exception Note 2 of TS 38.101 table 7.3A.4-1 |
|  | Mid / 3870 | 20/20 | Exception Note 3 of TS 38.101 table 7.3A.4-1 |
| n2A/n48A | Mid/Mid | 20/20 | Non-exception. To be used in test cases in 7.3A.1 to 7.3A.4 |
| n2A/n48A | UL 1860 MHz / DL 3690 MHz | 20/20 | Exception Note 3 of TS 38.521 Table 7.3A.0.4-1 |
| n2A/n77A | Mid / 3850 Mhz | 20/100 | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |
|  | UL 1860 MHz / DL 3720 MHz | 20/100 | Exception Note 2 of TS 38.101 Table 7.3A.4-1 |
|  | UL 1860 MHz / DL 3690 MHz | 20/20 | Exception Note 3 of TS 38.521 Table 7.3A.0.4-1 |
| n3A / n77A | TBD/TBD | Highest/Highest | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |
|  | TBD/TBD | Highest/Highest | Exception Note 2 of TS38.101 table 7.3A.4-1 |
|  | TBD/TBD | 20/20 | Exception Note 3 of TS38.101 table 7.3A.4-1 |
| n3A / n78A | Mid/High | Highest/Highest | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |
|  | Mid/3495 MHz | Highest/Highest | Exception Note 2 of TS38.101 table 7.3A.4-1 |
|  | Mid/3465 MHz | 20/20 | Exception Note 3 of TS38.101 table 7.3A.4-1 |
| n5A/n77A | Mid/Mid | 20/100 | Non-exception. To be used in test cases in 7.3A.1 to 7.3A.4 |
|  | UL 834 MHz / DL 3336 MHz | 20/100 | Exception Note 4 of TS 38.101-1 table 7.3A.4-1 |
|  | UL 834 MHz / DL 4170 MHz | 20/100 | Exception Note 5 TS 38.101-1 table 7.3A.4-1 |
| n5A / n78A | Mid/High | Highest/Highest | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 in TS 38.521-1 [2] |
|  | Mid/3346 MHz | Highest/Highest | Exception Note 4 of TS 38.101 [5] table 7.3A.4-1 |
|  | Mid/3316 MHz | 20/20 | Exception Note 5 of TS 38.101 [5] table 7.3A.4-1 |
| n8A / n78A | Mid/High | Highest/Highest | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |
|  | Mid/3590 MHz | Highest/Highest | Exception Note 5 of TS38.101 table 7.3A.4-1 |
| n41A/n71A | UL Low/DL Low | Highest/Highest | Non-exception. Not overlapping interference since BWINT=4*20 MHz, FINT (146) ≥ (80+100)/2 for HD4. To be used in test cases 7.3A.1 to 7.3A.4 |
|  | UL Low / DL 2662 MHz | UL 5 / DL 10 | Exception Note 4 of TS 38.101 Table 7.3A.4-1 |
|  | UL Low / DL High | UL 5 / DL 100 | Exception Note 4 of TS 38.101 Table 7.3A.4-1 |
| n66A/n77A | Mid/Mid | 5/10 20/100 | Non-exception. To be used in test cases in 7.3A.1 to 7.3A.4 |
|  | UL 1750 MHz / DL 3500 MHz | 5/10 | Exception Note 2 of TS 38.101 Table 7.3A.4-1 |
|  | UL 1750 MHz / DL 3500 MHz | 20/100 | Exception Note 2 of TS 38.101 Table 7.3A.4-1 |
|  | UL 1712.5 MHz / DL 3400 MHz | 5/10 | Exception Note 6 of TS 38.101 Table 7.3A.4-1 |
| n70A / n71A | Mid/High | Highest/Highest | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |
|  | Low/Low | Highest / 10 | Exception Note 9 of TS38.101 table 7.3A.4-1 n71 UL RB allocation 20@10 |
|  | Low/Low | 5 / 5 | Exception Note 9 of TS38.101 table 7.3A.4-1 n71 UL RB allocation 8@10 |
| NOTE 1: This selection is used in test case 7.3A.1_1 unless otherwise stated. NOTE 2: Aggressor band in bold. |  |  |  |

##### << End of changes>>