3GPP TSG-RAN5 Meeting #98	R5-231645

# Athens, Greece, Feb 27th – Mar 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2121 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding PC2 intra-band contiguous testing to 6.5A.3.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 has introduced PC2 spurious emission requirement for intra-band contiguous CA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding PC2 spurious emission minimum requirement - Adding test configuration table for intra-band contiguous CA for both PC3 and PC2 - Adding PC2 related information in test procedure - Adding message exception for intra-band contiguous CA for both PC3 and PC2. - Adding test requirement for intra-band contiguous CA for both PC3 and PC2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Intra-band contiguous PC2 CA couldn’t be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A.3.1.0, 6.5A.3.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR also impacts PC3 intra-band CA requirements introduced under RAN5 WI NR_RF_FR1-UEConTest. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-230824 |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5A.3.1	General spurious emissions for CA

Editor’s Note:. The following aspects are either missing or not yet determined:

- Extending the coverage of the TCs with intra-band non-contiguous CA scenarios is FFS

6.5A.3.1.0	Minimum conformance requirements

For inter-band carrier aggregation with uplink assigned to two NR bands, the spurious emission requirement Table 6.5.3.1.3-2 apply for the frequency ranges that are more than FOOB as defined in Table 6.5.3.1.3-1 away from edges of the assigned channel bandwidth on a component carrier. If for some frequency a spurious emission requirement of individual component carrier overlaps with the spectrum emission mask or channel bandwidth of another component carrier then it does not apply.

NOTE 1:	For inter-band carrier aggregation with uplink assigned to two NR bands the requirements in Table 6.5.3.1.3-2 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5.3.1.3-2 would be considered to be verified by the measurements verifying the one uplink inter-band CA spurious emission requirement.

For intra-band contiguous carrier aggregation the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5A.3.1.0-1 from the edge of the aggregated channel bandwidth. For frequencies ΔfOOB greater than FOOB as specified in Table 6.5A.3.1.0-1 the spurious emission requirements in Table 6.5.3.1.3-2 are applicable. For power class 2 intra-band contiguous carrier aggregation, the spurious emissions is measured as the sum from both UE transmit antenna connectors when UE indicates support for dualPA-Architecture IE.

Table 6.5A.3.1.0-1: Boundary between out of band and spurious emission domain for intra-band contiguous carrier aggregation

| Aggregated Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| BWChannel_CA | BWChannel_CA + 5 |

For intra-band non-contiguous carrier aggregation transmission the spurious emission requirement is defined as a composite spurious emission requirement. Composite spurious emission requirement applies to frequency ranges that are more than FOOB away from the edges of each carrier in the gap and out of the gap. Composite spurious emission requirement is defined as follows

a)	Composite spurious emission requirement is a combination of individual sub-block spurious emission requirements

b)	In case the sub-block consist of one component carrier the sub-lock spurious emission requirement and FOOB are defined in subclause 6.5.3.1

c)	If for some frequency an individual sub-block spurious emission requirement overlaps with the general spectrum emission mask or the sub-block bandwidth of another sub-block then it does not apply

For the signalling is absent for dualPA-Architecture IE, if carrier leakage or I/Q image lands inside the gap spectrum between 2 UL CCs when UL CCs are synchronized with frequencies in the gap, exception to the general spurious requirement applies.

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5A.3.1

6.5A.3.1.1	General spurious emissions for CA (2UL CA)

6.5A.3.1.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5A.3.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2UL CA.

6.5A.3.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.3.1.0.

6.5A.3.1.1.4	Test description

6.5A.3.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.3.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.1.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg for both PCC and SCC Highest NRB_agg for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  | Lowest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A | CP-OFDM QPSK | Outer_Full | Outer_Full |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A3-1. |  |  |  |  |

Table 6.5A.3.1.1.4.1-2: Intra-band contiguous CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  | Lowest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A | CP-OFDM QPSK | Outer_Full | N/A |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left | N/A |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right | N/A |
| 4 |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Right |
| 5 |  | CP-OFDM QPSK | Outer_Full | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A3-1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.3.1.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5A.3.1.1.4.3.

6.5A.3.1.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.1.1. Message contents are defined in clause 6.5A.3.1.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5A.3.1.1.4.1-1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5A.3.1.1.5-1. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5A.3.1.1.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. For power class 2 intra-band contiguous carrier aggregation, the spurious emissions is measured as the sum from both UE transmit antenna connectors when UE indicates support for dualPA-Architecture IE.

6.5A.3.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.5A.3.1.1.4.3-1: FrequencyInfoUL-SIB for inter-band CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 16 |  | Power class 3 and Inter-band CA |

Table 6.5A.3.1.1.4.3-2: FrequencyInfoUL-SIB for intra-band contiguous CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10.5 |  | Power class 3 and Intra-band contiguous CA with Bandwidth class C Test ID 4 |
|  | 5 |  | Power class 3 and Inter-band CA Bandwidth class C Test ID 5 |
|  | 8.5 |  | Power class 3 and Inter-band CA Bandwidth class B Test ID 4 |
|  | 3 |  | Power class 3 and Inter-band CA Bandwidth class B Test ID 5 |
|  | 11.0 |  | Power class 2 and Inter-band CA Bandwidth class C Test ID 4 |
|  | 5.5 |  | Power class 3 and Inter-band CA Bandwidth class C Test ID 5 |

6.5A.3.1.1.5	Test requirement

This clause specifies the requirements for the specified NR band for Transmitter Spurious emissions requirement with frequency range as indicated in Table 6.5A.3.1.1.5-1 for inter-band UL CA configuration and Table 6.5A.3.1.1.5-3 for intra-band contiguous UL CA configuration. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

For inter-band CA,  the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth. If for some frequency a spurious emission requirement of individual component carrier overlaps with the spectrum emission mask or channel bandwidth of another component carrier then it does not apply.

For intra-band contiguous CA, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5A.3.1.0-1 from the edge of the aggregated channel bandwidth.

The measured average power of spurious emission, derived in step 6, shall not exceed the described value in Table 6.5A.3.1.1.5-1 for inter-band UL CA configuration and Table 6.5A.3.1.1.5-3 for intra-band contiguous UL CA configuration.

Table 6.5A.3.1.1.5-1: General spurious emissions test requirements for inter-band UL CA

| Frequency Range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| Test requirements for CA_n2A-n5A Configuration |  |  |  |
| 152 MHz ≤ f ≤ 262 MHz | -36 dBm+TT | 100 kHz |  |
| 1001 MHz ≤ f ≤ 1086 MHz 2674 MHz ≤ f ≤ 2759 MHz 2851 MHz ≤ f ≤ 2996 MHz 3498 MHz ≤ f ≤ 3608 MHz 4524 MHz ≤ f ≤ 4669 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n2A-n48A Configuration |  |  |  |
| 9 kHz ≤ f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz ≤ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f < 270 MHz | -36 dBm | 100 kHz |  |
| 1640 MHz ≤ f ≤ 1850 MHz 5190 MHz ≤ f ≤ 5610 MHz 7250 MHz ≤ f ≤ 7520 MHz 8950 MHz ≤ f ≤ 9310 MHz | -30 dBm | 1 MHz |  |
| Test requirements for CA_n3A-n78A Configuration |  |  |  |
| 270 MHz ≤ f ≤ 380 MHz | -36 dBm+TT | 100 kHz |  |
| 1515MHz ≤ f ≤ 2090 MHz 4815 MHz ≤ f ≤ 5890 MHz 6720 MHz ≤ f ≤ 7370 MHz 8310 MHz ≤ f ≤ 9385 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n8A-n78A Configuration |  |  |  |
| 1470 MHz ≤ f ≤ 2040 MHz 2385 MHz ≤ f ≤ 2920 MHz 4180 MHz ≤ f ≤4715 MHz 5060 MHz ≤ f ≤5630 MHz 5685 MHz ≤ f ≤6720 MHz 7480 MHz ≤ f ≤8515 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n24A-n41A Configuration |  |  |  |
| 563 MHz ≤ f ≤ 825 MHz 835.5 MHz ≤ f ≤ 1000 MHz | -36 dBm+TT | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1063.5 MHz 3331.5 MHz ≤ f ≤ 3753.5 MHz 4122.5 MHz ≤ f ≤ 4350.5 MHz 5749 MHz ≤ f ≤ 6011 MHz 6618.5 MHz ≤ f ≤ 7040.5 MHz | -25 dBm+TT | 1 MHz | NS-01 signalled for n41 |
|  | -30 dBm+TT | 1 MHz | NS-04 signalled for n41 |
| Test requirements for CA_n24A-n48A Configuration |  |  |  |
| 229 MHz ≤ f ≤ 447 MHz | -36 dBm+TT | 100 kHz |  |
| 1889.5 MHz ≤ f ≤ 2073.5 MHz 5176.5 MHz ≤ f ≤ 5360.5 MHz 5439.5 MHz ≤ f ≤ 5773.5 MHz 6803 MHz ≤ f ≤ 7021 MHz 8726.5 MHz ≤ f ≤ 9060.5 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n24A-n77A Configuration |  |  |  |
| 21 MHz ≤ f < 30 MHz | -36 dBm+TT | 10 kHz |  |
| 30 MHz ≤ f < 947 MHz | -36 dBm+TT | 100 kHz |  |
| 1639.5 MHz ≤ f ≤ 2573.5 MHz 4926.5 MHz ≤ f ≤ 7521 MHz 8226.5 MHz ≤ f ≤ 10060.5 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n26A-n66A Configuration |  |  |  |
| 12 MHz ≤ f < 30 MHz | -36 dBm+TT | 10 kHz |  |
| 30 MHz ≤ f ≤ 152 MHz 861 MHz ≤ f ≤ 966 MHz | -36 dBm+TT | 100 kHz |  |
| 2524 MHz ≤ f ≤ 2746 MHz 3338 MHz ≤ f ≤ 3478 MHz 4234 MHz ≤ f ≤ 4409 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n26A-n70A Configuration |  |  |  |
| 3 MHz ≤ f < 30 MHz | -36 dBm+TT | 10 kHz |  |
| 30 MHz ≤ f < 82 MHz | -36 dBm+TT | 100 kHz |  |
| 846 MHz ≤ f ≤ 896 MHz | -36 dBm+TT | 100 kHz |  |
| 2509 MHz ≤ f ≤ 2606 MHz 3323 MHz ≤ f ≤ 3408 MHz 4204 MHz ≤ f ≤ 4269 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n28A-n41A Configuration |  |  |  |
| 1000MHz ≤ f ≤ 1284 MHz 1748MHz ≤ f ≤ 1987 MHz 3199MHz ≤ f ≤ 3438 MHz 3902MHz ≤ f ≤ 4186 MHz 4244MHz ≤ f ≤ 4677 MHz 5695MHz ≤ f ≤ 6128MHz | -25 dBm+TT | 1MHz |  |
| Test requirements for CA_n41A-n79A Configuration |  |  |  |
| 8 MHz ≤ f ≤30 MHz | -36 dBm+TT | 10kHz |  |
| 30 MHz ≤ f ≤980 MHz | -36 dBm+TT | 100kHz |  |
| 1710MHz ≤ f ≤ 2504MHz | -30 dBm+TT | 1MHz |  |
| 6110MHz ≤ f ≤ 7690MHz |  |  |  |
| 9392MHz ≤ f ≤ 10380MHz |  |  |  |
| 11296MHz ≤ f ≤ 12690MHz |  |  |  |
| Test requirements for CA_n48A-n66A Configuration |  |  |  |
| 10 MHz ≤ f < 30 MHz | -36 dBm+TT | 10 kHz |  |
| 30 MHz ≤ f < 280 MHz | -36 dBm+TT | 100 kHz |  |
| 1770 MHz ≤ f ≤ 1990 MHz 5260 MHz ≤ f ≤ 5690 MHz 6970 MHz ≤ f ≤ 7260 MHz 8810 MHz ≤ f ≤ 9180 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n48A-n70A Configuration |  |  |  |
| 130 MHz ≤ f ≤ 310 MHz | -36 dBm+TT | 100 kHz |  |
| 1840 MHz ≤ f ≤ 2005 MHz 5245 MHz ≤ f ≤ 5705 MHz 6940 MHz ≤ f ≤ 7120 MHz 8795 MHz ≤ f ≤ 9110 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n48A-n71A Configuration |  |  |  |
| 2154 MHz ≤ f ≤ 2374 MHz 2852 MHz ≤ f ≤ 3037 MHz 4213 MHz ≤ f ≤ 4398 MHz 4876 MHz ≤ f ≤ 5096 MHz 6402 MHz ≤ f ≤ 6737 MHz 7763 MHz ≤ f ≤ 8098 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n66A-n71A Configuration |  |  |  |
| 314 MHz ≤ f < 454 MHz | -36 dBm+TT | 100 kHz |  |
| 1012 MHz ≤ f ≤ 1117 MHz 2373 MHz ≤ f ≤ 2478 MHz 2722 MHz ≤ f ≤ 2897 MHz 3036 MHz ≤ f ≤ 3176 MHz 4083 MHz ≤ f ≤ 4258 MHz | -30 dBm+TT | 1 MHz |  |
| Test requirements for CA_n70A-n71A Configuration |  |  |  |
| 299 MHz ≤ f < 384 MHz 997 MHz ≤ f < 1000 MHz | -36 dBm+TT | 100 kHz |  |
| 1000 MHz ≤ f ≤ 1047 MHz 2358 MHz ≤ f ≤ 2408 MHz 2692 MHz ≤ f ≤ 2757 MHz 3021 MHz ≤ f ≤ 3106 MHz 4053 MHz ≤ f ≤ 4118 MHz | -30 dBm+TT | 1 MHz |  |

Table 6.5A.3.1.1.5-2: Test Tolerance for General spurious emissions

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0 | 0 |
| 40MHz < BW ≤ 100MHz | 0 | 0 |

Table 6.5A.3.1.1.5-3: General spurious emissions test requirements for intra-band contiguous UL CA

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz | 4 |
|  | -25 dBm | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band for which the upper frequency edge of the UL Band is greater than 2.55 GHz and less than or equal to 5.2 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz NOTE 3: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in sub-clause 5.2B of [4] when NS_04 is signalled. NOTE 4: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in subclause 5.2B of TS 38.101-3 [4] when NS_04 is signalled. |  |  |  |

< End of Changes >