3GPP TSG-RAN5 Meeting #98	R5-231657

# Athens, Greece, 27 February – 3 March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2067 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding UE maximum output power reduction for new NR bands n91, n92, n93, n94 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Missing UE maximum output power reduction for NR bands n91, n91, n93, n94. CR 1906 (R5-227920) was agreed in RAN5#97 but not implemented in 38.521-1 V17.7.0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UE maximum output power reduction requirements for NR bands n91, n91, n93, n94 into Table 6.2.2.5-1 and Table 6.2.2.5-6. Include 0.3 dB margin as Note 4 in Table 6.2.2.5-1 to be in line with core specification output power relaxation. Note 4 applies to test IDs 5, 9 and 39 with MPR = 0 dB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE maximum output power reduction requirements for NR bands n91, n91, n93, n94 will not be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Correct band numbers to n91, n92, n93, n94 in the cover sheet title |  |  |  |  |  |  |  |  |

-------- Start of changes --------

6.2.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1 to Table 6.2.2.5-9a.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1 and Table 6.2.2.5-3.

Table 6.2.2.5-1a: UE Power Class test requirements Band 24 for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | 25.0 + TT | 19.0 - TT | （17.5 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 – TT | （15.5 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 - TT | （15.5 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | 25.0 + TT | 17.5 - TT | （15.5 - TT2） |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | 25.0 + TT | 18.5 - TT | （17.0 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | 25.0 + TT | 18.0 - TT | （16.0 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 – TT | （14.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 – TT | （14.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | 25.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 3 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 3 | 25.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23 | （21.52） | 2.0 |  | 3 | 25.0 + TT | 20.0 - TT | （18.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-1: UE Power Class test requirements(for Bands n1, n2, n3, n5, n7, n8, n12, n14, n20, n25, n26, n30, n34, n38, n39, n40, n41, n50, n51, n53, n65, n66, n70, n74, n91, n92, n93, n94) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | （1.52） | 25.8 | （24.32） | 2.0 |  | 2 | 28.0 + TT | 23.8 - TT | （22.3 - TT2） |
| 2 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 3 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 4 | 23 | -3 | 1.2 | 0 | （1.52） | 24.8 | （23.32） | 2.0 |  | 2 | 28.0 + TT | 22.8 - TT | （21.3 - TT2） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 2 | 25.0 + TT | 19.5 - TT | （17.5 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | 25.0 + TT | 20.5 - TT | (19.0 - TT2) |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23 | （21.52） | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT4 | （19.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n25 n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: For bands n91, n92, n93 and n94 the maximum power reduction requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

Table 6.2.2.5-6: UE Power Class test requirements (for Bands n1, n2, n3, n5, n7, n8, n12, n14, n20, n25, n26, n30, n34, n38, n39, n40, n41, n50, n51, n53, n65, n66, n70, n74, n91, n92, n93, n94) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 23 | 0 | 1.5 | 1.5 | 0 | 1.52 | 20 | 18.52 | 2.5 | 42 | 2 | 25.0 + TT | 17.5 - TT | 14.5 - TT2 |
| 15 | 23 | 0 | 1.5 | 1 | 0 | 1.52 | 20.5 | 192 | 2.5 | 3.52 | 2 | 25.0 + TT | 18 - TT | 15.5 - TT2 |
| 26 | 23 | 0 | 3 | 1.5 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 27 | 23 | 0 | 3 | 1 | 0 | 1.52 | 19 | 17.52 | 3.5 | 52 | 2 | 25.0 + TT | 15.5 - TT | 12.5 - TT2 |
| 34 | 23 | 0 | 2 | 1.5 | 0 | 1.52 | 19.5 | 182 | 3.5 | 42 | 2 | 25.0 + TT | 16 - TT | 14 - TT2 |
| 35 | 23 | 0 | 2 | 1 | 0 | 1.52 | 20 | 18.52 | 2.5 | 42 | 2 | 25.0 + TT | 17.5 - TT | 14.5 - TT2 |
| 46 | 23 | 0 | 3 | 1.5 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 47 | 23 | 0 | 3 | 1 | 0 | 1.52 | 19 | 17.52 | 3.5 | 52 | 2 | 25.0 + TT | 15.5 - TT | 12.5 - TT2 |
| 56 | 23 | 0 | 3.5 | 1.5 | 0 | 1.52 | 18 | 16.52 | 4 | 52 | 2 | 25.0 + TT | 14 - TT | 11.5 - TT2 |
| 57 | 23 | 0 | 3.5 | 1 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 66 | 23 | 0 | 6.5 | 1.5 | 0 | 1.52 | 15 | 13.52 | 5 | 52 | 2 | 25.0 + TT | 10 - TT | 8.5- TT2 |
| 67 | 23 | 0 | 6.5 | 1 | 0 | 1.52 | 15.5 | 142 | 5 | 52 | 2 | 25.0 + TT | 10.5 - TT | 9 - TT2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n25, n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- End of changes --------