3GPP TSG-RAN5 Meeting #98	R5-231918

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0326 | rev | 1 | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional TC for location based functional alias |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2023-03-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There was no test case for the Rel-16 location based functional alias feature |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 5.9 for the location based functional alias feature |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MCPTT clients that support the location based functional alias feature may not operate properly in commerical markets |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This will trigger a release upgrade of the spec |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Final version of TDoc number R5-230773 |  |  |  |  |  |  |  |  |

<START OF CHANGES>

## 5.9	Configuration / Functional Alias / Location based functional alias status change

5.9.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorised for MCPTT Service }

ensure that {

when { UE enters into or exits from a location area that triggers one or more functional aliases to be activated according to the MCPTT user profile document }

then { sends a SIP PUBLISH message to activate a functional alias }

}

(2)

with { UE (MCPTT Client) registered and authorised for MCPTT Service and having already activated a functional alias }

ensure that {

when { UE enters into or exits from a location area that triggers one or more functional aliases to be deactivated according to the MCPTT user profile document }

then { UE (MCPTT Client) sends a SIP PUBLISH message to deactivate a functional alias }

}

5.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.379 clause 9A.2.1.2, 9A.2.1.4. Unless otherwise stated these are Rel-16 requirements.

[TS 24.379, clause 9A.2.1.2]

In order:

-	to indicate that an MCPTT user requests to activate one or more functional aliases;

-	to indicate that the MCPTT user requests to deactivate one or more functional aliases;

-	to refresh indication of an MCPTT user interest in one or more functional aliases due to near expiration of the expiration time of a functional alias with the status set to the "activated" state received in a SIP NOTIFY request in clause 9A.2.1.3;

-	to indicate that the MCPTT client entering into or exiting from a location area triggers one or more functional aliases to be activated;

-	to indicate that the MCPTT client entering into or exiting from a location area triggers one or more functional aliases to be deactivated; or

-	any combination of the above;

the MCPTT client shall generate a SIP PUBLISH request according to 3GPP TS 24.229 [4], IETF RFC 3903 [37], and IETF RFC 3856 [51].

When the MCPTT user requests to deactivate a functional alias, the MCPTT client shall first check the <manual-deactivation-not-allowed-if-location-criteria-met> element within the <anyExt> element of the <entry> element corresponding to the functional alias within the <FunctionalAliasList> list element of the <anyExt> element of the <OnNetwork> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.484 [50]). If the functional alias has been activated due to a location area trigger and the <manual-deactivation-not-allowed-if-location-criteria-met> element is set to a value of "true", the MCPTT client shall suppress the MCPTT user's request.

NOTE 1:	If the request is suppressed, a notification message can be displayed to the user.

In the SIP PUBLISH request, the MCPTT client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

2)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body. In the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT client shall include the <mcptt-request-uri> element set to the MCPTT ID of the MCPTT user;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9];

4)	if the MCPTT client requests to activate one or more functional aliases, shall set the Expires header field according to IETF RFC 3903 [37], to 4294967295;

NOTE 2:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

5)	if the MCPTT client requests to deactivate one or more functional aliases, shall set the Expires header field according to IETF RFC 3903 [37], to zero; and

NOTE 3:	Activation and deactivation of functional alias cannot be performed with the same PUBLISH request.

6)	shall include an application/pidf+xml MIME body indicating per-user functional alias information according to clause 9A.3.1. In the MIME body, the MCPTT client:

a)	shall include all functional aliases where the MCPTT user requests activation for the MCPTT ID;

b)	shall include the MCPTT client ID of the targeted MCPTT client;

c)	shall not include the "status" attribute and the "expires" attribute in the <functionalalias> element;

d)	if the MCPTT client has received an indication that take over of a functional alias is possible and intends to take over a functional alias, shall include a <take-over> child element set to "true"; and

e)	shall set the <p-id-fa> child element of the <presence> root element to a globally unique value.

The MCPTT client shall send the SIP PUBLISH request according to 3GPP TS 24.229 [4].

[TS 24.379, clause 9A.2.1.4]

If a location criterion for functional alias activation or de-activation is met, the MCPTT client shall initiate the functional alias status change procedure as specified in clause 9A.2.1.2.

5.9.3	Test description

5.9.3.1		Pre-test conditions

System Simulator:

-	SS (MCPTT server)

-	E-UTRA related parameters are set to the default parameters for the basic single cell environment, as defined in TS 36.508 [20] clause 4.4.

-	GNSS simulator to simulate a location.

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10, is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in TS 36.579-1 [2], subclause 5.4.2.

-	The MCPTT User performs the Generic Test Procedure for MCPTT Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCPTT Client Application has been activated and User has registered-in as the MCPTT User with the Server as active user at the Client.

5.9.3.2	Test procedure sequence

Table 5.9.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger the UE to reset UTC time and location. NOTE: The UTC time and location reset may be performed by MMI or AT command (+CUTCR). | - | - | - | - |
| 2 | Trigger the GNSS simulator to start step 2 of Scenario #4 to simulate the UE moving to location #7 inside Geographical area #1 as defined in TS 36.508 [24] Table 4.11.2-2. | - | - | - | - |
| 3 | Wait 10 sec to allow the simulated location to move approximately 100 m from the original location to location #7. | - | - | - | - |
| 4-6 | Check: Does the UE (MCPTT client) correctly perform test procedure 'UE initiated MCPTT functional alias status change' as described in TS 36.579-1 [2] Table 5.3A.10.3-1 from step 2a1 through step 4 to change the status of a functional alias to "activated"? NOTE: The UE (MCPTT client) sends a SIP PUBLISH message and the SS responds with a SIP 200(OK) message and then the RRC connection is released. | - | - | 1 | P |
| 7 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. (NOTE 1) | - | - | - | - |
| 8 | Trigger the GNSS simulator to start step 4 of Scenario #4 to simulate the UE moving to location #8 inside Geographical area #1 as defined in TS 36.508 [24] Table 4.11.2-2. | - | - | - | - |
| 9 | Wait 10 sec to allow the simulated location to move approximately 100 m from the original location to location #8. | - | - | - | - |
| 10-12 | Check: Does the UE (MCPTT client) correctly perform test procedure 'UE initiated MCPTT functional alias status change' as described in TS 36.579-1 [2] Table 5.3A.10.3-1 from step 2a1 through step 4 to change the status of a functional alias to "not activated"? NOTE: The UE (MCPTT client) sends a SIP PUBLISH message and the SS responds with a SIP 200(OK) message and then the RRC connection is released. | - | - | 2 | P |
| 13 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. (NOTE 1) | - | - | - | - |
| NOTE 1: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.9.3.3	Specific message contents

Table 5.9.3.3-1: SIP PUBLISH from the UE (step 11, Table 5.9.3.2-1;
step 3, TS 36.579-1 [2] Table 5.3A.10.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.3A.10.4-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  | TS 24.379 [9] clause 9A2.1.2 |  |
| value | "0" |  |  |  |

<END OF CHANGES>