3GPP TSG-RAN WG5 Meeting #98	R5-230044r1

Athens, Greece, 27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0402 | rev | 1 | Current version: | 16.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR PRS-based measurement requirements for NR RSTD and PRS-RSRP test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The TT analysis results for NR RSTD and PRS-RSRP test cases are not yet added to Annex C |  |  |  |  |  |  |  |  |
| Summary of change: |  | The TT analysis results have been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR RSTD and PRS-RSRP test cases will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Removed information of test case 16.3.2. |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

C.1.6	NR PRS-based measurement requirements

Table C.1.6-1: Maximum Test System Uncertainty for NR PRS-based Measurement Requirements

| Clause |  | Maximum Test System Uncertainty |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- | --- |
| 14.2.1 NR RSTD measurement period test case for single positioning frequency layer in FR1 SA |  | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig PRS Ês3 / Noc ±0.3 dB averaged over BWConfig Ês3 / Noc ±0.3 dB averaged over BWConfig Response Time = 300ms |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN PRS Ês3 / Noc and Ês3 / Noc are the ratios of cell 3 signal / AWGN Signal-to-noise ratio uncertainty ±0.3 dB |  |
| 14.2.2 NR RSTD measurement period test case for dual positioning frequency layers in FR1 SA |  | Same as 14.2.1 |  |  |  |
| 14.2.3 NR RSTD measurement period test case for single positioning frequency layer in FR2 SA |  | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig PRS Ês3 / Noc ±0.3 dB averaged over BWConfig Ês3 / Noc ±0.3 dB averaged over BWConfig Response Time = FFS |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN PRS Ês3 / Noc and Ês3 / Noc are the ratios of cell 3 signal / AWGN Signal-to-noise ratio uncertainty ±0.3 dB |  |
| 14.2.4 NR RSTD measurement period test case for dual positioning frequency layers in FR2 SA |  | Same as 14.2.3 |  |  |  |
| 14.3.1 NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA |  | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = 32 Tc |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |  |
| 14.3.2 NR RSTD measurement accuracy test case for dual positioning frequency layers in FR1 SA |  | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = FFS |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |  |
| 14.3.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA |  | Same as 14.3.1 |  |  |  |
| 14.3.4 NR RSTD measurement accuracy test case for dual positioning frequency layer in FR2 SA |  | Same as 14.3.2 |  |  |  |
| 15.2.1 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR1 SA |  | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Response Time = ± 300 ms |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN Tc = 1/(480000 x 4096), the basic timing unit defined in TS 38.211 [53] |  |
| 15.2.2 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR1 SA |  | Same as 15.2.1 |  | Same as 15.2.1 |  |
| 15.2.3 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA |  | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Response Time = ± 300 ms |  | PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN Tc = 1/(480000 x 4096), the basic timing unit defined in TS 38.211 [53] |  |
| 15.2.4 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA |  | Same as 15.2.3 |  | Same as 15.2.3 |  |
| 15.3.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA |  | Noc ±1.5 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = 32 Tc |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |  |
| 15.3.2 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA |  | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Cell Timing Difference = 32 Tc |  | Note: PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN |  |
| 16.2.1 PRS-RSRP measurement period test case for single positioning frequency layer in FR1 SA |  | FFS |  | FFS |  |
| 16.2.2 PRS-RSRP measurement period test case for dual positioning frequency layer in FR1 SA |  | FFS |  | FFS |  |
| 16.2.3 PRS-RSRP measurement period test case for single positioning frequency layer in FR2 SA |  | Noc ±5.65 dB averaged over BWConfig PRS Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig PRS Ês2 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig Response Time = ± 300 ms |  | PRS Ês1 / Noc and Ês1 / Noc are the ratios of cell 1 signal / AWGN PRS Ês2 / Noc and Ês2 / Noc are the ratios of cell 2 signal / AWGN Tc = 1/(480000 x 4096), the basic timing unit defined in TS 38.211 [53] |  |
| 16.2.4 PRS-RSRP measurement period test case for dual positioning frequency layer in FR2 SA |  | Same as 16.2.3 |  | Same as 16.2.3 |  |
| 16.3.1 PRS-RSRP measurement accuracy with PRS in FR1 |  | FFS |  | FFS |  |

< Unchanged sections omitted >

C.2.5	NR PRS-based Measurement requirements

Table C.2.5-1: Test Parameter Relaxations for NR PRS-based Measurement requirements

| Clause | Test Parameter Relaxation |  |
| --- | --- | --- |
| 14.2.1 NR RSTD measurement period test case for single positioning frequency layer in FR1 SA | Response time | 300ms |
| 14.2.2 NR RSTD measurement period test case for dual positioning frequency layers in FR1 SA | Response time | 300ms |
| 14.2.3 NR RSTD measurement period test case for single positioning frequency layer in FR2 SA | Response time | FFS |
| 14.2.4 NR RSTD measurement period test case for dual positioning frequency layers in FR2 SA | Response time | FFS |
| 14.3.1 NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA | Cell Timing Difference | 32 Tc |
| 14.3.2 NR RSTD measurement accuracy test case for dual positioning frequency layers in FR1 SA | Cell Timing Difference | 32 Tc |
| 14.3.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Same as 14.3.1 | Same as 14.3.1 |
| 14.3.4 NR RSTD measurement accuracy test case for dual positioning frequency layer in FR2 SA | Same as 14.3.2 | Same as 14.3.2 |
| 15.2.1 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR1 SA | Response time | 300ms |
| 15.2.2 UE Rx-Tx time difference measurement period for dual positioning frequency layer in FR1 SA | Response time | 300ms |
| 15.2.3 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA | Response time | 300ms |
| 15.2.4 UE Rx-Tx time difference measurement period for dual positioning frequency layer in FR2 SA | Response time | 300ms |
| 15.3.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA | Cell Timing Difference | 32 Tc |
| 15.3.2 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA | Cell Timing Difference | 32 Tc |
| 16.2.1 PRS-RSRP measurement period test case for single positioning frequency layer in FR1 SA | Response time | 300ms |
| 16.2.2 PRS-RSRP measurement period test case for dual positioning frequency layer in FR1 SA | Response time | 300ms |
| 16.2.3 PRS-RSRP measurement period test case for single positioning frequency layer in FR2 SA | Response time | 300ms |
| 16.2.4 PRS-RSRP measurement period test case for dual positioning frequency layer in FR2 SA | Response time | 300ms |
| 16.3.1 PRS-RSRP measurement accuracy with PRS in FR1 |  |  |

< Unchanged sections omitted >

Table C.4.5: Derivation of Test Requirements for NR RSTD Measurement tests

| Test | Minimum Requirement in TS 38.133 | Test Parameter Relaxation (TPR) | Test Requirement in TS 37.571-1 |
| --- | --- | --- | --- |
| 14.2.1 NR RSTD measurement period test case for single positioning frequency layer in FR1 SA | Response Time between 1 s and 660 s depending on UE capabilities | 300 ms | Time + TPR: between 1.3 s and 660.3 s depending on UE capabilities |
| 14.2.2 NR RSTD measurement period test case for dual positioning frequency layers in FR1 SA | Response Time between 2 s and 660 s depending on UE capabilities | 300 ms | Time + TPR: between 2.3 s and 660.3 s depending on UE capabilities |
| 14.2.3 NR RSTD measurement period test case for single positioning frequency layer in FR2 SA | Response Time between 6 s and 250 s depending on UE capabilities | 300 ms | Time + TPR: between 6.3 s and 250.3 s depending on UE capabilities |
| 14.2.4 NR RSTD measurement period test case for dual positioning frequency layers in FR2 SA | Response Time between 11 s and 210 s depending on UE capabilities | 300 ms | Time + TPR: between 11.3 s and 210.3 s depending on UE capabilities |
| 14.3.1 NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |
| 14.3.2 NR RSTD measurement accuracy test case for dual positioning frequency layers in FR1 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |
| 14.3.3 NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |
| 14.3.4 NR RSTD measurement accuracy test case for dual positioning frequency layer in FR2 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |

Table C.4.6: Derivation of Test Requirements for UE Rx–Tx time difference Measurement tests

| Test | Minimum Requirement in TS 38.133 | Test Parameter Relaxation (TPR) | Test Requirement in TS 37.571-1 |
| --- | --- | --- | --- |
| 15.2.1 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR1 SA | Response Time = 11s | 300ms | Time + TPR: 11.3s |
| 15.2.2 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR1 SA | Response Time = 20s | 300ms | Time + TPR: 20.3s |
| 15.2.3 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA | Response Time between 6 s and 170 s depending on UE capabilities | 300ms | Time + TPR: between 6.3 s and 170.3 s depending on UE capabilities |
| 15.2.4 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA | Response Time between 11 s and 84 s depending on UE capabilities | 300ms | Time + TPR: between 11.3 s and 84.3 s depending on UE capabilities |
| 15.3.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |
| 15.3.2 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA | Cell Timing Difference = 3µs | 32 Tc | Time + TPR: 3µs ± 32 Tc |

Table C.4.7: Derivation of Test Requirements for PRS-RSRP Measurement tests

| Test | Minimum Requirement in TS 38.133 | Test Parameter Relaxation (TPR) | Test Requirement in TS 37.571-1 |
| --- | --- | --- | --- |
| 16.2.1 PRS-RSRP measurement period test case for single positioning frequency layer in FR1 SA | FFS | FFS | FFS |
| 16.2.2 PRS-RSRP measurement period test case for dual positioning frequency layer in FR1 SA | FFS | FFS | FFS |
| 16.2.3 PRS-RSRP measurement period test case for single positioning frequency layer in FR2 SA | Response Time between 6 s and 170 s depending on UE capabilities | 300ms | Time + TPR: between 6.3 s and 170.3 s depending on UE capabilities |
| 16.2.4 PRS-RSRP measurement period test case for dual positioning frequency layer in FR2 SA | Response Time between 11 s and 84 s depending on UE capabilities | 300ms | Time + TPR: between 11.3 s and 84.3 s depending on UE capabilities |
| 16.3.1 PRS-RSRP measurement accuracy with PRS in FR1 | FFS | FFS | FFS |