3GPP TSG-RAN5 Meeting #98	R5-230052r1

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3430 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new NR Multi-SIM test case 8.1.2.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add new NR Multi-SIM test case 8.1.2.1.6 for RRC reconfiguration/ MUSIM / MUSIM- gap / Addition / Modification / Release |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case 8.1.2.1.6 for NR Multi-SIM devices is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.6(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ..38.523-2. CR .0325.. |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1:change the typo and specific IE definition as TF160 expert suggested. |  |  |  |  |  |  |  |  |

8.1.2.1.6	RRC reconfiguration/ MUSIM / MUSIM- gap / Addition / Modification / Release

8.1.2.1.6.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state}

ensure that {

when { UE receives the RRCReconfiguration message includes the musim-GapConfig with periodic musim-GapID involved in musim-GapToAddModList and the gap configuration is active in SS1 }

then { UE stops all interworking with SS1 during the configured gaps periodically }

}

(2)

with { UE in NR RRC_CONNECTED state}

ensure that {

when { UE receives the RRCReconfiguration message includes the musim-GapConfig with periodic musim-GapID involved in musim-GapToAddModList and an entry is matched with musim-GapID that exists in the musim-GapConfigLis in SS1}

then { UE stops all interworking with SS1 during the replaced new configured gaps periodically }

}

(3)

with { UE in NR RRC_CONNECTED state}

ensure that {

when { UE receives the RRCReconfiguration message includes the musim-GapConfig with periodic musim-GapID involved in musim-GapToReleaseList in SS1}

then { UE performs normal interworking with SS1 during the released gaps periodically }

}

(4)

with { UE in NR RRC_CONNECTED state}

ensure that {

when { UE receives the RRCReconfiguration message includes the musim-GapConfig with musim-AperiodicGap-r17}

then { UE stops all interworking with SS1 temporarily with configuration MUSIM-GapInfo-r17 }

}

8.1.2.1.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clauses  5.3.5.3, 5.3.5.9a and TS 38.133 clause 9.1.10. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO, CPA or CPC):

…

1>	if the RRCReconfiguration message includes the musim-GapConfig:

2>	perform the MUSIM gap configuration procedure as specified in 5.3.5.9a;

[TS 38.331, clause 5.3.5.9a]

The UE shall:

1>	if musim-GapConfig is set to setup:

2>	for each musim-GapId included in the received musim-GapToReleaseList:

3>	release the periodic MUSIM gap configuration associated with the musim-GapId;

2>	for each MUSIM-Gap included in the received musim-GapToAddModList:

3>	setup periodic MUSIM gap configuration indicated by the MUSIM-Gap in accordance with the received musim-GapRepetitionAndOffset (providing musim-GapRepetition and Offset value for the following condition) i.e. the first subframe of each periodic MUSIM gap occurs at an SFN and subframe of the NR PCell meeting the following condition:

SFN mod T = FLOOR(Offset/10);

subframe = Offset mod 10;

with T = musim-GapRepetition/10;

2>	if musim-AperiodicGap is included:

3>	setup aperiodic MUSIM gap configuration indicated by the musim-AperiodicGap in accordance with the received musim-Starting-SFN-AndSubframe, i.e. the first subframe of aperiodic MUSIM gap occurs at an SFN and subframe of the NR PCell meeting the following condition:

SFN = starting-SFN;

subframe = startingSubframe;

1>	else if musim-GapConfig is set to release:

[TS 38.133, clause 9.1.10]

If the UE requires gap patterns for MUSIM purpose, such as cell identification and measurement, paging monitoring, SIB acquisition, and/or on-demand SI request of the target cell in the target network, then the network may provide one or more per-UE MUSIM gap pattern(s) for concurrent monitoring of all frequency layers for MUSIM via MUSIM-GapConfig [2]. The UE can be configured with no more than three periodic MUSIM gap patterns and/or one aperiodic MUSIM gap pattern for MUSIM via MUSIM-GapConfig [2]. The MUSIM gap patterns specified in Table 9.1.10-1 are applicable only for MUSIM operation.

The UE is not required to perform cell identification and measurement, paging monitoring, SIB acquisition, and/or on-demand SI request of the target cell in the target network that is outside the MUSIM gaps.

UE supporting MUSIM capability shall support the MUSIM gap patterns listed in Table 9.1.10-1 based on UE’s capability specified in TS38.306[14] and the applicability specified in Table 9.1.10-2.

UE determines MUSIM gap timing based on gap offset configuration from serving cell provided by higher layer signalling as specified in TS 38.331 [2].

Table 9.1.10-1: MUSIM Gap Pattern Configurations

| MUSIM Gap Pattern Id | MUSIM Gap Length (MGL, ms) | MUSIM Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 10 | 80 |
| 13 | 20 | 160 |
| 14 | 6 | 320 |
| 15 | 6 | 640 |
| 16 | 6 | 1280 |
| 17 | 6 | 2560 |
| 18 | 10 | 320 |
| 19 | 10 | 640 |
| 20 | 10 | 1280 |
| 21 | 10 | 2560 |
| 22 | 20 | 320 |
| 23 | 20 | 640 |
| 24 | 20 | 1280 |
| 25 | 20 | 2560 |
| 26 | 20 | 5120 |
| 27 | 10 | NA |
| 28 | 20 | NA |
| Note 1: Measurement gap pattern #27, #28 are the aperiodic gap pattern without MGRP. |  |  |

Table 9.1.10-2: Applicability for MUSIM Gap Pattern Configurations supported by the UE with NR standalone operation (with single carrier, NR CA configuration)

| MUSIM gap pattern configuration | Serving cell | Gap Purpose | Applicable MUSIM Gap Pattern Id |
| --- | --- | --- | --- |
| Per-UE | FR1, FR2, or | MUSIM Note1 | 0-13, 14-26, 27, 28 |
| MUSIM gap | FR1 + FR2 |  |  |
| NOTE 1: Inclusion of MUSIM procedures for per-UE MUSIM gaps only in NR single carrier, NR CA mode: MUSIM purpose which includes cell identification and measurement, paging monitoring, SIB acquisition, and/or on-demand SI request of the target cell in the target network. |  |  |  |

8.1.2.1.6.3	Test description

8.1.2.1.6.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1(home PLMN1) and NR Cell 12 (home PLMN2) are configured according to table 4.4.2-3 in TS 38.508-1 [18] and belong to the different frequency.

-  NR Cell 1 (belongs to TAI-1, home PLMN1, MCC=001,MNC=01) is set to ''Serving cell''.

-  NR Cell 12 (belongs to TAI-3, home PLMN2, MCC=001,MNC=02) is set to '' Serving cell''.

UE:

- 	the UE is a MUSIM UE;

-    the UE is equipped with two USIMs with configuration as defined in TS 38.508-1 [4] Tables 6.4.1-27 and 6.4.1-28.

Preamble:

-	The UE performs a successful registration for MUSIM according to Table 4.9.36.2.2-1 of TS 38.508-1 [4].

8.1.2.1.6.3.2	Test procedure sequence

Table 8.1.2.1.6.3.2-1 illustrates a paging message to be sent for NR Cell 1 at various time instants of the test execution.

Table 8.1.2.1.6.3.2-1: Time instances of sending a paging message in conducted test environment

|  | Remark | Comment |
| --- | --- | --- |
| T0 | SFN mod T = FLOOR(Offset/10); subframe = Offset mod 10; with T = musim-GapRepetition/10; T0 = SFN + subframe + 5ms | When musim-GapLength-r17="ms6" in step 3, T0 is in the gap; When musim-GapLength-r17="ms4" in step 8, T0 is outside the gap. |
| T1 | SFN mod T = FLOOR(Offset/10); subframe = Offset mod 10; with T = musim-GapRepetition/10; T1 = SFN + subframe + 8ms | T1 is always outside the gap in Step 6,10,14. |
| T2 | SFN mod T = FLOOR(Offset/10); subframe = Offset mod 10; with T = musim-GapRepetition/10; T2 = SFN + subframe + 2ms | If the gap is not released successfully, T2 is in the gap when musim-GapLength-r17="ms4" in step 12. |
| T3 | SFN = starting-SFN; subframe = startingSubframe; T3 = SFN + subframe + 2ms | T3 is in the gap when musim-GapLength-r17="ms4" in step 16. |

Table 8.1.2.1.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-8 | The USIM A of UE performs the generic procedure by executing steps 1 to 8 to enter the NR RRC_CONNECTED state as described in TS 38.508-1 [4] table 4.5.4.2-3 | - | - | - | - |
| 9 | The SS transmits an RRCReconfiguration message to configure musim-GapLength-r17 = ‘ms6’ with musim-GapToAddModList-r17 IE to USIM A. | <-- | NR RRC: RRCReconfiguration | - | - |
| 10 | USIM A of the UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 11 | The SS sends UECapabilityEnquiry message to the USIM A at T0. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 12 | Check: Does the UE send a UECapabilityInformation message? | --> | NR RRC: UECapabilityInformation | 1 | F |
| 13 | The SS transmits an RRCReconfiguration message to configure musim-GapLength-r17 = ‘ms4’ with musim-GapToAddModList-r17 IE to USIM A at T1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 14 | USIM A of the UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 15 | The SS sends UECapabilityEnquiry message to the USIM A at T0. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 16 | Check: Does the UE send a UECapabilityInformation message? | --> | NR RRC: UECapabilityInformation | 2 | P |
| 17 | The SS transmits an RRCReconfiguration message to USIM A to configure musim-GapId-r17 = ‘0’ with musim-GapToReleaseList-r17 IE to USIM A at T1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 18 | USIM A of the UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 19 | The SS sends UECapabilityEnquiry message to the USIM A at T2. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 20 | Check: Does the UE send a UECapabilityInformation message? | --> | NR RRC: UECapabilityInformation | 3 | P |
| 21 | The SS transmits an RRCReconfiguration message to USIM A to configure starting-SFN-r17 = ‘0’, startingSubframe-r17 = ‘0’ and musim-GapLength-r17 = ‘ms4’ with musim-AperiodicGap-r17 IE to USIM A at T1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 22 | USIM A of the UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 23 | The SS sends UECapabilityEnquiry message to the USIM A at T3. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 24 | Check: Does the UE send a UECapabilityInformation message? | --> | NR RRC: UECapabilityInformation | 4 | F |

8.1.2.1.6.3.3	Specific message contents

Table 8.1.2.1.6.3.3-1: RRCReconfiguration (step 9, step 13, step 17,step 21,Table 8.1.2.1.6.3.2-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| musim-GapConfig-r17 SetupRelease { |  |  |  |
| setup | MUSIM-GapConfig-r17 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.1.6.3.3-2:MUSIM-GapConfig-r17(step 9, step 13, 8.1.2.1.6.3.3-1)

| Derivation Path: TS 38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MUSIM-GapConfig-r17 ::= SEQUENCE { |  |  |  |
| musim-GapToReleaseList-r17 | Not Present |  |  |
| MeasIdleCarrierNR-r16[1] SEQUENCE { |  |  |  |
| musim-GapToAddModList-r17 SEQUENCE (SIZE (1..3)) OF MUSIM-Gap-r17{ |  |  |  |
| MUSIM-Gap-r17[1] SEQUENCE { |  |  |  |
| musim-GapId-r17 | 0 |  |  |
| musim-GapInfo-r17 SEQUENCE { |  |  |  |
| musim-Starting-SFN-AndSubframe-r17 | Not Present |  |  |
| musim-GapLength-r17 | ms6 |  | Step 9 |
|  | ms4 |  | Step 13 |
| musim-GapRepetitionAndOffset-r17 CHOICE { |  |  |  |
| ms80-r17 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| musim-AperiodicGap-r17 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.1.6.3.3-3:MUSIM-GapConfig-r17(step 17, step 13, 8.1.2.1.6.3.3-1)

| Derivation Path: TS 38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MUSIM-GapConfig-r17 ::= SEQUENCE { |  |  |  |
| musim-GapToReleaseList-r17 SEQUENCE (SIZE (1..3)) OF MUSIM-GapId-r17 { |  |  |  |
| MUSIM-GapId-r17[1] SEQUENCE{ |  |  |  |
| MUSIM-Gap-r17[1] SEQUENCE { |  |  |  |
| musim-GapId-r17 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| musim-GapToAddModList-r17 | Not Present |  |  |
| musim-AperiodicGap-r17 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.1.6.3.3-4:MUSIM-GapConfig-r17(step 21, step 13, 8.1.2.1.6.3.3-1)

| Derivation Path: TS 38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MUSIM-GapConfig-r17 ::= SEQUENCE { |  |  |  |
| musim-GapToReleaseList-r17 | Not Present |  |  |
| musim-GapToAddModList-r17 | Not Present |  |  |
| musim-AperiodicGap-r17 SEQUENCE { |  |  |  |
| musim-Starting-SFN-AndSubframe-r17 SEQUENCE { |  |  |  |
| starting-SFN-r17 | 0 |  |  |
| startingSubframe-r17 | 0 |  |  |
| } |  |  |  |
| musim-GapLength-r17 | ms4 |  |  |
| musim-GapRepetitionAndOffset-r17 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |