3GPP TSG-RAN5 Meeting #98	R5-230122r1

# Athens, Greece, 27th Feb – 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5174 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to IMS eCall over LTE test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The stage 2 specification TS 23.167 clause 3.1 includes the following definition: “NG-eCall (eCall Over IMS): A manually or automatically initiated IMS emergency call, from a vehicle, supplemented with a minimum set of emergency related initial data (MSD). “ However, the stage 3 specification 24.229 clause 3.1 specifies that only one of these terms apply: “For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.167 [4B] apply: eCall over IMS” Test cases 11.3.6, 11.3.7 and 11.3.8 refer to the term ‘NG eCall’. This is confusing as this term is not used in the stage 3 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed ‘NG’ from all references to ‘NG eCall’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Referring to an ‘NG eCall’ in a 4G test specification may cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.6, 11.3.7, 11.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-1 CR 1513 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: CR number corrected on the CR coversheet. |  |  |  |  |  |  |  |  |

### 11.3.6	eCall Only mode / Limited service state / Call to URI for test service should not be attempted / eCall over IMS should be attempted

#### 11.3.6.1	Test Purpose (TP)

(1)

with { UE in eCALL-INACTIVE.LIMITED-SERVICE state }

ensure that {

when { UE is requested to make an eCall to URI for test service}

then { UE does not transmit any RRCConnectionRequest message }

}

(2)

with { UE in eCALL-INACTIVE.LIMITED-SERVICE state }

ensure that {

when { UE is requested to make a manual eCall}

then { UE transmits an ATTACH REQUEST message to attach for emergency bearer services, together with a PDN CONNECTIVITY REQUEST message with 'Request type' set to 'emergency’ }

}

#### 11.3.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122, clauses 2

[TS 23.122 clause 2]

If the MS is in eCall only mode, it attempts to camp on a suitable cell and enters an "eCall inactive" state in which it can only attempt an eCall over IMS, or a call to a non-emergency MSISDN or URI for test or terminal reconfiguration services as specified in 3GPP TS 31.102 [40].

If the MS is in eCall only mode and is unable to find a suitable cell to camp on, it attempts to camp on an acceptable cell in limited service state, and enters an "eCall inactive" state in which it can only attempt an eCall over IMS. The signalling procedure for the release of the NAS signalling connection is initiated by the network.

#### 11.3.6.3	Test description

11.3.6.3.1	Pre-test conditions

System Simulator:

-	Cell A

-	The PLMN is defined in Table 11.3.6.3.1-1.

Table 11.3.6.3.1–1: PLMN identifier

| Cell | PLMN name | MCC / MNC |
| --- | --- | --- |
| A | PLMN4 | 004 / 31 |

UE:

-	The eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 36.508 [18] Table 4.9.3.5-2.

-	The UE is equipped with a USIM containing values shown in Table 11.3.6.3.1-2.

Table 11.3.6.3.1-2: USIM configuration

| USIM field | Value |
| --- | --- |
| EFFPLMN | PLMN4 |

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

11.3.6.3.2	Test procedure sequence

Table 11.3.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Wait 60s for the UE to camp on Cell A as an acceptable cell. | - | - | - | - |
| 3 | An eCall to URI for test service is initiated. (Note 1) | - | - | - | - |
| 4 | Check: Does UE send an RRCConnectionRequest on Cell A within 120 seconds? | --> | RRC: RRCConnectionRequest | 1 | F |
| 5 | A manual eCall is initiated. (Note 2) | - | - | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'? | --> | RRC: RRCConnectionRequest | - | - |
| 7 | SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup | - | - |
| 8 | Check: Does the UE transmit an ATTACH REQUEST message to attach for emergency bearer services, together with a PDN CONNECTIVITY REQUEST message with 'Request type' set to 'emergency? | --> | NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST | 2 | P |
| 9-18 | Steps 5 to 14 of the generic test procedure for IMS Emergency call establishment in EUTRA: Limited Service (TS 36.508 subclause 4.5A.5.3-1). | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 19 to 23 below, the behaviour in table 11.3.6.2-2 occurs. | - | - | - | - |
| 19-23 | Steps 15 to 19 of the generic test procedure for IMS emergency call establishment in EUTRA: Limited Service (subclause 4.5A.5.3-1) | - | - | - | - |
| 24-27 | Release eCall over IMS using the generic procedure described in TS 34.229-1 [35] subclause C.33. | - | - | - | - |
| - | EXCEPTION: Step 27Aa1 and 27Aa2 describes optional behaviour that depends on the UE implementation. | - | - | - | - |
| 27Aa1 | The UE transmits a DETACH REQUEST message with the Detach type IE indicating “EPS detach” to regain normal service. | --> | DETACH REQUEST | - | - |
| 27Aa2 | The SS responds the DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| 28 | Void | - | - | - | - |
| 29 | The SS releases the RRC connection after 5secs of Step 27. | <-- | RRCConnectionRelease | - | - |
| Note 1: The request to originate an eCall to URI for test service may be performed by MMI or AT command. Note 2: The request to originate a manual eCall may be performed by MMI or AT command. |  |  |  |  |  |

Table 11.3.6.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1 describes behaviour that depends on the UE behaviour. | - | - |
| 1 | If initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| 2-8 | Steps 1-7 defined in annex C.47 of TS 34.229-1 [35] Generic procedure for eCall Setup. (See Note 1) | - | - |
| Note 1: Content of INVITE request as specified in TS 34.229-1 [35] Step-1 of Annex C.47 using conditions A6 and A20 |  |  |  |

11.3.6.3.3	Specific message contents

Table 11.3.6.3.3-1: Message SystemInformationBlockType1 (Preamble)

Derivation path: 36.508 table 4.4.3.2-3 Condition eCalloverIMS

### 11.3.7	eCall Only mode / SRVCC Handover to CS domain / UTRAN / MSD Update / Success

#### 11.3.7.1	Test Purpose (TP)

(1)

with { UE in eCall Only Mode and an IMS eCall is ongoing }

ensure that {

when { UE receives a MobilityFromEUTRACommand message and an UTRA Speech RAB combination is configured for an UTRA cell }

then { UE transmits a HANDOVER TO UTRAN COMPLETE message on the utra cell }

}

#### 11.3.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.4.3.3, TS 23.237, clauses 5.3.3, 6c.2.1, TS 23.216, clause 5.3.4.2, TS 26.267, clause 4.3, and TS 24.008, clause 5.2.4.2.

[TS 36.331, clause 5.4.3.3]

The UE shall:

1>	stop timer T310, if running;

1>	if the MobilityFromEUTRACommand message includes the purpose set to handover:

2>	if the targetRAT-Type is set to utra or geran:

3>	consider inter-RAT mobility as initiated towards the RAT indicated by the targetRAT-Type included in the MobilityFromEUTRACommand message;

3>	forward the nas-SecurityParamFromEUTRA to the upper layers;

3>	access the target cell indicated in the inter-RAT message in accordance with the specifications of the target RAT;

[TS 23.237, clause 5.3.3]

The Emergency Access Transfer Function (EATF) provides IMS-based mechanisms for enabling service continuity of IMS emergency sessions and eCall over IMS. It is a function in the serving (visited if roaming) IMS network, providing the procedures for IMS emergency session anchoring and PS to CS Access Transfer. The EATF acts as a routing B2BUA which invokes third party call control (3pcc) for enablement of Access Transfer.

When supporting PS to CS DRVCC for IMS emergency, the EATF provides the following functions:

-	Generates and sends an E-STN-DR to UE for session continuity procedure toward the CS domain. The E-STN-DR is used by the EATF to correlate two access legs, and is unique for each access transfer function within an EATF.

The EATF performs the session continuity when the Access Transfer request indicated by the E-STN-SR is received.

[TS 23.237, clause 6c.2.1]

Figure 6c.2.1-1 provides flow for SRVCC for IMS emergency session, when the IMS emergency session is active session.

Figure 6c.2.1-1: IMS level Call flow for SRVCC for IMS emergency session with E-STN-SR

1.	MSC Server initiates the session transfer with the E-STN-SR and it includes the equipment identifier.

2.	The I-CSCF routes the INVITE directly to the EATF via I5 by using similar procedures to that defined in TS 23.228 [4] for PSI based Application Server termination.

NOTE 1:	The use of indirect routeing for PSI based Application Server Termination as described in TS 23.228 [4] in clause 5.7.6 cannot be used for routing the INVITE to the EATF.

3.-4.	The EATF uses the E-STN-SR to determine that Access Transfer is requested. The EATF proceeds with the Access Transfer of the active session with bi-directional speech for the UE by updating the Remote Leg with the media description and other information using the Remote Leg Update procedure as specified in clause 6.3.1.5. For SRVCC session transfer of an eCall over IMS, the EATF indicates in the reINVITE that the EATF shall exclude INFO requests for any Info Packages related to eCall over IMS as defined in RFC 6086 [34] clause 5.2.2.

NOTE:	Indicating an unwillingness to receive INFO requests will prevent an emergency centre/PSAP from sending an INFO message to request an updated MSD from the UE.

5.	The E-CSCF forwards the Re-INVITE to the MGCF associated with the PSAP if the PSAP is located in the PSTN or CS Domain (the u-plane path is switched between the UE and the MGW) or the Re-INVITE is sent directly to an IP-capable PSAP (the u-plane path between the UE and the PSAP is switched end-to-end).

6.	When session modification procedures complete, the source access leg (i.e. the access leg previously established over IMS) is released as specified in clause 6.3.1.6.

NOTE 2:	If non-voice media was part of the original Multimedia emergency call session, the non-voice media will be released.

If the MS is in eCall only mode, it attempts to camp on a suitable cell and enters an "eCall inactive" state in which it can only attempt an eCall over IMS, or a call to a non-emergency MSISDN or URI for test or terminal reconfiguration services as specified in 3GPP TS 31.102 [40].

If the MS is in eCall only mode and is unable to find a suitable cell to camp on, it attempts to camp on an acceptable cell in limited service state, and enters an "eCall inactive" state in which it can only attempt an eCall over IMS. The signalling procedure for the release of the NAS signalling connection is initiated by the network.

[TS 23.216, clause 5.3.4.2]

3GPP SRVCC UE is needed to perform SRVCC (see clause 3.1 for 3GPP SRVCC UE definition). The interaction between UE and E-UTRAN is described in TS 36.300 [16] and between UE and UTRAN (HSPA) is described in TS 25.331 [19].

The SRVCC UE indicates to the network that the UE is SRVCC capable when being configured for using IMS speech service supported by the home operator, e.g. the IMS Multimedia Telephony Service for bi-directional speech as described in TS 22.173 [26].

For an eCall Over IMS after handover to GERAN/UTRAN is complete, the SRVCC UE shall support in-band transfer of updated MSD according to TS 26.267 [43].

[TS 23.267, clause 4.3]

After an emergency voice call has been (automatically or manually) established, the IVS modem receiver constantly monitors the incoming signal from the speech decoder output. When prompted by a request from the PSAP operator for MSD, the IVS connects the IVS data modem transmitter to the input of the speech coder and mutes any speech from the motorist for the duration of MSD transmission to prevent it from interfering with the eCall data transmission. Alternatively, it can be the IVS that may trigger the MSD transmission. In this case, the IVS asks the PSAP to request an MSD transmission.

The first operation mode shall be referred to as the pull mode whereas the latter one is the push mode. Essentially, push mode is realized by a request from the IVS to the PSAP to pull the MSD.

The requirement about the modem to be configured in either push or pull mode is beyond the scope of this specification. Refer to clause 4.2 for a reproduction of eCall service requirements.

[TS 24.008, clause 5.2.4.2]

If the MS supports single radio PS to CS access transfer for calls in alerting state as specified in 3GPP TS 24.237 [136] subclause 12.2.3B, and the MS has a single voice media stream over the PS domain that is handed over to the CS domain via SRVCC, and the call control entity in "null" state receives an indication "MM connection establishment due to SRVCC handover", then:

-	if the voice media stream is associated with a mobile originated session in the "early" state (defined in IETF RFC 3261 [137]) according to the conditions specified in 3GPP TS 24.237 [136] subclause 12.2.3B.3.2, the call control entity of the MS shall enter the "call delivered" state for this transaction. The MS and the network shall locally set the TI value of the call to "000" and the TI flag value as in mobile terminated call; and

...

If the MS has additional voice media streams carried over the PS domain that are handed over to the CS domain via SRVCC, the state for the transactions and the setting of the TI value and TI flag for these additional media streams is described in 3GPP TS 24.237 [136].

#### 11.3.7.3	Test description

11.3.7.3.1	Pre-test conditions

System Simulator:

-	Cell A and Cell 5.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

UE:

-	The eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 36.508 [18] Table 4.9.3.5-2.

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

11.3.7.3.2	Test procedure sequence

Table 11.3.7.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 11.3.7.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell A | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values are such that entering conditions for event B2 are not satisfied. |
|  | CPICH_Ec (UTRA FDD) | dBm/3.84 MHz | - | -88 |  |
|  | PCCPCH_Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -88 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -84 | - | The power level values are such that entering conditions for event B2 are satisfied. |
|  | CPICH_Ec (UTRA FDD) | dBm/3.84 MHz | - | -64 |  |
|  | PCCPCH_Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -64 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | Non-suitable “Off” | - |  |
|  | CPICH_Ec (UTRA FDD) | dBm/3.84 MHz | - | -64 |  |
|  | PCCPCH_Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -64 |  |

Table 11.3.7.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Wait 60s for the UE to camp on Cell A as an acceptable cell. | - | - | - | - |
| 3 | A manual eCall is initiated. (See Note 1). | - | - | - | - |
| 4-26 | Steps 3 to 25 of the generic test procedure for eCall over IMS establishment in EUTRA: eCall only mode (TS 36.508 4.5A.27). | - | - | - | - |
| 27 | The SS transmits an RRCConnectionReconfiguration message on Cell A to setup inter-RAT measurement and reporting for event B2. | <-- | RRCConnectionReconfiguration | - | - |
| 28 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell A. | --> | RRCConnectionReconfigurationComplete | - | - |
| 29 | The SS changes the power level for Cell A and Cell 5 according to the row "T1" in Table 11.3.7.3.2-1. | - | - | - | - |
| 30 | The UE transmits a MeasurementReport message on Cell A to report event B2 for Cell 5. | --> | MeasurementReport | - | - |
| 31 | The SS transmits a UECapabilityEnquiry message on Cell A to request UE radio access capability information for E-UTRA and UTRA. | <-- | UECapabilityEnquiry | - | - |
| 32 | The UE transmits a UECapabilityInformation message on Cell A. NOTE: The start-CS values received, should be used to configure ciphering on Cell 5. | --> | UECapabilityInformation | - | - |
| 33 | The SS transmits a MobilityFromEUTRACommand message on Cell A. | <-- | MobilityFromEUTRACommand | - | - |
| 34 | Check: Does the UE transmit a HANDOVER TO UTRAN COMPLETE message on Cell 5? | --> | HANDOVER TO UTRAN COMPLETE | 1 | P |
| - | EXCEPTION: In parallel to the events described in step 35 to 40 the steps specified in Table 11.3.7.3.2-3 takes place. | - | - | - | - |
| 35 | The SS transmits a SECURITY MODE COMMAND message for the CS domain on Cell 5. | <-- | SECURITY MODE COMMAND | - | - |
| 36 | The UE transmits a SECURITY MODE COMPLETE message on Cell 5. | --> | SECURITY MODE COMPLETE | - | - |
| 37 | The SS transmits an UTRAN MOBILITY INFORMATION message on Cell 5 to notify CN information. | <-- | UTRAN MOBILITY INFORMATION | - | - |
| 38 | The UE transmits an UTRAN MOBILITY INFORMATION CONFIRM message on Cell 5. | --> | UTRAN MOBILITY INFORMATION CONFIRM | - | - |
| 39 | The SS transmits a TMSI REALLOCATION COMMAND message on Cell 5. | <-- | TMSI REALLOCATION COMMAND | - | - |
| 40 | The UE transmits a TMSI REALLOCATION COMPLETE message on Cell 5. | --> | TMSI REALLOCATION COMPLETE | - | - |
| 41 | The CS traffic channel is kept alive by UE for at-least 5 seconds for in-band MSD transfer | - | - | - | - |
| 42 | SS adjusts cell levels according to row T2 of table 11.3.7.3.2-1. | - | - | - | - |
| - | The UE is in end state UTRA CS call. | - | - | - | - |
| Note 1: The request to originate a manual eCall may be performed by MMI or AT command. |  |  |  |  |  |

Table 11.3.7.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a ROUTING AREA UPDATE REQUEST message on Cell 5. | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 2 | The SS transmits a SECURITY MODE COMMAND message for the PS domain on Cell 5. | <-- | SECURITY MODE COMMAND | - | - |
| 3 | The UE transmits a SECURITY MODE COMPLETE message on Cell 5. | --> | SECURITY MODE COMPLETE | - | - |
| 4 | The SS transmits a ROUTING AREA UPDATE ACCEPT message on Cell 5. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 5 | The UE transmits a ROUTING AREA UPDATE COMPLETE message on Cell 5. | --> | ROUTING AREA UPDATE COMPLETE | - | - |

11.3.7.3.3	Specific message contents

Table 11.3.7.3.3-1: Message SystemInformationBlockType1 (Preamble)

Derivation path: 36.508 table 4.4.3.2-3 Condition eCalloverIMS

Table 11.3.7.3.3-2: ATTACH REQUEST (preamble)

| Derivation path: 36.508 Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MS network capability |  | SRVCC from UTRAN HSPA or E-UTRAN to GERAN/UTRAN supported |  |  |
| Mobile station classmark 2 |  | Any allowed value |  |  |
| Supported Codecs |  | Any allowed value |  |  |

Table 11.3.7.3.3-3: RRCConnectionReconfiguration (step 27, Table 11.3.7.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 11.3.7.3.3-4: MeasConfig (Table 11.3.7.3.3-3)

| Derivation Path: 36.508, Table 4.6.6-1, condition UTRAN |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| MeasConfig ::= SEQUENCE { |  |  |  |  |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  | 2 entries |  |  |  |  |  |
| measObjectId[1] |  | IdMeasObject-f1 |  |  |  |  |  |
| measObject[1] |  | MeasObjectEUTRA-GENERIC(f1) |  |  |  |  |  |
| measObject[1] |  | MeasObjectEUTRA-GENERIC(maxEARFCN) |  |  |  | Band > 64 |  |
| measObjectId[2] |  | IdMeasObject-f8 |  |  |  |  |  |
| measObject[2] |  | MeasObjectUTRA-f8 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { |  | 1 entry |  |  |  |  |  |
| reportConfigId[1] |  | IdReportConfig-B2-UTRA |  |  |  |  |  |
| reportConfig[1] |  | ReportConfigInterRAT-B2-UTRA (-72, -76) |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  | 1 entry |  |  |  |  |  |
| measId[1] |  | 1 |  |  |  |  |  |
| measObjectId[1] |  | IdMeasObject-f8 |  |  |  |  |  |
| reportConfigId[1] |  | IdReportConfig-B2-UTRA |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  | 2 entries |  |  |  | Band > 64 |  |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |  |  |  |  |
| carrierFreq-v9e0 |  | Same downlink EARFCN as used for f1 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE {} |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 11.3.7.3.3-5: MeasObjectUTRA-f8 (Table 11.3.7.3.3-4)

| Derivation Path: 36.508, Table 4.6.6-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectUTRA ::= SEQUENCE { |  |  |  |
| carrierFreq | Same downlink ARFCN as used for Cell 5 |  |  |
| cellsToAddModList CHOICE { |  |  |  |
| cellsToAddModListUTRA-FDD SEQUENCE (SIZE (1..maxCellMeas)) OF SEQUENCE { | 1 entry |  | UTRA-FDD |
| cellIndex[1] | 1 |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 5 |  |  |
| } |  |  |  |
| cellsToAddModListUTRA-TDD SEQUENCE (SIZE (1..maxCellMeas)) OF SEQUENCE { |  |  | UTRA-TDD |
| cellIndex[1] | 1 |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 5 |  |  |
| } |  |  |  |
| } |  |  |  |
| csg-allowedReportingCells-v930 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 11.3.7.3.3-6: MeasurementReport (step 30, Table 11.3.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] CHOICE { |  |  |  |
| fdd | PhysicalCellIdentity of Cell 5 |  | UTRA-FDD |
| tdd | PhysicalCellIdentity of Cell 5 |  | UTRA-TDD |
| } |  |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| utra-RSCP | (-5..91) |  |  |
| utra-EcN0 | Not present |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 11.3.7.3.3-7: UECapabilityEnquiry (step 31, Table 11.3.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 2 entries |  |  |
| RAT-Type[1] | eutra |  |  |
| RAT-Type[2] | utra |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.3.7.3.3-8: MobilityFromEUTRACommand (step 33, Table 11.3.7.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| mobilityFromEUTRACommand-r8 SEQUENCE { |  |  |  |
| cs-FallbackIndicator | False |  |  |
| purpose CHOICE { |  |  |  |
| handover SEQUENCE { |  |  |  |
| targetRAT-Type | utra |  |  |
| targetRAT-MessageContainer | HANDOVER TO UTRAN COMMAND(UTRA RRC message) |  |  |
| nas-SecurityParamFromEUTRA | The 4 least significant bits of the NAS downlink COUNT value |  |  |
| systemInformation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.3.7.3.3-9: HANDOVER TO UTRAN COMMAND (Table 11.3.7.3.3-8)

Derivation Path: 36.508, Table 4.7B.1-1, condition UTRA Speech

Table 11.3.7.3.3-10: SECURITY MODE COMMAND (step 35, Table 11.3.7.3.2-2)

| Derivation Path: 36.508, Table 4.7B.1-n |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Ciphering mode info | Not present |  |  |

Table 11.3.7.3.3-13: ROUTING AREA UPDATE ACCEPT (step 4, Table 11.3.7.3.2-3)

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDP context status | 0 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |

### 11.3.8	eCall Only mode / SRVCC Handover to CS domain / GERAN / MSD Update / Success

#### 11.3.8.1	Test Purpose (TP)

(1)

with { UE in eCall Only Mode and an IMS eCall is ongoing }

ensure that {

when { UE receives a MobilityFromEUTRACommand message and an UTRA Speech RAB combination is configured for an UTRA cell }

then { UE transmits a HANDOVER COMPLETE message on the geran cell }

}

#### 11.3.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.4.3.3, TS 23.237, clauses 5.3.3, 6c.2.1, TS 23.216, clause 5.3.4.2, TS 26.267, clause 4.3, and TS 24.008, clause 5.2.4.2.

[TS 36.331, clause 5.4.3.3]

The UE shall:

1>	stop timer T310, if running;

1>	if the MobilityFromEUTRACommand message includes the purpose set to handover:

2>	if the targetRAT-Type is set to utra or geran:

3>	consider inter-RAT mobility as initiated towards the RAT indicated by the targetRAT-Type included in the MobilityFromEUTRACommand message;

3>	forward the nas-SecurityParamFromEUTRA to the upper layers;

3>	access the target cell indicated in the inter-RAT message in accordance with the specifications of the target RAT;

[TS 23.237, clause 5.3.3]

The Emergency Access Transfer Function (EATF) provides IMS-based mechanisms for enabling service continuity of IMS emergency sessions and eCall over IMS. It is a function in the serving (visited if roaming) IMS network, providing the procedures for IMS emergency session anchoring and PS to CS Access Transfer. The EATF acts as a routing B2BUA which invokes third party call control (3pcc) for enablement of Access Transfer.

When supporting PS to CS DRVCC for IMS emergency, the EATF provides the following functions:

-	Generates and sends an E-STN-DR to UE for session continuity procedure toward the CS domain. The E-STN-DR is used by the EATF to correlate two access legs, and is unique for each access transfer function within an EATF.

The EATF performs the session continuity when the Access Transfer request indicated by the E-STN-SR is received.

[TS 23.237, clause 6c.2.1]

Figure 6c.2.1-1 provides flow for SRVCC for IMS emergency session, when the IMS emergency session is active session.

Figure 6c.2.1-1: IMS level Call flow for SRVCC for IMS emergency session with E-STN-SR

1.	MSC Server initiates the session transfer with the E-STN-SR and it includes the equipment identifier.

2.	The I-CSCF routes the INVITE directly to the EATF via I5 by using similar procedures to that defined in TS 23.228 [4] for PSI based Application Server termination.

NOTE 1:	The use of indirect routeing for PSI based Application Server Termination as described in TS 23.228 [4] in clause 5.7.6 cannot be used for routing the INVITE to the EATF.

3 - 4.	The EATF uses the E-STN-SR to determine that Access Transfer is requested. The EATF proceeds with the Access Transfer of the active session with bi-directional speech for the UE by updating the Remote Leg with the media description and other information using the Remote Leg Update procedure as specified in clause 6.3.1.5. For SRVCC session transfer of an eCall over IMS, the EATF indicates in the reINVITE that the EATF shall exclude INFO requests for any Info Packages related to eCall over IMS as defined in RFC 6086 [34] clause 5.2.2.

NOTE:	Indicating an unwillingness to receive INFO requests will prevent an emergency centre/PSAP from sending an INFO message to request an updated MSD from the UE.

5.	The E-CSCF forwards the Re-INVITE to the MGCF associated with the PSAP if the PSAP is located in the PSTN or CS Domain (the u-plane path is switched between the UE and the MGW) or the Re-INVITE is sent directly to an IP-capable PSAP (the u-plane path between the UE and the PSAP is switched end-to-end).

6.	When session modification procedures complete, the source access leg (i.e. the access leg previously established over IMS) is released as specified in clause 6.3.1.6.

NOTE 2:	If non-voice media was part of the original Multimedia emergency call session, the non-voice media will be released.

If the MS is in eCall only mode, it attempts to camp on a suitable cell and enters an "eCall inactive" state in which it can only attempt an eCall over IMS, or a call to a non-emergency MSISDN or URI for test or terminal reconfiguration services as specified in 3GPP TS 31.102 [40].

If the MS is in eCall only mode and is unable to find a suitable cell to camp on, it attempts to camp on an acceptable cell in limited service state, and enters an "eCall inactive" state in which it can only attempt an eCall over IMS. The signalling procedure for the release of the NAS signalling connection is initiated by the network.

[TS 23.216, clause 5.3.4.2]

3GPP SRVCC UE is needed to perform SRVCC (see clause 3.1 for 3GPP SRVCC UE definition). The interaction between UE and E-UTRAN is described in TS 36.300 [16] and between UE and UTRAN (HSPA) is described in TS 25.331 [19].

The SRVCC UE indicates to the network that the UE is SRVCC capable when being configured for using IMS speech service supported by the home operator, e.g. the IMS Multimedia Telephony Service for bi-directional speech as described in TS 22.173 [26].

For an eCall Over IMS after handover to GERAN/UTRAN is complete, the SRVCC UE shall support in-band transfer of updated MSD according to TS 26.267 [43].

[TS 23.267, clause 4.3]

After an emergency voice call has been (automatically or manually) established, the IVS modem receiver constantly monitors the incoming signal from the speech decoder output. When prompted by a request from the PSAP operator for MSD, the IVS connects the IVS data modem transmitter to the input of the speech coder and mutes any speech from the motorist for the duration of MSD transmission to prevent it from interfering with the eCall data transmission. Alternatively, it can be the IVS that may trigger the MSD transmission. In this case, the IVS asks the PSAP to request an MSD transmission.

The first operation mode shall be referred to as the pull mode whereas the latter one is the push mode. Essentially, push mode is realized by a request from the IVS to the PSAP to pull the MSD.

The requirement about the modem to be configured in either push or pull mode is beyond the scope of this specification. Refer to clause 4.2 for a reproduction of eCall service requirements.

[TS 24.008, clause 5.2.4.2]

If the MS supports single radio PS to CS access transfer for calls in alerting state as specified in 3GPP TS 24.237 [136] subclause 12.2.3B, and the MS has a single voice media stream over the PS domain that is handed over to the CS domain via SRVCC, and the call control entity in "null" state receives an indication "MM connection establishment due to SRVCC handover", then:

-	if the voice media stream is associated with a mobile originated session in the "early" state (defined in IETF RFC 3261 [137]) according to the conditions specified in 3GPP TS 24.237 [136] subclause 12.2.3B.3.2, the call control entity of the MS shall enter the "call delivered" state for this transaction. The MS and the network shall locally set the TI value of the call to "000" and the TI flag value as in mobile terminated call; and

...

If the MS has additional voice media streams carried over the PS domain that are handed over to the CS domain via SRVCC, the state for the transactions and the setting of the TI value and TI flag for these additional media streams is described in 3GPP TS 24.237 [136].

#### 11.3.8.3	Test description

11.3.8.3.1	Pre-test conditions

System Simulator:

-	Cell A and Cell 24.

-	System information combination 5 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

UE:

-	The eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 36.508 [18] Table 4.9.3.5-2.

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

11.3.8.3.2	Test procedure sequence

Table 11.3.8.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 11.3.8.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell A | Cell 24 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | - | The power level values are such that entering conditions for event B2 are not satisfied. |
|  | RSSI | dBm | - | -85 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | - | The power level values are such that entering conditions for event B2 are satisfied. |
|  | RSSI | dBm | - | -65 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | Non-suitable “Off” | - |  |
|  | RSSI | dBm | - | -65 |  |

Table 11.3.8.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Wait 60s for the UE to camp on Cell A as an acceptable cell. | - | - | - | - |
| 3 | A manual eCall is initiated. (See Note 1). | - | - | - | - |
| 4-26 | Steps 3 to 25 of the generic Test Procedure for eCall over IMS establishment in EUTRA: eCall only mode (TS 36.508 4.5A.27). | - | - | - | - |
| 27 | The SS transmits an RRCConnectionReconfiguration message on Cell A to setup inter-RAT measurement and reporting for event B2. | <-- | RRCConnectionReconfiguration | - | - |
| 28 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell A. | --> | RRCConnectionReconfigurationComplete | - | - |
| 29 | The SS changes the power level for Cell A and Cell 24 according to the row "T1" in Table 11.3.8.3.2-1. | - | - | - | - |
| 30 | The UE transmits a MeasurementReport message on Cell A to report event B2 for Cell 24. | --> | MeasurementReport | - | - |
| 31 | The SS transmits a MobilityFromEUTRACommand message on Cell A. | <-- | MobilityFromEUTRACommand | - | - |
| 32 | Check: Does the UE transmit a HANDOVER COMPLETE message on cell 24? | --> | HANDOVER COMPLETE | 1 | P |
| 33 | The UE transmits a GPRS SUSPENSION REQUEST message | --> | GPRS SUSPENSION REQUEST | - | - |
| 34 | The SS transmits a TMSI REALLOCATION COMMAND message. | <-- | TMSI REALLOCATION COMMAND | - | - |
| 35 | The UE transmits a TMSI REALLOCATION COMPLETE message. | --> | TMSI REALLOCATION COMPLETE | - | - |
| 36 | SS adjusts cell levels according to row T2 of table 11.3.8.3.2-1. | - | - | - | - |
| 37 | The CS traffic channel is kept alive by UE for at least 5 seconds for In-band MSD transfer | - | - | - | - |
| 38-52 | Steps 20 to 34 of the generic test procedure described in TS 36.508 subclause 6.4.3.8.1 are performed on Cell 24. NOTE: Call is released and UE performs a RAU procedure. | - | - | - | - |
| Note 1: The request to originate a manual eCall may be performed by MMI or AT command. |  |  |  |  |  |

11.3.8.3.3	Specific message contents

Table 11.3.8.3.3-1: Message SystemInformationBlockType1 (Preamble)

Derivation path: 36.508 table 4.4.3.2-3 Condition eCalloverIMS

Table 11.3.8.3.3-2: ATTACH REQUEST (preamble)

| Derivation path: 36.508 Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MS network capability | SRVCC from UTRAN HSPA or E-UTRAN to GERAN/UTRAN supported |  |  |
| Mobile station classmark 2 | Any allowed value |  |  |
| Supported Codecs | Any allowed value |  |  |

Table 11.3.8.3.3-3: RRCConnectionReconfiguration (step 27, Table 11.3.8.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 11.3.8.3.3-4: MeasConfig (Table 11.3.8.3.3-3)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 with condition GERAN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measurementConfiguration ::= SEQUENCE { |  |  |  |
| measObjectToAddModifyList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f11 |  |  |
| measObject[1] | MeasObjectGERAN-GENERIC(f11) |  |  |
| measObjectId[2] | IdMeasObject-f1 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModifyList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfigInterRAT-B2-GERAN |  |  |
| reportConfig[1] | ReportConfigInterRAT-B2-GERAN (-69, -75) |  |  |
| } |  |  |  |
| measIdToAddModifyList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f11 |  |  |
| reportConfigId[1] | IdReportConfigInterRAT-B2-GERAN |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE {} |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 11.3.8.3.3-5: MeasurementReport (step 30, Table 11.3.8.3.2-2)

| Derivation Path: 36.508, table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListGERAN SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId | PhysicalCellIdentity of Cell 24 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rssi | The value of rssi is present but contents not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.3.8.3.3-6: MobilityFromEUTRACommand (step 31, Table 11.3.8.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| mobilityFromEUTRACommand-r8 SEQUENCE { |  |  |  |
| cs-FallbackIndicator | False |  |  |
| purpose CHOICE{ |  |  |  |
| handover SEQUENCE { |  |  |  |
| targetRAT-Type | GERAN |  |  |
| targetRAT-MessageContainer | HANDOVER COMMAND(GERAN RR message) , see Table 11.3.8.3.3-7 |  |  |
| nas-SecurityParamFromEUTRA | The 4 least significant bits of the NAS downlink COUNT value |  |  |
| systemInformation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.3.8.3.3-7: HANDOVER COMMAND (Table 11.3.8.3.3-6)

| Derivation Path: 51.010, Table 40.2.4.33 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Cell Description |  |  |  |
| Network Colour Code | 1 |  |  |
| Base Station Colour Code | 5 |  |  |
| BCCH Carrier Number | The BCCH Carrier ARFCN as per table in clause 40.1.1 of 51.010-1. |  |  |
| Description of the First Channel, after time |  |  |  |
| Channel Description |  |  |  |
| Channel Type and TDMA offset | TCH/F + ACCH’s |  |  |
| Timeslot Number | Chosen arbitrarily, but not Zero. |  |  |
| Training Sequence Code | Same as the BCCH |  |  |
| Hopping channel | Single RF channel |  |  |
| ARFCN | The first ARFCN in the cell allocation as per table in clause 40.2.1.1.1 of 51.010-1 |  |  |
| Cipher Mode Setting | 1001xxxy | See TS 44.018 §9.1.15.10 xxx - px_GSM_CipherAlg y - px_GSM_CipheringOnOff |  |

Table 11.3.8.3.3-10: ROUTING AREA UPDATE ACCEPT (step 51, Table 11.3.8.3.2-2)

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PDP context status | 0 | NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |