3GPP TSG-RAN5 Meeting #98	R5-230127r1

# Athens, Greece, 27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0289 | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.3B - Generic test procedures for UE MCVideo operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Transmission Control Ack, Transmission End Request, Transmission End Response, Media Reception End Request and Media Reception End Response need either condition UPLINK or condition DOWNLINK - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Specific message content added for Transmission Control Ack, Transmission End Request, Transmission End Response, Media Reception End Request and Media Reception End Response specifying condition UPLINK/DOWNLINK - Editorial corrections, further editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.3B	Generic test procedures for UE MCVideo operation

### 5.3B.1	MCVideo CO session establishment/modification without provisional responses other than 100 Trying

5.3B.1.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3B.1.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.1.3	Procedure

Table 5.3B.1.3-1: MCVideo CO session establishment/modification without provisional responses other than 100 Trying

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCVideo call establishment described in clause 5.4.3 'MCX CO communication in E-UTRA' take place. | - | - | - | - |
| 2 | Check: Does the UE (MCVideo client) send a SIP INVITE requesting the establishment/modification of an MCVideo call? | --> | SIP INVITE | - | P |
| 3 | The SS sends SIP 100 Trying | <-- | SIP 100 (Trying) | - | - |
| 4 | The SS (MCVideo server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 5 | Check: Does the UE (MCVideo client) send a SIP ACK to acknowledge the session establishment/modification? | --> | SIP ACK | - | P |
| - | EXCEPTION: Steps 6a1-6a2 describe behaviour that depends on the test case requirements; the "lower case letter" identifies a step sequence that takes place if the UE requests implicit transmission control in step 2 (i.e. the "mc_implicit_request" fmtp attribute included in the SDP offer and the SS responded with the "mc_implicit_request" fmtp attribute included and the “mc_granted” fmtp attribute not present in the SDP answer. (NOTE 1) | - | - | - | - |
| 6a1 | The SS (MCVideo server) sends a Transmission Granted message with request for acknowledgement. | <-- | Transmission Granted | - | - |
| 6a2 | Check: Does the UE (MCVideo client) send a Transmission Control Ack message? | --> | Transmission Control Ack | - | P |
| NOTE 1: Possibilities in SDP-offer/answer depend on the test case requirements a. UE sends SDP offer with media description for transmission control but without implicit transmission request b. UE sends SDP offer with media description for transmission control and with implicit transmission request i. SDP answer from SS contains “mc_implicit_request” and “mc_granted” (Transmission is implicitly granted) ii. SDP answer from SS contains “mc_implicit request” and but no “mc_granted” (Transmission needs to be explicitly granted ar step 6a1) iii. SDP answer from SS contains no “mc_implicit_request”and no “mc_granted” (the UE needs to explicitly request the transmission ) c. UE sends SDP offer without media description for transmission control |  |  |  |  |  |

5.3B.1.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure with the following clarifications:

Table 5.3B.1.4-1: SIP INVITE (step 2, Table 5.3B.1.3-1)

Derivation Path: Table 5.5.2.5.1-1, condition MCVIDEO

Table 5.3B.1.4-2: SIP 200 (OK) (step 4, Table 5.3B.1.3-1)

Derivation Path: Table 5.5.2.17.1.2-1, condition INVITE-RSP and MCVIDEO

Table 5.3B.1.4-3: Transmission Granted (step 6a1, Table 5.3B.1.3-1)

Derivation Path: Table 5.5.11.2.1-1, condition ACK

Table 5.3B.1.4-4: Transmission Control Ack (step 6a2, Table 5.3B.1.3-1)

Derivation Path: Table 5.5.11.3.5-1, condition UPLINK

### 5.3B.2	MCVideo Transmission request – Transmission Granted

5.3B.2.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3B.2.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.2.3	Procedure

Table 5.3B.2.3-1: MCVideo Transmission Request – Transmission Granted

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCVideo client) send a Transmission Request message? | --> | Transmission Request | - | P |
| 2 | The SS (MCVideo server) sends a Transmission Granted message with request for acknowledgement. | <-- | Transmission Granted | - | - |
| 3 | Check: Does the UE (MCVideo client) send a Transmission Control Ack message? | --> | Transmission Control Ack | - | P |
| 4 | Check: Does the UE (MCVideo client) provide transmission granted notification to the user? (NOTE 1) | - | - | - | P |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3B.2.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3B.2.4-1: Transmission Granted (step 2, Table 5.3B.2.3-1)

Derivation Path: Table 5.5.11.2.1-1, condition ACK

Table 5.3B.2.4-2: Transmission Control Ack (step 3, Table 5.3B.2.3-1)

Derivation Path: Table 5.5.11.3.5-1, condition UPLINK

### 5.3B.3	MCVideo Media Transmission Notification and Request CT

5.3B.3.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3B.3.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.3.3	Procedure

Table 5.3B.3.3-1: MCVideo Media Transmission Notification and Request CT

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS (MCVideo server) sends a Media Transmission Notification message. | <-- | Media Transmission Notification | - | - |
| 2 | Check: Does the UE (MCVideo client) provide media transmission notification to the user? (NOTE 1) | - | - | - | P |
| - | EXCEPTION: Steps 3a1 – 3a4a1 describe behaviour that depends on the requirements of test case calling the present procedure. | - | - | - | - |
| 3a1 | IF the test case specifies the Reception Mode field of the Media Transmission Notification message to be 1 (indicating manual reception mode) THEN make the UE (MCVideo client) request permission to receive media. (NOTE 1) | - | - | - | - |
| 3a2 | Check: Does the UE (MCVideo client) send a Receive Media Request message? | --> | Receive Media Request | - | P |
| 3a3 | The SS (MCVideo server) sends a Receive Media Response message. | <-- | Receive Media Response | - | - |
| - | EXCEPTION: Step 3a4a1 describes behaviour that depends on the requirements of test case calling the present procedure. | - | - | - | - |
| 3a4a1 | IF the test case specifies the Receive Media Response message to request an acknowledgement THEN Check: Does the UE (MCVideo client) send a Transmission Control Ack message? | --> | Transmission Control Ack | - | P |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3B.3.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3B.3.4-1: Transmission Control Ack (step 3a4a1, Table 5.3B.3.3-1)

Derivation Path: Table 5.5.11.3.5-1, condition UPLINK

### 5.3B.4	MCVideo Transmission Request - Queue Position Info

5.3B.4.1	Initial conditions

As specified in the test case which calls the procedure.

5.3B.4.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.4.3	Procedure

Table 5.3B.4.3-1: MCVideo Transmission Request – Queue Position Info

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCVideo client) send a Transmission Request message? | --> | Transmission Request | - | P |
| 2 | The SS (MCVidao server) sends a Queue Position Info message indicating that the Transmission Request is queued. | <-- | Queue Position Info | - | - |

5.3B.4.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3B.5	MCVideo Queue Position Request

5.3B.5.1	Initial conditions

As specified in the test case which calls the procedure.

5.3B.5.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.5.3	Procedure

Table 5.3B.5.3-1: MCVideo Queue Position Request

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCVideo client) send a Queue Position Request message? | --> | Queue Position Request | - | P |
| 2 | The SS (MCVideo server) responds with a Queue Position Info message. | <-- | Queue Position Info | - | - |
| - | EXCEPTION: Step 3a1 describes behaviour that depends on the requirements of test case calling the present procedure. | - | - | - | - |
| 3a1 | IF the test case specifies the Queue Position Info message to request an acknowledgement THEN Check: Does the UE (MCVideo client) acknowledge receipt of the Queue Position Info message? | --> | Transmission Control Ack | - | P |

5.3B.5.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3B.5.4-1: Transmission Control Ack (step 3a1, Table 5.3B.5.3-1)

Derivation Path: Table 5.5.11.3.5-1, condition UPLINK

### 5.3B.6	MCVideo Transmission Request - Transmission Rejected

5.3B.6.1	Initial conditions

As specified in the test case which calls the procedure.

5.3B.6.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.6.3	Procedure

Table 5.3B.6.3-1: MCVideo Transmission Request – Transmission Rejected

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCVideo client) send a Transmission Request message? | --> | Transmission Request | - | P |
| 2 | The SS (MCVideo server) sends a Transmission Rejected message. | <-- | Transmission Rejected | - | - |
| 3 | Check: Does the UE (MCVideo client) provide Transmission Rejected notification to the user? (NOTE 1) | - | - | - | P |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3B.6.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

### 5.3B.7	MCVideo Transmission End Request CO

5.3B.7.1	Initial conditions

As specified in the test case which calls the procedure.

5.3B.7.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.7.3	Procedure

Table 5.3B.7.3-1: MCVideo transmission End Request CO

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCVideo client) send a Transmission End Request message? | --> | Transmission End Request | - | P |
| 2 | The SS (MCVideo server) responds with a Transmission End Response message with request for acknoledgement. | <-- | Transmission End Response | - | - |
| 3 | Check: Does the UE (MCVideo client) send a Transmission Control Ack message? | --> | Transmission Control Ack | - | P |
| 4 | The SS (MCVideo server) sends a Transmission Idle message. | <-- | Transmission Idle | - | - |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3B.7.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3B.7.4-1: Transmission End Request (Step 1, Table 5.3B.7.3-1)

Derivation Path: Table 5.5.11.3.1-1, condition UPLINK

Table 5.3B.7.4-2: Transmission End Response (Step 2, Table 5.3B.7.3-1)

Derivation Path: Table 5.5.11.3.2-1, condition DOWNLINK, ACK

Table 5.3B.7.4-3: Transmission Control Ack (step 3, Table 5.3B.7.3-1)

Derivation Path: Table 5.5.11.3.5-1, condition UPLINK

### 5.3B.8	MCVideo Media Reception End Request CO

5.3B.8.1	Initial conditions

As specified in the test case which calls the procedure.

5.3B.8.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.8.3	Procedure

Table 5.3B.8.3-1: MCVideo Media Reception End Request CO

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCVideo client) send a Media Reception End Request message? | --> | Media Reception End Request | - | P |
| 2 | The SS (MCVideo server) sends a Receive Media Reception End Response message. | <-- | Media Reception End Response | - | - |
| 3 | The SS (MCVideo server) sends a Transmission Idle message. | <-- | Transmission Idle | - | - |

5.3B.8.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3B.8.4-1: Media Reception End Request (Step 1, Table 5.3B.8.3-1)

Derivation Path: Table 5.5.11.3.3-1, condition UPLINK

Table 5.3B.8.4-2: Media Reception End Response (Step 2, Table 5.3B.8.3-1)

Derivation Path: Table 5.5.11.3.4-1, condition DOWNLINK

### 5.3B.9	MCVideo Transmission End Request CT

5.3B.9.1	Initial conditions

As specified in the test case which calls the procedure.

5.3B.9.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.9.3	Procedure

Table 5.3B.9.3-1: MCVideo Transmission End Request CT

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS (MCVideo server) sends a Transmission End Request message. | <-- | Transmission End Request | - | - |
| 2 | Void | - | - | - | - |
| 2A | Check: Does the UE (MCVideo client) respond with a Transmission End Response message? | --> | Transmission End Response | - | P |
| 3 | Void | - | - | - | - |
| 3A | Check Does the UE (MCVideo client) notify the user that the permission to send RTP media is being revoked? (NOTE 1) | - | - | - | P |
| 4 | The SS (MCVideo server) sends a Transmission Idle message. | <-- | Transmission Idle | - | - |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3B.9.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3B.9.4-1: Transmission End Request (Step 1, Table 5.3B.9.3-1)

Derivation Path: Table 5.5.11.3.1-1, condition DOWNLINK

Table 5.3B.9.4-2: Transmission End Response (Step 2, Table 5.3B.9.3-1)

Derivation Path: Table 5.5.11.3.2-1, condition UPLINK

### 5.3B.10	MCVideo Media Reception End Request CT

5.3B.10.1	Initial conditions

As specified in the test case which calls the procedure.

5.3B.10.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.10.3	Procedure

Table 5.3B.10.3-1: MCVideo Media Reception End Request CT

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS (MCVideo server) sends a Media Reception End Request message. | <-- | Media Reception End Request | - | - |
| 2 | Void | - | - | - | - |
| 2A | Check: Does the UE (MCVideo client) respond with a Media Reception End Response message? | --> | Media Reception End Response | - | P |
| 3 | Void | - | - | - | - |
| 3A | Check: Does the UE (MCVideo client) notify the user that the permission to send RTP media is being revoked? (NOTE 1) | - | - | - | P |
| 4 | The SS (MCVideo server) sends a Transmission Idle message. | <-- | Transmission Idle | - | - |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3B.10.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3B.10.4-1: Media Reception End Request (Step 1, Table 5.3B.10.3-1)

Derivation Path: Table 5.5.11.3.3-1, condition DOWNLINK

Table 5.3B.10.4-2: Media Reception End Response (Step 2, Table 5.3B.10.3-1)

Derivation Path: Table 5.5.11.3.4-1, condition UPLINK

### 5.3B.11	MCVideo CO session modification

5.3B.11.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3B.11.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.11.3	Procedure

Table 5.3B.11.3-1: MCVideo CO session modification

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCVideo client) send a SIP INVITE requesting the modification of the call? | --> | SIP re-INVITE | - | P |
| 2 | The SS sends SIP 100 Trying | <-- | SIP 100 (Trying) | - | - |
| 3 | The SS (MCVideo server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 4 | Check: Does the UE (MCVideo client) send a SIP ACK to acknowledge the session modification? | --> | SIP ACK | - | P |
| - | EXCEPTION: Steps 5a1-5a2 describe behaviour that depends on whether the UE has implicitly requested a grant at step 1 which has not implicitly been granted at step 3 (NOTE 1) | - | - | - | - |
| 5a1 | IF the media description for media control in the 200 OK contains fmtp parameter mc_implicit_request but no fmtp parameter mc_granted THEN the SS (MCVideo server) sends a Transmission Granted message with request for acknowledgement. | <-- | Transmission Granted | - | - |
| 5a2 | Check: Does the UE (MCVideo client) send a Transmission Control Ack message? | --> | Transmission Control Ack | - | P |
| NOTE 1: An implicit transmit media request may be requested in case of upgrade to an emergency or imminent peril MCVideo group call but not in case of a downgrade or any other re-INVITE |  |  |  |  |  |

5.3B.11.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3B.11.4-1: SIP 200 (OK) (step 3, Table 5.3B.11.3-1)

Derivation Path: Table 5.5.2.17.1.2-1, condition INVITE-RSP

Table 5.3B.11.4-2: Transmission Granted (step 5a1, Table 5.3B.11.3-1)

Derivation Path: Table 5.5.11.2.1-1, condition ACK

Table 5.3B.11.4-3: Transmission Control Ack (step 5a2, Table 5.3B.11.3-1)

Derivation Path: Table 5.5.11.3.5-1, condition UPLINK