3GPP TSG-RAN5 Meeting #98	R5-230178r2
Athens, Greece, 27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0464 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PC1 MU - General Update in 38.903 test case section B.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | PC1 MUs definition continues.The agreements made in sections B.3 to B.25 for specific test cases must be captured accordingly in section B.2.2. By the way, an error is observed in the SNR specified for PC3 in RX spurious test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Summarized the MU factors agreed for PC1 during this meeting. Original proposal is based on proposals made in R5-230161. Corrected SNR for RX spurious test case in PC3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There might be incosnsitency between PC1 MU values listed and section B.2.2 and the values provided in sections B.3 to B.25. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: -r1: -added Anritsu as co-source. -Updated with agreements made during offline disucssions. -r2: -Added new NOTE 3 to Table B.2.2.27-1 applicable to the new values corrected. -Corrected Influence of noise for MOP-TRP FR2b according to agreements (missed change in revision 1). |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### B.2.2.27	Influence of noise

See B.2.1.27.

The uncertainty value of influence of noise is estimated as below table and used across clause B.

Editor’s Note: For ACLR, all applicable configurations need to be added.

Table B.2.2.27-1: Uncertainty value for influence of noise for PC3 for IFF

| Test case |  | Frequency range |  | Noise floor |  | Minimum requirement |  | Estimated SNRtotal [dB/400MHz] |  | Relaxation |  | Influence of noise |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MOP-EIRP |  | FR2a |  | N/A |  | 20.7dBm/ChBW (22.4-1.7) |  | 16.33 (NOTE 1) |  | 0 |  | 0.1 |  |
|  |  | FR2b |  | N/A |  | 18.9dBm/ChBW (20.6-1.7) |  | 11.45 (NOTE 1) |  | 0 |  | 0.3 |  |
| MOP-TRP |  | FR2a |  | N/A |  | 23dBm/ChBW |  | 16.33 (NOTE 1) |  | 0 |  | 0.1 |  |
|  |  | FR2b |  | N/A |  | 23dBm/ChBW |  | 11.45 (NOTE 1) |  | 0 |  | 0.3 |  |
| MOP-Spherical |  | FR2a |  | N/A |  | 9.75dBm/ChBW (Spherical – MBR= 11.5-1.75) |  | 11.45 (NOTE 1) |  | 0 |  | 0.3 |  |
|  |  | FR2b |  | N/A |  | 7.6dBm/ChBW (Spherical – MBR=8-0.4) |  | 6.37 (NOTE 1) |  | 0 |  | 0.9 |  |
| MPR |  | FR2a |  | -7.6dBm/400MHz |  | 7.65dBm/ChBW (EIRP-MPB-MPR-T(MPR)=22.4-0.75-9-5) |  | 15.17 (NOTE 1) |  | 0 |  | 0.13 |  |
|  |  | FR2b |  | -5.5dBm/400MHz |  | 5.85dBm/ChBW (EIRP-MPB-MPR-T(MPR)=20.6-0.75-9-5) |  | 11.30 (NOTE 1) |  | 0 |  | 0.31 |  |
| Configured output power with power boost |  | FR2a |  | N/A |  | 21.7dBm/ChBW (22.4-1.7+1) |  | 16.33 (NOTE 1) |  | 0 |  | 0.1 |
|  |  | FR2b |  | N/A |  | 19.9dBm/ChBW (20.6-1.7+1) |  | 11.45 (NOTE 1) |  | 0 |  | 0.3 |
| Minimum output power |  | FR2a |  | -10.6dBm/400MHz |  | -13dBm |  | -2.54 (NOTE 1) |  | 8.4 |  | 1.0 (with relaxation) |  |
|  |  | FR2b |  | -5.5dBm/400MHz |  | -13dBm |  | -7.64 (NOTE 1) |  | 13.5 |  | 1.0 (with relaxation) |  |
| OFF power – TRP |  | FR2a |  | N/A |  | -35dBm/ChBW |  | -24.54 (NOTE 2) |  | 30.4 |  | 1.0 (with relaxation) |  |
|  |  | FR2b |  | N/A |  |  |  | <24.54 (NOTE 2) |  | N/A |  | Propose not to test |  |
| OFF power – EIRP |  | FR2a |  | -7.6dBm/400MHz |  | -30dBm/ChBW |  | -22.54 (NOTE 2) |  | 28.4 |  | 1.0 (with relaxation) |  |
|  |  | FR2b |  | -5.5dBm/400MHz |  |  |  | -24.64 (NOTE 2) |  | 30.5 |  | 1.0 (with relaxation) |  |
| Absolute power tolerance |  | Same as Minimum output power |  |  |  |  |  |  |  |  |  |  |  |
| Relative power tolerance |  | FR2a |  | -13.6dBm/100MHz |  | -7.6dBm/100MHz |  | 5.86 (NOTE 1) |  | 0 |  | 1.0 |  |
|  |  | FR2b |  | -11.5dBm/100MHz |  | -5.5dBm/100MHz |  | 5.86 (NOTE 1) |  | 0 |  | 1.0 |  |
| Aggregate power tolerance |  | Same as Relative power tolerance |  |  |  |  |  |  |  |  |  |  |  |
| Aggregate power tolerance |  | FR2a |  | -13.6dBm/100MHz |  | -7.6dBm/100MHz |  | 5.86 (NOTE 1) |  | 0 |  | 1.0 |  |
|  |  | FR2b |  | -11.5dBm/100MHz |  | -5.5dBm/100MHz |  | 5.86 (NOTE 1) |  | 0 |  | 1.0 |  |
| SEM |  | FR2a |  | N/A |  | -13dBm/1MHz |  | 8.14 (NOTE 1) |  | 0 |  | 0.62 |  |
|  |  | FR2b |  | N/A |  |  |  | 5.86 (NOTE 1) |  | 0 |  | 1.0 |  |
| ACLR (CP) |  | FR2a |  | -7.6dBm/400MHz |  | Highest testable MPR for 400MHz: 3dB 16.65dBm/ChBW (EIRP-MPB-MPR-T(MPR) =22.4-0.75-3-2) Actual lowest: 7.65dBm/ChBW (EIRP-MPB-MPR-T(MPR)=22.4-0.75-9-5) |  | 22.86 (with 3dB MPR) (NOTE 1) |  | 0 |  | N/A |  |
|  |  | FR2b |  | -5.5dBm/400MHz |  | Highest testable MPR for 400MHz: 2dB 16.35dBm/ChBW (EIRP-MPB-MPR-T(MPR) =20.6-0.75-2-1.5) Actual lowest: 5.85dBm/ChBW (EIRP-MPB-MPR-T(MPR)=20.6-0.75-9-5) |  | 21.86 (with 2dB MPR) (NOTE 1) |  | 0 |  | N/A |  |
| ACLR (ACP) |  | FR2a |  | -7.6dBm/400MHz |  | Highest testable MPR for 400MHz: 3dB -0.35dBm/ChBW (EIRP-MPB-MPR-T(MPR)-ACLR=22.4-0.75-3-2-17) Actual lowest: -9.35 dBm/ChBW (EIRP-MPB-MPR-T(MPR)-ACLR=22.4-0.75-9-5-17) |  | 5.86 (NOTE 1) |  | 0 |  | 1.0 |  |
|  |  | FR2b |  | -5.5dBm/400MHz |  | Highest testable MPR for 400MHz: 2dB 0.35dBm/ChBW (EIRP-MPB-MPR-T(MPR)-ACLR=20.6-0.75-2-1.5-16) Actual lowest: -10.15 dBm/ChBW (EIRP-MPB-MPR-T(MPR)-ACLR=20.6-0.75-9-5-16) |  | 5.86 (with 2dB MPR) (NOTE 1) |  | 0 |  | 1.0 |  |
| General Tx spurious |  | 6GHz <=f<=23.45GHz |  | N/A |  | -13dBm/1MHz |  | 10.0 (NOTE 1) |  | 0 |  | 0.41 |  |
|  |  | 23.45GHz<=f<=40GHz |  | N/A |  | -13dBm/1MHz |  | 10.0 (NOTE 1) |  | 0 |  |  |  |
|  |  | 40GHz<=f<=80GHz |  | N/A |  | -13dBm/1MHz |  | 10.0 (NOTE 1) |  | 0 |  |  |  |
| Tx spurious Co-existence |  | n260 (Aggressor band : n257, n261) |  | -23 |  | -2dBm/100MHz (-22dBm/MHz) |  | 0.86 (NOTE 2) |  | 5 |  | 1.0 (with relaxation) |  |
|  |  | n257, n261 (Aggressor band : n260) |  | -27.7 |  | -5dBm/100MHz (-25dBm/MHz) |  | 2.56 (NOTE 2) |  | 3.3 |  | 1.0 (with relaxation) |  |
|  |  | 23.6 GHz ≤ f ≤ 24.0GHz |  | -27.7 |  | 1dBm/200MHz (-22 dBm/MHz) |  | 5.56 (NOTE 2) |  | 0.3 |  | 1.0 (with relaxation) |  |
|  |  | 36 GHz ≤ f ≤ 37GHz |  | -23dBm/MHz |  | 7dBm/1000MHz (-23dBm/MHz) |  | -0.14 (NOTE 2) |  | 6 |  | 1.0 (with relaxation) |  |
|  |  | 57 GHz ≤ f ≤ 66GHz |  | N/A |  | 2dBm/100MHz (-18dBm/MHz) |  | 5.86 (NOTE 1) |  | 0 |  | 1.0 |  |
| Additional spurious emission |  | NS_202 (7.25GHz <=f <=12.75GHz) |  | -40 dBm/MHz |  | -10dBm/100MHz (-30 dBm/MHz) |  | 10 (NOTE 1) |  | 0 |  | 0.41 |  |
|  |  | NS_202 (12.75GHz <=f <=23.45GHz) |  | -23 dBm/MHz |  | -10dBm/100MHz (-30 dBm/MHz) |  | -7.14 (NOTE 2) |  | 13 |  | 1.0 (with relaxation) |  |
|  |  | NS_202 (23.6GHz <=f <=24.0GHz) |  | -27.7 dBm/MHz |  | 1dBm/200MHz (-22 dBm/MHz) |  | 5.56 (NOTE 2) |  | 0.3 |  | 1.0 (with relaxation) |  |
|  |  | NS_202 (23.45GHz <=f <=40.8GHz) |  | -23 dBm/MHz |  | -10dBm/100MHz (-30 dBm/MHz) |  | -7.14 (NOTE 2) |  | 13 |  | 1.0 (with relaxation) |  |
|  |  | NS_202 (40.8GHz <=f <=66GHz) |  | -23 dBm/MHz |  | -10dBm/100MHz (-30 dBm/MHz) |  | -7.14 (NOTE 2) |  | 13 |  | 1.0 (with relaxation) |  |
|  |  | NS_203 (23.6GHz <=f <=24.0GHz) |  | -27.7 dBm/MHz |  | +1dBm/200MHz (-22dBm/MHz) |  | 5.56 (NOTE 2) |  | 0.3 |  | 1.0 (with relaxation) |  |
| Rx spurious |  | 6GHz <=f<=20GHz |  |  |  | -47dBm/1MHz |  | -2.2 (NOTE 3) |  | 10.2 |  | 1.0 dB for 23.45~40.8GHz, 0.64dB for 6~23.45 and 40.8~80 GHz. |  |
|  |  | 20GHz<=f<=40GHz |  |  |  | -47dBm/1MHz |  | -11.34 (NOTE 2) |  | 17.2 |  |  |  |
|  |  | 40GHz<=f<=80GHz |  |  |  | -47dBm/1MHz |  | -25.1 (NOTE 3) |  | 33.1 |  |  |  |
| NOTE 1: Estimated SNR is calculated based on agreed influence of noise. NOTE 2: Estimated SNR is calculated based on agreed relaxation value: Estimated SNR = 6dB - relaxation. NOTE 3: Estimated SNR is calculated based on agreed relaxation value: Estimated SNR = 8dB - relaxation. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.2.2.27-2: Uncertainty value for influence of noise for PC1 for IFF

| Test case | Frequency range | Relaxation [dB] | Influence of noise [dB] |  |
| --- | --- | --- | --- | --- |
| MOP-EIRP | FR2a | 0 | 0.13 |  |
|  | FR2b | 0 | 0.2 |  |
| MOP-TRP | FR2a | 0 | 0.13 |  |
|  | FR2b | 0 | 0.27 |  |
| MOP-Spherical | FR2a | 0 | 0.2 |  |
|  | FR2b | 0 | 0.35 |  |
| MPR | FR2a | 0 | 0.1 |  |
|  | FR2b | 0 | 0.3 |  |
| Minimum output power | FR2a | 0 | 0.3 |  |
|  | FR2b | 0 | 0.6 |  |
| OFF power – TRP | FR2a | Same as defined for PC3 in Table B.2.2.27-1 |  |  |
|  | FR2b | Same as defined for PC3 in Table B.2.2.27-1 |  |  |
| SEM | FR2a | 0 | 1.81 (NOTE 1, 2) |  |
|  | FR2b | 0 | TBD |  |
| ACLR (ACP) | FR2a | 0 | 0.95 |  |
|  | FR2b | 0 | 0.95 |  |
| General Tx spurious | 6GHz <=f<12.75GHz | Same as defined for PC3 in Table B.2.2.27-1 |  |  |
|  | 12.75GHz<=f<66GHz | 0 | 1.08 (NOTE 1, 3) |  |
|  | 66GHz<=f<=80GHz | Same as defined for PC3 in Table B.2.2.27-1 |  |  |
| Tx spurious Co-existence | n260 (Aggressor band : n257, n261) | 5 (Same as PC3) | 2.34 (NOTE 1, 4) |  |
|  | n257, n261 (Aggressor band : n260) | 3.3 (Same as PC3) | 2.34 (NOTE 1, 4) |  |
|  | 23.6 GHz ≤ f ≤ 24.0GHz | 0.3 (Same as PC3) | 2.34 (NOTE 1, 4) |  |
|  | 57 GHz ≤ f ≤ 66GHz | Same as defined for PC3 in Table B.2.2.27-1 |  |  |
| Additional spurious emission | NS_202 (7.25GHz <=f <=12.75GHz) | Same as defined for PC3 in Table B.2.2.27-1 |  |  |
|  | NS_202 (12.75GHz <=f <=23.45GHz) | 13 (Same as PC3) |  | 2.34 (NOTE 1, 4) |
|  | NS_202, NS_203 23.6 GHz ≤ f ≤ 24.0GHz | 0.3 (Same as PC3) | 2.34 (NOTE 1, 4) |  |
|  | NS_202 (23.45GHz <=f <=40.8GHz) | 13 (Same as PC3) | 2.34 (NOTE 1, 4) |  |
|  | NS_202 (40.8GHz <=f <=66GHz) | 13 (Same as PC3) | 2.34 (NOTE 1, 4) |  |
| Rx spurious | 6GHz <=f<=20GHz | Same as defined for PC3 in Table B.2.2.27-1 |  |  |
|  | 20GHz<=f<=40GHz | Same as defined for PC3 in Table B.2.2.27-1 |  |  |
|  | 40GHz<=f<=80GHz | Same as defined for PC3 in Table B.2.2.27-1 |  |  |
| NOTE 1: values assuming up to 6% of grid points with EIRP > 43dBm. NOTE 2: values assuming SNR = -7.9dB for points with EIRP > 43dBm / SNR = 8.14dB otherwise. NOTE 3: values assuming SNR = -5dB for points with EIRP > 43dBm / SNR = 10dB otherwise. NOTE 4: values assuming SNR = -9dB for points with EIRP > 43dBm / SNR = 6dB otherwise and considering the proposed relaxation. |  |  |  |  |

## <<< Skip unchanged sections >>>

## <<< END OF CHANGES >>>