3GPP TSG-RAN5 Meeting #98	R5-230186r1

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0422 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS for ATSSS devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom,ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | PICS for NR ATSSS devices in TS 38.508-2 is missed and need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PICS for NR ATSSS devices has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR ATSSS test cases can not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR .38/523-2.. CR ..0300. |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1:change the name of the PICS to pc_5GC_ATSSS as ATSSS is a 5GC feature and add the missing information in coversheet. |  |  |  |  |  |  |  |  |

A.4.3.7	General Capabilities

Table A.4.3.7-1: UE General Capabilities

| Item |  |  |  | UE General Capabilities |  |  |  | Ref. |  |  |  | Release |  |  |  | Mnemonic |  |  |  | M |  |  |  | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release |  |  |  | Comments |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | Support UL transmission via either MCG path or SCG path for the split SRB as specified in TS 37.340[20] |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-15 |  |  |  | pc_splitSRB_WithOneUL_Path |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  | Support UL transmission via both MCG path and SCG path for the split DRB as specified in TS 37.340[20] |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-15 |  |  |  | pc_splitDRB_withUL_Both_MCG_SCG |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  | Support direct SRB between the SN and the UE as specified in TS 37.340[20] |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-15 |  |  |  | pc_srb3 |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  | Support of reflective QoS |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-15 |  |  |  | pc_as_ReflectiveQoS |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  | Support of NAS reflective QoS |  |  |  | 24.501, 6.2.5.1.4.1, 9.11.4.1 |  |  |  | Rel-15 |  |  |  | pc_nas_ReflectiveQoS |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  | Support of SMS over NAS |  |  |  | 24.501, 5.5.1.2 |  |  |  | Rel-15 |  |  |  | pc_sms_over_NAS |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  | Support of CMAS message on NR |  |  |  | 38.331, 5.2.2.2.2 |  |  |  | Rel-15 |  |  |  | pc_CMAS_NR |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  | Support of ETWS message on NR |  |  |  | 38.331, 5.2.2.2.2 |  |  |  | Rel-15 |  |  |  | pc_ETWS_NR |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  | The UE supports additional UE-requested PDU establishment |  |  |  | 24.501, 6.4.1.5 |  |  |  | Rel-15 |  |  |  | pc_Additional_PDU_establishment |  |  |  | No |  |  |  |  |  |  |  | pc_ExpectedNoOfPDUSessionsAtRegistration +1 |  |  |  |
| 10 |  |  |  | The UE includes the SM PDU DN request container IE in the PDU SESSION ESTABLISHMENT REQUEST message |  |  |  | 24.501, 6.4.1.2 |  |  |  | Rel-15 |  |  |  | pc_SM_PDU_DN_RequestContainer |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  | Support of emergency services fallback in NR connected to 5GCN |  |  |  | 24.501 |  |  |  | Rel-15 |  |  |  | pc_NR_5GC_EmergencyService_fallback |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  | Support of EPS fallback |  |  |  | 24.501, |  |  |  | Rel-15 |  |  |  | pc_EPS_fallback |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  | Support of UE requested PDU session modification |  |  |  | 24.501, 6.4.2.2 |  |  |  | Rel-15 |  |  |  | pc_MO_PDU_Session_Modification |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  | Support of emergency services in NR connected to 5GCN |  |  |  | 24.501 |  |  |  | Rel-15 |  |  |  | pc_NR_5GC_EmergencyServices |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  | Support of voiceFallbackIndication |  |  |  | 38.306, 4.2.13 |  |  |  | Rel-16 |  |  |  | pc_voiceFallbackIndication |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  |  | Support provision of referenceTimeInfo |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-16 |  |  |  | pc_referenceTimeProvision_r16 |  |  |  | No |  |  |  |  |  |  |  | specifically for TSC (time sensitive communication) services |  |  |  |
| 17 |  |  |  | Support of RACS |  |  |  | 24.501, 9.11.3.1 |  |  |  | Rel-16 |  |  |  | pc_5GC_RACS |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 18 |  |  |  | Support of RRC message Segmentation in the UL |  |  |  | 38.306, 5.4 |  |  |  | Rel-16 |  |  |  | pc_NR_UL_Segmentation |  |  |  | No |  |  |  |  |  |  |  | UE supports segmenation of UECapabilityInformation message, IF size > maximum supported size of a PDCP SDU |  |  |  |
| 19 |  |  |  | Support of RRC_INACTIVE as specified in TS 38.331 [9]. |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-15 |  |  |  | pc_inactiveState |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  | Support of UE local release when the security check is successful but SOR Transparent container indicates ACK has been NOT requested |  |  |  | 23.122 clause C.2 |  |  |  | Rel-15 |  |  |  | pc_SOR_ACKNotReqLocalRel |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 21 |  |  |  | Support of RRC connection release with deprioritisation |  |  |  | 38.306, 5.3 |  |  |  | Rel-15 |  |  |  | pc_NR_RRC_Release_With_Deprioritisation |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 22 |  |  |  | Support of RRC connection establishment failure with temporary offset |  |  |  | 38.306, 5.3 |  |  |  | Rel-15 |  |  |  | pc_NR_RRC_ConEstFail_With_TempOffset |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 23 |  |  |  | Support of Closed Access Group |  |  |  | 24.501, 9.11.3.1 |  |  |  | Rel-16 |  |  |  | pc_CAG |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 24 |  |  |  | Support of Stand-alone Non-Public Network |  |  |  | 23.501, 3.1 |  |  |  | Rel-16 |  |  |  | pc_SNPN |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  |  |  | Support of test function SET UL MESSAGE for using a preconfigured UE capability container over NR |  |  |  | 38.509, 5.9 |  |  |  | Rel-16 |  |  |  | pc_Set_UE_Cap_Info_NR |  |  |  | No |  |  |  |  |  |  |  | This test function is mandatory for UEs supporting UL RRC segmentation and whose maximum UECapabilityInformation message size is less than the allowed maximum supported size of a PDCP SDU. |  |  |  |
| 26 |  |  |  | Support of network slice-specific authentication and authorization |  |  |  | 24.501, 9.11.3.1 |  |  |  | Rel-16 |  |  |  | pc_5GC_NSSAA |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 27 |  |  |  | Support of EAP-AKA’ as EAP method for network slice-specific authentication and authorization |  |  |  | 24.501, 5.4.7 |  |  |  | Rel-16 |  |  |  | pc_5GC_NSSAA_EAP_AKA_Prime |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  |  |  | Support reduced control plane latency as defined in TS 38.331 [9] |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-15 |  |  |  | pc_reducedCP_Latency |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Support of release preference assistance information |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-16 |  |  |  | pc_releasePreference_r16 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 30 |  |  |  | Support of user initiated SNPN reselection in automatic mode |  |  |  | 23.122 |  |  |  | Rel-16 |  |  |  | pc_UserInitiated_SNPN_Reselection |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 31 |  |  |  | Support of autonomous search function to detect CAG cells on serving and non-serving frequencies |  |  |  | 38.304, 5.2.4.10 |  |  |  | Rel-16 |  |  |  | pc_Autonomous_search_function_nr_CAG |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 32 |  |  |  | Support IMS voice over NR |  |  |  | 38.306, 4.2.13 |  |  |  | Rel-15 |  |  |  | pc_voiceOverNR |  |  |  | No |  |  |  |  |  |  |  | A UE supporting IMS voice over NR shall support: - IMS emergency call over NR, and - IMS voice over E-UTRA/EPC if it supports E-UTRA/EPC. |  |  |  |
| 33 |  |  |  | Support of V2X communication |  |  |  | 24.501, 9.11.3.1 |  |  |  | Rel-16 |  |  |  | pc_V2X |  |  |  | No |  |  |  |  |  |  |  | UE support V2X communication over NR-Uu and/or NR-PC5. |  |  |  |
| 34 |  |  |  | Support of V2X communication over NR-PC5 |  |  |  | 24.501, 9.11.3.1 |  |  |  | Rel-16 |  |  |  | pc_V2XCNPC5 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 35 |  |  |  | Support of Manufacturer assigned Radio Capability ID |  |  |  | 23.501, 5.9.10 |  |  |  | Rel-16 |  |  |  | pc_5GC_RACS_Manufacturer_URCID |  |  |  | No |  |  |  |  |  |  |  | UE support of Manufacturer assigned radio capability ID |  |  |  |
| 36 |  |  |  | Support of 3GPP PS data off |  |  |  | 24.501, 6.2.10 |  |  |  | Rel-15 |  |  |  | pc_PS_data_off |  |  |  | No |  |  |  |  |  |  |  | UE support of 3GPP PS data off |  |  |  |
| 37 |  |  |  | Support of Network Slice Simultaneous Registration Group |  |  |  | 24.501, 9.11.3.82 |  |  |  | Rel-17 |  |  |  | pc_5GC_NSSRG |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 38 |  |  |  | Support of slice reselection information in SIB and on RRC release for slice based cell reselection in RRC _IDLE and RRC INACTIVE |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-17 |  |  |  | pc_sliceInfoforCellReselection_r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 39 |  |  |  | Support of reception of segmented DL RRC messages |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-16 |  |  |  | pc_NR_dl_DedicatedMessageSegmentation |  |  |  | No |  |  |  |  |  |  |  | The SS initiates the DL Dedicated Message Segment transfer procedure IF the encoded RRCReconfiguration or RRCResume message PDU size > maximum PDCP SDU size. |  |  |  |
| 40 |  |  |  | Support of unified access control configuration in the list of subscriber data, indicating for which access identities (see 3GPP TS 24.501 [64]) the ME is configured, when the MS accesses an SNPN. |  |  |  | 23.122, 4.9.3.0, 24.501 4.5.2A |  |  |  | Rel-16 |  |  |  | pc_SNPN_access_control_configuration |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 41 |  |  |  | Support of polarization signalling in NR NTN |  |  |  | 38.306, 5.4 |  |  |  | Rel-17 |  |  |  | pc_Polarization_Signalling_NR_NTN |  |  |  | No |  |  |  |  |  |  |  | UE supports polarization signalling in NR NTN |  |  |  |
| 42 |  |  |  | Supports receiving paging early indication and UE subgrouping indication with UEID |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-17 |  |  |  | pc_pei_SubgroupingSupportBandList_r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 43 |  |  |  | Support of Rel-17 extended DRX cycle up to 10485.76 seconds |  |  |  | 38.306, 5.8 24.501, 5.3.16 |  |  |  | Rel-17 |  |  |  | pc_NR_eDRX |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 44 |  |  |  | Support of (re-)configuration of an SCG during the resume procedure. |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-16 |  |  |  | pc_resumeWithSCG_Config_r16 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| XX |  |  |  | Support of ATSSS and MA PDU session |  |  |  | 24.501, 6.4.1.2 |  |  |  | Rel-16 |  |  |  | pc_5GC_ATSSS |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |