3GPP TSG-RAN5 Meeting #98	R5-230206r1

Athens, Greece, 27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0424 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Optional 6x6 PC5 Antenna Array Configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_FWA_Bn257_Bn258-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Optional vendor declaration allows test time reduction |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a vendor declaration for the alternate PC5 antenna configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test time reduction cannot be applied to BP searches and select test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.521-2 CR 0877 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Discussion paper in R5-230204 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: fix coversheet issue with version (remove leading V) |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

<<< START OF CHANGE >>>

### A.4.3.9	Additional capabilities for UE declared capability

Table A.4.3.9-1: UE declared capabilities

| Item | UE declared capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Enhanced Type 1 Receiver for NR | 38.101-4, 5 | Rel-15 | pc_nr_enh_type1_receiver | Support for Enhanced Type 1 Receiver (SU-MIMO Interference Mitigation advanced receiver) |
| 2 | Vehicular UE | 38.101-1, 3 | Rel-15 | pc_nr_vehicular_ue |  |

Table A.4.3.9-2: UE declared multi-band peak EIRP relaxation factors for Rel-15 FR2 power class 3 UE

| Item |  | Supported FR2 bands set |  | Ref. |  | Release |  | peak EIRP relaxation factor per band, MBp (dB) (Note 1) |  |  |  |  |  |  |  | Maximum sum of MBp, ∑MBP (dB) (Note 2) |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | n257 |  | n258 |  | n260 |  | n261 |  |  |  |  |  |
| 1 |  | n257, n258 |  | 38.101-2, 6.2.1.3 |  | Rel-15 |  |  |  |  |  | N/A |  | N/A |  | 1.3 |  |  |  |
| 2 |  | n257, n260 |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  | 1.0 |  |  |  |
| 3 |  | n258, n260 |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  | 1.0 |  |  |  |
| 4 |  | n258, n261 |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  | 1.0 |  |  |  |
| 5 |  | n260, n261 |  |  |  |  |  | N/A |  | N/A |  |  |  |  |  | 0.0 |  | No relaxation factor allowed |  |
| 6 |  | n257, n258, n260 |  |  |  |  |  |  |  |  |  |  |  | N/A |  | 1.7 |  |  |  |
| 7 |  | n257, n258, n261 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | 1.7 |  |  |  |
| 8 |  | n257, n260, n261 |  |  |  |  |  |  |  | N/A |  |  |  |  |  | 0.5 |  |  |  |
| 9 |  | n258, n260, n261 |  |  |  |  |  | N/A |  |  |  |  |  |  |  | 1.5 |  |  |  |
| 10 |  | n257, n258, n260, n261 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.7 |  |  |  |
| 11 |  | n257, n261 |  |  |  |  |  |  |  | N/A |  | N/A |  |  |  | 0.0 |  | No relaxation factor allowed |  |
| Note 1: UE vendor to fill in the needed relaxation factor per band that is ≥0 for Rel-15 UE supporting only Rel-15 FR2 bands. One row to be filled in, the one matching the supported FR2 bands of the UE as declared in Table A.4.3.1-3. Note 2: Max allowed sum of MBp over all supported FR2 bands as defined in TS 38.521-2 clause 6.2.1.1.3.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.3.9-3: UE declared multi-band peak EIRP Spherical coverage relaxation factors for Rel-15 FR2 power class 3 UE

| Item |  | Supported FR2 bands set |  | Ref. |  | Release |  | EIRP Spherical coverage relaxation factor per band, MBs (dB) (Note 1) |  |  |  |  |  |  |  | Maximum sum of MBs, ∑MBs (dB) (Note 2) |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | n257 |  | n258 |  | n260 |  | n261 |  |  |  |  |  |
| 1 |  | n257, n258 |  | 38.101-2, 6.2.1.3 |  | Rel-15 |  |  |  |  |  | N/A |  | N/A |  | 1.25 |  |  |  |
| 2 |  | n257, n260 |  |  |  |  |  |  |  | N/A |  |  |  | N/A |  | 0.75 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 3 |  | n258, n260 |  |  |  |  |  | N/A |  |  |  |  |  | N/A |  | 0.75 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 4 |  | n258, n261 |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  | 1.25 |  |  |  |
| 5 |  | n260, n261 |  |  |  |  |  | N/A |  | N/A |  |  |  |  |  | 0.75 |  | No relaxation allowed for n260 |  |
| 6 |  | n257, n258, n260 |  |  |  |  |  |  |  |  |  |  |  | N/A |  | 1.75 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 7 |  | n257, n258, n261 |  |  |  |  |  |  |  |  |  | N/A |  |  |  | 1.75 |  |  |  |
| 8 |  | n257, n260, n261 |  |  |  |  |  |  |  | N/A |  |  |  |  |  | 1.25 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 9 |  | n258, n260, n261 |  |  |  |  |  | N/A |  |  |  |  |  |  |  | 1.25 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 10 |  | n257, n258, n260, n261 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.75 |  | Maximum 0.4 dB relaxation allowed for n260 |  |
| 11 |  | n257, n261 |  |  |  |  |  |  |  | N/A |  | N/A |  |  |  | 0.0 |  | No relaxation factor allowed |  |
| Note 1: UE vendor to fill in the needed relaxation factor per band that is ≥0 for Rel-15 UE supporting only Rel-15 FR2 bands. One row to be filled in, the one matching the supported FR2 bands of the UE as declared in Table A.4.3.1-3 Note 2: Max allowed sum of MBs over all supported FR2 bands as defined in TS 38.521-2 clause 6.2.1.1.3.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.3.9-4a: FDD 4 Rx antenna ports Capabilities

| Item |  | Band |  | Ref. |  | Release |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | FDD Band n1 |  | 38.101-1, 7.3.2 |  | Rel-15 |  |  |  |
| 2 |  | FDD Band n2 |  | 38.101-1, 7.3.2 |  | Rel-15 |  |  |  |
| 3 |  | FDD Band n3 |  | 38.101-1, 7.3.2 |  | Rel-15 |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| 7 |  | FDD Band n7 |  | 38.101-1, 7.3.2 |  | Rel-15 |  | NOTE 2 |  |
| 8 |  | FDD Band n8 |  | 38.101-1.7.3.2 |  | Rel-17 |  | 4 Rx operation is targeted for FWA form factor |  |
| ... |  |  |  |  |  |  |  |  |  |
| 28 |  | FDD Band n28 |  | 38.101-1, 7.3.2 |  | Rel-16 |  | 4 Rx operation is targeted for FWA form factor |  |
| ... |  |  |  |  |  |  |  |  |  |
| 30 |  | FDD Band n30 |  | 38.101-1, 7.3.2 |  | Rel-16 |  |  |  |
| ... |  |  |  |  |  |  |  |  |  |
| 66 |  | FDD Band n66 |  | 38.101-1, 7.3.2 |  | Rel-15 |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| 70 |  | FDD Band n70 |  | 38.101-1, 7.3.2 |  | Rel-15 |  |  |  |
| 71 |  | FDD Band n71 |  | 38.101-1, 7.3.2 |  | Rel-16 |  | 4 Rx operation is targeted for FWA form factor |  |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/2. NOTE 2: Support of 4 Rx for this band is mandatory for non-vehicular UEs i.e. if support has NOT been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |  |  |  |  |  |  |

Table A.4.3.9-4b: TDD 4 Rx antenna ports Capabilities

| Item |  | Band | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 34 |  | TDD Band n34 | 38.101-1, 7.3.2 | Rel-15 |  |
| ... |  |  |  |  |  |
| 38 |  | TDD Band n38 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 39 |  | TDD Band n39 | 38.101-1, 7.3.2 | Rel-15 |  |
| ... |  |  |  |  |  |
| 40 |  | TDD Band n40 | 38.101-1, 7.3.2 | Rel-15 |  |
| 41 |  | TDD Band n41 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| ... |  |  |  |  |  |
| 48 |  | TDD Band n48 | 38.101-1, 7.3.2 | Rel-16 | NOTE 2 |
| … |  |  |  |  |  |
| 77 |  | TDD Band n77 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 78 |  | TDD Band n78 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 79 |  | TDD Band n79 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/3. NOTE 2: Support of 4 Rx for this band is mandatory for non-vehicular UEs i.e. if support has NOT been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |  |  |

Table A.4.3.9-4c: 2 Rx antenna ports Capabilities

| Item |  | Band |  | Ref. |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | FDD Band n1 |  | 38.101-1, 7.3.2 |  |  |  |
| 2 |  | FDD Band n2 |  | 38.101-1, 7.3.2 |  |  |  |
| 3 |  | FDD Band n3 |  | 38.101-1, 7.3.2 |  |  |  |
| 4 |  | FDD Band n5 |  | 38.101-1, 7.3.2 |  |  |  |
| 5 |  | FDD Band n7 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| 6 |  | FDD Band n8 |  | 38.101-1, 7.3.2 |  |  |  |
| 7 |  | FDD Band n12 |  | 38.101-1, 7.3.2 |  |  |  |
| 7b |  | FDD Band n14 |  | 38.101-1, 7.3.2 |  |  |  |
| 8 |  | FDD Band n20 |  | 38.101-1, 7.3.2 |  |  |  |
| 8d |  | FDD Band n24 |  | 38.101-1, 7.3.2 |  |  |  |
| 9 |  | FDD Band n25 |  | 38.101-1, 7.3.2 |  |  |  |
| 9a |  | FDD Band n26 |  | 38.101-1, 7.3.2 |  |  |  |
| 10 |  | FDD Band n28 |  | 38.101-1, 7.3.2 |  |  |  |
| 10a |  | SDL Band n29 |  | 38.101-1, 7.3.2 |  |  |  |
| 10b |  | FDD Band n30 |  | 38.101-1, 7.3.2 |  |  |  |
| 11 |  | TDD Band n34 |  | 38.101-1, 7.3.2 |  |  |  |
| 12 |  | TDD Band n38 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| 13 |  | TDD Band n39 |  | 38.101-1, 7.3.2 |  |  |  |
| 14 |  | TDD Band n40 |  | 38.101-1, 7.3.2 |  |  |  |
| 15 |  | TDD Band n41 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| 16 |  | TDD Band n48 |  | 38.101-1, 7.3.2 |  |  |  |
| 17 |  | TDD Band n50 |  | 38.101-1, 7.3.2 |  |  |  |
| 18 |  | TDD Band n51 |  | 38.101-1, 7.3.2 |  |  |  |
| 18a |  | Reserved |  |  |  |  |  |
| 18b |  | TDD Band n53 |  | 38.101-1, 7.3.2 |  |  |  |
| 19 |  | FDD Band n65 |  | 38.101-1, 7.3.2 |  |  |  |
| 20 |  | FDD Band n66 |  | 38.101-1, 7.3.2 |  |  |  |
| 21 |  | FDD Band n70 |  | 38.101-1, 7.3.2 |  |  |  |
| 22 |  | FDD Band n71 |  | 38.101-1, 7.3.2 |  |  |  |
| 23 |  | FDD Band n74 |  | 38.101-1, 7.3.2 |  |  |  |
| 24 |  | TDD Band n77 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| 25 |  | TDD Band n78 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| 26 |  | TDD Band n79 |  | 38.101-1, 7.3.2 |  | NOTE 2 |  |
| 27 |  | FDD Band n91 |  | 38.101-1, 7.3.2 |  |  |  |
| 28 |  | FDD Band n92 |  | 38.101-1, 7.3.2 |  |  |  |
| 29 |  | FDD Band n93 |  | 38.101-1, 7.3.2 |  |  |  |
| 30 |  | FDD Band n94 |  | 38.101-1, 7.3.2 |  |  |  |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/1. NOTE 2: Support of 2 Rx for this band is allowed only for vehicular UEs i.e. if support has been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |  |  |  |  |

Table A.4.3.9-4d: Enhanced transient capabilities

| Item | Band | Ref | Release | enhanced transient capability per band |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2us | 4us | 7us |  |
| 1 | FDD Band n1 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 2 | FDD Band n2 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 3 | FDD Band n3 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 4 | FDD Band n5 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 5 | FDD Band n7 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 6 | FDD Band n8 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 7 | FDD Band n12 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 8 | FDD Band n14 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 9 | FDD Band n20 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 10 | FDD Band n24 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 11 | FDD Band n25 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 12 | FDD Band n26 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 13 | FDD Band n28 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 14 | FDD Band n30 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 15 | TDD Band n34 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 16 | TDD Band n38 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 17 | TDD Band n39 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 18 | TDD Band n40 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 19 | TDD Band n41 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 20 | TDD Band n46 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 21 | TDD Band n48 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 22 | TDD Band n50 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 23 | TDD Band n51 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 24 | TDD Band n53 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 25 | TDD Band n65 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 26 | TDD Band n66 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 27 | FDD Band n70 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 28 | FDD Band n71 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 29 | FDD Band n74 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 30 | TDD Band n77 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 31 | TDD Band n78 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 32 | TDD Band n79 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| 33 | TDD Band n96 | 38.101-1, 6.4.2.1a | Rel-16 |  |  |  |  |
| NOTE 1: At least one band from those listed in the present table needs to be supported with enhanced transient capability of 2us, 4us or 7us if UE has indicated support of the capability defined in Table A.4.3.2-1/79. NOTE 2: Indicate transient capability for each band by ticking the cell corresponding to the smallest enhanced transient capability that the UE supports for that band. |  |  |  |  |  |  |  |

Table A.4.3.9-4e: 1 Rx antenna ports Capabilities

| Item | Band | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | FDD Band n1 | 38.101-1, 7.3I.2 |  |
| 2 | FDD Band n2 | 38.101-1, 7.3I.2 |  |
| 3 | FDD Band n3 | 38.101-1, 7.3I.2 |  |
| 4 | FDD Band n5 | 38.101-1, 7.3I.2 |  |
| 5 | FDD Band n8 | 38.101-1, 7.3I.2 |  |
| 6 | FDD Band n12 | 38.101-1, 7.3I.2 |  |
| 7 | FDD Band n13 | 38.101-1, 7.3I.2 |  |
| 8 | FDD Band n14 | 38.101-1, 7.3I.2 |  |
| 9 | FDD Band n18 | 38.101-1, 7.3I.2 |  |
| 10 | FDD Band n20 | 38.101-1, 7.3I.2 |  |
| 11 | FDD Band n24 | 38.101-1, 7.3I.2 |  |
| 12 | FDD Band n25 | 38.101-1, 7.3I.2 |  |
| 13 | FDD Band n26 | 38.101-1, 7.3I.2 |  |
| 14 | FDD Band n28 | 38.101-1, 7.3I.2 |  |
| 15 | FDD Band n30 | 38.101-1, 7.3I.2 |  |
| 16 | TDD Band n34 | 38.101-1, 7.3I.2 |  |
| 17 | TDD Band n39 | 38.101-1, 7.3I.2 |  |
| 18 | TDD Band n40 | 38.101-1, 7.3I.2 |  |
| 19 | TDD Band n50 | 38.101-1, 7.3I.2 |  |
| 20 | TDD Band n51 | 38.101-1, 7.3I.2 |  |
| 21 | TDD Band n53 | 38.101-1, 7.3I.2 |  |
| 22 | FDD Band n65 | 38.101-1, 7.3I.2 |  |
| 23 | FDD Band n66 | 38.101-1, 7.3I.2 |  |
| 24 | FDD Band n70 | 38.101-1, 7.3I.2 |  |
| 25 | FDD Band n71 | 38.101-1, 7.3I.2 |  |
| 26 | FDD Band n74 | 38.101-1, 7.3I.2 |  |
| 27 | FDD Band n85 | 38.101-1, 7.3I.2 |  |
| 28 | FDD Band n91 | 38.101-1, 7.3I.2 |  |
| 29 | FDD Band n92 | 38.101-1, 7.3I.2 |  |
| 30 | FDD Band n93 | 38.101-1, 7.3I.2 |  |
| 31 | FDD Band n94 | 38.101-1, 7.3I.2 |  |
| 32 | FDD Band n100 | 38.101-1, 7.3I.2 | NOTE 2 |
| 33 | TDD Band n101 | 38.101-1, 7.3I.2 |  |
| 34 | TDD Band n104 | 38.101-1, 7.3I.2 |  |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/1. NOTE 2: HD-FDD is not supported. |  |  |  |

Table A.4.3.9-5: Beam Peak Search Vendor Declarations with respect to test frequency range for single CC

| Item | Band | Intent | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | n257 | n257 single CC beam peak is leveraged from mid to low and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 2 | n258 | n258 single CC beam peak is leveraged from mid to low and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1. |
| 3 | n260 | n260 single CC beam peak is leveraged from mid to low and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 4 | n261 | n261 single CC beam peak is leveraged from mid to low and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 5 | n261 | n261 single CC beam peak is leveraged from n257 single CC mid channel to n261 low, mid and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 2 |
| NOTE 1: The beam peak searches shall be performed for every test frequency range by default unless the device manufacturer explicitly declares that the beam peak at the mid test frequency range is applicable for the remaining (low, high) test frequency ranges. NOTE 2: Beam peak search results can be re-used from bands that completely contain the target bands if explicitly declared by the manufacturer. |  |  |  |  |  |

Table A.4.3.9-6: Beam Peak Search Vendor Declarations with respect to test frequency range for different CA BW classes

| Item | Bands | NR CA bandwidth class | Intent | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258, n260, n261 | A, B, C, D, E, F, G, H, I, J, K, L, M, O, P, Q | The beam peak is leveraged from a reference (frequency band, CBW) or (frequency band combination, CA BW class) to a group of other intra-band contiguous combinations and CA BW classes | 38.521-2, K.1.1 & K.1.2 | Rel-15 | A beam peak search shall be performed for every intra-band contiguous combination and CA BW class by default unless the device manufacturer explicitly declares that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes. |

Table A.4.3.9-7: Beam Peak Search Vendor Declarations with respect to modulation for single CC

| Item | Band | Intent | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | n257 | n257 single CC beam peak is leveraged from QPSK modulation to 16QAM and 64QAM | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 2 | n258 | n258 single CC beam peak is leveraged from QPSK modulation to 16QAM and 64QAM | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 3 | n260 | n260 single CC beam peak is leveraged from QPSK modulation to 16QAM and 64QAM | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 4 | n261 | n261 single CC beam peak is leveraged from QPSK modulation to 16QAM and 64QAM | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| NOTE 1: The beam peak searches shall be performed for every modulation by default unless the device manufacturer explicitly declares that the beam peak at the QPSK modulation is applicable for the remaining 16QAM and 64QAM modulations. |  |  |  |  |  |

Table A.4.3.9-8: Beam Peak Search Vendor Declarations with respect to waveform for single CC

| Item | Band | Intent | Reference Waveform | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | n257 | n257 single CC beam peak is leveraged from the reference waveform to the other waveform | CP-OFDM or DFT-s-OFDM | 38.521-2, K.1.1 | Rel-15 | NOTE 1 |
| 2 | n258 | n258 single CC beam peak is leveraged from the reference waveform to the other waveform | CP-OFDM or DFT-s-OFDM | 38.521-2, K.1.1 | Rel-15 | NOTE 1 |
| 3 | n260 | n260 single CC beam peak is leveraged from the reference waveform to the other waveform | CP-OFDM or DFT-s-OFDM | 38.521-2, K.1.1 | Rel-15 | NOTE 1 |
| 4 | n261 | n261 single CC beam peak is leveraged from the reference waveform to the other waveform | CP-OFDM or DFT-s-OFDM | 38.521-2, K.1.1 | Rel-15 | NOTE 1 |
| NOTE 1: The beam peak searches shall be performed for every waveform by default unless the device manufacturer explicitly declares that the beam peak from one waveform is applicable for the other waveform. |  |  |  |  |  |  |

Table A.4.3.9-9: Reference Point Vendor Declaration for grey-box test approach

| Item | Band | Positioning Reference Point: Offset (x/y/z) from geometric centre of DUT [cm] | Minimum QZ required to contain all active antennas within the quiet zone | Ref. | Release |
| --- | --- | --- | --- | --- | --- |
| 1 | n257 |  |  | 38.508-1, B.2.2.2 | Rel-15 |
| 2 | n258 |  |  | 38.508-1, B.2.2.2 | Rel-15 |
| 3 | n260 |  |  | 38.508-1, B.2.2.2 | Rel-15 |
| 4 | n261 |  |  | 38.508-1, B.2.2.2 | Rel-15 |
| NOTE: The available QZ sizes are defined in TS 38.508-1, Clause B.2.2.2, i.e., 20cm, 30cm, 40cm, and 55cm |  |  |  |  |  |

Table A.4.3.9-10: Vendor Declarations with respect to PC3 antenna configuration

| Item | Band | Intent | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | n257 | n257 PC3 measurement grids can be relaxed based on 4x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 1 |
| 2 | n258 | n258 PC3 measurement grids can be relaxed based on 4x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 1 |
| 3 | n260 | n260 PC3 measurement grids can be relaxed based on 4x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 1 |
| 4 | n261 | n261 PC3 measurement grids can be relaxed based on 4x2 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 1 |
| NOTE 1: The fine PC3 measurement grids based on the 8x2 worst case configuration shall be applied by default unless the device manufacturer explicitly declares that all antenna arrays with M x N (M ≥ N) comply with M ≤ 4 and N ≤ 2 for each band. |  |  |  |  |  |

Table A.4.3.9-10a: Vendor Declarations with respect to PC5 antenna configuration

| Item | Band | Intent | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | n257 | n257 PC5 measurement grids can be relaxed based on 6x6 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 1 |
| 2 | n258 | n258 PC5 measurement grids can be relaxed based on 6x6 worst case antenna array configuration | 38.521-2, M.2 – M.4 | Rel-15 | NOTE 1 |
| NOTE 1: The fine PC5 measurement grids based on the 12x12 worst case configuration shall be applied by default unless the device manufacturer explicitly declares that all antenna arrays with M x N (M ≥ N) comply with M ≤ 6 and N ≤ 6 for each band. |  |  |  |  |  |

Table A.4.3.9-11: Antenna Aperture Vendor Declaration

| Item | Band | Antenna Aperture Declaration | Ref. | Release |
| --- | --- | --- | --- | --- |
| 1 | n257 | Maximum radiating aperture of any of the panels integrated in the DUTs is ≤5cm | 38.508-1, B.2 | Rel-15 |
| 2 | n258 | Maximum radiating aperture of any of the panels integrated in the DUTs is ≤5cm | 38.508-1, B.2 | Rel-15 |
| 3 | n260 | Maximum radiating aperture of any of the panels integrated in the DUTs is ≤5cm | 38.508-1, B.2 | Rel-15 |
| 4 | n261 | Maximum radiating aperture of any of the panels integrated in the DUTs is ≤5cm | 38.508-1, B.2 | Rel-15 |

Table A.4.3.9-12: NR FR1 UL MIMO Capabilities

| Item |  | RF Baseline Implementation Capabilities |  | Ref. |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | NR Frequency band: 1920-1980 MHz, 2110-2170 MHz |  | 38.101-1, 5.2D |  | NR FDD FR1 Band 1 |  |
| 2 |  | NR Frequency band: 1850-1910 MHz, 1930-1990 MHz |  | 38.101-1, 5.2D |  | NR FDD FR1 Band 2 |  |
| 3 |  | NR Frequency band: 1710-1785 MHz, 1805-1880 MHz |  | 38.101-1, 5.2D |  | NR FDD FR1 Band 3 |  |
| ... |  |  |  |  |  |  |  |
| 7 |  | NR Frequency band: 832-862 MHz, 791-821 MHz |  | 38.101-1, 5.2D |  | NR FDD FR1 Band 7 |  |
| ... |  |  |  |  |  |  |  |
| 25 |  | NR Frequency band: 1850-1915 MHz, 1930-1995 MHz |  | 38.101-1, 5.2D |  | NR FDD FR1 Band 25 |  |
| ... |  |  |  |  |  |  |  |
| 301 |  | NR Frequency band: 2305-2315 MHz, 2350-2360 MHz |  | 38.101-1, 5.2D |  | NR FDD FR1 Band 30 |  |
| ... |  |  |  |  |  |  |  |
| 34 |  | NR Frequency band: 2010-2025 MHz |  | 38.101-1, 5.2D |  | NR TDD FR1 Band 34 |  |
| ... |  |  |  |  |  |  |  |
| 38 |  | NR Frequency band: 2570-2620 MHz |  | 38.101-1, 5.2D |  | NR TDD FR1 Band 38 |  |
| 39 |  | NR Frequency band: 1880-1920 MHz |  | 38.101-1, 5.2D |  | NR TDD FR1 Band 39 |  |
| 40 |  | NR Frequency band: 2300-2400 MHz |  | 38.101-1, 5.2D |  | NR TDD FR1 Band 40 |  |
| 41 |  | NR Frequency band: 2496-2690 MHz |  | 38.101-1, 5.2D |  | NR TDD FR1 Band 41 |  |
| ... |  |  |  |  |  |  |  |
| 46 |  | NR Frequency band: 5150-5925 MHz |  | 38.101-1, 5.2D |  | NR TDD FR1 Band 46 |  |
| ... |  |  |  |  |  |  |  |
| 48 |  | NR Frequency band: 3550-3700 MHz |  | 38.101-1, 5.2D |  | NR TDD FR1 Band 48 |  |
| ... |  |  |  |  |  |  |  |
| 66 |  | NR Frequency band: 1710-1780, 2110-2200 MHz |  | 38.101-1, 5.2D |  | NR FDD FR1 Band 66 |  |
| ... |  |  |  |  |  |  |  |
| 70 |  | NR Frequency band: 1695-1710, 1995-2020 MHz |  | 38.101-1, 5.2D |  | NR FDD FR1 Band 70 |  |
| 712 |  | NR Frequency band: 663-698 MHz, 617-652 MHz |  | 38.101-1, 5.2D |  | NR FDD FR1 Band 71 |  |
| ... |  |  |  |  |  |  |  |
| 77 |  | NR Frequency band: 3300–4200 MHz |  | 38.101-1, 5.2D |  | NR TDD FR1 Band 77 |  |
| 78 |  | NR Frequency band: 3300–3800 MHz |  | 38.101-1, 5.2D |  | NR TDD FR1 Band 78 |  |
| 79 |  | NR Frequency band: 4400–5000 MHz |  | 38.101-1, 5.2D |  | NR TDD FR1 Band 79 |  |
| 95 |  | NR Frequency band: 2010–2025 MHz |  | 38.101-1, 5.2D |  | NR SUL FR1 Band 95 |  |
| 97 |  | NR Frequency band: 2300–2400 MHz |  | 38.101-1, 5.2D |  | NR SUL FR1 Band 97 |  |
| 98 |  | NR Frequency band: 1880–1920 MHz |  | 38.101-1, 5.2D |  | NR SUL FR1 Band 98 |  |
| 99 |  | NR Frequency band: 1626.5–1660.5 MHz |  | 38.101-1, 5.2D |  | NR SUL FR1 Band 99 |  |
| NOTE 1: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. NOTE 2: UL MIMO is targeted for FWA form factor. |  |  |  |  |  |  |  |

Table A.4.3.9-13: NR FR2 UL MIMO Capabilities

| Item | RF Baseline Implementation Capabilities | Ref. | Comments |
| --- | --- | --- | --- |
| 257 | NR Frequency band: 26500-29500 MHz | 38.101-2, 5.2D | NR TDD FR2 Band 257 |
| 258 | NR Frequency band: 24250-27500 MHz | 38.101-2, 5.2D | NR TDD FR2 Band 258 |
| 259 | NR Frequency band: 39500-43500 MHz | 38.101-2, 5.2D | NR TDD FR2 Band 259 |
| 260 | NR Frequency band: 37000–40000 MHz | 38.101-2, 5.2D | NR TDD FR2 Band 260 |
| 261 | NR Frequency band: 27500–28350 MHz | 38.101-2, 5.2D | NR TDD FR2 Band 261 |

<<< Skip unchanged sections >>>

<<< END OF CHANGES >>>