3GPP TSG-RAN5 Meeting #98 	R5-230211r2

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0878 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of PC1 MU and TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MU and TT are not defined for PC1 in some test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following tables were corrected/added to define TT for PC1 FR2a, and corresponding editor’s note was updated accordingly. - Table 6.2.1.2.5-1a for MOP (Sphericla coverage) - Table 6.2.2.5-1b for MPR - Table 6.3.1.5-1 for Minimum output power - Table 6.4.2.1.5-4 for EVM (PUSCH) - Table 6.5.2.3.5-1d for ACLR - Table 7.3.4.5-1a for EIS spherical coverage Editor’s note in the following test cases were updated to complete PC1 FR2a. - 6.5.1 OBW - 6.5.3.1 General Tx Spurious - 6.5.3.2 Spurious UE co-existence - 6.5.3.3 Additional spurious - 7.9 Rx Spurious MTSU for MOP (Spherical coverage), MPR, Minimum output power, EVM (PUSCH) for FR2a and Rx Spurious (66 GHz – 80 GHz), OBW, Additional spurious, EIS spherical coverage was defined for PC1 FR2a in Annex F.1. [ ] was removed from MTSU of ACLR, General Tx Spurious, Spurious UE co-existence, Rx Spurious for PC1 FR2a in Annex F.1. TT for MOP (Spherical coverage), MPR, Minimum output power, EVM (PUSCH), ACLR, EIS spherical coverage was defined for PC1 FR2a in Annex F.3. Typo corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases cannot be completed for PC1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.2, 6.2.2, 6.3.1, 6.4.2.1, 6.5.1, 6.5.2.3, 6.5.3.1, 6.5.3.1_1, 6.5.3.2, 6.5.3.3, 7.3.4, 7.9, F.1.2, F.1.3, F.3.1, F.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on the outcome of MU discussion R5-230209 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: - Changes on MOP (Sphericla coverage), MPR, Minimum output power, OBW, ACLR, General Tx Spurious, Spurious UE co-existence, Additional spurious, EIS spherical coverage were removed to solve overlaps with R5-230162 to R5-230167, R5-230169, R5-230225. - Editorial correction to 6.5.3.1_1 was removed to solve overlaps with R5-230222 and R5-230564. - Changes in 7.9 were removed. r2: PC1 MTSU of Rx Spurious (66 GHz to 80 GHz) was defined. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.4.2.1	Error vector magnitude

Editor’s note:

- This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainty and Test Tolerance are FFS except for PUSCH, PC1 in FR2a, PC3 in FR2a and FR2b.

- For a transition period until RAN#102 meeting (Dec 2023), the implementation of note 4 in Table 6.4.2.1.4.1-1 in test equipment is not applicable to avoid lack of test coverage until testcase 6.4.2.1_1 is available.

<<Unchanged sections skipped>>

6.4.2.1.5	Test requirement

The PUSCH EVM, derived in Annex E.4.2, shall not exceed the values in Table 6.4.2.1.5-1.

The PUSCH, derived in Annex E.4.6.2, shall not exceed the values in Table 6.4.2.1.5-1 when embedded with data symbols of the respective modulation scheme.

The PUCCH EVM derived in Annex E.5.9.2 shall not exceed the values for QPSK in Table 6.4.2.1.5-1.

The PRACH EVM derived in Annex E.6.9.2 shall not exceed the values for QPSK in Table 6.4.2.1.5-1.

Table 6.4.2.1.5-1: Test requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level |
| --- | --- | --- | --- |
| Pi/2 BPSK | % | 30+TT | 30+TT |
| QPSK | % | 17.5+TT | 17.5+TT |
| 16 QAM | % | 12.5+TT | 12.5+TT |
| 64 QAM | % | 8+TT | 8+TT |

Table 6.4.2.1.5-2: Test Tolerance (TT) for PUSCH, PC3, FR2a

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 1.61% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 2.18% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 0.00% | 0.00% | 1.53% | 4.29% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 0.00% | 0.00% | 1.67% | 4.29% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 1.06% | 1.97% | 3.61% | NA |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 1.44% | 2.68% | NA | NA |
| 9 | CP-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 3.66% |
| 10 | CP-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 1.37% | 3.66% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 0.00% | 1.35% | 2.57% | NA |
| 12 | CP-OFDM 16 QAM | Outer_Full | 0.00% | 1.35% | 2.57% | NA |
| 13 | CP-OFDM 64 QAM | Inner_Full | 2.19% | 3.97% | NA | NA |
| 14 | CP-OFDM 64 QAM | Outer_Full | 2.19% | 3.97% | NA | NA |
| NOTE 1: Test combinations without TT defined must be skipped as not testable. |  |  |  |  |  |  |

Table 6.4.2.1.5-3: Test Tolerance (TT) for PUSCH, PC3, FR2b

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 2.50% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 1.31% | 2.49% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 1.79% | 4.01% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 0.00% | 1.48% | 2.85% | NA |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 1.00% | 1.92% | 3.60% | NA |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 2.49% | NA | NA | NA |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 3.35% | NA | NA | NA |
| 9 | CP-OFDM QPSK | Inner_Full | 0.00% | 1.42% | 2.73% | 8.42% |
| 10 | CP-OFDM QPSK | Outer_Full | 0.00% | 1.58% | 3.04% | 8.42% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 1.72% | 3.25% | 5.92% | NA |
| 12 | CP-OFDM 16 QAM | Outer_Full | 1.72% | 3.25% | 5.92% | NA |
| 13 | CP-OFDM 64 QAM | Inner_Full | NA | NA | NA | NA |
| 14 | CP-OFDM 64 QAM | Outer_Full | NA | NA | NA | NA |
| NOTE 1: Test combinations without TT defined must be skipped as not testable. |  |  |  |  |  |  |

Table 6.4.2.1.5-4: Test Tolerance (TT) for PUSCH, PC1, FR2a

| Test ID | Modulation | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- |
| 1-2 | PI/2 BPSK | 0.00% | 0.00% | 0.00% | 0.00% |
| 3-4, 9-10 | QPSK | 0.00% | 0.00% | 0.00% | 1.35% |
| 5-6, 11-12 | 16 QAM | 0.00% | 0.00% | 0.94% | 1.83% |
| 7-8, 13,14 | 64 QAM | 0.00% | 0.73% | 1.41% | 2.63% |
| NOTE 1: Test combinations without TT defined must be skipped as not testable. |  |  |  |  |  |

<<Unchanged sections skipped>>

Annex F (normative):
Measurement uncertainties and Test Tolerances

F.1	Acceptable uncertainty of Test System (normative)

The maximum acceptable uncertainty of the Test System is specified below for each test, where appropriate. The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified range, and the equipment under test to be measured with an uncertainty not exceeding the specified values. Care should be taken to ensure that each conformance test implementation including the OTA chamber aspects meets the specified measurement uncertainty for each test case by requiring the test laboratory to maintain a detailed measurement uncertainty test report showing compliance to all the measurement uncertainty requirements. The detailed measurement uncertainty report would contain the justification for each measurement uncertainty component and its value and distribution. The derivation of these values is based on the minimum conformance requirements plus relaxation, i.e., test tolerance is not to be considered. All ranges and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

The downlink signal uncertainties apply at the defined quiet zone with the UE properly positioned in the quiet zone. The uplink signal uncertainties apply at the measurement equipment with the UE positioned properly in the quiet zone.

F.1.1	Measurement of test environments

Editor’s note: Various measurement accuracies for UE test environments, e.g., pressure, relative humidity, DC&AC voltage, vibration, and vibration frequency, are FFS:

The measurement accuracy of the UE test environments defined in TS 38.508-1 [5] subclause 4.1, Test environments shall be

-	Temperature			±4 degrees.

The above values shall apply unless the test environment is otherwise controlled and the specification for the control of the test environment specifies the uncertainty for the parameter.

F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| <<Unchanged rows omitted>> |  |  |
| 6.4.2.1 Error vector magnitude | PUSCH, PC3, FR2a: As defined in Table F.1.2-2. PUSCH, PC3, FR2b: As defined in Table F.1.2-3. PUSCH, PC1, FR2a: ±2.48 [%CBW] (BW 50MHz) ±3.50 [%CBW] (BW 100MHz) ±4.95 [%CBW] (BW 200MHz) ±7.00 [%CBW] (BW 400MHz) Otherwise: TBD |  |
| <<Unchanged sections skipped>> |  |  |
| NOTE 1: FR2a: 23.45GHz ≤ f ≤ 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz FR2c: 40.8GHz ≤ f ≤ 44.3GHz |  |  |

Table F.1.2-2: EVM Measurement Uncertainty (MU) for PUSCH, PC3, FR2a (23.45GHz <= f <= 32.125GHz)

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 2.78% | 3.85% | 5.44% | 7.69% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 3.10% | 4.16% | 5.88% | 8.99% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 2.78% | 3.85% | 5.44% | 7.69% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 3.10% | 4.16% | 5.88% | 8.99% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 3.31% | 4.50% | 6.36% | 11.21% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 3.60% | 4.73% | 6.68% | 11.21% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 4.26% | 5.96% | 8.41% | 15.84% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 5.01% | 7.08% | 9.99% | 15.84% |
| 9 | CP-OFDM QPSK | Inner_Full | 3.60% | 4.73% | 6.68% | 11.89% |
| 10 | CP-OFDM QPSK | Outer_Full | 3.71% | 4.99% | 7.07% | 11.89% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 4.26% | 5.96% | 8.41% | 15.84% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 4.26% | 5.96% | 8.41% | 15.84% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 6.31% | 8.91% | 12.59% | 21.13% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 6.31% | 8.91% | 12.59% | 21.13% |

Table F.1.2-3: EVM Measurement Uncertainty (MU) for PUSCH, PC3, FR2b (32.125GHz < f <= 40.8GHz)

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 3.56% | 4.83% | 6.91% | 9.65% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 4.15% | 5.69% | 8.11% | 12.50% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 3.56% | 4.83% | 6.91% | 9.65% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 4.15% | 5.69% | 8.11% | 12.50% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 4.54% | 6.26% | 8.91% | 18.06% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 5.09% | 7.19% | 10.15% | 18.06% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 6.78% | 9.58% | 13.54% | 25.50% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 8.06% | 11.38% | 16.09% | 25.50% |
| 9 | CP-OFDM QPSK | Inner_Full | 5.09% | 7.19% | 10.15% | 19.13% |
| 10 | CP-OFDM QPSK | Outer_Full | 5.39% | 7.61% | 10.75% | 19.13% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 6.78% | 9.58% | 13.54% | 25.50% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 6.78% | 9.58% | 13.54% | 25.50% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 10.14% | 14.33% | 20.25% | 34.01% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 10.14% | 14.33% | 20.25% | 34.01% |

F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty (MTSU) for receiver tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| <<Unchanged sections skipped>> |  |  |
| 7.9 Spurious emissions | Max Device size ≤ 30 cm Maximum in-band BW ≤ 400MHz PC3: For Band n257, n258, n260, n261: ±5.50dB (6GHz ≤ f < 12.75GHz) ±5.46dB (12.75GHz ≤ f < 23.45GHz) ±6.11dB (23.45GHz ≤ f < 40.8GHz) ±7.65dB (40.8GHz ≤ f < 66GHz) ±7.95 dB (66GHz ≤ f ≤ 80GHz) PC1: For Band n257, n258, n260, n261: ±5.63dB (6GHz ≤ f < 12.75GHz) ±5.59dB (12.75GHz ≤ f < 23.45GHz) ±6.10dB (23.45GHz ≤ f < 40.8GHz) ±7.64dB (40.8GHz ≤ f < 66GHz) ±7.95 dB (66GHz ≤ f ≤ 80GHz) | MTSU = 1.00 x MU (from Table B.25-1 in TR 38.903) |
| NOTE 1: FR2a, FR2b and FR2c are specified in Table F.1.2-1. |  |  |

F.2	Interpretation of measurement results (normative)

The actual measurement uncertainty of the Test System for the measurement of each parameter shall be included in the test report.

The recorded value for the Test System uncertainty shall be, for each measurement, equal to or lower than the appropriate figure in clause F.1 of the present document.

If the Test System using one of the permitted test methods defined in TR38.903 [20] for a test is known to have a measurement uncertainty greater than that specified in clause F.1, it is still permitted to use this apparatus provided that an adjustment is made value as follows:

Any additional uncertainty in the Test System over and above that specified in clause F.1 shall be used to tighten the Test Requirement, making the test harder to pass. For some tests, for example receiver tests, this may require modification of stimulus signals. This procedure will ensure that a Test System not compliant with clause F.1does not increase the chance of passing a device under test where that device would otherwise have failed the test if a Test System compliant with clause F.1 had been used.

F.3	Test Tolerance and Derivation of Test Requirements (informative)

TBD

F.3.1	Measurement of test environments

TBD

F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Influence of noise is subtracted from MTSU before calculating the TT for lower limit Tx test cases.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| <<Unchanged rows omitted>> |  |  |
| 6.4.2.1 Error vector magnitude | PUSCH, PC3, FR2a: As defined in Table 6.4.2.1.5-2. PUSCH, PC3, FR2b: As defined in Table 6.4.2.1.5-3. PUSCH, PC3, FR2a: As defined in Table 6.4.2.1.5-4. PUSCH, PC3, FR2b: TBD | Minimum requirement + TT EVM_meas_Increase = sqrt(Minimum requirement^2 + MTSU^2) - Minimum requirement; it is the increase of measured EVM due to test equipment uncertainty. EVM_meas_Increase_Relative = EVM_meas_Increase / Minimum requirement [%] If (EVM_meas_Increase_Relative < 7.5%) TT = 0% Else if (7.5% ≤ EVM_meas_Increase_Relative ≤ 50%) TT = EVM_meas_Increase Else Skip the test as not testable. |
| <<Unchanged rows omitted>> |  |  |
| NOTE 1: FR2a: 23.45GHz ≤ f ≤ 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz FR2c: 40.8GHz ≤ f ≤ 44.3GHz |  |  |

<<End of change>>