3GPP TSG-RAN5 Meeting #98	   R5-230270r1

# Athens, Greece, 27th Feb – 03rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2695 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of CG SDT Configuration message contents for 3GPP SDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SDT Information Elements values of CG-SDT-Configuration-r17 are missing in ConfiguredGrantConfig IE, for 3GPP SDT test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | ##### Updated SDT Information elements values of CG-SDT-Configuration-r17 inside ConfiguredGrantConfig IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the work plan will remain unfulfilled |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Secretary to resolve the “xxy” for the new Table for CG-SDT-Configuration-r17 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-230270r1 – removed second condition from BWP-UplinkDedicated table. Also, removed CG_Config_Type2 condition from table 4.6.3-26. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

#### –	BWP-UplinkDedicated

Table 4.6.3-15: BWP-UplinkDedicated

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| Setup | PUCCH-Config |  |  |
| } |  |  |  |
| pucch-Config | Not present |  | SUL_NUL, RF AND Scell_Add |
| pusch-Config | Not present |  | RESUME |
| pusch-Config CHOICE { |  |  |  |
| Setup | PUSCH-Config |  |  |
| } |  |  |  |
| configuredGrantConfig | Not present |  |  |
|  | ConfiguredGrantConfig |  | CG_Config |
| srs-Config | Not present |  | Short_DCI |
|  | SRS-Config |  |  |
| beamFailureRecoveryConfig | Not present |  |  |
| sl-PUCCH-Config-r16 | Not present |  |  |
| sl-PUCCH-Config-r16 CHOICE { |  |  | SIDELINK |
| Setup | PUCCH-Config |  |  |
| } |  |  |  |
| cp-ExtensionC2-r16 | Not present |  |  |
| cp-ExtensionC3-r16 | Not present |  |  |
| useInterlacePUCCH-PUSCH-r16 | Not present |  |  |
| pucch-ConfigurationList-r16 | Not present |  |  |
| lbt-FailureRecoveryConfig-r16 | Not present |  |  |
| configuredGrantConfigToAddModList-r16 | Not present |  |  |
| configuredGrantConfigToReleaseList-r16 | Not present |  |  |
| configuredGrantConfigType2DeactivationStateList-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Short_DCI | Used in test scenarios requiring DCI formats 0-0 and 1-0 on USS |
| SUL_NUL | On the NUL carrier when supplementary carrier is configured |
| RESUME | Used in RRCResume Message |
| SIDELINK | Used for sidelink communication |
| SCell_add | Add Scell |
| CG_Config | Configured Grant Configuration |

<<< End OF CHANGES >>>

<<< Sections skipped >>>

<<< Start OF CHANGES >>>

#### –	ConfiguredGrantConfig

Table 4.6.3-26: ConfiguredGrantConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| frequencyHopping | Not present |  |  |
| cg-DMRS-Configuration | DMRS-UplinkConfig |  |  |
| mcs-Table | Not present |  |  |
| mcs-TableTransformPrecoder | Not present |  |  |
| uci-OnPUSCH | Not present |  |  |
| resourceAllocation | resourceAllocationType1 |  |  |
| rbg-Size | Not present |  |  |
| powerControlLoopToUse | n0 |  |  |
| p0-PUSCH-Alpha | 0 |  |  |
| transformPrecoder | Not present |  |  |
| nrofHARQ-Processes | 16 |  |  |
| repK | n1 |  |  |
| repK-RV | Not present |  |  |
| periodicity | Sym40x14 | 40ms | SCS15 |
|  | Sym80x14 | 40ms | SCS30 |
|  | Sym160x14 | 40ms | SCS60 |
|  | Sym320x14 | 40ms | SCS120 |
| configuredGrantTimer | Not present |  |  |
| rrc-ConfiguredUplinkGrant | Not present |  |  |
| rrc-ConfiguredUplinkGrant SEQUENCE { |  |  | CG_Config_Type1 |
| timeDomainOffset | 9 |  |  |
| timeDomainAllocation | 0 |  |  |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | Equals to NBWPsize * (NBWPsize – LRB + 1) + (NBWPsize -1 - RBstart), where LRB = 24 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in TS.38.508-1 [4] clause 6.2.3.1 |  |
|  | BIT STRING (SIZE(18) | Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 24 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in TS.38.508-1 [4] clause 6.2.3.1 |  |
| antennaPort | 2 |  |  |
| dmrs-SeqInitialization | Not present |  |  |
| precodingAndNumberOfLayers | 1 |  |  |
| srs-ResourceIndicator | Not present |  |  |
| mcsAndTBS | 1 |  |  |
| frequencyHoppingOffset | Not present |  |  |
| pathlossReferenceIndex | 0 |  |  |
| pusch-RepTypeIndicator-r16 | Not present |  |  |
| frequencyHoppingPUSCH-RepTypeB-r16 | Not present |  |  |
| timeReferenceSFN-r16 | Not present |  |  |
| pathlossReferenceIndex2-r17 | Not present |  |  |
| srs-ResourceIndicator2-r17 | Not present |  |  |
| precodingAndNumberOfLayers2-r17 | Not present |  |  |
| timeDomainAllocation-v1710 | Not present |  |  |
| timeDomainOffset-r17 | Not present |  |  |
| cg-SDT-Configuration-r17 | Not present |  |  |
|  | CG-SDT-Configuration-r17 |  | pc_cg_SDT_r17 |
| } |  |  |  |
| cg-RetransmissionTimer-r16 | Not present |  |  |
| cg-minDFI-Delay-r16 | Not present |  |  |
| cg-nrofPUSCH-InSlot-r16 | Not present |  |  |
| cg-nrofSlots-r16 | Not present |  |  |
| cg-StartingOffsets-r16 | Not present |  |  |
| cg-UCI-Multiplexing-r16 | Not present |  |  |
| cg-COT-SharingOffset-r16 | Not present |  |  |
| betaOffsetCG-UCI-r16 | Not present |  |  |
| cg-COT-SharingList-r16 | Not present |  |  |
| harq-ProcID-Offset-r16 | Not present |  |  |
| harq-ProcID-Offset2-r16 | Not present |  |  |
| configuredGrantConfigIndex-r16 | Not present |  |  |
| configuredGrantConfigIndexMAC-r16 | Not present |  |  |
| periodicityExt-r16 | Not present |  |  |
| startingFromRV0-r16 | Not present |  |  |
| phy-PriorityIndex-r16 | Not present |  |  |
| autonomousTx-r16 | Not present |  |  |
| cg-betaOffsetsCrossPri0-r17 | Not present |  |  |
| cg-betaOffsetsCrossPri1-r17 | Not present |  |  |
| mappingPattern-r17 | Not present |  |  |
| sequenceOffsetForRV-r17 | Not present |  |  |
| p0-PUSCH-Alpha2-r17 | Not present |  |  |
| powerControlLoopToUse2-r17 | Not present |  |  |
| cg-COT-SharingList-r17 | Not present |  |  |
| periodicityExt-r17 | Not present |  |  |
| repK-v1710 | Not present |  |  |
| nrofHARQ-Processes-v1700 | Not present |  |  |
| harq-ProcID-Offset2-v1700 | Not present |  |  |
| configuredGrantTimer-v1700 | Not present |  |  |
| cg-minDFI-Delay-v1710 | Not present |  |  |
| harq-ProcID-Offset-v1730 | Not present |  |  |
| cg-nrofSlots-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CG_Config_Type1 | Configured Grant Configuration Type1 |

#### –	CG-SDT-Configuration-r17

Table 4.6.3-xxy: CG-SDT-Configuration-r17

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CG-SDT-Configuration-r17 ::= SEQUENCE { |  |  |  |
| cg-SDT-RetransmissionTimer | Not present |  |  |
| sdt-SSB-Subset-r17 | Not present | If this field is absent, UE assumes the SSB set includes all actually transmitted SSBs configured by SIB1. |  |
| sdt-SSB-PerCG-PUSCH-r17 | one |  |  |
| sdt-P0-PUSCH-r17 | 0 |  |  |
| sdt-Alpha-r17 | alpha08 |  |  |
| sdt-DMRS-Ports-r17 CHOICE { |  |  |  |
| dmrsType1-r17 | ’100000000000’B | Port0 |  |
| } |  |  |  |
| sdt-NrofDMRS-Sequences-r17 | 1 |  |  |
| } |  |  |  |

<<< END OF CHANGES >>>