3GPP TSG-RAN5 Meeting #98	R5-230447r1

# Athens, Greece, 27th Feb 2023 – 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2164 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test case 5.5.3.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | ENDC FR2 direct Scell activation has been defined in 38.133 but is missing in RAN5 test specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added ENDC FR2 test case 5.5.3.7. - Added cell configuration mapping in E.3 for above test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case would be missing in the test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3, E.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.522 CR 0241 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Updated the title of CR. Updated clauses affected. Updated other specs affected. Corrected cell mapping configuration as SCell FR2 is intra-band |  |  |  |  |  |  |  |  |

## 5.5	Signaling characteristics

### 5.5.3	SCell activation and deactivation delay

-------- Skipped unchanged clauses --------

----- start of first change --------

#### 5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2

Editor’s note: This test case is incomplete. The following aspects are missing

- Test Tolerances and measurement uncertainty are missing

- CSI-RS config is missing

5.5.3.7.1		Test Purpose

The purpose of this test is to verify that the direct SCell activation time is within the requirements, when the SCell in FR2 is known by the UE at the time of activation.

5.5.3.7.2		Test applicability

This test applies to all types of NR UE supporting E-UTRA and EN-DC from Release 15 onwards and supporting 2DL CA.

5.5.3.7.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 4.5.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause 8.3 and A.5.5.3.7.

5.5.3.7.4		Test description

5.5.3.7.4.1		Initial Conditions

This test shall be tested using any of the test configurations in Table 5.5.3.7.4.1-1.

Table 5.5.3.7.4.1-1: Supported test configurations for FR2 SCell activation case with FR2 PSCell

| Test Case ID | Description |
| --- | --- |
| 5.5.3.7-1 | FDD LTE PCell, Cell 2&3 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 5.5.3.7-2 | TDD LTE PCell, Cell 2&3 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to pass in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 5.5.3.7.4.1-2 and Table 5.5.3.7.4.1-3.

Table 5.5.3.7.4.1-2: Initial conditions for known FR2 SCell activation case with FR2 PSCell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.1-1 and TS 38.508-1 [14] clause 4.3.1 for E-UTRA and 7.2.3 for NR. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.7.1.1.2-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 5.5.3.7.4.1-3: General test parameters for FR2 SCell activation case with FR2 PSCell

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2, 3 | Two radio channels are used for this test. One for E-UTRA cell and two for NR Cell |
| Active PCell |  | Cell1 | PCell on RF channel number 1. As specified in clause A.3.7.2.2 |
| Active PSCell |  | Cell2 | PSCell on RF channel number 2. |
| SCell |  | Cell3 | secondary cell on RF channel number 3 |
| DRX |  | OFF | Continuous monitoring of PCell/PSCell |
| PRACH configuration on cell2 |  | FR2 configuration 2 | Captured in A.3.8.3.2 |
| PSCell CQI/PMI periodicity and offset configuration index |  | slot5 | CQI reporting for PSCell every uplink slot |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on carrier frequency of cell1. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on carrier frequency of cell2. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on carrier frequency of cell3. |
| T1 | s | 7 | During this time the PCell/PSCell shall be known and cell3 is detected, and UE shall report a valid CQI for PCell/PSCell. |
| T2 | s | Ndirect | During this time the UE shall be configured with directly activated SCell1. |
| T3 | s | 1 | During this time the UE shall report a valid CQI for PCell/PSCell and SCell. |

1.	Message contents are defined in clause 5.5.3.7.4.3.

2.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 and Cell 3 are NR FR2 cells in the same frequency. Cell 2 is the PSCell and Cell 3 is the SCell.

5.5.3.7.4.2	Test procedure

Same as in 4.5.3.5.4.2 with the following exception:

2.	Set the parameters according to T1 in Tables 5.5.3.7.5-1 and A.6.1.2-1. Propagation conditions are set according to Annex C clauses C.2.2.

3.	T1 starts. Immediately after, the SS shall configure SCell (Cell 3) on the SCC as per TS 38.508-1 [14] clause 7.5.2, with the message content exceptions defined in clause 5.5.3.7.4.3. NR RRCReconfiguration message is contained in RRCConnectionReconfiguration and NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete.

5.5.3.7.4.3		Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the exception as described in section 4.5.3.5.4.3.

5.5.3.7.5		Test requirement

Table 5.5.3.7.5-1 defines the cell specific test parameters and Table 5.5.3.7.5-2 defines OTA related test parameters.

Table 5.5.3.7.5-1: Cell specific test parameters for FR2 SCell activation case with FR2 PSCell

| ParameterNote 5 |  | Unit |  | Cell 2 |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 | T3 |  | T1 | T2 | T3 |
| SSB ARFCN |  |  | freq1 |  |  |  |  | freq2 |  |  |
| Duplex mode |  |  | TDD |  |  |  |  | TDD |  |  |
| TDD configuration |  |  | TDDConf.3.1 |  |  |  |  | TDDConf.3.1 |  |  |
| BWchannel | MHz |  | 100: NRB,c = 66 |  |  |  |  | 100: NRB,c = 66 |  |  |
| PDSCH Reference measurement channel |  |  | SR.3.1 TDD |  |  |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET Reference Channel |  |  | CR.3.1 TDD |  |  |  |  | CR.3.1 TDD |  |  |
| RMC CORESET Reference Channel |  |  | CCR.3.1 TDD |  |  |  |  | CCR.3.1 TDD |  |  |
| DL initial BWP configuration |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |
| DL dedicated BWP configuration |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |
| UL initial BWP configuration |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |
| UL dedicated BWP configuration |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |  |  |
| SSB configuration |  |  | SSB.1 FR2 |  |  |  |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB |  | 0 |  |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |
| Propagation conditions |  |  | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: All parameters apply for configuration 1 and 2. |  |  |  |  |  |  |  |  |  |  |

Table 5.5.3.7.5-2: OTA related test parameters for FR2 SCell activation case with FR2 PSCell

| ParameterNote 6 | Unit | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Angle of arrival configuration |  | Setup 1 according to A.3.15.1 |  |  |  |  |  |
| Assumption for UE beamsNote 7 |  | Rough |  |  | Rough |  |  |
| Note1 | dBm/15kHzNote4 | -104.7 |  |  | -104.7 |  |  |
| Note1 | dBm/SCSNote3 | -95.7 |  |  | -95.7 |  |  |
|  | dB | 7 |  |  | 7 |  |  |
| SS-RSRPNote2 | dBm/SCS Note4 | -88.7 |  |  | -88.7 |  |  |
|  | dB | 7 |  |  | 7 |  |  |
| IoNote2 | dBm/95.04 MHz Note4 | -58.92 |  |  | -58.92 |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: All parameters apply for configuration 1 and 2 Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

The UE shall accomplish the activation of the SCell no later than subframe m+Ndirect as defined in TS 38.133[6], clause 8.3.4. Time period T3 starts at (m+ Ndirect), at which point UE shall be reporting a valid CQI for both PSCell and SCell.

During T3 the UE shall send CSI reports for SCell with non-zero CQI index and continue to send CSI reports for SCell 1 with non-zero CQI index until the end of T3. All of the above test requirements shall be fulfilled in order for the observed SCell1 direct activation delay to be counted as correct. The rate of correct observed SCell1 direct activation delay during repeated tests shall be at least 90%.

NOTE:	In this test case the placement of interruptions is only verified in NR PSCell.

----- End of first change --------

-------- Skipped unchanged clauses --------

----- Start of second change --------

# E.3	Cell configuration mapping for EN-DC FR2 test cases in Chapter 5

Table E.3-1 defines the cell configuration mapping for EN-DC FR2 test cases in chapter 5 of this test specification.

Table E.3-1: Cell configuration mapping for EN-DC FR2 RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 5.3.2.2.1 | EN-DC FR2 contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.3.2.2.2 | EN-DC FR2 non-contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.3.2.2.3 | EN-DC FR2 2-step contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.3.2.2.4 | EN-DC FR2 2-step non-contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.4.1.1 | EN-DC FR2 UE transmit timing accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.4.3.1 | EN-DC FR2 timing advance adjustment accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.1 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 6 |  |  |  |
| 5.5.1.2 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 6 |  |  |  |
| 5.5.1.3 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 6 |  |  |  |
| 5.5.1.4 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 6 |  |  |  |
| 5.5.1.5 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.6 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.7 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.8 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.9 | EN-DC FR2 radio link monitoring UE scheduling restrictions | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.2.1 | EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.2.2 | EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.2.3 | EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 5.5.2.4 | EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 5.5.2.5 | EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 5.5.2.6 | EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 5.5.3.1 | EN-DC FR2 SCell activation and deactivation intra-band in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
| 5.5.3.7 | Direct SCell activation at SCell addition of known SCell in FR2 | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
| 5.5.5.1 | EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.2 | EN-DC FR2 SSB-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.3 | EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.4 | EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.5 | EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.6 | EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  |  |
| 5.5.5.7 | EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  |  |
| 5.5.8.1 | EN-DC FR2 MAC-CE based active TCI state switch | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.8.2 | EN-DC FR2 RRC based active TCI state switch | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.1.1 | EN-DC FR2 event-triggered reporting without gap in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.6.1.2 | EN-DC FR2 event-triggered reporting without gap in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.6.1.3 | EN-DC FR2 event-triggered reporting with gap in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.6.1.4 | EN-DC FR2 event-triggered reporting with gap in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.6.2.1 | EN-DC FR2-FR2 event-triggered reporting in non-DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.2 | EN-DC FR2-FR2 event-triggered reporting in DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.3 | EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.4 | EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.5 | EN-DC FR1-FR2 event-triggered reporting in non-DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.6 | EN-DC FR1-FR2 event-triggered reporting in DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.7 | EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.8 | EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.3.1 | EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.3.2 | EN-DC FR2 SSB-based L1-RSRP measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.3.3 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.3.4 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.6.1 | EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.6.2 | EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.6.3 | EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.7.1.1 | EN-DC FR2 SS-RSRP measurement accuracy | LTE Cell 1 | NR Cell 489 | NR Cell 1 |  |  |
| 5.7.1.2 | EN-DC FR2-FR2 SS-RSRP measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.7.1.3 | EN-DC FR1-FR2 SS-RSRP measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.7.2.1 | EN-DC FR2 SS-RSRQ measurement accuracy | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.7.2.2 | EN-DC FR2-FR2 SS-RSRQ measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.7.3.1 | EN-DC FR2 SS-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.7.3.2 | EN-DC FR2-FR2 SS-SINR measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.7.6.1 | EN-DC FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.7.6.2 | EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.7.6.3 | EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |

----- End of second change --------