3GPP TSG-RAN5 Meeting #98	R5-230460r1

# Athens, Greece, Feburary 27th - March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2171 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RedCap RRM TC 16.3.1.2 - intra known HO 2Rx with TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To add RedCap RRM TC 16.3.1.2- NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 2 Rx UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RedCap RRM TC 16.3.1.2- NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 2 Rx UE is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.1.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0244 TS 38.533 CR 2225, 2226 TS 38.903 CR 0472 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: To align with updated associated RAN4 CR (R4-2303185, revised from R4-2301823). |  |  |  |  |  |  |  |  |

<Start of modified section 1>

16.3.1.2	NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 2 Rx UE

16.3.1.2.1	Test purpose

To verify the requirement for the NR FR1-NR FR1 intra frequency handover requirements for 2 Rx RedCap UE as specified in 38.133 [6] clause 6.1D.1.2

16.3.1.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.3.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.3.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.1.2.

16.3.1.2.4	Test description

16.3.1.2.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.1.1.4.1 with the following exceptions:

-	Table 6.3.1.1.4.1-1 is replaced by Table 16.3.1.2.4.1-1.

-	Table 6.3.1.1.4.1-2 is replaced by Table 16.3.1.2.4.1-2.

-	Table 6.3.1.1.4.1-3 is replaced by Table 16.3.1.2.4.1-3

Table 16.3.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.1.2-1 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.1.2-2 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.1.2-3 | Source cell:NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.1.2-4 | Source cell:NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.3.1.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 16.3.1.2.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | -1 |  |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

16.3.1.2.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.1.1.4.2.

16.3.1.2.4.3	Message contents

Same as the message contents specified in 6.3.1.1.4.3 with following exceptions:

-	Table 7.3.1-3 in TS 38.508-1 [14] is used with condition SMTC.1.

16.3.1.2.5	Test requirement

Table 16.3.1.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 16.3.1.2.5-1: Cell specific test parameters

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  |  |  | 1 |  |  | 1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
|  |  | Config 4 |  | HD-FDD |  |  |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| BWP BW |  | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference |  | Config 1,4 |  | SR.1.1 FDD |  |  |  |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |  |  |  |  |
| CORESET Reference |  | Config 1,4 |  | CR.1.1 FDD |  |  |  |  |  |
| Channel |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  |  |  |  |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |  |  | SSB.1 RedCap FR1 |  |  |
| PDSCH/PDCCH subcarrier |  | Config 1,2,4 | kHz | 15 kHz |  |  |  |  |  |
| spacing |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PUCCH/PUSCH subcarrier |  | Config 1,2,4 | kHz | 15 kHz |  |  |  |  |  |
| spacing |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PRACH configuration |  |  |  | PRACH.1 FR1 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  |  | DLBWP.0.1 |  |  |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/ 15kHz | -98 |  |  |  |  |  |
| Note2 |  | Config 1,2,4 | dBm/ SCS | -98 |  |  |  |  |  |
|  |  | Config 3 |  | -95 |  |  |  |  |  |
|  |  |  | dB | 8 | -2.41 | -2.41 | -∞ | 1.36 | 1.36 |
|  |  |  | dB | 8 | 8 | 8 | -∞ | 10 | 10 |
| SSB_RP |  | Config 1,2,4 | dBm/ SCS | -90 | -90 | -90 | -∞ | -88 | -88 |
|  |  | Config 3 | dBm/ SCS | -87 | -87 | -87 | -∞ | -85 | -85 |
| IoNote3 |  | Config 1,2,4 | dBm/ 9.36MHz | -61.41 | -57.67 | -57.67 | -61.41 | -57.67 | -57.67 |
|  |  | Config 3 | dBm/ 18.36MHz | -58.49 | -54.74 | -54.74 | -58.49 | -54.74 | -54.74 |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  |  |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 92 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

-	RRC procedure delay = 10 ms, is the RRC processing delay specified in TS 38.331 [2] clause 12.

-	Tinterrupt = 62 ms, is the interruption time specified in TS 38.133 [6] clause 6.1D.1.2.

This gives a total of 72 ms.

<End of modified section 1>