3GPP TSG-RAN5 Meeting #98	R5-230609r3

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3510 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ATSSS test case 10.4.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test case 10.4.1.3 needs to be added according to ATSSS work plan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 10.4.1.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Updated the test case conformace requirements and test procedures. Revision 2: Updated confromance requirements to “Same Conformance requirements as in table10.4.1.1.2”. Revision 3: Corrected editorial mistake in the test case. |  |  |  |  |  |  |  |  |

10.4.1.3	UE-requested MA PDU session establishment / ATSSS / Registered to different PLMNs over 3GPP and non-3GPP accesses simultaneously/ Success

10.4.1.3.1	Test Purpose (TP)

(1)

with { UE in registered over 3GPP and non-3GPP accesses and SS informs the UE that it supports ATSSS feature in the 5GS network feature support information element }

ensure that {

when { UE is configured to request a new MA PDU session }

then { UE sends a PDU Session Establishment Request over one access and after this PDU Session is established, UE sends another PDU Session Establishment Request over the other access and provides the same PDU Session ID and Request Type as "MA PDU Request" in the UL NAS Transport message }

}

10.4.1.3.2	Conformance requirements

Same Conformance requirements as in clause 10.4.1.1.2.

10.4.1.3.3	Test description

10.4.1.3.3.1	Pre-test conditions

System Simulator:

-  WLAN Cell 27 is configured according to TS 38.508-1 [4]

-  NR Cell 1 is configured according to TS 38.508-1 [4], Table 4.4.2-3.

-  NR Cell 1 and WLAN Cell 27 belong to 2 different PLMNs and Both NR Cell 1 and WLAN cell 27 are set to '' Serving cell''.

UE:

- 	 UE is registered over both 3GPP access and non-3GPP access in 2 different PLMNs.

Preamble:

-	The UE is brought to state 3W-A and state 3N-A according to TS 38.508-1 [4], Table 4.4A.2-3

10.4.1.3.3.2	Test procedure sequence

Same Test procedure sequence as in clause10.4.1.1.3.2.

10.4.1.3.3.3	Specific message contents

Same Specific message contents as in clause 10.4.1.1.3.3.