3GPP TSG-RAN5 Meeting #98	R5-230622r1

# Athens, 27th Feb 2023- 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3518 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MBS Multicast TC 14.2.1.1.7-NACK-only |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MBS TC 14.2.1.1.7 are not completed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of MBS TC 14.2.1.1.7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MBS TC 14.2.1.1.7 can’t be executed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.1.1.7 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-2 CR 0317 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 1st revision: Correct the number of Table 14.2.1.1.8.3.3-8 to Table 14.2.1.1.7.3.3-8 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.2.1.1.7	MBS Multicast/ MAC / DL Data Transfer/ RRC-based enabling-disabling HARQ feedback for Multicast / NACK-only

14.2.1.1.7.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state and Multicast MRB established with RLC-UM entity for PTM transmission and HARQ feedback for Multicast with NACK-only mode is enabled by RRC }

ensure that {

when { UE receives downlink assignment with MAC PDU scheduled for UE's G-RNTI and successfully decodes it }

then { UE does not send ACK for the corresponding HARQ process and UE forwards MAC PDU to higher layer }

}

(2)

with { UE in RRC_Connected state and Multicast MRB established with RLC-UM entity for PTM transmission and HARQ feedback for Multicast with NACK-only mode is enabled by RRC }

ensure that {

when { UE receives downlink assignment with MAC PDU scheduled for UE's G-RNTI and decodes it failure }

then { UE sends NACK for the corresponding HARQ process }

}

(3)

with { UE in RRC_Connected state and Multicast MRB established with RLC-UM entity for PTM transmission and HARQ feedback for Multicast with NACK-only mode is enabled by RRC and pdsch-AggregationFactor > 1 }

ensure that {

when { UE receives downlink assignment on the PDCCH for the UE’s G-RNTI and receives data in the associated slot and successive pdsch-AggregationFactor – 1 HARQ retransmissions within a bundle and UE coud not successfully decode the data }

then { UE sends NACK on the HARQ process }

}

(4)

with { UE in RRC_Connected state and Multicast MRB established with RLC-UM entity for PTM transmission and HARQ feedback for Multicast is disabled by RRC }

ensure that {

when { UE receives downlink assignment with MAC PDU scheduled for UE's G-RNTI }

then { UE does not send HARQ feedback }

}

(5)

with { UE in RRC_Connected state and Multicast MRB established with RLC-UM entity for PTM transmission and HARQ feedback for Multicast with NACK-only mode is enabled by RRC and moreThanOneNackOnlyMode is configured }

ensure that {

when { UE needs to provide more than one HARQ-ACK information bits in a PUCCH }

then { UE provide more than one HARQ-ACK information bits in a PUCCH by selecting a resource from a set of resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in 38.213 Table 18-1 }

}

(6)

with { UE in RRC_Connected state and Multicast MRB established with RLC-UM entity for PTM transmission and HARQ feedback for Multicast with NACK-only mode is enabled by RRC and moreThanOneNackOnlyMode is not configured }

ensure that {

when { UE needs to provide more than one HARQ-ACK information bits in a PUCCH }

then { UE provide more than one HARQ-ACK information bits in a PUCCH according to ACK-NACK mode }

}

14.2.1.1.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.3.2; TS 38.213, clause 18; TS 38.214, clause 5.1.2.1; TS 38.331, clause 6.3.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321, clause 5.3.2]

For each received TB and associated HARQ information, the HARQ process shall:

1>	if the NDI, when provided, has been toggled compared to the value of the previous received transmission corresponding to this TB; or

…

2>	consider this transmission to be a new transmission.

1>	else:

2>	consider this transmission to be a retransmission.

The MAC entity then shall:

1>	if this is a new transmission:

2>	attempt to decode the received data.

1>	else if this is a retransmission:

2>	if the data for this TB has not yet been successfully decoded:

3>	instruct the physical layer to combine the received data with the data currently in the soft buffer for this TB and attempt to decode the combined data.

…

1>	if the HARQ process is associated with a transmission indicated with a G-RNTI or a G-CS-RNTI for MBS multicast and HARQ feedback is disabled; or

1>	if the HARQ process is associated with a transmission indicated with a G-RNTI or a G-CS-RNTI for MBS multicast and NACK only HARQ feedback is configured and the data for this TB is successfully decoded; or

…

1>	if the HARQ process is configured with disabled HARQ feedback:

2>	not instruct the physical layer to generate acknowledgement(s) of the data in this TB.

1>	else:

2>	instruct the physical layer to generate acknowledgement(s) of the data in this TB.

[TS 38.213, clause 18]

For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. The second HARQ-ACK reporting mode is not applicable for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration, or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception.

For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is one, a UE transmits a PUCCH only when the HARQ-ACK information bit has NACK value. The UE determines a PUCCH to provide the HARQ-ACK information as described in clause 9.2.1 when UE is not provided moreThanOneNackOnlyMode, or as the first PUCCH in Table 18-1 when UE is provided moreThanOneNackOnlyMode. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0.

A UE that is indicated the second HARQ-ACK reporting mode, and for the case when the UE reports more than one HARQ-ACK information bits, the UE can be indicated to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode when the UE is not provided moreThanOneNackOnlyMode or, for only one G-RNTI or only one G-CS-RNTI, according to the second HARQ-ACK reporting mode by selecting a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1 when the UE is provided moreThanOneNackOnlyMode. The UE generates HARQ-ACK information bits for the second HARQ-ACK reporting mode according to a Type-2 HARQ-ACK codebook as described in clause 9.1.3.1. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0.

For a UE that is indicated the second HARQ-ACK reporting mode, the UE does not expect to be provided pdsch-HARQ-ACK-Codebook = semi-static for multicast HARQ-ACK information.

For a UE that is indicated the second HARQ-ACK reporting mode and moreThanOneNackOnlyMode, all PUCCH resources associated with the second HARQ-ACK reporting mode have same starting symbol and same number of symbols and, when PUCCH resources in Table 18-1 are located in more than one PRBs, the more than one PRBs are adjacent and are associated with a same MPR value [8-1, TS 38.101-1].

Table 18-1: Mapping of values of HARQ-ACK information bits to PUCCH resources for the second HARQ-ACK reporting mode

| Value of HARQ-ACK information bits |  |  |  | PUCCH resource |
| --- | --- | --- | --- | --- |
| {0} | {0,0} | {0,0,0} | {0,0,0,0} | 1st PUCCH resource from resourceList/sps-PUCCH-AN-ListMulticast |
|  | {1,0} | {1,0,0} | {1,0,0,0} | 2nd PUCCH resource from resourceList/sps-PUCCH-AN-ListMulticast |
|  | {0,1} | {0,1,0} | {0,1,0,0} | 3rd PUCCH resource from resourceList/sps-PUCCH-AN-ListMulticast |
|  |  | {1,1,0} | {1,1,0,0} | 4th PUCCH resource from resourceList |
|  |  | {0,0,1} | {0,0,1,0} | 5th PUCCH resource from resourceList |
|  |  | {1,0,1} | {1,0,1,0} | 6th PUCCH resource from resourceList |
|  |  | {0,1,1} | {0,1,1,0} | 7th PUCCH resource from resourceList |
|  |  |  | {1,1,1,0} | 8th PUCCH resource from resourceList |
|  |  |  | {0,0,0,1} | 9th PUCCH resource from resourceList |
|  |  |  | {1,0,0,1} | 10th PUCCH resource from resourceList |
|  |  |  | {0,1,0,1} | 11th PUCCH resource from resourceList |
|  |  |  | {1,1,0,1} | 12th PUCCH resource from resourceList |
|  |  |  | {0,0,1,1} | 13th PUCCH resource from resourceList |
|  |  |  | {1,0,1,1} | 14th PUCCH resource from resourceList |
|  |  |  | {0,1,1,1} | 15th PUCCH resource from resourceList |

[TS 38.214, clause 5.1.2.1]

When receiving PDSCH scheduled by DCI format 4_1, or 4_2 in PDCCH with CRC scrambled by G-RNTI for multicast, if the UE is configured with pdsch-AggregationFactor in the MBS-RNTI-SpecificConfig associated with the corresponding G-RNTI for multicast, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots.

[TS 38.331, clause 6.3.2]

moreThanOneNackOnlyMode

Indicates the mode of supporting more than one NACK-only feedback in the same PUCCH transmission. Mode 1 means UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. Mode 2 means UE transmitting a specific sequence or a PUCCH transmission corresponding to the combination of more than one NACK-only HARQ feedback. If multicast CFR is not configured, this field is not included. Otherwise, if the field is absent, UE uses mode 1 for multicast CFR.

14.2.1.1.7.3	Test description

14.2.1.1.7.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	The SS configures the NR Cell 1 as the "Serving cell".

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-3 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

-	The UE is made interested in receiving MBS Multicast service with MBS service ID '000101'H.

14.2.1.1.7.3.2	Test procedure sequence

Table 14.2.1.1.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1a1-1b12a1 | Steps 1a1 to 1b12a1 of the generic procedures described in TS 38.508-1 subclause 4.9.34 are performed on NR Cell 1 to establish an associated PDU Session to the MBS DNN and join in MBS Multicast session. | - | - | - | - |
| 2a1-2a2 | Steps 9a1 to 9a2 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 1 with condition UE TEST LOOP MODE C and Multicast MRB. | - | - | - | - |
| 3 | The SS indicates a new transmission addressed to the G-RNTI assigned to the UE. | <-- | (PDCCH (G-RNTI)) | - | - |
| 4 | The SS transmits a MBS Packet on the MTCH with LCID matched with the LCID configured for receving PTM transmission. The CRC is calculated in such a way, it will result in CRC pass on UE side. | <-- | MBS Packet | - | - |
| 5 | Check: Does the UE transmit a HARQ ACK/NACK? | --> | HARQ ACK/NACK | 1 | F |
| 6 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 7 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 8 | Check: Is the number of reported MBS Packets received on the MRB in step 7 equal to 1? | - | - | 1 | P |
| 9 | The SS indicates a new transmission addressed to the G-RNTI assigned to the UE. | <-- | (PDCCH (G-RNTI)) | - | - |
| 10 | The SS transmits a MBS Packet on the MTCH with LCID matched with the LCID configured for receving PTM transmission. The CRC is calculated in such a way, it will result in CRC error on UE side. | <-- | MBS Packet | - | - |
| 11 | Check: Does the UE transmit a HARQ NACK? | --> | HARQ NACK | 2 | P |
| 12 | The SS indicates a new transmission addressed to the G-RNTI assigned to the UE in SFN x, slot 2 and the "PDSCH-to-HARQ_feedback timing indicator" is configured to require UE to feedback HARQ-ACK information in SFN x, slot 9. | <-- | (PDCCH (G-RNTI)) | - | - |
| 13 | The SS transmits a MBS Packet on the MTCH with LCID matched with the LCID configured for receving PTM transmission. The CRC is calculated in such a way, it will result in CRC pass on UE side. | <-- | MBS Packet | - | - |
| 14 | The SS indicates a new transmission addressed to the G-RNTI assigned to the UE in SFN x, slot 5 and the "PDSCH-to-HARQ_feedback timing indicator" is configured to require UE to feedback HARQ-ACK information in SFN x, slot 9. | <-- | (PDCCH (G-RNTI)) | - | - |
| 15 | The SS transmits a MBS Packet on the MTCH with LCID matched with the LCID configured for receving PTM transmission. The CRC is calculated in such a way, it will result in CRC pass on UE side. | <-- | MBS Packet | - | - |
| 16 | Check: Does the UE transmit {HARQ ACK, HARQ ACK}? | --> | {HARQ ACK, HARQ ACK} | 6 | P |
| 17 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 18 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 19 | Check: Is the number of reported MBS Packets received on the MRB in step 18 equal to 3? | - | - | 6 | P |
| 20 | The SS transmits RRCReconfiguration to configure pdsch-AggregationFactor-r17 to n4 for multicast. | <-- | NR RRC: RRCReconfiguration | - | - |
| 21 | The UE transmits RRCReconfigurationComplete. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 22 | The SS indicates a new transmission addressed to the G-RNTI assigned to the UE. | <-- | (PDCCH (G-RNTI)) | - | - |
| 23 | The SS transmits a MBS Packet on the MTCH with LCID matched with the LCID configured for receving PTM transmission. The CRC is calculated in such a way, it will result in CRC error on UE side. | <-- | MBS Packet. | - | - |
| 24 | In the following 3 consecutive slots, the SS transmits same MBS Packet in step 23. The CRC is calculated in such a way, it will result in CRC error on UE side. | <-- | MBS Packet. | - | - |
| 25 | Check: Does the UE transmit a HARQ NACK on slot n3+k1? (Note 2) | --> | HARQ NACK | 3 | P |
| 26 | The SS transmits RRCReconfiguration to disable HARQ feedback for multicast | <-- | NR RRC: RRCReconfiguration | - | - |
| 27 | The UE transmits RRCReconfigurationComplete. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 28 | The SS indicates a new transmission addressed to the G-RNTI assigned to the UE. | <-- | (PDCCH (G-RNTI)) | - | - |
| 29 | The SS transmits a MBS Packet on the MTCH with LCID matched with the LCID configured for receving PTM transmission. The CRC is calculated in such a way, it will result in CRC pass on UE side. | <-- | MBS Packet | - | - |
| 30 | Check: Does the UE transmit a HARQ ACK/NACK? | --> | HARQ ACK/NACK | 4 | F |
| 31 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 32 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 33 | Check: Is the number of reported MBS Packets received on the MRB in step 32 equal to 4? | - | - | 4 | P |
| - | EXCEPTION: Steps 34a1-34a12 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place if pc_nack_OnlyFeedbackSpecificResourceForMulticast-r17 is configured | - | - | - | - |
| 34a1 | The SS transmits RRCReconfiguration to enable NACK only HARQ feedback for multicast and configure moreThanOneNackOnlyMode | <-- | NR RRC: RRCReconfiguration | - | - |
| 34a2 | The UE transmits RRCReconfigurationComplete. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 34a3 | The SS indicates a new transmission addressed to the G-RNTI assigned to the UE in SFN y, slot 2 and the "PDSCH-to-HARQ_feedback timing indicator" is configured to require UE to feedback HARQ-ACK information in SFN y, slot 9. | <-- | (PDCCH (G-RNTI)) | - | - |
| 34a4 | The SS transmits a MBS Packet on the MTCH with LCID matched with the LCID configured for receving PTM transmission. The CRC is calculated in such a way, it will result in CRC pass on UE side. | <-- | MBS Packet | - | - |
| 34a5 | The SS indicates a new transmission addressed to the G-RNTI assigned to the UE in SFN y, slot 5 and the "PDSCH-to-HARQ_feedback timing indicator" is configured to require UE to feedback HARQ-ACK information in SFN y, slot 9. | <-- | (PDCCH (G-RNTI)) | - | - |
| 34a6 | The SS transmits a MBS Packet on the MTCH with LCID matched with the LCID configured for receving PTM transmission. The CRC is calculated in such a way, it will result in CRC pass on UE side. | <-- | MBS Packet | - | - |
| 34a7 | Check: Does the UE transmit HARQ ACK/NACK? | --> | {HARQ ACK, HARQ ACK} | 5 | F |
| 34a8 | The SS indicates a new transmission addressed to the G-RNTI assigned to the UE in SFN z, slot 2 and the "PDSCH-to-HARQ_feedback timing indicator" is configured to require UE to feedback HARQ-ACK information in SFN z, slot 9. | <-- | (PDCCH (G-RNTI)) | - | - |
| 34a9 | The SS transmits a MBS Packet on the MTCH with LCID matched with the LCID configured for receving PTM transmission. The CRC is calculated in such a way, it will result in CRC pass on UE side. | <-- | MBS Packet | - | - |
| 34a10 | The SS indicates a new transmission addressed to the G-RNTI assigned to the UE in SFN z, slot 5 and the "PDSCH-to-HARQ_feedback timing indicator" is configured to require UE to feedback HARQ-ACK information in SFN z, slot 9. | <-- | (PDCCH (G-RNTI)) | - | - |
| 34a11 | The SS transmits a MBS Packet on the MTCH with LCID matched with the LCID configured for receving PTM transmission. The CRC is calculated in such a way, it will result in CRC fail on UE side. | <-- | MBS Packet | - | - |
| 34a12 | Check: Does the UE transmit {HARQ ACK, HARQ NACK} in the 2nd PUCCH resource from resourceList? | --> | {HARQ ACK, HARQ NACK} | 5 | P |
| 34a13 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 34a14 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 34a15 | Check: Is the number of reported MBS Packets received on the MRB in step 34a14 equal to 7? | - | - | 5 | P |
| Note 1: The value 4 for the maximum number of HARQ retransmissions has been chosen based on an assumption that, given the radio conditions used in this test case, a UE soft combiner implementation should have sufficient retransmissions to be able to successfully decode the data in its soft buffer. Note 2: n0 is the index of slot when 1st transmission of MBS Packet in step 23 happens, n1, n2, n3 are indices of slots when 2nd, 3rd, 4th transmission of MBS Packet in step 24 may happen, k1 is obtained from "PDSCH-to-HARQ_feedback timing indicator" of downlink assignment in step 22. |  |  |  |  |  |

14.2.1.1.7.3.3	Specific message contents

Table 14.2.1.1.7.3.3-1: ACTIVATE TEST MODE (preamble, Table 14.2.1.1.7.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.2.1.1.7.3.3-2: RRCReconfiguration (step 1a15, Table 14.2.1.1.7.3.2-1)

| Derivation Path: TS 38.508-1 [4],Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition MRBm and UM_PTM | m=1 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 14.2.1.1.7.3.3-4 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.1.1.7.3.3-3: RRCReconfiguration (step 1b10, Table 14.2.1.1.7.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition DRBn and MRBm and UM_PTM | n is chosen as the next available number higher or equal to 2 m=1 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 14.2.1.1.7.3.3-5 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.1.1.7.3.3-4: CellGroupConfig (Table 14.2.1.1.7.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19, condition MRBm (m=1) and UM_PTM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig with condition MBS_Multicast and RRC_Enable_HARQFeedback and NACK_ONLY |  |  |
| } |  |  |  |

Table 14.2.1.1.7.3.3-5: CellGroupConfig (Table 14.2.1.1.7.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19, condition MRBm_DRBn and UM_PTM (Note 1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig with condition MBS_Multicast and RRC_Enable_HARQFeedback and NACK_ONLY |  |  |
| } |  |  |  |
| Note 1: n is set to the same value as for the radioBearerConfig IE in Table 14.2.1.1.7.3.3-3 and m=1 |  |  |  |

Table 14.2.1.1.7.3.3-6: CLOSE UE TEST LOOP (step 2a1, Table 14.2.1.1.7.3.2-1)

Derivation Path: 38.508-1 [4], Table 4.7A-3, condition UE TEST LOOP MODE C and Multicast MRB

Table 14.2.1.1.7.3.3-7: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST (step 6, step 17, step31 and step 34a14, Table 14.2.1.1.7.3.2-1)

Derivation Path: 36.508 [6], Table 4.7A-9

Table 14.2.1.1.7.3.3-8: Physical layer parameters for DCI format 4_1 ( step 12, step 14, step 34a3, step34a5, step34a8 and step 34a10, Table 14.2.1.1.8.3.2-1)

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| PDSCH-to-HARQ_feedback timing indicator | K1 = 7 | “110” | Step 12, Step34a3 and Step 34a8 |
|  | K1 = 4 | “011” | Step 14, Step34a5 and Step 34a10 |
| Note: K1 set for DCI 4_1 is { 1, 2, 3, 4, 5, 6, 7, 8 } because dl-DataToUL-ACK-MulticastDCI-Format4-1 is not provided |  |  |  |

Table 14.2.1.1.7.3.3-9: RRCReconfiguration (step 20, Table 14.2.1.1.7.3.2-1)

| Derivation Path: TS 38.508-1 [4],Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 14.2.1.1.7.3.3-10 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.1.1.7.3.3-10: CellGroupConfig (Table 14.2.1.1.7.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| rlc-BearerToReleaseList | Not present |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig | Table 14.2.1.1.7.3.3-11 |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig | Not present |  |  |
| } |  |  |  |

Table 14.2.1.1.7.3.3-11: MAC-CellGroupConfig (Table 14.2.1.1.7.3.3-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-68, condition MBS_Multicast and RRC_Enable_HARQFeedback and ACK_NACK |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| g-RNTI-ConfigToAddModList-r17 SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF MBS-RNTI-SpecificConfig-r17 { | 1 entry |  |  |
| MBS-RNTI-SpecificConfig-r17[1] SEQUENCE { |  | entry 1 |  |
| pdsch-AggregationFactor-r17 | n4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.1.1.7.3.3-12: RRCReconfiguration (step 26, Table 14.2.1.1.7.3.2-1)

| Derivation Path: TS 38.508-1 [4],Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 14.2.1.1.7.3.3-13 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.1.1.7.3.3-13: CellGroupConfig (Table 14.2.1.1.7.3.3-12)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| rlc-BearerToReleaseList | Not present |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig with condition MBS_Multicast |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig | Not present |  |  |
| } |  |  |  |

Table 14.2.1.1.7.3.3-14: RRCReconfiguration (step 34a1, Table 14.2.1.1.7.3.2-1)

| Derivation Path: TS 38.508-1 [4],Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 14.2.1.1.7.3.3-15 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.1.1.7.3.3-15: CellGroupConfig (Table 14.2.1.1.7.3.3-14)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| rlc-BearerToReleaseList | Not present |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig with condition MBS_Multicast |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 14.2.1.1.7.3.3-16 |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.1.1.7.3.3-16: ServingCellConfig (Table 14.2.1.1.7.3.3-15)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| tdd-UL-DL-ConfigurationDedicated | Not present |  |  |
| initialDownlinkBWP | Not present |  |  |
| downlinkBWP-ToReleaseList | Not present |  |  |
| downlinkBWP-ToAddModList | Not present |  |  |
| firstActiveDownlinkBWP-Id | Not present |  |  |
| bwp-InactivityTimer | Not present |  |  |
| defaultDownlinkBWP-Id | Not present |  |  |
| uplinkConfig | Not present |  |  |
| supplementaryUplink | Not present |  |  |
| pdcch-ServingCellConfig | Not present |  |  |
| pdcch-ServingCellConfig | Not present |  |  |
| pdsch-ServingCellConfig | Not present |  |  |
| csi-MeasConfig | Not present |  |  |
| sCellDeactivationTimer | Not present |  |  |
| crossCarrierSchedulingConfig | Not present |  |  |
| tag-Id | 0 |  |  |
| dummy1 | Not present |  |  |
| pathlossReferenceLinking | Not present |  |  |
| servingCellMO | Not present |  |  |
| moreThanOneNackOnlyMode-r17 | mode2 |  |  |
| } |  |  |  |

<End of modified section 1>